交通部運輸研究所

合作研究計畫出版品摘要表

| 出版品名稱：高速公路出口匝道區域車流特性資料收集與分析 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 國際標準書號(或叢刊號) |  |  | 政府出版品統一編號 |  |  | 運輸研究所出版品編號 |  |
| 本所主辦單位：運輸計畫組 主管： 計畫主持人： 研究人員：蘇振維 聯絡電話：(02)349-6806 傳真號碼：(02)545-0428 |  |  |  | 合作研究單位：鼎漢國際工程顧問股份有限公司 計畫主持人：王國材 研究人員：徐宏昇、唐文斌、翁忠川、 黃淑美 地址：台北市信義路四段306號13樓 聯絡電話：(02)2704-4369 ext 34 |  |  | 研究期間 自85年9月 至86年6月 |
| 關鍵詞：高速公路、出口匝道、減速車道、流率、速率、佔有率、容量 |  |  |  |  |  |  |  |
| 摘要： 本研究主要目的在於收集高速公路出口匝道區域內(含主線及匝道)車流特性資料及出口匝道號誌化路口停等資料，工作共分為前、後兩階段，第一階段重點在於尋找出口匝道區域內主線服務水準最差地點處並評估Autoscope系統自動偵測之適用性，第二階段工作則根據上一階段所發現之地點進行調查，並分析出口匝道區域內主線各車道及匝道中點之主要車流參數，如速率、流率及佔有率等資料，以建立各參數間之基本關係圖。同時，利用各車道之調查流量以評選後之最適方法構建出口匝道區域內車道車輛分佈模式。 由本研究之初步研究成果發現， 高速公路出口匝道區域內，服務水準最差地點約莫發生於減速車道終點區域。而出口匝道區域尖峰時段內紊亂之車流行為且因 Autoscope系統穩定度尚未證實，經檢討後並不適用自動偵測方式分析資料，而改採人工判讀方式進行分析。從所得之車流參數可知，出口匝道區域內主線車道之運作，的確受到出口匝道車輛行為之影響。另外，由本研究所建立之車道車輛分佈模式，經檢定後發現，除外側車道外其他車道均可通過檢定，但若將最外側兩車道流率予以合併後模式將可獲得較佳之解釋能力。 |  |  |  |  |  |  |  |
| 出版日期 | 頁數 | 工本費 |  |  | 本出版品取得方式 |  |  |
| 88年4月 |  |  |  |  | 凡屬機密或限閱性出版品均不對外公開。一般性出版品，公營、公益機關團體及學校可函洽本所免費贈閱；私人及私營機關團體可按工本費價購。 |  |  |
| 管制等級： □機密 (□解密日期 年 月 日□主辦單位視情況辦理解密) □限閱 (□解限日期 年 月 日□主辦單位視情況辦理解限) □一般 |  |  |  |  |  |  |  |
| 備註：本研究之結論與建議不代表交通部之意見 |  |  |  |  |  |  |  |

PUBLICATION ABSTRACT OF RESEARCH PROGRAM

INSTITUTE OF TRANSPORTATION

MINISTRY OF TRANSPORTATION AND COMMUNICATIONS

| TITLE： The Study on the Flow Characteristics in the Off-ramp Area of Urban Expressway and Freeway |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ISBN(OR ISSN) | UNIFORM SERIAL CODE FOR GOVERNMENT PUBLICATIONS |  |  |  | IOT SERIAL NUMBER |
| DIVISION：Transportation Planning Division DIVISION CHIEF： PRINCIPAL INVESTIGATOR： PROJECT STAFF: Cheng-Wei Su PHONE：(02)349-6806 FAX：(02)545-0428 |  |  |  |  | PROJECT PERIOD FROM September, 1996 TO June, 1997 |
| RESEARCH AGENCY：THI Consultants Inc. PRINCIPAL INVESTIGATOR：Kwo-Tsai Wang PROJECT STAFF：Hung-Sheng Hsu,Wen-Ping Tang,Chung-Chuan Weng, May Wang ADDRESS：13F-1,No.306,Sec 4,Shin-Yi Road, Taipei, Taiwan, R.O.C. PHONE：(02)2704-4369 ext 34 |  |  |  |  |  |
| KEY WORDS：Freeway, Off-Ramp, Flow Rate, Speed, Occupancy, Capacity |  |  |  |  |  |
| ABSTRACT: The purpose of this project is to collect data of flow characteristics in off-ramp area of highway and in signalized intersections of off-ramp . This project is divided into two phases. The first phase is to identify the critical point in off-ramp area, also to evaluate whether the Autoscope system is adaptable or not. The main work of second phase is to collect data within those areas identified from the former phase, and to analyze some variables of flow ,like speed(V)、flow rate(Q)、occupancy(Occ%) .etc., in order to find the basic relationships among them. At the same time ,it is also expected to utilize the survey volume to build a vehicle distribution model of each main lane in off-ramp area by an appropriate methodology. It is concluded from the research of this project that the critical point in off-ramp area occurs in the end of the exit lane . The Autoscope system is not suitable for the data collection work in this project because of two reasons: 1.the behavior of vehicles are out of control within the peak hours in off-ramp areas 2.the reliability of this system hasn’t been proved. As a result, analysis of data in this project is all done by man-power. The operation of traffic flow of main lanes is definitely influenced by the diverge flow of exit lane from this study. In addition, the vehicle distribution model built by stepwise regression in this project has a good result of calibration except the outer lanes. However, the result is much better if the two outer lanes are combined into one when running this model. |  |  |  |  |  |
| DATE OF PUBLICATION December, 1998 |  | NUMBER OF PAGE | PRICE | CLASSIFICATION □SECRET □CONFIDENTIAL □UNCLASSIFIED |  |
| The views expressed in this publication are not necessarily those of Ministry of Transportation and Communications |  |  |  |  |  |