交通部運輸研究所出版品摘要表

| 出版品名稱：交通寧靜區之交通工程方法應用 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 國際標準書號（或叢刊號） |  |  |  | 政府出版品統一編號 1009100293 | 運輸研究所出版品編號 91-5-3218 |  |
| 主辦單位：運輸安全組 主管：林豐福 計畫主持人：林豐福 研究人員：葉祖宏 聯絡電話：（02）2349-6856 傳真號碼：（02）2545-0429 |  |  |  |  |  | 研究期間 自88年08月 至89年12月 |
| 關鍵詞：交通寧靜方法、交通安全、穿越性交通、公共參與、減速丘、設置準則、設置條件 |  |  |  |  |  |  |
| 摘要： 交通寧靜方法的發展，可謂高度工業化國家早期過度重視道路建設發展與私人機動車輛急速增加而造成居民生活品質惡化後，民眾與政府單位逐漸重視環境之「可居性」與「安全性」的一項反思過程。為利於國際間溝通及研究發展，ITE於1997年對交通寧靜區所下定義為：「降低機動車輛使用的負效果，改變駕駛人行為與改善街道上非機動車輛使用者環境的重要實體設施組合。」 本研究主要針對各國交通寧靜區之發展歷程與設置規範、規劃程序、民眾參與、設置方法、設置方法選擇與效果、設置條件與準則、及其他相關主要議題等分別進行探討，以提供國內推動相關概念之參考。並建議國內值得重視交通寧靜方法的發展與應用的理由應建構在人本交通觀念漸受重視、交通安全改善標的之移轉、及傳統交通工程方法之改善極限等基本邏輯上。最後針對未來國內推動交通寧靜方法所可能面臨的問題，提出七項推動課題包括建立明確道路等級分類、建立本土性之研究課題、考量設置準則定位、由點、線、面逐步擴大實施規模、建立設置交通寧靜區之民眾參與機制、加強交通寧靜方法觀念宣導、及列入常態性改善計畫推動等，作為國內發展交通寧靜方法的建議。 |  |  |  |  |  |  |
| 出版日期 | 頁數 | 工本費 | 本出版品取得方式 |  |  |  |
| ９1年1月 | 84 |  | 凡屬機密性出版品均不對外公開。普通性出版品，公營、公益機關團體及學校可函洽本所免費贈閱；私人及私營機關團體可按工本費價購。 |  |  |  |
| 機密等級： □限閱 □密 □機密 □極機密 □絕對機密 （解密【限】條件：□ 年 月 日解密，□公布後解密，□附件抽存後解密， □工作完成或會議終了時解密，□另行檢討後辦理解密） 普通 |  |  |  |  |  |  |
| 備註：本研究之結論與建議不代表交通部之意見。 |  |  |  |  |  |  |

PUBLICATION ABSTRACTS OF RESEARCH PROJECTS

# INSTITUTE OF TRANSPORTATION

MINISTRY OF TRANSPORTATION AND COMMUNICATIONS

| TITLE: Preliminary Study of Traffic Calming Measures |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ISBN(OR ISSN) |  | GOVERNMENT PUBLICATIONS NUMBER 1009100293 |  | IOT SERIAL NUMBER 91-5-3218 |  |
| DIVISION: Transportation Safety Division DIVISION CHIEF: Fong-Fu Lin PRINCIPAL INVESTIGATOR: Fong-Fu Lin PROJECT STAFF: Tsu-Hurng Yeh PHONE: 886-2-23496856 FAX: 886-2-25450429 |  |  |  | PROJECT PERIOD FROM August 1999 TO December 2000 |  |
| KEY WORDS: Traffic Calming, Traffic Safety, Through Traffic, Public Involvement, Speed Humps, Guidelines, Warrants |  |  |  |  |  |
| ABSTRACT： The development of Traffic Calming was a rethinking process for rebuilding a livable and safe environment in industrialized countries, which had heavily developed the road systems for motor vehicles and failed to control the amount of motor vehicles properly. To enhance communication and research development between different countries, the Institute of Transportation Engineering (ITE) has defined Traffic Calming in 1997 as “the combination of mainly physical measures that reduce the negative effects of motor vehicles use, alter drivers’ behavior and improve conditions for non-motorized users.” Firstly, this study focused on the introduction of related Traffic Calming concepts among advanced countries, including the evolutionary history and installation guidelines, the planning process, public involvement, traffic engineering measures, the choice suggestion for measures and their effectiveness, the installation warrants and criteria, and goal negotiation with emergency vehicles. Secondly, this study suggested the basic logic for Traffic Calming measures to be implemented in Taiwan. There are three reasons: the protection of vulnerable users such as pedestrians and cyclists is becoming more important, the improving safety focus is shifting to local streets and collector roads, and the limitation of traditional traffic engineering methods needs a breakthrough. Finally, to overcome the problems hindering the Traffic Calming implementation in Taiwan, six issues were suggested as follows: 1.constructing the specific road classification, 2.implementing the locally related study topics, 3.considering the locus of Traffic Calming guidelines, 4.accumulating the installing areas step by step, 5.designing the public involvement mechanism, 6.enhancing the promotion of Traffic Calming concepts, and 7.integrating into the long-term administrative plan. |  |  |  |  |  |
| DATE OF PUBLICATION January 2002 | NUMBER OF PAGES 84 |  | PRICE |  | CLASSIFICATION □SECRET □CONFIDENTIAL UNCLASSIFIED |
| The views expressed in this publication are not necessarily those of the Ministry of Transportation and Communications. |  |  |  |  |  |

目錄

| 第一章 緒論…………..…………………………………………………… 1.1 研究緣起………..……..…………………………………………. 1.2 研究目的……………………………………………………………. 1.3 研究方法與內容….………………………………………………… 第二章 交通寧靜區相關文獻回顧………………………………………… 2.1 交通寧靜區概述與設置目標……………….……………………… 2.2 交通寧靜區發展歷程與設置規範………….……………………… 2.3 交通寧靜區之規劃程序…………………….……………………… 2.4 公眾參與之重要性…………………………………………………. 2.5 交通寧靜區之主要交通工程方法…………………………………. 2.6 交通寧靜區設置條件分析…………………………………………. 2.7 其他重要相關課題…………………………………………………. 第三章 交通寧靜區之設置方法與效果分析……………………………… 3.1 流量管制方法………………………………………………………. 3.2 速率管制方法………………………………………………………. 3.3 交通寧靜方法之選擇………………………………………………. 3.4 交通寧靜方法之設置效果…………………………………………. 3.5 交通寧靜方法之發展趨勢…………………………………………. 第四章 國內推動交通寧靜區之發展課題………………………………… 4.1 國內發展交通寧靜區之需要性……………………………………. 4.2 國內發展交通寧靜區所面臨問題…………………………………. 4.3 國內推動交通寧靜區之發展課題建議……………………………. 第五章 結論與建議………………………………………………………… 5.1 結論…………………………………………………………………. 5.2 建議…………………………………………………………………. 參考文獻……………………………………………………………………. 附錄 簡報資料（P1~P15） | 1 1 2 2 5 5 7 12 16 18 19 22 27 27 30 37 42 45 49 49 50 52 57 57 58 59 |
| --- | --- |

圖目錄

| 圖1 澳洲LATM之規劃程序…………………………………………… 圖2 日本社區區劃計畫之規劃程序…………………………………… 圖3 街道全封閉設施…………………………………………………… 圖4 街道半封閉設施……………………………………………………. 圖5 路口轉向半封閉設施………………………………………………. 圖6 路口對角線封閉設施………………………………………………. 圖7 中央分隔阻斷路口設施……………………………………………. 圖8 強迫轉向槽化島……………………………………………………. 圖9 減速丘(12英呎)側面圖……………………………………………. 圖10 減速台(22英呎)側面圖……………………………………………. 圖11 墊高的行人穿越設施………………………………………………. 圖12 墊高的交岔路口……………………………………………………. 圖13 紋理化舖面設施……………………………………………………. 圖14 鄰里圓環…………………………………………………………. 圖15 圓環…………………………………………………………………. 圖16 波紋路型設施………………………………………………………. 圖17 交岔路口線形修正設計……………………………………………. 圖18 路口寬度縮減設施…………………………………………………. 圖19 路面寬度縮減之中央分隔島設施………………………………………. 圖20 路段寬度縮減設施…………………………………………………. | 13 15 27 28 28 29 29 30 31 32 32 33 33 34 34 35 35 36 36 37 |
| --- | --- |

表目錄

| 表1 設置交通寧靜區之目標與標的………………………………..…….. 表2 澳洲LATM程序中社區居民參與技巧之應用效果………….…….. 表3 Sarasota市之一般性設置授權條件………………………………….. 表4 Montgomery郡之減速丘設置授權條件…………………………….. 表5 減速丘設置條件要求（42個調查機構）……….………………….. 表6 Bellevue市之多項交通寧靜方法「控制矩陣」…………………….. 表7 美國部份都市緊急反應時間目標………………………………..….. 表8 美國測試交通寧靜方法對緊急救援車輛所造成之延滯效果研究.... 表9 加拿大交通寧靜區主要設置方法間之潛在優點比較……………… 表10 加拿大交通寧靜區主要設置方法間之潛在缺點比較……………… 表11 澳洲交通寧靜方法與交通管理策略應用矩陣……………………… 表11 澳洲交通寧靜方法與交通管理策略應用矩陣（續）……………… 表12 相關研究中交通寧靜區交通工程方法之設置效果………………… 表13 不同交通寧靜方法對速度之影響…………………………………… 表14 不同交通寧靜方法對流量之影響…………………………………… 表15 不同交通寧靜方法對事故碰撞次數之影響……………………….. | 5 18 20 21 21 22 23 24 38 39 40 41 42 43 44 45 |
| --- | --- |