國際港裝卸機具調查研究

交通部運輸研究所

財團法人中華顧問工程司

合作辦理

中華民國八十九年四月

交通部運輸研究所合作研究計畫出版品摘要表

| 出版品名稱：國際港裝卸機具調查研究 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 國際標準書號（或叢刊號） |  |  |  |  | 政府出版品統一編號 009104890314 | 運輸研究所出版品編號 89-31-264 |  |
| 本所主辦單位：運輸工程組 主管：曾志煌 計畫主持人：曾志煌 研究人員：翁國和、張昭芸 聯絡電話：(02)2349-6821、24 傳真號碼：(02)2545-0427 |  |  |  | 合作研究單位：財團法人中華顧問工程司 計畫主持人：劉宏道 研究人員：廖學瑞、朱志誠、張欽森、張徐鍚、孫君偉、陳建中 地址：台北市辛亥路二段185號28樓 聯絡電話：(02)2736-3567 |  |  | 研究期間 自87年 8月 至88年 6月 |
| 關鍵詞：裝卸機具、資料庫系統、作業效率 |  |  |  |  |  |  |  |
| 摘要：本計畫係針對台灣各國際港裝卸機具之硬體現況、作業效率及APEC港埠資料庫系統等項目作一探討及介紹，而得如下總結： 1. 高雄港及台中港因為開放部份碼頭給民營業者專用，所以此二港有部份機具為民營業者所自購；基隆港、花蓮港及蘇澳港則皆為港務局自購。 2. 各港機具可用率大部份可維持在80%以上，顯示機具狀況良好；而使用率方面，則是以民營業者之機具使用率較高。在各港裝卸作業逐步開放由民營業者辦理之時，應該增購高效率之機具，全面汰除老舊過期之設備，以增加競爭力。 3. 各港裝卸作業效率資料蒐集不易；貨櫃起重機之作業毛效率上，各港差異不大，而淨效率則是以民營業者較高。在作業資料蒐集方面應該建立定期回報資料之制度，同時可在碼頭出租契約或開放特許發放許可證時要求業者提供相關資料，甚至可由交通部修正棧埠管理規則，強制業者辦理申報手續。 4. 建立初步之機具資料庫系統，以方便使用者之查詢，未來並可結合網際網路(Internet)之特性，建立網路資料庫系統，增加資料流通之便利性，同時也有利於資料之更新。 |  |  |  |  |  |  |  |
| 出版日期 | 頁數 | 工本費 | 本出版品取得方式 |  |  |  |  |
| 89年5月 | 290 |  | 凡屬機密性出版品均不對外公開。普通性出版品，公營、公益機關團體及學校可函洽本所免費贈閱；私人及私營機關團體可按工本費價購。 |  |  |  |  |
| 機密等級： □限閱 □密 □機密 □極機密 □絕對機密 （解密【限】條件：□ 年 月 日解密，□公布後解密，□附件抽存後解密， □工作完成或會議終了時解密，□另行檢討後辦理解密） □普通 |  |  |  |  |  |  |  |
| 備註：本研究之結論與建議不代表交通部之意見。 |  |  |  |  |  |  |  |

PUBLICATION ABSTRACTS OF RESEARCH PROJECTS

#### INSTITUTE OF TRANSPORTATION

MINISTRY OF TRANSPORTATION AND COMMUNICATIONS

| TITLE: THE INVESTIGATION OF LOADING AND UNLOADING FACILITIES OF INTERNATIONAL COMMERCIAL HARBORS IN TAIWAN |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ISBN(OR ISSN) |  | GOVERNMENT PUBLICATIONS NUMBER 009104890314 |  | IOT SERIAL NUMBER 89-31-264 |  |
| DIVISION: Transportation Engineering DIVISION CHIEF:C. H. Tseng PRINCIPAL INVESTIGATOR:C. H. Tseng PROJECT STAFF:G. H. Wung、C. Y. Chang PHONE:(02)2349-6821、24 FAX:(02)2545-0427 |  |  |  | PROJECT PERIOD FROM 1988.8 TO 1999.6 |  |
| RESEARCH AGENCY: China Engineering consultants, Inc. PRINCIPAL INVESTIGATOR:H. D. Liu PROJECT STAFF: S.R. Liaw, J.C. Chu, C.C. Chang, C.C. Chang, C.W. Sun, C.C. Chen ADDRESS: 30FL. 185 Hsin-Hai Rd. Sec. 2 Taipei Taiwan, R.O.C. PHONE:(02)2736-3567 |  |  |  |  |  |
| KEY WORDS: load and unload machine, database system, produce efficiency |  |  |  |  |  |
| ABSTRACT: The project is an investigation and introduction of the existing loading & unloading facilities in Taiwan’s international ports and the APEC harbor database system. The conclusion is shown as follows. - In Kaohsiung and Taichung ports, private companies rent some of the wharves, for which the private companies bought some of the loading and unloading facilities by themselves, but the condition is totally different in Keelung, Hualien and Suao ports. The port bureaus own the whole loading & unloading facilities. - From investigation, all the available ratio of facilities remains above 80% in every port. This states that all facilities are well-maintained in ports. In operation efficiency, the facilities owned by the private sector are higher than those owned by the port bureau. Hence, to increase the competition ability of ports, it should replace old and inefficient facilities to new higher efficient facilities when the wharves are gradually opened to the private sector. - Although the gross operation efficiency of gantry cranes is not too different among ports, the net operation efficiency of gantry cranes owned by private wharves is significantly higher than those owned by the port bureau. Meanwhile, to collect the accurate data of operating facilities is difficult. To increase the competition of each port in Taiwan, building a period data feedback system is necessary. This can be achieved by requesting the companies to provide the related information when the rent agreement was signed or the operating permit was authorized. In addition, the Transport Department may modify the rules of cabin management to force the private companies to provide the regular information when their ships approach and are then operated on the wharf. - The initial database system should be built as soon as possible when the port bureaus enhance the efficiency of facility operation. It will be more convenient for the user or merchant to browse and find the optimum berthing solution. To increase the convenience of data circulation, the database system can also combine the characteristic of the Internet to build an Internet database system in the future. |  |  |  |  |  |
| DATE OF PUBLICATION May 2000 | NUMBER OF PAGES 290 |  | PRICE |  | CLASSIFICATION □SECRET □CONFIDENTIAL □UNCLASSIFIED |
| The views expressed in this publication are not necessarily those of the Ministry of Transportation and Communications. |  |  |  |  |  |

- 國際港裝卸機具調查研究

目    錄

第一章  前言

1.1	研究緣起	1-1

1.2	研究範圍	1-2

1.2.1	台灣國際港裝卸機具	1-2

1.2.2	亞太經濟合作組織(APEC)港埠資料庫系統	1-3

1.3	研究目的	1-4

1.4	研究執行方式及流程	1-5

1.4.1	執行方式	1-5

1.4.2	工作流程	1-6

第二章  各國際商港簡介

2.1	基隆港	2-1

2.1.1	基隆港基本資料	2-3

2.1.2	基隆港運量資料	2-4

2.1.3	基隆港經營方式	2-10

2.1.4	基隆港未來發展	2-12

2.2	台中港	2-13

2.2.1	台中港基本資料	2-16

2.2.2	台中港運量資料	2-17

2.2.3	台中港經營方式	2-19

2.3	高雄港	2-24

2.3.1	高雄港基本資料	2-27

2.3.2	高雄港運量資料	2-28

2.3.3	高雄港經營方式	2-30

2.4	花蓮港	2-36

2.3.1	花蓮港基本資料	2-36

2.4.2	花蓮港運量資料	2-39

2.4.3	花蓮港經營方式	2-40

第三章  裝卸機具基本資料蒐集與分析

3.1	資料蒐集	3-1

3.1.1	蒐集種類	3-1

3.1.2	面臨的問題	3-1

3.2	裝卸機具簡介	3-3

3.2.1	貨櫃裝卸機具	3-3

3.1.2	一般裝卸機具	3-7

3.3	機具基本資料	3-15

3.4	現有機具種類及數量	3-16

3.4.1	貨櫃裝卸機具	3-16

3.4.2	一般裝卸機具	3-17

3.5	基本作業能量分析	3-19

3.6	使用齡期分析	3-26

第四章  作業狀況資料分析

4.1	各港資料現況	4-1

4.2	績效評比項目	4-2

4.3	可用率、使用率及故障率分析	4-2

4.3.1	計算公式與定義	4-3

4.3.2	統計結果	4-4

4.3.2	比較分析結果	4-7

4.4	毛效率及淨效率分析	4-29

4.4.1	計算公式與定義	4-29

4.3.2	統計結果	4-30

第五章  裝卸機具資料庫查詢系統

5.1	系統簡介	5-1

5.2	系統基本架構	5-3

5.2.1	機具資料顯示	5-3

5.2.2	查詢項目	5-5

5.3	基本使用簡介	5-8

5.4	使用基本軟硬體設施	5-9

5.4.1	資料庫及發展介面軟體	5-9

5.4.2	硬體設備	5-11

5.5	未來發展及資料更新	5-12

5.5.1	未來發展	5-12

5.5.2	資料更新	5-12

第六章	亞太經濟合作組織(APEC)港埠資料庫系統

6.1	成立目的	6-1

6.2	範圍	6-1

6.3	港埠資料庫系統架構及內容	6-3

6.3.1	港埠一般資訊	6-4

6.3.2	貨櫃中心、碼頭營運資料	6-10

6.3.3	港埠環境	6-12

6.4	使用上需注意之事項	6-15

第七章  結論與建議

7.1	結論	7-1

7.2	建議	7-3

附錄一  機具基本資料

附錄二  期末簡報意見辦理情形

附錄三  期末簡報

- 圖  目  錄

圖	1-3-1	裝卸機具調查工作流程圖	1-6

圖	1-3-2	APEC港埠資料庫系統修改資料流程圖	1-7

圖	2-1-1	基隆港、蘇澳港、台北港地理位置圖	2-1

圖	2-1-2	基隆港港區平面圖	2-2

圖	2-1-3	基隆港進出港船舶艘數及貨運量統計圖	2-6

圖	2-1-4	基隆港1997年貨櫃進出港地區圖	2-9

圖	2-2-1	台中港位置圖	2-14

圖	2-2-2	台中港港區及未來發展圖	2-15

圖	2-2-3	台中港進出港船舶艘數及貨運量統計圖	2-18

圖	2-2-4	台中港1997年貨櫃進出港地區圖	2-22

圖	2-3-1	高雄港位置圖	2-25

圖	2-3-2	高雄港港區平面圖	2-26

圖	2-3-3	高雄港進出港船舶艘次及貨運量統計圖	2-31

圖	2-3-4	高雄港1997貨櫃進出港地區圖	2-34

圖	2-4-1	花蓮港位置圖	2-37

圖	2-4-2	花蓮港港區平面圖	2-38

圖	2-4-3	花蓮港進出港船舶艘次及貨運量統計圖	2-41

圖	3-2-1	花蓮港堆高機平均作業狀況圖	3-7

圖	3-2-2	基隆港貨櫃起重機平均作業狀況圖	3-8

圖	3-2-3	蘇澳港堆高機平均作業狀況圖	3-11

圖	4-3-1	基隆港機具平均可用率圖	4-9

圖	4-3-2	基隆港機具平均使用率圖	4-12

圖	4-3-3	台中港機具平均可用率圖	4-14

圖	4-3-4	台中港機具平均使用率圖	4-16

圖	4-3-5	高雄港機具平均可用率圖	4-18

圖	4-3-6	高雄港機具平均使用率圖	4-20

圖	4-3-7	花蓮港機具平均可用率圖	4-22

圖	4-3-8	花蓮港機具平均使用率圖	4-23

圖	4-3-9	蘇澳港機具平均可用率圖	4-25

圖	4-3-10	蘇澳港機具平均使用率圖	4-26

圖	4-3-11	貨櫃起重機平均使用率比較圖	4-27

圖	4-3-12	堆高機平均使用率比較圖	4-28

圖	4-4-1	貨櫃起重機平均毛效率圖	4-32

圖	4-4-2	貨櫃起重機平均淨效率圖	4-33

圖	5-1-1	資料庫查詢系統	5-2

圖	5-2-1	功能點選區	5-3

圖	5-2-2	文字描述區	5-4

圖	5-2-3	資料顯示區	5-4

圖	5-2-4	使用齡期分佈圖	5-5

圖	5-2-5	機具可用率圖	5-6

圖	5-2-6	機具使用率圖	5-6

圖	5-2-7	機具毛效率圖	5-7

圖	5-2-8	機具淨效率圖	5-7

圖	6-2-1	APEC港埠資料庫進入畫面	6-2

圖	6-2-2	港埠選擇畫面	6-2

圖	6-3-1	港埠資料庫項目	6-3

圖	6-3-2	地理位置	6-4

圖	6-3-3	港口氣象條件	6-4

圖	6-3-4	港埠管理者資訊	6-5

圖	6-3-5	港勤服務資料	6-5

圖	6-3-6	貨物吞吐量	6-6

圖	6-3-7	航線資訊	6-7

圖	6-3-8	區域運輸資料	6-8

圖	6-3-9	航商資訊	6-8

圖	6-3-10	其它服務項目	6-9

圖	6-3-11	貨櫃中心資訊	6-10

圖	6-3-12	營運碼頭資訊	6-11

圖	6-3-13	港埠環境資料	6-12

圖	6-3-14	港埠管理機關資料	6-14

表  目  錄

表	2.1.1	基隆港進出港船舶艘數及貨運量統計表	2-6

表	2.1.2	基隆港歷年(1988至1998)貨櫃裝卸量統計表	2-7

表	2.1.3	基隆港1997年貨櫃進出港地區統計表	2-8

表	2.2.1	台中港進出港船舶艘數及貨運量統計表	2-18

表	2.2.2	台中港歷年(1988至1998)貨櫃裝卸量統計表	2-20

表	2.2.3	台中港1997年貨櫃進出港地區統計表	2-21

表	2.3.1	高雄港進出港船舶艘數及貨運量統計表	2-31

表	2.3.2	高雄港歷年(1988～1998)貨櫃裝卸量統計表	2-32

表	2.3.3	高雄港1997貨櫃進出港起迄統計表	2-33

表	2.4.1	花蓮港進出港船舶艘數及貨運量統計表	2-41

表 3.4.1	台灣各國際港現有裝卸機具數量表(88年)	3-18

表 3.5.1	貨櫃起重機作業能量分佈表	3-20

表 3.5.2	貨櫃跨載機作業能量分佈表	3-20

表 3.5.3	貨櫃門式機作業能量分佈表	3-21

表 3.5.4	貨櫃牽引機(大拖車)作業能量分佈表	3-21

表 3.5.5	貨櫃堆積機作業能量分佈表	3-21

表 3.5.6	卡車式起重機作業能量分佈表	3-22

表 3.5.7	拖車(中、小)作業能量分佈表	3-22

表 3.5.8	堆高機作業能量分佈表	3-23

表 3.5.9	卡車作業能量分佈表	3-23

表 3.5.10	抓斗作業能量分佈表	3-24

表 3.5.11	漏斗作業能量分佈表	3-24

表 3.5.12	堆煤機作業能量分佈表	3-24

表 3.5.13	原木裝卸機作業能量分佈表	3-25

表 3.5.14	卸煤機作業能量分佈表	3-25

表 3.5.15	吸穀機作業能量分佈表	3-25

表 3.6.1	貨櫃起重機使用齡期分佈表	3-27

表 3.6.2	貨櫃跨載機使用齡期分佈表	3-28

表 3.6.3	貨櫃門式機使用齡期分佈表	3-29

表 3.6.4	貨櫃車架使用齡期分佈表	3-30

表 3.6.5	貨櫃牽引機(大拖車)使用齡期分佈表	3-31

表 3.6.6	堆積機使用齡期分佈表	3-32

表 3.6.7	卡車式起重機使用齡期分佈表	3-33

表 3.6.8	堆高機使用齡期分佈表	3-34

表 3.6.9	鏟裝機使用齡期分佈表	3-35

表 3.6.10	輸送機使用齡期分佈表	3-36

表 3.6.11	拖車(中、小)使用齡期分佈表	3-37

表 3.6.12	卡車使用齡期分佈表	3-38

表 3.6.13	漏斗使用齡期分佈表	3-38

表 3.6.14	抓斗使用齡期分佈表	3-39

表 3.6.15	堆煤機使用齡期分佈表	3-40

表 3.6.16	原木裝卸機使用齡期分佈表	3-40

表 3.6.17	卸煤機使用齡期分佈表	3-41

表 3.6.18	吸榖機使用齡期分佈表	3-41

表	4.3.1	基隆港機具平均可用率表	4-9

表	4.3.2	基隆港機具平均使用率表	4-11

表	4.3.3	台中港務局機具平均可用率表	4-13

表	4.3.4	德隆倉儲公司機具平均可用率表	4-13

表	4.3.5	萬海航運公司機具平均可用率表	4-13

表	4.3.6	台中港務局機具平均使用率表	4-15

表	4.3.7	德隆倉儲公司機具平均使用率表	4-15

表	4.3.8	萬海航運公司機具平均使用率表	4-15

表	4.3.9	高雄港機具平均可用率表	4-17

表	4.3.10	高雄港機具平均使用率表	4-19

表	4.3.11	花蓮港機具平均可用率表	4-21

表	4.3.12	花蓮港機具平均使用率表	4-21

表	4.3.13	蘇澳港機具平均可用率表	4-24

表	4.3.14	蘇澳港機具平均使用率表	4-24

表	4.4.1	貨櫃起重機毛效率平均表	4-31

表	4.4.2	貨櫃起重機平均淨效率	4-31