85-42-3144

無障礙道路交通工具改裝與運輸服務示範計畫（五）

固定路線無障礙運輸服務

之運轉操作

µ §

交 通 部 運 輸 研 究 所

中 華 民 國 八 十 五 年 六 月

交通部運輸研究所合作研究計畫出版品摘要表

出版品名稱: 固定路線無障礙運輸服務之運轉操作

國際標準書號（或叢刊號）

政府出版品統一編號

009104850414

運輸研究所出版品編號

85-42-3144

| 本所 |  |  |  |  |
| --- | --- | --- | --- | --- |

主辦單位：運輸安全組

主管：林組長豐福

計畫主持人：林豐福、張開國

研究人員：周永暉、王穆衡

聯絡電話：（02）3496868

（02）3496854

傳真號碼：（02）5450429

合作研究單位：汎誠科技有限公司

計畫主持人：黃書強

研究人員：黃信隆、黃貴鴻、張聖

張良安、張玉卿

地址：台北市新生南路一段138-1號二樓

聯絡電話：（02）3511138

研究期間

自84年11月

至85年 6月

關鍵詞：無障礙公共運輸、固定路線

摘要： 本計畫係依據「固定路線無障礙運輸服務之規劃設計」之研究架構所研提之各項規劃及設計成果，再配合台北市公車處作為示範運轉操作對象，以實作與規劃之差異再作修正以建立具體運轉制度，期望作為全國各縣市實施固定路線無障礙運輸服務之借鏡。 本次試辦係依據運轉操作示範計畫分無障礙公車及無障礙專車實施依據實地運轉評估，台北市由於公車班次密集，以加裝昇降設備之公車服務，由於操作時不但費時且會影響車流速度及車輛週轉率，造成尖峰班次無法提高，屢遭民眾詬病。故對於重殘同胞本研究建議宜由政府補助增購無障礙專車加入服務；而無障礙公車方面，宜以法律規定業者在汰換新車時，逐步以低底盤公車取代，配合無障礙空間之改善，期望能達到全面無障礙運輸服務之目標。 出

版日期

頁數

工本費

本出版品取得方式

85年6月

97

100

凡屬機密或限閱性出版品不得對外公開。一般性出版品，公營、公益機關團體及學校可函洽本所免費贈閱；私人及私營機關團體可按工本費價購。

管制等級

□機密（□解密日期為  年  月  日，□主辦單位視情況解密）

□限閱（□解限日期為  年  月  日，□主辦單位視情況

| 解限 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

）

(一般

備註：本研究之結論與建議不代表交通部之意見。

PUBLICATION ABSTRACTS OF RESEARCH PROGRAM

INSTITUTE OF TRANSPORTATION

MINISTRY OF TRANSPORTATION AND COMMUþþCATION

S

TITLE:An Operation Practice for the Fixed Route Barrier-Free Transportation Service

ISBN(OR ISSN)

UNIFORM SERIAL CODE FOR GOVERNMENT PUBLICATIONS

009104850414

IOT SERIAL NUMBER

85-42-3144

DIVISION:Transportation Safety

DIVISION CHIEF: Lin, Fong-Fu

PRINCIPAL INVESTIGATOR:Lin,Fong-Fu Chang, Kai-Kuo

PROJECT STAFF: Chou, Yung-Hui ,Wang, Mu-Han

PHONE:  (02) 349-6854

FAX:     (02)545-0429

PROJECT PERIOD

FROM  Nov. 1995

TO     June. 1996

RESEARCH AGENCY:  FORTUNE Technology Corp.

PRINCIPAL INVESTIGATOR: Huang ,Shu-Chiang

PROJECT STAFF: Huang ,Hsin-Lung etc..

ADDRESS: 2 FL., NO. 138-1 Hsin Sheng S. Rd. Sec. 1, Taipei ,Taiwan, R.O.C.

PHONE: (02) 351-1138

FAX::  (02)341-4907

KEY WORDS:Barrier-Free Public Transportataion，Fixed Route.

ABSTRACT:

The operation practice is following the proposed structure made by the study of “The Plan and Design for Fixed Route Barrier-Free Public Transportation Service”.Thanks for the assistance from the Taipei Bus Division,the study can have the chance to practice this special project in Taipei.Through the practice the study can find if anything needed to be improved before drawing the conclusions. The conclusions of this study are expected to be a fine example for all cities and counties in this country whenever they would like to implement the similar service in their jurisdictions.

During this practice two special services are provided which are regular barrier-free bus service and special barrier-free exclusive bus service for disabled people only. Through the practice in Taipei, it is found because the operating time for the new equipment installed on regular buses is too long, the traffic in a busy city like Taipei can be disturbed significantly. Besides,the bus turn over rate can also be affected signficantly. It is the reason why the frequency of the former special service is difficult to increase especially during rush hours. To improve this situation, it is suggested that (1) the government can subside bus companies to buy more special designed barrier-free buses in order to better serve people of higher levels of disability,and (2)the government can issue regulations to encourage bus companies to replace thier regular buses by low-floor buses. In coordinating with the improvement of barrier-free walking environment ,it is expected that the goal of a completely barrier-free transportation service will be achieved.

DATE OF PUBLICATION

NUMBER OF PAGES

PRICE

CLASIFICATION

JUNE  1996

97

NT 100

□SECRET

□CONDFIDENTIAL

(UNCLASSIFIED

The views expressed in this publication are not necessaritly those of Ministry of Transportation and Communications

固定路線無障礙運輸服務之運轉操作

目         錄

第一章   緒  論    .......................................1

1.1   計畫緣起.........................................1

1.2   計畫範圍與對象...................................4

1.3   計畫目標.........................................5

第二章   無障礙運輸運轉現況...............................7

2.1   國內運轉情況.....................................7

2.1.1 台北市復康巴士辦理經過...........................7

2.1.2 復康巴士辦理績效.................................8

2.1.3 復康巴士評估....................................12

2.1.4 博愛公車........................................14

2.2   國外運轉情況....................................15

第三章   運轉操作示範計畫說明............................18

3.1   固定路線方案....................................18

3.1.1 固定路線殘障公車定義............................18

3.1.2 殘障者運輸需求型態..............................19

3.1.3 固定路線殘障公車服務種類........................21

3.1.4 路線方案研擬....................................22

3.1.5 最佳路線方案....................................26

3.2   車輛人員調配計畫................................34

3.2.1 無障礙公（專）車調派模式..........................35

3.2.2 路線運轉操作方案................................37

3.2.3 車輛改加裝......................................38

3.2.4 人員車輛排班....................................40

3.2.5 車輛保養制度....................................42

3.3   人員訓練計畫....................................44

3.4   營運操作補貼計畫...............................,48

3.5   無障礙運輸服務運轉試行計畫......................51

第四章   車輛輔助設備配置計畫............................53

4.1   採購規範........................................53

4.2   安裝規範........................................56

4.3   車輛監理業務....................................68

第五章   運轉操作之評估..................................70

5.1   評鑑指標之選擇..................................70

5,2   績效評鑑制度之建立..............................74

5.3   運轉操作檢討及修正..............................77

第六章   結論與建議......................................83

6.1   結  論..........................................83

6.2   建  議..........................................86

參考文獻.................................................87

附  錄...................................................89

洛山磯 CITYRIDE 時刻表...................................89

洛山磯 CITYRIDE 路線圖...................................90

昇降機維修查核表.........................................91

昇降機維修/潤滑時間表....................................95

表    目    錄

表1.1 台北市公民營公車八十三年營運狀況年報表..............2

表2.1 復康巴士營運狀況表(載客人數)........................9

表2.2 復康巴士營運狀況表(收支情形).......................10

表2.3 八十四年復康巴士受益方式統計表.....................10

表2.4 八十四年復康巴士(小型車)殘障類別統計表.............11

表2.5 復康巴士營運比較表.................................11

表2.6 博愛公車與277一般公車營運績效比較表...............14

表3.1 無障礙運具配備種類.................................20

表3.2 固定路線無障礙公車服務種類.........................21

表3.3 無障礙公車路線篩選及評比結果.......................26

表3.4 無障礙專車主幹線示意...............................32

表3.5 277無障礙公車班次表................................41

表3.6 無障礙公車服務人員甄選評分表.......................45

表3.7 無障礙公（專）車服務考核表..........................47

表4.1 無障礙專車車輛規範建議草案,........................54

表4.2 各類別殘障人士所需之輔助設備.......................57

表4.3 美國殘障法案ADA對輪椅升降機標準規格..............58

表5.1 無障礙公車營運服務指標評定基準表...................75

表5.2 道奇牌無障礙專車規格表.............................78

表5.3 博愛公車運轉績效比較表.............................82

圖    目    錄

圖3-1 288線路線示意圖....................................27

圖3-2 277線路線示意圖....................................28

圖3-3 601線路線示意圖....................................29

圖3-4 606線路線示意圖....................................30

圖3-5 22線路線示意圖.....................................31

圖3-6 無障礙專車主幹線路線圖.............................34

圖3-7改裝無障礙國瑞牌公車內部佈置圖......................39

圖4-1改裝無障礙公車（國瑞牌）內部佈置圖...................61

圖4-2升降機設備配置圖....................................62

圖4-3輪椅及乘客安全帶配置圖..............................63

圖4-4無障礙專車（道奇牌）外觀圖..........................64

圖4-5無障礙專車（道奇牌）內部佈置圖（一）.................65

圖4-5無障礙專車（道奇牌）內部佈置圖（二）................66

圖4-6無障礙專車可折合式簡便型座椅........................67

圖4-7汽車檢驗作業程序....................................69