交通部運輸研究所合作研究計畫出版品摘要表

| 出版品名稱：基隆地區鐵路立體化可行性之檢討研究 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 國際標準書號（或叢刊號） |  |  |  |  | 政府出版品統一編號 009104890186 | 運輸研究所出版品編號 89-18-258 |  |
| 本所主辦單位：運輸工程組 主管：曾志煌 計畫主持人：曾志煌 研究人員：邱雅莉 聯絡電話：(02)23496828 傳真號碼：(02)25450427 |  |  |  | 合作研究單位：國立台北科技大學 計畫主持人：王隆昌 協同主持人：蕭再安 研究人員：徐承義、張建祥、何明美、陳怡誠 地址：忠孝東路3段1號 聯絡電話：(02)27712171 ext.1371 |  |  | 研究期間 自87年10月 至88年6月 |
| 關鍵詞：基隆、鐵路立體化、可行性評估 |  |  |  |  |  |  |  |
| 摘要： 基隆市地勢起伏大，可供發展的平地很少，市內公路系統不僅擴建不易；加上基隆市的平面鐵路軌道及站區阻隔，影響基隆市的交通運輸運輸系統發展。另外，其列車運送時所產生的振動與噪音，也破壞了沿線居民的生活品質。 鐵路立體化為重大公共工程，不僅對區域發展具有高度影響，而且興建成本甚為可觀，故於工程實施前時有必要進行全盤的研究分析。 本研究考量現今基隆地區社經條件下，提出四個鐵路路線改善方案(平面改善、鐵路高架、鐵路地下化甲、乙)。在路線工程分析中，地下化方案在各種工程控制點之考量分析下，均對於現況造成重大影響，是故將鐵路地下化方案予以排除；其他方案皆屬工程可行。然對於平面改善、鐵路高架方案之成本效益項目進行估算後，均為經濟不可行之方案。 |  |  |  |  |  |  |  |
| 出版日期 | 頁數 | 工本費 | 本出版品取得方式 |  |  |  |  |
| 89年04月 | 304 |  | 凡屬機密性出版品均不對外公開。普通性出版品，公營、公益機關團體及學校可函洽本所免費贈閱；私人及私營機關團體可按工本費價購。 |  |  |  |  |
| 機密等級： □限閱 □密 □機密 □極機密 □絕對機密 （解密【限】條件：□ 年 月 日解密，□公布後解密，□附件抽存後解密， □工作完成或會議終了時解密，□另行檢討後辦理解密） 普通 |  |  |  |  |  |  |  |
| 備註：本研究之結論與建議不代表交通部之意見。 |  |  |  |  |  |  |  |

PUBLICATION ABSTRACTS OF RESEARCH PROJECTS

INSTITUTE OF TRANSPORTATION

MINISTRY OF TRANSPORTATION AND COMMUNICATIONS

| TITLE: The Review Feasibility Study of Railroad Grade Separation in Keelung Area |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ISBN(OR ISSN) |  | GOVERNMENT PUBLICATIONS NUMBER 009104890186 |  | IOT SERIAL NUMBER 89-18-258 |  |
| DIVISION: Transportation Engineer Division DIVISION CHIEF: James C.H. Tseng PRINCIPAL INVESTIGATOR: James C.H. Tseng PROJECT STAFF: Chiu Ya-Li PHONE: (02)23496828 FAX: (02)25450427 |  |  |  | PROJECT PERIOD FROM October 1998 TO June 1999 |  |
| RESEARCH AGENCY: National Taipei University of Technology PRINCIPAL INVESTIGATOR: Wang Long-Chang ASSOCIATE INVESTIGATOR: Shiau Tzay-An PROJECT STAFF: Hsu Cheng-Yi, Chung Jian-Shiun, Ho Ming-Mei, Chen Yi-Cheng ADDRESS: 1 Chunshiou E. Rd, Section 3, Taipei, Taiwan, R.O.C PHONE: (02)27712171 ext. 1371 |  |  |  |  |  |
| KEY WORDS: Keelung, Railroad grade separation, evaluation of feasibility |  |  |  |  |  |
| ABSTRACT: The land rises and falls in Keelung area. It provides less flats for people and road system to extend. Keelung is cut off by the tracks and station area of railroad, and this hinders the extension of transportation system in Keelung. Furthermore, the vibration and the noise made by the trains running on the rail worens the quality of life along the rail line. The construction of rail grade separation is a serious public work. It not only has greatly influence on urban developing, but also cost substantially. Therefore, careful planning and evaluation before construction is very important. Under the consideration of social and economic terms in Keelung area, this study brings up four alternatives for rail grade separation (the rail flat surface reform alternative, the rail flyover alternative, and two subterranea alternatives). Considering the check points of route engineer in rail subterranea alternatives, they both will bring out graveness affect to Keelung area. So the rail subterranea alternatives are rejected. And finally we estimate the cost and benefit terms of two others alternatives. The outcome shows these two alternatives are economically unworkable alternatives. |  |  |  |  |  |
| DATE OF PUBLICATION April 2000 | NUMBER OF PAGES 304 |  | PRICE |  | CLASSIFICATION □SECRET □CONFIDENTIAL UNCLASSIFIED |
| The views expressed in this publication are not necessarily those of the Ministry of Transportation and Communications. |  |  |  |  |  |

基隆地區鐵路立體化可行性之檢討研究

目錄

中文摘要……………………………………………………………I

英文摘要………………………………………………………… Ⅱ

目錄………………………………………………………………Ⅲ

表目錄…..………………………………………………………Ⅷ

圖目錄……………………………….……………………………XI

第一章 緒論

1.1 研究緣起與目的 ………………………………1-1

1.2 研究範圍…………………………………………1-2

1.2.1 研究空間範圍………………………………1-2

1.2.2 目標年之設定………………………………1-2

1.3 研究內容與流程…………………………………1-4

第二章 研究地區發展現況分析與相關發展計畫

2.1 研究地區背景概述………………………………2-1

2.1.1 土地…………………………………………2-1

2.1.2 人口…………………………………………2-4

2.1.3 地形…………………………………………2-4

2.1.4地質與土壤…………………………………2-7

2.1.5 氣候…………………………………………2-9

2.2發展現況分析……………………………………2-12

2.2.1 社經特性…………………………………2-12

2.2.2 土地使用……………………………………2-16

2.2.3 交通現況…………………………………2-17

2.3 相關發展計畫……………………………………2-20

2.3.1 公路建設計畫……………………………2-20

2.3.2 鐵路建設計畫……………………………2-23

2.3.3 基隆港發展計畫  ………………………2-26

2.3.4 基隆市火車站地區交通運輸系統與

土地使用整體規劃………………………2-29

2.3.5 台北南港捷運延伸至基隆專案…………2-31

2.3.6 鐵路地下化東延南港專案………………2-33

2.4 小結………………………………………………2-34

第三章 運輸需求分析

3.1 運輸需求現況分析………………………………3-1

3.2 運輸需求預測分析……………………………3-12

3.3 小結………………………………………………3-23

第四章 鐵路沿線相關重要課題探討

4.1 鐵路沿線環境概述及其影響……………………4-1

4.1.1鐵路沿線環境概述…………………………4-1

4.1.2基隆火車站環境概述……………………4-24

4.1.3鐵路沿線所造成之影響…………………4-32

4.2 規劃範圍內鐵路沿線之都市發展概況分析…4-40

4.2.1土地使用分析………………………………4-40

4.2.2沿線人口分析………………………………4-45

4.2.3沿線產業人口分析…………………………4-45

4.2.4沿線地價分析……………………………4-45

4.3 鐵路沿線之工程探討……………………………4-49

4.4 鐵路運輸及基隆火車站未來之定位…………4-50

4.5 小結………………………………………………4-52

第五章 鐵路路線改善方案之研擬

5.1 鐵路路線改善方案之規畫策略分析…………5-1

5.2 鐵路路線改善方案之目標與研擬原則………5-3

5.2.1 鐵路路線改善方案之目標……………… 5-3

5.2.2 鐵路路線改善方案之研擬原則………… 5-3

5.3 方案研擬…………………………………………5-5

5.3.1 平面改善方案…………………………… 5-5

5.3.2 鐵路高架改善方案……………………… 5-9

5.3.3 鐵路地下化改善方案甲案…………… 5-12

5.3.4 鐵路地下化改善方案乙案….………… 5-15

第六章 路線改善方案之工程可行性分析

6.1 概述………………………………………………6-1

6.2 工程資料之蒐集分析…………………………6-1

6.3 工程設計標準……………………………………6-3

6.3.1 鐵路工程設計標準…………………………6-3

6.3.2 公路工程設計標準…………………….…6-10

6.4 路線工程研究……………………………………6-11

6.4.1 路線說明……………………………………6-11

6.4.2 鐵路沿線控制點說明……………………6-15

6.4.3 路線工程研究小結………………………6-17

6.5 結構工程研究……………………………………6-33

6.5.1 工程地質分析……………………………6-33

6.5.2 高架橋樑工程……………………………6-34

6.6 施工期間鐵路營運維持…………………………6-38

6.6.1 現有場站營運設施………………………6-38

6.6.2 平面改善方案……………………………6-38

6.6.3 鐵路高架方案……………………………6-44

6.7 道路相關配合措施及施工期間交通維持計畫6-45

6.7.1 道路相關配合措施………………………6-45

6.7.2 施工期間交通維持計畫…………………6-46

6.8 工程成本估算……………………………………6-51

6.9 小結………………………………………………6-52

第七章 路線改善方案之環境影響分析

7.1 環境影響項目……………………………………7-1

7.2 環境影響說明……………………………………7-2

7.3 替選方案之環境影響分析比較…………………7-6

7.4 小結………………………………………………7-7

第八章 路線改善方案之經濟可行性分析

8.1評估流程……………………………………………8-1

8.2評估之基本假設…………………………………8-2

8.3效益及成本參數…………………………………8-3

8.4效益及成本項目之估算…………………………8-4

8.4.1影響群體分析………………………………8-4

8.4.2經濟成本之估算……………………………8-5

8.4.3經濟效益之估算……………………………8-6

8.5成本效益比較……………………………………8-17

8.6敏感度分析………………………………………8-19

第九章 路線改善方案之財務可行性分析

9.1財務分析方法及基本假設……………………9-1

9.2財務成本分析……………………………………9-2

9.2.1 財務成本項目……………………………9-2

9.2.2 財務成本估算……………………………9-2

9.3財務收益分析……………………………………9-3

9.3.1 財務收益項目………………………………9-3

9.3.2 財務收益估算………………………………9-3

9.4財務可行性分析…………………………………9-4

9.4.1 財務評估指標分析………………………9-4

9.5 小結………………………………………….……9-5

第十章 結論與建議

10.1結論………………………………………………10-1

10.2建議……………………………………………10-5

附錄I 期中簡報審查意見辦理情形彙整表………………I-1

附錄II 期中簡報審查意見辦理情形彙整表……………II-1

附錄III 末段高架道路…………………………………III-1

附錄VI 修正後之簡報資料…………………………………IV-1

參考文獻………………………………………………………V-1

表 目 錄

表1 基隆市人口數一覽表…………………………………….2-5

表2 基隆地區風象資料……………………………………….2-10

表3 基隆地區歷年氣象資料…………………………………2-11

表4 基隆市都市之土地使用分區面積與人口現況………2-16

表5 基隆市聯外道路服務水準分析………………………2-18

表6 基隆市區道路系統分類…………………………………2-20

表7 目前基隆市之公路相關發展建設計畫………………..2-24

表8 基隆港碼頭基本資料與使用情形……………………2-27

表9 基隆港之貨物裝卸型態分析表………………………2-28

表10 其他各相關計畫對本研究之影響………………….2-35

表11 基隆市現況人旅次起迄分布表………………………….3-5

表12 現況小客車旅次起迄分佈表…………………………….3-8

表13 現況機車旅次起迄分佈表…………………….……….3-9

表14 現況大客車旅次起迄分佈表……………………………3-10

表15 基隆市大貨車旅次起迄現況………………………….3-11

表16 民國110年各車種旅次產生吸引預測…………………3-17

表17 民國110年小客車旅次起迄分佈表……………………3-18

表18 民國110年機車旅次起迄分佈表…………………….3-19

表19 民國110年大客車旅次起迄分佈表……………………3-20

表20 民國110年公路客運總旅次起迄表………………3-21

表21 民國110年大貨車總旅次起迄分佈表…………….3-22

表22 基隆火車站歷年旅客人數統計表…………….………4-26

表23 基隆火車站歷年貨物噸數統計表……………………4-27

表24 基隆火車站每日各級列車次數統計表………………4-28

表25 基隆火車站各級列車每日旅客人數統計表………..4-29

表26 假日與非假日之車輛交通量統計表………………..4-36

表27 假日與非假日之行人交通量統計表………………..4-36

表28 假日期間各交叉點於各時段之交通量…………….4-37

表29 假日期間各交叉點於各時段之行人與車輛

全日交通量………………………………………………4-37

表30 精一路平交道(非假日)平均受阻延滯時間分析表…4-38

表31 精一路平交道(假日)平均受阻延滯時間分析表….4-38

表32 成功路平交道(非假日)平均受阻延滯時間分析表.4-38

表33 成功路平交道(假日)平均受阻延滯時間分析表……….4-39

表34 精一路平交道(非假日)受阻率分析表………..…….4-39

表35 精一路平交道(假日)受阻率分析表………………….4-39

表36 成功路平交道(非假日)受阻率分析表……………….4-40

表37 成功路平交道(假日)受阻率分析表…………….……4-40

表38 基隆市鐵路沿線之人口概況……………………..…..4-45

表39 基隆市鐵路沿線之產業人口概況…………………4-45

表40 鐵路沿線地價……………………………………..4-48

表41 鐵路立體化路線方案建議採用之工程標準規範……6-4

表42 公路路線設計標準表……………………………………6-12

表43 平面改善方案工程成本估算………………………….6-53

表44 平面改善方案工程數量估算…………………………6-54

表45 鐵路高架化方案工程成本估算……………………..6-55

表46 鐵路高架化方案工程數量估算………………………6-56

表47 道路工程可能產生之環境影響項目……………………7-1

表48 各方案在施工期間及營運期間所產生

主要的空氣污染…………………………………………..7-2

表49 各方案在施工期間及營運期間

所產生主要的噪音震動………………………………7-4

表50 各方案在施工期間及營運期間對鄰近交通的影響…..7-5

表51 各方案在施工期間及營運期間對環境景觀的影響…7-6

表52 替選方案環境影響分析比較表………………………….7-7

表53 影響群體分析…………………………………………8-4

表54 方案分年建造成本………………………………………8-5

表55 研究年期各方案營運維修之效益比較表………………8-7

表56 研究年期各方案路網指派運輸績效………….……….8-12

表57 研究年期各方案運輸績效之效益統計表……………8-12

表58 研究年期高架方案平交道維修

成本節省效益統計表……………………………………8-15

表59 研究年期高架方案平交道肇事

成本節省效益統計表………………………….………8-16

表60 方案成本效益比較表……………………………………8-19

表61 敏感度分析……………………………………………….8-20

表62 鐵路沿線土地增值……………………………………….9-4

表63 各替選方案財務評估……………...……………………9-5

圖 目 錄

圖1 研究範圍………………………………………………….1-3

圖2 研究流程圖………………………………………………1-5

圖3 基隆市各行政區位置圖…………………………………2-1

圖4 基隆市各行政區域面積百分比圖…………………2-2

圖5 基隆市各項已登陸土地面積所佔百分比……………2-3

圖6 基隆市地形圖…………………………………………2-6

圖7 基隆市地質分佈圖……………………………………2-8

圖8 土地使用現況分析圖………………………………2-17

圖9 基隆市聯外道路建設計畫位置圖…………………2-25

圖10 交通分區對照圖…………………………………………3-2

圖11 民國87年旅次產生、吸引強度分佈圖………………3-4

圖12 民國110年旅次產生、吸引強度分佈圖…………….3-15

圖13 研究範圍鐵路路線分佈圖………………………………4-4

圖14 各區段鐵路路線圖……………………………………4-5

圖15 鐵路路線五堵地區概況………………………………..4-6

圖16 七堵調車場用地之現況況(a)…………………………4-6

圖17 七堵調車場用地之現況況(b)…………………………4-7

圖18 七堵火車站………………………………………………4-7

圖19 基起4K+400處公路與鐵路交會處…………………4-8

圖20 八堵火車站……………………………………………4-8

圖21 八堵火車站月台及軌道佈設狀況……………………4-9

圖22 宜蘭線鐵路…………………………………………4-9

圖23 八堵鐵橋………………………………………………4-10

圖24 竹子嶺隧道北口………………………………………4-10

圖25 龍安街…………………………………………………4-11

圖26 龍安街區段之軌道……………………………………4-11

圖27 成功陸橋………………………………………………4-12

圖28 西岸高架橋及光一陸橋………………………………4-12

圖29 忠四陸橋西側………………………………………4-13

圖30 忠一陸橋西側………………………………………4-13

圖31 基隆火車站……………………………………………4-14

圖32 港區線鐵路佈設情形…………………………………4-14

圖33 中華路路口之鐵路路線………………………………4-15

圖34 基隆港西岸28號碼頭之港區線鐵路路線………4-15

圖35 規劃範圍鐵路沿線衝突點位置圖…………………4-17

圖36 各點距離示意圖………………………………………4-18

圖37 竹子嶺隧道基隆方向出口處…………………………4-19

圖38 竹子嶺隧道基隆方向出口處鐵路沿線概況………4-19

圖39 龍安街概況……………………………………………4-19

圖40 龍門里涵洞東向出口…………………………………4-19

圖41 龍門里涵洞東向出口街景(左側)……………………4-20

圖42 龍門里涵洞東向出口街景(右側)……………………4-20

圖43 龍門里涵洞西向出口街景(左側)……………………4-20

圖44 龍門里涵洞西向出口街景(右側)……………………4-20

圖45 德厚里涵洞西側出口…………………………………4-21

圖46 德厚里涵洞上方之鐵路沿線景象……………………4-21

圖47 德厚里涵洞至精一路平交道口間

之土地使用情形(A)…………………………………4-21

圖48 德厚里涵洞至精一路平交道口間

之土地使用情形(B) ………………………………4-21

圖49 精一路平交道…………………………………………4-22

圖50 成功一路平交道………………………………………4-22

圖51 成功高架橋下之一景…………………………………4-22

圖52 忠四陸橋………………………………………………4-22

圖53 忠一陸橋………………………………………………4-23

圖54 基隆火車站之後站一景(A) …………………………4-23

圖55 基隆火車站之後站一景(B) …………………………4-23

圖56 基隆火車站平面圖……………………………………4-25

圖57 基隆火車站歷年旅客人數統計區線圖………………4-26

圖58 基隆火車站歷年貨物噸數數統計區線圖…………4-27

圖59 基隆火車站列車每日班次次數………………………4-28

圖60 基隆火車站列車車次每日率客人數統計區線圖4-29

圖61 基隆火車站大廳………………………………………4-30

圖62 行李包裹託運處………………………………………4-30

圖63 行李包裹託運處一景…………………………………4-30

圖64 島式月台………………………………………………4-31

圖65 列車調度區……………………………………………4-31

圖66 忠一路橋及西岸高架橋………………………………4-31

圖67 成功平交道之人力配置圖……………………………4-34

圖68 鐵路沿線的三個區段位置圖………………………4-43

圖69 第一與第二區段內的商業區與住宅區位置圖……4-44

圖70 鐵路沿線地價調查區域……………………………4-47

圖71 平面改善方案…………………………………………5-6

圖72 基隆火車站場站配置圖…………………………………5-7

圖73 高架化改善方案…………………………………………5-11

圖74 地下化改善方案……………………………………5-14

圖75 平面路軌標準斷面圖…………………………………6-5

圖76 新建電化路線隧道建築界限圖………………………6-8

圖77 情境一之鐵路地下化縱剖面圖………………………6-20

圖78 地下化高程說明圖………………………………………6-21

圖79 情境二與情境三之鐵路地下化縱剖面圖……………6-22

圖80 道路高架方案平面及縱斷面圖……………………6-23

圖81 鐵路高架方案平面及縱斷面圖………………………6-28

圖82 基隆鐵路立體化研究地區地質柱狀剖面圖………6-35

圖83 鐵路高架橋樑上部結構衡斷面圖…………………6-39

圖84 型式A：預力箱型橋樑………………………………6-40

圖85 型式B：預鑄預力I型橋樑………………………6-40

圖86 型式C：場鑄懸臂施工預力箱型橋樑……………6-41

圖87 型式D：連續鋼箱型橋樑……………………………6-41

圖88 型式E：鋼I型橋樑…………………………………6-42

圖89 型式F：鋼床鈑鋼箱型橋樑…………………………6-42

圖90 基隆火車站軌道現況分布圖…………………………6-43

圖91經濟效益評估工作流程圖…………………………8-1

圖92多車種多重路徑指派求解流程圖……………………8-11

圖93目標年指派路網圖………………………………………8-12