PUBLICATION ABSTRACTS OF RESEARCH PROGRAM

INSTITUTE OF TRANSPORTATION

MINISTRY OF TRANSPORTATION AND COMMUNICATIONS

| TITLE： The Test and Research Regarding the Relationship between Braking Distance and Vehicle Speed |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ISBN (OR ISSN) | UNIFORM SERIAL CODE FOR GOVERNMENT PUBLICATION |  |  |  |  | IOT SERIAL NUMBER |  |
| DIVISION：Transportation Safety DIVISION CHIEF：Fong-Fu Lin PRINCIPAL INVESTIGATOR：Fong-Fu Lin PROJECT STAFF：Grace Y.J. Liu, Y.M.Tien ADDRESS：240, Tunhua N.Rd.,Taipei ,Taiwan,R.O.C. PHONE：(02)349-6853 FAX：(02)545-0429 |  |  |  | PROJECT PERIOD FROM August 1996 TO June 1997 |  |  |  |
| RESEARCH AGENCY：Automotive Research & Testing Center PRINCIPAL INVESTIGATOR：Liao, Ching-Chiu PROJECT STAFF：Chung, Kuo-Liang, Huang, Pin-Cheng, Chung, Michael, Tsai, Aristo S.M., Won, Guo-Liang, Liu, David, Lung, Jiing-Sen ADDRESS：No. 415,Chen Kung Rd., Hu Kou Hsiang, Hsin Chu Hsien, Taiwan, R.O.C. PHONE：(03)599-4171 ext350 |  |  |  |  |  |  |  |
| KEY WORDS：Brake skid mark, Drag factor, Vehicle speed, Coefficient of friction of roadway surface, Slip ratio, Air brake system, Jack-knife of semi-combined vehicle |  |  |  |  |  |  |  |
| ABSTRACT： 1. This proposal is a continuous project from 〝Study on review, modification, and test of the relationship between braking distance and vehicle speed〞in FY83 and 〝Vehicle Braking Performance Test and Analysis of the Influnced Parameters for Braking Distance.〞in FY84.The purpose is to examine the suitability of existing table of 〝the relationship of braking stopping distance,vehicle speed and road friction coefficient〞. 2. The process of this proposal initially setup the modified corresponding table referring to international research attainment, then confirms its accuracy by the results of planned vehicle road braking tests of two popular domestic vehicles（passenger car and van）. The road test conditions of braking test combine following various states: dry/wet road surfaces on new/old asphalt paved road, including two kinds of weight distribution－light and heavy load/G.V.W.(gross vehicle weight), and test speeds of 30, 40,50,.........,100 km/h. 3. The table corresponding to vehicle speed used in traffic accident is based on the formular V2=2aL=2fgL. Namely, it is the way to estimate vehicle initial speed after deciding drag factor(f) and length of brake skid marks(L). 4. This proposal also collects relative characteristics of air brake system and motorcycle brake system, in order to have a concept for setup corresponding table relative to these motor vehicles. |  |  |  |  |  |  |  |
| DATE OF PUBLICATION November 1997 |  | NUMBER OF PAGES 149 | PRICE |  | CLASSIFICATIN oSECRET oCONFIDENTIAL þUNCLASSIFIED |  |  |
| REMARKS:The views Expressed in this publication are notnecessarily those of Ministry of Transportation and Communications |  |  |  |  |  |  |  |