交通部運輸研究所出版品摘要表

出版品名稱：台北松山機場計程車交通問題暨高雄

小港機場周邊停車問題改善建議報告

國際標準書號（或叢刊號）政府出版品統一編號

運輸研究所出版品編號

主辦單位

主管

計畫主持人

研究人員

電話

傳真：運輸安全組

：林豐福

：林豐福

：周永暉、葉祖宏、王穆衡

：(02)349-6856

：(02)545-0429研究期間

自84年10月

至85年2月

關鍵詞：排班計程車、巡迴計程車、停車週轉率

摘要： 本研究分於84年10月9日及85年1月9日奉 次長指示辦理民眾反映小港機場航廈附近自用小汽車嚴重違規停車暨松山機場計程車所衍生之交通問題，經實地前往會勘以瞭解問題之癥結，並進而研擬改善措施供相關業務單位參考。 在松山機場計程車交通問題方面，本研究針對航廈前實體設施之重新配置、並配合巡迴計程車上車處之移置、與相關標誌、號誌之設置，將可產生下列預期效益：1.增加航站大廈前之空間使用效率、2.減少巡迴計程車違規載客之現象、3.紓緩車流交織與改善行車秩序、4.增進旅客通行之安全。 在高雄機場周邊違規停車問題方面，本研究透過停車週轉率、車位平均使用率、及停車費率分析發現，欲有效改善高雄機場周邊交通現況，已非局部調整停車收費方式即能奏效，短期而言，應加強公車服務路線規劃與班次之提供、建立停車場指示標誌與停車資訊顯示系統、提高原路邊及公園路外停車場之收費標準等措施著手。長期而言，建請民航局儘速完成二期航站停車場之興建，及高雄市政府規劃公園立體停車場之建設計畫等策略進行。

出版日期頁數工本費本出版品取得方式

85 年 7月

26

凡屬機密或限閱性出版品均不對外公開。一般性出版品，公營、公益機關團體及學校可函洽本所免費贈閱；私人及私營機關團體可按工本費價購。

管制等級：

¨機密（¨解密日期為  年  月  日，¨主辦單位視情況辦理解密）

¨限閱（¨解密日期為  年  月  日，¨主辦單位視情況辦理解限）

þ一般

備註：本研究之結論與建議不代表交通部之意見。

PUBLICATION ABSTRACTS OF RESEARCH PROGRAM

INSTITUTE OF TRANSPORTATION

MINISTRY OF TRANSPORTATION AND COMMUNICATIONS

TITLE: The Improvement Suggestions for the Taxi Allocation at the Taipei Sung-Shan International Airport and the Parking Problem around the Kaoshung International Airport

ISBN(OR ISSN)

UNIFORM SERIAL CODE FOR GOVERNMENT PUBLICATIONS

IOT SERIAL NUMBER

DIVISION: Transportation Safety Division

DIVISION CHIEF: Fong-Fu Lin

PRINCIPAL INVESTIGATOR: Fong-Fu Lin

PROJECT STAFF: Yung-Hui Chou, Tsu-Hurng Yeh, Mu-Han Wang

PHONE：886-2-3496856

FAX：886-2-5450429PROJECT PERIOD

FROM  Oct., 1995

TO     Feb, 1996

KEY WORDS: Airport Taxi, City Taxi, Parking Turn Over Rate

ABSTRACT：

The study is conducted according to the Vice Minister's （MOTC） assignments respectively in October 9, 1995 and January 9, 1996. The assignments are in response to citizens' complaints regarding the parking problems and the problems related to taxi service around the Kaoshung International Airport and the Taipei Sung-Shan International Airport. To understand the problems, investigation teams have visited both airports. Improvement suggestions have been made according to the investigation results and the extensive discussions among the team members.

The traffic problem at the Taipei Sung-Shan International Airport is mainly related to the taxi service. Improvement suggestions for this special case are to relocate the physical barriers in front of the terminal, to move forward the reserved boarding space for airport taxi, and to redesign traffic signs and signals. Expected benefits include: (1) enhancing the efficiency of the space utilization in front of the terminal, (2) reducing the accurances of city taxi violating the    parking regulations, (3) minimizing the chance of crossing traffic and improving traffic situation, and (4) enhancing the safety for walking passengers.

The traffic problem of the Kaoshung International Airport is the serious situation of illegal parking around the airport. By analyzing the parking turn over rate, the rate of average parking lot usage, and the parking fee, it is concluded that it is unlikely to improve the situation by only adjusting the policy of parking fee collection. To improve the situation, in the short term, it is necessary to improve the designs of bus routes and schedules, to install new parking lot indication signs and parking information system, and to increase the parking fee for roadside parking around the terminal and the park near the airport. In the long term, it is suggested that the Civil Aeronautical Administration can complete the construction of new parking lot at the second terminal as soon as possible, and the Kaoshung Municipal Government can start the parking garage construction plan at the park near the airport.

DATE OF PUBLICATION

July, 1996NUMBER OF PAGES

26PRICECLASSIFICATION

oSECRET

oCONFIDENTIAL

þUNCLASSIFIED

The views expressed in this publication are not necessarily those of Ministry of Transportation and Communications

台北松山機場計程車交通問題暨高雄小港機場周邊停車問題改善建議報告

目   錄

壹、台北松山機場計程車交通問題改善分析....................1

一、問題背景說明........................................1

二、計程車暨航站前交通問題現況探討......................1

三、改善方案之規劃原則..................................4

四、研擬改善方案........................................5

五、結論與建議.........................................10

貳、高雄小港機場周邊停車問題改善分析......................11

一、問題背景說明.......................................11

二、停車現況探討.......................................11

三、停車費率分析.......................................14

四、改善策略之研擬.....................................18

五、結論與建議.........................................21

參考文獻..................................................22

附錄一....................................................23

附錄二.....................................................25

表 目 錄

表1.1 松山機場旅客到達／離開運具選用統計表................3

表1.2 改善措施說明表................................8

表2.1 高雄機場附近停車場車位容量.........................14

表2.2 高雄機場附近停車場費率結構.........................14

表2.3 高雄機場停車需求調查結果...........................15

表2.4 不同費率結構表.....................................15

表2.5 不同費率結構營收分析表.............................16

表2.6 高雄機場旅客到達／離開運具選用統計.................18

表2.7 高雄機場停車設施品質影響因素統計...................19

表2.8 高雄機場停車設施品質影響因素統計...................21

圖 目 錄

圖1-1 台北松山機場各類運具轉乘設施及位置現況圖............2

圖1-2 台北松山機場航站前交通改善方案佈設圖................7

圖2-1 高雄機場轉乘設施配置及交通動線現況圖...............12

圖2-2 高雄機場聯外道路系統現況圖.........................13

圖2-3 高雄機場旅客出入機場時間分佈.......................17