運輸計劃季刊

第三十卷  第一期

民國九十年三月

頁237  ～  頁252

Transportation Planning Journal

Vol. 30  No. 1

March 2001

PP. 237～252

A SIMULATION STUDY ON ENERGY SAVING EFFECT OF TRAIN OPERATION1

李治綱 Chi-Kang Lee2

孫志煌 Chin-Hung Sun3

（88年12月20日收稿，89年7月21日修改，89年12月6日定稿）

ABSTRACT

Most energy resources used in Taiwan are imported, and Taiwan has been encountering the power shortage problem in recent years. The railway operator is a heavy user of the power system, and energy saving is an important topic for the railway operation. Therefore, an object-oriented programming (OOP) simulation system is developed to assess the energy saving efficiency of train operation. The simulation system was built by Visual C++ on the basis of object modeling technique (OMT). Three models of OMT are utilized, and they are object model, dynamic model, and function model. The simulation system consists of a train movement simulator, a train dispatching simulator, and a power consumption simulator. We test the following energy saving strategies: (1) maximum speed reduction, (2) reduced acceleration rate, (3) anticipatory coasting, and (4) saw-tooth coasting. The major findings in the case study of Taiwan’s high-speed  following.  (1) The relationship of running time and power consumption is not linear, (2) the reduction of maximum speed is effective for

1.The authors are grateful to the financial support from National Science Council (NSC89-2211-E-006-047).

2.Professor, Department of Transportation and Communication Management, National Cheng Kung University.

3.Master, Department of Transportation and Communication Management, National Cheng Kung University.

railway are as follows. (1) The relationship of running time and power consumption is not linear, (2) the reduction of maximum speed is effective for all-stop train service, (3) the anticipatory coasting is effective for three-stop or direct train service, and (4) the anticipatory coasting is also effective for a test of timetable operation.

Key Words: Railway transportation; Object modeling technique; Energy management

I. INTRODUCTION

Almost all energy resources are imported in Taiwan, and Taiwan has the power shortage problem during summer for some years. Since the power supply can not be increased in a short period, demand management of power consumption becomes one important policy of Taiwan government. In the near future, Taiwan railway systems will grow fast in the urban area for mass transit and in the inter-city area for high-speed rail. Because the railway system is a heavy user of power supply, the energy saving strategy of train operation is essential, not only for the operator but also for the energy management of the society. Therefore, the objective of this study is to investigate the energy saving effect of train operation.

A simulation system was developed for the study, and it consists of a train movement simulator, a train dispatching simulator, and a power consumption simulator. The system is a tool to study the performance of train operation, so as to calculate the power consumption. The simulation system is an object-oriented Programming (OOP) model, built by Visual C++, on the basis of object modeling technique (OMT). The OMT utilized in the study includes object model, dynamic model, and function model. In the study, we test the following energy saving strategies: (1) maximum speed reduction, (2) reduced acceleration rate, (3) anticipatory coasting, and (4) saw-tooth coasting. Each strategy or a combined strategy is tested, for one train simulation with a specific stop pattern, or for a timetable simulation with multiple trains and various stop patterns.

The organization of the paper is the following. Section 2 describes the simulation model, and Section 3 states the OMT used in the modeling and programming process. Then, section 4 introduces energy saving strategy and the data of Taiwan high-speed rail. After that, section 5 gives the testing results and discussion. Finally, concluding remarks are made in section 6.

II. THE MODEL

In order to calculate the performance of train movement on a railway line, a simulation system was developed. It mainly consists of three parts: a train movement simulator, a train dispatching simulator, and a power consumption simulator. They are respectively described briefly in the following.

2.1 Train Movement Simulator

The heart of the simulation model is a train movement simulator. It simulates the motion of a train along a railway so as to obtain the trajectory of the train, given the input data representing the track configurations, the vehicle operating characteristics, electric supply system characteristics, and operation conditions. That is, the train movement simulator can generate the output, which describes the running speed, running time and the traction of the train over distance.

It is well known that the speed and running time of a train can be calculated by solving the differential equations derived from Newton’s law of motion [1,2]. However, it is much easier and more efficient to obtain the trajectory of a train by discrete simulation techniques[3]. Assuming the acceleration of the train is a constant over a very short section, the equations of motion can be integrated to compute the speed of the train at the end of the section with a given speed at the beginning of the section. Similarly, given the train speed at the end of the section, the speed of the train at the beginning of the section can be calculated backward. In summary, three basic characteristics the simulator are listed in the following. (1) The simulator simulates the train movement with consideration of the function of the railway system, instead of every operation of a system unit. (2) The train movement simulator is used to plot the trajectory of a train movement, in terms of velocity-distance, acceleration-distance, and so on. (3) There is no consideration of the interactions among the trains on the railway line or network.

2.2 Train Dispatching Simulator

| The objective of train dispatching model is to simulate the commands of the railway traffic control system to each train moving on the railway network. In general, the traffic control system consists of a fixed block signaling system and a centralized control system. According to the traffic condition at a time interval, the train dispatching simulator updates the display of each block signal or the display of cab signal, as well as generates commands from the dispatcher in the centralized traffic control center to the drivers[4,5,6]. The result of the train dispatching simulator at the time interval will create constrains for calculating the trajectory of train movements at and after that time interval. An example of the effect of the block signal on the calculation of the trajectory of two trains is illustrated in Figure 1. Note that there are two types of speed constrain. One is for signal and the other is for speed limit. The curves due to the signal are computed in the train dispatching simulator and dependent on the movement of the preceding train. Hence, the curves have an effect on the movement of a successive train. However, the curves due to station stop or speed limit are independent on train movement. R YY | ：red ：double yellow |
| --- | --- |
| Y G | ：single yellow ：green |
| S | ：signal position |

Figure 1  An Example of Backward Envelope Curves

With the consideration of the traffic control system in the train dispatching simulator, the whole simulation model can simulate the relationships of all trains on the railway network at each time interval, where the movement is calculated by the train movement simulator. In summary, the simulator is a tool to represent the traffic control system of the railway network, and it is used to simulate the interactions among train movements.

2.3 Power Consumption Simulator

The objective of the power consumption simulator is to compute the power consumption with the train movement on the railway. In the literature, several methods have been developed, given various input data and assumptions, for calculating the power consumption and the voltage drop with a DC or AC electric supply system [7,8,9,10]. For example, Chang’s method need some detail information of the electric system of the train and power supply system, Hill’s method assumed the train moves at a constant speed during a section of two auto-transformers, and HSRB’s method can only simulate the power consumption for one train operation. In this study, due to the data availability of the test problem, a simplified calculation method is selected for the case of multi-train operation. Firstly, the traction force F and speed S, which computed in the train movement simulator at each interval, given the train characteristic of traction force and speed, can calculate the required power rate P for the train movement in that interval by P=FS=VIθ. Secondly, electric current I can be computed with the train characteristic curve of traction force and current. Thirdly, given the power factor θ, the voltage V is obtained by FS/Iθ. Then, the voltage value is checked whether it is in the acceptable range or not. If the result is negative, the capacity of power supply system is not enough for the operation and the train has to decrease its speed. In summary, the power consumption simulator used in this study is only to compute power consumption and to check the voltage drop. The total energy consumption of the train E is computed according to the following formula E=Pdt, where t is the time. Moreover, the power consumption of a timetable is the sum of the total energy consumption of train running in the timetable.

III. OBJECT MODELING TECHNIQUE

Object modeling technique (OMT) is useful for building the simulation computer program. It is different from the structure analysis in traditional programming process [11], primarily due to its emphasis on the identification and organization of application domain [12]. There are many advantages in using OMT, such as encapsulation, inheritance, polymorphism, and so on.  In brief, the computer program developed on the base of OMT is very much flexible in its implementation with various changes of system characteristics.

3.1 Object Model

The object model illustrated in Figure 2 represents the static, structural, and data aspect of the simulation model described in section 2. There are two main facilities, or two types of data, in the simulation system: train and track. Track characteristics include transforming station, block signal, route configuration, and station. Train characteristics include traction-speed curve, brake-speed curve, traction-current curve, and others. The train speed during each simulation interval is dependent on the forward movement trajectory, the speed limit backward envelope curve, and the block signal backward envelope curve, which is described in Figure 1. Hence, there are some operations used to generate these objects: e.g. calculation of envelope curve of speed limit, calculation of envelope curve of block signal, and so on. These calculations are primarily based on kinematics equations with the input data stated above. Therefore, some links in Figure 2 represent data providence and some links represent data operation.

3.2 Dynamic Model

A dynamic model describes the aspects of the simulation system concerned with time and the sequencing of operations. An example of state and event diagram is shown in Figure 3. There are 4 states for a train in the simulation process, and they are: stop at a station or in front of a signal, acceleration as the result of forward calculation, running in accordance with an envelope curve of speed limit, and running on a envelope curve of signal. The envelope curves were already described in Figure 1 and in section 2. At each time interval, a train speed is computed in accordance to the process illustrated in Figure 5, with given initial speed and the data of the train and the track. When the train speed is compared with the speed at the envelope curves at the train location, there are many possible results and each result is one event. As shown in Figure 3, one event may have an effect on the change of the state of the train. For example, assume one train is at the state of acceleration. If the speed of forward calculation S is higher than the speed at the envelope curve of speed limit SSL, and it is lower than the speed at the envelope curve of block signal SBS; then the train will follow the speed at the envelope of speed limit. Therefore, the state of the train will change to the state of speed limit envelope curve.

Track

Kinematics

Equations

Power Consumption

Equations

Speed Limit

Envelope Curve

Block Signal

Envelope Curve

Movement Trajectory

Current, Voltage,

Power Rate, etc.

Length,

Weight, etc.

Traction-speed

Curve

Brake-speed

Curve

Traction-current Curve

Route

Signal

Station

Transformer

Grade

Curve

Train

Figure 2  The Object Model of Train Movement and Power Consumption

Start

SBS>S>SSL

Acceleration

Speed Limit

Envelope

Curve

Stop

Block Signal

Envelope Curve

S=SSL=0

SBS,SSL>S

END

SSL>S>SBS

S=SBS=0

SBS,SSL>S

S>0

S: speed; SSL: Speed at Speed Limit Envelope Curve; SBS: Speed at Block Signal Envelope Curve.

Figure 3  The State Diagram of Train Movement

Figure 4  The Function Model of Train Movement and Power Consumption

3.3 Function Model

A function model describes the way of value transformation. It captures what the simulation does, but not how and when. In other words, a function model is used to explain the operation in an object model and the action in a dynamic model. For example, Figure 4 is the function model for train movement and power consumption, and Figure 5 is the process to find the end location of train with its initial location and other related data.

Figure 5  The Process for Calculating Train Speed and Location

IV. ENERGY SAVING STRATEGY

4.1 Energy Saving Strategy

Many researchers have studied the energy saving effect of train operation with various methods. Eash [13] considered the effect of coasting by a simple formula. Uher et al. [14] made a survey of energy saving strategies used in several western cities. Mellitt et al., [4] found a promising result by simulation for the usage of an expert system to judge the place and time for coasting. Hsieh, et al. [15] calculated the most efficient operation pattern of train operation for time or energy consumption by optimal control method. In this study, the following four concepts are tested for energy saving operation:

(1)the reduction of maximum speed, e.g. running the train with the maximum speed of 280 Km/h instead of 300 Km/h;

(2)the reduction of maximum acceleration rate, e.g. running the train with 90% of the maximum acceleration when it is in the state of acceleration;

(3)coasting, e.g. starting the train to the coasting state at the place 50% earlier than its original initial place of deceleration; and

(4)Saw-tooth coasting, e.g. running the train in the coasting state during a speed range between 300Km/h and 275 Km/h.

Since various parameters can be used for each strategy, and a new combined strategy can be defined, the total number of possible experiments is very big.

4.2 Data and Experiment Design

The basic data used in the planning and design stages for Taiwan high-speed rail by Taiwan High Speed Rail Bureau is the input data in this study. The input data is listed as follows. (1) Taiwan high speed-rail is about 360 Km long. (2) There are 10 stations and 7 electric transforming stations. (3) There are detail data for grade, curve, and geometric design. (4) The maximum train speed is 300 Km/h.  (5) The speed limit is roughly 4.4 times root square of the curve. (6) The resistant force is 11000 plus 50 times speed, and 1.1333 times speed square. (7) Train characteristic curves are available for traction force and speed, brake force and speed, as well as traction force and current. (8) The length and weight of the train are given. (9) The characteristic of the transforming station is given (25 kilo-volt A.C.). (10) The acceptable range for voltage drop is given. Note that, the train is not finally selected yet for Taiwan high speed-rail system, the train characteristic curves of German ICE train is used in the study.

The input data described above are basic uncontrolled variables in a simulation run. However, there are many other variables, which can be controlled in the study. They are (1) load factor of the train (50%, 75%, and 100%), (2) stop pattern of the train (all-stop train, three-stop train, and direct express train), (3) operation pattern (one train operation or the timetable operation), and (4) a specific energy saving strategy with a set of specified parameters. A set of the control variables defines one simulation experiment. Because the simulation model in the study is deterministic in nature, only one computer run is required for one experiment.

V. TESTING RESULTS

The basic result of one simulation run for an all-stop train includes a distance-speed graph, a distance-current graph, a distance-voltage graph, a distance-power rate graph, etc. The running time and energy cost form the initial station to the final station on Taiwan High Speed Line are about 110 minutes and 30,000 New Taiwan dollars ($NT) for an all stop train. After many test runs, the load factor is usually not sensitive to the running time and power consumption. The trade-off relationship between energy consumption and running time is illustrated in Figure 6 for the strategy of reduction of maximum speed and the case of all stop train. It is clear that the relationship is not linear. For example, if the maximum speed is decreased from 300 Km/h to 270 Km/hr, the energy consumption decreases by 11% and the running time increase by 3.9%. In other words, the operator saves about $NT 3000, but the passenger spends 4 more minutes in the train. For the strategy of reduction of acceleration, when 80% acceleration is used, the increase of running time is 3.8 minutes and the saving of power consumption is only 4.7%. Hence, for the case of all stop train, the effect of the acceleration reduction is lesser than that of the reduction of maximum speed. Moreover, the effect of coasting or that of saw-tooth coasting is also not as good as that of the reduction of maximum speed. However, the effect of coasting or saw-tooth coasting may improve quickly if the number of stops is decreased.

For the case of three-stop train service, the simulation results of the four energy saving strategies are graphed in Figure 7. It is evident that the anticipatory coasting strategy gives the most promising result. Furthermore, if one-hour timetable is simulated, the effect of the four strategies is graphed in Figure 8. The one-hour timetable has 10 departure trains: 2 direct trains, 3 3-stop trains, 4 5-stop trains, and 1 all-stop train. It is evident that the coasting strategy is still the best.  Many other tests can be done, for example, simulate the strategies with various values of control variables. Besides, there are many possible combined strategies, and there are various operation patterns (stop pattern and train frequency) in a timetable. In other words, a good experiment design is required, so as to produce some practical rules for selecting energy saving strategies in accordance with a given operation pattern.

Figure 6  The Effect of Maximum Speed Reduction for an All-stop Train

Figure 6  The Effect of Maximum Speed Reduction for an All-stop Train

Max Speed Reduction

Reduced Acceleration

Saw-tooth Coasting

Anticipatory Coasting

Figure 7  The Effects of the 4 Strategies for a 3-stop Train

Figure 8  The Effects of the 4 Strategies for 1-hour Timetable Operation

VI. Concluding Remarks

The paper presented a study on the energy saving effect of train operation, with the data of Taiwan high-speed rail by an OOP simulation model. In the study, the simulation model was developed on the basis of object modeling technique. It is a data oriented modeling and programming technique, so that the simulation system can be very flexible in various application situations. The major findings of the study are the following. (1) The trade-off between time and energy is nonlinear. (2) One strategy may not effective for all operation patterns; e.g. maximum speed reduction is effective for all stop train but not effective for direct express train. (3) Anticipatory coasting is effective for the test timetable operation. However, many and detailed experiments should be run to obtain a strategy plan for energy saving operation in practice. Moreover, we should discuss the trade-off between energy (cost) and time not only from the point of view of operator, but also from the passenger’s viewpoint.

REFERENCES

| - 1. | - Andrews, H. I., Railway Traction, Elsevier Science Publishers, U.K.,1992. |
| --- | --- |
| - 2. | - Inada, N., Koga, S., and Tanifuji, K., “Development of Program System for Train Performance Computation”, Quarterly Report of RTRI, Vol. 16, No. 3, 1975, pp. 119-123. |
| - 3. | - Uher, R. A. and Disk, D. R., “A Train Operations Computer Model”, In Murthy, T.K.S. et al. (eds.), Computers in Railway Operations, pp. 253-266, WIP Press, U.K., 1987. |
| - 4. | - Lee, C. K., “Delay Analysis of Train Operation”, In Mellitt, B. et al. (eds.), Computers in Railway VI, WIP Press, U.K. 1998, pp. 515-522. |
| - 5. | - Lee, C. K., “A Comparison on the Headway Estimates of Kaohsiung Orange Line”, In Van de Voorde, E. et al. (eds.), World Transport Research, Elesevier Science Ltd., 1999. |
| - 6. | - Nock, O. S., Railway Signaling, Institution of Railway Signaling Engineers, U.K., 1993. |
| - 7. | - Chang, C. S., “Digital Computer Simulation and Optimisation of 50 HZ AC Electrification”, Ph.D. thesis, University of Birmingham, U.K., 1975. |
| - 8. | - Hill, R. I. and Cevik, I. H., “On-line Simulation of Voltage Regulation in Autotransformer-fed AC Electric Railroad Traction Network”, IEEE Traction on Vehicular Technology, Vol. 42, No. 3, 1993, pp. 365-372. |
| - 9. | - HSRB, “Impact Assessment of High Speed Rail Traction Load on Taiwan Power Quality”, HSRB, DOTC, Taiwan, R.O.C., 1997. |
| - 10. | - Majumdar, J., The Economics of Railway Traction, Gower Publishing CO., 1985. |
| - 11. | - Yourdon, E., Modern Structured Analysis, Prentice-Hall, Inc., U.S.A., 1989. |
| - 12. | - Rumbaugh, J., Blaha, M., Premerlani, W., Eddy, F., and Lorensen, W., Object-oriented Modeling and Design, Prentice-Hall, Inc., U.S.A., 1991. |
| - 13. | - Eash, R. W., “Energy Efficient Rail Transit Operation”, Transportation Research Record, Vol. 662, 1978, pp. 1-7. |
| - 14. | - Uher, R. A., Reduction of Peak-power Demand for Electric Rail Transit Systems, National Cooperative Transit Research and Development Program, Transportation Research Board, U.S.A., 1983. |
| - 15. | - Hsieh, W. J, and Lee, C. K., “Calculation of the Fastest and Most Energy Saving Running Times of Train Operation”, Journal of Chinese Institute of Transportation, 1997. |
| - 16. | - Lee, C. K., Ding, K. L., and Hwang, T. S., “A Train Movement Simulation Model”, Journal of the Chinese Institute of Transportation”, Vol. 10, No. 2, 1997, pp. 119-134. |
| - 17. | - Yoshikawa, K., “Systemization of Train Planning and Operation Tasks－the Present and Future Situations”, Quarterly Report of RTRI, Vol. 33, No. 4, pp. 280-284. |