交通部運輸研究所委託研究計畫出版品摘要表

出版品名稱：

我國都市地區運輸系統管理策略對

於能源消耗與環境(空氣)污染之影響研究 國際標準書號（或叢刊號） 政府出版品統一編號 運輸研究所出版品編號 本所主辦單位：綜合技術組 主管： 計畫主持人： 研究人員： 委託研究單位：邱穀工程顧問有限公司 計畫主持人：賴俊良 研究人員：陳昌顯、張杏珍、林宜達、林肇光。 地址：台北市南京東路五段399號8樓 聯絡電話：(02)7699612 研究期間 自85年8月 至86年5月

關鍵詞：運輸系統管理、能源消耗、環境污染 摘要： 都市地區之交通壅塞為許多國家所面臨之共同問題，其對於能源及環境保護層面之影響，不僅頻繁的車輛停等造成運輸能源使用效率低，所排放出來的廢氣亦影響空氣品質，世界各國乃研擬各種運輸系統管理策略因應，期能利用現有道路設施，以低成本、時間短的管理方法，以提高道路的使用效率，進而減少能源消耗及降低環境污染。 本計畫即以台北市為範圍，選擇五項運輸系統管理策略，包括：南京東路公車專用道、重慶南路之號誌連鎖、八德路及長安東路之配對單行道、陽明山花季假日交通管制、光復橋之調撥車道，除此之外也選擇仁愛路及信義路之配對單行道及公車專用道作為組合管理策略，以上述路段來比較有無此項策略對於交通之影響。本研究亦利用本土化之資料探討當交通條件(如車速、車流組成)改變時，能源及空氣污染排放之變化，期比較策略實施前後對於能源消耗及空污排放之影響，並計算各策略之成本效益。結果顯示，有些策略有正面的效益，有些則為負面。 出版日期 頁數 工本費 本出版品取得方式 86年 月 凡屬機密或限閱性出版品均不對外公開。一般性出版品，公營、公益機關團體及學校可函洽本所免費贈閱；私人及私營機關團體可按工本費價購。 管制等級： □機密 (□解密日期 年 月 日□主辦單位視情況辦理解密) □限閱 (□解密日期 年 月 日□主辦單位視情況辦理解限) □一般 備註：本研究之結論與建議不代表交通部之意見 PUBLICATION ABSTRACTS OF RESEARCH PROGRAM INSTITUTE OF TRANSPORTATION MINISTRY OF TRANSPORTATION AND COMMUNICATIONS TITLE: EFFECTS OF TRANSPORTATION SYSTEM MANAGEMENT ON ENERGY CONSUMPTION AND ENVIRONMENTAL POLLUTION ISBN(OR ISSN) UNIFORM SERIAL CODE FOR GOVERNMENT PUBLICATION IOT SERIAL NUMBER DIVISION: DIVISION CHIEF: PRINCIPAL INVESTIGATOR: PROJECT STAFF: PHONE: FAX: PROJECT PERIOD FROM: August 1996 TO: May 1997 RESEARCH AGENCY:CKS Consulting Engineers & Planners, LTD. PRINCIPAL INVESTIGATOR: Jiun-Liang Lai PROJECT STAFF: Chang-Shen Chen, Shin-Jan Chang, Yi-Dar Lin, Jaw-Kuan Lin ADDRESS:8F, No. 399, Nan-King E. Road, Sec. 5, Taipei, Taiwan R.O.C. PHONE: (02)7699612 KEY WORD: Transportation System Management, Energy Consumption, Environmental Pollution EFFECTS OF TRANSPORTATION SYSTEM MANAGEMENT ON ENERGY CONSUMPTION AND ENVIRONMENTAL

POLLUTION

The heavy traffic in urban areas usually is a major problem to many countries.  It reduces the efficiency of energy and increases the load of air pollutants.  Many countries develop different kinds of the Transportation System Management (TSM) strategies to solve the problem.  TSM strategies are short-term but low-cost solutions to improve the efficiency of function of existed traffic facilities in urban areas.  Furthermore, they can also lower the energy use and decrease the air pollution.

In this research some arterials of Taipei city are chosen as cases to study the effects of TSM strategies.  Five TSM strategies included bus lane, signal coordination, one-way street system, area license, and reversible lane are selected for the purpose.  The effects of the traffic with and w/o five TSM strategies are compared.  By using local traffic characteristics and transportation management model, this research simulates some traffic major of effectivenesses (MOE‘s; such as traffic travel speeds and traffic flow combinations) in each street.  The relationships between MOE‘s and fuel consumption / air pollution are also found in this research.  Finally energy consumption, air pollution emission and cost-effect in different TSM strategies are estimated and compared.  The results show that not all of the effects of the five TSM strategies are positive; some are negative.

DATE OF PUBLICATION

NUMBER OF PAGE

PRICE

CLASSIFICATION

□SECRET

□CONFIDENTIAL

□UNCLASSIFIED

The views expressed in this publication are not necessarily those of Ministry of Transportation and Communications