用路人駕駛模擬器軟硬體之規劃研究

交通部運輸研究所

中華民國八十七年七月

交通部運輸研究所合作研究計畫出版品摘要表

| 出版品名稱：用路人駕駛模擬器軟硬體之規劃研究 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 國際標準書號（或叢刊號） |  |  |  | 政府出版品統一編號 009104870290 |  | 運輸研究所出版品編號 87-32-3185 |  |
| 本所主辦單位：運輸安全組 主管：林豐福 計畫主持人：林豐福 研究人員：張開國、曹瑞和、 賴靜慧 聯絡電話：(02)2349-6855 傳真號碼：(02)2545-0429 |  |  |  |  | 合作研究單位：國立中央大學機械系 計畫主持人：董基良 研究人員：何明雄、黃維信、張劭卿、 簡志雄、李灝賸、呂岱穎 地址：中壢市五權里38號 聯絡電話：(03)422-0575 |  | 研究期間 自86年8月 至87年5月 |
| 關鍵詞：虛擬實境、駕駛模擬器、車輛動態、互動式 |  |  |  |  |  |  |  |
| 摘要： 本研究參考國內及國外與駕駛模擬器有關之各種軟硬體架構，以及使用駕駛模擬器所完成之相關研究與應用，規劃設計一套車輛駕駛模擬器之軟硬體架構。本研究之重點在於系統雛型之建立，本系統雛型之建立包含駕駛模擬器規格之訂定、電腦軟硬體設備之建立、北二高虛擬場景之建立、車輛動態分析、監控系統之規劃。研究成果如下：(1)收集國外駕駛模擬器資料及研究項目，並對國內不同領域的研究需求做一完整的評估。(2)在所規劃的北二高場景中，已完成部份場景，並加入車輛及簡易車流模式的場景，後續可依規劃持續增加。虛擬實境軟體與資料庫的結合已完成初步規劃。(3) 車輛動態模擬，目前已完成直線及轉彎的分析推導，並和國外研究結果做一完整的驗證工作。(4)駕駛艙感應訊號有方向盤、油門、煞車及排檔。訊號與虛擬實境軟體的搭配，也已完成測試。(5)系統雛型機已做過系統整合測試。而後續的研究課題則可考慮國內汽車對整體車流的影響，再依此系統根據不同的研究目的做相關之研究探討。 |  |  |  |  |  |  |  |
| 出版日期 | 頁數 | 工本費 | 本出版品取得方式 |  |  |  |  |
| 87年 7月 | 209 |  | 凡屬機密或限閱性出版品均不對外公開。一般性出版品，公營、公益機關團體及學校可函洽本所免費贈閱；私人及私營機關團體可按工本費價購。 |  |  |  |  |
| 管制等級： □機密（□解密日期為 年 月 日， □主辦單位視情況辦理解密） □限閱（□解限日期為 年 月 日， □主辦單位視情況辦理解限） 一般 |  |  |  |  |  |  |  |
| 備註：本研究之結論與建議不代表交通部之意見。 |  |  |  |  |  |  |  |

PUBLICATION ABSTRACTS OF RESEARCH PROJECTS

INSTITUTE OF TRANSPORTATION

MINISTRY OF TRANSPORTATION AND COMMUNICATIONS

| TITLE: Study on the Software and Hardware of Driving Simulator |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ISBN(OR ISSN) | UNIFORM SERIAL CODE FOR GOVERNMENT PUBLICATIONS 009104870290 |  |  | IOT SERIAL NUMBER 87-32-3185 |  |
| DIVISION: Transportation Safety DIVISION CHIEF: Lin, Fong-Fu PRINCIPAL INVESTIGATOR: Lin, Fong-Fu PROJECT STAFF: Chang, Kai-Kuo, Tsaur, Ray-Her, Lai, Ching-Huei PHONE: (02)2349-6855 FAX: (02)2545-0429 |  |  |  | PROJECT PERIOD FROM Aug., 1997 TO May 1998 |  |
| RESEARCH AGENCY: Department of Mechanical Engineering, National Central University PRINCIPAL INVESTIGATOR: Doong , Ji-Liang PROJECT STAFF: Ho, Ming-Shiung, Huang, Wei-Shin, Chang, Shao-Ching, Chien, Chih-Shiung, Lee, Hao-Sheng, Lu, Dai-Ying ADDRESS: 38, Wu-Chuan Li, Jung-Li, 32054, Taiwan, R.O.C. PHONE: 886-3-4220575 |  |  |  |  |  |
| KEY WORDS: Virtual Reality, Driving Simulator, Vehicle Dynamic, Interactive Mode |  |  |  |  |  |
| ABSTRACT: The achievements of this current project are as follows : (1) The related research topics and information of driving simulator on broad had been collected. The domestic need of research in different areas had been thoroughly evaluated. (2) Some proposed virtual environments of the Northern Taiwan Second Freeway (No. 3 Freeway), some vehicles and simple traffic flow pattern had been added into this prototype system which will be combined with the data bank to increase the virtual environments as the proposed plan. (3) The vehicle dynamics simulation, the derived mathematics and the analysis in straight line and curve driving had been finished, and was verified with the research works on broad too. (4) The interactive test of the virtual reality software with the reaction signals of steering wheel, acceleration paddle, brake paddle and transmission shift had been performed. (5) The system integration test of the driving simulator prototype was performed. |  |  |  |  |  |
| DATE OF PUBLICATION July, 1988 |  | NUMBER OF PAGES 209 | PRICE |  | CLASSIFICATION □SECRET □CONFIDENTIAL UNCLASSIFIED |
| The views expressed in this publication are not necessarily those of the Ministry of Transportation and Communications. |  |  |  |  |  |