交通部運輸研究所合作研究計畫出版品摘要表

| 出版品名稱: 台中市公車專用道系統規劃與設置可行性研究 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 國際標準書號(或叢刊號) |  |  | 政府出版品統一編號 1009005833 |  |  | 運輸研究所出版品編號 90-75-4140 |  |
| 本所主辦單位: 運輸經營管理組 主管: 林繼國 計畫主持人: 林繼國 研究人員: 張贊育、楊弘道 聯絡電話: (02)2349-6842 傳真號碼: (02)2545-0431 |  |  |  | 合作研究單位：鼎漢國際工程顧問公司 計畫主持人:李俊賢 研究人員:周榮昌﹙顧問﹚、張景興、陳良俊、鍾宜螢、徐千雲、葉南廷 地址:高雄市博愛一路28號7樓 聯絡電話: (07) 322-1772 |  |  | 研究期間 自2000年1月 至2000年11月 |
| 關鍵詞: 公車專用道、大眾運輸系統 |  |  |  |  |  |  |  |
| 摘 要: 為有效提昇台中市大眾運輸系統之經營績效及服務品質，本所依據行政院84.8.23核頒之「促進大眾運輸發展方案」，進行「台中市公車專用道系統規劃與設置可行性研究」(以下簡稱本研究)，以瞭解台中市是否具有設置公車專用道之條件，作為台中市政府未來推動公車專用道之依據。依據本研究結果顯示： 一、在可行性分析方面：(一)工程可行性-台中市市區道路較為狹窄，主要幹道之道路寬度達四十公尺以上者有八條，符合設置公車專用道之標準；另路寬達二十五公尺以上、但未達四十公尺之路段，有41條，亦可彈性設置公車專用道，因此，以台中市目前道路狀況，設置公車專用道尚屬可行；(二)經濟可行性-依據本研究所作成本效益分析結果顯示，本研究所規劃之三套公車專用道路網可行方案之總效益(包括公車乘客時間節省效益、私人運具行車成本節省效益及減少空氣污染之效益)均大於總建造成本，因此，台中市實施公車專用道具有經濟效益；(三)社會接受性-依據本研究所作調查結果顯示，平均六成六之民眾贊成設置公車專用道；(四)財務可行性-依據初步評估，本研究所建議之長期路網方案所需建造總成本為1.4億元。以台中市政府財政狀況，推動公車專用道應無特別困難。 二、在公車專用道路網規劃方面 本研究經過工程、經濟、財務及社會接受性之可行性評估後，評選出台中市短期公車專用道路網為由中正路、中港路﹙捷運藍線﹚、北屯路、雙十路、國光路﹙捷運紅線﹚及振興路﹙捷運藍線﹚所組成之路網方案，並選擇中正路-中港路作為台中市政府最優先推動之示範公車專用道，進行初步之規劃與設計，作為未來台中市政府推動公車專用道之參考。 |  |  |  |  |  |  |  |
| 出版日期 | 頁數 | 工本費 |  |  | 本出版品取得方式 |  |  |
| 90年12月 | 278 | 240元 |  |  | 凡屬機密或限閱性出版品均不對外公開。一般性出版品，公營、公益機關團體及學校可函洽本所免費贈閱；私人及私營機關團體可按工本費價購。 |  |  |
| 管制等級: 限閱 密 機密 極機密 絕對機密 （解【限】密條件：日期為 年 月 日解密；公佈後解密；附件抽存後解密； 工作完成或會議終了時解密； 另行檢討後辦理解密） 普通 |  |  |  |  |  |  |  |
| 備註︰本研究之結論與建議不代表交通部之意見。 |  |  |  |  |  |  |  |

PUBLICATION ABSTRACTS OF RESEARCH PROJECT

INSTITUTE OF TRANSPORTATION

###### MINISTRY OF TRANSPORTATION AND COMMUNICATIONS

| TITLE: A Feasibility Study and Planning on the Bus-exclusive-lane System in Taichung City |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ISBN(OR ISSN) | GOVERNMENT PUBLICATIONS NUMBER 1009005833 |  |  |  | IOT SERIAL NUMBER 90-75-4140 |  |
| DIVISION：Transportation Operations And Management DIVISION CHIEF: Chi-Kuo Lin PRINCIPAL INVESTIGATOR: Chi-Kuo Lin PROJECT STAFF: Tsan-Yu Chang, Hung-Daw Yang PHONE: (02)2349-6842 FAX: (02)2545-0431 |  |  |  | PORJECT PERIOD FROM January 2000 TO November 2000 |  |  |
| RESEARCH AGENCY：THI Consultants PRINCIPAL INVESTIGATOR: Jun-Hsien Lee PROJECT STAFF: Rong-Chang Jou, Ching-Shing Chang, Liang-Chun Chen, Yi-Ying Chung, Chien-Yun Hsu, Nan-Ting Yeh ADDRESS:7F ,.28 Po-Ai 1st Road, Kaohsiung TAIWAN PHONE: 886-7-332-1772 |  |  |  |  |  |  |
| KEYWORDS: Bus Exclusive Lane, Mass Transit System |  |  |  |  |  |  |
| ABSTRACT: In accordance with ”The Alternatives for the Promotion of the Development of Public Transportation Sectors” approved by the Executive Yuan in 1995, this research aims to evaluate the feasibility of a bus-exclusive-lane system in Taichung City. So to improve the operational performance and the bus system’s level of service in Taichung. Based on the results, we concluded that the proposed bus-exclusive-lane system in Taichung is feasible in the assessment of engineering, economic, finance and social acceptance. In addition, three networks of the bus-exclusive-lane system for short-term, mid-term and long-term development in Taichung City are further proposed in this study. Last but not least, the Chung-cheng road and Chung-gang road are selected to be the demonstrative roads for the first priority of bus-exclusive-lane deployment and their sketch planning and design are provided as a reference for further implementation by the Taichung City Government. |  |  |  |  |  |  |
| DATE OF PUBLICATION Dec 2001 |  | NUMBER OF PAGES 278 | PRICE 240 |  |  | CLASSIFICATION  SECRET  CONFIDENTIAL  UNCLASSIFICATION |
| The views expressed in this publication are not necessarily those of the Ministry of Transportation and Communications. |  |  |  |  |  |  |

目　錄

第一章　緒論

1.1 計畫緣起………………………………………………………………………………..…1

1.2 研究範圍…………………………………………………………………………………..1

1.3 研究目的…………………………………………………………………………………..2

1.4 研究流程與內容…………………………………………………………………………..2

第二章　文獻回顧

2.1 公車專用道之分類與設置要件…………………………………………………………….5

2.1.1 公車專用道之定義與分類………………………………………………………..5

2.1.2 公車專用道之設置條件…………………………………………………………..7

2.1.3 設置公車專用道應考慮之因素…………………………………………………..9

2.2 國內實施公車專用道經驗………………………………………………………………….9

2.2.1 台北公車專用道之實施情況……………………………………………………..9

2.2.2 台北經驗之啟示………………………………………………………………….12

2.3 國外實施公車專用道之經驗………………………………………………………………17

2.4 公車動態資訊系統實施之實例……………………………………………………………18

2.4.1 國外公車動態資訊系統實施情形……………………………………………….19

2.4.2 台北市公車動態資訊系統……………………………………………………….19

2.4.3 新竹市公車動態資訊系統……………………………………………………….19

2.4.4 小結……………………………………………………………………………….19

第三章　基本資料蒐集與分析

3.1 台中市道路幾何特性與交通特性…………………………………………………………21

3.1.1 道路幾何特性…………………………………………………………………….21

3.1.2 交通特性………………………………………………………………………….22

3.2 台中市大眾運輸路網與營運狀況之檢討…………………………………………………23

3.2.1 大眾運輸營運路線……………………………………………………………….23

3.2.2 市區公車營運狀況之檢討……………………………………………………….30

3.3 現況與未來運輸需求特性分析……………………………………………………………32

3.3.1 現況運輸需求特性……………………………………………………………….32

3.3.2 未來運輸需求特性……………………………………………………………….33

3.4 相關交通建設………………………………………………………………………………35

3.5 台中市實施公車專用道之限制與機會分析………………………………………………38

3.5.1 整體運輸環境..………………………………………………………………….38

3.5.2 供給與需求……………………………………………………………………….39

第四章　台中市設置公車專用道意見調查

4.1 調查作業……………………………………………………………………………………41

4.2 問卷調查結果………………………………………………………………………………42

第五章　大眾運輸政策

5.1 研擬大眾運輸政策之思路………………….………………………………………………57

5.2 大眾運輸系統整合………………………….………………………………………………58

5.2.1 整合目標與指標………………….……………………………………………….58

5.2.2 路網整合之目標………………….……………………………………………….60

5.3 漸進整合的大眾運輸路網……………….……………………………….……………...60

5.4 公車專用道在大眾運輸路網中之角色….…………………………………………….….63

5.4.1 公車專用道之角色定位……….………………………………………………….63

5.4.2 推動公車專用道之策略……….………………………………………………….64

第六章　方案研擬與規劃

6.1 道路供給面之考慮………………………………………………………………………….67

6.2 需求面之考慮……………………………………………………………………………….72

6.2.1 大眾運輸需求之現況分析…………………………………………………………72

6.2.2 未來運輸需求之分析………………………………………………………………77

6.3 公車專用道路網方案研擬………………………………………………………………….81

6.3.1 公車專用道路網方案之構思………………………………………………………81

6.3.2 公車專用道路網方案………………………………………………………………83

6.4 公車專用道路網方案評估………………………………………………………………….87

6.4.1 評估標的、準則與衡量結果………………………………………………………87

6.4.2 評估結果……………………………………………………………………………90

6.5 公車專用道系統分期分區方案…………………………………………………………….92

第七章　優先示範路段初步工程規劃

7.1 優先路段道路現況與車流特性…………………………………………………………….97

7.1.1 車流特性……………………………………………………………………………97

7.1.2 道路幾何……………………………………………………………………………99

7.1.3 交通管制………………………………………………………………………….101

7.1.4 公車路線………………………….………………………………………………101

7.2 相關交通建設及時程……………………………………………………………………….102

7.3 專用道佈設方式規劃………………………………………………………………………103

7.3.1 規劃考量因素…………………………………………………………………….103

7.3.2 道路工程設計規範及設計要素………………………………………………….104

7.3.3 路線佈設限制之探討…………………………………………………………….105

7.3.4 專用道佈設方式之規劃………………………………………………………….107

7.3.5 專用道鋪面規劃………………………………………………………………….127

7.3.6 排水規劃………………………………………………………………………….127

7.3.7 車道隔離方式…………………………………………………………………….129

7.3.8 公車站位及站台規劃…………………………………………………………….130

7.3.9 景觀規劃………………………………………………………………………….132

7.3.10 乘客進出動線與安全及使用資訊系統規劃……………………………………134

7.4 經費估算與財源籌措………………………………………………………………………135

7.4.1 經費概估………………………………………………………………………….135

7.4.2 財源籌措方式…………………………………………………………………….136

第八章　相關配合措施研議

8.1 公車專用道推動組織………………………………………………………………………137

8.2 施工期間與營運期間交通管理……………………………………………………………142

8.2.1 施工期間配合措施……………………………………………………………….142

8.2.2 營運期間交通管理配合措施…………………………………………………….143

8.3 公車動態系統之規劃與設計………………………………………………………………144

8.3.1 系統技術………………………………………………………………………….144

8.3.2 系統架構與運作模式…………………………………………………………….146

8.3.3 優先示範路段公車動態資訊系統之規劃與設計……………………………….152

8.4 公車專用道行銷計畫研擬…………………………………………………………………155

第九章　結論與建議

9.1 結論……………………………………………………………………………………….157

9.2 建議……………………………………………………………………………………….159

參考文獻………………………..………………………………………………..……………161

附錄

附錄一　實施公車專用道意見調查表……………….……………………………….…….165

附錄二　期初專家學者座談會會議紀錄…………………………………………………… 167

附錄三　期中專家學者座談會會議紀錄…………………………………………………….171

附錄四　期中簡報審查意見辦理情形說明………………………………………………….177

附錄五　期末專家學者座談會會議紀錄…………………………………………………….183

附錄六　期末簡報審查意見第一次辦理情形說明………………………………………….187

附錄七　期末簡報審查意見第二次辦理情形說明………………………………………….193

附錄八　期末簡報資料……………………………………………………………………….199

表 目 錄

表1  工作內容……..……………………………………………………………….……………….3

表2　 各類公車專用道之定義…………………………………………………….……………….6

表3　 公車專用道佈設方式優缺點分析………………………………………….……………….7

表4　 相關文獻中公車專用道之設置條件……………………………………….……………….8

表5　 相關文獻中公車專用道最低設置條件之彙整.………………………….………………..8

表6　 台北市公車專用道之實施概況………………………………………….…………………10

表7　 台北市公車專用(優先)道佈設型式優缺點比較……..……………….…………………11

表8　 台北市公車專用道佈設方式及建設經費……………………………….…………………12

表9　 行經台北市公車專用道公車路線載客數及營收成長情形…………..………………….12

表10　台北市聯營公車84-86年公車行駛輛數成長情形………………………….…………….13

表11　台北市公車專用道實施前後公車平均行駛速率成長……………………….…………..14

表12　公車專用道實施前後相鄰車道上小汽車平均行駛速率變化……………….…………..15

表13　公車專用道實施前後聯營公車肇事統計…………………………………….…………..16

表14　公車專用道各路段交通事故統計…………………………………………….…………..16

表15  公車專用道各項實施績效總結表…………………………………………….…………..17

表16　歐美各國公車專用道之實施經驗……………………………………………….………..18

表17　台中市市區現行單行道路寬彙整表…………………………………………….………..21

表18　台中市25公尺以上道路路寬彙整表……………………………………………….………22

表19　台中市主要幹道服務水準……………………………………………………….………..23

表20　台中客運國道長途客運營運概況……………………………………………….………..25

表21　台中都會區中短途客運營運概況……………………………………………….………..26

表22　台中市市區公車路線營運概況………………………………………………….………..29

表23　台中都會區旅次目的分佈……………………………………………………….………..32

表24　台中都會區運具使用比例……………………………………………………….………..32

表25　旅次目的與使用運具別………………………………………………………….………..32

表26　台中都會區各旅次目的百分比(現況與預測)……..………………………….………..33

表27　台中都會區未來之運具選擇情形……………………………………………….………..34

表28　運具分配率之情境分析………………………………………………………….….………39

表29　有效樣本結構…………………………………………………………………….………..42

表30　各群體對公車專用道之瞭解程度………………………………………………….……..42

表31　提示台北經驗情境下受訪者對公車專用道之支持度……………………………….…..43

表32　實施公車專用道後各運具使用者改搭公車之比例………………………………….…..44

表33　雙向道路上實施公車專用道之民眾支持度………………………………………….…..45

表34　單行道上實施公車專用道之民眾支持度…………………………………………….…..50

表35　取消路邊停車以佈設公車專用道之意見調查結果………………………………….…..51

表36　佔用車道以佈設公車專用道之意見調查結果……………………………………….…..52

表37　移植行道樹以佈設公車專用道之意見調查結果…………………………………….…..53

表38　民眾對各公車專用道實施時段之接受性…………………………………………….…..54

表39  民眾支持之公車專用道行駛方向…………………………………………………….…..55

表40　台中市公車專用道工程可行之道路……………………………………………………...68

表41　台中市市中心區現行單行道路寬彙整表………………………………………………...69

表42　研究範圍內各分區私人及大眾旅次分佈量……………………………………….……..73

表43　研究範圍內各分區旅次運具使用比例…………………………………………….……..75

表44　台中都會區民國110年無捷運系統方案之旅次需求……………………………….…….79

表45　公車專用道路網方案一……………………………………………………………….…..83

表46　公車專用道路網方案二……………………………………………………………….…..85

表47　公車專用道路網方案三……………………………………………………………….…..86

表48　公車專用道路網評估指標之衡量方式……………………………………………….…..87

表49　台中客運22號路線公車各方案績效值……………………………………………….……88

表50　公車專用道路網各方案之建造成本…………………………………………………….…88

表51  經濟效益分析參數彙整表…………………………….……………………………….….89

表52　各方案之經濟效益………………………………………………………………………...90

表53　公車專用道路網各方案所需移植行道樹之長度………………………………………….90

表54　各方案評估指標績效值與標準化值……………………………………………………….91

表55　路網方案綜合評估表……………………………………………………………………….92

表56　第一期公車專用道實施路線……………………………………………………………...93

表57　第二、三期公車專用道實施路線……………….………………………………………..93

表58　路口轉向交通量─下午尖峰…………………….………………………………………..98

表59　台中港路路段實質幾何配置情形彙整表……….……………………………………….100

表60　中正路及台中港路現況市公車招呼站位及尖峰流量.………………………………….102

表61　公車專用道設計要素………………………………….………………………………….104

表62　台中港路及中正路專用道佈設型式優缺點比較…….………………………………….107

表63　台中港路專用道路段橫斷面規劃配置……………….………………………………….111

表64　台中港路公車專用道站台席位數及長度估算……….………………………………….130

表65　坡道高低差與坡度關係……………………………….………………………………….135

表66　優先示範路段建設經費估算………………………….………………………………….136

表67　不同發車班距情境下之車隊規模估計…………………………………………………..141

表68　各類定位技術優缺點比較表………………………….………………………………….145

表69　無線通訊技術特性…………………………………….………………………………….146

表70　乘客、公車業者與主管機關與各子系統之需求關係.………………………………….147

圖 目 錄

圖1 　本計畫規劃及研究範圍………………………………………………………………………2

圖2 　工作流程圖……………………………………………………………………………………4

圖3 　公車專用道之分類……………………………………………………………………………5

圖4 　台中市區公車路線兩側300公尺服務範圍…………………………………………………31

圖5 　旅次目的與使用運具別…………………………………………………………………….33

圖6 　民國110年運輸走廊運量分佈情況…………………………………………………………34

圖7 　台中市主要道路及重大建設位置圖……………………………………………………….37

圖8 　各群體對公車專用道之瞭解程度………………………………………………………….43

圖9 　提示台北經驗情境下受訪者對公車專用道之支持度…………………………………….44

圖10　實施公車專用道後各運具使用者改搭公車之比例……………………………………….45

圖11　各種道路寬度條件下，實施公車專用道之民眾支持度………………………………….46

圖12　取消路邊停車以佈設公車專用道之意見調查結果……………………………………….51

圖13　佔用車道以佈設公車專用道之意見調查結果………….…………………………………52

圖14　移植行道樹以佈設公車專用道之意見調查結果………………………………………….53

圖15　民眾對各公車專用道實施時段之接受性………………….………………………………54

圖16　民眾支持之公車專用道行駛方向………………………………………………………….55

圖17　大眾運輸系統服務水準降低之惡性循環………………………………………………….59

圖18　大眾運輸路網整合發展示意圖…………………………………………………………….61

圖19　台北市南京東路公車專用道……………………………………………………………….62

圖20　研擬可行系統方案之程序與架構………………………………………………………….67

圖21　台中市公車專用道工程可行之道路……………………………………………………….69

圖22　40公尺寬雙向道之公車專用道橫斷面配置………………………….……………………70

圖23　30公尺寬雙向道之公車專用道橫斷面配置……………………………………………….70

圖24　30公尺寬雙向道之公車專用道平面配置圖……………………………………………….70

圖25　30公尺寬單行道之公車專用道橫斷面配置…………………………………………….…71

圖26　30公尺寬單行道之公車專用道平面配置圖…………………………………………….…71

圖27　25公尺寬單行道之公車專用道橫斷面配置…………………………………………….…72

圖28　都會區尖峰小時總旅次分佈…………………………………………………………….…77

圖29　都會區尖峰小時大眾運輸旅次分佈…………………………………………………….…77

圖30　台中都會區民國110年大眾旅次需求分佈圖……………………………………………..80

圖31　台中都會區主要運輸走廊示意圖………………………………………………………….81

圖32　台中市路幅大於25公尺之道路………………………………………………….…………82

圖33　台中公車專用道路網方案示意圖……………………………………………….…………83

圖34　公車專用道路網替選方案一…………………………………………………….…………84

圖35　公車專用道路網替選方案二………………………………………………………….……85

圖36　公車專用道路網替選方案三………………………………………………………….……86

圖37　分期分區方案之路網圖……………………………………………………………….……94

圖38　台中港路慢車道佈設現況公車站…………………………………………………….…100

圖39　台中捷運藍線出土段配置(惠華路~惠中路間之台中港路)……………………………103

圖40　光明路跨越陸橋及南側平面側車道現況……………………………………………….105

圖41　中正路車道佈設現況…………………………………………………………………….106

圖42a 五權路～民權路現況斷面圖…………………………………………………………….108

圖42b 五權路～民權路公車專用道路段斷面圖……………………………………………….108

圖43a 民權路～忠明南路現況斷面圖………………………………………………………….109

圖43b 民權路～忠明南路公車專用道路段斷面圖…………………………………………….109

圖44a 高速公路西側匝道～國際街現況斷面圖……………………………………………….110

圖44b 高速公路西側匝道～國際街公車專用道路段斷面圖………………………………….110

圖45a 忠明南路～朝富路現況斷面圖………………………………………………………….111

圖45b 忠明南路～朝富路公車專用道路段斷面圖…………………………………………….111

圖46a 80米外環道～高速公路西側匝道現況斷面圖………………………………………….112

圖46b 80米外環道～高速公路西側匝道公車專用道路段斷面圖…………………………….112

圖47　光明陸橋現況斷面圖…………………………………………………………………….112

圖48a 中正路現況斷面圖……………………………………………………………………….113

圖48b 中正路公車專用道路段斷面圖………………………………………………………….113

圖48c 中正路逆向中央式公車專用道路段斷面圖…………………………………………….114

圖49  五權路～民權路公車專用道路口斷面圖……………………………………………….114

圖50  民權路～忠明南路公車專用道路口斷面圖…………………………………………….115

圖51  忠明南路～朝富路公車專用道路口斷面圖…………………………………………….115

圖52  高速公路西側匝道～國際街公車專用道路口斷面圖………………………………….116

圖53a 中正路逆向路緣式公車專用道路口斷面圖－路口西側……………………………….116

圖53b 中正路逆向路緣式公車專用道路口斷面圖－路口東側……………………………….117

圖53c 中正路逆向中央式公車專用道路口斷面圖－路口西側……………………………….117

圖54  台中港路公車專用道平面佈設─五權路路口…..…………………………………….119

圖55  台中港路公車專用道平面佈設─民權路路口…………………………..…………….120

圖56  台中港路公車專用道平面佈設─英才路路口………………………………..……….121

圖57  台中港路公車專用道平面佈設─美村路路口……………………………………..….122

圖58  台中港路公車專用道平面佈設─忠明南路路口……………………………..……….123

圖59  台中港路公車專用道平面佈設─漢口路路口……………………………………..….124

圖60  中正路逆向公車專用道平面佈設(五權路～中華路)………………………………….125

圖61  中正路逆向公車專用道平面佈設(中華路～三民路)………………………………….126

圖62  改良後瀝青混凝土示意圖……………………………………………………………….128

圖63  剛性路面橫斷面圖……………………………………………………………………….128

圖64  專用車道分隔設施之排水示意範例…………………………………………………….129

圖65  不同鋪面顏色之車道區隔……………………………………………………………….129

圖66  台中港路公車專用道停靠站位置圖…………………………………………………….131

圖67  台中港路路樹樹種現況………………………………………………………………….132

圖68  國外公車亭、站牌及廣告參考範例…………………………………………………….133

圖69  日本街道及公車站照明範例…………………………………………………………….133

圖70  日本地坪舖面範例……………………………………………………………………….134

圖71　公車專用道之權責部門…………………………………………………………………..138

圖72　相關配合標誌及標字附牌示意圖……………………………………………….………144

圖73　台中市公車動態顯示系統之規劃架構………………………………………….………147

圖74　調度站子系統之資訊傳入示意………………………………………………….………151

圖75　台北市公車動態資訊顯示系統架構…………………………………………….………153

圖76　公車候車亭示意圖(立面圖)…………………………………………………….………154

圖77　公車候車亭示意圖(剖面圖一)………………………………………………….………154

圖78　公車候車亭示意圖(剖面圖二)………………………………………………….………155

圖79　公車專用道行銷廣告折頁(正面)……………………………………………….………156

圖80　公車專用道行銷廣告折頁(反面)……………………………………………….………156