A Reasonable Site Selection Process of Public Facility

Student : Kuo-Cheng Lee              Advisor : Dr. Cheng-Min Feng

# Institute of Traffic and Transportation

National Chiao Tung University

## Abstract

Public facilities having characteristics of social welfare are commonly necessities in our daily life within a community .Therefore , the selection of the facility site must consider both efficiency and equity .The objective of the study is to develop a mechanism in corporating both efficiency and equity in the site selection process.

In this study , Indices were established first and used in this study to evaluate if a site allocation plan is efficient and equitable .A decision model is then established through a joint application of location analysis model and GIS for information processing ,analysis and display. A site selection plan of a community park in an urban plan area in Kelung city was used as a case study.

Indices used to evaluate site selection plan are all scaled to the range of 0 and 1.The closer the index value is to 1 represents the more efficient and equitable is the facility plan Contrarily ,the closer the index value is to zero stands for the lesser efficient and equitable is the facility site plan.

An overall decision process on the site selection of a facility system comprises of following steps:

1.Define study area.

2.Delineate sub-zone , sub-zone centroid and demand measure.

3.Determine facility size.

4.Preliminary screen of alternative facility sites.

5.Propose alternative site plan for a public facility system.

6.Evaluate alternative plan.

7.Recommend best alternative plan.

Basically , above said indices can be used to evaluate alternative site selection plans of any public facility system . In this study ,a modified P-median model serves as an alternative generator and an allocation model written in Lingo languages used to evaluate alternative plans. Both make overall analysis easy and effective.

Case studies for a public facility located in a new urban plan area and one in an old urban plan area were conducted respectively .

Keywords : public facility, location ,urban plan , equity , efficiency.