A Study on Timed Transfer Model for Connection of Railway and Bus Routes at Transfer Terminals

# Srudent: Ching-Fung Chen                 Advisor: William Jen

Institute of Transportation Engineering and Management

National Chiao Tung University

## ABSTRACT

This study attempts to develop a timed transfer model for railway and bus route operations. Schedule synchronization with various routes in a transfer terminal may reduce transfer delay in the terminal. Since vehicle arrivals are stochastic, we may design slack time in the vehicle schedule to reduce the probability of missed connection of the transfer passengers. However, when slack time is in the schedule, it may result in vehicle operation cost increase, so we must consider it systematically.

In the past, most of the literatures allowed focus on timed transfer models for buses. Because of the differences of operation characteristics of trains and buses, the factors considered in the timed transfer model for both modes are also different. The cost components in our timed transfer model include non-transfer cost and transfer cost. The non-transfer cost includes (1) vehicles operating cost;(2) passenger waiting cost in origin terminals; and(3) passenger in-vehicle cost. The total transfer cost function includes all cost components that are influenced by slack time and holding time. For example, if holding the bus departures at the transfer terminal, the induced cost on some compensated strategies (e.g. signal preemption) must be considered.

Trains operate along a fixed track. As a result, they are not affected by other traffic situations and have higher schedule adherence. The rider-ship and operation cost of railway operations are also higher than buses, so it is not suitable for introducing a slack time in the train schedule. As train is late, we can adopt holding bus departure time to increase the successful transfer probability of train passengers. Similarly, holding time may increase bus operation cost and non-transfer passenger waiting cost. Besides, if we don’t recall the initial bus headway, buses may possibly bunch together on the routes and will eventually influence the bus company’s operation.

There are three alternatives for integrating the schedules of railway and buses at the transfer terminal. They are (1) uncoordinated operating; (2) coordinated operating with common headway; and (3) coordinated operating with integer-ratio headway. We have applied an analytical model to explore the effects of different schedule coordination options. We also built an example for sensitivity tests. The results showed that the transfer cost of the system is higher than coordinated operations if the schedules were not coordinated. The policy of coordinated train schedules should be train schedule based. In addition, rather than a non-coordinated strategy, it is advantageous to use a common headway or an inter-ratio headway in our transfer system.