| ### Title of Thesis |  |  |  | Total pages:143 |
| --- | --- | --- | --- | --- |
| A study of vehicle classification and tracking of continuous traffic images at night |  |  |  |  |
| Key words: night vehicle detection, night traffic flow images, vehicle tracking, vehicle classification, segmentation of vehicle rear light |  |  |  |  |
| Name of Institute: |  |  |  |  |
| ## Graduate Institute of Transportation Science, Tamkang University |  |  |  |  |
| Graduate date: | Jan 2002 |  | Degree conferred:Master Degree |  |
| Name of student: | Sheng-Iuan Chuang |  | Advisor: Dr. Chun-Hai Fan |  |
|  | 莊盛淵 |  | 范俊海 博士 |  |
| Abstract: The image process of daytime or night has big difference. In daytime, the light is sufficient, however, the shadow is the main problem. At night, the problem of light source such as the street lights, the reflection of vehicle lights… is complicated. In this case, this research is especially on the emphasis the random-discussed night traffic flow images, and the first step investigation is proceeding in the sunny day. “The algorithm in night vehicle detection” developed in this research is separated into two periods. The first period is operated independently in each car lane and the work includes these following main parts, “ moving detection”, “vehicle tracking” and “vehicle classification”. “Moving detection” is mainly according to the beam coming from the night vehicle（whatever it is the vehicle light or the reflected light from the vehicle body）. In the continual images, the characters of the light blob itself and of the relation such as distance or angle between each could be the bases of classification. To do the work of “vehicle tracking”, the light blobs can be told from one vehicle because of the somehow stable advance of a mass of lights. Working on the “vehicle classification” jobs, we covert the width of the frame, which covers the mass of lights of one vehicle to the width in the real space, then we know the real width oh vehicle. Thereupon, the size of vehicles could be classified in accord with the width of small or big vehicle, which is defined in this research. However, the independent operation in each car lane may result in the mistake over the vehicles, which change lanes. The second period “grouping changing-lane vehicle” is brought up. That is to say, in the bases of the character of the same time points, we classify those light blobs, which are of changing-lane vehicle as one vehicle. |  |  |  |  |
| In the process of blob-marked, this research develops “Second-Minimum-Slop Method” to segment the rear lights in order to meet the need for getting the threshold. There are our discoveries after practically researching: (1) In one main experimental area, the super high way, the rates of success in vehicle detection of the big car, small car and the whole vehicles in the experimental area are separately as 90%, 100% and 99.21%. After modified the rates of success are 88.89%, 99.13% and 98.39%. (2) In another main experimental area, the rapid high way, the rates of success in vehicle detection of the big car, small car and the whole vehicles in the experimental area are 100%. And the rates of success in identification are 100%, 96.25% and 96.30%. However, there’s one big vehicle passing by. Take it for reference. (3) In the secondary experimental area, the suburb road, the rates of success in vehicle detection of the small cars, motorcycles and the whole vehicles in the experimental area are separately as 100%, 93.75% and 94.48%. And the rates of success in identification are 96%, 100% and 96.92%. The accuracy of vehicle speed is 91.97%, no matter where the experiment is held. |  |  |  |  |