國立交通大學

交通運輸研究所

碩 士 論 文

高速公路瓶頸路段車流模式之研究

研 究 生：張瑞玲

指導教授：卓訓榮

中華民國八十六年六月

高速公路瓶頸路段車流模式之研究

The Traffic Flow Models of Freeway Bottlenecks

研 究 生：張瑞玲	Student：Ruey-Ling Chang

指導教授：卓訓榮	Advisor：Hsun-Jung Cho

國 立 交 通 大 學

交  通  運  輸  研  究  所

碩 士 論 文

A Thesis

Submitted to Institute of Traffic and Transportation

College of Management

National Chiao Tung University

in Partial Fulfillment of the Requirements

for the Degree of

Master of Engineering

in

Traffic and Transportation

June 1997

Hsinchu, Taiwan, Republic of China.

中華民國八十六年六月

高速公路瓶頸路段車流模式之研究

學生：張瑞玲	指導教授：卓訓榮

國立交通大學交通運輸研究所

摘  要

在正常車流狀態運行下，高速公路能提供用路人快速、便捷的高水準服務；然而一旦發生交通事故或進行道路的養護施工時，常必須封閉部份車道，造成道路容量的降低，因而影響到正常的車流，使得該路段造成擁擠、旅行時間的增長及其他財物損失。若能即時將此類擁擠區域、路段或旅行時間等路況資訊提供予管理者研擬交通管制計畫，或提供予用路人以誘導其改道行駛，應可減低該路段對車流的干擾及降低該路段的交通擁擠。

瓶頸路段車流特性的分析乃是對瓶頸路段進行交通管制計畫的基本工作；故本研之目的在建構高速公路瓶頸路段的車流行為模式，以作為將來分析瓶頸路段對上游車流延滯情形的基礎。

本研究首先考慮高速公路上一均質、無進出口匝道的二車道路段，其中有一車道的部份路段封閉，觀察未封閉車道的車流行為，以流量守恆律為基礎，構建該瓶頸路段的車流行為模式，再以特徵曲線法求解模式得車波軌跡函數及密度函數，並以數值模擬方法模擬二車道瓶頸路段車流模式，所得結果與預期路況密度相近。

接著，探討多車道瓶頸路段的車流行為，並分別以流量守恆律、構建多車道瓶頸路段一階車流模式及高階車流模式。

The Traffic Flow Models of Freeway Bottlenecks

Student：Ruey-Ling Chang	Advisor：Hsun-Jung Cho

Institute of Traffic and Transportation

National Chiao Tung University

ABSTRACT

Freeways provide exclusively for through movement of traffic at high speed under normal situation. However, whether the bottlenecks are caused by incidents or roadwork, may reduce the capacity of freeway and generate traffic congestion. Soon after an incident happened, if the speed information for the bottleneck could be given to the upstream drivers immediately, which can reduce the emission and fuel consumption, in addition to lowering traffic congestion and travel cost.

An analytical procedure to study the effect of freeway bottlenecks is the need for effective traffic control of the road section under construction which is one of the major tasks in the process of freeway construction and maintenance. The purpose of this research is to construct a traffic flow model to describe the characteristics of traffic flow at bottlenecks.

In this study, a special case of an uniform two-lane freeway with no entrances or exists, and with bottlenecks is discussed. The mathematical model of the traffic flow describing the dynamic evolution of traffic variables along the normal lane are developed by conservation law and the momentum equation. The models are solved by characteristic method and simulated by adaptive finite difference method. Then, the multilane freeway bottleneck models are extended. Finally, the existence and uniqueness properties of the solutions are also discussed.

誌    謝

本論文得以順利完成，承蒙恩師  卓訓榮教授兩年來學業上的悉心指導與協助，舉凡題目之訂定、觀念之啟迪、論文架構之匡正、乃至論文完稿之過程，均蒙恩師之耐心指引與逐字之斧正；在為學處世及待人接物方面亦多蒙恩師之教誨，使學生在治學方法及處世態度上受益良多，在此謹獻上最誠摯的感謝與敬意。

論文口試期間，承蒙交通部張次長家祝博士與台灣大學土木所張學孔博士撥冗細審與斧正，提供寶貴意見，使本論文更臻完備，謹表由衷之謝忱。論文進度審查與報告期間，承蒙所上吳水威老師與韓復華老師之細心審閱，張新立老師、許巧鶯老師及陳亭羽學姐於進度報告期間提供改進意見與建議，在此亦深表謝意。

受業期間，承蒙所上韓復華老師、張新立老師的關懷與指導，以及吳水威老師、許巧鶯老師、謝尚行老師、吳宗修老師、張隆憲老師、任維廉老師、高凱老師與清華大學王信華老師等諸位師長在學業上的悉心指導，於此一併致謝。

論文進行期間，感謝同窗好友的朝夕相伴，使生活增添了許多歡笑。特別感謝學長習平、志賢、柏丞、裕仁，學姐李霞、亭羽的照顧幫忙，同門仕京、志忠、姿婷於修業期間互相砥礪，志忠在程式撰寫過程的指導，在此一併致謝。

學生能繼續求學，完成研究所學業，全靠父母親的呵護，大姐、姊夫、芮榛、祺鈞、又升的鼓勵與支持，好友志忠在生活上的照顧，謝謝您們在我學習過程或是生活上所給予的支持與關懷，使我在學的這段日子裡，能安心地面對種種困難與挑戰，在此由衷感謝。最後，謹將這份成果獻給辛勤養育與栽培我的父母親！

張 瑞 玲  謹誌

中華民國八十六年六月

於風城交大

目    錄

頁次

中文摘要	...........................................................................................................	i

英文摘要	...........................................................................................................		ii

誌謝	...................................................................................................................		iii

目錄	...................................................................................................................		iv

表目錄	..............................................................................................................		vi

圖目錄	..............................................................................................................		vii

緒論	..................................................................................................	1

- 研究動機	............................................................................................	1

- 研究目的	............................................................................................	1

- 研究範圍	............................................................................................	2

- 研究方法	............................................................................................	2

- 研究內容與流程	................................................................................	2

- 文獻回顧	..................................................................................................	5

- 車流之基本特性	................................................................................	5

- 車流曲線模式的發展	.......................................................................	7

- 車流波動方程式	................................................................................	9

- LWR模式及其他後續相關研究	............................................	9

- 高階波動方程式	.......................................................................		12

- 瓶頸路段之相關研究	.......................................................................	14

- 三、車流流量守恆律與動量方程式	..............................................................	17

- 車流流量守恆律	................................................................................	17

單一車道、無進出口匝道路段	............................................	17

- 單一車道、有進出口匝道路段	............................................	18

- 多車道、無進出口匝道路段	..................................................	19

- 多車道、有進出口匝道路段	..................................................	21

- 車流動量方程式	................................................................................	23

- Payne的原始高階模式	...........................................................	23

- 多車道下的高階波動方程模型	...............................................	24

- 四、二車道瓶頸路段之車流模式	....................................................................	27

- 二車道瓶頸路段之車流概況	..............................................................	27

- 二車道瓶頸路段一階車流模式之構建	............................................	29

- 二車道瓶頸路段一階車流模式之求解	............................................		30

- 準線性一階偏微分方程式的求解	.........................................	31

- 模式求解	...................................................................................	32

- 二車道瓶頸路段一階車流模式之數值模擬	......................................	40

- 二車道瓶頸路段高階車流模式	........................................................	44

- 五、多車道瓶頸路段之車流模式	....................................................................	48

- 多車道瓶頸路段之車流概況	..............................................................	48

- 多車道瓶頸路段一階車流模式之構建	............................................	50

- 多車道瓶頸路段高階車流模式之構建	............................................	54

- 六、結論與建議	...............................................................................................	57

- 結論	.....................................................................................................	57

- 建議	.....................................................................................................	57

- 參考文獻	...........................................................................................................	59

表 目 錄

頁次

- 單一階段車流模式方程式	.................................................................	7

- 多階段車流模式方程式與分界點	.....................................................	8

- 圖 目 錄

- 頁次

- 研究架構	...........................................................................................	3

- 研究流程圖	.........................................................................................	4

- 於µ §時間內車流移動示意圖	...........................................................		6

- 流量（q）、速度（u）與密度（k）之基本關係圖	.................	6

- 瓶頸路段上游之流量小於瓶頸路段之容量	...................................	16

- 瓶頸路段上游之流量大於瓶頸路段之容量	...................................	16

- 車輛在單一車道且無進、出口匝道路段上的進出情形	.................	18

- 車輛在單一車道且有進、出口匝道路段上的進出情形	.................	19

- 二車道無進、出口匝道路段上的進出情形	...................................	21

- 二車道有進、出口匝道路段上的進出情形	...................................	22

- 兩車道道路環境示意圖	.....................................................................	26

- 瓶頸路段車流狀況示意圖	.................................................................	28

- 特徵曲線與初始值函數	....................................................................	31

- 二車道瓶頸路段一階車流模式之數值模擬-密度	..........................	42

- 二車道瓶頸路段一階車流模式之數值模擬-速度	..........................	43

- 二車道瓶頸路段高階車流模式之數值模擬	......................................	47

多車道瓶頸路段的車流示意圖	........................................................	49