國  立  交  通  大  學

交 通 運 輸 研 究 所

碩士論文

百貨公司廣場行人移動

自我組織現象之研究

Self-organization Phenomenon of Pedestrian Movement in Department Store Plaza

研 究 生：邱詩純

指導教授：馮正民 曾平毅

中 華 民 國 九 十 一 年 六 月

百貨公司廣場行人移動自我組織現象之研究

Self-organization Phenomenon of Pedestrian Movement in Department Store Plaza

研 究 生：邱詩純              Student : Shih-Chen Chiu

指導教授：馮正民              Advisor : Cheng-Min Feng                               曾平毅                      Pin-Yi Tseng

國立交通大學

交通運輸研究所

碩士論文

# A Thesis

## Submitted to Institute of Traffic and Transportation

College of Management

National Chiao Tung University

in Partial Fulfillment of the Requirement

for the Degree of

Master of Engineering

in

Traffic and Transportation

June 2002

Taipei, Taiwan, Republic of China

中華民國九十一年六月

百貨公司廣場行人移動自我組織現象之研究

研究生：邱詩純          指導教授：馮正民、曾平毅

國立交通大學交通運輸研究所

摘  要

過去行人相關研究多以聚集人群為主，侷限於描述規律性的移動行為，無法描述人群因個體間的互動所帶來的多變化結果，亦無法探討是否存在自我組織現象。有鑑於此，本研究藉由錄影調查台北車站新光三越百貨之行人廣場，探討該廣場的行人移動特性與現象，並從觀察中找到互動規則，以建構行為者基礎模型模擬現況，並探討其自我組織現象。經研究發現，該廣場之行人具有資訊不完全、有知性、密度適中的特性，行人之間以『吸引力』與『進入阻力』作為傳遞資訊，在不同時段觀察發現行人分佈均收斂成某一固定形態，另外，駐足人群的群落規模與數量的關係呈現冪次趨勢，為自我組織臨界的特徵現象。本研究進一步以NetLogo作為模擬軟體，構建模擬模型以分析行人之移動行為，其中環境狀態變數為『吸引力』與『進入阻力』，行人會選擇吸引力最大與進入阻力最小的方向。模擬模式經以廣場現況在低、中、高三種行人密度的駐足人群分佈驗證，顯示模擬模型具有良好的詮釋能力。本研究利用模擬模型分析行人密度與延遲時間的關係發現，穿越行人延遲時間會隨著密度的增加而增加，並行人移動系統會從秩序變成自我組織再轉變到混亂。本研究另分析廣場內設置咖啡座的影響，該應用以增加平均駐足吸引力與避免激增穿越行人延遲時間作考量原則，分析得咖啡座之適當設置位置及其對廣場行人移動之影響情形。由本研究之應用分析案例，顯示出該模擬模型可作為輔助廣場空間配置，分析廣場家具與行人移動互動影響之工具。

關鍵詞：百貨公司廣場、自我組織、行為者基礎模型、行人移動

Self-organization Phenomena of Pedestrian Movement in Department Store Plaza

Student : Shih-Chen Chiu               Advisors： Cheng-Min Feng

Pin-Yi Tseng

Institute of Traffic and Transportation

National Chiao-Tung University

Abstract

Most of pedestrian movement models were based on macroscopic behaviors of aggregative pedestrians and focused on regular movement behaviors. These models could not describe various interactions of individuals or analyze self-organization phenomena. The study starts with observing and investigating on pedestrian movements in Shin-Guang department store plaza. It is found that these pedestrians move under incomplete information, rational reaction and medium density situations. The major communication ways of transfer information are attraction and walk-ability. Pedestrians always cluster together with some patterns in different observation periods. It is found that the relationship of cluster scale and its frequency follows to Power Law, which is one of the evidences of self-organization phenomena. Based on the characteristics of pedestrian movement behaviors and interaction rules, a pedestrian agent-based model used by NetLogo simulation software is constructed. This model uses three modules to direct pedestrian movement that pedestrian takes the way of maximum attraction and minimum walk-ability, and can simulate pedestrian movements actually. From the empirical study on movement delay according to pedestrian density, it is discussed that the phenomena of pedestrian system changes from order to self-organization, and to chaos. The second empirical study simulates and discusses the influence of allocating a coffee shop in plaza, and gets the proper position. This pedestrian agent-based model can be a good tool to analyze the movements, interactions between layout of street furniture and pedestrian movements.

Keywords: department store plaza, self-organization, Agent-based model, pedestrian movement