Integrating Smartcards in to The Fare System in Buses

And Rapid Transportation Systems in Taipei

Metropolitan Area

Student: Li-Chu Lin          Advisor: Dr. Cheng-ch'uan Huang

Institute of Traffic and Transportation

National Chiao Tung University

ABSIRACT

The application of smartcards is increasing. In transportation, we use it in systems including railroads, buses, MRT, Taxis, parking, tolls, etc. To solve the problem of fare system in buses and MRT systems in Taipei effectively, we integrate smartcards into the fare system. Therefore, the goal in this stage is to reach the point where "one single cards can need to get around in Taipei".

The purpose of this study is to discuss the development and application in automatic electronic fare collection systems in MT systems all over the world, and examine the present fare system in buses and MRT systems in Taipei metropolitan area. Secondary, to analyze the effect of the fare integration system and the need of users, operators, and governmental managers. The establishment of appropriate type of organization is also discussed and evaluated. According to the result, the best alternative selling Center clearing center, multiple cards and selling units. According to the results of benefit/cost analysis, this alternative will provide IRR (internal rate of return) higher than the reasonable rate of return on investment used by the operators of buses and MRT systems. The present value of annual net benefit is positive which means this is a feasible solution.

Keyword:	smartcard, fare integration, benefit/cost analysis