中文摘要

本研究探討線性軸輻路網之接駁／轉運課題，首先利用遺傳演算法之編解碼技術，分別構建同時分組、逐步分組及種子分組三模式，俾進行路線分組，以決定路線間之接駁轉運關係，並與層級分組模式比較；其次，構建路網最佳化及排班最佳化模式，求解各路線分組之最佳路網型式及班距。目標函數係求解乘客候車成本（含起站、接駁站及轉運站）、乘客懲罰成本（含轉乘、接送）、業者成本（含營運、車隊），以及場站設置營運成本（含接駁站及轉運站）之總和最小化。

因路線分組模式本質上屬NP-hard問題，為瞭解各模式之尋優效度與效率，本研究以亂數產生之200個樣本進行測試，結果顯示種子分組之尋優效度最佳，但效率最差；逐步分組模式之尋優效度次佳，效率亦次佳。同時分組模式因染色體較長，尋優效度最差。以台汽公司60條國道直達路線為實例應用，發現種子分組尋優效果明顯優於其他模式，經接駁轉運設計後，形成1組直達集合（32條路線）、3組接駁集合（6條路線）及7組轉運集合（22條路線），總成本降低5.18%，顯示接駁轉運確能改善現行之直達服務方式。由敏感度分析知，車輛容量與乘客單位轉乘懲罰成本係影響接駁轉運成功與否之關鍵。因此，若能增加車輛容量，以提高接駁／轉運可能性，並妥為設計接駁站及轉運站之轉乘方式，以減少乘客之不便，將更能加強接駁／轉運系統之功效。

關鍵詞：線性軸輻路網、接駁、轉運、遺傳演算法

ABSTRACT

This study discusses the development and application of an integrated feeder/transfer system model in a linear hub-and-spoke network. At first, genetic algorithms are employed to determine the feeder/transfer relationship among routes by clustering the routes into different partition sets. Three models- simultaneous clustering method (SICM), stepwise clustering method (STCM) and cluster seed points method (CSPM)- basing on different coding/decoding techniques are developed and compared with a conventional statistics clustering model- the agglomerative hierarchical clustering method. Then, an algorithm integrating feeder/transfer location selecting, routing, and scheduling is proposed to solve the optimal network and headway for each partition set. The objective function is to minimize the total sum of passenger waiting cost (at origin ends, feeder stops and transfer stations), passenger inconvenience penalty cost (including change vehicles, pick-up and delivery), operator cost (including operation cost and fleet cost), set-up cost (including feeder stops and transfer stations).

Because of the NP-hard character of the routes clustering problem, it is essential to analyze the effectiveness and efficiency of these methods. A testing experiment of 200 randomly generated objects is designed to perform the comparison. The testing result shows that CSPM is the most effective but least efficient method, STCM is second most effective and efficient, SICM is least effective because of its long chromosome. A case study of 60 direct service routes operated by Taiwan Motor Transport Company shows an optimal condition can be reached by clustering these 60 direct-service routes into three feeder sets with 6 routes, seven transfer sets with 22 routes and one direct service with 32 routes. As a result, the total cost is lowered by 5.18%, illustrating that feeder/transfer systems can surely improve a direct service system. The sensitivity analysis indicates that bus seat capacity and change vehicle penalty cost are two key factors in designing a feeder/transfer system. It implies that expanding the bus seat capacity to increase the possibility of feeder/transfer passengers boarding and well designing the transfer arrangement to reduce the inconvenience in changing vehicles would further enhance the performance of an inter-city bus feeder/transfer system.

Keywords: Linear hub-and-spoke network, feeder, transfer, genetic algorithms

目錄

中文摘要…………………………………………………………...…Ⅰ

英文摘要……………………………………………….………...…….Ⅱ

目錄……………………………………………………………...…….Ⅲ

表目錄……………………………………………………………...…Ⅴ

圖目錄……………………………………………………………...…Ⅶ

第一章  緒論……………………………………………………...…….1

1.1  研究背景與動機……………………………………………...1

1.2  研究目的……………………………………………………...2

1.3  研究對象與範圍………………………..…………………….2

1.4  研究方法與架構………………………..…………………….3

1.5  研究內容與流程………………………………..…………….4

第二章  相關文獻評析……………………..…………………...……...9

2.1  接駁／轉運相關文獻………..…………………………..…...9

2.2  群落分析相關文獻………………………………………….19

第三章  基本假設與模式架構……………………………………..…29

3.1  問題分析………………………………………….….……...29

3.2  基本假設………………………………………………….....29

3.3  目標函數……………………….………………………...….31

3.4  模式架構………………………………………………….…34

3.5  符號及名詞定義………………………………………….…35

第四章  路線分組最佳化模式……………………………………..…39

4.1  問題界定………………………………………….….……...39

4.2  數學模式………………………………………………….....40

4.3  遺傳演算法…………………….………………………...….41

4.4  問題模化………………………………………………….…44

4.5  簡例驗證一：群落分析…………………………….……..…51

4.6  簡例驗證二：P-median………………………………………56

4.7  小結………………………………………………………….62

第五章  路網最佳化模式…………………………………………63

5.1  路網特性…………………………………………………….63

5.2  路網型式……….…………………………..………………..65

5.3  路網尋優演算法……….………………………………….75

第六章  排班最佳化模式……………………………………..……77

6.1  問題界定……………………….………………..………..…77

6.2  模式推導……………………….…………………….……78

6.3  簡例說明……….……………………………………………81

第七章  簡例驗證與實例應用…….…………………..……………91

7.1  簡例驗證………………………………………….…………91

7.2  實例應用…………………………………………………….94

7.3  討論…………………….………………………………….113

第八章  結論與建議…………………….…………………………117

8.1  結論………………………………………………….……117

8.2  建議……………………………………………….………119

參考文獻…………………………….……………………….………123

附錄一	…………………………………………………….………..129

作者簡歷……………………………………………………………..137

表目錄

表2.1  接駁／轉運相關文獻彙整表………………………………….16

表2.2  群落分析相關文獻彙整表…………………………………….26

表3.1  基本假設之合理性分析……………………………………….30

表3.2  符號彙整表…………………………………………………….36

表4.1  分組集合關係矩陣………………………………………...…..45

表4.2  基因與指派集合之關係……………………………………….46

表4.3  分組集合關係矩陣………………………………………….....46

表4.4  四種模式在不同問題規模下之尋優效度與效率……….……53

表4.5  四種混合模式在不同問題規模下之尋優效度與效率…….…55

表5.1  轉運路網與接駁路網之差異分析………………………….…65

表5.2  轉運集合之路網型式及其特性……………………………….73

表6.1  接駁路網各項成本計算公式彙整表………………………….78

表6.2  不同排班策略下接駁路網之最佳班距……………………….81

表6.3  簡例之6條路線基本資料…………………….….…………....82

表6.4  直達路線改為接駁路網之成本變化………………………….83

表6.5  直達路線改為一對多單轉運路網之成本變化…………….....84

表6.6  直達路線改為多對一單轉運路網之成本變化…………….....85

表6.7  直達路線改為多對多單轉運路網之成本變化………….……86

表6.8  直達路線改為混合型轉運路網之成本變化……………….…88

表7.1  接駁／轉運尋優結果（簡例）…………………………………93

表7.2  台汽公司60條國道客運路線資料……………………………95

表 7.3  台汽60條國道客運路線營運現況與排班最佳化之班距與各項成本……………………………….…………………………97

表7.4  種子分組模式在突變率0.03下之尋優結果………..……..102

表7.5  各路線分組集合之路網特性………………………………103

表A.1中山高交流道及休息站之編號與里程………………………130

表A.2 台汽公司國道客運路線資料…………………………………131

表A.3 台汽高速公路客運路線車站至交流道行駛時間分析………134

圖目錄

圖1.1  研究架構圖……………….…………………………………….4

圖1.2  研究流程圖………………..…..………………………………..7

圖2.1  同一轉運站但不同轉運路線組合之釋例………………….…18

圖3.1  模式架構圖……………….……………………………………35

圖4.1  單點交配示意圖……………………….………………………42

圖4.2  雙點交配示意圖……………………………………………….43

圖4.3  均勻交配示意圖……………………………………………….43

圖4.4  基因突變與移轉突變示意圖………………………………….44

圖4.5  逐步分組模式示意圖………………………………………….47

圖4.6  分組種子模式之尋優架構示意圖…………………………….50

圖4.7  200個2維度樣本之分佈情形………………………………..51

圖4.8  四種模式於各測試群體規模下之尋優結果(小規模問題)…..53

圖4.9  種子分組模式之50個樣本分組結果………………………..54

圖4.10  層級分組、逐步分組及種子分組於各測試群體之尋優結果

(中大規模問題)……….………………………………..…..55

圖4.11  α=0.5單位／座之設計結果………………..……….….…...57

圖4.12  α=1單位／座之設計結果……………………….……..…...58

圖4.13  α=2單位／座之設計結果………………………………..…58

圖4.14  α=3單位／座之設計結果……………………………….….59

圖4.15  α=4單位／座之設計結果…………………………………..59

圖4.16  α=5單位／座之設計結果……………………………….….60

圖4.17  α=10單位／座之設計結果…………………………………60

圖4.18  α=15單位／座之設計結果…………………………………61

圖4.19  公共設施數、總距離成本與α值之關係……………….……61

圖5.1  直達、接駁及轉運路網示意圖……………………………….63

圖5.2  一對一單轉運路網示意圖（未繪交流道位置）……………….66

圖5.3  一對多單轉運路網示意圖（未繪交流道位置）……………….66

圖5.4  多對多單轉運路網示意圖（未繪交流道位置）……………….67

圖5.5  一對一雙轉運路網示意圖（未繪交流道位置）……………….68

圖5.6  一對多雙轉運路網示意圖（未繪交流道位置）……………….68

圖5.7  多對多雙轉運路網示意圖（未繪交流道位置）……………….69

圖5.8  單面對雙面混合型路網示意圖（未繪交流道位置）………….70

圖5.9  雙面對雙面混合型路網示意圖（未繪交流道位置）………….71

圖5.10  一對多複雜雙轉運路網示意圖（未繪交流道位置）……...…72

圖5.11  多對多複雜雙轉運路網示意圖(未繪交流道)……………....72

圖5.12  一對多複雜混合型路網示意圖（未繪交流道位置）………...72

圖5.13  多對多複雜混合型路網示意圖（未繪交流道位置）……….73

圖6.1  共同班距策略示意圖（以接駁路網為例）…………………….77

圖6.2  整數班距策略示意圖（以接駁路網為例）…………………….77

圖6.3  簡例之高速公路幾何佈設示意圖………………..…………...82

圖6.4  直達路線改為接駁路網之路網示意圖…………..…………...83

圖6.5  直達路線改為一對多單轉運路網之示意圖……..…………...84

圖6.6  直達路線改為多對一單轉運路網之示意圖……..………...…85

圖6.7  直達路線改為多對多單轉運路網之示意圖……..…………...86

圖6.8  直達路線改為混合型轉運路網之示意圖………..…………...87

圖6.9  不同乘客需求下各路網之總成本減少率…………………….89

圖7.1  簡例之各模式尋優過程……………………………………….92

圖7.2  λ、ρ、φ、ψ參數變化對總成本減少率之影響……………...93

圖7.3  各模式在不同突變率下之尋優結果………………………100

圖7.4  同時分組、逐步分組及種子分組之尋優過程……………….101

圖7.5  不同路線規模下各模式之尋優效果………………………107

圖7.6  ρ、φ、ψ參數變化對總成本減少率之影響………………108

圖7.7  α、β、γ參數變化對總成本減少率之影響………………108

圖7.8  δ、η、θ參數變化對總成本減少率之影響………………109

圖7.9  V、v、C參數變化對總成本減少率之影響…………………109

圖7.10 ρ參數變化對直達、接駁及轉運路線比率變化之影響…….110

圖7.11 α參數變化對直達、接駁及轉運路線比率變化之影響…….111

圖7.12 γ參數變化對直達、接駁及轉運路線比率變化之影響…….111

圖7.13 δ參數變化對直達、接駁及轉運路線比率變化之影響…….112

圖7.14 η參數變化對直達、接駁及轉運路線比率變化之影響…….112

圖7.15 C參數變化對直達、接駁及轉運路線比率變化之影響……113

圖A.1 台汽國光號及中興號歷年運輸效率變化情形…..…..………130