第八章  結論與建議

8.1  結論

8.1.1 主要貢獻

1.理論面

本研究之主要貢獻為建構一線性軸輻路網整合決策模式，整合路線分組、路網規劃及排班設計三層面問題，以克服以往接駁轉運之相關文獻缺乏整合決策模式、未能確立路線間接駁轉運關係以及排班設計過於複雜等問題，並解決集合分割相關文獻之分群組數無法同時求解以及模式應用彈性有限等問題。此模式具有獨立性及整合性兩項特性，說明如下：

(1)獨立性

本整合決策模式包括路線分組最佳化、路網最佳化及排班最佳化三個子模式。各模式均具獨立性，可分別求解其特定問題。例如路線分組最佳化模式可應用於求解一般群落分析問題，本研究以亂數產生之200組二維樣本加以測試，結果驗證其尋優效度甚佳。此外，排班最佳化模式可針對直達路線進行排班設計，本研究將台汽60條路線進行排班最佳化設計，使總成本降低達41.62%。

(2)整合性

本整合決策模式之各子模式均以同一目標函數最小化為規劃方向，該目標函數包括乘客候車成本（含起站、接駁站及轉運站）、乘客懲罰成本（含轉乘、接送）、業者成本（含營運、車隊），以及場站設置營運成本（含接駁站及轉運站）。目標函數值必須俟路線分組、路網型式及協調班距三者均告確定後，方可計算，故此三個子模式係整合一體而非程序性分別求解。

2.實務面

本研究以台汽之國道路線為例，先針對其直達路線進行排班設計，使總成本降低達41.62%，再進一步研提實施接駁／轉運之具體計畫（接駁站及轉運站設置區位、各路線之營運方式及其班距）與預估效益（總成本可再降低5.18%），足供主管機關及業者推動相關計畫之參考。

8.1.2  主要發現

本研究之主要發現如下：

1.因路線分組最佳化模式本質上屬群落分析問題，本研究利用遺傳演算法，分別利用不同之編解碼方式，構建同時分組、逐步分組及種子分組等三模式，並與傳統統計之層級分組模式比較。經以亂數產生之200組二維樣本之測試結果顯示，種子分組模式係尋優效度最佳之模式。

2.針對路網最佳化模式，本研究依據接駁及轉運路網特性，研擬接駁路網、一對多單轉運、多對一單轉運、多對多單轉運、多對多雙轉運、單面對雙面混合型轉運及雙面對雙面混合型轉運等七種線性軸輻路網，並針對各路網之特性，設計轉運站之最佳區位選擇方式。最後，研擬一路網最佳化尋優演算法，俾針對各路線分組，規劃最佳路網型式。

3.至於排班最佳化模式，本研究則利用數學解析法推導各路網之幹線最佳班距及支線之班距整數倍值之計算公式，俾針對一特定路網設計最佳幹、支線班距。

4.以6條路線之簡例驗證中，同時分組及逐步分組模式均能求得整體最佳解，但種子分組及層級分組模式則因受組別合併先後或路線指派順序之影響，無法求得整體最佳解，惟求解所得之總成本仍低於原直達成本。

5.挑選台汽公司60條國道客運路線，經以排班最佳化模式進行其直達路線之排班設計，則總成本可降低達41.62 %。且經由現行班距與最佳班距之比較知，目前台汽各路線之班次明顯偏多，導致營運成本偏高。

6.以經排班最佳化設計後之台汽公司60條國道路線為實例，進一步分析接駁／轉運之績效，結果顯示種子分組模式之尋優效果最佳，經接駁轉運設計後，共形成1組直達集合（32條路線）、3組接駁集合（6條路線）及7組轉運集合（22條路線），總成本降低5.18%，顯示轉運接駁之營運方式確能改善現行直達之經營型態。至於此一接駁／轉運系統中，接駁站共有三座，分別設置在中壢、頭份及大雅等三處交流道附近，轉運站有四座，分別設置在中壢、泰安、西螺及仁德等四處休息站或服務區，以便於乘客及公車進行接送或轉乘。

7.由敏感度分析知，車輛容量與乘客單位轉乘懲罰成本係影響接駁轉運成功與否之關鍵。因此，若能提高車輛容量，並妥為設計接駁、轉運站之轉乘方式，以減少乘客轉乘之不便，將更能發揮接駁轉運系統之效果。

8.本研究之實例應用，雖僅以台汽公司之營運路線為例，惟亦可適用其他客運業者。甚至可將所有國道路線（或調查預測之乘客旅次起迄需求分佈情形）一併加以考量，俾使接駁站及轉運站之設置區位，均可適用所有客運業者。

9.本研究係假設各路線之起、迄站固定不變，故無法分析因接駁／轉運之設計後，路線新增之可行性（例如，原偏遠虧損路線可經由接駁轉運，減少空駛延座公里，提高路線獲利能力及業者增闢意願）。因此，在設計上可將此種路線設計為虛擬路線，與其他路線一併進行接駁／轉運設計，即可分析其效益。

8.2  建議

本研究之建議及後續研究方向如下：

1.本研究所建構之路線分組最佳化模式，雖屬整合模式之一環，用於路線分組，但仍可應用於一般群落分析問題，顯示遺傳演算法之應用彈性。惟同時分組、逐步分組及種子分組均尚存有缺失，有待後續研究加以改善，分別說明如下：

(1)同時分組模式為能一次將所有路線加以分組，其編解碼方式將使染色體過長，故僅能適合規模較小之問題。以60條路線之規模而言，其可行解個數達2300，致尋優效果較差。後續研究若能結合其它啟發式尋優理論，如模擬退火，應可進一步改善尋優結果，或進一步研究如何降低編解碼之重覆，以儘量減少染色體長度。

(2)逐步分組模式之染色體長度雖較同時分組模式短，但因其程序性進行二元分組之概念，導致一旦分組確定後，屬於不同分組之路線，即使非常適合互為接駁或轉運，但於後續階段均不可能再重新合併為一組。因此，為改善此一問題，後續研究可考慮在所有路線分組均探測到底後，再設計一分組集合間路線互換之演算法，以作局部調整之用。

(3)種子分組模式雖染色體長度亦較同時分組短，程序上也較逐步分組單純，但每次進行非種子路線之分組指派時，均必須重覆計算目標值 m×(N-m)次（m為種子路線數），並加以比較，以決定目標值最佳之指派組別，形同蒐尋一可行解，導致其尋優效率最差。因此，後續研究可針對問題之特性，設計合理且容易計算之指派準則，將可大幅改善種子分組之尋優效率。

2.本研究簡例驗證及實例應用，所設定之參數值，未來實際運用時，應再予審視。其中，部分參數仍需進一步之調查分析（如乘客起站候車時間係數、乘客候車單位成本、乘客接送或轉乘之懲罰成本等）方可據以估算。

3.本研究規劃轉運站及接駁站之區位時，並未針對現有休息站及交流道之幾何條件與可行性進行評估，未來經評選為接駁站或轉運站之交流道或休息站，有必要進一步檢視其對鄰近交通之衝擊，研提必要之交通衝擊分析。甚至於規劃設計前，先行評估各交流道及休息站設置接駁站或轉運站之可行性，俾將不可行者先予以刪除。

4.本研究為簡化問題，假設行駛速率為確定性，乘客需求無尖離峰現象且固定，後續研究可針對此假設加以解除，以更符實際狀況。

5.本研究進行公車容量之敏感度分析時，係假設公車容量變化並不改變其車輛成本，此一假設或許較適合於底盤規格不變，僅車廂內裝稍作改變（例如，容量減少時，係將四排座位改為三排或二排；容量增加時，係將二排座位改為三排或四排），致車輛成本變化不大之情況。未來宜進一步探析公車容量與成本間之關係，或依據張學孔[1991]所設定之公車容量與成本特性之關係，再進行公車容量之敏感度分析，俾據以決定最適公車容量。

6.本研究僅分析到達率及時段T為固定之狀況，若將一日分成數個時段T，本模式仍可分別求解各時段之最佳班距，惟因一日各班距間將涉及複雜之車輛及人員調度排班問題，本研究並未加以探討，建議納入後續研究課題。

7.本研究進行接駁／轉運系統之規劃設計係以特定客運公司之營運路線為基礎，未來在接駁路網之設計上可由不同客運公司以聯盟方式營運，其幹線係由城際客運公司營運，支線則由地區客運公司營運。

8.由於本研究所建立之線性軸輻路網之接駁／轉運整合決策模式，僅針對單一高、快速公路進行規劃。惟台灣西部運輸走廊隨著中南二高、十二條東西向快速道路及西部濱海快速道路之完工通車，逐漸形成高速公路網。因此，為能規劃整體高速公路網之接駁／轉運系統，後續研究可修改本研究之路網最佳化模式，建構更為複雜之接駁／轉運路網型式及其區位尋優方式。另因非線性軸輻路網之接駁／轉運系統，其路線行駛里程及乘客搭車里程可能與原直達路線不同，故亦必須於目標函數中加以反映。

參考文獻

一、中文部份

吳清醮[1987]，「大眾運輸準確轉車系統車輛排班之研究」，成功大學交通管理科學研究所，碩士論文。

張學孔[1991]，「最佳公車容量與成本特性之分析」，運輸計劃季刊，第二十卷第四期，頁373~405。

朱珮芸[1994]，「公車與捷運的協調發車時間」，台灣大學土木工程學研究所，碩士論文。

黃泰林[1994]，「構建智慧型適應性網路號誌控制模式之研究」，博士論文，成大交管所。

台灣省政府交通處[1996]，「高速公路交流道附近設置轉運站可行性研究」，委託亞聯工程顧問股份有限公司辦理。

王慶瑞[1996]，運輸系統規劃，亞聯工程顧問公司。

曾國雄、邱裕鈞、許書耕[1997]，「主線欄柵式收費站最佳區位遺傳演算尋優法與逐步尋優法之比較分析」，中國土木水利工程學刊，第九卷第一期，頁171-178。

許書耕、陳茂南、邱裕鈞[1997]，「高速公路客運轉運接駁系統運轉規劃」，運輸計劃季刊，第二十卷第三期，頁315-338。

林祥生[1997]，「管制環境下城際國道客運服務策略之最佳化分析」，交通大學交通運輸研究所，博士論文。

藍武王、林祥生[1997a]，「均質環境下城際國道客運服務策略之最佳化分析」，運輸學刊，第十卷第三期，頁39-78。

林祥生、藍武王[1997b]，「異質環境下城際國道客運服務策略之最佳化分析」，運輸學刊，第十卷第四期，頁21-58。

李克聰[1995]，「大眾運輸接駁系統最佳化研究」，國科會計畫。

藍武王、邱裕鈞[1999]，「線性軸輻路網接駁/轉運區位、路線與排班之規劃─遺傳演算法之應用」，將於運輸計劃季刊刊出。

二、英文部份

Abkowitz, M. D. [1987], “Operational feasibility of timed transfer in transit systems,” Journal of Transportation Engineering, 113, pp.68-177.

Abramowitz, M., and Stegun, I. A. [1964], eds., Handbook of Mathematical Functions, U.S. Department if Commerce, National Bureau of Standards Applied Mathematical Series. 55, 1964.

Anderberg, M. R. [1973], Cluster Analysis for Applications, New York: Academic Press.

Andreasson, I. [1977], “Volvo approaches to computer-aided transportation planning,” Transportation Research Record, 657, pp.9-14.

Ali, A.I. and Thiagarajan, H. [1989], “A network relaxation based enumeration algorithm for set partitioning,” European Journal of Operational Research, 38, pp.76-85.

Arisawa, S. and Elmaghraby, S. E. [1977], “The hub and wheel scheduling problems: I. the hub scheduling problem: the myopic case,” Transportation Science, 11, pp.124-146.

Assad, A. A. [1980], “Modelling of rail networks: toward a routing/makeup model. Transportation Research, 14B, pp.101-114.

Aykin, T. [1995], ”Networking policies for hub-and-spoke systems with application to the air transportation system,” Transportation Science, 29, pp.201-221.

Aytug, H., Koehler, G. and Snowdon, J. L. [1994], “Genetic learning of dynamic scheduling within a simulation environment,” Computers and Operations Research, 21, pp.909-925.

Baker, J.J., J. Calkin and Sylvester, S. [1988], ”Multi-centered time transfer system for capital metro, Austin, Texas,” Transportation Research Record, 1202, pp.22-28.

Bookbinder, J. H. and Desilets, A. [1992], ”Transfer optimization in a transit network,” Transportation Science, 26, pp.106-118.

Brucker, P. [1978], ”On the complexity of clustering problems,” in:Beckmenn, M. and Kunzi, H.P.(eds.), Optimization and Operations Research. Lecture Notes in Economics and Mathematical Systems 157, Springer-Berlin, pp.45-54.

Cattrysse, D. G. and Wassenhove L.N.V. [1994], ”A set partitioning heuristic for the generalized assignment problem,” European Journal of Operational Research, 72, pp.167-174.

Chang S.K. [1990], Analytic Optimization of Bus Systems in Heterogeneous Environments, Ph.D. Dissertation, the University of Maryland.

Chang S.K. and P.M. Schonfeld[1991a], “Multiple period optimization of bus transit system,” Transportation Research, Vol. 25B, No.6, pp453-478.

Chang S.K. and P.M. Schonfeld[1991b], “Optimization models for comparing conventional and subcription bus feeder services,” Transportation Science, Vol. 25, No.4, pp281-298.

Conway, D. G. and Venkataramanan, M. A. [1994], “Genetic search and dynamic facility layout problem,” Computers and Operations Research, 21, pp.955-960.

Dobson, G. and Lederer, P. J. [1993], “Airline scheduling and routing in a hub-and-spoke system,” Transportation Science, 27, pp.281-297.

El-Darzi, E. [1988], Methods for Solving the Set Covering and Set Partitioning Problems Using Graph Theoretic (Relaxation) Algorithm, Ph.D. Dissertation, Brunel University.

El-Darzi, E. and Mitra G. [1995], “Graph theoretic relaxations of set covering and set paritioning problems,” European Journal of Operational Research, 87, pp.109-121.

Etcheberry, J. [1977], ”The set covering problem: a new implicit enumeration algorithm,” Operations Research, 25, pp.760-772.

Fisher, M.L., Jaikumar, R. and Van Wassenhove, L.N. [1986], ” A multiplier adiustment method for the generalized assignment problem,” Management Science, 32, 9, pp.1095-1103.

Garey, M.R., Johnson, D.S. and Stockmeyer, L. [1976], “Some simplified NP-complete graph problems,” Theoretic Computer Science, 1, pp.237-267.

Goldberg, D. [1989], Genetic Algorithm in Search, Optimization and Machine Learning, Addison Wesley.

Hall, R. W. [1985], “Vehicle scheduling at a transportation terminal with random delay en route,” Transportation Science, 19, pp.308-320.

Hagen, L. and Kahng, A. [1991], ”Fast spectral methods for ratios cut partitioning and clustering. Proceedings IEEE International Conference on Computer Aided Design, pp.10-13.

Hagen, L. and Kahng, A. [1992], ”New spectral methods for ratios cut partitioning and clustering. IEEE Trans-CAD, 11, 9, pp.1074-1085.

Hansen, P. and Delattre, M.[1978], ”Complete-link cluster analysis by graph coloring,” Journal of the American Statistical Association, 73, pp.297-403.

Hansen, P. and Jaumard, B. [1987], “Minimum sum of diameters clustering,” Journal of Classification, 4, pp.215-226.

Holland, J. H. [1975], Adaptation in Nature and Artificial Systems, University of Michigan press, Ann Arbor.

Jensen, R. E. [1969], “A dynamic programming algorithm for cluster analysis,” Operations Research, 12, pp.1034-1057.

Johnson, D.S., Aragon, L.A. McGeoch, L.A. and Schevon, C. [1989], ” Optimization by simulated annealing: an experimental evaluation; Part I. Graph partitioning,” Operations Research, 37, 6, pp.865-892.

Johnson, R.A. and Wichern D. W. [1988], Applied Multivariate Statistical Analysis, 2nd ed., Prentice-Hall Inc.

Karabakal, N., Bean, J.C. and Lohmann, J.R. [1992], A Steepest Descent Multiplier Adjustment Method for Generalized Assignment Problem. Report 91-11, University of Michigan, Ann Arbor, MI.

Kernighan, B.W. and Lin, S. [1970], “An efficient heuristic procedure for partitioning graphs, Journal of Bell System Technology, 49, pp.291-307.

Klastorin, T.D. [1985], “The P-median problem for cluster analysis: a comparative test using the mixture model approach,” Management Science, 31, 1, pp.84-95.

Kohonen, T. [1989], Self-Organization and Associative Memory (3rd ed.), Berlin: Springer-Verlag.

Lee, K. K. T. and Schonfeld, P. M. [1991], “Optimal slack time for timed transfer at a transit terminal,” Journal of Advanced Transportation, 25, pp.281-308.

Lin, F. T., Kao C. Y. and Hsu, C. C. [1993], “Applying the genetic approach to simulated annealing in solving some NP-hard problems,” IEEE Transactions on Systems, Man and Cybernetics, 23, pp.1752-1767.

MacQueen, J. [1967], ”Some methods for classification and analysis of multivariate observations,” Proceedings of Fifth Berkeley Symposium on Mathematical Statistics and Probability, 1, pp.281-297.

Martello, S. and Toth, P. [1981], “An algorithm for the generalized assignment problem,” Operational Research’81, J.P. Brams(Ed.), North-Holland, New York.

Michalewicz, Z. [1992], Genetic Algorithms + Data Structures = Evolution Programs, 2nd, Springer-Verlag.

Moshe, E. C., Tovey, C. A. and Ammons, J. C. [1996], “Circuit partitioning via set partitioning and column generation,” Operations Research, 44, 1, pp. 65-76.

Mulvey J. M. and Crowder H. P. [1979],” Cluster analysis: An application of Lagrangian relaxation,” Management Science, 25, pp.329-340.

Nawijn, W.M. [1988], ”Optimizing the performance of a blood analyser: application of the set partitioning problem,” European Journal of Operational Research, 36, pp.167-173.

Nemhauster, G.L. and Weber, G.M. [1979], ”Optimal set partitioning, matching and Lagrangian duality,” Naval Research Logistics Quarterly, 26, pp.553-563.

Nordstrom, A. L. and Tufekci, S. [1994], “A genetic algorithm for the talent scheduling problem,” Computers and Operations Research, 21, pp.941-954.

Pirkul, H. and Rolland E. [1994], ”New heuristic solution procedures for the uniform graph partitioning problem: extensions and evaluation,” Computers and Operations Research, 21, pp.895-907.

Pinte`r, J. and Pesti, G. [1991], “Set partition by globally optimized cluster seed points,” European Journal of Operational Research, 51, pp.127-135.

Rapp M. H. and Gehner, C. D. [1976], ”Transfer optimization in an interactive graphic system for transit planning,” Transportation Research Record, 619, pp.22-29.

Reynolds, M. M. and Hixson, C. D. [1992], ”Transit vehicle meets system: a method for measuring transfer time between transit routes,” Transportation Research Record, 1349, pp.35-41.

Salzborn, F. J. M. [1980], ”Scheduling bus system with interchange,” Transportation Science, 14, pp.211-231.

Shih, M. C. [1994], A Design Methodology for Bus Transit Route Networks with Coordinated Operations, Ph.D. Dissertation, The University of Texas at Austin.

Savelsbergh M. [1997], “A branch-and-price algorithm for the generalized assignment problem,” Operations Research, 45, pp. 831-841.

Ting, C. J. [1997], Transfer Coordination in Transit Networks, Ph.D. Dissertation, The University of Maryland at College Park.

Trick, M. A. [1992], “A linear relaxation heuristic for the generalized assignment problem,” Naval Research Logistic, 39, pp.137-152.

van Nes, R., Hamerslag R. and Immer, B.H. [1988], “The design of public transport networks,” Transportation Research Record, 1202, pp.74-83.

Vuchic, V. R., Clarke, J. and Molinero, A. [1983], Timed Transfer System Planning, Design and Operation, US. Department of Transportation, DOT-I-83-28.

Ward, J.H. [1963], “Hierarchical grouping to optimize an objective function,” Journal of the American Statistical Association, 58, pp.236-244.

Welch, J. W. [1983], “Algorithmic complexity: three NP-hard problems in computational statistics,” Journal of Statistical Computation and Simulation, 15, pp.17-25.

附錄一　台汽高速公路客運路線經營現況分析

台汽高速公路客運路線包括行駛中山高及北二高之路線，惟本研究以中山高路線為主。以下先概述中山高速公路之幾何現況，再分析台汽中山高路線之營運狀況。

A.1  中山高速公路幾何現況

中山高速公路大致係位於西部走廊之中央，與台1及台3省道平行，主線自基隆至高雄小港，全長約 373公里，另有機場支線（註：中正國際機場聯絡道路）長 8.5公里。其間主要設施包括：

1.交流道：包含基隆、八堵等43交流道，扣除2座系統交流道，一般交流道共計41處。

2.服務區：包含湖口、泰安及新營等 3處服務區。

3.休息站：包含中壢、西螺及仁德等 3處休息站。

4.收費站：包括汐止、泰山、楊梅、造橋、后里、員林、斗南、新營、新市、岡山等10處收費站。

5.車道數：台北交流道至林口交流道路段，全長15公里，佈設 8車道；圓山交流道至台北交流道、林口交流道至機場系統交通道，以及楠梓交流道至高雄端等三路段，全長30公里，佈設6 車道，其餘路段均佈設 4車道。

6.坡  度：全線除五股交流道至桃園交流道、新竹交流道至頭份交流道、苗栗交流道至豐原交流道等路段之最大縱坡度大於4%外，其餘路段則皆低於4%。

中山高沿線交流道、服務區及休息站等設施之編號（本研究自行編定）及里程如表A.1所示。

表A.1  中山高交流道及休息站之編號與里程

| 編號 | 交流道 | 休息站 服務區 | 里程 (公里) | 編號 | 交流道 | 休息站 服務區 | 里程(公里) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 基隆端 |  | 0 | T3 | ─ | 泰安 | 158 |
| 2 | 基隆 |  | 1 | 23 | 豐原 |  | 168 |
| 3 | 八堵 |  | 2 | 24 | 大雅 |  | 174 |
| 4 | 五堵 |  | 6 | 25 | 台中 |  | 179 |
| 5 | 汐止 |  | 10 | 26 | 王田 |  | 189 |
| 6 | 內湖 |  | 17 | 27 | 彰化 |  | 198 |
| 7 | 圓山 |  | 23 | 28 | 員林 |  | 211 |
| 8 | 台北 |  | 25 | T4 | ─ | 西螺 | 229 |
| 9 | 三重 |  | 27 | 29 | 西螺 |  | 230 |
| 10 | 五股 |  | 33 | 30 | 斗南 |  | 240 |
| 11 | 林口 |  | 41 | 31 | 嘉義 |  | 264 |
| 12 | 桃園 |  | 49 | 32 | 水上 |  | 270 |
| T1 | ─ | 中壢 | 55 | T5 | ─ | 新營 | 284 |
| 13 | 機場 |  | 52 | 33 | 新營 |  | 288 |
| 14 | 內壢 |  | 57 | 34 | 麻豆 |  | 304 |
| 15 | 中壢 |  | 62 | 35 | 永康 |  | 320 |
| 16 | 幼獅 |  | 67 | 36 | 台南 |  | 327 |
| 17 | 楊梅 |  | 69 | T6 | ─ | 仁德 | 335 |
| 18 | 湖口 |  | 83 | 37 | 路竹 |  | 338 |
| T2 | ─ | 湖口 | 86 | 38 | 岡山 |  | 349 |
| 19 | 新竹 |  | 95 | 39 | 楠梓 |  | 356 |
| 20 | 頭份 |  | 110 | 40 | 高雄 |  | 367 |
| 21 | 苗栗 |  | 132 | 41 | 高雄端 |  | 373 |
| 22 | 三義 |  | 150 | ─ | ─ | ─ | ─ |

A.2 台汽國道路線營運狀況

圖A.1為台汽國光號及中興號歷年之運輸效率（延人公里／延座公里）變化情形。由圖知，國光號之運輸效率雖較中與號為高，但兩者均自民國70年度以後，逐年遞減，至民國80至82年間，運輸效率更低於50%，顯示台汽實施接駁／轉運措施之必要性日益增高。

圖A.1  台汽國光號及中興號歷年運輸效率變化情形

以民國87年度而言，台汽營運路線計有177條，其中，行駛一般公路者計有96條，行駛高速公路者計有81條，營運概況如表A.2所示。由表知，81條路線中僅12條之運輸效率高於50%，比例尚不及15%，顯示台汽現行點到點直達之營運方式，雖可使乘客迅速地到達目的地，但因各路線獨立運作，無法整合併班，導致多數路線之班車運輸效率（即乘載率）偏低，未能充分發揮疏運乘客功能，同時也導致台汽公司之營運虧損。

表A.2  台汽公司國道客運路線資料

| 路線名稱 | 區間里程 (公里) | 日駛班次 (班) | 每車公里營運實績 |  | 運輸效率 (%) | 備註 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 收入(元) | 成本(元) |  |  |
| 基隆新竹 | 100.5 | 12 | 16.2 | 42.9 | 41 |  |
| 基隆台中 | 191.5 | 8 | 24.7 | 32.5 | 24 |  |
| 基隆南投 | 223.9 | 2 | 25.8 | 31.9 | 25 |  |
| 基隆高雄 | 381.5 | 2 | 19.8 | 32.4 | 18 |  |
| 台北南投 | 197.7 | 6 | 26.1 | 29.7 | 26 |  |
| 台北-台14-南投 | 197.7 | 46 | 26.1 | 29.7 | 26 |  |
| 台北嘉義 | 250.2 | 58 | 21.2 | 29.2 | 21 |  |
| 台北-大林-嘉義 | 248.3 | 10 | 21.2 | 29.2 | 21 |  |
| 北阿里山 | 324.9 | 2 | - | - | - |  |
| 台北新營 | 272.2 | 6 | 21.2 | 29.2 | 26 |  |
| 台北台南 | 308.9 | 24 | 21.8 | 30.3 | 20 |  |
| 台北高雄 | 354.9 | 46 | 23.9 | 29.4 | 24 |  |
| 台北-岡山-高雄 | 364.8 | 6 | 23.9 | 29.4 | 24 |  |
| 西站中正 | 45.8 | 156 | 34.7 | 39.4 | 35 |  |
| 台北-中清-台中 | 165.6 | 88 | 29.5 | 32.9 | 28 |  |

表A.2  台汽公司國道客運路線資料（續）

| 路線名稱 | 區間里程 (公里) | 日駛班次 (班) | 每車公里營運實績 |  | 運輸效率 (%) | 備註 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 收入(元) | 成本(元) |  |  |
| 台北-中港-台中 | 165.6 | 91 | 29.5 | 32.9 | 28 |  |
| 台北桃園 | 35.8 | 144 | 24.5 | 38.3 | 46 |  |
| 台北中壢 | 43.0 | 62 | 34.0 | 34.9 | 62 |  |
| 台北楊梅 | 52.2 | 30 | 21.5 | 30.5 | 44 |  |
| 台北新竹 | 82.8 | 140 | 19.6 | 30.4 | 44 |  |
| 台北-埔心-新竹 | 80.2 | 39 | 13.9 | 32.7 | 22 |  |
| 台北沙鹿 | 172.2 | 32 | 21.7 | 29.3 | 34 |  |
| 台北員林 | 193.5 | 51 | 18.1 | 30.7 | 19 |  |
| 台北日月潭 | 247.7 | 2 | 28.5 | 32.8 | 27 |  |
| 台北埔里 | 228.0 | 22 | 28.5 | 32.8 | 27 |  |
| 台北竹南 | 96.1 | 52 | 23.3 | 26.3 | 51 |  |
| 台北苗栗 | 119.1 | 46 | 24.9 | 28.1 | 49 |  |
| 中崙豐原 | 152.0 | 10 | 13.9 | 29.2 | 30 |  |
| 松山中正 | 45.8 | 202 | 31.8 | 39.4 | 35 |  |
| 松山-大園-中正 | 47.7 | 22 | 31.8 | 35.8 | 35 |  |
| 板橋台中 | 163.4 | 32 | 24.7 | 33.3 | 24 |  |
| 板橋新竹 | 77.3 | 18 | 21.1 | 35.4 | 43 |  |
| 桃園台中 | 142.7 | 28 | 20.0 | 29.5 | 42 |  |
| 桃園嘉義 | 221.2 | 4 | 11.7 | 32.2 | 35 |  |
| 桃園高雄 | 325.1 | 8 | 18.8 | 30.3 | 20 |  |
| 中正台中 | 149.1 | 47 | 29.1 | 39.4 | 21 |  |
| 中正-豐-台中 | 149.1 | 3 | 29.1 | 33.8 | 21 |  |
| 中壢台中 | 125.8 | 32 | 24.9 | 30.1 | 51 |  |
| 新竹台中 | 95.7 | 48 | 29.5 | 32.1 | 57 |  |
| 新竹高雄 | 288.8 | 4 | 17.8 | 30.5 | 19 |  |
| 新竹台南 | 238.2 | 2 | 15.3 | 27.5 | 20 |  |
| 竹南台中 | 80.4 | 4 | 12.1 | 31.6 | 28 |  |
| 苗栗台中 | 58.6 | 42 | 21.2 | 35.8 | 39 |  |
| 台中台南 | 151.9 | 64 | 21.6 | 33.3 | 20 |  |
| 台中嘉義 | 94.5 | 60 | 18.1 | 31.9 | 37 |  |
| 台中高雄 | 197.9 | 28 | 17.5 | 30.7 | 19 |  |
| 台中-岡山-高雄 | 196.5 | 28 | 17.5 | 30.7 | 19 |  |
| 埔里高雄 | 260.4 | 8 | 17.8 | 30.1 | 37 |  |
| 嘉義高雄 | 117.9 | 56 | 13.2 | 30.4 | 30 |  |
| 嘉義台南 | 67.6 | 32 | 16.7 | 34.4 | 61 |  |
| 台南屏東 | 64.7 | 36 | 17.1 | 43.1 | 32 |  |
| 方澳台北 | 136.6 | 2 | 19.0 | 38.3 | 35 |  |
| 羅東台北 | 126.4 | 17 | 20.1 | 38.3 | 35 |  |
| 頭城台北 | 102.6 | 1 | 20.1 | 38.3 | 35 |  |
| 中崙金青 | 49.4 | 109 | 26.5 | 37.9 | 38 |  |
| 基隆新店 | 34.1 | 168 | 27.8 | 39.9 | 47 |  |
| 基隆-石牌-護專 | 33.7 | 114 | 28.7 | 39.7 | 46 |  |
| 基隆中壢 | 62.2 | 50 | 33.0 | 46.2 | 52 |  |
| 基隆三重 | 29.4 | 181 | 33.0 | 46.2 | 52 |  |
| 基隆台北 | 28.8 | 346 | 16.2 | 42.9 | 57 |  |
| 基隆-安樂-台北 | 29.7 | 14 | 16.2 | 42.9 | 57 |  |
| 中崙基隆 | 26.9 | 114 | 16.2 | 42.9 | 57 |  |
| 台北北斗 | 207.0 | 10 | 15.2 | 27.0 | 19 |  |

表A.2  台汽公司國道客運路線資料（續）

| 路線名稱 | 區間里程 (公里) | 日駛班次 (班) | 每車公里營運實績 |  | 運輸效率 (%) | 備註 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 收入(元) | 成本(元) |  |  |
| 台北彰化 | 177.3 | 13 | 16.8 | 31.4 | 17 |  |
| 台北屏東 | 364.8 | 24 | 21.7 | 30.9 | 22 |  |
| 新店-二高-竹東 | 83.4 | 16 | 6.3 | 29.5 | 15 | 北二高路線 |
| 中正-中壢休息站 | 15.7 | 60 | 6.3 | 42.0 | 7 | 轉運路線 |
| 台北-二高-大溪 | 40.0 | 3 | 24.5 | 46.6 | 38 | 北二高路線 |
| 板橋-二高-嘉義 | 242.3 | 2 | 12.7 | 31.1 | 2 | 北二高路線 |
| 台北-二高-竹東 | 72.1 | 60 | 28.9 | 33.3 | 56 | 北二高路線 |
| 台北-大溪-竹東 | 83.0 | 4 | 28.9 | 33.3 | 56 | 北二高路線 |
| 新竹-林口 | 60.1 | 10 | 19.6 | 35.4 | 43 |  |
| 台中屏東 | 209.1 | 24 | 17.8 | 33.4 | 19 |  |
| 彰化高雄 | 179.1 | 8 | 9.0 | 28.3 | 20 |  |
| 溪州-北斗-台北 | 209.9 | 5 | 15.2 | 27.0 | 19 |  |
| 高雄阿里山 | 192.4 | 2 | 11.0 | 36.2 | 20 |  |
| 台東台中 | 368.9 | 6 | 26.9 | 36.1 | 38 |  |
| 台中溪頭 | 70.3 | 2 | 20.2 | 44.6 | 26 | 聯營路線 |
| 台中杉林 | 88.3 | 2 | 20.2 | 44.6 | 26 | 聯營路線 |
| 潮州楠梓 | 48.8 | 10 | 1/2 | 53.5 | 8 | 聯營路線 |

註：1.資料來源:台汽公司。

2.網底者為運輸效率高於50%之路線。

A.3 台汽營運場站分佈概況

公路汽車客運之場站主要包括招呼站、車站、停車場及保養場，一般而言，公路汽車客運業均會選取重要之交通產生及到達地點設置車站；至於停車場及保養場，因其主要係提供車輛之停放及維修保養，故大多位於非交通繁忙地區。目前大多數長途客運之起迄場站係位於市中心區，一般旅客均須利用地區性公共運輸系統進出長途客運場站，由於市區道路交通原本就較為壅塞，致長途客運起迄站附近通常存在相當多的交通問題。

台汽客運現有場站約96處，西部走廊共設有車站58處，站址主要分佈於交通便利之市鄉鎮中心區，且大多與鐵路車站及市區公車或地區性公路客運車站毗鄰，以方便旅客之集散轉運。目前台汽客運各區之場站分佈，簡要說明如下：

1.北部區域

含基隆市、台北縣、市、桃園縣及新竹縣、市，共設有19處車站，其中行駛高速公路之路線計使用12處車站，包括台北地區6 站，桃園縣 4站，基隆市及新竹市皆設一站。

2.中部區域

含苗栗縣、台中縣、市、彰化縣、南投縣及雲林縣等六縣市，共設有24處車站，其中，行駛高速公路之路線使用12處車站，包括中縣市及南投縣各設 6站，彰化及雲林兩縣各設 3站，苗栗縣設1 站。

3.南部區域

含嘉義縣、市、台南縣、市、高雄縣、市、屏東縣等地，共設有13處車站，其中行駛高速公路之路線使用其中 8站，除屏東外，四大縣市各設2 站。

至於台汽高速公路客運路線由車站至高速公路交流道間之行駛時間，則如表A.3 所示。由表知，目前車站至交流道之行駛時間有70%以上需要20分鐘以上，加上其週邊道路交通狀況普遍不佳，嚴重影響車輛進出站區，對於行駛高速公路客運路線之營運效率均產生不良影響。

表A.3 台汽高速公路客運路線車站至交流道行駛時間分析

| 行駛時間 | 車站數 | 車站名稱 |
| --- | --- | --- |
| 小於20分 | 11 | 基隆、中正機場、楊梅、頭份、苗栗、豐原、員林、西螺、斗南、新營、松山 |
| 20~40分 | 16 | 台北西站、台北北站、中崙、桃園、中壢、新竹、大甲、台中、台中南站、干城、彰化、斗六、嘉義、台南、高雄東站、高雄南站 |
| 40~60分 | 7 | 淡水、板橋、草屯、南投、北港、屏東、北斗 |
| 大於60分 | 5 | 埔里、日月潭、水里、阿里山、恆春 |

註；資料來源：台灣省政府交通處[1996]

A.4 高速公路客運接駁／轉運運作現況

目前高速公路之客運服務雖無轉運站之設置，卻已有轉運之實際營運服務。例如統聯客運已於台中大雅交流道附近設置有一中途站，原係作為台北─高雄、台北─台南及十三條通運路線作為中途停靠休息使用，惟此一中途站實際亦扮演著該公司班車之轉運整合功能，即提供乘客在此車站上、下及轉車的功能。另外，在高雄楠梓交流道附近亦設有車站，用以轉運台北─高雄、高雄─屏東二條路線之乘客。

事實上，最能發揮高速公路客運接駁功能者應屬違規遊覽車（即所謂「野雞車」）。現違規經營高速公路客運業務之遊覽車，主要係利用中山高速公路沿線交流道之匝道或附近聯絡道路進行停靠，以利乘客上、下（即進行轉運），而其聯絡停靠方式，大多以燈號、無線電對講機、旗幟或固定停靠等方式來提供乘客轉運搭乘。

台汽公司在高速公路轉運的應用上，基本上可歸納出下列三種：

1.調度支援

如台北─高雄營運路線班次，在若干時段內之旅客量並不多，然而在台北─台中營運路線卻有相當多的等候旅客，於是將台北─高雄預備班次轉調台北─台中路線支援。此時於實際運作中，該班車通常僅運送乘客至交流道附近（如台中朝馬站），而後繼續將車開回至高雄。

2.乘載支援

除上述班車調度支援外，亦有利用不同營運路線之班車進行乘載支援之情況，如台北─高雄營運路線之班次，利用其未完全坐滿之座位，讓大排長龍之台北─台中的乘客上車，且於台中交流道附近（如台中朝馬站），讓乘客下車轉搭地區客運，而該班車再繼續折返中山高速公路繼續南行到高雄。

3.聯運運送

若干營運路線（如台北─嘉義及已移轉營運之台北─北港路線），亦有利用中途休息站（如頭份車站），讓乘客上、下車，以增加班車之乘載率。