國  立  交  通  大  學

運 輸 科 技 與 管 理 學 系

博 士 論 文

車流動力模式之構建與模擬 ─ 以波茲曼輸運方程為基礎

Modeling and Simulation of Vehicular Kinetic Flow ─ from the Viewpoint of Boltzmann Transport Equation

| 研究生 | ：羅仕京 | 撰著 |
| --- | --- | --- |
| 指導教授 | ：卓訓榮 | 博士 |

中華民國九十一年七月

# CHAPTER 1

# Introduction

There are many indications of the complexity of living in today’s world. The behavior of road using is one of them. As the trend of increasing travel demand, planning, design, prediction, control and management of the transportation system become more and more important. Traffic flow theory provides the description of the fundamental traffic flow characteristics and analytical techniques. This new science has addressed questions related to understand traffic processes and to optimize these processes through proper design and control. The former questions could be described as basic research and the latter as applied research. Dynamic traffic management is an efficient tool to solve traffic congestion. Among the research topics of dynamic traffic management, dynamic traffic flow theory is an important part. Therefore, a new dynamic traffic flow model is proposed to describe the multilane multiclass traffic in this study. In the introduction, we will discuss motivations, objectives, approaches, overview and contributions of the research in detail. In the following section, the research motivations are presented first.

## 1.1 Research Motivations

The worsening of traffic congestion in developed countries causes great economical loss every year. Thus, transportation management becomes a wild research topic. In the evolution of transportation researches, dynamic traffic management seems to be a feasible way to relieve congestion. According to the types of collecting data and applications, dynamic traffic researches can be classified to three categories; that is, dynamic origin/destination (dynamic O/D) estimation, dynamic traffic flow theory, and dynamic traffic assignment. A complete dynamic management system includes the three kernels; the relation between them is illustrated in Figure 1.1. Since the whole procedure is a cycle, each part can be the initial step. If the dynamic travel cost (or travel time) of each path in the network is known, the dynamic origin/destination table will be generated from the dynamic O/D model. If the dynamic travel cost (or travel time) of each link and the dynamic O/D table in the network are known, the path flow will be obtained by the dynamic assignment model. If the flow on each path is known, the travel time and dynamic link flow are obtained from the dynamic traffic flow model. Therefore, dynamic traffic flow theory is an important part of dynamic traffic management.

Figure 1.1 The architecture of dynamic traffic management system

Dynamic traffic flow theory is useful not only in the whole dynamic traffic management architecture, but also in individual applications. The applications of dynamic traffic flow theory are dynamic traffic information prediction, signal control and ramp metering, incident management, evaluation of alternatives, and so on. Dynamic traffic flow researches vary from a very simple equation to a complex simulation model. Usually, we can develop traffic analytical techniques from three different points of view, microscopic, macroscopic and mesoscopic analysis. Microscopic analysis may be selected for moderate-sized systems where the number of transport units passing through the system is relatively small, and there is a need to study the behavior of individual units in the system. Macroscopic system may be selected for higher-density or large-scale systems. Mesoscopic analysis is an intermediate method, which does not distinguish individual vehicles, but specifies the behavior of individuals; for instance, in probabilistic terms. As we discuss the three methods, microscopic modeling involves complicated interaction of vehicles and time-consuming computation. Macroscopic modeling method provides continuous properties for analyses. However, the lack of behavioral discussion is the drawback of macroscopic models. A mesoscopic model combines the computational efficiency of macroscopic models with the opportunity to derive properties that refer to individual vehicles. Therefore, in this dissertation, the modeling methodology starts with a mesoscopic model, and then a macroscopic system is derived from the model.

Traffic problems in the real world involve many aspects, such as geometric design, different driving behavior, different types of vehicles, weather, lane usage, network topology and so on. There are diverse research topics of traffic flow. Generally, multilane multiclass traffic is a common situation in the real world. Multilane traffic involves acceleration, deceleration, lane-changing, and passing of vehicles. Multilane traffic is not the only dilemma; there are different types of vehicles and driving behavior on a road. Each type of vehicle and driving behavior has its own acceleration, deceleration, lane-changing and passing characteristics. To improve the performance of traffic controls, describing and predicting the influence of each type of user is necessary. Therefore, this study tries to develop a dynamic multilane multiclass macroscopic traffic flow model and its numerical simulation.

## 1.2 Framework of Research and Research Objectives

Since dynamic traffic flow is an important research topic in transportation development, a modeling approach is presented in this study. First, the review of related researches is presented. According to the review and real traffic situations, a dynamic traffic flow model is proposed. The main considerations of modeling are:

(i) Interaction among vehicles.

(ii) Multilane traffic flow.

(iii) Multiple types of driving behavior.

(iv) Rapid computation.

Owing to the considerations, a systematic model, which includes an interacting equation and a vehicular kinetic model, is proposed. After the systematic model is developed, a numerical method is proposed. Numerical examples are also employed to explain the model. Consequently, the main research objectives of this dissertation are as follows:

- Developing a modeling procedure of dynamic traffic flow research.

- Modeling the interaction among vehicles and the equilibrium distribution.

(ii) Modeling multilane multiclass users traffic flow.

- Presenting a numerical simulation method for the systematic model.

- Discussing and developing stochastic dynamic traffic.

## 1.3 Research Approach

In respect to the dynamic traffic flow, since spatial and temporal characteristics are concerned, this study tries to apply partial differential equations to modeling. The model is a Boltzmann equation (see Appendix C) and it is a mesoscopic model. After multiplying the model by the moment function and integrating over the individual velocity, a macroscopic system is obtained. The equations of the macroscopic system are systematic conservation laws; i.e., conservation of vehicle numbers, conservation of momentum (motion equation) and conservation of energy (variance equation). Thus, the evolution of traffic flow is described by the system. The system is the transient equations. A complete system includes transient equations and state equations to make the system self-consistent. The state equation is derived from the car-following theory in this study. The equation is named as the traffic dispersion model and is in the form of a nonlinear Poisson equation.

However, the systematic model cannot be solved analytically. Numerical schemes for the equations are developed separately and the whole system is approximated by the decoupling scheme. The numerical scheme presented in this study is based on the finite difference method and the finite volume method. With regard to the traffic dispersion model, a finite volume based monotone iterative scheme is employed. On the other hand, the numerical scheme of the kinetic system is developed under the steady state first. After the spatial discretization scheme is derived, the time variant problem is discretized by the Crank-Nicolson method.

In the case of stochastic dynamic traffic flow model, a simple model based on the LWR model is presented first. Considering the complexity of computing, we decompose the nonlinear term by a decomposed polynomial. Then, the stochastic model is formulated by the Itô equation. The modeling and decomposition method will be the foundation of extending the whole deterministic system to stochastic one.

## 1.4 Overview of Thesis

This dissertation is organized as follows. First, the introduction chapter gives an overview of the motivation, research objectives, and the research approach of this study. A literature review of related researches in the relevant areas is presented in the second chapter. The topics discussed in the chapter includes: (i) static and time series traffic flow model, (ii) the car-following theory, (iii) the LWR-like models, (iv) the PW-like models, (v) gas-kinetic models, and (vi) particle hopping models.

In the third chapter, the conformity of multi-dimensional macroscopic traffic flow variables is discussed. Multilane traffic flow model proposed in this study is developed in two-dimensional space in terms of the definitions of multi-dimensional traffic flow variables defined in this chapter. A new variable (flow density) is introduced to make the fundamental diagram consistent in multi-dimensional space.

As the variables are redefined, the traffic dispersion model is proposed in chapter four. The model is derived from the car-following theory and describes the interaction among vehicles. A linear model, which considers the influence of density as the interaction (represented by the traffic field), is formulated first. Then, under the equilibrium assumption, the density is distributed by the interaction. Thus, a nonlinear dispersion model is derived.

The applications and the analyses of the dispersion model are discussed in Chapter 5. The analytical solution and analyses of the linear model are shown and a monotone iterative scheme, which is derived from the divergence theorem (see Appendix B), is introduced to solve the nonlinear dispersion model. A numerical example is employed to explain the model and traffic problems.

According to the multi-dimensional traffic flow variables and the dispersion model, the self-consistent vehicular kinetic model is presented in the sixth chapter. The modeling procedure is shown as follows. The first step is multiplying the Boltzmann equation by the moment functions. The second step is integrating both sides of the equations and using the chain rules. At last, the macroscopic quantities are obtained. By coupling the dispersion model to the kinetic system, a self-consistent system is obtained. Then, the model is extended to multiclass traffic.

Since the system includes several partial differential equations, the numerical method of a simplified model is developed first. The model is named as the drift-diffusion model, which includes two equations; i.e., the continuity equation and the traffic dispersion model. Numerical scheme of steady state problem and time variant problem are proposed in the seventh and eighth chapter, respectively.

After discussing the dynamic model, we extend the model to stochastic dynamic one. The ninth chapter derives nonlinear stochastic models from the LWR model. Since the stochasticity and the nonlinearity cause the difficulty of solving the model, a polynomial decomposition is presented to approximate the stochastic dynamic model. In addition, a computational procedure is shown to compute the decomposed polynomial.

The last chapter presents the conclusions and perspectives of this dissertation. The content of each chapter and its relationship to the other chapters are given in Figure 1.2.

## 1.5 Research Contributions

The principal contributions of this study are as follows:

- Development of a dynamic traffic flow modeling procedure and architecture. The modeling procedure can also be employed to develop more complicated situations and behavior.

- Presenting an interacting equation (the dispersion model) so as to describe the influence of density on a road. The dispersion model simplifies the discussion of multilane and multiclass traffic. Also, the model implies the finite-space requirement so as to make the application to multiple driving behaviors possible.

- Multilane multiclass dynamic traffic flow model. By coupling the dispersion model to the kinetic system, we derive a self-consistent model.

Figure 1.2 Overview of the dissertation and the relationship between chapters.

- Development of numerical simulation method for the dynamic traffic flow model. From the numerical scheme and architecture proposed in this study, the numerical scheme of the whole can be derived. In addition, the decoupling scheme can be applied to other partial differential systematic equations.

- Modeling stochastic dynamic traffic. A decomposed polynomial is employed to approximate the nonlinearity. The simplest stochastic model based on the LWR model is presented. The motion equation and the variance equation can also be formulated as stochastic models in the same way,.

In this study, a modeling procedure, a self-consistent multilane multiclass traffic flow model and its simulation are presented to describe and predict dynamic traffic flow. The results can be the foundation of further dynamic traffic flow and dynamic assignment researches.

# CHAPTER 2

# Literature Review

Traffic flow characteristics include time headway, flow, time-space projectory, speed, distance headway, and density. Traffic flow analytical techniques include kinematics modeling, shock wave analysis, car-following theory, and simulation modeling. The analytical process varies from a very simple equation to a complex simulation model. This chapter reviews topics relevant to traffic flow models and their applications. The discussion of dynamic traffic flow model is the most important part herein. Section 2.1 presents a discussion of static models and time series models. Section 2.2 presents the essential microscopic dynamic traffic model; i.e., the car-following theory. The LWR model and relevant researches are discussed in section 2.3. The extended models, the PW model and the gas-kinetic models, are reviewed in section 2.4 and 2.5, respectively. In section 2.6, a newly analytical microscopic technology, which is named the particle hopping method (cellular automation), is discussed. A brief summary and conclusion is given in section 2.7.

## 2.1 Static Models and Time Series Models

As the revolutions of mathematical models, traffic flow researches also start with static models. Although the applications of static models are restricted, they are the first step to traffic researches. Developing mathematical models can start with different viewpoints. The first one is developed with a microscopic viewpoint, which focuses on individual behavior; the main characteristics they describe and predict are time headway, individual speeds, and distance headway (Oliver, 1962, Foote, 1963, Daou, 1964, Greenberg, 1970, Sullivan and Troutbeck, 1997).

Static traffic flow models are mostly developed through the Poisson process, which is used to describe the probability of vehicles passing a point on a roadway. In its simplest form, the Poisson model does not take into account the finite size of the vehicles nor does it properly describe the characteristic bunching of traffic that commonly occurs, for example, on a two-lane road when passing is hindered. (Newell 1970, Kurkjian, 1980) Dependency among vehicles has also been expressed through different headway models. However, simple Poisson models cannot be validated for platooning conditions, more complex models based on a mixed distribution have been adopted (Cowan, 1975, Buckley, 1968, Branston, 1976, Wasielewski, 1974, 1979).

Macroscopic system describes higher-density, large-scale systems. The basic variables of macroscopic traffic flow are speed (u), density (k), and flow (Q), and the relationship between them is Q=k×u. Greenshield (1934) and Olcott (1955) show that an approximate linear relationship between speed and density of traffic exists, whereby, as the average speeds of vehicles increase, the density of the traffic stream decreases. The relationship is used to establish a series of theoretical equations that, in turn, permit the determination of optimum of speed and spacing required to produce maximum volumes under given conditions of traffic composition. Greenberg (1959) suggested that an equation describing the stead-state relation between Q and k, or between u, the velocity of the stream, and k could be derived by assuming a particular equation of state for the fluid. However, Greenberg’s model is exhibited by the lack of an upper limit on stream velocity, since k→0, u→lnkj/0→∞. Edie (1961) modified Greenberg’s model by a two-regime model. He treated congested and non-congested flows separately. Also, Edie mentioned that the data, which is available for both regions are rather limited; therefore, employing different models for congested and non-congested traffic is reasonable. From empirical studies, this opinion is observed.

Characteristics and distributions of hourly volumes and average speed are discussed by Vaughan (1970), Wright (1970), Gazis, and Knapp (1971), Makigami, et al. (1971), Nahi, Trivedi (1973), and Branston (1979). In addition, their efforts also include developing more economic methods to collect data and calibrating parameters from raw data. Compared to microscopic models, which are better in behavior discussion and safety analyses, macroscopic models can be used in signal and network planning. Other research results of the static macroscopic traffic flow model are summarized in Table 2.1 and Table 2.2.

Table 2.1 Table of single-regime models

| Single-regime models | Equations |
| --- | --- |
| Greenshields model (1934) | u=uf1−kkj |
| Greenberg model (1959) | u=u0lnkjk |
| Underwood model (1961) | u=ufe−k/k0 |
| Northwestern's model (1967) | u=ufe−1/2k/ko2 |
| Drew model (1968) | u=uf1−kkjn+1/2 |
| Pipes-Munjal model (1967) | u=uf1−kkjn |
|  | uf：free flow speed kj：congested density uo：critical speed ：critical density |

reference: May(1990)

Table 2.2 Table of multi-regime models

| Multiregime models | Free-flow regime | Transitional-flow regime | Congested-flow regime |
| --- | --- | --- | --- |
| Edie model (1961) | u=ufe−k/k0 | － | u=u0lnkjkEMBED Equation.2 \* mergeformat |
| Two-regime linear model (1967) | embed Equation u=uf1−kkj EMBED Equation.2 \* mergeformatEMBED Equation.2 \* mergeformatk≤k1 | － | u=uf1−kkj EMBED Equation.2 \* mergeformatEMBED Equation.2 \* mergeformatEMBED Equation.2 \* mergeformat |
| Modified Greenberg model (1967) | constant speed EMBED Equation.2 \* mergeformatEMBED Equation.2 \* mergeformatk≤k2 | － | u=u0lnkjk EMBED Equation.2 \* mergeformatEMBED Equation.2 \* mergeformat |
| Three-regime linear model (1967) | u=uf1−kkj EMBED Equation.2 \* mergeformat | u=uf1−kkj | u=uf1−kkjEMBED Equation.2 \* mergeformat |
|  | ki：specified traffic density ，i=1.2.3.4 |  |  |

reference: May(1990)

Between dynamic forecasting and static planning is short-term forecasting, which is also an essential part of traffic control and information systems. A considerable amount of effort has therefore been expended on this problem, with a variety of different algorithms proposed. These include Kalman filters: (Chang and Gazis, 1975, Okutani and Stephanedes, 1984), and spectral analysis (Lam and Rothery, 1970, Nicholoson and Swann, 1974), ARIMA (Kirby et al., 1984, Clark et al, 1993, Kirby et al. 1997) and others.

In contrast to models, which ignore the interaction and dependency between successive vehicles. Mahalel and Hakkert (1985) presented time-series models for the spot speeds of vehicles on a road section, which incorporate information on the extent of existing dependency between the speeds of successive vehicles. However, the model can only describe a vehicle speed distribution at a certain point or section in space and does not provide full time-space diagram.

Kalman filter can also be applied to multilane traffic condition. Chang and Gazis (1975) developed a method by the Kalman filter for the density estimation of a road section and extended to include explicit consideration of lane-changing on a multilane freeway. Kirby, Watson and Dougherty (1997) discussed the relative merits of neural networks and time series methods for traffic forecasting and summaries the findings from a comparative study of their performance for motorway traffic in France. They concluded that neural networks perform slightly worse than the traditional ARIMA time series when predicting 30 min ahead, and the improvement to ARIMA projections achieved by including upstream data is only slight.

Since, time series models consider the evolution of traffic variables with time, they are more contributive to traffic prediction and control than static models. However, the validity of time series models strongly depends on historical data and can only be applied to similar condition. Therefore, time series can hardly be applied to evaluation of traffic strategies and distinct situations.

## 2.2 Car-Following Theory

The existence of dependency among vehicles traveling a platoon is well known in the literature as discussed before. The most popular expressions for these relationships are based on the car-following theory, which are microscopic dynamic models. These models, which represent the behavior of individual cars as they fight for survival and a place in a line of cars moving along a highway, were first considered by Reuschel (1950) and Pipes (1953) in the early 1950s. Herman Montroll, Potts Rothery (1959) developed a series model of car-following. Every driver who finds himself in a single-lane traffic situation is assumed to react mainly to a stimulus from his immediate environment. The stimulus was assumed to be a function of the position of the car, the position of its neighbors, and the time-derivatives of these positions. It was conjectured, and verified experimentally, that the strongest stimulus was the relative speed of the car with respect to the car in front. If we further assumes that the sensitivity, λ, is constant, we’ll obtain the “linear car-following model”:

d2xnt+Tdt2=λdxn−1tdt−dxntdt                                  (2.1)

in which n denotes the position of a car in a line of cars, t denotes time and xn denotes the position of the nth car along a highway. The model of Eq. (2.1) was used to investigate the following questions:

- Assuming a pair of cars, one following the other, under what circumstances would a maneuver cause a collision?

- Assuming a long line of a cars, under what circumstances does a perturbation by the car in front get simplified as it propagates down the line of cars?

The first question refers to the “local stability” and the second to the “asymptotic stability” of traffic.

Gazis, Herman and Rothery (1961) checked the linear car-following model and discussed nonlinear car-following models. In studies of nonlinear car-following model, the main difference between them is the formulation of sensitivity (Chandler et al., 1958, Gazis et al., 1959, 1961, Newell, 1961, Evans and Rothery, 1977, Gipps, 1981). They combined Eqs (2.1) and (2.2)

λ=ax't+Tn+1mxnt−xn+1tl                                         (2.2)

and obtained

xn+1''t+T=ax't+Tn+1mxn't−xn+1'txnt−xn+1tl.                            (2.3)

Equation (2.3) is a generalized sensitivity function, which can be simplified to specific models proposed before. In the study, the model is also related to macroscopic model such as Greenshield’s model, Greenberg’s model and Edie’s model. Car-following models can be summaries as Table 2.3.

Table 2.3 Governing equations of car-following models

| Model | Governing equation |
| --- | --- |
| Bando et al. (1995) | xn''=aVxn+1−xn−xn' |
| Chandler et al. (1958) | xn''t+T=λxn+1'−xn' |
| California Chandler et al. (1958) | xn''t+T=λxn+1−xn+c−T1xn' |
| Edie (1961) | xn''t+T=λxn'xn+1'−xn'xn+1−xn2 |
| Gazis et al. (1959) | xn''t+T=λxn+1'−xn'xn+1−xn |
| Gazis et al. (1961) | xn''t+T=λxn'lxn+1'−xn'xn+1−xnm |
| Herman et al. (1959) | xn''t+T=λ1xn+1'−xn'¦t−T1+λ2xn+2'−xn'¦t−T2 |
| Newell (1961) | xn''t+T=Vxn+1−xn |

There are still different considerations of car-following modeling. Lee (1965) considered that the reaction of a follower depends not only on the stimuli of his leader, but also his historical experience. Simonsson (1993) suggested a new car-following model. The hypothesis is that a driver reacts to the stimuli deceleration, which is proportional to two components: the difference between the preferred time gap and the actual time gap, and the difference between the preferred speed and the actual speed. The model estimated a user impact and environmental effects with a knowledge-based expert system and showed the result with a numerical example. Todosiev (1963) and Wiedemann (1974) introduced psycho-physiological considerations into the car-following models. Wiedemann considers so-called reaction thresholds to distinguish different regions of driver behavior and Fritsche (1994) extended the model further.

Mostly, applications of car-following theory are done by simulation such as signal control, effect of small cars (McClenahan and Simkowitz, 1969) and motorcycles and cars mixed traffic flow. Researches of automatic highway system (AHS) are based on the car-following theory, too. The advantage of car-following theory is that it has behavioral meaning. On the other hand, it is also a disadvantage because the vehicular behavior is hard. Another disadvantage of the car-following theory is robust computation. The car-following theory can hardly provide real-time prediction, especially in large-scale traffic network (Zhang and Owen, 1998).

## 2.3 The LWR-like models

Lighthill and Whitham (1955) and Richards (1956) are the first people that presented the macroscopic kinetic traffic flow model. They used kinematic concepts to describe waves in traffic. The basic premises of their model are that traffic is conversed and that there exists a one-to-one relationship between velocity and density. The LWR model can be viewed as a good and basic approximation. Mathematically, LWR model states that the density k and flow Q satisfy

∂kx,t∂t+∇⋅Qx,t=0,                                           (2.4)

where t denotes time and x denotes position. Equation (2.4) expresses the conservation of vehicles. In addition, Q, k and velocity u are assumed to satisfy Q=ku(k). From these assumptions, Eq. (2.4) has the solution k=F(x-ct), where F is an arbitrary function (the initial condition), c is the wave speed and c=dQ/dk. Equation (2.4) implies that in homogeneities, such as changes in density of cars, propagate along a stream of cars with constant wave speed c with respect to a stationary observer.

Those who doubt that traffic behaves like a fluid, not only does traffic flow like a fluid but sometimes it appears to leak and flood an entire area. However, Forbes and Simpson (1968), Lam and Rothery (1970) and Persaud and Hurdle (1988) found deceleration waves propagating back from the bottleneck by aerial photo records. Also, longer average time headways, which related to driver uncertainty, were found after deceleration waves.

Shock wave analysis is the useful analytical technique extended from kinematical wave. Greenberg and Daou (1960) applied Greenberg’s model to control tunnel-traffic flow under the traveling backward shock waves caused by a bottleneck. The control of the traffic input by platooning the entering vehicles is shown to eliminate the shock waves and allow higher flows. Michalopoulos and Pisharody (1980) and Michalopoulos et al. (1980, 1981) also employed the LWR model and shock wave analysis to derive an isolated real time signal control.

To make the simulation and empirical data of dynamic models more understandable, Makigami, and Newell (1971) tried to represent traffic flow on three-dimensional space and interpret the three dimensional model. The three dimensional variables are space, time and density. They illustrated the vehicles’ space-time trajectory on the same x-t graph; then, this family of curves can be interpreted as the contours of a three-dimensional surface for which the third dimension is the vehicle number. Newell (1993,Part I, II, III) evaluated flows or densities by cumulative flow A(x, t) past any point x by time t. It is shown here how a formal solution for A(x, t) can be evaluated directly from boundary or initial conditions without evaluations at intermediate times and positions. Daganzo (1994) also presented a model on a highway with a single entrance and exit. The model proposed can be used to predict traffic’s evolution over time and space, including transient phenomena such as the building, propagation, and dissipation of a queue. The easy-to-solve difference equations are used from a special case of the hydrodynamic model of traffic flow. (Daganzo, 1995a, 1995b) extended the difference equations to network traffic. Traditionally, LWR model is discussed in one-dimensional space. Pearce (1975) proposed a two-dimensional LWR model. However, the variables defined are still in one-dimensional space and the discussion in his work cannot find the advantage of two-dimensional model, either.

The LWR model is a simple but sufficient traffic theory if the size and end location of a queue are the things that one only cares about such as the time-space trajectory of a shock. Unfortunately, traffic flow phenomena are very complex, and some important phenomena that we are also interested in elude the LWR model. One such phenomenon is the stop-start waves in long queues often observed on congested freeways. Therefore, the development of continuum models extends the LWR model by replacing the instantaneous flux function with a dynamic one. This is referred to higher order models, which is also named the PW model.

## 2.4 The PW-like models

The assumption of u=u(k) is a steady state assumption of velocity, which means that velocity changes instantaneously as density changes. It is certainly not valid in some traffic flow situations. To overcome the steady state assumption of velocity, Payne (1979) used a motion equation to obtain time variant speed.

∂u∂t+u∇⋅u=−1k∇⋅Pek+1τuek−u,                            (2.5)

where uek is an equilibrium speed-density relation and τ is the relaxation time. The motion equation (or so-called momentum conservation) coupled with the continuity equation is referred to PW model since the concept of employing a higher order function in LWR model is proposed by Whitham (1974) first. The term −∇⋅Pek/k=−Pe'k∇⋅k/k is an anticipation term, which takes it into account that drivers beware the preceding traffic condition, where Pek is an equilibrium traffic pressure. Payne used an anticipation term determined by Pe'k=1/2τue'k. In general, we have Pek=kθek, where θek denotes the equilibrium velocity variance. Analogous to the situation for uek, there have been different suggestions for the function θek. For example, Kühne (1991), and Kerner and Kohnhäuser (1993, 1994) suggested it takes a constant value

θek=c02,                                                      (2.6)

whereas Phillips (1977) suggested to take a linear relation

θek=θm1−kkm.                                               (2.7)

The dependence of speed uek and variance θek on the density in the equilibrium case can be derived by means of kinetic theory from the equilibrium distribution function. After that, Papageorgiou et al. (1982) substituted Eq. (2.8), which is an Euler-like discretize form for Eq. (2.5).

∂u∂t+ςu∇⋅u=−ντk+κ∇⋅k+1τuek−u,                        (2.8)

where ς,κ are predicted parameters. This modification can speed up numerical computing. Sanwal et al. (1996) presented a model based on Papageorgiou’s work and test it on field data. They extended the model to flow under the influence of traffic-obstructing incidents. Michalopoulos et al. (1993) developed a semi-viscous model, which substitutes a viscosity term into Eq. (2.5).

∂u∂t+u∇⋅u=−akb∇⋅k+1τuek−u,                           (2.9)

where a and b are constants.

However, Hauer et al. (1979) mentioned that the above model shows unrealistic results for strong changes of the densities (e.g. for shock waves). To overcome this difficulty, a viscosity term is introduced in the above equations smoothing out the solutions. For example Michalopoulos et al. (1993) and Kühne (1991) modified Eq.(2.5) by Eqs (2.10) and (2.11)

∂u∂t+u∇⋅u=−akb∇⋅k+hkp∇⋅∇k,                             (2.10)

∂u∂t+u∇⋅u=−1k∇⋅Pek+1τuek−u+μk∇⋅∇u,                (2.11)

where a, b, h, p and μ are parameters.

There are still other researches based on the PW model. Choi and Lee (1995) examined the possibility of such complex behaviors in the traffic flow modeled by Eq. (2.12),

k∂u∂t+u∇⋅u=μ∇∇⋅u−∇⋅P+kτuek−u,                     (2.12)

together with their stability against random fluctuations. The power spectra of the current fluctuations are also computed, which shows 1/f fluctuations in the density-wave phase. Pue (1982) applied a discrete PW model in large automated transportation network. The model employed a nonlinear function of densities in adjacent links to compute vehicle flow across link boundaries. The function is introduced to help preserve the representation of bunch input flows, at merge junctions, otherwise flows tended to smooth out to average values. Del Castillo et al. (1994) proposed an expression for the reaction time of drivers as a function of traffic density in the PW model. Later, Del Castillo, and Benitez (1995a, 1995b) presented a functional form for the speed-density relationship. This functional form is made up of a nondimensional spacing the equivalent spacing and of a function, the generating function, whose argument is the equivalent spacing. This functional form is derived by means of two different arguments. The first argument is based on the set of properties that the volume-speed-density relationship should satisfy. The second one arises when applied to a dimensional analysis of a generic car-following model.

Since higher order models cost a lot of computation, Chronopoulos and Johnston (1997, 1998) Johnston and Chronopoulos (1999a, 1999b) proposed a series effort on real-time traffic simulation by paralleling the numerical algorithm of the semi-viscous model. After paralleling the numerical algorithm, the computing time reduces from more than one hour to about one and a half minute. Thus, parallelization is a solution to realize real-time simulation of traffic flow simulation, especially in higher order models.

In another aspect, Jou and Lo (2001) derived nonlinear stochastic dynamic models, which are based on LWR and PW model, and employed Adomian’s polynomial (Adomian, 1983, 1986) to decompose the nonlinear term so as to reduce the numerical complexity. Also, Weits (1992) presented a stochastic dynamic traffic flow model under the assumption of stationary, which allows the model to be linearlized.

Daganzo (1995) mentioned that although the result of higher order model is a little better than first order model, it needs more computation. He also pointed out higher order models bring the wrong result; that is, in some cases, vehicle speed will be negative. Aw and Rascle (2000) explained that the phenomena is caused by the wrong assumption of traffic pressure. Bonzani (2000) and recently, Günther et al. (2001) presented a modeling procedure to ensure that the PW-like models describe all situations correctly.

## 2.5 The Gas-Kinetic Models

Gas-kinetic theory is a further modeling methodology. This kind of model is first employed to describe traffic flow by Prigogine, Herman and their colleagues (Prigogine and Andrews, 1960, Anderson et al., 1962, Prigogine and Herman, 1971) and is referred to the Boltzmann-like model. Hoogendoorn and Bovy (2000) classified the gas-kinetic models as the mesoscopic models. Boltzmann equation is widely applied in applied science such as gas dynamics, population analysis, traffic flow, semiconductor and so on (Bellome, 1997). Generally, the Boltzmann equation is used to describe properties of a fluid in the large domain by examining the statistics of motion of constituent particles. Prigogine described a traffic fluid with a probability density for the velocity (v) of an individual car, fx,v,t, which may vary with a function of time t and the coordinate x along the highway. This density is assumed to satisfy the equation

∂f∂t+v∂f∂x=∂f∂trelaxation+∂f∂tinteraction.                              (2.13)

The first term of the right-hand element of Eq. (2.13) stems from the fraction that fx,v,t differs from some desired velocity distribution f0v. The second term describes that a fast car will slow down owing to the influence of a slow car. The interaction term has been criticized. It has been argued that the collision term is only valid in the situation, which describes the incoming vehicle passing each single car in the queue independently. Therefore, Paveri-Fontana (1975) proposed an improved model to overcome the shortcoming of Prigogine’s approach. He generalized the phase-plane distribution as fx,v,v0,t, where v0 is the individual desired velocity. fx,v,t=fx,v,v0,tdv0. Nevertheless, his model still considered that the maneuver on the lanes of a multilane road is the same and his model does not take queuing effects into account. Transitory and stationary solutions and numerical simulations are proposed by researches (Illner et al., 1999, Klar et al., 1996, 1997, 1999a, 1999b, 2999, Wegner and Klar, 1996) under different interaction, relaxation, and adjustment terms.

The nature of the equilibrium solutions of a Boltzmann-like equation is likely to be reflected in the nature of the associated hydrodynamic models (Paveri-Fontana, 1975, Helbing, 1995, 1996, 1997, 1999, 2000, Hoogendoorn and Bovy 1998, 2000, 2001). Helbing (1995) presents a gas-kinetic model for traffic operations. In contrast to the model of Paveri-Fontana (1975), the model of Helbing (1995) considers additional terms. The first is a velocity diffusion term, which takes the individual fluctuations of the velocity into account due to imperfect driving. The other one is the rate of vehicles entering and leaving the roadway. The macroscopic systematic equation Helbing (1995) obtained from the mesoscopic equation has one more equation than PW-like model. The equation is the velocity variance equation. The previous models handle the velocity variance as an equilibrium quantity. However, in non-equilibrium situations, the velocity variance may be better treated as a dynamic variable with a further equation describing its evolution. In particular, to predict traffic jams, an increase of the variance appears to be a very important indicator. The velocity variance equation is shown as:

∂θ∂t+u∇⋅θ=−2θ∇⋅u+2μk∇⋅u2+2τθek−θ+μk∇⋅∇u+κk∇⋅∇θ,  (2.14)

where θ is velocity variance, μ,κ are coefficients, τ is relaxation time and θe is equilibrium velocity variance. θe is assumed to depend on k only as it is assumed in the PW-like models. The numerical simulation of the model obtained following results:

- The section of high density induces low speed and small speed variance.

- The section of low density induces high speed and large speed variance.

- The largest speed variance takes place at the highest speed behind a platoon.

Empirical studies were also employed by Helbing (1996, 1997) to validate the model. Multilane traffic can also be extended by the model (1997, 2000). Besides, Hoogendoorn and Bovy (1998, 2000, 2001) derived a multiple user-classes traffic flow model from Boltzmann equation by a similar approach. From the works of Paveri-Fontana, Helbing, Hoogendoorn and Bovy, macroscopic models can be derived from Boltzmann-like model, which is a mesoscopic model. Boltzmann-like models can be developed with behavioral analysis. Therefore, the macroscopic models derived from them improve the lack of behavioral analyses of macroscopic models.

## 2.6 Particle Hopping (Cellular Automation)

A more recent addition to the development of vehicular traffic flow theory is cellular automation (CA) or particle hopping method. Although CA is first proposed long ago (Gerlough, 1956), CA has begun to receive wide attention of statistical physics community only after the simple formulation by Nagel and Schreckemberg (1992). In CA, a road is represented as a string of cells, which are either empty or occupied by exactly one vehicle. Movement takes place by hopping between cells.

CA is defined as follows. Each vehicle can have an integer velocity between 0 and umax. The complete configuration at time step t is stored, and the configuration at time step t+2 is computed from that, i.e., using a parallel or synchronous update. All vehicles execute in parallel the following steps:

- Let g (gap) equal the number of empty sites ahead.

- If u>g (too fast), then slow down to u=g (rule 1); otherwise if u<g (enough headway) and u<umaz, then accelerate by one u=u+1 (rule 2).

- Randomization: If after the above steps the velocity is larger than zero (u>0), then, with probability p, reduce u by one (rule 3).

- Vehicle propagation: Each vehicle moves u sites ahead (rule 4).

Pesheva et al. (1997) proposed a CA (particle hopping) to describe 1D traffic flow under a bottleneck situation. Then Rickert (1996) extended CA to a two lane traffic condition. Furthermore, Chowdhury (1997) a two-lane traffic with two kinds of vehicles. CA also can be built as a control model. Hattori (1999) and Jin et al. (1999) applied CA to signalize intersection with periodic boundary condition. With respect to the capacity of the signalized intersection, Nagel (1996) and Nagatani (1996) considered the several different acceleration effects of vehicles in CA models.

Nagel (1996) compared the other models to CA and made some conclusions as follow:

- Robust computing: CA is known to be numerically robust especially in complex geometries.

- University: Intuitively, a relatively simple microscopic model should be able to show the essential features of traffic jams. One might even speculate that the critical exponents of traffic jam formation are universal.

- Towards minimal models: The present results show that close-up car-following behavior is not the most important aspect to traffic model. The important crucial aspect is to model deviations from the optimal (smooth) behavior and the ways in which they lead to jam formation. Another important aspect, which seems far from obvious, is the acceleration behavior, especially when there are other vehicles ahead, since it is the acceleration behavior that mostly determines the maximum flow out of a jam (which may be a simple traffic light).

- Traffic dynamics: Fast running and easy to implement CA can be very useful in interpreting measurements.

- Microscopic simulation: CA is inherently microscopic, which allows one to add individual properties to each vehicle.

- Stochastic and fluctuations: Last but not least, CA are stochastic in nature, thus producing different results when using different random seeds even when starting from identical initial conditions. The traffic system is inherently stochastic and the variance of the outcomes is an important variable itself.

CA is a kind of microscopic models and inherits the advantages and disadvantages of them. Although its concept is posed many years ago, the physical models are presented recently. It is claimed that it has the potential to model and describe real problems well. However, in motorcycles and vehicles mixed traffic, motorcycles are not restricted in single lane. Their motion should be treated as two-dimensional behavior. As researches, which are proposed by car-following theory in Taiwan, the model may become very complicate and the behavior may not be easily modeled anymore. In addition, the more complicate CA induce much more robust numeric can be forecasted.

## 2.7 Brief Review of Finite Difference Approximation

In this section, the historical development of numerical methods is presented. The main references are Mitchell and Griffitns (1980), Ames (1992), Humi and Miller (1992), LeVeque (1992) and Morton (1996). Numerical calculation is commonplace today in fields where it was virtually unknown before 1950. The high-speed computing machine has made possible the solution of scientific and engineering problems of great complexity. This capability has, in turn, stimulated research in numerical analysis since effective utilization of such devices depends strongly upon the continual advance of research in relevant areas of mathematical analysis. One measure of the growth is the upsurge of books devoted to the subject in the years after 1953. A second measure is the development, during the same period, of many research journals and societies, whose primary concern is numerical analysis. Whole new disciplines, such as numerical fluid mechanics and computational mechanics, have arisen.

Finite difference approximations for derivatives were already in use by Euler in 1768. The simplest finite difference procedure for dealing with the problem dx/dt=gx,t, x0=a is obtained by replacing dx/dtn−1 with the crude approximation xn−xn−1/Δt. This leads to the recurrence relation x0=a, xn=xn−1+Δtgxn−1,tn−1 for n>0. This procedure is known as Euler’s method. Thus, we see that for one-dimensional systems the finite difference approach has been deeply ingrained in computational algorithms for quite some time.

For two-dimensional systems, the first computational application of finite difference methods was probably carried out by Runge in 1908. He studied the numerical solution of the Poisson equation uxx+uyy=constant. At approximately the same time Richardson, in England, was carrying on similar research. His 1910 paper was earliest work on the application of iterative methods to the solution of continuous equilibrium problems by finite differences. In 1918, Liebmann, in considering the finite difference approximation to Laplace’s equation, suggested an improved method of iteration. Today the name of Liebmann is associated with any method of iteration by single steps in which a fixed calculation sequence is followed.

The algebra solution of finite difference approximations is usually accomplished by some iteration procedure. Various schemes have been proposed to accelerate the convergence of the iteration. A summary of those that were available in 1950, and that are adaptable to automatic programming, is given by Frankel. Other methods require considerable judgment on the part of the computer and are therefore better suited to hand computation. In 1949, Higgins gives an extensive bibliography of such techniques. In the latter category, the method of relaxation has received the most complete treatment. Relaxation was the most popular method in the decade of the thirties. Two books by Southwell (1940, 1946) describe the process and detail many examples. The successive over-relaxation method, extensively used in some areas, is an outgrowth of this highly successful hand computation procedure.

For any given continuous system there are a multiplicity of discrete models, which are usually comparable in terms of their relative truncation errors. Early approximations were second order- that is, Oh2- and these still play an important role today. Higher order procedures were promoted by Collatz and Fox in 1940s. The relative economy of computation and accuracy of second-order processes utilizing a small interval size, compared with higher order procedures using larger interval sizes, has been discussed in the paper of Southwell (1949).

The ultimate goal of discrete methods is the reduction of continuous systems to “equivalent” discrete (lumped parameter) systems that are suitable for high-speed computer solution. One is initially deceived by the seeming elementary nature of the techniques. Their usage is certainly widespread but also they are often misapplied and abused. A little knowledge is dangerous since these approximations raise many serious and difficult mathematical questions of adequacy, accuracy, convergence, and stability.

Discretization of the governing equations and boundary conditions of the continuous problem may be accomplished physically, but is more often carried out mathematically. The specialist sometimes finds the physical approach useful in motivating further analyses. In such a modus operandi the discrete (physical) model is given the lumped physical characteristics of the continuous system. For example, a heat conducting slab could be replaced by a network of heat conducting rods. The governing equations are then developed by direct application of the physical laws to the discrete system.

On the other hand, in the mathematical approach the continuous formulation is transformed to a discrete formulation by replacing derivatives by, say, finite difference approximations. When the continuous problem formulation is already available this procedure is simpler and more flexible.

Partial derivatives can be approximated by finite differences in many ways. Several simple approximations will be introduced here. Let the problem under consideration be the one-dimensional wave equation

Ut+AUx=0,             −∞<x<∞, t≥0                    (2.15)

Ux,0=U0x.                                                 (2.16)

We discretize the x-t plane by choosing a mesh width hx≡Δx and a time step ht≡Δt. For simplicity we take a uniform mesh, with hx and ht constant, although most of the methods discussed can be extended to variable meshes. The finite difference methods we will develop produce approximations Ujn∈Rm to the solution Uxj,tn at the discrete grid points. There are a wide variety of finite difference methods that can be used. Table 2.4 and Table 2.5 show the difference schemes and their stencils.

Table 2.4 Explicit finite difference methods for Eq. (2.15)

| Name | Difference Equations | Stencil |
| --- | --- | --- |
| Explicit Method |  |  |
| Right One-sided | Ujn+1=Ujn−hthxA(Ujn−Uj−1n) |  |
| Left One-sided | Ujn+1=Ujn−hthxA(Uj+1n−Ujn) |  |
| Lax-Friedrichs | Ujn+1=12(Uj−1n+Uj+1n)−ht2hxA(Uj+1n−Uj−1n) |  |
| Leapfrog | Ujn+1=Ujn−1−ht2hxA(Uj+1n−Uj−1n) |  |
| Lax-Wendroff | Ujn+1=Ujn−ht2hxA(Uj+1n−Uj−1n)+ht22hx2A2(Uj+1n−2Ujn+Uj−1n) |  |
| Beam-Warming | Ujn+1=Ujn−ht2hxA(3Ujn−4Uj−1n+Uj−2n)+ht22hx2A2(Ujn−2Uj−1n+Uj−2n) |  |

Reference: LeVeque (1980)

Table 2.5 Implicit finite difference methods for Eq. (2.15)

| Implicit Method |  |  |
| --- | --- | --- |
| BTBS | Ujn=Ujn+1+hthxA(Ujn+1−Uj−1n+1) |  |
| BTCS | Ujn=Ujn+1+hthxA(Uj+0.5n+1−Uj−0.5n+1) |  |
| Crank-Nicolson | 1−ht/hx2Ujn+ht/2hx2A(Uj+1n+Uj−1n)1+ht/hx2Ujn+1−ht/2hx2A(Uj+1n+1+Uj−1n+1) |  |

Reference: LeVeque (1980)

## 2.8 Summary and Discussions

Dynamic traffic flow model and dynamic traffic assignment are closely related in dynamic traffic management. Essentially, a traffic assignment model needs the information of link cost, which is in terms of link travel time mostly. The relationship between dynamic traffic flow models and dynamic traffic assignment models are illustrated as Figure 1.1. Link cost or link travel time can be estimated directly by a flow/link travel time function (Davidson, 1974, Taylor, 1977, Cowan, 1980, Fisk, 1990, 1991, HCM, 1994, Daganzo, 1995, Shimizu et al., 2000) or by a traffic flow models. Therefore, traffic flow models are the basic part of transportation applications. However, dynamic traffic assignment models are complicated in computing, link cost estimation methods are chosen simple one so as to relieve computing burden. Occasionally, a static traffic flow model is employed to estimate the link cost in a dynamic traffic assignment model and cause inconsistency. (Mahmassani, Chang and Herman, 1986, Mahmassani and Peeta, 1993; Mahmassani et al., 1994, Wie et. al., 1995, 1998, Chen, 1998). Thus, developing a dynamic traffic flow model, which possesses the ability to describe traffic conditions more realistically and provide real-time prediction, is a necessary research. From the discussion above, the related research topics of dynamic traffic flow theory for dynamic or real-time prediction and large scale networks are illustrated in Figure 2.1.

-28-

Figure 2.1 Research topics of macroscopic dynamic traffic model for ITS

The conclusions and comments of traffic flow researches can be illustrated as follows.

- As the LWR-like model is employed, one has to give externally the relation between speed, flow and density. This unsatisfying in terms of development of a theory (Nelson, 1995, Negal, 1996, Papageorgiou, 1998, Zhang, 2001).

- Microscopic simulation models usually been adopted for the simulation of relatively small or simple systems and macroscopic simulation can be used to simulated the large network or complex system. Numerous runs of microscopic simulations may be necessary to achieve such a long time. This limitation makes applications of these simulation models problematic for the provision of real-time traffic information. (Chi, Lee and Kim, 1995, Negal, 1996, Sheu, 1997, Zhang and Owen, 1998). Daganzo (1994) mentioned that microscopic simulation usually keep track of each vehicle’s destination, their assumption is difficult to validate because humans’ behavior in real traffic is difficult to observe and measure.

- Higher order macroscopic models should be modeled carefully or unreasonable situation will be induced, such as wrong way traveling flow. In addition, the higher order models also need numerous computing. Fortunately, the accelerating methods of numerical simulation for higher order models are more achievable than the methods for microscopic models.

- Mesoscopic models have the advantages of macroscopic and microscopic models. According to statistical mechanism, macroscopic models can be derived from related mesoscopic models. The relations among mesoscopic, microscopic and macroscopic models are illustrated in Figure 2.2.

Consequently, this study proposes a Boltzmann equation and derives it to macroscopic models. There are three main differences between this study and previous studies. The first one is that a multilane road is considered as a two-dimensional domain. The second one is that the acceleration effect of Boltzmann equation is considered as the influence of traffic field. The last one is that the second order moment function considered herein is individual velocity variance. Traffic field is derived from the car-following theory, which describes the interaction between vehicles. The detail of derivation of traffic field is illustrated in Chapter three. The concept of traffic field not only describes the interaction between vehicles but also makes the macroscopic system consistent.

As mentioned above, finite difference method is a simple and efficient numerical method. Numerical schemes, which are based on the finite difference method, are suggested to solve our systematic equations.

Figure 2.2 Relations between microscopic, mesoscopic and macroscopic traffic flow models (reference: Hoogendoorn, 1999)

# CHAPTER 3

# Definitions of Multi-Dimensional Traffic Flow

This chapter verifies the conformity of multi-dimensional macroscopic traffic flow variables. Dynamic traffic flow models are usually extended to multilane models by coupling several one-dimensional models (Gazis et al., 1960, Miller, 1970, Chang, 1975, Habermann, 1977, Edie et al., 1980, Holland, 1997). Since the lane-changing probability or function can hardly be determined and formulated, the treatment of multi-dimensional motion is necessary. However, the basic variables of traffic flow theory, flow, density, and speed are originally defined in one dimension. When the flow-density-speed relation is introduced to the multi-dimension, the units become inconsistent. The purpose of this chapter is to define traffic flow variables in multi-dimensional space of the rectangular Cartesian coordinate system. Moreover, the applications are also discussed herein.

The chapter is organized in the following way. Section 3.1 shows the applications of two-dimensional models and the mapping of one-dimensional variables to a two-dimensional space. Section 3.2 defines three-dimensional traffic flow variables and its applications. Suggestions for further researches and the conclusion are discussed in section 3.3.

## 3.1 Two-Dimensional Traffic Flow

Definitions and the relation among one-dimensional macroscopic traffic flow variables, which are flow, density, and speed, are required to be discussed before the discussion of a multi-dimensional model. They are as follows: (The unit of each variable is presented in the brackets.)

Flow [pcu×time-1 or pcu/hr]: the number of vehicles (passenger car unit, pcu) passing through a certain point in a given time period. Flow is denoted by Q.

(3.1)

Density [pcu×length-1 or pcu/km]: the number of vehicles (pcu) occupying a length of road. Density is denoted by k.                            (3.2)

Speed [length×time-1 or km/hr]: the length that vehicles passing through during a specified time period. Speed is denoted by u.                        (3.3)

The relation among the three variables is the fundamental diagram and is shown as:

Q=ku.                                                        (3.4)

Equation (3.4) is an important relation of macroscopic models. Considering the LWR model:

∂kx,t∂t+∂Qx,t∂x=0   or   ∂kx,t∂t+∇Qx,t=0,                   (3.5)

where x is the spatial independent variable and t is the temporal independent variable.

Equation (3.4) should be imposed into ∂Q/∂x. If the speed is assumed to be a function of density, Eq. (5) can be solved then. The model describes single lane roads, which conforms to one-dimensional space. Multilane traffic flow models were extended by Eq. (3.5) (Chang, 1975, Haberman, 1977). If there are N lanes, the modified model is shown as:

∂kix,t∂t+∂Qix,t∂x=Gix,t−Rix,t,      ,              (3.6)

,                ,              (3.7)

where Gix,t is the number of vehicles entering lane i, and Rix,t is the number of vehicles exiting lane i. ki denotes the density of lane i and Qi denotes the flow of lane i. Eq. (3.7) means that the number of vehicles on a certain road section is conserved, if there is no entrance or exit from roadsides. However, Gix,t and Rix,t are hardly observed, therefore Eqs (3.6) and (3.7) fail to represent actual situations.

Considering multiple lanes as a plane is another modeling philosophy. Generally, the way motorcycles move is not the same as cars, which follow one after another. The movement of motorcycles is not restricted within a single lane. They do not behave as vehicles, which follow one another. The movement of motorcycles among cars is like the movement, which small particles pass through the spacing of large particles. Consequently, viewing a multi-lane road as a plane in the two-dimensional rectangular Cartesian coordinate system can replace Eqs (3.6) and (3.7) and it is a more practical viewpoint to solve a mixed traffic flow problem. The three basic variables are required to describe characteristics of plane motion. Definitions (3.1) to (3.3) are extended as follows:

Flow [pcu×time-1 or pcu/hr]: the number of vehicles (pcu) passing through a certain section during a given time period. Flow is a scalar and is denoted by Q.

(3.8)

Density [pcu×length-2 or pcu/km⋅m or pcu/km⋅lane]: the number of vehicles (pcu) occupying a section of road. Density is a scalar and is denoted by k.                                                           (3.9)

Velocity [length×time-1 or km/hr]: the length that a vehicle passing through during a specified time period. Velocity is a vector and is denoted by u=ux,uy, where ux and uy means velocity in x- and y-direction.    (3.10)

Figure 3.1 shows an imaginary line L, which is vertical to the roadside. Vehicles pass through L during time Δt, implying

Q=LkuxΔtΔt=kLux=kLu⋅n,                                  (3.11)

where n is the normal unit vector of the imaginary line L and the inner product u⋅n=ux,uy⋅1,0=ux. Eq. (3.11) is the correct fundamental diagram in two-dimensional space. If we substitute the definitions (3.8) for (3.10) into Eq. (3.4), it shows that the unit of the left-hand side is unequal to the unit of the right-hand side. A new variable, “flow density”, is introduced into two-dimensional traffic flow. Flow density is defined as:

Flow density [pcu×(length×time)-1 or pcu/hr⋅lane or pcu/hr⋅m]: the number of vehicles (pcu) passing through a unit width of a road in a given time period. Flow density is a vector and is denoted by q=qx,qy, where qx and qy means flow in x- and y-direction, respectively. qy can only be zero, if there is no entrance or exit from the roadside.                            (3.12)

Thus, q, k, and u satisfy Eq. (3.4); that is, q=ku, where ku=kux,kuy=qx,qy. As a result of Eq. (3.11) and the fundamental diagram, the relation between flow and flow density can be shown as:

Q=q⋅dln.                                             (3.13)

Figure 3.1 Two-dimensional traffic flow of a multilane highway

Assuming that qx and qy are constants, Eq. (3.13) can then be simplified: Q=q⋅ln. Substituting q for Q in Eq. (3.5) results in a two-dimensional traffic flow model:

∂kx,y,t∂t+∇qx,y,t=0.                                       (3.14)

Two-dimensional traffic flow variables extended to a three-dimensional model are discussed in the following section.

## 3.2 Three-Dimensional Traffic Flow

As postulated in Section 3.1, definitions (3.8), (3.9), (3.10), and (3.12) can be extended to a three-dimensional rectangular Cartesian coordinate system. Regarding traffic flow on a road as a two-dimensional motion is reasonable. However, why is a three-dimensional model needed? The third dimension herein referred to follows Baker’s work (1983). Baker treats the third dimension (z-direction) as capacity, which is defined as: the maximum hourly flow rate can be served on a road. Thus, the three-dimensional models are allowed to describe effective capacity and traffic flow. Effective capacity measures the operational capacity, which depends on weather, time, flow, and road conditions. The three-dimensional model is also proposed by Cho et al. (2001). Capacity and flow have equivalent units. For convenience, capcity is used as the unit of capacity instead of [pcu×time-1]. The three-dimensional definition is as below:

Flow [pcu×time-1 or pcu/hr]: the number of vehicles (pcu) passing through a certain volume in a given time period. Flow is a scalar and is denoted by Q.

(3.15)

Density [pcu×length-2×capacity-1 or pcu/km⋅m⋅capacity or pcu/km⋅lane⋅capacity]: the number of vehicles (pcu) occupying a certain volume of road. Density is a scalar and is denoted by k.              (3.16)

Velocity [length×time−1 or km/hr]: the length that vehicles pass through during a time period. Velocity is a vector and is denoted by u=ux,uy,uz, where ux, uy, and uz means velocity in x-, y- and z-direction, respectively.       (3.17)

Figure 3.2 shows an imaginary plane A that is vertical to the road. Vehicles pass through A during time Δt, implying

Q=AkuxΔtΔt=kAux=kAu⋅n.                                  (3.18)

where n is the normal unit vector of A and the inner product u⋅n=ux,uy⋅1,0,0=ux. For the same reason as with the two-dimensional model, flow density should be introduced.

Flow density [pcu×length-1×capacity-1 or pcu/hr⋅lane⋅capacity or pcu/hr⋅m⋅capacity]: the number of vehicles (pcu) passing through a unit plane, which is normal to the road in a given time period. Flow density is a vector and is denoted by q=qx,qy,qz, where qx, qy, qz means flow in x-, y- and z-direction, respectively. qy can only be zero, if there is no entrance or exit from the roadside.                                          (3.19)

Thus, q, k, and u satisfies Eq. (3.4)in three dimensional space; that is, q=ku, where ku=kux,kuy,kuz=qx,qy,qz. From Eq. (3.18), the relation between flow and flow density is shown as:

Q=q⋅dAn                                             (3.20)

If qx, qy, and qz are constants, Eq. (3.20) can be simplified to Q=q⋅An. Substituting q for Q in Eq. (3.5) shows a three-dimensional traffic flow model.

∂kx,y,z,t∂t+∇qx,y,z,t=0                                     (3.21)

Considering the capacity, we can discuss and analyze much more traffic situations.

Figure 3.2 Three-dimensional traffic flow of a multilane highway

## 3.3. Summary

In this study, multilane traffic is suggested being considered as a multi-dimensional problem. Since the units of the multi-dimensional traffic flow variables and the fundamental diagram are inconsistent, the traffic flow variables in two-dimensional and three-dimensional space are defined in this chapter. A new variable, flow density, is introduced to make the fundamental diagram consistent in multi-dimensional space. Also, the simplest kinetic model (LWR model) is presented as an example of multi-dimensional modeling in this chapter. Higher order kinetic models, such as the PW-like model and gas-kinetic model, can still be extended to multiple dimensions in the same way.

Another method of making variables consistent is to remove the dimensions from the variables (Del Castillo, Benitez, 1995). This occurs when each variable is divided by an average value such as:

ρ=kk, ω=qq, η=uu.                                         (3.22)

Therefore, the extension of models to a higher dimensional space is consistent. However this kind of model loses some important information, such as flow and speed in y-direction, which is described by a multi-dimensional model. For this reason, extending traffic flow variables to higher dimensions is a necessary step before they are modeled.

# CHAPTER 4

# Traffic Dispersion Model

A dynamic system includes transient equations and state equations. Transient equations describe the dynamic behavior and state equations determine the state of each time step. Since most roads are multilane in each direction in the real world, the traffic dispersion model presented in this chapter is proposed to describe multilane traffic flow. Traffic flow researches are also extended to multilane models instead of single lane models in the four main methodologies: car-following models, kinetic equations, Boltzmann-like models and cellular automation. However, the multilane traffic model is a difficult problem because of observing and modeling lane-changing behavior and model computation. On the other hand, simplifying a multilane traffic model to a single lane model by taking an average is too simple to describe the multilane traffic phenomena. Fortunately, traffic information for management focuses on macroscopic quantities. If a multilane traffic flow model can describe the macroscopic phenomena well, we do not have to formulate the lane-changing behavior in detail. A vehicular dispersion model, which describes how traffic density distributes on a multilane road, is suggested herein. There are several important applications of the dispersion model. Firstly, the model is employed to describe the state at each time point. As mentioned above, the multilane traffic model is a difficult problem. The dispersion model provides a simple modeling approach. Secondly, the dispersion model also implies the finite-space requirements. If there are multiple types of driving behaviors, the model can be employed to evaluate the aggregated influence of density. This advantage is useful when vehicles, such as motorcycles and bicycles, are not restricted to drive along one by one in a single lane. Another importance of the dispersion model is the model relates the basic variables in traffic flow and makes the systematic equations self-consistent (Cho and Lo, 2002c, 2002d, 2002e, 2002f). In the following section, the linear dispersion model is derived first.

## 4.1 Linear Dispersion Model

The vehicular dispersion model is derived in a two-dimensional space so as to distribute density on the road directly. Modeling a two-dimensional multilane model is much easier than modeling lane-changing behaviors. The derivation of the multilane dispersion model is started with the discussion of the interaction between a single vehicle and other vehicles by car-following theory (Herman et al., 1959, Gazis et al. 1960, Newell, 1961, Gipps, 1981). The car-following theory describes that each driver reacts mainly to a stimulus from his immediate environment according to the general relationship as (Reaction)t+T=λ(Stimulus)t, where λ is a sensitivity coefficient and T is a reaction time lag. The relation can be treated as Newton’s law, which implies that acceleration is induced by external force. On the other hand, the external force can also be described by field, especially the isolated force. Thus, it is assumed that there is an external field applied to a road and vehicles on the road will be pushed forward along the direction of the external field. Nevertheless, one vehicle on a road adjusts its velocity and spacing according to the relative position between other cars for the sake of safety. It is assumed that each vehicle has its own field. As mentioned above, the traffic force (or in the form of traffic pressure) is a resistance against the external field. From the assumptions mentioned above, the external field determines the free flow acceleration (or velocity) of vehicles moving along the road and the interaction of field produced by vehicles is the resistant. The velocity of vehicles depends on the summation of external field and the resistant (interaction). By Newton’s law, traffic field E is in proportion to acceleration dv/dt, where v is the individual velocity. From the discussion of the car-following theory, it can also be assumed that the traffic field (E) depends on velocity and headway. To simplify the complication of the problem, E is assumed to depend on spacing and to satisfy the inverse-square law. It is reasonable to assume that the influence of cars in the same lane be larger than that in the adjacent lanes. Let the influence of cars in the same lane be M times larger than that in the adjacent lanes. If we consider the interaction between two vehicles (vehicle 0 and 1), the traffic field produced by vehicle 1 (leader) will act on vehicle 0 (follower). The traffic field acting on vehicle 0 can be formulated as:

E01=eε0x0−x1x0−x13i+y0−y1M2y0−y13j,                              (4.1)

where e is the passenger car equivalent, ε0 is the interacting parameter and x0,y0 and x1,y1 are the position of vehicle 0 and 1, respectively. The influence between two vehicles is larger as the distance between them decreases. Therefore, the assumption of the inverse-square law is reasonable herein. The traffic field and the interaction are illustrated in Figure 4.1. For the convenience’s sake, we transform the domain from Ω to Ω, that is, let x=x,y=My. The transformation is employed for the convenience of computing. After the result of the transformed problem is obtained, it will be transformed back to the original space. The traffic field acting on vehicle 0 in Ω is denoted by

E=iNeiXi/εiXi3,                                              (4.2)

where N is the number of vehicles on the road, Xi denotes the spacing. ei is the passenger car equivalent of vehicle i and εi is the interacting parameter of vehicle i. Figure 4.2 illustrates the transformed traffic field. In the continuous space, Eq. (4.2) can be represented as

E=eεk−ks/X2dΩ,                                         (4.3)

where e denotes the passenger car equivalent and ε denotes the interacting parameter. It is assumed that all vehicles have the same characteristics and driving behavior, i.e., the passenger car equivalent and the interacting parameter are constants. In the following paragraph, the assumption will be released and the multi-class user traffic situation will be discussed. k is the actual density and ks is the unrestrained density that is the density, which vehicles do not interfere with each other. When the actual density is equal to the unrestrained density, the traffic flow is moving freely. E is a conservative field, therefore, a potential function φ exists by the potential theory. The potential function φ satisfies E=−∇xφ. Thus, the magnitude of traffic field is illustrated as

divE=−Δφ=ek−ks/ε+Ka,                                      (4.4)

where divE denotes the magnitude of traffic field, ε=ε/4π, Ka=Kax, which depends on the position x, is the adjust term of the road condition if the road condition is ideal Ka=0.

Figure 4.1 (a)elliptic traffic field        (b)interacting between vehicles

Equation (4.4) is a linear Poisson equation, which implies that density is independent of the traffic potential. The linear model implies several facts. The first one is the magnitude of traffic field is increasing when density is increasing. The second one is the direction of field, which is from high potential to low potential. It means that density is distributed from high potential to low potential by the traffic field. Therefore, density should be distributed by the field (or potential). The relationship between density and potential is obtained from the assumption that density will tend to become its equilibrium state, which is defined as the most possible microscopic state under a given macroscopic state. From the theory of permutation and combination, the most possible microscopic state is the uniform distribution if there is no other external influence. This means vehicles will not always drive forward bumper to bumper. They will adjust their spacing in order to separate from each other uniformly. This is a common phenomena occurring at the downstream section of a signalized interaction or a merging intersection. The equation of the equilibrium state, which is named the equilibrium distribution, is derived in the following section.

Figure 4.2 Figure of circular traffic field after transforming.

## 4.2 Equilibrium Distribution and Nonlinear Dispersion Model

The main purpose of the equilibrium distribution is describing the most possible microscopic state under a given macroscopic traffic phenomena, i.e., under a given macroscopic condition, such as total number of vehicles or density on a road, the vehicles will tend toward to distribute uniformly on the road. In other words, we assume that the distribution of vehicles on a road will tend toward its maximum entropy condition. The derivation starts with the following mathematical programming:

MaxW=N!ini!,                               ∀i∈Ω         (4.5)

s.t.ini=n1+n2+⋯+nm=N,                  ∀i∈Ω         (4.6)

iniΘi=n1Θ1+n2Θ2+⋯+nmΘm=Θtol,            ∀i∈Ω         (4.7)

where i is the number of intervals, N is the number of total vehicle, ni is the vehicle number of interval i, Θtol is the total velocity variance, and Θi is the velocity variance in invertal i. The velocity variance of individual car is defined as ui−ue2, where ui is the average velocity of interval i and ue is the equilibrium velocity. Under the equilibrium state, the total velocity variance Θtol is assumed to be equal to NΘe, where Θe is the equilibrium velocity variance. Equation (4.5) is the objective function, and Eqs (4.6)~(4.7) are given macroscopic phenomena. Equation (4.5) finds the most possible number of combinations of ni if the total number of cars on the road is N. Equation (4.5) is in a simplified form, which neglects the all-possible combinations of the denominator, since the denominator is a constant. Equation (4.6) is the conservation of vehicle numbers, and Eq. (4.7) is the conservation of total velocity variance, which means that the traffic situation is equilibrium. Both N and Θtol are assumed to be constants because the macroscopic situation is given. The conservation of vehicle numbers is the essential constraint of traffic researches. Average velocity is not considered herein because it is not a conservative quantity. The reason of considering the conservation of velocity variance is that traffic force acting on the road is induced by velocity variance, and velocity variance is influenced by traffic potential. The relationship between traffic force, traffic potential, and velocity variance gives ∇xΘ∝e∇xφ. Since e is a scalar proportion, which is not an absolute value, the relationship can be simplified as. As discussed above, the traffic field (the magnitude of traffic field) will influence the distribution of density. Therefore, the influence of velocity variance should be considered in the mathematical programming.

The mathematical programming (4.5)~(4.7) can be solved by the following steps. First, use Stirling’s equation to approximate Eq. (4.5), and lnW=NlnN−inilnni is obtained. From the first order condition, when W is the maximum, ∂Wmax/∂ni=0 is true, which implies ∂lnWmax/∂ni=0. By Kuhn-Tucker condition, α and β are introduced as Lagrange multipliers. We have ilnni+α+βΘi∂ni=0. Thus,

ni=exp−α−βΘi.                                              (4.8)

If we treat velocity variance as a continuous function, then Eq. (4.8) can be rewritten as

n=IΘexp−α−βΘ,                                             (4.9)

where IΘ is the number of intervals, whose velocity variance is Θ. nΘdΘ=IΘexp−α−βΘdΘ is the vehicle’s velocity variance between Θ and Θ+dΘ. Before determining the coefficient α and β, IΘ should be determined first. Using an indirect way is easier. Let U=Ux,Uy=u−ue and U=u−ue, and then Θ=U2. That is, a single velocity variance corresponds to a velocity magnitude U, and the number of velocity variance state IΘdΘ between Θ and Θ+dΘ is the same as the number of velocity state IUdU between U and UdU. The number of velocity state IUdU between U and UdU can be measured by the area of a circular ring in velocity space U in radius and dU in width, which is 2πUdU, in the phase plane. Hence,

IUdU=1h2dxdydUxdUy=Ah22πUdU,                      (4.10)

where A is the area of the whole section, and h2 is the area of each interval. Because IΘdΘ=IUdU, U=Θ1/2 and dU=1/2Θ−1/2dΘ, Eq. (4.10) becomes

IΘdΘ=πAh2dΘ.                                                 (4.11)

The number of vehicles, whose velocity variance is between Θ and Θ+dΘ is

nΘdΘ=πAh2exp−α−βΘdΘ.                                    (4.12)

From the total number of vehicles, nΘdΘ=N, πAexp−α/h2=βN, let Θe be the equilibrium velocity variance, which is also the average velocity variance under ideal condition. Then the total velocity variance is NΘe under an ideal condition. Therefore, ΘnΘdΘ=NΘe, that is, β=1/Θe. Finally, the number of vehicles with velocity variance Θ in an equilibrium section is

nΘ=NΘeexp−ΘΘe.                                           (4.13)

The traffic variable concerned in this study is density (k). Let the essential density K0 be Nπ/Θeh2, and then density becomes

kΘ=K0exp−ΘΘe.                                           (4.14)

Equation (4.14) describes that as velocity variance increases, density decreases, which implies that vehicles with large velocity variance can spread out easily. Also, when velocity variance increases, traffic flow will become unstable. However, in the real traffic condition, the mobile density does not spread out immediately as the velocity variance increases. There exists a threshold Θ0. When the velocity variance is larger than Θ0, the mobile density will be less than the essential density; otherwise the mobile density will be larger than the essential density. The statement means that if Θ−Θ0>0, vehicles can move away freely and the density in the interval will be less than the essential density. On the other hand, if Θ−Θ0<0, vehicles are trapped in the platoon and the density will be larger than the essential density. The modified model is

kΘ=K0exp−Θ−Θ0Θe.                                        (4.15)

Therefore, vehicles are distributed on the road by Eq. (4.15). In the traffic flow study, not all traffic conditions need to consider the velocity variance. Some simple cases can only consider density or density and velocity at the same time. For this reason, converting velocity variance to traffic potential is necessary. Since Θ=eφ, Eq. (4.15) becomes

kφ=K0expeψ−eφΘe,                                         (4.16)

where ψ is the potential equivalent of the velocity variance threshold Θ0; that is, ψ=Θ0/e. ψ is named as potential barrier here. Equation (4.16) gives the equilibrium distribution of density and implies several facts. The first one is that density decreases as traffic potential increases. The second one is as the equilibrium velocity variance increases, the variation of density increases, which means the traffic is sensitive. The third one is as the potential barrier is low, the mobile density is small. That is, drivers are aggressive. They can spread out easily although the velocity variance is small. Figure 4.3 illustrates the three aspects, where k1=K0expeψ1−eφ/Θe1, k2=K0expeψ1−eφ/Θe2 and k3=K0expeψ2−eφ/Θe1. Θe1>Θe2 and ψ1>ψ2. Coupling Eqs (4.4) to (4.16), the nonlinear dispersion model is given as

−Δφ=eεK0expeψ−eφΘe−ks+Ka.                              (4.17)

Solving the model with boundary conditions, the potential function is determined. Thus, the traffic field is obtained by Eq. (4.17) and density is obtained by Eq. (4.16). From the traffic field, we can observe how traffic field acts on vehicles, which is driving along the road. Also, the variation of velocity variance can be observed. With traffic force, velocity variance and density, the multilane traffic behavior can be described.

k3

k2

k1

Figure 4.3 The density curves of different potential barrier and equilibrium velocity variance.

## 4.3 Applications of the Dispersion Model

Solving the model with boundary conditions, the potential function is determined. From the traffic field, we can observe how traffic force acts on vehicles driving along the road. Also, the variation of velocity variance can be observed. With traffic force, velocity variance and density, the multilane traffic behavior can be described. Figure 4.4 is an example of a basic section of freeway (Cho and Lo, 2002b, 2002d), which is behind an on-ramp. The length of the arrow is the intensity of the traffic field. Vehicles tend toward the inside lane and the trend of the vehicles in the outside lane is stronger than that of the vehicles in the median lane. That is, there are more vehicles trying to change lanes in the front part of the outside lane. However, the lane-changing trend keeps a longer distance in the median lane. If the uninterrupted section of the freeway is long enough, the traffic flow will become uniform. The information is also useful to design the location of the interchanges of the freeway so as to optimize the traffic volume.

Figure 4.4 The traffic field of the numerical example.

The system can be extended to multi-class users model by employing the concept of Hoogendoorn and Bovy (1998, 2000). They considered that each class has different behavior, and share the same limited space. The concept can be applied to Poisson equation easily. If there are i classes of users or vehicles on the road, Eq. (4.17) is modified as Eq. (4.18) and the aggregated density is given by Eq. (4.19).

divE=−Δφ=ieiKi0εiexp−eiφ−eiψiΘie−eεks+Ka,                 (4.18)

kx,t=ieikix,t.                                             (4.19)

Equations (4.18)~(4.19) are very useful as vehicles, such as the motorcycles in Taiwan, are not restricted to move along a single lane. Figure 4.5 illustrates the mixed traffic flow of motorcycles and cars.

The most important application of the dispersion model is making a dynamic system self-consistent. A solving procedure of a self-consistent dynamic system is illustrated in Figure 4.6 (Cho and Lo, 2002c, 2002f). The concept of modeling can handle the different characteristics and distribute density on a road.

Figure 4.5 An example of the mixed traffic flow of motorcycles and cars.

Figure 4.6 Self-consistent traffic flow modeling.

## 4.4 Summary

A traffic dispersion model is proposed to describe the state of traffic flow in this chapter. The model is derived from the car-following theory and the equilibrium assumption. We propose the model to obtain the steady state multilane traffic information (density) without the microscopic analysis. The nonlinear dispersion model is developed to discuss the relation between density and traffic potential; that is, density and potential depend on each other. A traffic system will tend to its equilibrium state and it is the main assumption of the model. Generally speaking, Poisson equation is employed to describe the diffusive phenomena, such as heat and density, in physical researches. It is introduced into traffic flow research to explain the dispersion of vehicles in this study.

# CHAPTER 5

# Numerical Simulation of the Dispersion Model

In this chapter, the applications and the analyses of the dispersion model are discussed in detail. There are several important applications of the dispersion model. Firstly, the model is employed to describe the state at each time interval. As mentioned above, the multilane traffic model is a difficult problem. The dispersion model provides a simple modeling approach. Secondly, the dispersion model also implies the finite-space requirements. If there are multiple types of driving behaviors, the model can be employed to evaluate the aggregated influence of density. This advantage is useful when vehicles are not restricted to drive along one by one in a single lane. Another importance of the dispersion model is the model relates the basic variables in traffic flow and makes the systematic equations self-consistent. The analytical solution and analyses are presented in the following section firstly.

## 5.1 Analytical Solution of Linear Dispersion Model

The dispersion model needs to couple with a set of boundary conditions to make a model complete. Since vehicles move forward along a specific direction, the boundary condition of the model should be given in order to consist with the situation. If vehicles move from left to right, the left boundary condition must be larger than the right boundary condition. The reason is that vehicles moving along the direction of traffic field, which is directed from high potential to low potential. However, the traffic potential cannot be measured directly. The boundary conditions have to be transformed from density by Eq. (4.4). For example, if a model with a set of boundary conditions are given as follows:

Δφ(x,y)=−ek−ks/ε+Ka                                in Ω∂φ∂n(x,0)=∂φ∂n(x,Ly)=0                 0≤x≤Lx       on ∂Ω,                (5.1)

where Lx and Ly are the length of the research domain in the x- and y-direction. The boundary conditions mean that there is no entrance and exit on the both roadsides. In addition, inflow density, velocity, outflow density and velocity are controlled by specific function, such as signal or speed limited. Solving the following three sub-problems, we will obtained the solution of problem (5.1) by superposing the three results.

Δφ(x,y)=0                                                           in Ω∂φ∂n(x,0)=∂φ∂n(x,Ly)=0                0≤x≤Lx       on ∂Ω,                 (5.2)

Δφ(x,y)=0                                                            in Ω∂φ∂n(x,0)=∂φ∂n(x,Ly)=0                 0≤x≤Lx       on ∂Ω,                 (5.3)

Δφ(x,y)=−ek−ks/ε+Ka                                in Ω∂φ∂n(x,0)=∂φ∂n(x,Ly)=0                 0≤x≤Lx       on ∂Ω.                 (5.4)

The problems can be solved by separation of variables. Solutions of problems (5.2), (5.3) and (5.4) are shown as Eqs (5.5), (5.6) and (5.7), respectively.

φ(x,y)=B0(1−x)+n=1∞Bnsinhnπ(1−x)cos(nπy),                      (5.5)

where Bi, for i=0,1,…, are determined by the Fourier coefficients of g(y).

φ(x,y)=f0x+n=1∞sinhnπxsinh(nπ)cos(nπy),                                (5.6)

where f0 is determined by the Fourier coefficients of f(y).

φ(x,y)=n=1∞m=0∞−kmn(m2+n2)π2sinnπxcosmπy,                          (5.7)

where kmn, for m=0,1,…, n=0,1,…, are determined by the Fourier coefficients of ek−ks/ε+Ka.

Therefore, the general form of the analytic solution is shown as

φx,y=A0+i=1nAixn+i=1nBiyn+i=0nj=0m[Cisinhnπx/Lx+Djcoshmπy/Lx+Eisinnπx/Lx+Ficosnπy/Ly]sinnπx/Lx+cosnπy/Ly,       (5.8)

where  Ai, Bi, Ci, Di, Ei and Fi are coefficients, which are determined by the boundary conditions and the explicit form of density function. Also, the dispersion model can be solved by the numerical methods such as finite difference methods, finite elements and finite volume methods. Typical cases are illustrated as follows: The existence and uniqueness can be proved by the maximum principle. However, the nonlinear model can not be solved analytically. Therefore, a numerical scheme is presented in the following section.

## 5.2 Monotone Iterative Scheme of the Traffic Dispersion Model

Monotone iterative techniques are widely applied in solving nonlinear differential equation (Heikkiläand and Lakshmikantham, 1994, Ladde et al., 1985). Pao (1995) mentioned that the point method is not efficient computation in two or higher space dimension and proposed a block monotone iterative scheme for nonlinear elliptic equations. Since the dispersion model is a nonlinear elliptic equation, the block monotone iterative scheme is employed. In order to demonstrate the fundamental of monotone iterative method a simplified form of nonlinear dispersion model and the boundary condition are shown as follows:

Δφ=fx,y,φ,              in Ω,                       (5.9)

a∂φ/∂n+bφ=gx,y         on ∂Ω                           ,

where Ω is a two-dimensional rectangular domain Ω=0,Lx×0,Ly with boundary ∂Ω, where ∂φ/∂n is the outward normal derivative of φ on ∂Ω, a≡ax,y and b≡bx,y are nonnegative functions on ∂Ω with a+b>0, and f and g are prescribed nonlinear function in its domain. Let hx=Lx/Mx, hy=Ly/My and xi=ih, yj=jh for i=0,1,...,Mx, j=0,1,...,My, where Mx and My are the total number of intervals along the x and y directions, respectively. The set of points xi,yj in Ω and Ω=Ω∪∂Ω are denoted, respectively, by Λ and Λ. Define φi,j=φxi,yj, fi,jφi,j=fxi,yj,φxi,yj and gi,j=gxi,yj, then the standard finite difference approximation for the equations in (5.9) leads to a finite difference system in the form

aijφij−bijφi−1,j−cijφi+1,j−dijφi,j−1−eijφi,j+1=hkfijφij+gi,j, for all i,j∈Λ, (5.10)

where the coefficients aij, bij, etc. are associated with the coefficient of the discretized Laplace operator as well as the boundary coefficients a≡ax,y and b≡bx,y, gi,j is associated with the boundary functions of gi,j. In a more compact form

,                                               (5.11)

where A is a M×M matrix, where M=Mx+1My+1, FjΦj=−hxhyf0,jφ0,j,...,−hxhyfMx,jφMx,jT, Gj=−hxhyg0,j,...,−hxhygMx,jT and Φ=φ0,j,...,φMx,jT for j=0,1,2,…My. Denoting F and Φ by M, then FΦ≡−F0Φ0,...,−FMΦMT, G≡−G0,...,−GMTand Φ≡φ0,j,...,φMx,jT.

Typical choice of the coefficients in Eq. (5.10) can be easily derived if a finite difference approximation is specified. For the boundary points xi,yj where i=0, Mx and j=0, My. If the coefficients aij, bij, cij, dij, eij are associated with the boundary coefficients aij and bij, and possess the following property for the case aij>0.

aij≥bij+cij+dij+eij,                                            (5.12)

for i=0, Mx and j=0, My. If the mixed boundary condition holds, then strict inequality in the above relation holds for at least one i,j. In either case, these coefficients satisfy the condition:

aij≥0,bij≥0,cij≥0,dij≥0,eij≥0,                                  (5.13)

aij≥bij+cij+dij+eij,            for  all  i,j∈Λ.

Condition (5.13) is the fundamental hypothesis for our discussion of the monotone iterative method for the nonlinear algebraic system (5.11). It is clear from the matrix A in Eq. (5.11) and condition (5.13) that A is a M-matrix, i.e., A−1≥0 is a strictly diagonal dominant and invertible matrix provided that strict inequality in the relation aij≥bij+cij+dij+eij in condition (5.13) holds for at least one i,j and the domain Λ is connected. The monotone iterative method is based on the system (5.11) with A in the split form A=D−L−U, where D, L and U are block diagonal, lower triangular and upper triangular matrices of A respectively. In addition, we observe that D is a M-matrix, L≥0, and U≥0. A direct fact that equation (5.11) has at most a solution holds, if FjΦj∈C1, for 0≤j≤M and ∂FjΦj/∂φ≥0, −∞<φ<∞.

It is assumed that there is a function S0c such that

FjΦ1−FjΦ2≤S0cΦ1−Φ2,                                  (5.14)

for 0≤j≤M provided that Φ1,Φ2≤c. FjΦj is bounded uniformly if there exists a constant Z>0 such that FjΦj≤Z, Φ<∞, j=1,…,M. Let λ=S0c, where c0=Φ0 and Φ0 is the solution of AΦ0=Z+1E−G, where E=ej for 0≤j≤M. The following result is the well-known theorem and has been reported by Pao (1995).

Theorem 1 Let the functions FjΦ be bounded uniformly, and Φn+1, n=0, 1, 2,…, be the solution of

D+λIΦn+1=L+UΦn−FΦn−G+λΦn,                      (5.15)

then, there exists a solution of Eq. (5.11).

As discussion above, let the coefficient ξi,j associated with the Laplace operator in Eq. (5.9) satisfies the condition (5.13), i.e., ξi,j≥0 for all i,j∈Λ and ξi,j≥ξi+1,j+ξi−1,j+ξi,j+1+ξi,j−1. As last, when using the corresponding finite difference and monotone iterative methods for the nonlinear Poisson equation, we obtain the following finite difference system, i.e.,

φi,jξi,j=e/εξi+1,jφi+1,j+ξi−1,jφi−1,j+ξi,j+1φ1,j+1+ξi,j−1φi,j−1+K0expeψi,j−φi,j/Θe−ksi,j+Kai,j                (5.16)

is the finite difference system corresponding to the nonlinear Poisson equation, and

φi,jn=e/ελ+ξi,jξi+1,jφi+1,jn+ξi−1,jφi−1,jn+ξi,j+1φ1,j+1n+ξi,j−1φi,j−1n+K0expeψi,j−φi,jn/Θe−ksi,j+λφi,jn+Kai,j/λ+ξi,j           (5.17)

is the associated monotone iterative equations where λ=∂f/∂φ for all x,y,φ∈Ω×infφ,supφ, where φ and φ is so-called a subsolution and supersolution respectively.

Theorem 2 If a monotone iterative number λ is chosen, and the coefficients ξi,j satisfy the condition (5.13), then sequences φi,jnn=0∞ which is generated by Eq. (5.17) converge to a solution φi,j of Eq. (5.16) monotonically.

Theorem 1 and 2 give us the existence and the uniqueness of solution of the monotone iterative scheme for our nonlinear dispersion model. The implementation of the monotone iterative method is illustrated in Figure 5.1. The procedure is described as follows.

Step 1 initial guess.

Step 2 set Error bound and stop criterion.

Step 3 set iterative number and node.

Step 4 calculate the monotone number.

Step 5 compute the present solution.

Step 6 check the stop criteria: if yes, then stop; if no, then goto Step 4.

Step 7 stop.

Furthermore, the initial guess of the iteration is determined by the physical meaning of the model and the boundary conditions in general. Fortunately, the nonlinear dispersion model is a monotonic function, and then any arbitrarily initial guess will converge to the unique solution. A numerical example is employed to explain the nonlinear dispersion model and to show the convergence of the scheme in the following section.

Figure 5.1 Flowchart of monotone iterative method.

## 5.3 Numerical Results and Discussions

In this section, the numerical results of the numerical scheme for the traffic dispersion model are presented. In order to describe the traffic situation, the model and the boundary conditions (see Appendix B) should be considered together. Since the boundary conditions of potential can’t be obtained directly, we have to transform the boundary conditions of density into the boundary conditions of potential by Eq. (4.17). Therefore, the boundary conditions of density in traffic flow problems have to be discussed first. If the traffic condition on a boundary can be described by a deterministic function, a Dirichlet condition is chosen, such as deterministic inflow or outflow (kx,y=fy) and no entrance and exit on the roadsides (kx,y=0). If we only have the changing rate of density on a boundary, a Neumann condition is chosen. The situation takes place behind the on-ramp, the signal intersection and the toll collection station where the service rate is easily obtained. Mostly, a Robin condition mixed by both Dirichlet and Neumann condition is employed. A traffic flow problem often involves two or three types of boundary conditions, which is a mixed boundary condition problem. A boundary condition of density can be corresponded to a boundary condition of potential when it is formulated.

The numerical example in this study considers a basic section of freeway, which is behind an on-ramp. The research domain is a three-lane section with 1 km in length. Under the uncongested traffic, the boundary conditions are given as follows:

∂φx,0/∂n=∂φx,3/∂n=0,φ0,y−∂φ0,y/∂n=4−2×Θe/e×lnks0,y/K0,φ1,y=−Θe/e×lnks1,y/K0,   x∈∂Ωy∈∂Ωy∈∂Ω,       (5.18)

where the passenger car equivalent e=1, the equilibrium velocity variance Θe=25 km2/h2 and the K0=3, which is about 108 passenger car unit per kilometer per lane (pcu/km⋅lane) in the original problem (before transforming to the conservative field). Assume the interacting parameter ε=10 and the domain is ideal in geometry, then Ka=0. The unrestrained density ks and the research domain are given as Figure 5.2. The first condition in Eq. (5.18) means that there is no entrance and exit on both roadsides; that is, k=0, which implies ∇φ=0. The second condition is obtained from the Robin condition of density, which means the inflow depends on the existent density and the inflow density. The third condition means that the outflow is determined by the unrestrained density. Through the boundary conditions, traffic flow is forced to move forward and the mobility of the vehicles is blocked by the density (platoon of vehicles). The unrestrained density is assumed to describe that the inside lane makes more vehicles pass through than outside lanes, since the outside lanes are disturbed by the inflow of the on-ramp.

In one-dimensional space, the movement of traffic flow is restricted to forward moving and backward moving. The dispersive effect of traffic flow is hardly observed. Thus, we only discuss the multilane case in this section. The numerical results are illustrated in Figures 5.3 to 5.5. In the figures, each interval is 20m in x-direction and 0.4m in y-direction. 0≤y≤10 is the outside lane; 10≤y≤20 is the median lane; 20≤y≤30 is the inside lane. By solving the model, traffic potential is obtained. The traffic field is derived from the traffic potential after it is computed. The traffic field is illustrated in Figure 5.4, which implies the following behavior. Vehicles tend toward the inside lane and the trend of the vehicles in the outside lane is stronger than that of the vehicles in the median lane. That is, there are more vehicles trying to change lanes in the front part of the outside lane. However, the lane-changing trend keeps a longer distance in the median lane. At last, the traffic flow will become uniform. This case is an uncongested flow, which means that if the traffic is disturbed, the density will decrease. Figure 5.5 shows the density distributed on the road. Since the front part (left hand side) of the domain is disturbed by the inflow of the on-ramp, the density there is lower than the rear part (right hand side) of the domain. Behind the merging area, the traffic flow becomes uniform. This information can help to design the on-ramp area (the length of the auxiliary lane). Sup norm error is chosen to check the convergence. The numerical result supports the conclusions of Theorem 1 and 2 that the monotone iterative scheme converges to the unique solution. This fact is inferred in Figure 5.6, which also illustrates the speed of convergence is very fast.

Figure 5.2 The research domain and the unstrained density (ks).

Figure 5.3 The potential of the numerical example. Each interval is 20m in x-direction and 0.4m in y-direction. 0≤y≤10 is the outside lane; 10≤y≤20 is the median lane; 20≤y≤30 is the inside lane.

inside lane

median lane

outside lane

x-direction

Figure 5.4 The traffic field of the numerical example

Figure 5.5 The density of the numerical example.

Figure 5.6 Plot of the convergence (log(error108) v.s. number of iterations).

## 5.4 Summary

In this chapter, a numerical scheme for multilane traffic dispersion model is presented to solve the traffic dispersion model, which is a nonlinear Poisson equation. The monotone iterative scheme is firstly employed to solve the traffic problem. If the model is monotone, it will converge rapidly. According to the two theorems mentioned previously, the numerical approximation will converge to the exact solution. A numerical example is employed to explain the multilane traffic by the model, and it also shows the convergence of the monotone iterative scheme is fast. Scenarios of boundary conditions should be formulated according to different traffic situations. The model developed herein is a stationary dispersion model. The time dependent dispersion model is left for further researches.

# CHAPTER 6

# Modeling of Self-Consistent Vehicular-Kinetic Flow

Since Lighthill, Whitham (1955) and Richards (1956) firstly proposed their kinetic model, the related subjects are broadly researched and debated. The LWR model was extended to second order model, which includes the continuity equation and a phenomenological velocity equation. The second order model was named PW model. However, this kind of models has a lot of arguments, so families of gas-kinetic models are presented. The development of gas-kinetic models includes the discussion of multilane, multiclass users and overtaking, lane-changing, relaxation and interaction maneuvers. As the review of Boltzmann equation, Boltzmann equation is a phase-plane distribution. The macroscopic quantities are derived as follows. The first step is multiplying Boltzmann equation by the moment functions. The second step is integrating both sides of the equations and using the chain rules. At last, the macroscopic quantities are obtained. Therefore, the resulting macroscopic quantity and the moment function must have physical meanings. From the previous researches, the second order moment function multiplied to Boltzmann equation is the square of individual velocity. Nevertheless, the square of individual velocity, which denotes the individual kinetic energy in physics, is meaningless in traffic. Although the second order expansion results in the velocity variance equation, additional terms may be generated. For this reason, we multiply Boltzmann equation by the individual velocity variance in order to modify the derivation of velocity variance equation herein.

A complete dynamic system should include motion equations and state equations. The state equation considered in this study is the traffic dispersion model, which is presented in Chapter four. By coupling the dispersion model to the kinetic system, a self-consistent system is obtained. The derivation of the self-consistent traffic flow model is started with the definitions of the variables in this chapter.

## 6.1 Definitions

By reason of some driving behavior of road users cannot be restricted in one lane or even they derive in one lane they still not be restricted to drive one after one, such as, driving behavior of motorcycles. Therefore, a multilane highway is regarded as a two-dimensional space in this study. We assume that there exists a phase-plane distribution function fx,v,t at a given time and at a given point, where x=x,y denotes position, v=vx,v denotes individual velocity and t denotes time. Since v denotes individual velocity, it is impossible to restrict a specific velocity at a specific time and place. Thus, v is independent to position x and t, i.e. ∇xv=0, ∇x⋅v=0, ∂v/∂t=0.

In addition, dx/dt=v and dv/dt=eE, where E denotes traffic field and has been derived in detail in Chapter 4. By Taylor’s expansion or total derivative of f, the changing of f is shown by

dfx,v,tdt=∂fx,v,t∂t+v⋅∇xfx,v,t+eE⋅∇vfx,v,t=∂fx,v,t∂tcoll,  (6.1)

fx,v,t|∂Ωv=0,                                                 (6.2)

where f is defined on Ω and ∂Ω is the boundary of Ω. ∂Ωv is the boundary of individual velocity. Since f is a distribution function, it is reasonable to assume that f is equal to 0 at the extreme value (i.e. boundary ∂Ωv). Thus, density is given by

kx,t=fx,v,tdv                                             (6.3)

and flow density, which is defined by Cho and Lo (2002b), is given by

qx,t=vfx,v,tdv=kx,tux,t,                                 (6.4)

where ux,t denotes average velocity (or so-called group velocity), which is defined as

ux,t=vfx,v,tdvfx,v,tdv.                                          (6.5)

Flow is denoted by Qx,t=qx,tdy, where y is the width of the road.

Next, three kinds of velocity variance are defined. The first one is total velocity variance, which is velocity variance between individual velocity and equilibrium velocity. Total velocity variance is defined by

Θx,t=v−ue2fx,v,tdvfx,v,t.                                     (6.6)

The second one is individual velocity variance, which is the velocity variance between individual velocity and group velocity. Individual velocity variance is given by

θax,t=v−ux,t2fx,v,tdvfx,v,t.                                 (6.7)

The last one is group velocity variance, which is the velocity variance between group velocity and equilibrium velocity. Group velocity variance is given by

θex,t=ux,t−ue2fx,v,tdvfx,v,t=ux,t−ue2fx,v,tdvfx,v,t.         (6.8)

If traffic flow is uniform, individual velocity is equal to average velocity θax,t=0. If traffic flow is equilibrium, average velocity is equal to equilibrium velocity θex,t=0. The relationship among three different velocity variances is

Θx,t=v−ux,t+ux,t−ue2fx,v,tdvfx,v,tθax,t+θex,t,                         (6.9)

since 2v−ux,t⋅ux,t−uekfx,v,tdv=0.

That implies: (i) total velocity variance is equal to the summation of individual and group velocity variance, (ii) Θ≥θa≥0 and Θ≥θe≥0, (iii) Θ=0 implies θa=θe=0.

Furthermore, equilibrium average velocity and equilibrium average variance are denoted by uek,u,Θ and Θek,u,Θ, respectively. An equilibrium state is defined as the most possible microscopic state under a specific macroscopic state. Since ue depends upon k, u and Θ only, ∇xue=0, ∇x⋅ue=0, ∂ue/∂t=0.

Another variable, which appears in the derivation, is skewness. It is defined by

Γx,t=v−ukv−uk2fx,v,tdvfx,v,t,                           (6.10)

which is the bias of the distribution. From the empirical study by Helbing (1996, 1997b), Γx,t≈0. Since the influences of the third or higher order moment functions are negligible, we do not have to expand the higher order conservation laws. The basic variables k, Q, Θ, θa and θe are scalar and q, u, Γx,t are vectors.

The basic idea of deriving macroscopic models from Boltzmann equation is the same as finding the expectation of a random variable. Therefore, finding the individual variables that are meaningful and multiplying them to distribution f will obtain macroscopic variables (average or group quantities). The individual variables are named as moment functions and denoted as χx,v,t. The moment functions chosen by related researches are 1, v, and v2. However, v2 doesn’t have physical meaning in traffic. For this reason, the moment functions chosen herein are 1, v, and v−ue2, where v−ue2 is individual velocity variance. Thus, multiplying Eq. (6.1) by χx,v,t, we have

∂fx,v,t∂tχx,v,t+v⋅∇xfx,v,tχx,v,t+eE⋅∇vfx,v,tχx,v,t∂fx,v,t∂tcollχx,v,t.  (6.11)

The integration form of Eq. (6.11) is illustrated as

dfx,v,tdtχx,v,tdv∂fx,v,t∂tχx,v,tdv+v⋅∇xfx,v,tχx,v,tdv+eE⋅∇vfx,v,tχx,v,tdv∂fx,v,t∂tcollχx,v,tdv. (6.12)

By substituting 1, v, and v−ue2 for χx,v,t in Eq. (6.12), the macroscopic system will be obtained. The derivation is shown from Section 6.2 to 6.4.

## 6.2 Continuity Equation (Conservation of Vehicle Numbers)

After introducing traffic field and Poisson equation, derivation from Boltzmann equation to the macroscopic system is presented. Firstly, let χ=1, so Eq. (6.12) becomes Eq. (6.13), which is expectation of density.

∂fx,v,t∂tdv+v⋅∇xfx,v,tdv+eE⋅∇vfx,v,tdv=∂fx,v,t∂tcolldv.  (6.13)

Each term in Eq. (6.13) is discussed separately; the first term of left hand side (LHS) can be obtained from the definition (6.3):

∂fx,v,t∂tdv=∂∂tfx,v,tdv=∂kx,t∂t.                         (6.14)

Since individual velocity is independent of position and Eq. (6.4), the second term of LHS is given by

v⋅∇xfx,v,tdv=∇x⋅fv−f⋅∇xvdv=∇x⋅qx,t.               (6.15)

From Eq. (6.2), the third term of LHS is represented as

eE⋅∇vfx,v,tdv=eE⋅∇vfx,v,tdveE⋅fx,v,t|∂Ωv0.                         (6.16)

At last, the collision term is assumed to be equal to zero. Thus right hand side (RHS) of Eq. (6.13) is given by

∂fx,v,t∂tcolldv=0.                                          (6.17)

Therefore, Eqs (6.14), (6.15), (6.16), and (6.17) give the first conservation law; that is, conservation of vehicle numbers.

∂kx,t∂t+∇x⋅qx,t=0.                                         (6.18)

## 6.3 Motion Equation (Conservation of Momentum)

The motion equation is derived by substituting the first order moment function, v, for χ in Eq. (6.12). Then the expectation function of velocity is illustrated as

∂fx,v,t∂tvdv+v⋅∇xfx,v,tvdv+eE⋅∇vfx,v,tvdv∂fx,v,t∂tcollvdv.         (6.19)

Each term in Eq. (6.19) is also discussed separately; the first term of LHS is obtained

∂fx,v,t∂tvvdv=∂∂tfx,v,tvvdv−fx,v,t∂∂tvvdv∂ku∂t.       (6.20)

By vector analysis (see Appendix D), Eqs (6.21) and (6.22) are true,

v⋅∇fx,v,t=∇⋅fv−f⋅∇v=∇⋅fv,                          (6.21)

v⋅∇fx,v,tvv=∇⋅fvv=∇⋅fvv−fv⋅∇v=∇⋅fvv.                     (6.22)

Thus,

v⋅∇xfx,v,tvvdv=∇x⋅fvvdv∇x⋅fvvdv,                           (6.23)

where

vv=v−u+u−ue+uev−u+u−ue+uev−uu−ue+v−uue+u−uev−u+uev−u+uu+v−uv−u            (6.24)

is a tensor. With fv−uu−uedv=ku−uu−ue=0 and fv−uuedv =ku−uue=0. Equation (6.23) becomes

v⋅∇xfx,v,tvvdv=∇x⋅fvvdv∇x⋅fv−uv−udv+fuudv.          (6.25)

Assume the influence of velocity is dominated by the component of the same direction, i.e. vx−ux2ii>>vx−uxvy−uyij and vy−uy2jj>>vx−uxvy−uyji. Define fv−uv−udv≈fvv−uuIdv≡kθa, where I is the 22 identity matrix. The summation of diagonal of θa is equal to θa, i.e. trθa=θa.

Since fuudv=uufdv=kuu, the second term of LHS becomes

v⋅∇xfx,v,tvvdv=∇x⋅kθa+kuu.                          (6.26)

The third term of LHS is computed by

eE⋅∇vfx,v,tvdv=eE⋅∇vfvdv−eE⋅f∇vvdveE⋅fv|∂Ωv−eE⋅fdv=−ekE.              (6.27)

The RHS of Eq. (6.19) is assumed to satisfy the relaxation time approximation, i.e.

∂fx,v,t∂tcollvdv=∂∂tkucoll=−ku−kueτm,                      (6.28)

where τm is the velocity relaxation time. Equation (6.28) means that if the average velocity is not equal to the equilibrium velocity, it will become the equilibrium velocity gradually after a period of time τmk. Therefore, Eq. (6.19) becomes

∂ku∂t+∇x⋅kθa+kuu=ekE−ku−kueτm.                          (6.29)

From Eq. (6.28), Eq. (6.29) is represented as

∂u∂t+u∇x⋅u=−1k∇x⋅kθa+eE−u−ueτm,                         (6.30)

if k0. Thus, the conservation of momentum is obtained, which describes the changing of group velocity. Therefore, Eq. (6.30) also is known as a motion equation.

From Eq. (6.29), the explicit form of flow density can be derived as follows:

q=ku=−τmk∂u∂t−τmku∇x⋅u−τm∇x⋅kθa+τmeE+kue.            (6.31)

Under steady state and homogeneous velocity assumption, it’s reasonable to assume that u∝ue. Let ku−kue=ςku and μ=τm/ς. Equation (6.31) becomes

q=−μ∇x⋅kθa+ekμE=ekμE−ν⋅∇xk,                           (6.32)

where average velocity variance matrix equals to the equilibrium velocity variance (θa=Θe), and ν=μΘe. The first term of Eq. (6.32) is called the drift term, which is induced by velocity (or field). The second term of Eq. (6.32) is called the diffusion term, which is induced by distribution of density. Equation (6.32) is the same as the fundamental diagram with diffusion effect q=ekudrift−ν⋅∇xk, where udrit=μE =τmE/ς. The result can also be derived from Fick’s law. Eq. (6.32) can be employed when velocity and velocity variance are homogeneous and stationary. Generally, Eq. (6.30) is employed as the explicit function of flow density.

## 6.4 Variance Equation (Conservation of Energy)

The last equation considered herein is the variance equation, which is obtained by substituting the second moment function, v−ue2, for χ in Eq. (6.13). Then the expectation function of velocity variance is illustrated as

∂fx,v,t∂tv−ue2dv+v⋅∇xfx,v,tv−ue2dv+eE⋅∇vfx,v,tv−ue2dv∂fx,v,t∂tcollv−ue2dv. (6.33)

Also, the derivation is done by each term of Eq. (6.33). The first term of LHS is

∂fx,v,t∂tv−ue2dv=∂∂tfv−ue2dv−f∂∂tv−ue2dv∂kΘ∂t.       (6.34)

The second term of LHS is

v⋅∇xfv−ue2dv=∇x⋅fvv−u2+2v−u⋅u−ue+u−ue2dv−fv⋅∇xv−ue2dv∇x⋅kΓ+kθau+2ku−ueθa+kθeu∇x⋅kΘu+2kθa⋅u−ue,   (6.35)

since fuv−u⋅u−uedv=fv−udv⋅u−ueu=0, Γ≈0, and  fv⋅∇xv−ue2dv=0 as mentioned before.

The third term of LHS is

eE⋅∇vfx,v,tv−ue2dv=eE⋅∇vfv−ue2dv−eE⋅f∇vv−ue2dveE⋅fv−ue2|∂Ωv−eE⋅f2v−2uedv=−2eE⋅ku−ue.  (6.36.)

The last term is the RHS of Eq. (6.33), which is also assumed to satisfy the relaxation time approximation, i.e.,

∂fx,v,t∂tcollv−ue2dv=∂∂tkΘcoll−fx,v,tcoll∂∂tv−ue2dv−kΘ−kΘeτe, (6.37)

where τe is the relaxation time of velocity variance. Therefore,

∂kΘ∂t+∇x⋅kΘu+2ku−ueθa=−2ekE⋅u−ue−kΘ−kΘeτe        (6.38)

is obtained. From Eq. (6.18), Eq. (6.38) is represented as

∂Θ∂t+u⋅∇xΘ=−2eE⋅u−ue−Θ−Θeτe−2k∇x⋅ku−ueθa,           (6.39)

if k0. Thus, the conservation of energy is obtained, which describes the changing of group velocity variance.

Other conservation laws can be derived by multiplying higher order moment functions to Boltzmann equation as long as the moment functions are meaningful and the macroscopic quantities obtained are significant. As mentioned by Helbing (1996, 1997), Boltzmann equation multiplied by the third order moment function produces the expectation function of skewness, which is near zero. Therefore, higher order moment functions are not discussed in this study.

The system equations developed above includes three conservation laws derived from Boltzmann equation. These four equations are transient equations, which describe the changing of variables. However, a complete dynamic system not only needs transient equations, but also needs state equations, which describe the state of variables. A state equation is needed so as to make the system self-consistent. In this study, the dynamic system is assumed to become the equilibrium state gradually. Therefore, the traffic dispersion model proposed in Chapter 4 is employed to be the state equation in the system. Hence, all variables of the system can be solved by the systematic equations.

## 6.5 Closure Relations

The system presented herein also needs closure relations so as to determine the equilibrium velocity uek,u,Θ, equilibrium variance Θek,u,Θ, and relaxation time τm and τe in Eqs (6.29) and (6.38). There are a variety of possible closure relations, which could be adopted from previous studies. (Payne, 1979, Kerner and Kohnhäuser, 1993, 1994, Phillips, 1977, Hoogendoorn and Bovy, 2001, Helbing, 2001). The ue and Θe proposed in study are represented by

uek,u,Θ=u0−τmpbkkΘ                                      (6.40)

and

Θek,u,Θ=τeppk⋅kΘu,                                       (6.41)

respectively. u0 is the average desired velocity, pbk∈0,1 is the braking probability vector, and ppk∈0,1 is the passing probability vector. The explicit forms are obtained by specifying expressions for pbk and ppk. Equation (6.40) means that the equilibrium velocity decreases as pbk increases. On the other hand, the equilibrium variance increases as ppk increases. Since uek and Θek,u are equilibrium equations, the functions suggested in this study are

uek=vfex,v,tdvfex,v,tdv                                           (6.42)

and

Θek=v−ue2fex,v,tdvfex,v,tdv                                     (6.43)

respectively. fe denotes the steady state homogeneous solution of Boltzmann equation. The relaxation time τm and τe are shown as

τmk=Tmgk   and   τek=Tegk,                              (6.44)

which are modified from the suggestion of Helbing (1997). g(k) is the proportion of freely moving vehicles, Tm is the reaction time of velocity and Te is the reaction time of variance. As fe, g(k) Tm and Te are determined, the closure relations are expressed specifically. Then, the self-consistent system is complete.

## 6.6 Multiclass users model

Dynamic multilane traffic flow model proposed in the previous studies includes three conservation laws, which control the motion of vehicles, and a nonlinear Poisson equation, which distribute the density on the road. The set of equations can only describe one driving behavior. Fortunately, the system can be extended to multi-class users model by employing the concept of Hoogendoorn and Bovy (1998, 2000, 2001). They considered that:

(i) each class has different behavior and is described by different conservation laws.

(ii) space of a road section is limited and all class of users share the space.

The first assumption is easy to achieve. According to different driving behavior, a specific Boltzmann equation is derived. With the same procedure mentioned above, the macroscopic kinetic system of each class of user is derived from the specific Boltzmann equation. The concept of Hoogendoorn and Bovy’s second assumption is the same as Poisson equation of our system. Therefore, the traffic dispersion model for multi-class users proposed in Chapter 4 can be employed herein. If there are i classes of users or vehicles on the road, the traffic dispersion model for multi-class users Eq. (4.18) is shown as

divE=−Δφ=ieiKi0εiexp−eiφ−eiψiΘie−eεks+Ka,                 (4.18)

The other equations in the multi-class users’ system are illustrated as follows:

qix,t=kix,tuix,t,                                           (6.47)

∂kix,t∂t+∇x⋅qix,t=0,                                        (6.48)

∂ui∂t+ui∇x⋅ui=−1kc∇x⋅kiθia−ei∇φ−ui−uieτim,                  (6.49)

∂Θi∂t+ui⋅∇xΘi=2ei∇φ⋅ui−uie−Θi−Θieτie−2ki∇x⋅kiui−uieθia,   (6.50)

ki=Ki0exp−eiφ−eiψiΘie.                                       (6.51)

The system equations above mean that each class of user is controlled by his own conservation laws. By coupling with the Poisson equation, a self-consistent multi-class users dynamic traffic flow model is obtained. Total flow density and total density are represented as

qx,t=ieiqix,t=ieikix,tuix,t                             (6.52)

and

kx,t=ieikix,t,                                             (6.53)

respectively.

## 6.7 Summary

In this chapter, we have derived a macroscopic multilane traffic model for multiple classes users. The system is a self-consistent system; it can be solved with proper initial conditions and boundary conditions. Our consideration is based on the following assumptions:

(i) a multilane road is considered as a two-dimensional domain.

(ii) the whole system will tend toward equilibrium.

(iii) each class of user has different behavior and is described by different conservation laws (the gas-kinetic model).

(iv) the individual velocity variance is employed as the second moment function.

(v) the space of a road section is limited and all classes of users share the space.

Considering a multilane road as a two-dimensional domain allows us to handle the driving behaviors, which are not restricted to drive one by one in a single lane. Another advantage of this consideration is to avoid modeling complicated lane-changing behavior. Lane-changing behavior is controlled by the nonlinear Poisson equation. If the research area is only a single lane road, the system can be reduced to a one-dimensional model.

The dynamic traffic flow mode is derived from Boltzmann equation and the Boltzmann equation employed herein includes accelerated effect, which is controlled by Poisson equation. Helbing, and Hoogendoorn and Bovy followed the study of Paveri-Fontana. Their methodology is multiplying Boltzmann equation by 1, v and v2 so as to derive macroscopic systems. However, the second order moment v2 does not make sense in traffic flow. Although multiplying Boltzmann equation by v2 can obtain velocity variance equation, it may also generate some meaningless terms. This study modifies the second moment function as v−ue2, which is individual velocity variance, and reformulates velocity variance equation to be more reasonable. This study exposes three moment functions. If the influence of the other meaningful moment is significant, the moment should be considered.

In addition, the Poisson equation plays an important role in the system. Since Poisson equation distributes vehicles on a road, it becomes the key point to extend the system to multiclass user traffic. Besides, the equilibrium distribution is employed to determine the state of traffic flow. Thus, the system is self-consistent. The solving procedure of the self-consistent system is illustrated as Figure 4.6.

The system equations and the relationship among them are illustrated in Figure 6.1. From the figure, the system equations can be simplified to adapt different traffic condition because not all variables are significant in each traffic condition. For example, in uniform and equilibrium traffic flow, the influence of velocity variance equation may be ignored. The simplification is needed because computation of the whole system takes a lot of time. In the following content of this dissertation, a simplification that is named as drift-diffusion model, is presented. Also, numerical method for the model is developed.

Figure 6.1 Modeling architecture and the relationship between models.

# CHAPTER 7

# Numerical Simulation of Steady State Drift-Diffusion Traffic Flow Model

The whole self-consistent vehicular kinetic model includes five partial differential equations. It’s a complicated system to solve. Therefore, a simplified model is presented first. After that, the numerical scheme of the model is derived under the steady state. Numerical simulation of the time variant model will be proposed in the next chapter.

## 7.1 Drift-Diffusion Model

According to Figure 6.1, the simplest model in the architecture is the drift-diffusion model. The model is named as the drift-diffusion model because the fundamental diagram (6.32) is employed. In addition, the drift-diffusion model includes the continuity equation and the nonlinear dispersion model; i.e.,

−Δφ=eεK0expeψ−eφΘe−ks+Ka.                              (4.17)

∂kx,t∂t+∇x⋅qx,t=0.                                         (6.18)

The continuity equation is derived from the conservation of vehicle numbers or the zeroth moment expansion of Boltzmann equation and describes the transient of density. The equation is a simple but sufficient traffic theory if velocity and variance are time invariant. The second equation is the dispersion model, which describes the interaction among vehicles and the distribution of density. The explicit form of the fundamental diagram is employed under steady state and homogeneous assumption of velocity and variance; i.e.,

q=−μ∇x⋅kθa+ekμE=ekμE−ν⋅∇xk,                           (6.32)

In this chapter, we discuss the model under the steady state; i.e., ∂k/∂t=0 in Eq. (6.18). Coupling Eq (6.32) to (6.18), we can rewrite the system as follows.

∇x⋅−ekμ∇xφ−ν⋅∇xk=0,                                       (7.1)

−Δφ=eεK0expeψ−eφΘe−ks+Ka.

After showing the model, the numerical algorithm is illustrated in the following section.

## 7.2 Decoupling Scheme

Because there are two equations in the model, we present a decoupling algorithm first in this section. That is, the Poisson equation and the continuity equation are solved separately. After the both equations converge, the convergence of the whole system is examined. If the system converges, then the algorithm stops to compute the flow and velocity. Decoupling scheme is one of efficient solution methods in simulation (Selberherr, 1984). The method decouples the system as two independent partial differential equations and then solves each partial differential equation iteratively. The basic idea of decoupling method is that the decoupled equations are solved sequentially, which is illustrated in Figure 7.1. If we denote φn as the approximated potential of the nth outer loop iteration. In addition, kn denotes the nth outer loop iteration.

Solving procedure of the decoupling scheme is shown as follows. Firstly, Poisson’s equation is solved for φn by given the previous states kn−1. Secondly, the continuity equation is solved for kn by given φn. Then, the convergent criterion of outer loop is examined. Sup norm error is the chosen convergent criterion in this study. Each decoupled partial differential equation is solved by the inner iterative loop. The scheme of inner loop is presented in the next section.

Figure 7.1 A flowchart for decoupling scheme in traffic flow simulation.

## 7.3 Discretization of the Model

In this section, the continuity equation is discretized by the Scharfetter-Gummel scheme (Selberherr, 1984, Markowich, 1986), whereas the monotone iterative scheme of the nonlinear dispersion model is presented. The numerical scheme of drift-diffusion model is derived in one-dimensional space first. The two-dimensional scheme can be derived by the same manner. The scheme of the nonlinear Poisson equation is already obtained in Chapter 5. The discretization of the continuity equation is much more crucial. First we write the approximation of flow as

qx∈xi,xi+1=q|i+1/2+x−xi−hi2⋅∂∂xq|i+1/2+Oh2∂2∂x2q|i+1/2.                           (7.2)

We obtain the approximation by ignoring the Oh2 terms. For the interval xi,xi+1 we have

μk∂φ∂x−ν∂k∂x=q|i+1/2+x−xi−hi2⋅∂∂xq|i+1/2.                            (7.3)

This equation is solved to determine the variation of the density along the path xi,xi+1. We have to assume that the partial derivative of the traffic potential is constant on the path under consideration, which is the assumption we have already invoked for the Poisson equation. Therefore, Eq. (7.3) can be considered as a first order ordinary differential equation of density k. The solution is

kx∈xi,xi+1=Cexp−eφxΘe+hiq|i+1/2μ1−exp−eφxΘeeφi+1−φi+hi2Θeφi+1−φ1−exp−eφxΘee2φi+1−φi∂∂xq|i+1/2μ.                     (7.4)

Assuming that φi+1−φi=Oh, shows that the last term in Eq. (7.4) is Oh3. By ignoring this term, we obtain

kx∈xi,xi+1=1−gik,φki+gik,φki+1.                                (7.5)

with

gik,φ=1−expeφi−eφi+1Θex−xihi1−expeφi−eφi+1Θe.                                     (7.6)

If φi+1=φi, the growth function (7.6) degenerates to a linear function.

gik,φ=x−xihi.                                                 (7.7)

Therefore, the discretized scheme of continuity equation is represented as

qi+1/2−qi−1/2hi+hi−1/22hi+hi−1[μeki+11+expφi−φi+12Θe/e+ki1+expφi+1−φi2Θe/eφi+1−φihi+μeki−11+expφi−φi+12Θe/e+ki1+expφi−1−φi2Θe/eφi−1−φihi−1−νφi+1−φiΘe/eexpφi−φi+12Θe/e−expφi+1−φi2Θe/eki+1−kihi−νφi−1−φiΘe/eexpφi−1−φi2Θe/e−expφi−φi−12Θe/eki−1−kihi−1]0.        (7.8)

Since density is function of potential, the approximation ki is

ki=K0iexpψi−φiΘe/e                                              (7.9)

By substituting Eq. (7.9) for ki in Eq. (7.8), we can derive the iterative scheme. In this scheme, a finite difference approximation with exponentially fit the dependent variables is employed for the continuity equation. Because traffic potential is expected to change smoothly, whereas density may change dramatically. In addition, the two-dimensional continuity equation can be discretized by the same manner.

## 7.4 Numerical Results and Discussions

In this section, the numerical results of the numerical scheme for the steady state traffic drift-diffusion model are presented. The first numerical example in this study is a one-dimensional problem. It considers a basic section of highway, which is behind an intersection. The research domain is a single lane section with 1 km in length. Under the uncongested traffic, the boundary conditions are given as follows:

12φ0−∂φ∂n=3−Θeelnks0K0φ1=−Θee×lnksK0k0−13∂k∂n=50,                                 (7.10)

where the passenger car equivalent e=1, the equilibrium velocity variance Θe=16 km2/h2 and the K0=100 pcu/km⋅lane. Assume the interacting parameter ε=3,600 and the domain is ideal in geometry then Ka=0. The first condition in Eq. (7.10) means that vehicles, which is going to enter the interval is resisted by the existent vehicles. In addition, vehicles in the interval are forced to drive forward by an imaginary external potential. The second condition means the outflow depends on the existent density. The third one means that the density of inflow is determined by the inflow density from the front interval. The last condition gives that the outflow density is uniform. Through the boundary conditions, traffic flow is forced to move forward and the mobility of the vehicles is blocked by the density (platoon of vehicles). The unstrained density is assumed to describe that the front part of the research interval is disturbed by the inflow vehicles from different direction. Gradually, it gets back to the average value. The unrestrained density ks is assumed to be

ks=30+0.125∗x,x∈0,0.8.                                     (7.11)

The numerical results are illustrated in Figures 7.2 to 7.5. Traffic potential decays, whereas density increases in the front part of the interval and then decreases. It means that the interaction among vehicles in the front part is large and forms a platoon. This results in the increase of the density and decrease of the velocity, which is shown in Figure 7.5. Velocity, which is obtained from u=q/k, decreases firstly and then increases. Flow rate is illustrated in Figure 7.4. It is almost a constant. The result is reasonable, since the model solved herein is under steady state.

Figure 7.2 The plots of potential in one-dimensional numerical example.

Figure 7.3 The plots of density in one-dimensional numerical example.

Figure 7.4 The plots of flow in one-dimensional numerical example.

Figure 7.5 The plots of velocity in one-dimensional numerical example.

Next, the problem is extended to a three-lane road. It is considered in a two-dimensional space. The boundary conditions employed herein are modified as

12φ0,y−∂φ∂n=3−ΘeelnksK0φ1,y=−Θee×lnksK0∂φx,0∂n=∂φx,3∂n=0k0,y−13∂k∂n=50k1,y=30                                 (7.12)

The numerical results are illustrated in Figures 7.6 to 7.11. Traffic potential decays, whereas density increases in the front part of the interval and then decreases. The result is similar to the one-dimensional problem because the case is an uncongested case and the difference among lane is neglected. Thus, the movement of vehicles in each of the three lanes will be the same. Figure 7.11 is the steady state flow density in the form of flow field. From Figures 7.10 and 7.11, we can find that the propagation of traffic flow is mainly forward.

Figure 7.6 The plot of potential in two-dimensional numerical example.

Figure 7.7 The plot of flow density in x-direction in two-dimensional numerical example.

Figure 7.8 The plot of velocity in two-dimensional numerical example.

Figure 7.9 The plot of density in two-dimensional numerical example.

Figure 7.10 The plot of traffic field in two-dimensional numerical example.

Figure 7.11 The plot of flow density in two-dimensional numerical example.

Generally, the movement of different lanes is different. For example, vehicles can pass through with less disturbance in the inside lane than in the outside lane. Therefore, the distinction among different lanes can be measured by the unstrained density. The unrestrained density ks is assumed to be

ks=37.5+21x,∀x∈0,0.6,y∈insidelane35+20x,∀x∈0,0.75,y∈medianlane30+26.7x∀x∈0,0.75,y∈outsidelane50,otherwise,                        (7.13)

which is illustrated in Figure 7.12. The numerical results are illustrated in Figures 7.13 to 7.18. Traffic potential still decays. From Figure 7.14, the density of each lane is different. Since the disturbance, which is produced by the merging and diverging, of outside lane is the largest, the density is large in the outside lane. The density increases in the front part of the interval and then will tend toward uniform after a certain distance. Still, the flow is almost constant because the problem is under steady state. A descend trend is illustrated in Figure 7.14 may cause by the error of potential and density. Velocity and the flow density are illustrated in Figure 7.15. Figures 7.17 and 7.18 are the results of the traffic field and the flow density, respectively. From the two figures, the moving trend of vehicles is shown. In this problem, vehicles will tend toward inside lane.

Figure 7.12 The plot of unstrained density and the research domain in two-dimensional numerical example with lane-distinction.

Figure 7.13 The plot of potential in two-dimensional numerical example with lane-distinction.

Figure 7.14 The plot of density in two-dimensional numerical example with lane-distinction.

Figure 7.15 The plot of flow in two-dimensional numerical example with lane-distinction.

Figure 7.16 The plot of velocity in two-dimensional numerical example with lane-distinction.

Figure 7.17 The plot of traffic field in two-dimensional numerical example with lane-distinction.

Figure 7.18 The plot of flow density in two-dimensional numerical example with lane-distinction.

## 7.5 Summary

A drift-diffusion traffic flow model is simplified from the modeling architecture presented in Chapter 6 herein. The model is also a self-consistent one, which describes the evolution of density by the continuity equation and determines the interaction among vehicles by the density. A decoupling scheme is also suggested to solve the system under the steady state. Furthermore, a numerical example of uncongested highway is employed to explain the traffic condition.

There are still further works left for further researches, such as numerical scheme and simulations of time-dependent problem, scenarios of different traffic situation, the application to traffic control, extension of multilane traffic model and so on. In addition, as providing the real-time information and prediction become more and more important today, the efficiency and accuracy of numerical simulation are further important research topics.

# CHAPTER 8

# Numerical Simulation of Time Variant Drift-Diffusion Traffic Flow Model

The decoupling scheme of steady-state drift-diffusion model is proposed in Chapter 7. In this chapter, the numerical scheme of time variant drift-diffusion model is presented. The scheme is based on the decoupling scheme suggested in the previous chapter and the time-matching approach. Therefore, the time-matching method is introduction briefly in Section 8.1. Then, the time variant scheme of the drift-diffusion model is presented in Section 8.2. At last, a numerical example is employed to explain the model.

## 8.1 Brief Intorduction of Time Variant Scheme

The time variant drift-diffusion model is shown as follows.

−Δφ=eεK0expeψ−eφΘe−ks+Ka.                              (4.17)

∂kx,t∂t+∇x⋅qx,t=0.                                         (6.18)

From Eqs (4.17) and (6.18), it is found that only Eq. (6.18) depends on time, i.e., we only need to consider the time-matching scheme of Eq. (6.18). For the sake of convenience, we discuss the time-matching method of Eq. (6.18) in one-dimensional space, which is represented by Eq. (8.1).

∂k∂t+∇xq=0.                                                   (8.1)

Thus, the numerical scheme can be obtained by following steps. Firstly, the space discretization is derived by the finite difference scheme, i.e.,

∇xq≈qx+hx,t−qx,thx,                                         (8.2)

where hx is the length of the mesh. By the same manner, the scheme of time discretization is represent as

∂k∂t≈kx,t+Δt−kx,tΔt,                                          (8.3)

where Δt is size of time step. If we denote the approximation of density and flow density at time m as km and qm, respectively, Eq. (8.1) can be discretized as

km+1−kmΔt+λ∇xqm+1+1−λ∇xqm=0,                           (8.4)

where λ is a weighting parameter. If λ=0, we have a Forward Euler Method (explicit method). If λ=1, we have a Backward Euler Method (implicit method). If λ=1/2, we have a Crank-Nicolson Method. Generally, the approximation of the Crank-Nicolson Method is better than the result of the Forward Euler Method and the result of the Backward Method (Morton, 1996, Liu, 2000). Therefore, The Crank-Nicolson Method is chosen to solve the time variant drift-diffusion traffic flow model in this study. By coupling Eqs (8.2) and (8.4), the full discretization of Eq. (8.1) is obtained.

kim+1−kimΔt+λqi+1m+1−qim+1hx+1−λqi+1m−qimhx=0,                   (8.5)

where kim+1 denotes the approximation of density at potion i when time is m+1. qim+1 denotes the approximation of flow density at potion i when time is m+1. Equation (8.5) is the discretization of time dependent problem. In the following section, the Scharfetter-Gummel scheme will be substituted for Eq. (8.3) in Eq. (8.5) and the numerical scheme of the dynamic drift-diffusion traffic flow model will be presented.

## 8.2 Numerical Scheme of Time Variant Drift-Diffusion Model

Before deriving the time dependent Scharfetter-Gummel scheme, the implementation of the numerical simulation is mentioned. Basically, we have to solve a steady state drift-diffusion traffic flow model at each time step. Hence, the decoupling scheme is still employed to solve the steady state problem. By time matching method, the numerical approximation of each time step is obtained. The implementation of the monotone iterative method is illustrated in Figure 8.1. The procedure is described as follows.

Step 1 initial guess.

Step 2 set error bound and stop criterion.

Step 3 set time step

Step 4 set iterative number and node.

Step 5 calculate the monotone number.

Step 6 compute the present solution.

Step 7 check the stop criteria: if yes, then goto Step 8; if no, then goto Step 4.

Step 8 check the time step: if the time step is equal to the total research time, then stop; if no, then increase the time step by one and goto Step 4.

Figure 8.1 A flowchart for time variant drift-diffusion traffic flow model.

After introducing the solving procedure, the discretized scheme is derived. The discretized scheme of the dispersion model is the same as Eq. (5.17) because the dispersion is independent of time. According to the steady state discretization of the continuity equation derived in Section 7.3, the full discretization of the continuity equation is obtained by the time matching method mentioned in the previous section, i.e., substituting Eq. (7.8) for Eq. (8.3) in Eq. (8.5). Then, we have

kim+1−kimΔt+12qi+1/2m+1−qi−1/2m+1hi+hi−1/2+12qi+1/2m−qi−1/2mhi+hi−1/2=0,                    (8.6)

where

qi+1/2m−qi−1/2mhi+hi−1/22hi+hi−1[μeki+1m1+expφim−φi+1m2Θe/e+kim1+expφi+1m−φim2Θe/eφi+1m−φimhi+μeki−1m1+expφim−φi+1m2Θe/e+kim1+expφi−1m−φim2Θe/eφi−1m−φimhi−1−νφi+1m−φimΘe/eexpφim−φi+1m2Θe/e−expφi+1m−φim2Θe/eki+1m−kimhi−νφi−1m−φimΘe/eexpφi−1m−φim2Θe/e−expφim−φi−1m2Θe/eki−1m−kimhi−1]        (8.7)

and

kim=K0iexpψim−φimΘe/e.                                            (8.8)

Furthermore, the two-dimensional scheme of the continuity equation can be obtained by the same way.

## 8.3 Numerical Results and Discussions

In this section, the numerical results of the numerical scheme for the time-variant drift-diffusion model are presented. The time variant numerical example considered in this study is a one-dimensional problem. It still considers a basic section of highway, which is behind an intersection. The research domain is a single lane section with 1 km in length. Under the uncongested traffic, the unrestrained density ks is assumed to be

ks=30+5x,x∈0,1,                                            (8.11)

which is shown in Figure 8.2. The initial and boundary conditions are given as follows:

kx,0=40−10xx∈0,1,                                       (8.12)

φ0=−Θeelnks0K0φ1=−Θee×lnks1K0k0=40,                                      (8.13)

which are illustrated in Figures 8.3 and 8.4. the passenger car equivalent e=1 and the K0=100pcu/km⋅lane. Assume the interacting parameter ε=3,600 and the domain is ideal in geometry then Ka=0. To test the algorithm, the conditions are given for easy solving. The numerical results are illustrated in Figures 8.5 to 8.8. Traffic potential and density decay. It means that the interaction among vehicles in the front part is large. This results in large density and small velocity, which is shown in Figures 8.6 and 8.8. Since the problem is a time variant problem, the traffic flow will tend to equilibrium after a certain time period. In this case, the transient time is about 5 minutes. According to the transient time and the inflow rate of the boundary, traffic control strategies can be made and evaluated.

Figure 8.2 The plot of unrestrained density and research domain in one-dimensional time variant numerical example.

Figure 8.3 The plot of initial condition of potential in one-dimensional time variant numerical example.

Figure 8.4 The plot of initial condition of density in one-dimensional time variant numerical example.

Figure 8.5 The plot of potential in one-dimensional numerical example.

Figure 8.6 The plot of density in one-dimensional numerical example.

Figure 8.7 The plot of flow in one-dimensional numerical example.

Figure 8.8 The plot of velocity in one-dimensional numerical example.

## 8.4 Summary

A numerical scheme of the time variant drift-diffusion traffic flow model is proposed in this chapter. The scheme is based on the steady state scheme presented in Chapter 7 and the time matching method. The transient behavior of traffic flow can be observed by the dynamic model. Thus, the traffic control strategies can be made or evaluated. Consequently, the optimization of the strategies is the further researches. In the aspect of computation, time variant problem takes a lot of computing time. According to the computational experience we have got, the computing time of a one-dimensional dynamic problem with a set of smooth initial and boundary conditions is about 40 minutes. It is not sufficient for providing dynamic traffic information and control. Therefore, improving the efficiency of numerical computation is the following research.

# CHPTER 9

# Modeling of Stochastic Dynamic Traffic Flow

Stochasticity and the nonlinearity are always present in traffic and cause considerable mathematical difficulty. A dynamic system with random fluctuations can be modeled by stochastic differential equations, which can involve stochastic processes as coefficients of the differential operator, as initial conditions, and as forcing functions. In the limiting case where randomness vanishes, the deterministic result is obtained. However, if nature is really stochastic, a deterministic theory will always be a crude approximation and may prevent a deeper understanding. Nonlinearity is another problem. However, only some ad hoc techniques are available for nonlinear operators. Thus, many nonlinear traffic models are solved by means of linearization so as to employ the well-developed knowledge about linear systems, linear equations, and linear operator theory to analyze the system. This occasionally introduces incorrect results. That is, mathematical solution is not the phenomena of the real world, although it may occasionally be a useful approximation or a first result yielding some insights.

Since the importance of stochasticity and nonlinearity and conventional techniques such as linearization and perturbation are not sufficient or realistic, the purpose of this chapter is to formulate a nonlinear stochastic dynamic traffic flow model without unphysical assumptions, perturbation, linearization, truncation, or discretization. The differences between the model proposed herein and previous studies are the nonlinearity of our model is decomposed by polynomials and the stochasticity is formulated by Itô equation. In the following section, the decomposition of nonlinear dynamic traffic flow model is presented.

## 9.1 Decomposition of Traffic Flow Model

Stochastic models discussed in this chapter is based on the traditional LWR model, which is the same with Eq. (6.18) in this dissertation. With the fundamental diagram and the speed-density relation, Eq. (6.18) can be solved and analyzed. Equation (6.18) can be represented as

kt+q'kkx=0,                                                 (9.1)

where q'k=dku/dk. It has been found that even the simplest speed-density relation, including the Greenshield’s model, which is substituted into Eq. (9.1) becomes a nonlinear partial differential equation. Therefore, before converting a deterministic equation to its stochastic form, the nonlinear terms should be decomposed first. The decomposed polynomial employed here is suggested by Adomian (1983, 1986). The decomposition is shown as follows.

First, let Lt=∂/∂t and Q'kkx=fk,kx+bkx. Equation (9.1) is represented as

Ltk=−fk,kx−bkx,                                             (9.2)

where b is a constant and fk,kx is the nonlinear part of Eq. (9.2).

Lt=−fk,kx−b∂∂x.                                          (9.3)

Let Lt−1 as the corresponding integration from 0 to t. Then

,                          (9.4)

.                                   (9.5)

Parametrizing the differential operator Lt−1 with the parameter λ, Lt−1is represented as

Lt−1=−λLt−1fk,kx+b∂∂x,                                      (9.6)

where λ is a parameter, which is not small. Therefore, Eq. (9.5) becomes

k=kt=0−λLt−1fk,kx−λbLt−1kx.                                  (9.7)

Since fk,kx is nonlinear and in product form as a portion of Eq. (9.2), it should be parametrized and decomposed first. Let k=n=0∞λnkn with k0=kt=0, where λ is a parameter. Let the nonlinear term be represented as fk,kx=fkfkx and decomposed by n=0∞λnAnn=0∞λnBn, where An is the decomposition polynomial of fk and Bn is the decomposition polynomial of fkx. Then, k is decomposed as

k=k0−λLt−1n=0∞λnAnn=0∞λnBn−λLt−1b∂∂xn=0∞λnkn.                   (9.8)

Since k=n=0∞λnkn, that is

k=k0+λk1+λ2k2+⋯+λnkn+⋯.                                  (9.9)

Comparing equation (9.8) and (9.9) reveals that once the An and Bnare determined, any component of k for m≥1 is given by

km=−Lt−1i,j=0i+j=m−1AiBj−Lt−1b∂∂xkm−1.                                  (9.10)

An and Bn are generated specifically for the nonlinearity according to the suggestion of Adomian (1983) which is shown as

An=1n!dndλnfkλ|λ=0,                                       (9.11)

Bn=1n!dndλnfkxλ|λ=0.                                      (9.12)

To evaluate the coefficients An and Bn, observe that k=kλ and hence

,

,

,                 (9.13)

,

,

,

,               (9.14)

,

where kix=∂ki/∂x ∀i=1,...,n. These are defined such that each An depends only on k0,k1,⋯,kn and each Bn only depends on k0x,k1x,⋯,knx, where knx=∂kn/∂x. The simplest case,fk=k, is an example to show An=kn.

An=1n!dndλnfkλ|λ=0=1n!dndλnk|λ=0=1n!dnkdλn|λ=01n!n!kn=kn             (9.15)

is true. The convergence ofAn polynomials is proved in Adomian (1986). limn→∞Φn−Φn−1=limn→∞yn=0 is presented.

To complete a traffic flow model, a speed-density relation should be introduced into Eq. (9.1). The Greenshield’s model (1934), the Greenberg’s model (1959) and the Underwood’s model (1961) which are the most common speed-density relations, are considered herein. The Greenshield’s model, which is

u=uf1−kkj                                                 (9.16)

is considered first. uf is free flow speed and kj is jam density. Both uf and kj are constants. Substituting equation (9.16) into q=ku becomes

q=ku=ufk1−kkj.                                            (9.17)

Then,

Q'k=dqdk=uf−2ufkjk.                                         (9.18)

Thus, fk,kx=fkfkx=−2ufkjkkx, fk=k, fkx=−2ufkjkx and b=uf. From Eq. (9.11), An is determined as

A0=k0, A1=k1, A2=k2, A3=k3, … .                             (9.19)

From Eq. (9.12), Bn is determined as

B0=−2ufkjk0x, B1=−2ufkjk1x, B2=−2ufkjk2x, B3=−2ufkjk3x, … .      (9.20)

Thus,

k1=−Lt−1−2ufkjk0k0x−Lt−1uf∂∂xk0,

k2=−Lt−1−2ufkjk1k0x+k0k1x−Lt−1uf∂∂xk1,                        (9.21)

k3=−Lt−1−2ufkjk2k0x+k1k1x+k0k2x−Lt−1uf∂∂xk2,

Therefore, the n term approximation of k is

Φn=i=0n−1ki.                                                    (9.22)

Next is the Greenberg’s model, which is illustrated by

u=u0lnkjk,                                                  (9.23)

where uo is the optimal speed, which is a constant. Substituting Eq. (9.23) into q=ku becomes

q=uoklnkjk.                                                  (9.24)

Then,

q'k=u0lnkj−1−uolnk.                                   (9.25)

Thus, fk,kx=fkfkx=−uolnkkx fk=lnk, fkx=−uokx and b=uolnkj−1. From Eq. (9.11), An is determined as

A0=lnk0, A1=k1k0, A2=12−k12k02+2k2k0, A3=1612k13k03−6k1k2k02+6k3k0, … . (9.26)

However, as the limit of logarithm k0 should be greater than zero. From Eq. (9.12), Bn is determined as

B0=−uok0x, B1=−uok1x, B2=−uok2x, B3=−uok3x, … .               (9.27)

From Eq. (9.10), the approximation Eq. (9.22) is obtained by substituting Eqs (9.26) and (9.27) into it.

The third is the Underwood’s model, which is illustrated as

,                                                   (9.28)

where kopt is the optimal density, which is a constant. Substituting Eq. (9.28) into q=ku becomes

q=ufke−kkopt.                                                  (9.29)

Then,

q'k=ufe−kkopt−kkopt+ufe−kkopt,                                   (9.30)

Thus, fk,kx=fkfkx=ufe−kkopt−kkopt+ufe−kkoptkx fk=e−kkopt1−kkopt, fkx=ufkx and b=0. From Eq. (9.11), An is determined as

,

A1=−1kopte−k0kopt1−k0koptk1,

A2=12−1kopte−k0kopt−1kopt3−k0koptk12+22−k0koptk2,                (9.31)

A3=16−1kopte−k0kopt62−k0koptk3+6−1kopt3−k0koptk1k2+−1kopt24−k0koptk13,

From Eq. (9.12), Bn is determined as

B0=ufk0x, B1=ufk1x, B2=ufk2x, B3=ufk3x, … .                   (9.32)

From Eq. (9.10), the approximation Eq. (9.22) is obtained by substituting Eqs (9.31) and (9.32) into it.

Adomian (1986) reveals the accurate decomposition of fk=k,e−k by simple numerical examples, which are illustrated briefly as below. In the case of fk=k, consider the trivial equation k=8. Rewriting it as 2k−fk−8=0, such that k=fk/2+4=An/2+4. Thus Φn is the approximation of k=8. Let n=7, therefore, the approximation is Φ7=7.93, which will evidently improve with more terms. In the case of fk=e−k, consider k=2+e−k. A seven-term approximation has an error of 0.0007%, which is evaluated by 100Φ7−x/x%. However, Adomian did not discuss the case of fk=lnk, in this study k=lnk+3.390562088 is considered. The exact solution of k is 5. The approximation is Φ7=4.976078602, which has an error of 0.478%. The convergence of decomposed polynomial of polynomial type nonlinearity has been proved in Adomian (1986). However, the convergence of exponential and logarithm nonlinearity can only be proved numerically.

Another viewpoint of decomposition is considering qx of the continuity equation as the nonlinear part and decomposes it directly. Since q is assumed to be a function of k, the continuity equation is illustrated as

kt+gkx=0.                                                  (9.33)

Thus, Eq. (9.3) becomes

Ltk=−gkx.                                                  (9.34)

Take the Greenshield's model for example, g(k)=ufk−k2/kj and Eq. (9.8) is represented as

,                                              (9.35)

where  is decomposed by . From Eq. (9.11),

,

,

,                                     (9.36)

,

Then both the Greenberg's and the Underwood's model can also be decomposed by the same manner. If diffusive effect is considered in a traffic model, the relation q=ku should be modified as

,                                                   (9.37)

which is suggested by Whitham (1974). Thus, Eq. (9.2) becomes

or  ,                    (9.38)

where  is a diffusion coefficient which is a constant. As mentioned above, substituting a specific speed-density relation into Eq. (9.38), a specific model is obtained. The nonlinear part of Eq. (9.38) can be decomposed by the polynomial. It is represented as

.    (9.39)

As long as An and Bn are obtained, kn is calculated. Therefore, the n term approximation Eq. (9.22) is obtained. Before converting the deterministic model into a stochastic form, a systematic computation process of decomposed polynomial is introduced in the next section.

## 9.2 Computation of Decomposition Polynomial

The systematic computation process of decomposed polynomial is suggested by Adomian (1983). Let D=d/dλ=dk/dλd/dk, Eq. (9.11) is simplified as

.                                              (9.40)

From k=n=0∞λnkn, the following relations are true.

.                                            (9.41)

.                                 (9.42)

The Dnf term for n>0 can be written as a summation from m=1 to n of term dmf/dkm with coefficients, which are polynomials in the dmk/dλm. Denote the mth coefficient as cm,n, the nth derivative Dnf becomes

,                                           (9.43)

where Fm=dmf/dkm. The first index in the cm,n progresses from 1 to n along with the index of F, and the second index is the order of the derivative. The cm,n coefficients are calculated as follows: For 1≤i,j≤n,

(9.44)

with c0,0=1, and c1,0=0. The first condition is obtained by comparing with D0f=c0,0d0f/dk0=f, and the second condition is true due to ci,j=0, for i>j. Let

(9.45)

and Fi=dif/dki as defined previously are explicit derivatives, which also represent Dnf. Consider n=3, for example, the illustration of Dnf by ci,j and ϕi,j is shown as

.             (9.46)

Now, An polynomials can be computed by ci,j or ϕi,j coefficients. Considering coefficients:

,

.                                    (9.47)

Noting that c0,j=0 for j>0,

,

,                                (9.48)

To calculate any Dnf, simply multiply the cm,n coefficients by Fm, for the range from 1 to n and sum. Thus

,

,                           (9.49)

,

Thus, the coefficients of decomposition polynomials are calculated and the nonlinear terms of dynamic traffic flow model can be solved.

## 9.3 Stochastic Dynamic Traffic Flow Model

After decomposing the nonlinear term, the next step is to transfer the deterministic model to a stochastic one. The stochastic case is formulated by Itô equation, which is used as a model for a dynamical system perturbed by a special stochastic process with simplifying mathematical properties. This process is called white noise. The stochastic case of Eq. (9.2) is expressed as

,                                     (9.50)

where wt is the Wiener process. This equation is therefore rewritten as

,                                      (9.51)

where bt,w=bdwt/dt is a general stochastic process since

,                                     (9.52)

where  is the expected value,  are deterministic functions of time and  are uncorrelated random variables. In the sense of Pugachev (1965) or a Karhunen expansion, each individual  is an elementary random function. Analogously,

,                                (9.53)

where  is a white noise variation of the  parameter and  is a deterministic function with parameter t and argument . The integral corresponds to the sum with each infinitely small-uncorrelated elementary random function being . Thus we have a representation of a stochastic process in terms of a white noise. Let Lt=∂/∂t and  so that operation by  on  results in

.             (9.54)

Now

,

,

,                          (9.55)

.

The approximant , which approaches k; i.e., , is obtained. The expectation is given by . The correlation is found by writing .

By the same manner, the model with diffusion effect can also be converted to a stochastic form. Converting Eq. (9.38) to Itô form is represented as

(9.56)

and rewritten as

.                                  (9.57)

Equation (9.39) in stochastic form is rewritten as

.  (9.58)

With a specific speed-density relation, Eqs (9.54) and (9.58) are the nonlinear stochastic dynamic traffic flow model proposed herein.

## 9.4 Summary

The stochastic and nonlinear properties are found everywhere in the real world. However, both behaviors can hardly be captured, analyzed and modeled. Even if each of them can be modeled, solving such a system is still difficult. This chapter derives nonlinear stochastic models from the LWR model. The models are derived by the following procedure. Firstly, Adomian’s (1983) polynomial is employed to decompose the nonlinear part of each model. Then, the deterministic models are transformed into stochastic models by Itô form. Three common speed-density relations are employed herein. Other relations can be treated as the combination of these three relations and can also be decomposed by Adomian’s polynomials with the combination form as mentioned previously. Thus, nonlinear stochastic dynamic traffic flow models are obtained. Adomian (1986) proved the convergence of the polynomials and the convergence of polynomial nonlinearity. However, a numerical aspect can only be presented to prove the convergence of exponential and logarithm nonlinearity. The theoretical proofs of convergence are left for further researches.

# CHAPTER 10

# Conclusions and Perspectives

In this chapter, we make some conclusions and perspectives of this research. Dynamic traffic management is one of the solutions for the traffic congestion. Among the fundamental theory and controlling strategies, dynamic traffic flow theory is an important kernel. Since traffic flow problems in the real world involve multilane and multi-classes driving behavior, the studies of dynamic traffic flow include diverse topics. The main objective of this dissertation is to develop a framework of dynamic traffic flow models and the numerical scheme. According to the framework, dynamic traffic flow models, which can be extended to describe multilane multi-classes users traffic flow, are proposed. In this closing chapter, the conclusions of this dissertation are mentioned. They are categorized into conclusions with respect to dynamic traffic flow theory, continuum modeling and numerical scheme. Also, the future researches of this study are presented.

## 10.1 Conclusions

The work presented in this dissertation has evolved into a broad scope. We established a new dynamic traffic flow theory, developed a modeling framework, proposed numerical schemes and presented some interesting results. Therefore, the conclusions presented in this section are categorized into four groups, that is, theory development, macroscopic dynamic traffic flow modeling, numerical schemes and model applications. Overall, in opposition to current aggregate-class aggregate-lane macroscopic models, the multilane generalization presented in this thesis enables macroscopic model application to describe problems, which class and lane distinction have to be considered is necessary.

Firstly, one main result of this thesis is the development of a realistic and tractable traffic flow model, which accurately describes the multiclass traffic flow operations on multilane roadways. The theoretical framework is followed by the concept of dynamical systems, which include transient equations and state equations. Due to the modeling and computational consideration, macroscopic models are preferred. Thus, the framework and the theory are designed for macroscopic modeling. We can summarize the results of theory development as follows:

- Extension and definitions of multiple dimensional traffic flow are presented. According to the definitions and the introduction of the new variable (flow density) studying multilane traffic flow in multi-dimensional space becomes possible and consistent.

- The dynamic traffic flow system is consisted of transient equations and state equations. The transient equations, which are in the form of conservation laws, are derived from the Boltzmann equation. On the other hand, a dispersion model is suggested to be the state equation in the dynamic system. The dispersion model provides the equilibrium distribution of density and the magnitude of interaction among vehicles on the road.

- The dynamic traffic flow modeling procedure and framework can be employed to develop more complicated situations and driving behavior. On the other hand, the framework provides an insight of the assumption of dynamic traffic flow models. Therefore, the chosen or simplification of dynamic traffic flow models will become more suitable to the traffic situation that we want to describe.

- To overcome the disadvantages of macroscopic models; that is, lack of behavior discussion and inconsistency of model, a modeling procedure is suggested. A mesoscopic model, which is based on Boltzmann equation, is presented first. The driving behavior is modeled and discussed in the equation. Boltzmann equation is widely used to describe the interaction and propagation of particles and species. Then, the macroscopic model is derived from the Boltzmann equation by the moment expansion. Hence, the macroscopic model is derived with behavioral analysis and the equations are consistent with each other.

- According to the systematic equations, all variables can be solved by the equations. Therefore, the model is a self-consistent model.

As mentioned above, there are transient equations and state equations of the systematic model. The transient equations describe the macroscopic propagation of traffic variables and the state equation determines the state. The transient equations are in the form of conservation laws and the dispersion model is a nonlinear Poisson equation. We can summarize the results of dynamic traffic flow models as follows:

- The dispersion model is derived from the car-following theory and the equilibrium assumption. It describes the influence of density on a road. The dispersion model simplifies and generalizes the discussion of multilane. Also, the model implies the finite-space requirement so as to make the application to multiple driving behaviors possible.

- From the Boltzmann equation, the transient equations derived in this study include conservation of vehicle numbers (continuity equation, LWR model), conservation of momentum (motion equation) and conservation of energy (velocity variance equation).

- A multilane multiclass users dynamic traffic flow model is proposed. Density, velocity, velocity variance and the traffic potential are the interested variables. The basic idea of the multiclass users model is the propagation of each type of user is described by an independent set of conservation laws. The total influence of all users, which is restrained by the finite-space constraint, is measured by the dispersion model. Generally, not all variables are significant enough to influence the traffic flow. Suitable simplifications should be made to increase the numerical computation. An explicit equation of the fundamental diagram is simplified from the complete system. The simplified model is named as drift-diffusion model because it describes the drift and diffusion effect of traffic flow.

- A stochastic modeling method is presented in this study, too. The LWR model is employed as an example. In the case of stochastic dynamic traffic flow model, we decompose the nonlinear term by a decomposed polynomial to reduce the complexity of the stochastic model.

Since the model is a systematic equation, a solving procedure, which is named as decoupling scheme, is suggested. Furthermore, the discretized schemes of the steady state and time variant drift-diffusion model are proposed. We can summarize the results of numerical schemes as follows:

  - Since the model is a set of systematic equation, a decoupling scheme is presented to solve the model. The basic idea is fixing other variables, when one of them is being solved. The decoupling scheme is the outer loop of the solving procedure.

  - The dispersion model is a nonlinear Poisson equation. It has to be solved by the numerical method. A monotone iterative method is employed to discretize the equation. Two theorems are presented to show that if an equation satisfies them, the numerical result will converge monotonically. In the whole solving procedure, the monotone iterative scheme is a inner loop.

  - A Scharfetter-Gummel scheme is derived to solve the steady-state continuity equation. An exponential fitting is employed to approximate the density because the density is expected to change dramatically and the traffic potential is expected to change smoothly. The Scharfetter-Gummel scheme is also a inner loop of the whole solving procedure.

  - Numerical simulation of the time variant problem is solved by time matching of the steady-state scheme. Among plenty of the time matching method, Crank-Nicolson method is simple but sufficient so we employ the Crank-Nicolson method to solve the time variant drift-diffusion traffic flow model.

At last, the applications of the model are discussed as follows:

- The applications and the analyses of the dispersion model are discussed in Chapter five. The analytical solution and analyses of the linear model are shown and a monotone iterative scheme, which is derived from the divergence theorem, is introduced to solve the nonlinear dispersion model. The numerical example of the dispersion employed in this thesis shows the dispersion model can describe the multilane traffic flow. The moving trend of traffic flow is described. By the simulation result, we can observe where the traffic flow will become stable (or equilibrium) behind an intersection or an on-ramp. Hence, the traffic control and geometric of the road can be designed.

- Three numerical examples of the steady-state drift-diffusion traffic flow model are presented. The results show that the two-dimensional traffic flow model can describe the diffusion of the density. On the other hand, the one-dimensional case only shows the average change of the density. When the difference of the lanes is considered, the moving trend of the traffic flow is more realistic. Also, the applications and analyses of the steady-state case focus on the planning and design of traffic engineering, such as signal control and geometric design. If the traffic phenomenon is in the transient section, such as a section behind an interaction and an on-ramp or a section ahead an interaction and an off-ramp, is the most interesting aspect, a time variant model should be considered.

## 10.2 Persectives

In this research, the fundamental dynamic traffic flow theory is developed. Our long-term objective is to develop a dynamic traffic management simulator, which involves the applications to Advanced Transportation Management Systems (ATMS) and Advanced Transportation Information Systems (ATIS). This research is the beginning. The future studies are illustrated as Figure 10.1 in the time sequential order. The fruitful research areas are categorized into four groups, that is, modeling development, numerical simulation, architecture of advanced traffic management and information center and dynamic traffic management simulator. Firstly, the further development of dynamic traffic model is mentioned as follows.

- Dynamic traffic flow modeling framework developed in this study is based on the relaxation time approximation. There is still complicated behavior to be formulated. Thus, analyses of multilane multiclass users traffic flow behavior and formulating them in mathematical model are one of the further research topics. The relation between the dynamic traffic flow model and the dynamic traffic information is illustrated as Figure 10.2.

- Next step is extending the link model to network model. The application of link flow model is limited. A network traffic flow model has much more applications than a link flow model. Therefore, developing a dynamic network traffic model, which is based on the dynamic traffic flow model proposed in this study, is necessary and important. The relation between the dynamic link traffic flow model and the dynamic network traffic flow model is illustrated as Figure 10.3.

- As mentioned above, determining an optimal set of traffic management strategies is one main objective. Consequently, the traffic management strategies should be formulated as mathematical models. Generally, the strategies are shown in the form of boundary conditions. In addition, the optimization of the strategies is another research interest.

Secondly, numerical simulation is also an important work. According to the computational experience of this study, the computing time will increase rapidly when the model becomes complicated. Therefore, rapid numerical simulation is vital.

- Developing suitable discretized schemes of each equation, which can converge stably and rapidly, is one method to increase computing efficiency. However, improving the convergence and the accuracy of the schemes are difficult works.

- Another method is employing parallelized techniques, such as distributed computing, message passing interface (MPI) techniques and grid computing. Each kind of technique has its own advantages and limits. Different techniques should be employed due to the applications. For instance, the distributed computing and MPI can be employed as a local center because the setup cost of the computational environment is cheap. If the traffic management strategies involve large area or several local control centers, a grid computing techniques should be used.

- Just as a network traffic model is necessary to develop, the simulation method of a network model is needed. If the rapid computing techniques of dynamic link traffic flow model are developed, they can be the bases of the simulation of network traffic.

In the above paragraphs, perspectives of modeling and numerical simulation are mentioned. Next part is the researches for the architecture of advanced traffic management and information center. As mentioned previously, different rapid computational environments have different cost and computing efficiency. The architecture of the simulation center should be discussed under different budget and the need of computation and information. From the Figure 10.1, the distributed computing and MPI technique, which is based on PC cluster, are suggested to develop local traffic management and simulation center. Grid computing technique is suitable to establish a global traffic management and simulation center. At last, a nation-wide dynamic traffic management simulation system can be established.

-127-

Figure 10.1 Future studying topics and their relation in time sequential order.

Figure 10.2 The relation between the dynamic traffic flow model and the dynamic traffic information.

Figure 10.3 The relation between the dynamic link traffic flow model and the dynamic network traffic flow model.

# References

Adomian, G., Stochastic System, Academic Press, 1983.

Adomian, G., Nonlinear Stochastic Operator Equations, Academic Press, 1986.

Alvarez, A., J. J. Brey, and J. M. Casado, “A Simulation Model for Traffic Flow with Passing,” Transportation Research Part B, Vol. 24, No. 3, pp.193-202, 1990.

Ames, W. F., Numerical Methods for Partial Differential Equations, Academic Press, Inc., 1992.

Anderson, R. L., R. Herman, and I. Prigogine, “On the Statistical Distribution Function Theory of Traffic Flow,” Operations Research, Vol. 10, pp.180-196, 1962.

Araki, S. and S. Tremain, “The Dynamics of Dense Particle Disks,” Icarus, Vol. 65, pp.83-109, 1986.

Arlotti, L. and N. Bellomo, “Population Dynamics with Stochastic Interactions,” Transport Theory Statistical Physics, Vol. 24, pp.431-443, 1995.

Arlotti, L. and N. Bellomo, “Solution of a Nonlinear Kinetic Models of Population Dynamics,” Applied Mathematical Letters, Vol. 9, No. 2, pp.65-70, 1996.

Aw, A. and M. Rascle, “Resurrection of “Second Order” Models of Traffic Flow,” SIAM Journal of Applied Mathematics, Vol. 60, No. 3, pp.916-938, 2000.

Baker, R. G. V., “A Model of Traffic Dispersion from a Congested Road,” Transportation Research Part B, Vol. 15, No. 5, pp.319-327, 1981.

Bando, H., K. Hasebe, A. Nakayama, A. Shibata, and Y. Sugiyama, “A Dynamic Model of Traffic Congestion and Numerical Simulation,” Physical Review E, Vol. 51, No. 2, pp.1035-1042.

Bellomo, N., A. Palczewski and Toscani, Mathematical Topics in Nonlinear Kinetic Theory, 1988.

Bellomo, N. and G. Forni, “Dynamics of Tumor Interaction with the Host Immune System,” Mathematical Computer Modelling, Vol. 20, No. 1, pp.107-122, 1994.

Bellomo, N., G. Forni and L. Preziosi, “On a Kinetic (Cellular) Theory for the Competition between Tumors and Host-Immune System,” Journal of Biological Systems, Vol. 4 pp.479-502, 1996.

Bellomo, N. and M. L. Schiavo, “From the Boltzmann Equation to Generalized Kinetic Models in Applied Science,” Mathematical and Computer Modelling, Vol. 26, No. 7, pp.43-76, 1997.

Bonzani, I, “Hydrodynamic Models of Traffic Flow: Drivers’ Behaviour and Nonlinear Diffusion,” Mathematical and Computer Modelling, Vol. 31, pp.1-8, 2000.

Branston, D., “Models of Single Lane Time Headway Distributions,” Transportation Science, Vol. 10, No. 2, pp.125-148, 1976.

Branston, D. “A Method of Estimating the Free Speed Distribution for a Road,” Transportation Science, Vol. 13, No. 2, pp.130-145, 1979.

Buckley, D. J., “A Semi-Poisson Model of Traffic Flow,” Transportation Science, Vol. 2, No. 2, pp.107-133, 1968.

Cercignani, C., Theory and Application of the Boltzmann Equation, Springer, 1988.

Cercignani, C., R. Illner and M. Pulvirenti, Theory and Application of the Boltzmann Equation, Springer, 1993.

Chandler, R. E., R. Herman, and E. W. Montroll, “Traffic Dynamics: Studies in Car Following,” Operations Research, Vol. 6, pp.165-184, 1958.

Chang, M. F., “Traffic Density Estimation With Consideration Of Lane-Changing,” Transportation Science, Vol. 19, No. 4, pp. 308~320, 1975.

Chen, H. K., Dynamic Travel Choice Models: A Variational Inequality Approach, Lecture Notes in Economics and Mathematical Systems, Springer-Verlag, 1998.

Cho, H. J., F. Y. Lai and S. C. Lo, “An Exploring Three-Dimensional Traffic Dispersion Model with Potential Theory,” Transportation Planning Journal Quarterly, Vol. 30, No. 2, pp.303-322, 2001. (in Chinese)

Cho, H. J., and S. C. Lo, “The Definitions of Multi-Dimensional Traffic Flow Variables,” appears to Journal of Chinese Institute of Civil and Hydraulic Engineering, 2002.

Cho, H. J., and S. C. Lo, “Solving Bilevel Network Design Problem Using Linear Reaction Function without Nondegeneracy Assumption,” Transportation Research Record, No. 1667, pp.96-106, 1999.

Cho, H. J., and S. C. Lo, “Numerical Simulation of Dynamic Drift-Diffusion Traffic Flow Model,” Applied Numerical Mathematics, 2002a (submitted paper).

Cho, H. J., and S. C. Lo, “Numerical Simulation of the Multilane Drift-Diffusion Traffic Flow Model,” Mathematics and Computers in Simulation, 2002b (submitted paper).

Cho, H. J., and S. C. Lo, “Modeling of Self-consistent Multi-class Dynamic Traffic Flow Model,” Physica A, 2002c (accepted).

Cho, H. J., and S. C. Lo, “Monotone Iterative Simulation of Traffic Dispersion Model,” Computer Physics Communications, 2002d (accepted).

Cho, H. J., and S. C. Lo, “Numerical Simulation of Drift-Diffusion Traffic Flow Model,” WSEAS Transactions on Systems, Vol. 1, No.2, pp.254-259, 2002e.

Cho, H. J., and S. C. Lo, “The Analyses and Applications of the Traffic Dispersion Model,” WSEAS Transactions on Systems, Vol. 1, No.2, pp.153-158, 2002f.

Chow, T. S., “Operational Analysis of a Traffic-Dynamics Problem,” Operations Research, Vol. 6, pp.827-834, 1958.

Chowdhury, D., D. E. Wolf and M. Schreckenberg, “Particle Hopping Models for Two-Lane Traffic with Two Kinds of Vehicles: Effect of Lane-Changing Rules,” Physica A, Vol. 235, pp. 417-439, 1997.

Chronopoulos, A. T. and C. M. Johnston, (1997) “Parallel Solution of a Traffic Flow Simulation Problem,” Parallel Computing, Vol. 22, pp.1965-1983.

Chronopoulos, A. T. and C. M. Johnston, “A Real-Time Traffic Simulation System,” IEEE Transactions on Vehiclar Technology, Vol. 47, No. 1, pp.321-331, 1998.

Clark, S. D., M., S. Dougherty and H. R. Kirby, “The Use of Neural Networks and Time Series Models for Short Term Traffic Forecasting: A Comparative Study,” Proceedings of the PTRC Summer Meeting, Manchester, 1993.

Cowan, R., “Further Results on Single Lane Traffic Flow,” Journal of Applied Probability, Vol. 17, pp. 523-531, 1980.

Daganzo, C. F., “The Cell Transmission Model: A Dynamic Representation of Highway Traffic Consistent with the Hydrodynamic Theory,” Transportation Research Part B, Vol. 28, No. 4, pp.269-287, 1994.

Daganzo, C. F., “Properties of Link Travel Time Functions under Dynamic Loads,” Transportation Research Part B, Vol. 29, pp.95-98, 1995.

Daganzo, C. F., “A Finite Difference Approximation of the Kinematic Wave Model of Traffic Flow,” Transportation Research Part B, Vol. 29, No. 4, pp.261-276, 1995.

Daou, A., “The Distribution of Headways in a Platoon,” Operations Research, Vol. 12, pp.360, 1964.

Davidson, K. B., “A Flow Travel Time Relationship for Use in Transportation Planning,” Proceeding of Australian Road Research Board, Vol. 3, No. 1, pp. 183-194, 1974.

De Angelis, E., “Nonlinear Hydrodynamic Models of Traffic Flow Modeliling and Mathematical Problems,” Mathematical and Computer Modelling, Vol. 29, pp.83-95, 1999.

De Boer, R. J., L. Segel and A. S. Perelson, “Pattern Formation in One or Two dimensional Shape-Space Models of the Immune System,” Journal of Theoretical Biology, Vol. 155, pp.295-333, 1992.

De Boer, R. J., M. C. Boerlijst, B. Sulzer and A. S. Perelson, “A New Bell Shaped Functions for idiotypic Interactions Based on Cross Linking,” Bulletins of Mathematics Biology, Vol. 58, pp.285-312, 1996.

Del Castillo, J. M., P. Pintado, and F. G. Benitez, “The Reaction Time of Drivers And The Stability of Traffic Flow,” Transportation Research Part B, Vol. 28, No. 1, pp.35-60, 1994.

Del Castillo, J. M., and F. G. Benitez, “On the Functional Form of the Speed-Density Relationship-I: General Theory,” Transportation Research Part B, Vol. 29B, No. 5, pp.373-389, 1995.

Del Castillo, J. M., and F. G. Benitez, “On the Functional Form of the Speed-Density Relationship-II: Empirical Investigation,” Transportation Research Part B, Vol. 29, No. 5, pp.391-406, 1995.

Drew, D. Traffic Flow Theory and Control, McGraw-Hill Book Company, pp223-254, 1968.

Dunne, M. C., and R. W. Rothery, “A Discrete Markov Model of Vehicular Traffic,” Transportation Science, Vol. 2, No. 3, pp.233-251, 1968.

Edie, L. C., “Car-Following and Steady-State Theory for Noncongested Traffic,” Operations Research, Vol. 9, pp.66-76, 1961.

Edie, L. C., R. Herman, and T. N. Lam, “Observed Multilane Speed Distribution and the Kinetic Theory of Vehicular Traffic,” Transportation Science, Vol. 14, No. 1, pp.55-76, 1980.

Esteban, M. and B. Perthame, “On the Modified Enskog Equation for Elastic and Inelastic Collisions,” Annuals of Institute of Poincaré, Vol. 8, pp.289-308, 1991.

Evans, L., and R. Rothery, “Perceptual Thresholds in Car-Following-A Comparison of Recent Measurements with Earlier Results,” Transportation Science, Vol. 11, No. 1, pp.60-72, 1977.

Fisk, C. F., “Effects of Heavy Traffic on Network Congestion,” Transportation Research Part B, Vol. 24, No. 5, pp.391-404, 1990.

Fisk, C. F., “Link Travel Time Functions for Traffic Assignment,” Transportation Research Part B, Vol. 25, No. 2, pp.103-113, 1991.

Foote, R. S., “Single Lane Traffic Flow Control,” presented at Second International Symposium on the Theory of Traffic Flow, Lincoln, England, June, 1963.

Forbes, T. W., “Effect of Small and Compact Cars on Traffic Flow and Safety,” Highway Research Board Bull, Vol. 351, pp.1-17, 1962.

Forbes, T. W., and M. E. Simpson, “Driver-and-Vehicle Response in Freeway Deceleration Waves,” Transportation Science, Vol.2, No. 1, pp.77-104, 1968.

Forni, G., R. Foa, A. Santoni and L. Frati, Editors, Cytokine Induced Tumor Immunogeneticity, Academic Press, 1994.

Freeway Management Handbook, Texas Transportation Institute, DOT, and FHWA, 1997.

Fritsche, H. T., “A Model for Traffic Simulation,” Traffic Engineering and Control, Vol. 35, 1994.

Gazis, D. C., R. Herman, and R. B. Potts, “Car-Following Theory of Steady-State Traffic Flow,” Operations Research, Vol. 7, pp.499-505, 1959.

Gazis, D. C., R. Herman, and R. W. Rothery, “Nonlinear Follow-the-Leader Models of Traffic Flow,” Operations Research, Vol. 9, pp.545-567, 1960.

Gazis, D. C., and C. H. Knapp, “On-Line Estimation of Traffic Densities from Time-Series of Flow and Speed Data,” Transportation Science, Vol. 5, No. 3, pp.281-301, 1971.

Gerlough, D. L., “Simulation of Freeway Traffic by an Electronic Computer,” Proceedings of 35th Annual Meeting of Highway Research Board, Washington, D. C., 1956.

Ghali, M. O. and Swith, M. J, “Optimal Dynamic Traffic Assignment of a Congested City Network,” Proceedings of the Second International Capri Seminar on Urban Traffic Network, Capri, Italy, July 1992.

Gipps, P. G., “A Behavioural Car-Following Model for Computer Simulation,” Transportation Research Part B, Vol. 15, No. 4, pp.105-111, 1981.

Greenberg, H., “An Analysis of Traffic Flow,” Operations Research, Vol. 7, pp.79-85, 1959.

Greenberg, H., and A. Daou, “The Control of Traffic Flow to Increase the Flow,” Operations Research, Vol. 8, pp.524-532, 1960.

Greenberg, I., “Pareto and Traffic,” Transportation Science, Vol. 4, No. 2, pp.217, 1970.

Greenshield, B., "A Study of Traffic Capacity,” Highway Research Board Proceedings, Vol. 14, pp.468-477, 1933.

Günther, M., A. Kalr, T. Materne, R. Wegener, “An Explicit Solvable Kinetic Model for Vehicular Traffic and Associated Macroscopic Equations,” appears in Computing Mathematical Applications.

Haberman, R., Mathematical Models~Mechanical Vibrations, Population Dynamics and Traffic Flow, Prentice-Hall, 1977.

Hauer, E., and V. F. Hurdle, “Freflo: Discussion of the Freeway Traffic Model,” Transportation Research Record, No. 722, pp.75, 1979.

Hattori, Y, T. Hashimoto and S. Inoue, “A Study for the Traffic Flow Control Considering Capacity of the Road by Cellular Automation Method,” IEEE SMC’99 Conference Proceedings, Vol. 4, pp.569-573, 1999.

Heikkiläand S. and V. Lakshmikantham, “Monotone Iterative Techniques for Discontinuous Nonlinear Differential Equations,” Marcel Dekker INC, 1994.

Helbing, D., “Improved Fluid-Dynamic Model for Vehicular Traffic,” Physical Review E, Vol. 51, No. 4, pp.3164-3169, 1995.

Helbing, D., “Derivation and Empirical Validation of a Refined Traffic Flow Model,” Physica A, Vol. 233, pp.253-282, 1996.

Helbing, D., “Gas-Kinetic Derviation of Speed-Flow Concentration Relationships Using Catastrophe Theory,” Physical Review E, Vol. 535, pp.2366-2381, 1996.

Helbing, D., “Modeling Multi-Lane Traffic Flow with Queueing Effects,” Physica A, Vol. 242, pp.175-194, 1997.

Helbing, D. and A. Greiner, “Modeling and Simulation of Multilane Traffic Flow,” Physical Review E, Vol. 55, No. 5, pp.5498-5508, 1997.

Helbing, D., “Empirical Traffic Data and Their Implications for Traffic Modeling,” Physical Review E, Vol. 55, pp. R25-R28, 1997.

Helbing, D., “MASTER: Macroscopic Traffic Simulation Based on a Gas-Kinetic, Non-Local Traffic Model,” Transportation Research Part B, Vol. 35, pp.183-211, 2001.

Herman, R., E. W. Montroll, R. B. Potts, and R. W. Rothery, “Traffic Dynamics: Analysis of Stability in Car Following,” Operations Research, Vol. 7, pp.86, 1959.

Herman, R. and G. Weiss, “Comments on the Highway-Crossing Problem,” Operations Research, Vol. 9, pp.828-840, 1961.

Herman, R., and T. Lam, “On the Mean Speed in the ‘Boltzmann-Like’ Traffic Theory: Analytical Derivation,” Transportation Science, Vol. 5, No. 3, pp.314-326, 1971.

Herman, R., and T. Lam, “On the Mean Speed in the ‘Boltzmann-Like’ Traffic Theory: A Numerical Method,” Transportation Science, Vol. 5, No. 4, pp.418-429, 1971.

Herman, R., and I. Prigogine, “A-Two-Fluid Approach to Town Traffic,” Science, Vol. 204, No. 13, pp.148-151, 1979.

Hilliges, M. , and M. Weidlich, “A Phenomenological Model for Dynamic Traffic Flow in Network,” Transportation Research Part B, Vol. 29, No. 6, pp.407-431, 1995.

Highway Capacity Manual, Special Report 209, TRB, National Research Council, Washington, DC, 1994.

Holland, H. J., “A Stochastic Model for Multilane Traffic Flow,” Transportation Science, Vol. 1, pp.184-205, 1967.

Holland, E. D., and A. W. Woods, “A Continuum Model for the Dispersion of Traffic on Two-Lane Roads,” Transportation Research Part B, Vol. 31B, No. 6, pp.473-485, 1997.

Hoogendoorn, S. P. and P. H. L. Bovy, “Modeling Multiple User-Class Traffic,” Transportation Research Record, Vol. 1644, pp.57-70, 1998.

Hoogendoorn, S. P., Multiclass Continuum Modelling of Multilane Traffic Flow, Doctoral Dissertation, University of Delft, 1999.

Hoogendoorn, S. P. and P. H. L. Bovy, “Continuum Modeling of Multiclass Traffic Flow,” Transportation Research Part B, Vol. 34, pp.123-146, 2000.

Hogweg, P. and P. Hesper, “The Ontogeny of the Interaction Structure in Bumble Bee Colonies,” Behavor Economical Sociobiol, Vol. 12, pp.171-183, 1983.

Hu, T. Y, and H. S. Mahmassani, “Day-to-Day Evolution of Network Flows under Real-Time Information and Reactive Signal Control,” Transportaiton Research Part C, Vol. 5, No. 1, pp.51-69, 1997.

Humi, M. and W. B. Miller, Boundary Value Problem and Partial Differential Equation, Pws-Kent Publishing Company, 1992.

Illner, R., A. Klar, H. Lange, A. Unterreiter and R. Wegener, “A Kinetic Model for Vehicular Traffic: Existence of Stationary Solution,” Journal of Mathematical Analysis and Applications, Vol. 237, pp.62-643, 1999.

Jager, E. and L. Segel, “On the Distribution of Dominance in a Population of Interacting Anonymous Organisms,” SIAM Journal of Applied Mathematics, Vol. 52, pp.1442-1468, 1992.

Jayakrishnan, R., and H. S. Mahmassani, “Dynamic Simulaion Assignment Methodology to Evaluate In-Vehicle Information Strategies in Urban Traffic Networks,” Proceedings of the 1990 Winter Simulation Conference, pp.763-769, 1990.

Jayakrishnan, R., W. K. Tsai, and A. Chen, “A Dynamic Traffic Assignment Model with Traffic-Flow Relationships,” Transportation Research Part C, Vol. 3, No. 1, pp.51-72, 1995.

Jin, W. Y. Zheng and J. Li, “Microscopic Simulation of Traffic Flow at Signalized Intersection Based on Cellular Automata,” Proceedings of IEEE International Vehicle Electronics Conference, Vol.1, pp.106-109, 1999.

Johnston, C. M., and, A. T. Chronopoulos, “A Communication Latency Parallelization of a Traffic Flow Simulation,” Proceedings of IEEE , pp.688-695, 1999.

Johnston, C. M., and, A. T. Chronopoulos, “The Parallelization of a Highway Traffic Flow Simulation ,” Proceedings of IEEE , pp.192-199, 1999.

Jou, Y. J., and S. C. Lo, “Modeling of Nonlinear Stochastic Dynamic Traffic Flow,” Transportation Research Record, No. 1771, pp.83-88, 2001.

Kagesawa, M. and T. Sadao, “Simulation of Road Traffic Management System with Dynamic Information,” 1994 Vehicle Navigation and Information System Conference Proceedings, pp.223-228, 1994.

Karandikar, R. L., and V. G. Kulkarni, “Second-Order Fluid Flow Models: Reflected Brownian Motion in a Random Environment,” Operations, Research, Vol. 43, No. 1, pp.77-88, 1995.

Karmeshu, and R. K. Pathria, “A Stochastic Model for Highway Traffic,” Transportation Research Part B, Vol. 15, No. 4, pp.285-294, 1981.

Kersch A. and J. Morokoff, Transport Simulation in Microelectronics, Birkhäuser, 1995.

Kerner, B. S. and P. Kohnhäuser, “Cluster Effect in Initially Homogeneous Traffic Flow,” Physical Reviews E, Vol.48, pp.2335, 1993.

Kerner, B. S. and P. Kohnhäuser, “Structure and Parameters of Clusters in Traffic Flow,” Physical Reviews E, Vol.50, pp.54-83, 1994.

Kirby, H. R., M. S. Dougherty and R. D. Watson, “Should We Use Neural Networks or Statistical Models for Short Term Motorway Traffic Forcasting?,” Proceedings of the second DRIVE-II Workshop on Short-Term Traffic Forecasting, 1994.

Klar, A., R. D. Kühne and R. Wegener, “Mathematical Models for Vehicular Traffic,” Surveys Mathematics Industry, Vol. 6, pp.215-232, 1996.

Klar, A. and R. Wegener, “Enskog-Like Kinetic Models for Vehicular Traffic,” Journal of Statistical Physics, Vol. 87, pp.91-114, 1997.

Klar, A. and R. Wegener, “A Hierarchy of Models for Multilane Vehicular Traffic I: Modeling,” SIAM Journal of Applied Mathematics, Vol.59, No. 3, pp.983-1001, 1999.

Klar, A. and R. Wegener, “A Hierarchy of Models for Multilane Vehicular Traffic II: Numerical Investigations,” SIAM Journal of Applied Mathematics, Vol.59, No. 3, pp.1001-1011, 1999.

Klar, A. and R. Wegener, “Kinetic Derivation of Macroscopic Anticipation Models for Vehicular Traffic,” SIAM Journal of Applied Mathematics, Vol.60, No. 5, pp.1749-1766, 2000.

Klein, M. J., “The Development of Boltzmann’s Statistical Ideas,” in The Boltzmann Equation Theory and Applications (Ed. by E. G. D. Cohen and W. Thirring), Spring-Verlag, New York, 1973.

Kühne, R. D., “Traffic Patterns in Unstable Traffic Flow on Freeways,” Highway Capacity and Level of Service, pp.211, 1991.

Kühne, R. D. and M. B. Rödiger, “Macroscopic Simulation Model for Freeway Traffic with Jams and Stop-Start Waves,” Proceedings of the 1991 Winter Simulation Conference, pp.762-770, 1991.

Lachowicz, M, “Asymptotic Analysis of Nonlinear Kinetic Equations – The Hydrodynamic Limit,” in Lectures Notes on the Mathematical Theory of the Boltzmann Equation, (Ed. by N. Bellomo), pp. 65-148, World Science, 1995.

Ladde, G. S., V. Lakshmikantham and A. S. Vatsala, Monotone Iterative Techniques for Nonlinear Differential Equations,” Pitman Publishing INC, 1985.

Lam, T. and R. Rothery, “The Spectral Analysis of Speed Fluctuations on a Freeway,” Transportation Science, Vol. 4, No. 3, pp.293-310. 1970.

Lampis, M., “On the Kinetic Theory of Traffic Flow in the Case of a Nonnegligible Number of Queueing Vehicles,” Transportation Science, Vol. 12, No. 1, pp.16-28, 1978.

Lattanzio, C. and P. Marcati, “The Zero Relaxation Limit for the Hydrodynomic Whitham Traffic Flow Model,” Journal of Differential Equations, Vol. 141, pp.150-178, 1997.

Leo, C. J., and R. P. Pretty, “Numerical Simulation of Macroscopic Continuum Traffic Models,” Transportation Research Part B, Vol. 26, No. 3, pp.207-220, 1992.

Leutzbach, W., An Introduction to the Theory of Traffic Flow, Springer-Verlag, Berlin, 1988.

LeVeque, R. J., Numerical Methods for Conservation Laws, Birkhäuser Verlag, 1992.

Lighthill, M. J., and G. B Whitham, “ On Kinematics Waves II. A Theory of Traffic Flow on Long Crowded Road ,” London, Proceedings Royal Society, A229, pp.317-345, 1955.

Liu, J. L., An Introduction to Mathematics in Semiconductor Industry, National Center for Theoretical Sciences, Taiwan, 2000.

Mahalel, D., and A. S. Hakkert, “Time-Series Model for Vehicle Speeds,” Transportation Research Part B, Vol. 19, No. 3, pp.217-225, 1985.

Mahmassani, H. S., G. L. Chang, and R. Herman, “Individual Decisions and Collective Effects in a Simulated Traffic System,” Transportation Science, Vol. 20, No. 4, pp.258-271, 1986.

Mahmassani, H. S., and Peeta, S., “Network Performance under System Optimal and User Equilibrium Dynamic Assignments: Implications for ATIS,” Transportation Research Record, No. 1408, pp. 83-93, 1993.

Mahmassani, H. S., Hu, T. and Peeta, S., “Microsimulation-Based Procedures for Dynamic Network Traffic Assignment,” Proceedings of PTRC Conference, London, UK, September 1994.

Makigami, Y., G. F. Newell, and R. Rothery, “Three-Dimensional Representation of Traffic Flow,” Transportation Science, Vol. 5, No. 3, pp.302-313, 1971.

Markowich, P. A., The Stationary Semiconductor Device Equations, Springer-Verlag INC, 1986.

May, A. D., Traffic Flow Fundamentals, Prentice Hall, Englewood Cliffs, NJ, 1990.

McClenahan, J. W., and H. J. Simkowitz, “The Effect of Short Cars on Flow and Speed in Downtwon Traffic: A Simulation Model and Some Results,” Transportation Science, Vol. 3, No. 2, pp.126-139, 1969.

Michalopoulos, P. G., G. Stephanopoulos, and V. B. Pisharody, “Modeling of Traffic Flow at Signalized Links,” Transportation Science, Vol. 14, No. 1, pp.9-41, 1980.

Michalopoulos, P. G., and V. Pisharody, “Plaoon Dynamics on Signal Controlled Arterial,” Transportation Science, Vol. 14, No. 4, pp.365-396, 1980.

Michalopoulos, P. G., G. Stephanopoulos, and G. Stephanopoulos, “An Application of Shock Wave Theory to Traffic Signal Control,” Transportation Research Part B, Vol. 15, No. 1, pp.35-51, 1981.

Michalopoulos, P., D. Beskos and Y. Yamauchi, “Multilane Traffic Flow Dynamics: Some Macroscopic Consideration,” Transportation Research Part B, Vol. 18, pp. 377-393, 1984.

Michalopoulos, P. G., P. Yi, and A. D. Lyrintzis, “Continuum Modelling of Traffic Dynamics for Congested Freeways,” Transportation Research Part B, Vol. 27, No. 4, pp.315-352, 1993.

Miller, A. J., ”Analysis of Bunching in Rural Two-Lane Traffic,” Operations Research, Vol. 11, pp.236-247, 1963.

Miller, A. J., “An Empirical Model for Multilane Road Traffic,” Transportation Science, Vol. 4, No. 2, pp.164-186, 1970.

Miller, H. J., Y. H Wu and M. C. Hung, “GIS-Based Dynamic Congestion Modeling to Support Time-Critical Logistics,” Proceedings of the 32nd Hawaii International Conference on System Science, pp.1-13, 1999.

Mitchell, A. R. and D. F. Griffitns, The Finite Difference Method in Partial Differential Equations, John Wiley and Sons Ltd., 1980.

Morton, K. W., Numerical Solution of Convection-Diffusion Problems, London, Chapman and Hall Inc., 1996.

Munjal, P. K. and L. A. Pipes, “Propagation of On-Ramp Density Waves on Nonuniform Unidirectional Two-Lane Freeways,” Transportation Research, Vol. 5, pp. 241-255, 1971.

Munjal, P. K., Y. S., Hsu and R. L. Lawrence, “Analysis and Validation of Lane-Drop Effects of Multilane Freeways,” Transportation Research, Vol. 5, pp. 257-266, 1971.

Nagatani, T., “Effect of Car Acceleration on Traffic Flow in 1D Stochastic CA Model,” Physica A, Vol. 223, pp. 137-148, 1996.

Nagel K. and M. Schreckenberg, “A Cellular Automaton Model for Freeway Traffic,” Journal of Physics I, Vol. 2, pp. 2221-2229, 1992.

Nagel, K. “Partical Hopping Models and Traffic Flow Theory,” Physical Review E, Vol. 53, No. 5, pp. 4655-4672, 1996.

Nelson, P., “On the Deterministic Developments of Traffic Stream Models,” Transportation Research Part B, Vol. 29, No. 4, pp.297-302, 1995.

Nelson, P. and A. Sopasakis, “The Prigogine-Herman Kinetic Model Predicts Widely Scattered Traffic Flow Data at High Concentrations,” Transportation Research Part B, Vol. 32, No. 8, pp.589-604, 1998.

Newell, G. F., “Mathematical Models for Freely Flow Highway Traffic,” Operations Research, Vol. 3, pp.176-186, 1955.

Newell, G. F., “Nonlinear Effects in the Dynamics of Car Following,” Operations Research, Vol. 9, pp.209-229, 1961.

Newell, G. F., “A Theory of Platoon Formation in Tunnel Traffic,” Operations Research, Vol. 12, pp.589-598, 1964.

Newell, G. F., “Some Properties of Presence Detectors,” Transportation Science, Vol. 4, No. 3, pp.311-321, 1970.

Newell, G. F., “A Simplified Theory of Kinematic Waves in Highway Traffic, Part I: General Theory,” Transportation Research Part B, Vol. 27, No. 4, pp.281-287, 1993.

Newell, G. F., “A Simplified Theory Of Kinematic Waves in Highway Traffic, Part II: Queueing at Freeway Bottlenecks,” Transportation Research Part B, Vol. 27, No. 4, pp.289-303, 1993.

Newell, G. F., “A Simplified Theory of Kinematic Waves in Highway Traffic, Part III: Multi-Destination Flows,” Transportation Research Part B, Vol. 27, No. 4, pp.305-313, 1993.

Newell, G. F., Theory of Highway Traffic Flow 1945-1965, University of California, Berkeley, 1994.

Nicholoson, H. and C. D. Swann, “The Prediction of Traffic Flow Volumes Based on Spectral Analysis,” Transportation Research, Vol. 8, pp.533-538, 1974.

Niver, E., K. C. Mouskos, T Batz and P. Dwyer, “Evaluation of the TRANSCOM’s System for Managing Incidents and Traffic (TRANSMIT),” IEEE Transzctions on Intelligent Transportation Systems, Vol.1, No. 1, pp.15-31, 2000.

Okutani, I., and Y. J. Stephanedes, “Dynamic Prediction of Traffic Volume through Kalman Filtering Theory,” Transportation Research Part B, Vol. 18, No. 1, pp.1-11, 1984.

Olcott, E. S., “The Influence of Vehicular Speed and Spacing on Tunnel Capacity,” Operations Research, Vol. 3, pp.147-167, 1955.

Oliver, R. H., ”Distribution of Gaps and Blocks in a Traffic Stream,” Operations Research, Vol. 10, pp.197-217, 1962.

Pant, P. D. and M. M. Polycarpou, “Intelligent Traffic Control System for Highway Work Zones,” Proceedings of the IEEE 1995 National Areospace and Electronic Conference, Vol. 1, pp.348-351, 1995.

Pao, C. V., “Block Monotone Iterative Methods for Numerical Solutions of Nonlinear Elliptic Equations,” Numerische Mathematik, Vol. 72, pp.239-262, 1995.

Papageorgiou, M., J. M. Blosseville, and H. Hadj-Salem, “Macroscopic Modelling of Traffic Flow on the Boulevard Peripherique in Paris,” Transportation Research Part B, Vol. 16, No. 2, pp.125-142, 1982.

Papageorgiou, M., “Some Remarks on Macroscopic Traffic Flow Modelling,” Transportation Research Part A, Vol. 32, No. 5, pp.323-329, 1998.

Patel, M. I. and N. Ranganathan, “An Intelligent System Architecture for Urban Traffic Control Applications,” Eighth IEEE Symposium on Parallel and Distributed Processing, pp.10-17, 1996.

Paveri-Fontana, S. L., “On Boltzmann-Like Treatments for Traffic Flow: A Critical Review of the Basic Model and an Alternative Proposal for Dilute Traffic Analysis,” Transportation Research, Vol. 9, pp.225-235, 1975.

Payne, H. J., “Freflo: A Macroscopic Simulation Model of Freeway Traffic,” Transportation Research Record, Vol.722, 1979.

Pearce, C. E. M., “Time Dependence in Commuter Traffic Models,” Transportation Science, Vol. 19, No. 4, pp.289-307, 1975.

Persaud, B., and V. O. Hurdle, “Freeway Traffic Data from Time-Lapse Film,” Transportation Science, Vol. 22, No. 4, pp.251-258, 1988.

Preziosi, L., “From Population Growth,” Mathematical Models Methodologies of Applied Science, Vol. 4, pp.107-120, 1994.

Pesheva, N. C., D. P. Daneva and J. G. Brankov, “Self-Organized Criticality in 1D Stochastic Traffic Flow Model,” Reports on Mathematical Physics, Vol. 40, No. 3, pp. 509-520, 1997.

Phillips, W. F., “Kinetic Model for Traffic Flow,” National Technical Information Service, Springfield, Virginia, 1977.

Pipes, L. A., “An Operational Analysis of Traffic Dynamics,” Journal of Applied Physics, Vol. 24, No. 3, pp.274-287, 1953.

Prigogine, I., and F. C. Andrews, ”A Boltzmann-Like Approach for Traffic Flows,” Operations Research, Vol. 8, pp.789-797, 1960.

Prigogine, I., and R. Herman, Kinetic Theory of Vehicular Traffic, American Elsevier Publishing Company, 1971.

Pue, A. J., “Macroscopic Traffic Models for Vehicle-Follower Automated Transportation System,” Transportation Research Part B, Vol. 16, No. 2, pp.125-142, 1982.

Pugachev, V. S., Theory of Random Functions, Addison-Wesley, 1965.

Reuschel, A., “Vehicles Moves in a Platoon,” Oesterreichisches Ingenieur-Archir, Vol. 4, pp.193-215, 1950.

Richards, P. I., “Shock Waves on the Highway,” Operation Research, Vol. 4, No. 1, pp.42-51, 1956.

Ricket, M., K. Nagel, M. Schreckenberg and A. Latour, “Two Lane Traffic Simulations Using Cellular Automation,” Physica A, Vol. 231, pp.534-550, 1996.

Ross, P., “Traffic Dynamics,” Transportation Research Part B, Vol. 22, No. 6, pp.421-435, 1988.

Sanwal, K. K., K. Petty, J. Walrand, and Y. Fawaz, “An Extended Macroscopic Model for Traffic Flow,” Transportation Research Part B, Vol. 30, No. 1, pp.1-9, 1996.

Segel, L. A. and A. S. Perelson, “Computations in Shape Space: A New Approach to Immune Network Theory,” in Theoretical Immunology, Part Two, SFI Studies in the Science of Complexity, (Ed. by A. S. Perelson), pp.321-342, Addison-Wesley, 1988.

Selberherr S., Analysis and Simulation of Semiconductor Devices, Springer-Verlag, Wein-New York, 1984.

Shimizu, H., M. Kobayashi and Y. Yonezawa, “An Analysis of Mean Link Travel Time for Urban Traffic Networks,” 2000 IEEE 51st Vehicular Technology Conference Proceedings, Vol. 1, pp.318-322, 2000.

Sheu, J. B, Stochastic Estimation of Lane-Changing Probabilities and Its Application to Incident Detection, Doctoral Dissertation, University of California, Irvine, CA., 1997.

Southwell, R. V., Relaxation Methods in Engineering Science Oxford University Press, London and New York, 1940.

Southwell, R. V., Relaxation Methods in Theoretical Physics Oxford University Press, London and New York, 1946.

Southwell, R. V., in Numerical Methods of Analysis in Engineering (L. E. Grinter, ed.), pp.66-74, Macmillan, London and New York, 1949.

Sullivan, D. P., and R. J. Troutbeck, “An Exponential Relationship for The Proportion of Free Vehicles on Arterial Roads,” Transportation Research Part A, Vol. 31, No. 1, pp.21-33, 1997.

Taylor, M. A. P., “Parameter Estimation and Sensitivity of Parameter Values in a Flow-Rate/Travel-Time Relation,” Transportation Science, Vol. 1, No. 3, pp.1275-292, 1977.

Todosiev, E. P. and L. C. Barbosa, “A Proposed Model for the Driver Vehicle System,” Traffic Engineering, Vol. 34, No. 17, 1963.

Trietsch, D., G. Y. Handler, “On Highway Fuel and Time Expenditures,” Transportation Science, Vol. 19, No. 3, pp.293-307, 1985.

Troutbeck, R. J., “Average Delay at an Unsignalized Intersection with Two Major Streams Each Having a Dichotomized Headway Distribution,” Transportation Science, Vol. 20, No. 4, pp.272-286, 1986.

Truesdell, C. and R. Mucaster, Fundamentals of Maxwell’s Kinetic Theory of a Simple Monoatomic Gas, Academic Press, 1980.

Underwood, R. T., “Speed, Volume, and Density Relationships, Quality and Theory of Traffic Flow,” Yale Bureau of Highway Traffic, New Haven, Conn., pp.141-188, 1961.

Van Aerde, M., INTERGRATION: A Model for Simulating Integrated Traffic Networks, Transportation System Research Group, 1994.

Vaughan, R. J., “The Distribution of Traffic Volumes,” Transportation Science, Vol. 4, No. 1, pp.97-110, 1970.

Wasielewski, P., “A Integral Equation for the Semi-Poisson Headway Distribution Model,” Transportation Science, Vol. 8, pp.237-247, 1974.

Wasielewski, P., “Car-Following Headways on Freeways Interpreted by the Semi-Poisson Headway Distribution Model,” Transportation Science, Vol. 13, No. 1, pp.36-55, 1979.

Wasielewski, P., “The Effect of Car Size on Headways in Freely Flowing Freeway Traffic,” Transportation Science, Vol. 15, No. 4, pp.364-378, 1981.

Weits, E., “Stationary Freeway Traffic Flow Modelled by a Linear Stochastic Partial Differential Equation,” Transportation Research Part B, Vol. 26, No. 2, pp.115-126, 1992.

Wegener, R. and A. Klar, “A Kinetic Model for Vehicular Traffic Derived from a Stochastic Microscopic Model,” Transport Theory and Statistical Physics, Vol.25, No. 7, pp.785-798, 1996.

Whitby, W. B., “Small-Car Speeds and Spacings on Urban Expressways,” Highway Research Board Bull, Vol. 351, pp.18-23, 1962.

Whitham, G. B., Linear and Nonlinear Waves, John Wiley & Sons Inc., 1974.

Wie, B. W., R. L. Tobin, D. Bernstein, and T. R. Freisz, “A Comparison of System Equilibrium Dynamic Traffic Assignments with Schedule Delays,” Transportation Research Part C, Vol. 3C, No. 6, pp.381-411, 1995.

Wie, B. W., and R. L. Tobin, “Dynamic Congestion Pricing Models for General Traffic Networks,” Transportation Research Part B, Vol. 32B, No. 5, pp.313-327, 1998.

Wiedemann, R., Simulation des Strabenverkehrsflusses. Schriftenreihe des Instituts für Verkehrswesen der Karlruhe, Vol. 8, 1974.

Wright, C. C., “Some Characteristics of Traffic Leaving a Signalized Intersection,” Transportation Science, Vol. 4, No. 4, pp.331-346, 1970.

Yserentant, H., “A New Class of Particle Methods,” Numerical Mathematics, Vol. 76, pp.87-109, 1997.

Zawack, D. J., and G. L. Thompson, “A Dynamic Space-Time Network Flow Model for City Traffic Congestion,” Transportation Science, Vol. 21, No. 3, pp.153-162, 1987.

Zhang, X., and D. F. Jarrett, “Stability Analysis of the Classical Car-Following Model,” Transportation Research Part B, Vol. 31, No. 6, pp.441-462, 1997.

Zhang, Y. and L. E. Owen, “An Advanced Traffic Simulation Approach for Modelling ITS Application,” 1998 IEEE International Conference on Systems, Man, and Cybernetics, Vol. 4, pp.3228-3233, 1998.

Zhang, H. M., “A Theory of Nonequilibrium Traffic Flow,” Transportation Research B, Vol. 32, No. 7, pp.485-498, 1998.

Zhang, H. M., “New Perspectives on Continuum Traffic Flow Models,” Networks and Spatial Economics, Vol. 1, No.1, pp.9-33, 2001.

# APPENDIX A

# Glossary of Symbols

| x | : location x=x,y in two dimensional space. |
| --- | --- |
| X | : spacing among vehicles. |
| t | : time. |
| Ω | : research domain before transformation. |
| Ω | : research domain. |
| k | : density. |
| ki | : density of user class i. |
| ks | : unrestrained density. |
| Ka | : adjust density, which depends on weather, road conditions and so on. |
| K0 | : essential density. |
| q | : flow density. |
| qi | : flow density of user class i. |
| Q | : flow. |
| u | : average velocity or group velocity. |
| ui | : average velocity or group velocity of user class i. |
| ue | : equilibrium velocity. |
| v | : individual velocity. |
| E | : traffic field. |
| F | : traffic force. |
| φ | : traffic potential. |
| ψ | : potential threshold. |
| ς | : proportion of average velocity to the difference between average velocity and equilibrium velocity. ςku=ku−kue |
| τm | : relaxation time of velocity. |
| τe | : relaxation time of velocity variance. |
| μ | : mobility, μ=τm/ς. |
| e | : passenger car equivalent. |
| ε | : interacting parameter. |
| Θ | : velocity variance. |
| Θe | : equilibrium velocity variance. |
| ν | : diffusion coefficient. ν=μΘe. |
| θa | : individual velocity variance. |
| θe | : average velocity variance. |

# APPENDIX B

# Partial Differential Equations

## B.1 Basic Theorems

Green’s Theorem. Let D be a bounded plane domain with a piecewise C1 boundary curve C=bdy D. Consider C to be parametrized so that it is traversed once with D on the left. Let p1x,y and p2x,y be any C1 functions defined on D=D∪C. Then

p2x−p1ydxdy=p1dx+p2dy                                 (B.1)

The line integral on the right side may also be written as p1,p2⋅tds, where t is the tangent vector along the curve and ds is the element of arc length. If p1,p2 is the velocity field of a fluid flow, the line integral is the circulation of the flow. If, for instance, D is an annulus, then bdy D is a pair of circles traversed in opposite directions.

A completely equivalent formulation of Green’s theorem is obtained by substituting p1=−g1 and p2=+g2. If g=g1,g2 is any C1 vector field in D, then g2x+g1ydxdy=−g1dx+g2dy. If n is the unit outward-pointing normal vector on C, then n=+dy/ds,−dx/ds. Hence Green’s theorem take the form

∇⋅gdxdy=g⋅nds,                                          (B.2)

where ∇⋅g=g1x+g2y denotes the divergence of g.

In three dimensions we have the divergence theorem, also known as Gauss’s theorem, which is the natural generalization of Eq. (B.2).

Divergence Theorem. Let D be a bounded spatial domain with a piecewise C1 boundary surface S. Let n be the unit outward normal vector on S. Let gx be any C1 vector field on D=D∪S. Then

∇⋅gdx=g⋅ndS,                                           (B.3)

where ∇⋅g is the three-dimensional divergence of g and dS is the element of surface are on S.

Superposition Principle. If L is a linear operator, Lu=0 is called linear. The advantage of linearity for the equation Lu=0 is that if u and v are both solutions, so is u+v. If u1,⋯,un are all solutions, so is any linear combination

c1u1+⋯+cnun=j=1ncjuj.                              (B.4)

where cj are constants.

## B.2 Boundary Conditions

From the physical intuition, it is necessary to specify some boundary condition if the solution is to be determined. The three most important kinds of boundary conditions are:

Dirichlet Condition. If a specific function is given, the boundary condition is called Dirichlet condition.

Neumann Condition. If the normal derivate is specified, the boundary condition is called Neumann condition.

Robin Condition. A boundary condition, which is given by the combination of Dirichlet and Neumann conditions.

# APPENDIX C

# A Brief Introduction of Boltzmann Equation

In the appendix, a brief introduction of Boltzmann equation is presented (Klein, 1973, Bellomo and Lo Schiavo, 1997). In 1866 Boltzmann began his scientific career with an attempt to give a purely mechanical explanation of the second law of thermodynamics. He gradually recognized the need to introduce statistical concepts in order to understand irreversibility and the second law. Now, Boltzmann equation, which is also called generalized kinetic models, is widely developed towards the modeling and simulation in applied science. These models are started in terms of nonlinear integro-differential equations and refer to systems constituted by a large number of interacting individuals. The model provides the evolution of the statistical distribution of the sate of the system.

As known (Cercignani, 1988, Cercignani, Illner and Pulvirenti, 1993, Truesdell and Mucaster, 1980, Bellomo, Palczewski and Toscani, 1988) the Boltzmann equation is an evolution equation for the one particle distribution function of a diluted gas of identical physical particles modeled as point masses. Macroscopic variables can be recovered from moments weighted by the distribution function. The evolution equation is derived from suitable balance equations, at a molecular level, and is an integro-differential equation with quadratic nonlinearities. Actually, the derivation requires some heuristic assumptions, that depend also on the particular gas examined and which have to be regarded as an approximation of physical reality.

By generalized model, we mean an evolution equation for the one body density distribution of a large population of identical interacting individuals. For instance, as in the Boltzmann equation, the distribution function may represent the number of particles that occupy the corresponding element of the phase space at the given time. On the other hand, the distribution function in models of traffic flow refers to the number of cars that at a certain time are found in an elementary tract of an arterial road. Analogously, in cellular dynamics, the distribution function represents the number of cells that at a certain time are found in an elementary volume with certain values of the activation of the cells themselves.

Generalized models are derived by means of arguments similar to those followed by the heuristic (or phenomenological) derivation of the Boltzmann equation. As we shall see, their derivation is based on a balance on phase space of a loss and a gain, which yields the evolution equation of the distribution function. The idea of Boltzmann equation is based on the one particle distribution function

f=fx,v,t: R3×R3×R+→R+,                                (C.1)

such that the total number of particles is

N=fx,v,tdxdv,                                          (C.2)

(R+ means, here and afterwards, the set (x∈R,x≥0) and in deriving an evolution equation for such a distribution. In deed if f is known such that vf as well as v2f are in L1R3×R3, then the macroscopic variables can be computed as expectation values of the corresponding microscopic functions. In particular,

kx,t=fx,v,tdv,                                           (C.3)

and

ux,t=1kx,tvfx,v,tdv,                                     (C.4)

are, respectively, the density and the velocity, while the mean translational energy is given by

Ex,t=1kx,tv2fx,v,tdv,                                   (C.5)

The mean energy can then related to the temperature by suitable assumptions on the thermodynamics of the system.

Actually, one has to accept that this type of modeling brings only to an approximation of physical reality. In particular and referring to the classic literature, the phenomenological derivation of the Boltzmann equation consists in equating the time derivative of f in a reference volume dxdv to the difference between the gain and loss terms of the particles which, due to the collisions enter into the said volume and leave it.

The above idea writes

dfdtdxdv=Gf,f−Lf,fdxdv,                                 (C.6)

In particular, the loss term is evaluated computing all collusions experienced by the test particle with velocity v in its encounters with the field particle with velocity w. Performing the calculations, that is integrating over the impact angular and linear variables a and b, yields

Lf,fx,v,tdxdv=dxdvw−vfx,v,tfx,w,tdabdbdw.      (C.7)

Similar calculations can be developed for the gain term, so that one obtains

Gf,fx,v,tdxdv'=dxdv'w'−v'fx, {v',tfx, {w',tda'b'db'dw'. (C.8)

Exploiting Liouville’s theorem of conservation of phase volume: dv'dw'=dvdw and suitable symmetry properties of the collision dynamics, a=a', b=b', and w−v=w'−v' yields the Boltzmann equation that is an evolution equation for the one particle distribution function. This model can be written as

∂∂t+v,∇x+E,∇vf=Jf,f=J1f,f−J2f,f,                (C.9)

where

J1f,fx,v,tdxdv=Bn,Vfx, {v',tfx, {w',tdndw             (C.10)

and

J2f,fx,v,tdxdv=Bn,Vfx,w,tdndw.                    (C.11)

The following notations have been used.

- E is the external field acting on each of the identical particles.

- n is the unit vector in the direction of the apse-line bisecting the angle between velocities V=w−v and V'=w'−v'.

- S+2 is the integration domain of n, S+2=n∈R3:n=1,n,V≥0.

- B is a collision kernel, which depends upon the interaction potential. Detained expressions of B can be recovered in the pertinent literature, e.g., Cercignani (1988).

- v, w are the precollision velocities of the test and field particles, respectively, and v', w' are the post-collision velocities, that are related to v and w by the relations v'=v+nn,V, w'=w−nn,V.

- denotes Cartesian inner product.

Solving the mathematical problems related to the Boltzmann equation provides the distribution function, and consequently, the macroscopic variables. We also recall that the equation Jf,f=0 is a functional equation, which admits the so-called Maxwellians equilibrium solutions. In the following paragraphs, the applications of Boltzmann equation are mentioned. The applications of Boltzmann equation for traffic flow theory are mentioned in Chapter 2. Thus, the related researches are neglected herein. The first one is the application of population dynamics.

Jager and Segel (1992) employed Boltzmann equation to describe the population dynamics firstly. They studied the evolution of an evolution of a physical state, called dominance, and characterized certain populations of insects. The experimental analysis of the behavior of these populations is due to Hogeweg and coworkers (Hogeweg, 1983). Some developments and generalizations of this model were studied in (Arlotti and Bellomo, 1995) with particular attention to the dynamics of several populations interacting with binary encounters, and to the qualitative analysis of the initial value problem. Further studies can be found in (Arlotti and Bellomo, 1996) where models with multiple interactions are proposed and the existence analysis of equilibrium solution is developed. In particular, the model consists of an evolution equation, in terms of a nonlinear system of integro-differential equations, on the probability density functions one per each population. To the applications of the model, there are two different types of applications can be mentioned. The first one is due to Jager and Segel and refers to the description of phenomenon of social behaviors of populations of anonymous interacting organisms; the second one refers to diffusion of epidemics. A phenomenon of social separations was observed and analyzed (Hogeweg, 1983), it was shown that for certain populations of insects, interactions divide the populations into dominating and dominated. Interactions modify the attitude to dominate. The application is developed in (Preziosi, 1996) and refers to diffusion of epidemics. Interactions modify the pathological state of individuals. In particular, the model shows that long time interactions may drive strong populations to overcome weak ones. In any case, the structure of the model is very interesting since it can be further developed to the analysis of phenomena concerning the competition between cells of the immune system and of an aggressive host.

A further development of Jager and Segel model leads to the modeling of extremely complex phenomena in theoretical immunology and, in particular, the competition between immune cells and tumor cells. The system is constituted by several interacting populations, where interactions not only modify the state of each cell, but can also generate destruction and proliferation of cells. The evolution of a cell is regulated by the genes contained in its nucleus (Forni et al., 1994). The genes are activated, or suppressed, by signals detected by receptors localized on the cell surface and transducted to its nucleus. Capability of recepting particular signals, can modify the usual behavior of a cell. In extreme situations, particular signals can induce a cell to reproduce itself in the form of identical descendants, that is the so-called clonal expansion, or to reproduce itself in the form of identical descendant, that is the so-called clonal expansion, or to die and disappear apparently without trace, that is the so-called apoptosis or programmed death. When the clonal expansion appears, a competition between tumor cells and cells of the immune system is activated. If the immune system is active and able to recognize and tumor cells, then it may hopefully be able to develop destruction mechanisms, that possibly end up with destruction of the aggressive host. Otherwise, tumor growth may develop progressively. Activation and disactivation of immune cells can be regulated by cytokine signals. A modeling of kinetic-type was proposed in two recent papers (Bellomo and Froni, 1994, Bellomo et al., 1996).

Boltzmann equation is also employed to describe the statistical distribution of the binding property of antigen (De Boer, et al., 1992, 1996). Cellular interaction produce modification of shape by mechanisms related to image recognition. It is of interest in the description of the time evolution of the shape distribution in populations of antigens. One of the purposes of modern theoretical immunology is to provide a detailed description of the above outlined physical phenomenon. A mathematical model towards such an objective was proposed by Segel and Perelson (1988) on the basis of a detailed analysis of the exchanges and interactions at a molecular level.

An interesting development of the Boltzmann equation towards applications in technology is the analysis of flows of disparate gas mixtures, say bubbles, droplets, clusters, or solid particles. The development of kinetic models is possible if the physical conditions characterizing the system are similar to the ones of the Boltzmann equation. In particular, the dimensions of the interacting particles need be small compared with the mean free path and/or of the vessel containing it. Referring to general aspects, we focus our attention on three features characterizing the system and the mathematical model. The first aspect refers to the size of the interacting particles. One can either model each element as a point and obtain a Boltzmann-like model, or by elements with finite size and in this case interactions occurs at a certain distance from the center of mass of the particles. The model is then developed in the same fashion of the Enskog equation. This means that two-body correlation function has to be introduced to modify the collusion frequency due to the fact that particles occupy part of the volume available for the collision. A second aspect refers to interactions between particles, which may be perfectly elastic or, in alternative, energy dissipative. Indeed, it is reasonable to include dissipation for particles, which have a large dimension with respect to the others. For instance, interactions of clusters may be such that part if the kinetic energy is converted into vibrational energy with some dissipation. Finally, the last point refers to modeling of the mixture; the model can either refer to a system of equal particles or to a disparate mass mixture (Esteban and Perthame, 1991, Araki and Tremaine, 1986).

The last application we are going to mention is the “quantum kinetic models”, which are Boltzmann-type evolution models such that local particle interactions are ruled by quantum mechanics rather than classical mechanics. This kind of modeling is motivated by technological applications related to the distribution of carriers in semiconductor devices. As known (Lachowics, 1995, Markowich, 1989, Kersch and Morokoff, 1995), the flow of carriers can be simulated by hydrodynamic type equations and, in particular, by the so-called Maxwell drift diffusion model. Experiments on modern technological devices show that the continuum description is not always valid, and kinetic type models may be necessary. However, although a description at the molecular scale appears necessary, the problem of an accurate modeling is still open due to the complexity of the hydrodynamic limit that is obtained when the Knudsen number tends to zero. The analysis should give a hydrodynamic description consistent with the microscopic description as an alternative to phenomenologically derived hydrodynamic models.

This appendix provides a background of Boltzmann equation so as to apply the model to applied science. With this in mind, the presentation of the contents does not claim to be complete. On the other hand, some effort was spent to include a large variety of examples in different branches of applied science and, for each subclass of models, deal with the following topics.

- Methodological framework towards modeling.

- Mathematical aspects arise by the models, e.g., initial and/or boundary value problems, and qualitative and computational analysis of their solutions.

- Review of applications known in the literature and perspectives for future developments.

As already mentioned, further research activity can be motivated by specific mathematical purposes. Indeed, each class of models generates interesting mathematical problems related both reference to each class of models, and several interesting and challenging problems have been shown to be open. Again we stress that very little has been done, even when a qualitative analysis was available, to relate ti to a quantitative analysis and simulations.

Although the literature on Boltzmann equation and closely related models in extensively developed, several mathematical problems are still open. In addition, very little is done on generalized kinetic models. A research line in this direction would be of great interest since it would significantly contribute in having macroscopic modeling of systems follow from microscopic information instead of phenomenological way, which may be not sufficiently motivated.

# APPENDIX D

# Vector Analysis

Current trends in mathematics continue to emphasize its applications to engineering and physics. In this appendix, the basic concepts and operation are mentioned in brief (Davis and Snider, 1987).

## D.1 Scalar and Vector

Scalar. If to each point x,y,z of a region in space there is made to correspond a number hx,y,z, we say that h is a scalar, for example, hx,y,z=x+2y−3z. In this study, density and flow are scalar.

Vector. Vector concept is closely related to the geometrical idea of a directed line segment. Roughly speaking a vector is a quantity that has direction as well as magnitude. For example, hx,y,z=h1i+h2j+h3k is a vector, where h1, h2 and h3 are scalar.

Gradient. The concept of gradient describes the rate of change of a scalar. The gradient of a scalar is defined to be the vector

gradh=∇hx,y,z=∂h∂xi+∂h∂yj+∂h∂zk.                              (D.1)

Divergence. There are two fundamental measures of changing of a vector: the divergence and curl. The divergence of a vector is a scalar, that tells us, at each point, the extent to which the function explodes, or diverges, from that point. The divergence of a vector is denoted by

divh=∇⋅hx,y,z=∂h∂x+∂h∂y+∂h∂z.                                 (D.2)

Curl. Another measure of a vector is curl, which expresses the tendency to swirl of a vector. The curl of a vector is a vector. The curl of a vector is denoted by

curlh=∇×hx,y,z=ijk∂∂x∂∂y∂∂zh1h2h3.                               (D.3)

## D.2 Vector Identities

The basic operation of vectors and scalar are summarized in sixteen vector identities in Table D.1.

Table D.1 Vector operation identities

| α denotes a scalar function, R denotes a vector function and A is any constant vector, g is any differentiable function of a single variable and β is a constant. |  |
| --- | --- |
| ∇α1α2=α1∇α2+α2∇α1 | (D.4) |
| ∇gα=dgdα∇α | (D.5) |
| ∇⋅R−A=β | (D.6) |
| ∇×R−A=0 | (D.7) |
| ∇R−An=nR−An−2R−A | (D.8) |
| ∇A⋅R=A | (D.9) |
| ∇⋅αR=α∇⋅R+R⋅∇α | (D.10) |
| ∇×αR=α∇×R+R×∇α | (D.11) |
| R1⋅∇R2−A=βR1 | (D.12) |
| ∇⋅R1×R2=R1⋅∇×R2−R2⋅∇×R1 | (D.13) |
| ∇×R1×R2=R2⋅∇R1−R1⋅∇R2+∇⋅R2R1−∇⋅R1R2 | (D.14) |
| ∇×∇×R=∇∇⋅R−∇2R | (D.15) |
| ∇⋅R1⋅R2=R1⋅∇R2+R2⋅∇R1+R1×∇×R2+R2×∇×R1 | (D.16) |
| ∇×∇α=0 | (D.17) |
| ∇⋅∇×R=0 | (D.18) |
| ∇⋅α1×α2=0 | (D.19) |

# Vita

Shih-Ching Lo was born in Taoyuan, Taiwan, on May 24, 1973. After graduation from The Affiliated Senior High School of National Taiwan Normal University, he entered National Chiao Tung University, Hsinchu, and received his Bachelor degree in Transportation Engineering and Management in June of 1995. He entered the graduate school of National Chiao Tung University in September of 1995. Then, he kept on studying his education in the Ph.D. program of Transportation Technology and Management of National Chiao Tung University in February of 1997. His major interests are Dynamic Traffic Flow Theory, Numerical Simulation, Intelligent Transportation Systems, Traffic Engineering, Physical Transport Theory and Network Traffic Flow. He received his Ph.D. degree in July of 2002.

| Address | ：No. 8, Lane 282, Wenhua St., Pingjen City, 324, Taiwan |
| --- | --- |
| Tel | ：+886-3-4923307 |
| Email | ：shihchinglo.tem84g@nctu.edu.tw |

# 簡    歷

| 中文姓名 | ：羅仕京 |
| --- | --- |
| 英文姓名 | ：Shih-Ching Lo |
| 籍貫 | ：台灣省新竹縣 |
| 出生日期 | ：民國62年5月24日 |
| 聯絡地址 | ：324平鎮市文化街282巷8 |
| 聯絡電話 | ：0931040227 |
| 電子郵件 | ：shihchinglo.tem84g@nctu.edu.tw |
| 學歷 | ： 民國91年6月 國立交通大學運輸科技與管理學系博士班畢業 民國86年2月 國立交通大學運輸工程與管理學系碩士班肄業 民國84年6月 國立交通大學運輸工程與管理學系畢業 民國80年6月 國立台灣師範大學附屬高級中學畢業 |