# 車流動力模式之構建與模擬—以波茲曼輸運方程為基礎

學生：羅仕京                                 指導教授：卓訓榮

國立交通大學運輸科技與管理學系博士班

# 摘  要

本研究提出一由波茲曼方程 (Boltzmann equation) 推導出巨觀動態車流模式之完整架構。波茲曼方程由Prigogine和Herman (1971)引用於描述動態車流，許多後續研究延續其觀念探討不同之車流行為。相關研究著眼於車流鬆弛行為、調整行為與互動行為之模式構建。由Prigogine和Herman所發展之車流波茲曼方程僅考慮了隨著時間與空間變化的相空間分布，但忽略了加速度之影響。Paveri-Fontana (1975) 修正了他們的模式，研究中加入加速度與隨時間變化之期望速度分配之影響。Helbing(1995, 1997, 2000)、Hoogendoorn與Bovy (1998, 2000, 2001)等人則延伸Paveri-Fontana之研究，推導出一系列之巨觀動態模式。然而，其研究中引用的二階動差函數在交通上並無實質意義。因此，本研究首先修正二階動差函數，以個別車輛之速度變異數代入模式推導當中，推導巨觀車流動力模式。關於車輛加速度，本研究藉由描述車輛間互動之交通場 (traffic field) 構建模式，所構建出之車流擴散模式 (traffic dispersion model) 除了描述加速度、車輛間之互動之外，亦在系統趨向均衡之假設下，將密度按均衡狀態分配 (equilibrium distribution) 於多車道道路上。本研究於二維平面空間探討多車道車流，主要優點在於可簡化討論車輛超車與變換車道行為，針對車道內不滿足跟車行為之車種，如：機車，亦能藉著二維空間之車流擴散模式描述。將車流擴散模式與有限空間限制條件結合，模式可應用至多種駕駛行為車流。將車流擴散模式與巨觀車流動力模式結合，即為動態多駕駛行為多車道車流模式。本研究亦發展模式求解之數值方法，所解之模式為偏微分方程系統，因此本研究引用分解演算法 (decoupling scheme) 進行模擬，並以簡化之漂移擴散模式 (drift-diffusion model) 為例探討。隨機動態模式構建為另一探討重點，鑒於模式之複雜度，模式推導時以多項式近似非線性項，期簡化隨機動態模式之計算複雜度。最後，本研究以動態未來研究方向與自我評述作為結論，期望作為相關研究之參考與基礎。

# Modeling and Simulation of Vehicular Kinetic Flow—from the Viewpoint of the Boltzmann Transport Equation

Student：Shih-Ching Lo                  Advisor：Dr. Hsun-Jung Cho

Department of Transportation Technology and Management

### National Chiao Tung University

# ABSTRACT

This study proposes a whole derivation of dynamic traffic flow model from Boltzmann equation to macroscopic models. Boltzmann-like equation is employed to describe dynamic traffic flow by Prigogine and Herman (1971). Many researches follow their concept and discuss different traffic behavior. Mostly, they endeavor to formulate the relaxation, adjusting and interaction between vehicles. Boltzmann-like equation developed by Prigogine and Herman only discussed distribution with time and spatial evolution but without acceleration effect. Paveri-Fontana (1975) modified their model and considered the influence of acceleration and time dependent desired speed distribution. Helbing (1995, 1997, 2000), Hoogendoorn and Bovy (1998, 2000, 2001) developed a series of macroscopic systems from Paveri-Fontana’s model. However, the second order moment function they employed does not have physical meaning in traffic flow. In this study, individual velocity variance is employed as the second moment function to modify the model. Furthermore, this study introduces the concept of traffic field, which describes the acceleration effect interacting among vehicles so as to distribute density on a multilane road, into Boltzmann equation. The multilane domain is considered as a two-dimensional space. The reason we consider a multilane domain as a two-dimensional space is that the driving behavior of road users may not be restricted by lanes, especially motorcyclists. According to the traffic field and the equilibrium assumption, a traffic dispersion model is obtained. The finite-space requirement of multiclass users can also be described by the dispersion model. By coupling the gas-kinetic model and the dispersion model, a new self-consistent multiclass multilane traffic model is presented. Numerical simulation methods are also developed in this study. Since the model is a system of partial differential equations, a decoupling scheme is suggested and employed to simulate the drift-diffusion model, which is a simplified model of the self-consistent model. Stochastic dynamic traffic flow modeling is another research topic discussed in this study. A decomposed polynomial is employed to approximate the nonlinear term of the model because of the computing complexity. Conclusions of this study and the perspectives of traffic flow researches are discussed in the end of this dissertation.

# Contents

| ## Abstract (in Chinese) ## Abstract ## List of Figures ## List of Tables ## CHAPTER 1 Introduction 1.1 Research Motivations 1.2 Framework of Research and Research Objectives 1.3 Research Approach 1.4 Overview of Thesis 1.5 Research Contributions CHAPTER 2 Literature Review 2.1 Static Models and Time Series Models 2.2 Car-Following Theory 2.3 The LWR-like models 2.4 The PW-like models 2.5 The Gas-Kinetic Models 2.6 Particle Hopping (Cellular Automation) 2.7 Brief Review of Finite Difference Approximation 2.8 Summary and Discussions ## CHAPTER 3 Definitions of Multi-Dimensional Traffic Flow 3.1 Two-Dimensional Traffic Flow 3.2 Three-Dimensional Traffic Flow 3.3. Summary ## CHAPTER 4 Traffic Dispersion Model 4.1 Linear Dispersion Model 4.2 Equilibrium Distribution and Nonlinear Dispersion Model 4.3 Applications of the Dispersion Model 4.4 Summary CHAPTER 5 Numerical Simulation of the Dispersion Model 5.1 Analytical Solution of Linear Dispersion Model 5.2 Monotone Iterative Scheme of the Traffic Dispersion Model 5.3 Numerical Results and Discussions 5.4 Summary ## CHAPTER 6 Modeling of Self-Consistent Vehicular-Kinetic Traffic Flow 6.1 Definitions 6.2 Continuity Equation (Conservation of Vehicle Numbers) 6.3 Motion Equation (Conservation of Momentum) 6.4 Variance Equation (Conservation of Energy) 6.5 Closure Relations 6.6 Multiclass Users Model 6.7 Summary CHAPTER 7 Numerical Simulation of Steady State Drift-Diffusion Traffic Flow Model 7.1 Drift-Diffusion Model 7.2 Decoupling Scheme 7.3 Discretization of the Model 7.4 Numerical Results and Discussions 7.5 Summary CHAPTER 8 Numerical Simulation of Time Variant Drift-Diffusion Traffic Flow Model 8.1 Brief Introduction of Time Variant Scheme 8.2 Numerical Scheme of Time Variant Drift-Diffusion Model 8.3 Discretization of the Model 8.4 Summary CHPTER 9 Modeling of Stochastic Dynamic Traffic Flow 9.1 Decomposition of Traffic Flow Model 9.2 Computation of Decomposition Polynomial 9.3 Stochastic Traffic Flow Model 9.4 Summary CHAPTER 10 Conclusions and Perspectives 10.1 Conclusions 10.2 Perspectives ## References APPENDIX A Glossary of Symbols APPENDIX B Partial Differential Equations B.1 Basic Theorems B.2 Boundary Conditions APPENDIX C A Brief Introduction of Boltzmann Equation APPENDIX D Vector Analysis D.1 Scalar and Vector D.2 Vector Identities ## Vita | I II VI IX 1 1 3 4 5 6 9 9 13 15 17 20 22 23 27 31 31 34 36 39 40 43 48 50 51 51 53 58 62 63 63 67 67 70 72 73 74 77 77 78 79 81 93 95 95 96 99 103 105 105 113 115 117 119 119 123 129 145 147 147 148 149 157 157 158 159 |
| --- | --- |

# List of Figures

| Figure 1.1 The architecture of dynamic traffic management modeling. Figure 1.2 Overview of the dissertation and the relationship between chapters. Figure 2.1 Research topics of macroscopic dynamic traffic model. Figure 2.2 Relations between microscopic, mesoscopic and macroscopic traffic flow models Figure 3.1 Two-dimensional traffic flow of a multilane highway. Figure 3.2 Three-dimensional traffic flow of a multilane highway. Figure 4.1 Figure of (a) elliptic traffic field (b) interacting between vehicles. Figure 4.2 Figure of circular traffic field after transforming. Figure 4.3 The density curves of different potential barrier and equilibrium velocity variance. Figure 4.4 The traffic field of the numerical example Figure 4.5 Motorcycles and cars mixed flow Figure 4.6 Self-consistent traffic flow modeling Figure 5.1 Flowchart of monotone iterative method Figure 5.2 The research domain and the unstrained density (ks) Figure 5.3 The potential of the numerical example Figure 5.4 The traffic field of the numerical example Figure 5.5 The density of the numerical example Figure 5.6 Plot of the convergence Figure 6.1 Modeling architecture and the relationship between models. Figure 7.1 A flowchart for decoupling scheme in traffic flow simulation. Figure 7.2 The plots of potential in one-dimensional numerical example Figure 7.3 The plots of density in one-dimensional numerical example Figure 7.4 The plots of flow in one-dimensional numerical example Figure 7.5 The plots of velocity in one-dimensional numerical example Figure 7.6 The plot of potential in two-dimensional numerical example Figure 7.7 The plot of flow density in x-direction in two-dimensional numerical example Figure 7.8 The plot of velocity in two-dimensional numerical example Figure 7.9 The plot of density in two-dimensional numerical example Figure 7.10 The plot of traffic field in two-dimensional numerical example Figure 7.11 The plot of flow density in two-dimensional numerical example Figure 7.12 The plot of unstrained density and the research domain in two-dimensional numerical example with lane-distinction Figure 7.13 The plot of potential in two-dimensional numerical example with lane-distinction Figure 7.14 The plot of density in two-dimensional numerical example with lane-distinction Figure 7.15 The plot of flow in two-dimensional numerical example with lane-distinction Figure 7.16 The plot of velocity in two-dimensional numerical example with lane-distinction Figure 7.17 The plot of traffic field in two-dimensional numerical example with lane-distinction Figure 7.18 The plot of flow density in two-dimensional numerical example with lane-distinction Figure 8.1 A flowchart for time variant drift-diffusion traffic flow model Figure 8.2 The plot of unrestrained density and research domain in one-dimensional time variant numerical example Figure 8.3 The plot of initial condition of potential in one-dimensional time variant numerical example Figure 8.4 The plot of initial condition of density in one-dimensional time variant numerical example Figure 8.5 The plot of potential in one-dimensional numerical example Figure 8.6 The plot of density in one-dimensional numerical example Figure 8.7 The plot of flow in one-dimensional numerical example Figure 8.8 The plot of velocity in one-dimensional numerical example Figure 10.1 Future studying topics and their relation in time sequential order Figure 10.2 The relation between the dynamic traffic flow model and the dynamic traffic information Figure 10.3 The relation between the dynamic link traffic flow model and the dynamic network traffic flow model | 2 7 28 30 34 36 42 43 47 48 49 50 57 60 60 61 61 62 76 79 83 83 84 84 86 86 87 87 88 88 89 90 90 91 91 92 92 97 100 100 101 101 102 102 103 125 126 127 |
| --- | --- |

# List of Tables

| Table 2.1 Table of single-regime models Table 2.2 Table of multi-regime models Table 2.3 Governing equations of car-following models Table 2.4 Explicit finite difference methods for Eq. (2.15) Table 2.5 Implicit finite difference methods for Eq. (2.15) Table D.1 Vector operation identities | 11 11 14 26 27 158 |
| --- | --- |