摘  要

採行彈性、錯開工作時間制度與徵收道路擁擠費，以分散尖峰交通需求達到減輕交通擁擠之效果是運輸需求管理(TDM)所強調的重要管理策略。本研究利用使用者均衡原理構建數學分析模式，探討單一瓶頸路段於不同工作時間制度(包括固定、彈性與錯開工作時間制度)及不同道路擁擠費收費方式(包括不收費、最佳與次佳階梯式收費)等分散尖峰擁擠策略對系統等候延滯等績效值之影響，並以簡例及敏感度分析進行測試。

由研究結果顯示，在工作時間制度策略方面，相較於固定工作時制，彈性工作時制對於等候延滯之減少為彈性時間長度與瓶頸路段擁擠時間長度之比例平方(e/S)2呈正比；均勻式錯開工作時制對於等候延滯之減少為錯開時間長度與擁擠時間長度之比例(d/S)呈正比。無論採彈性或均勻錯開工作時制，最大等候長度之減少均為彈性（錯開）時間長度與瓶頸路段擁擠時間長度之比例(e/S, (d/S))呈正比。至於分段式錯開工作時制，在人數控制得宜情況下(即分段人數安排為相互之錯開工作時間乘以瓶頸路段設施容量)，更可獲致較彈性或均勻式錯開工作時制更佳之效果。

在徵收擁擠費策略方面，於彈性工作時制下採階梯式擁擠費對系統等候延滯之減少，較固定工作時制更有效果。在均勻錯開工作時制下，其最適階梯式費率均較固定工作時制小。採最佳階梯式擁擠費會發生部分通勤者因不願繳擁擠費，而於收費站前暫時等待，直到費率降低時才通過之現象。本文另提出次佳階梯式擁擠費，將可完全消除此一等待現象，惟系統績效值較最佳階梯式略差。

關鍵詞：使用者均衡；固定工作時間；彈性工作時間；錯開工作時間；階梯式擁擠費；出發時間

# ABSTRACT

Flexible working schedules, staggered working hours and congestion tolls are highly emphasized tactics in transportation demand management (TDM) which are used to reshape peak traffic demand to reduce congestions. This study attempts to employ user equilibrium principle and queueing theory to formulate analytic models at a single bottleneck to evaluate and compare peak traffic delays resulted from such alternative work schedules and congestion tolls. Numerical examples and sensitivity analysis are performed.

In the subjects of working schedules, the results show that, traffic delays reduced by flexible working hours, compared with conventional fixed working hours, are in squared proportion to the flextime length - peak duration ratio (e/S)2. By contrast, the reduced traffic delay incurred by staggered working hours of uniform-type is linearly proportional to the ratio of staggered time length to peak hour period, (d/S). The maximum queue length reduced by flexible/staggered working schedules is also linearly proportional to the ratio of flexible or staggered time length to peak hour period, (e/S or d/S). Moreover, the staggered working hours of step-type can yield even better performance if the steps are optimized in such a manner that the staggered time multiplied by the bottleneck saturation flow rate controls the number of commuters in each step.

In the subject of congestion tolls, the results show that, the benefit brought by optimal step-tolls under flexible working hours is more than the ceiling of fixed working hours. The optimal step-toll fees under staggered working hours should be lower than those under fixed working hours. At optimal step-tolls, it appears that some commuters would be reluctant to pass through the toll station at the transitions from higher step-tolls to lower ones. Introducing suboptimal step-tolls will completely remove such “reluctant queues,” however the systems performance is slightly less effective than the optimal step-tolls.

Key Words: User equilibrium; Fixed working hours; Flexible working hours; Staggered working hours; Congestion step-tolls; Departure time

目   錄

中文摘要...............................................Ⅰ

英文摘要...............................................Ⅱ

目錄...................................................Ⅲ

表目錄.................................................Ⅵ

圖目錄.................................................Ⅶ

第一章  緒論

1.1  研究動機.......................................1

1.2  研究目的.......................................1

1.3  研究範圍與限制.................................2

1.4  研究方法與架構.................................3

1.5  研究內容與流程.................................6

- 文獻回顧

2.1出發時間.........................................9

2.2工作時間制度....................................16

2.3道路擁擠費......................................18

2.4動態使用者均衡模式..............................20

2.5  文獻綜合評析與啟示............................27

第三章  模式架構與分析方法

3.1  工作時間制度意義與種類........................30

3.2  道路收費類型..................................33

3.3  假設條件......................................35

3.4  分析方法......................................36

第四章  工作時間制度對通勤旅次延滯影響之分析模式建立

4.1  固定工作時間模式..............................40

4.2  彈性工作時間模式..............................42

4.3  錯開工作時間模式..............................44

4.4  模式比較與工作時間制度之關聯性................52

第五章  不同工作時間制度下課徵擁擠費對通勤旅次延滯影響之模式建立

5.1  固定工作時制下課徵擁擠費模式..................58

5.2  彈性工作時制下課徵擁擠費模式..................69

5.3  錯開工作時制下課徵擁擠費模式..................78

5.4  小結..........................................82

第六章  簡例及敏感度分析

6.1  工作時間制度..................................84

6.2  不同工作時間制度下課徵道路擁擠費..............90

6.3  政策涵義與執行因素............................98

第七章  通勤者旅次總成本函數之修正

7.1  函數之修正...................................103

7.2  工作時間制度.................................104

7.3  不同工作時間制度下課徵道路擁擠費.............110

7.4  小結.........................................128

第八 章  結論與建議

8.1  結論.........................................129

8.2  建議.........................................132

參考文獻..............................................134

附錄A  參(變)數一覽表................................A-1

表 目 錄

表2.1 通勤旅次均衡模式考量之範圍.....................22

表3.1  彈性工作時制的種類..............................32

表3.2 本研究模式考量之因素組成.......................37

表4.1 工作時間制度對通勤者旅次成本、總等候延滯與最大等候長度之比較.........................................55

表5.1 固定工作時制採行階梯式收費之最適擁擠費與系統績效比較..............................................68

表5.2　彈性工作時制採行階梯式收費之最適擁擠費與系統績效比較..............................................77

表6.1 不同工作時間制度之簡例計算結果...................86

表6.2 彈性與錯開工作時制之通勤旅次成本中之平均分項成本.86

表6.3 不同工作時間制度下課徵道路擁擠費之簡例計算結果...92

圖 目 錄

圖1.1  研究範圍說明.....................................3

圖1.2  研究架構圖.......................................5

圖1.3  研究流程圖.......................................8

圖3.1  工作時間型態示意圖..............................31

圖3.2  道路擁擠費類型..................................34

圖3.3  單一瓶頸路段示意圖..............................35

圖3.4　通勤者早到或遲到之時程延滯成本示意圖............38

圖4.1  固定工作時制之均衡到達曲線......................41

圖4.2　彈性工作時制之均衡到達曲線......................44

圖4.3　兩段式錯開工作時制之均衡到達曲線................46

圖4.4  最適三段式錯開工作時間之錯開安排(可遲到情況) .... 48

圖4.5  最適兩段式錯開工作時間之錯開安排(不可遲到情況) ..48

圖4.6　均勻式錯開工作時制之均衡到達曲線................50

圖4.7  均等多段式錯開工作時制之均衡到達曲線............51

圖4.8　最適多段式錯開工作時制之均衡到達曲線............52

圖4.9  各種工作時間制度之關聯性........................57

圖5.1  固定工作時制最佳階梯式(單段)收費均衡達曲線.......60

圖5.2  最佳階梯式收費(單段)之系統等候長度...............60

圖5.3  固定工作時制最佳階梯式(兩段)收費均衡到達曲線.....62

圖5.4  固定工作時制次佳階梯式(單段)收費均衡到達曲線.....65

圖5.5  彈性工作時制下隨時間變化之擁擠費率..............69

圖5.6  彈性工作時制最佳階梯式(單段)收費均衡到達曲線.....71

圖5.7  彈性工作時制次佳階梯式(單段)收費均衡到達曲線.....74

圖5.8  彈性工作時制下最佳階梯式與次佳階梯式收費(單段)之通勤旅次成本比較..................................78

圖5.9  錯開工作時制下隨時間變化之擁擠費率..............79

圖5.10  錯開工作時制最佳階梯式(單段)收費均衡到達曲線....80

圖5.11 錯開工作時制次佳階梯式(單段)收費均衡到達曲線.....82

圖6.1  不同工作時間制度之等候延滯變化情形..............87

圖6.2  彈性與錯開工作時間長度對系統等候延滯之影響......87

圖6.3  不同工作時制下瓶頸路段容量對系統等候延滯之影響..88

圖6.4  不同工作時制下早到時程延滯與等候延滯單位成本對系統等候延滯之影響..................................89

圖6.5  不同工作時制下早到與遲到時程延滯單位成本對系統等候延滯之影響......................................90

圖6.6  不同工作時間制度下課徵道路擁擠費之等候長度變化情形..............................................93

圖6.7  彈性工作制下採最佳階梯式收費之系統等候延滯改變情形..............................................94

圖6.8  不同工作時制採最佳、次佳階梯(分段數)收費之等候延滯..............................................94

圖6.9  不同工作時制與最佳、次佳階梯收費下彈性與錯開時間長度之等候延滯....................................95

圖6.10 不同工作時制與最佳、次佳階梯收費下瓶頸路段容量對系統等候延滯之影響................................96

圖6.11 不同工作時制與最佳、次佳階梯收費下早到時程延滯與等候延滯單位成本對系統等候延滯之影響..............97

圖6.12 不同工作時制與最佳、次佳階梯收費下早到與遲到時程延滯單位成本對系統等候延滯之影響..................98

圖7.1  通勤者早到與遲到成本(修正)之示意圖..............103

圖7.2  固定工作時制之均衡到達曲線(修正)................105

圖7.3  遲到特別懲罰成本對均衡到達曲線之影響...........106

圖7.4　彈性工作時制之均衡到達曲線(修正)................107

圖7.5　均勻式錯開工作時制之均衡到達曲線(修正)..........109

圖7.6  固定工作時制單段最佳階梯式收費均衡達曲線(修正)..113

圖7.7  固定工作時制單段次佳階梯式收費均衡到達曲線(修正) ............................................115

圖7.8  彈性工作時制單段最佳階梯式收費均衡到達曲線(修正) ............................................119

圖7.9 彈性工作時制單段次佳階梯式收費均衡到達曲線(修正)............................................122

圖7.10 錯開工作時制單段最佳階梯式收費均衡到達曲線(修正) ............................................126

圖7.11 錯開工作時制單段次佳階梯式收費均衡到達曲線(修正) ............................................128