Applying Fuzzy Multiobjective Combinatorial Programming Through Genetic Algorithm to Promote the Reaction Efficiency of Post-disaster Transportation Systems

Contents

1. Introduction

1.1 Research Motivation and Background		 1

1.2 Problem Definition and Purposes		 2

1.3 Research Limitation and Assumption		 3

1.4 Research Methods		 4

1.5 Research Process and Overview of Dissertation		 7

2. Literatures Review

2.1 Issues of Promoting the Reaction Efficiency in a Post-disaster Transportation System		 10

2.1.1 Northridge Earthquake		 21

2.1.2 Kobe Earthquake		 28

2.1.3 Crash of China Airline		 36

2.2 Fuzzy Multi-objective Programming		 39

2.2.1 Fuzzy Multi-objective Linear Programming		 40

2.2.2 Fuzzy Multi-objective Nonlinear Programming		 42

2.2.3 Fuzzy Multi-objective Bi-level Programming		 46

2.3 Meta-heuristics & Genetic Algorithm		 51

2.4 Network Equilibrium		 64

2.4.1 Separable Cost Models		 66

2.4.2 Non-separable Cost Models		 68

2.4.3 Non-optimization Models		 69

2.4.4 Computational Methods		 70

2.5 Discussions and Summary  		 72

3. Model Building and Implementation: Application of Two Cases

3.1 Case Introduction and Model Implementation		 74

3.1.1 Airport Firestation Problem		 74

3.1.2 Network Restoration Problem (NRP)		 76

3.2 Case Application Ⅰ– The Optimal Firestation Location at an Airport   		 81

3.2.1 Problem Characteristics and the Fire-protection Standards Suggested by ICAO  		 82

3.2.2 Model Building and Implementation by GA		 84

3.2.3 Results and Discussions		 97

3.3 Case Application II- The Network Restoration Problem (NRP)		 97

3.3.1 A NRP Model with single objective, traffic assignment and no work-troops 		 97

3.3.2 A NRP Model with multiple work-troops, fuzzy multi-objectives and no traffic assignment		 109

3.3.3 A NRP Model with multiple work-troops, multiple objectives and traffic assignment		 125

3.3.4 A NRP Model with multiple work-troops, fuzzy multi-objectives and traffic assignment		 138

3.4 Discussions and Summary		 143

4. Conclusions and Recommendations

4.1 Conclusions		 146

4.2 Recommendations		 147

References    		 151

### Appendix

A: The Assumed Data of Section 3.3.1		 165

B: The Assumed Data of Section 3.3.2		 169

C: The Assumed Data of Section 3.3.3		 171

D: The Assumed Data of Section 3.3.4		 172

Vita         		 173

Table List

Table 2.1 Relevant Literatures of Pre-disaster prevention, Disaster Response and Post-disaster recovery		 20

Table 2.2  Comparison of Popular Heuristics		 62

Table 3.1  The Airport Category of Fire-protection Ability		 83

Table 3.2  The Minimal Requirement of the Extinguishers at Each Airport Category		 84

Table 3.3  The Statistical Analysis of Accident at Taipei International Airport  		 93

Table 3.4  The Risk Rank of Area at Taipei International Airport		 94

Table 3.5  The Firestation Sites proposed by GA and Corresponding λ values  		 96

Table.3.6  The Assumed Traffic Demand in Figure 3.7		 107

Table 3.7  The Assumed Traffic Demand in Figure 3.22		 142

Figure List

Figure 1.1 Computation of Reaction Efficiency by Secured Property, Lives and Travel Convenience at each Time-interval		 3

Figure 1.2  The Research Process and Overview of Dissertation		 9

Figure 2.1  The Japan’s Damage-survey Process by Air and Satellites		 17

Figure 2.2  Northridge Earthquakes and their PGA Magnitude		 23

Figure 2.3  Slump of No. 14 Freeway		 24

Figure 2.4  Shear Failures of the Abutment of Connectors in Bridge I-5/14   		 24

Figure 2.5  Decimated Flares of Bridge 118/Mission-Gothic		 25

Figure 2.6  Shear Collars were Destroyed by Shear Failures		 25

Figure 2.7  Flexure-shear Failures of No. 118 and No. 210 Freeway		 26

Figure 2.8  Destroyed Bridges crossing Venice and La Cienega		 26

Figure 2.9  Shear Failures in Bridges crossing Fairfax and Washington		 27

Figure 2.10  Kobe Earthquakes and their Magnitudes		 28

Figure 2.11  Destroyed Road-networks in Kobe Earthquakes		 29

Figure 2.12  Collapsed Freeway between Kobe and Osaka		 30

Figure 2.13  Flexure-shear Failure in Collapsed Freeway between Kobe and Osaka		 30

Figure 2.14  Parallel Road-networks were Blocked by Slump Wastes		 31

Figure 2.15  Inner Lanes were Collapsed		 32

Figure 2.16  A Truck is Hampered by the Destroyed Pavement		 32

Figure 2.17  One-way Liquefaction of Road		 32

Figure 2.18  Tracks were Heavily Skewed by Shakes and Refugees were Evacuated by Walking and Riding Bicycles		 33

Figure 2.19  Tracks were Heavily Skewed by Shakes		 34

Figure 2.20  Fires Caused by Collision Accidents		 34

Figure 2.21  Liquefied and Cracked Harbor Areas		 35

Figure 2.22  Simulation of China Airline Crash		 37

Figure 2.23  Fire-fighters Searched and Rescued Lives on the Scene		 38

Figure 2.24  Fire-fighters Put off Fires on the Scene		 38

Figure 2.25  The Accident Area was in a Mess		 39

Figure 2.26  The Achievement Level for Fuzzy Objectives		 41

Figure 2.27  A strictly monotone decreasing function with respect to zi		 44

Figure 2.28  The Modified GA Process		 63

Figure 3.1 A Real Situation of Post-earthquake Road-network in Northridge		 78

Figure 3.2 The Trade-off between Total Setup Cost and Weighted Total Risk Cost		 86

Figure 3.3 The Achievement Level of Distance Constraint between any two Firestations		 88

Figure 3.4  The Achievement Level for Fuzzy Objectives (k=1,2,3)		 90

Figure 3.5  The Statistical Analysis of Air Crashes at the airport in the World  		 93

Figure 3.6  The Risk Rank Cubic Map of Taipei International Airport		 94

Figure 3.7  An Abstracted Situation of Post-earthquake Road-network		 99

Figure 3.8  A Heuristic Reconstruction Priority for Each Damage Point		 107

Figure 3.9  An Abstracted Situation of Post-earthquake Road-network		 110

Figure 3.10 Achievement Level of Minimizing the Travel-time for Travelers in a Road-network During Reconstruction		 111

Figure 3.11  Achievement Level of Minimizing the Total Time Needed for Reconstruction		 112

Figure 3.12  Achievement Level of Minimizing the Work-load Variance between any Two Work-troops		 113

Figure 3.13  An Assumed Situation of Post-earthquake Freeway-network		 116

Figure 3.14  A Heuristic Reconstruction Schedule for Each Work-troop		 125

Figure 3.15  The GA Evolution between Generations of the Best Run		 125

Figure 3.16  The Gene of a Reconstruction Plan		 131

Figure 3.17  The Crossover of a Reconstruction Plan		 133

Figure 3.18  The Mutation of a Reconstruction Plan		 133

Figure 3.19  A Heuristic Reconstruction Schedule for Each Work-troop		 136

Figure 3.20  The Modified GA Process		 138

Figure 3.21  The Achievement Level for Fuzzy Objectives (k=1,2,3)		 138

Figure 3.22  An Abstracted Situation of Post-earthquake Road-network		 140

Figure 3.23  A Heuristic Reconstruction Schedule for Each Work-troop		 143

Applying Fuzzy Multiobjective Combinatorial Programming Through Genetic Algorithm to Promote the Reaction Efficiency of Post-disaster Transportation Systems

Student: Chen Yuh-Wen                    Advisor: Prof. Tzeng Gwo-Hshiung

#### Abstract

Natural disasters or man-error often cause negative impacts to a transportation system. People injured or died in these disasters because of the inefficient reaction in a post-disaster transportation system. Past transportation-safety researches have focused almost exclusively or identifying the concepts of disaster prevention, disaster response and disaster recovery of a transportation system. But no further attention has been paid to the question of how reasonably, scientifically and logically secure the efficient reaction of a post-disaster transportation system. Since many activities of rescuing lives, e.g., fire-fighting, restoration, refugees’ evacuation, etc. are mainly supported by transportation systems (networks), if the reaction efficiency of a post-disaster transportation system is unacceptable, mass travelers, e.g., doctors, fire-fighters, policemen, work-troops for reconstruction, patients, etc. can’t be efficiently conducted via the destroyed transportation network. Thus, this study is far from many traditional transportation-safety researches, which heavily relied on conceptual introduction, statistical analysis or standards establishment. Thus, to promote the reaction efficiency of a post-disaster transportation system, we creatively apply the fuzzy multi-objective programming in such a study so as to implement appropriate strategies for promoting the reaction efficiency of a post-disaster transportation system. Two practical transportation-safety issues: airport firestation location and network restoration problem are considered in this study and resolved. Study results show that satisfying solutions can be easily derived by our modified genetic algorithms (GAs). Moreover, we confirm that our efforts devoted in this study can be a powerful decision support basis for pre-simulation against disasters.

Keywords: fuzzy, multi-objective, safety, firestation, network, genetic algorithms (GAs).

模糊多目標組合規劃基因演算法應用於提昇

運輸系統災後應變效率之研究

研究生: 陳郁文                        指導教授: 曾國雄博士

#### 摘要

自然或人為的災禍，如颱風、空難、車禍等對運輸系統常造成極大之負面影響。若運輸系統災後效率低落，則救災資源如民防團、消防隊、醫療小組、土木工程隊等將無法及時趕赴災區提供必要支援，民眾則因此遭受生命財產之巨額損失。以往運輸安全之研究多著重於災前、災時及災後應變復舊觀念之文字宣導，但應如何合理化、科學化及邏輯化地落實這些應變原則卻付之闕如。為合理化、系統化暨科學化地進行大規模救災及復舊，因此，本運輸安全研究不同於以往許多根據統計分析或建立標準應變程序之相關研究方法，而是創新地提出應用模糊多目標規劃以求解此類問題。復因大規模救災及復舊問題亦為一組合最佳化問題(Combinatorial Optimization Problems），而國際上風行之基因演算法 (GAs)具有平行搜尋及自我演化的特性，已在理論基礎上被證實在求解組合最佳化問題上最具效益且特別適合應用到大規模的實務問題上。故本研究應用基因演算法求解大規模救災及復舊問題，以提昇運輸系統災後應變效率。研究中並提出二個案進行探討，第一個案為機場最適消防站之區位規劃，冀使機場內事故損失最小化;第二個案為震災後路網復舊之最佳化排程，欲使破損路網修復時間最小化，且同時修復期間之路網交通擁塞最少。研究結果顯示，利用基因演算法於此二個案可簡便地求出其近似最佳解，而本論文將可提供決策單位研擬相關應變策略之科學基礎。

關鍵詞: 模糊多目標規劃、運輸安全、消防隊、路網、基因演算法 (GAs) 、震災復舊、組合最佳化。

國立交通大學

交通運輸研究所

博士論文

No. 21

模糊多目標組合規劃基因演算法應用於提昇

運輸系統災後應變效率之研究

Applying Fuzzy Multiobjective Combinatorial Programming Through Genetic Algorithm to Promote the Reaction Efficiency of Post-disaster Transportation Systems

研 究 生:陳郁文

指導教授:曾國雄 博士

中華民國八十八年七月

模糊多目標組合規劃基因演算法應用於提昇

運輸系統災後應變效率之研究

Applying Fuzzy Multiobjective Combinatorial Programming Through Genetic Algorithm to Promote the Reaction Efficiency of Post-disaster Transportation Systems

研 究 生:陳郁文                Student: Chen Yuh-Wen

指導教授:曾國雄 博士           Advisor: Tzeng Gwo-Hshiung

國立交通大學

交通運輸研究所

博士論文

A Dissertation

Submitted to Institute of Traffic and Transportation

College of Management

National Chiao Tung University

in Partial Fulfillment of the Requirements

for the Degree of

Doctor of Philosophy

in

Traffic and Transportation

July, 1999

Taipei, Taiwan, Republic of China

中華民國八十八年七月

誌 謝

這一路求學生涯走來，要感謝的人實在太多了。首先若無恩師曾國雄教授挖空心思暨三更半夜「廢寢忘時」之指導，今日就無法在此從容地寫出這篇誌謝了。另口試期間承蒙交通部民航局張有恆局長、台灣大學林建元教授、清華大學溫于平教授暨王小璠教授、交通大學張保隆教授暨林進燈教授、中央大學李允中教授等之指正，在此致上最深的謝意。

求學四年來，很感謝所長馮正民教授、藍武王教授、黃承傳教授、徐淵靜教授、陳武正教授、汪進財教授及黃台生教授在學問及做人處事上多所提攜及指導。回想修取博士學位來時路，雖然辛苦但卻覺得萬分充實。就如同恩師曾國雄教授所訓示：「博士學位只是求學過程的中點而非終點！」，未來還有太多的研究需進行，尚有許多的功課需去學習，我當秉持「學海無涯、唯勤近岸」的精神，繼續努力。正在求學之勁毅兄、成蔚兄、士展兄及麗敏學妹，學業尚未成功，同志仍須努力，祝福諸位心想事成！

老爹及老媽，辛苦您們了！這段求學生涯的低潮總是讓您們煩惱我許多不成熟的抱怨，如今「陳博士」已成真，希望老爹及老媽能分享這一份榮耀及喜悅。老弟，你也很快會當上「陳主任」啦，加油！小琳兒，感謝妳這一路陪我走來，帶給我許多歡樂，相信我們的美夢即將成真！最後，真的想感謝的人太多了，僅以此本論文獻給這些關心及幫助過我的諸位貴人，感謝您們給我的一切，我會記得並珍惜的，謝謝。