Abstract

Real-time coordinated ramp metering control and management is vital to development of advanced traffic management systems in ITS. This methodology presented here is divided into two parts, including: (1) a real-time coordinated ramp clustering, and (2) a multi-objective optimization with stochastic optimal control.

The proposed method executes a modified generalized sequential decision algorithm utilizing the strategic elements of traffic states those are estimated on the detectors. In the process of sequential decision making, the method can classify those ramps in the system into several groups.

In the multi-objective optimization with stochastic optimal control development, time-varying traffic state variables and control variables are specified. Following specification of system states, a stochastic model which comprises four types of equations, including: (1) recursive equations, (2) measurement equations, (3) boundary constraints, and (4) objective equations, is formulate. Based on the proposed stochastic model, a stochastic optimal control algorithm is then developed in an effort to update the time-varying control variables as well as traffic state variables in real time.

To generate efficiently traffic data used in model tests, an advanced microscopic traffic simulator, Paramics, Version 3.0, which is developed to model and analyze ITS traffic flow conditions is employed. The preliminary test results indicate that the proposed method is promising to accomplish the goal of real-time coordinated ramp metering control.

Expecting to have an efficient traffic management of highways, the research presented here propose a new method of real-time coordinated ramp metering control. We expect that will help to develop ITS technologies.

Keywords: Ramp Metering Control, Clustering Control, Multi-objective Optimization With Stochastic Optimal Control, Real-Time Prediction