3. Model Building and Implementation : Application of Two Cases

In this Chapter, we will introduce two practical cases: the location problem of airport firestation and network restoration problem (NRP), which are very important in transportation-safety field. First of all, backgrounds and basic models of these cases are briefly presented. Secondly, the formulation process and relevant GAs are also illustrated. Finally, computational results are showed and discussed to validate our optimization process.

3.1 Case Introduction and Model Implementation

3.1.1 Airport Firestation Problem

In many countries, the number of flight take-off and landings has increased proportionally with the gross domestic product (GDP). For example, the traffic at Taipei International Airport in Taiwan has greatly increased since 1988. Flights to Taiwan have already exceeded 80,000 flights in 1998. Thus, in the event of an accident, if the emergent reaction to airport accidents is not appropriate, injuries and deaths would also greatly increase. The emergent reaction mainly includes the fire protection and rescue mission, which are both executed against time by members of firestations at the airport. Thus, the relevant authorities have the duty not only to provide adequate resources of fire fighting and rescue, but also to provide these resources at minimal time to access the accident area.

An existing location model of firestations for any type of facility, is the expansion of location problem for a single objective or multi-objective. Also, relevant researches in the area which consider topics such as determining the ambulance-hospital locations (Berlin et al., 1976), locating the facilities by fuzzy multi-criteria (Bhattacharya et al., 1992), achieving maximal covering for locations (Church et al, 1991), analyzing the trade-off between cost and accessibility (Current et al., 1987), studying for essential air service program (Flynn and Ratick, 1988), locating new facilities in a competitive environment (Hakimi, 1983), expanding with multiple fuzzy goals (Hannan, 1981), finding the location for emergency facility (Toregas, 1971), etc. To solve our model for integrating the optimal number and sites of firestations, a number of optimizing techniques learned genetic algorithm (GA) has received a great deal of attention and performs well in many optimizing problems (Sakawa et al., 1997). For validating our model, a revised GA is applied. This GA is similar in spirit to that of Sakawa's work. Furthermore, the GA is useful in a large-scale location problem because of its self-evolution mechanism (Goldberg, 1989; Michalewicz, 1996).

To assist the relevant authority for an efficient reaction of fire protection and rescue, a well considered location of firestations at the airport is needed. An appropriate firestation location should bring the following benefits: (a) it can shorten the distance between firestations and the high risk area as to improve reaction efficiency; (b) firestations can be located by considering the interaction between all firestations– this can minimize the overlap of firestation sites, as to efficiently utilize the firestation resources; (c) it holds the ability to determine the reasonable number of firestations at the airport because that there will be an economical trade-off between the accident risk cost and the total set-up cost of the firestation. Based on the aforementioned reasons (a)-(c), the location model of firestation in this study is with a fuzzy multi-objective characteristic.

Since our airport firestation problem is with multi-objective characteristics, a general form of our model is shown as follows:

Min   z(x) = (z1(x),z2(x),...,zk(x))T               (3.1)

st     x∈A

where

zi(x): the objective function, i =1,2,…,k;

x: the decision variable, x is used to indicate a firestation is set on a location or not;

A: the feasible location set of firestation.

For each of the objective function zi(x), i =1,2,…,k; of this problem, assume that the decision maker has a fuzzy goal– the objectivezi(x)should be substantially less than or equal to some valuepi. Thus, the corresponding linear membership functionμiL(zi(x)) is defined as:

0         ; zi(x)≥zi−

μiL(zi(x))=   zi(x)−zi−zi+−zi−  ; zi+≤zi(x)<zi−                (3.2)

1         ; zi(x)≤zi+

where zi− denotes the objective value of pessimistic expectation by a decision maker, and zi+ denotes the objective value of optimistic expectation by a decision maker. Using such linear membership functionμiL(zi(x)), i =1,2,…,k; and apply the λoperator (see Section 2.2), the original problem can be changed as:

Max λ                               (3.3)

st  λμiL(zi(x)), i =1,2,…,k;

x∈A

Moreover, the GA is used to make x undergo crossover, mutation,…,etc. so as to let the Equation (3.3) has the largest λ– these operational details will be illustrated in Section 3.2.

3.1.2 Network Restoration Problem (NRP)

On January 17, 1995, the Hyogo-Ken Nambu earthquake shook the Kobe, Japan, causing a tremendous loss of life and property, including extensive damage to the city’s transportation network. Thus, the destroyed transportation network leads to traffic chaos  (Jain, et al., 1996). Moreover, on January 17, 1994, Northridge California, earthquake threw people from their beds, shifted transportation networks off their foundations and seriously decimate the road-network system (Finn, et al, 1995). Therefore, the commuters in Northridge had to spend at least quadruple travel-time than a normal working day in a post-quake road-network. If the strategy for road-network restoration is not available and effective, mass travelers will waste their valuable working time because of the congested traffic.

We use assumed seismic situation as a numerical example to show how we formulate our model and resolve the network restoration problem. As we know, Taiwan, Japan and America are all located in the vivid earthquake region, where big quakes are possible to occur. The post-earthquake experiences of Northridge and Osaka taught us that if the reconstruction strategies coping with seismic situation were not available, mass travelers couldn’t be efficiently conducted via the post-earthquake road-network. Nevertheless, with the purpose of promoting transportation effectiveness for the post-earthquake road-network in Taiwan, a well considered reconstruction plan for road network has become a must.

To design an appropriate reconstruction plan of seismic road-network, first of all, we should take the damage characteristics of seismic road-network into consideration. Many damage points will be scattered throughout the post-earthquake road-network- this real situation of Northridge earthquake in America is shown in Figure 3.1.

The Figure 3.1 is copied from special reports of 1994 Northridge earthquake (Finn, et al., 1995), where the black spot denotes the scale of the earthquake measured by local earthquake-meter, and the gray spot shows the slump area of the road-network. Thus, the reconstruction priority for each damage point should be reasonably arranged as to maximize the reconstruction effectiveness of a post-earthquake road-network. The reconstruction effectiveness can be maximized by achieving the shortest total reconstruction time and minimal inconvenience for travelers on a road-network during reconstruction periods.

Figure 3.1 A Real Situation of Post-earthquake Road-network in Northridge

Secondly, a reconstruction priority model of a post-earthquake road-network is a combinatorial optimization problem, which should be sophisticatedly developed and solved. The priority model for scheduling in our study, in actuality, is the expansion of a scheduling model to incorporate the shortest travel time of travelers during reconstruction perids. We can easily find relative researches by many authors, for example, there is scheduling problem solved by the branch and bound method (Demeulemeester and. Herroelen, 1992; Stinson, 1979), by the genetic approach (Gonway and Venkataramanan, 1994; Ishibuchi, et al., 1994; Houck, et al., 1996; Kawakami, et al., 1994; Sato and Ichii, 1996), by the multi-objective approach (Mohanty and Siddiq, 1989; Patterson and Roth, 1976; Prisker, et al., 1969; Kurtulus and Narula, 1985), and so on. The original concept of genetic algorithm (Mitchell, 1997; Goldberg, 1989) is also applied in this study to obtain a heuristic solution from the fuzzy multi-objective scheduling model mentioned above. Although aforementioned problems related to scheduling are well known to be combinatorial optimization problems (Sakawa, et al., 1994; Sakawa, et al., 1997; Michalewicz, 1996), but by the recently-developed genetic algorithm, a satisfying solution can be more easily and efficiently derived.

Thirdly, this study is far different from the traditional scheduling model in that the reconstruction state of damage point will dynamically affect the traffic volume in the road-network during reconstruction; thus, a bi-level model is employed to solve our optimal reconstruction plan (LeBlanc and Boyce, 1986). The upper level of this bi-level model is the priority schedule model, and the lower level is the model of asymmetric traffic assignment (Nagurney, 1984, 1986, 1993; Smith, 1979, 1983). An asymmetric traffic assignment is proved to be more appropriate for the congested traffic (Dafermos, 1980, 1984; Dafermos and Nagurney, 1984)― this congested situation will obviously happen on a broken road-network after a large earthquake. Thus, our model is somewhat similar to the network design problem (NDP), but is more complicated than the traditional NDP (LeBlanc and Boyce, 1986). Because of aforementioned resolution complexity, the revised genetic algorithm (Mitchell, 1997; Goldberg, 1989) is also applied in this study as to obtain a heuristic solution. Although our problem related to scheduling are well known to be combinatorial optimization problems, but by the recently-developed genetic algorithm, a heuristic solution can be more easily and efficiently derived compared with traditional random search (Sakawa, et al., 1994; Sakawa, et al., 1997; Michalewicz, 1996).

Therefore, assuming that a decision maker has fuzzy expectation for the objectives in the upper level of our bi-level model (see Section 2.2.3), and the corresponding linear membership functionμi(zl,i(x)) is defined as:

0         ; zl,i(x)≥zl,i−

μi(zl,i(x))=   zl,i(x)−zl,i−zl,i+−zl,i−  ; zl,i+≤zl,i(x)<zl,i−        (3.4)

1         ; zl,i(x)≤zl,i+

where zl,i− denotes the objective value in the upper level by a decision maker’s pessimistic expectation, and zl,i+ denotes the objective value in the upper level by a decision maker’s optimistic expectation. Here, we introduce the original fuzzy bi-level programming by Equation (2.23) again:

Maxx λ

where y solves                                 (2.23)

Miny  zf(x,y)

st  λμi(zl,i(x,y)), i =1,2,…,k

x,y∈XY =x,y∈Rn|gj(x,y)≤0,j=1,2,...,m

Since the traffic assignment problem in the lower level in our NRP model is with no clear objective function– because it is in a Variational Inequalities Problem (VIP) form (Roy, 1991), thus we extend Equation (2.38) by replacing Miny zf(x,y). Please recall Equation (2.38) as follows:

Find f∈Ω, such that:

aca(f)(fa−fa)≥0,∀f∈Ω                                   (2.38)

Ω=

The fuzzy multi-objective bi-level model is shown as follows:

Maxx λ

where f satisfies Equation (2.38)                   (3.5)

st  λμi(zl,i(p,f)), i =1,2,…,k

p,f∈PF =p,f∈Rn|gj(p,f)≤0,j=1,2,...,m

where

zl,i(p,f): the objective function of the upper level (leader), i = 1,2,…,k;

p, f: the decision variable, p is used to indicate the restoration priority and f is used to express the traffic flow in network equilibrium of Equation (2.38);

PF: the feasible solution set;

gj(p,f): the inequality constraint, j = 1,2,…,m.

Equation (3.5) is much harder to be resolved than that in Equation (3.3), because the solution space of Equation (3.5) is unclear and unknown. Thus, we use modified GA to help find satisfying solutions (see Chapter 3.3).

3.2 Case Application Ⅰ– The Optimal Firestation Location at an Airport

In the previous section 3.1.1, the background, purpose, literature reviews and methods of solving problems related to the location problem are briefly discussed. In Section 3.2.1, we describe the problem characteristics and the International Civil Aviation Organization (ICAO) standards of fire protection.  In Section 3.2.2, the mathematical model is constructed and a resolution approach is proposed. In Section 3.2.3, a revised GA is employed and the International Airport of Taiwan is introduced to illustrate the practicability and effectiveness of our model. A comparison of resolution efficiencies between the total enumeration method and our revised GA is performed. Finally, a short discussion is presented.

3.2.1 Problem Characteristics and the Fire-protection Standards Suggested by ICAO

Today's airplanes carry hundreds of passengers on each flight. If an airplane crashes at a location near the airport, lacking the appropriate strategies of fire fighting and rescue for the crashed plane could be disastrous. Since the members of firestations at the airport are responsible for fires and rescuing lives, a well performed fire protection and well considered sites for firestations can help the relevant authorities in improving reaction efficiency. First, a well-performed fire protection is achieved by obeying the standards suggested by ICAO. The ICAO concept classifies airports into specified levels by the number of flights at an airport. Each level corresponds in turn to a level of fire protection resources (e.g., number of fire engine, amount of fire extinguishers, etc.) recommended by ICAO. The ICAO standards are briefly illustrated as follows:

(a) Categories of fire-protection

The category are classified into ten levels, and the classification rule depends on the airplane size, flights taking off and landing, and the length and width of the aircraft body in an airport– these categories are shown in Table 3.1.

It is obvious that a bigger airplane will be categorized at a higher fire-protection level. This also implies that we should prepare more fire-protection resources for saving passengers in a larger aircraft body.

(b) Extinguishers to be equipped

Table 3.2 gives the minimal requirement of extinguishers at an airport at each fire protection level (from 1 to 10). Level A in Table 3.2 is the stricter standard for fire-protection levels of airports.

Table 3.1 The Airport Category of Fire-protection Ability

| Airport Category | Length of Airplane（m） | Width of Airplane（m） |
| --- | --- | --- |
| 1 | 0-9 | 2 |
| 2 | 9-12 | 2 |
| 3 | 12-18 | 3 |
| 4 | 18-24 | 4 |
| 5 | 24-28 | 4 |
| 6 | 28-39 | 5 |
| 7 | 39-49 | 5 |
| 8 | 49-61 | 7 |
| 9 | 61-76 | 7 |
| 10 | 76-90 | 8 |

Source: ICAO, 1995

(c) Reaction time

The tolerable reaction time for an airplane accident should be not longer than three minutes. This reaction time is the time needed for 50% of available fire engines arriving at the accident area and executing fire fighting and rescue. Furthermore, according to our on – scene survey, the current reaction time at Taipei airport (See Figure 3.6) is acceptable.

Table 3.2  The Minimal Requirement of the Extinguishers at Each Airport Category

|  | A Level |  | B Level |  | Complementary Agents |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Airport Category | Water(ℓ) | Discharging Rate(ℓ/min) | Water(ℓ) | Discharging Rate(ℓ/min) | Chemical Powder (kg) | Halons (kg) | Carbon Dioxide (kg) |
| 1 | 350 | 350 | 230 | 230 | 45 | 45 | 90 |
| 2 | 1000 | 800 | 670 | 550 | 90 | 90 | 180 |
| 3 | 1800 | 1300 | 1200 | 900 | 135 | 135 | 270 |
| 4 | 3600 | 2600 | 2400 | 1800 | 135 | 135 | 270 |
| 5 | 8100 | 4500 | 5400 | 3000 | 180 | 180 | 360 |
| 6 | 11800 | 6000 | 7900 | 4000 | 225 | 225 | 450 |
| 7 | 18200 | 7900 | 12100 | 5300 | 225 | 225 | 450 |
| 8 | 27300 | 10800 | 18200 | 7200 | 450 | 450 | 900 |
| 9 | 36400 | 13500 | 24300 | 9000 | 450 | 450 | 900 |
| 10 | 48200 | 16600 | 32300 | 11200 | 450 | 450 | 900 |

Source: ICAO, 1995

(d) Firestations

The firestations should be adequately established. Furthermore, the sites of firestations should provide fire engines reach to the accident area in minimum time.

To sum up, the ICAO standards provide a principle for promoting the reaction efficiency of fire fighting and rescue, but ICAO did not give a clear solution to the relevant authority how to decide the optimal number and sites of firestation at an airport. Thus, the problem to be solved in this study is trying to find the optimal number and sites of firestation at an airport so as to meet ICAO standards. This problem will be mathematically formulated in next section.

3.2.2 Model Building and Implementation by GA

To integrate the optimal number and sites of firestations, many objectives need to be considered during optimization. The concept and formulation are illustrated as follows:

3.2.2.1 Model Construction

A location model is established for optimizing the number and sites of firestations. Decision variables for this model are of the binary (0-1) type– a firestation will be set up at the corresponding coordinate if this variable equals 1. This model objectives and constraints are illustrated in detail as follows:

(a) Minimizing the sum of corresponding costs from firestation and risk costs

In sufficient number of firestations will lead to inefficient reaction times ― the loss caused by reacting inefficiently may be defined as the total risk cost (TRC). If there is no firestation to be set up at the airport, the total risk cost will equal to TRC; on the contrary, the total risk cost can be reduced when more firestations are set up. Meanwhile, overestimating the need for firestations will also result in wasting of fire protection resources. Therefore, there will be an optimal number of firestations after the trade-off between the total setup cost (TSC) from firestations and total risk cost is weighted by negative exponential form. The conceptual cost resulting from the total setup cost of firestation plus the total risk cost should be minimized, this concept is displayed in Equation (3.6).

Min  f1＝ijSij×SC+TRC×e−ijSij                           (3.6)

- where,

- Sij: the decision variable, if a firestation is set up on a x-y coordinate (i, j), then Sij=1, otherwise, Sij=0; and ijSij≥1;

- SC: the setup cost for a single firestation;

- TRC: the total risk cost.

- Moreover, although Equation (3.6) is subjectively established by assumption, Equation (3.6) is theoretically reasonable because that f1 has a U-Shape curve ― this represents a trade-off between the TSC and weighted TRC. A simple example in Figure 3.2 indicates the optimal number of firestation is 2 when we assume TRC=7 and SC=1.

- Figure 3.2 The Trade-off between Total Setup Cost and Weighted Total Risk Cost

(b) Minimizing the farthest distance from the firestation to any point at the airport

This objective considers the accessibility from any firestation to any point at the airport. This objective is shown in Equation (3.7).

- Min  f2＝(i,j)|Sij=1,∀i,j                         (3.7)

- where,

- Sij: the decision variable, if a firestation is set up on a x-y coordinate (i, j), then Sij=1, otherwise, Sij=0;

- x,y : if Sij =1, then x = i and y = j

(c) Minimizing the longest distance from any firestation to the higher risk area.

The distance from any firestation to the higher risk area should be minimized in order to increase reaction efficiency. This objective function is shown in Equation (3.8).

Min  f3＝Max(i,j)|Sij=1ijrijx−i+y−j                          (3.8)

- where,

- Sij: the decision variable, if a firestation is set up on a x-y coordinate (i, j), then Sij=1, otherwise, Sij=0;

- x,y : if Sij =1, then x = i and y = j.

- rij: the risk rank for x-y coordinate (i, j).

The risk rank rij, is computed based on accident statistics from different areas at the airport ― this will discussed in detail later.

In addition to the aforementioned three objectives, the first constraint in our model applied to the summation of Sij should be greater than or equal to 1. This implies that we should set up at least one firestation at the airport. This is shown in Equation (3.9).

- ijSij≥1                                                     (3.9)

- where,

- Sij: the decision variable, if a firestation is set up on a x-y coordinate (i, j), then Sij=1, otherwise, Sij=0;

- Secondly, from a theoretical point of view, there should be a reasonable distance (dabr) between any two firestations; e.g., firestation a and firestation b. This distance should not only be too long (dabl) for firestations failing to support each other, but also not be too short (dabs) for emphasizing the defectives of overlap for firestation sites. The dabr, dabl and dabs are utilized to construct fuzzy constraints. This membership function μd(dab) of distance between any two firestations is shown in Equation (3.10) and Figure 3.3

μd(dab)= 1,                  if  dab= dabr

μd(dab)=(dabl-dab)/ (dabl- dabr),  if  dabldab > dabr           (3.10)

μd(dab)=(dab-dabs)/ (dabr- dabs),  if  dabsdab < dabr

μd(dab)= 0,                   otherwise

Achievement Level

1.0

Distance between any Two Firestations

0

Figure 3.3 The Achievement Level of Distance Constraint between any two Firestations

Let {Sij} denote the set ofSij=1, let also a and b be any two different elements taken from {Sij}, then the aforementioned constraint is shown in Equation (3.11).

xa−xb+ya−yb≈dabr                                          (3.11)

where,

xa: the i value of element a taken from {Sij};

xb: the i value of element b taken from {Sij};

ya: the j value of element a taken from {Sij};

yb: the j value of element b taken from {Sij};

: the symbol of fuzzy equal.

Furthermore, the i and j values are expressed in a x-y coordinate form. This presentation coincides with the cubic map for rescuing people in an airport. In the next section, an international airport is taken to represent a numerical example, which shows the establishment and workings of our model.

3.2.2.2 Fuzzy Multi-objective Optimization Model

A fuzzy multi-objective approach is employed for the following reasons: first, the model should be simutaneously optimized for the number and sites of firestation at the airport. Thus, the techniques for optimizing only one single objective will not be suited for such a problem(Cohon, 1976; Fendel and Spronk, 1983; Yu, 1985; Zeleny, 1982). Secondly, the fuzzy multi-objective approach is quite simpler when compared with traditional weighing method for multi-objective optimization (Bellman and Zadeh, 1976; Fedrizzi et al., 1991; Lai and Hwang, 1994; Sakawa, 1993; Sakawa et al., 1997). Finally, the fuzzy multi-objective approach is more efficient when compared with traditional methods related to utility function in multi-objective optimization (Chen and Hwang, 1992; Sakawa, 1993).

The basic concept of fuzzy multi-objective optimization is to find the maximal achievement level among constraints of conflicting objectives（Sakawa, 1993）. Using Equations (3.6) - (3.8) as an example, and assuming that the optimistic value of kth (k=1,2,3) objective is fk+, while the pessimistic value is fk−, the achievement level then can be expressed as in Figure 3.4 (Hannan, 1981).

Where μkfk represents the fuzzy membership function of fk, which also reveals the achievement level of fk. By using the λ transformation（Sakawa, 1993）, and λ= Minkλ, the Equations (3.6)-(3.11) can be rewritten as in Equation (3.12).

1.0

fk+

fk−

0

fk

Figure 3.4  The Achievement Level for Fuzzy Objectives (k=1,2,3)

MaxSijλst    λ≤fk−fk−fk+−fk−,k=1,2,3                    (3.12)

and Equations (3.9)-(3.11)

To solve the model in Equation (3.12), a two step optimization approach can reduce the complexity. The first step is to compute the optimal number of firestations on the first objective (k=1), then the second step is to optimize Equation (3.12) with the optimal number of firestations from the previous step. In next Section, our revised GA and enumeration method are both applied and compared for resolution.

3.2.2.3 Model Resolution by Revised GA

To solve the mathematical model in Equation (3.12), a two-step approach is necessary. The first step is to decide the optimal number of firestation at the airport, the second step, then uses the results from first step as to maximize the global λ under other constraints. For this study, a GA is also developed because of following reasons: (a) The concept of GA is easy to be understood for readers, who lack a strong background of mathematics; (b) the GA is fully transferable ― the other problems which are similar to our study can apply almost the same steps for resolution; (c) our model is a NP-hard problem, and GA has proved to be effective in solving such combinatorial optimization problems (Sakawa et al., 1994, 1997). The traditional and basic notion of gene type in GA, e.g., 0010 (Goldberg, 1989) is also applied to form our gene population. But considering the characteristics of our model, we omit the crossover and reproduction in GA to prevent infeasible solution and promote self-evolution efficiency– this revised approach is also called simple GA (SGA). Thus, mutation and performance evaluation are re-defined as follows:

(a) Gene Type

From Figure 3.4, we observe that the range of the x-axis is from 0 to 16, while the range of y-axis is from 0 to 7; therefore, there are 136 (17  8 =136) available sites to be selected for setting up firestations at the airport. A 0-1 string of 136 bits is proposed in order to present whether to be set up a firestation or not on any specified location (i, j). All Sij are listed from left to right, and the corresponding x-y coordinate (i, j) to Sij are (0,0), (1,1), (1,2),…,(16,7), respectively. Thus, if Sij= 0, no firestation is located on the corresponding x-y coordinate (i, j), and if Sij= 1, a firestation is to be set up on the corresponding x-y coordinate (i, j). This is shown as follows:

Gene Type ( 136 bit string): 101…1

Corresponding Coordinate (i, j): (0,0), (1,1), (1,2),…, (16,7)

(b) Mutation

The mutation is defined as the recombination for a randomly selected gene. First, two cut-points are randomly selected. Secondly, the content between two cut-points is preserved and shifted to left-hand side. Finally, the gene is recombined from left to right. Mutation in this study is shown as follows:

Parent Gene: 1010010001…1      Offspring Gene 1: 1001010001…1

(c) Performance Evaluation

The self-evolution mechanism in generations is the important characteristic in GA. A generation is defined as a process to make the gene population undergo rank-selection, and mutation once (no crossover and reproduction are applied). The rank selection applied in this study tries to find the optimal sets of x-y coordinate, which will finally make theλ have the largest value among all possible values ofλ in Equation (3.12).

3.2.2.4 Numerical Example of Taipei International Airport

After our location model is well constructed, we follow the proceeding steps for a practical application of the Taipei International Airport in Taiwan– this will validate the practical applicability and effectiveness of our model.

1 Problem Description and Data Collecting

First, to compute the risk rank rij of Equation (3.8), the accidents at the International Airport in Taiwan are statistically analyzed. Because different accidents occur at the airport, the accident frequency is calculated and shown in Table 3.3.

Secondly, the area where accidents occur are taken into consideration and combine the ICAO statistical and geographical analysis of air crashes at the airport in Figure 3.5. Thus, after a conditional probability and normalized computation, the accident frequency rate is obtained from different area, and a risk rank table is established– this is shown in Table 3.4.

Thirdly, rijis assigned to the corresponding area at the airport, this is shown in the cubic map of Figure 3.6. The lower value of rij indicates that there is more chance for serious accidents occurring in the area of x-y coordinate (i, j).

Table 3. 3 The Statistical Analysis of Accident at Taipei International Airport

| Accident | Frequency | Frequency Rate﹪ |
| --- | --- | --- |
| 1. Airplane mechanical failure | 30 | 11.26 |
| 2. Airplane engine failure | 36 | 13.52 |
| 3. Airplane landing failure | 6 | 2.25 |
| 4. Airplane flap failure | 8 | 3.00 |
| 5. Airplane tire failure | 9 | 3.30 |
| 6. Airplane goes off runway | 2 | 0.75 |
| 7. Oil or fuel leaking | 41 | 15.40 |
| 8. Building fire | 29 | 10.90 |
| 9. Ground crew accident | 12 | 4.51 |
| 10. Passenger accident | 40 | 15.02 |
| 11. Runway poorly maintained | 11 | 4.14 |
| 12. Hijack accident | 2 | 0.75 |
| 13. Bomb threat | 3 | 1.12 |
| 14. Others | 37 | 14.36 |
| Total | 276 | 100% |

Figure 3.5  The Statistical Analysis of Air Crashes at the airport in the World

Source: ICAO, 1995

Table 3.4  The Risk Rank of Area at Taipei International Airport

| Accident Area | Risk Rank rij=1/AFR | Accident Frequency Rate﹪ AFR |
| --- | --- | --- |
| 1. Runway end to the nearby area | 1 | 1.00 |
| 2. Approach over 500m to the runway end | 2 | 0.50 |
| 3. Approach less than 500m | 3 | 0.33 |
| 4. Building and aprons | 4 | 0.25 |
| 5. Others | 5 | 0.20 |

x-axis  axis

0    1   2    3   4   5   6   7   8    9   10  11  12   13  14   15  16

7

6

5

4

3

2

1

0

y-axis  axis

### Figure 3.6  The Risk Rank Cubic Map of Taipei International Airport

Finally, the risk cost TRC and setup cost SC are reasonably computed:

- Computing Total Risk Cost TRC

If there is only one firestation at the airport the passengers in an air crash have a significantly lower chance of survival due to a lack of sufficient rescue and fire fighting resources. We compute the cost by estimating the number of deaths multiplied by the insurance payment for each of them. Assuming that 250 deaths result from an air accident, and each of them will be paid NT$13.5 million dollars from life insurance, the total risk cost TRC of them is then computed to be about NT$3.4 billion dollars ( NT$ 13.5 million×250 deaths ).

- Computing Setup Cost SC

If we try to set up one single firestation at the airport, the hardware (e.g., fire-engines, firestation, extinguishers, etc) and software (e.g., salary and life insurance of fire fighters, training cost, etc) cost are summed to be NT$ 165 million dollars by the relevant authority at Taipei’s International Airport; therefore the cost of SC is estimated to be NT$ 165 million dollars.

Since TRC equals NT$ 3.4 billion dollars and SC equals NT$ 165 million dollars, the optimal setup number of firestation around the International Airport would be 3 by optimizing the first objective (k=1). Moreover, after consulting form fire fighters in the airport, the distances are to be set dabr= 8, dabl=15 and dabs=1, respectively. Nevertheless, the risk weighted cubic map of the International Airport will be our initial input for Equations (3.6)-(3.12) and two solving approaches are implemented: GA and enumeration method.

2 Model Resolution by Revised GA and Enumeration Method

To solve the mathematical model in Equation (3.12), we first apply GA to solve this location problem. The traditional and basic notion of gene type in GA is binary, e.g., 0-1 type (Goldberg, 1989). Because the decision variable Sij is also 0-1 type, Goldberg`s encoding way is applied in a direct sense. After computing the optimal value of f1, and the ceiling and bottom values for f2 and f3, the Equation (3.12) can be rewritten as in Equation (3.13) with the λ transformation (Sakawa, 1994).

MaxSijλst    λ≤1λ≤51−f251−30λ≤3481−f33481−1427λ≤μd(d12)λ≤μd(d13)λ≤μd(d23)ijSij=3               (3.8)

The rank selection applied in this study tries to find 3 sets of x-y coordinate, namely the ones which make theλ have the largest value among all possible values of λ. Thus, λ is our primal index of rank selection in GA. The gene population are sorted by the λ value of each gene in a descending order. 20 runs of this GA derive satisfying solutions after 30 generations of our experiment. The final result is shown in Table 3.5.

Table 3.5  The Firestation Sites proposed by GA and Corresponding λ values

| Firestation Sites proposed by GA |  |  | λ value |
| --- | --- | --- | --- |
| Firestation 1 | Firestation 2 | Firestation 3 |  |
| (1, 6) | (9, 6) | (6, 2) | 0.75 |
| (6, 1) | (9, 3) | (0, 3) | 0.71 |
| (1, 1) | (6, 5) | (7, 0) | 0.71 |
| (9, 4) | (1, 6) | (5, 0) | 0.71 |
| (5, 6) | (13, 6) | (9, 4) | 0.70 |
| (12, 0) | (5, 3) | (11, 6) | 0.70 |
| (5, 0) | (9, 4) | (2, 6) | 0.70 |
| (5, 1) | (4, 7) | (0, 3) | 0.68 |
| (7, 1) | (4, 4) | (11, 7) | 0.60 |
| (11, 6) | (11, 0) | (5, 3) | 0.60 |

In computer speed terms, 20.33 minutes are spent to find the aforementioned result on a Pentium computer.

Since the enumeration method can exactly find the optimal location of firestations around the International Airport, this method is used for comparison with GA. This enumeration method proceeds by examining all possible combination of setup sites of three firestations as to achieve the maximal λ value. It took 73.57 minutes to find an optimal λ of 0.78.

3.2.3 Results and Discussion

From GA, three sets of firestation site are obtained, which x-y coordinates are (1, 6), (9, 6) and (6, 2). The final achievement level is 0.75. These three firestations can be easily located in Figure 3.6 by the triangle symbols. Comparing the GA and enumeration method, GA uses less time than enumeration method is firstly concluded (20.33 min vs. 73.57 min), yet still can perform a satisfying achievement level between conflicting objectives (λ=0.75 vs. λ=0.78). Therefore, the GA did show its feasibility and effectiveness in this study. Finally, the GA has potential of efficiently solving large-scale location problem of other facilities. This means many optimal sites (e.g., three or much more than three sites) of firestations can be easily derived through our model, which is very easy to be operated and can be a powerful support basis for location decision.

3.3 Case Application II- The Network Restoration Problem (NRP)

We will illustrate some cases and corresponding GAs in the following sections– from the simple NRP to the complex example.

3.3.1 A NRP Model with single objective, traffic assignment and no work-troops

In Section 3.3.1.1, the NRP characteristics are briefly described and the mathematical model is constructed. In Section 3.3.1.2, a revised genetic algorithm (GA) is proposed. In Section 3.3.1.3, the numerical example of an assumed network is presented, and the effectiveness between a subjectively experienced and our systematical strategy is compared.

### 3.3.1.1 Problem Description and Model Construction

According to the seismic experiences of Japan and America, an earthquake varied with time can be simply divided into three periods: the first period can be defined as a chaos period, which begins as the earthquake suddenly occurs. At the mean time, the computer-networks, electric power lines and telephone lines will be seriously destroyed. This chaos period ends as the instant rescue activities are proposed. After the chaos period, the second period can be defined as a rescue period, which allows the fire-extinguishing, emergency rescue for lives based on damage reports. Finally, the third period can be defined as a restoration period, which begins after the necessary rescue for lives are adequately provided. In this period, a long-term plan of reconstruction should be systematically drew out and executed. This study is trying to establish a simulation model as to efficiently design a schedule for road-network reconstruction during restoration periods.

After a large-scale earthquake occurring, there might be many damage points scattered throughout a road-network (see Figure 3.1), this situation is abstracted in Figure 3.7 to clarify our modeling concept. The priority guide of reconstruction for each damage point must be arranged appropriately as to save the travel-time for travelers during reconstruction.

To formulate the mathematical model of road-network reconstruction, firstly, the function (Z) reflects the convenience of traveling, Z is then defined as the computed value for total travel-time of travelers on road-network during reconstruction periods. This measurement is done by summing up the multiplying of traffic volume and travel-time in each link during reconstruction. Furthermore, the total reconstruction time of this model is fixed (a constant) for convenience. Assuming that a damage point is continuously reconstructed till this point is well-reconstructed; therefore, from a view of bi-level programming, our priority model in this study can be shown as in Equations (3.14) - (3.26).

4

1

C

5

8

10









7

6

9

B



3

2

A

: Work-troop for Reconstructing Damage Points

 : City

: Road-network

: Damage Points

Figure 3.7 An Abstracted Situation of Post-earthquake Road-network

Source: Modified from the Original Network of Nagurney (Nagurney, 1984).

The Upper Level

This Level achieves to minimize the travel-time of travelers and total working time by the single objective and constraints:

Min Z =i∈ok=1hl=1nlQkl(xik)×tkl(xik)　　                            (3.14)

s.t

if xik≠0, then xik=1, ∀i, ∀k                                (3.15)

k=1hi∈oxik≥WTi                                              (3.16)

k∈uxik=WTi,∀i                                             (3.17)

k=1hi∈oxik=i∈oWTi                                             (3.18)

k=1h                                            (3.19)

o ={i| the damage point i is assigned to the work-troop }

u ={k| the damage point i is continuously reconstructed by the work-troop within the time-interval k}

xik∈0,1; yik=1–xik, ∀i, ∀k

if l is a damage link including the damage point i, then this link volume at any given time-interval k is subject to:

Qkl(xik)=0,ifτ=1ki≠si,s∈o,∀i,∀k,                   (3.20)

but if l is a well link, then the link volume is subject to:

Qkl(xik)=Qkl∗,∀k                                            (3.21)

and the varying travel-time of a well link l in each time-interval k is subject to:

tkl(xik)=βlLlQkl(xik),∀l,∀k                                  (3.22)

and we assume that the total working time is subject to TRT:

i∈ok=1h(xik+yik)=TRT                                          (3.23)

The Lower Level

This Level achieves to obtain a convergent link flows under the reconstruction states of road-network from the upper level. Thus, A variational inequality problem (VIP) is solved as follows:

Find f∈Ω, such that:

aCa(f)(fa−fa)≥0,∀f∈Ω                                 (3.24)

Ω=                                 (3.25)

if  fa∈l then Qkl∗is subject to:

Qkl∗= afa,                                                  (3.26)

where:

i：damage points, i =1,2,…,m;

k：time-interval, k =1,2,…,h;

l：all physical links in a post-earthquake road-network, l = 1,2,…,nl;

xik：the decision variable, if damage point i is reconstructed at time interval k, then its value is 1, otherwise is 0;

yik：the dummy variable, if xik= 1, this value equals 0, otherwise is 1;

WTi：the time needed for work-troop j to completely reconstruct damage point i ;

TTio：the travel time for a work-troop traveling from a well-reconstructed point i to another damage point o;

tkl(xik)：the travel-time function of link l at time-interval k, this function is related to the reconstruction state of any damage point i;

Qkl(xik)：the traffic-volume function of link l at time-interval k, this function is also related to the reconstruction state of any damage point i;

Ll：the length of link l;

Qkl∗: the traffic volume of a well link l at time interval k, this volume is recomputed by asymmetric assignment when a damage point is completely reconstructed;

βl：the translation coefficient between traveling-speed and traffic-volume of a well link l, whereLlQkl∗/tkl=1/βl,βlis set to 0.0001 for all physical links in this study;

TRT：the total reconstruction time, a constant;

fa: the flow in link a of the detailed network during asymmetric traffic assignment, and fa represents the convergent flow in link a, frepresents the convergent flow in network;

Ca : the travel-cost of link a in the detailed network during asymmetric traffic assignment;

hr: the traffic volume of route r in a detailed network during traffic assignment;

δar: the dummy variable, if route r is used and link a is passed, then δar= 1; otherwise δar = 0;

Txy: the traffic demand between node x and y in a physical network.

The aforementioned bi-level model is essentially with the work priority model in the upper level. Furthermore, the lower level is expressed in a variational inequality problem (Dafermos, 1980; Nagurney, 1993) for asymmetric traffic assignment. Meaning of equations are explained as follows:

Equation (3.14): the objective function for minimizing the travel time of all travelers in a road-network during reconstruction periods;

Equation (3.15): any damage point is exactly reconstructed by only one work-troop;

Equation (3.16): any work-troop should at least reconstruct one damage point;

Equation (3.17): any damage point will be continuously reconstructed till this point is recovered;

Equation (3.18)-(3.19): total damage points should all be tackled during reconstruction;

Equation (3.20)-(3.21): a test rule established as to take the recovered links into the first objective consideration;

Equation (3.22): a function established based on the traffic-flow theory in order to compute the travel-time of all travelers for the first objective;

Equation (3.23): a function established to compute the reconstruction time consumed by each work-troop;

Equation (3.24)-(3.25): a variational inequality form of the asymmetric traffic assignment;

Equation (3.26): summing up the link flows in the detailed network as to form the physical link flow after the asymmetric traffic assignment.

After inputting the available data, e.g., origin/destination (OD) pattern of traffic demand on pre-earthquake road-network, locations of damage points, physical link length, etc., to this model, which will output a plan in order to find the optimal reconstruction priority for all damage points. Furthermore, because the connection status of the physical network varies with the reconstruction states; thus, Equations (3.24)-(3.26) are continuously resolved by diagonalization method (Dafermos, 1980, 1984; Dafermos and Nagurney, 1984; Roy, 1991) to obtain the assigned traffic in physical network. The assigned traffic in each reconstruction period is used as a performance index for evaluating the effectiveness of reconstruction priority.

In next section, we will illustrate how we resolve the schedule problem of Equations (3.14)-(3.26).

#### 3.3.1.2 Model Resolution by Genetic Algorithm

To solve the mathematical model above, the concept to optimize a bi-level model is necessary (LeBlanc and Boyce, 1986). The upper level in Equations (3.14)-(3.23) will firstly output a reconstruction plan, which will instruct in what order to reconstruct each damage point. Secondly, the reconstruction plan from the upper level will be an input to the lower level in Equations (3.24)-(3.26). The lower level uses this input to find the convergent link flows by asymmetric traffic assignment. The convergent link flows will be feedback to the objective of the upper level as to evaluate the priority effectiveness.

A GA is applied in this study because that the concept of GA is easy to be understood for readers, the GA is full of transferability and the GA is proved to be effective in solving such NP-hard problems (Sakawa, et al., 1994, 1997). Considering the characteristics of our model, the gene type, crossover, and mutation are re-defined for reconstruction order as follows:

- Gene type

The reconstruction order for each damage point can be defined as a string-gene of Arabic numerals. For example, if there are 5 damage points:1,2,…,5 needed to be reconstructed, then the string of “13245” indicates that the damage point 1 will be reconstructed first, the damage point 3 will be reconstructed secondly,…,and the damage point 5 will be reconstructed finally ― this encoding will be used in this study.

(2) Crossover

The crossover is similar to that of Sato’s and Ichii’s paper in 1996. First, a father gene and a mother gene are randomly chosen from the gene population. Secondly, two cut-points are randomly selected from the father gene as well as the mother gene ― the location of cut-points are identical for both father gene and mother gene. Thirdly, the content between two cut-points from father gene is then preserved and transmitted to the coordinate location of the offspring gene. Finally, the mother gene deletes the preserved part from the father gene, then Arabic numerals remained in the latest mother gene are inversely transmitted to the offspring gene one by one. This crossover can be illustrated as follows:

Father Gene    |123|45  ;   Mother Gene    |241|35

Offspring Gene    |123|54

| : cut-point

(3) Mutation

The mutation is proceeded as follows: first, a gene from the gene population is randomly chosen. Secondly, two cut-points are randomly selected for this gene and the content between cut-points is then preserved. Finally, the Arabic numerals preserved are inversely transmitted to the offspring gene one by one. This mutation can be illustrated as follows:

Parent Gene    |123|45

Offspring Gene  |321|45

| : cut-point

(4) Selection

The elitism selection is also used in this study (Goldberg, 1989). This means the gene which makes the objective function has the lower value is more preferred, and will be retained.

The aforementioned GA for work priority in this study will derive a satisfying solution through 30 generations after 20 runs ― a generation is defined as a process to make gene population undergo elitism-selection, crossover and mutation once ( no reproduction is applied ). A run is defined as a re-starting process with different random seed to iterate GA for 30 generations. Our initial gene population is set to be 10 genes. This special optimization steps of our model is illustrated as follows:

Step 1

Inputting the relevant data required for reconstruction order and work assignment. For example, the OD traffic demand of pre-earthquake road-network, damage points need to be reconstructed, and so on.

Step 2

Using the relevant crossover, mutation to extend offspring genes. The achievement level of each gene will be evaluated by Equation (3.27), the evaluation index (EI) for any gene in population is presented as follows:

EIg=Zg,∀g                                                   (3.27)

where :

g: the gene in population, g=1,2,…,10;

EIg: the evaluation index of g-th gene in population;

Zg: the objective value by Equation (3.14) for the g-th gene in population.

Since our model is a minimization problem, the lower value of EI is more preferred and highly ranked. The best ten genes will be kept by elitism-selection after Step 2 terminated.

Step 3

Repeating Step 2 for 20 runs. The heuristic optimal value of our objective and EI will be dynamically changed in each run if a better reconstruction priority is found. Furthermore, the gene with the lowest EI is our final solution.

After aforementioned three steps, a heuristic solution can be easily derived. A simple example is used to validate our model and optimizing process in next Section.

#### 3.3.1.3 Numerical Example and Discussions

Table.3.6  The Assumed Traffic Demand in Figure 3.7

| From\To | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- |
| 1 | 0 | 235 | 230 | 220 |
| 2 | 240 | 0 | 235 | 225 |
| 3 | 230 | 220 | 0 | 235 |
| 4 | 235 | 225 | 240 | 0 |

Time Interval

Figure 3.8  A Heuristic Reconstruction Priority for Each Damage Point

An assumed road-network after earthquake in Figure 3.7 is used to validate our model and revised GA. The reconstruction time for a damage point is estimated from 3 to 18 time-intervals by civil engineers, and each link length is assumed (see Appendix A). For reflecting the dynamics of traffic affected by different reconstruction states, the traffic volume in each link will be re-assigned if a damage point is fully recovered. The pre-quake traffic demand of Figure 3.7 is expressed in an OD matrix, which is shown in Table 3.6. The detailed network and the detailed link costs in Figure 3.7 are identical to that of Nagurney (Nagurney, 1984; also see Appendix A).

After inputting the desired parameters of Equations (3.14)-(3.23), loading traffic demand from Table 3.6, and applying three steps in the previous Section, a heuristic reconstruction schedule is finally obtained in Figure 3.8. The heuristic objective value, Z = 208,783 (traffic volume×travel time). The Figure 3.8 can be recognized as follows: we start our reconstruction priority form the damage point 4 first, then reconstruct the damage point 1. After damage point 1 is completely reconstructed, the damage point 3 is continuously recovered, and then the damage point 2 is reconstructed. Finally, the damage point 5 is tackled. It is very clear that when a damage point is completely recovered, the travel time of travelers will fall down. The final travel time in Figure 3.8 will reduce to 3347, and the dark gray area bounded in Figure 3.8 is the objective value (Z).

Furthermore, a subjectively experienced strategy and the plan in Figure 3.8 are compared for reconstruction effectiveness by Equation (3.14). The subjectively experienced strategy is proposed based on the traffic demand pattern before quake ― the original result of traffic assignment with no damage points in Figure 3.8. Firstly, the link volume affected by each damage point is computed, and is ranked with a descending order. If the damage point affects more link volume, then this point is ranked at a higher priority and earlier reconstructed. Thus, this subjective reconstruction plan finally make the objective value with 259,475 (traffic volume×travel time). The subjectively experienced plan can be recognized as follows: the damage point 2 is firstly restored, then the damage point 1 is secondly reconstructed, then the damage point 5 is thirdly reconstructed. After the damage point 5 is completely recovered, the damage point 3 is continuously tackled, and the damage point 4 is finally reconstructed. Since the objective value of the subjectively experienced plan is larger than that of our systematical approach (259,475 > 208,783), the result of our model resolved by GA is clearly more effective. Moreover, this also implies that the subjective and traditional restoration concept- firstly reconstructing the damage point, which heavily affects the link volume, is not appropriate.

In next section, we use Taiwan’s practical freeway-network as a numerical example. Furthermore, many work-troops will be considered, but the traffic condition will not be involved because of lacking real traffic data.

3.3.2 A NRP Model with multiple work-troops, fuzzy multi-objectives and no traffic assignment

This section is designed to expand and validate our model in the previous section. Our original model in Equations (3.14)-(3.26) will be modified for the situation of multiple work-troops and objectives. Thus, this part is organized as follows: in Section 3.3.2.1, the problem characteristics are described and the mathematical model is constructed. In Section 3.3.2.2, a revised genetic algorithm (GA) is proposed and a practical freeway-network is applied to validate the effectiveness of our model.    3.3.2.1 Problem Description and Model Construction

This study is trying to establish a simulation model so as to efficiently design a schedule for road-network reconstruction during the aforementioned restoration period. Moreover, our model may be installed on a portable computer to be a radioactive command center– this will prevent the command difficulty through a line communication (e.g., telephones) after a large-scale earthquake.

After a large-scale earthquake occurs, there might be many damage points scattered throughout a road-network (see Figure 3.1)– this abstracted situation is shown in Figure 3.9 to clarify our formulation concept. In the mean time, suppose that there are also a few homogeneous work-troops available for reconstructing damage points– this condition is also shown in Figure 3.9. In order to reconstruct each damage point by each work-troop which is eventually constrained in resources, the guide of reconstruction for each work-troop must be arranged appropriately so as to save total reconstruction time, travel-time for travelers and balance work-load between homogeneous work-troops during reconstruction. The guide of reconstruction should not only instruct each work-troop how many damage points should be tackled but also in what order these damage points are to be reconstructed. Furthermore, the work-troops for reconstruction only can arrive at any damage point by a well-reconstructed or non-decimated route.

: Work-troop

X : Damage point

X

X

X

X

Figure 3.9  An Abstracted Situation of Post-earthquake Road-network

The travel-time in a post-quake road-network will be sharply increase and unacceptable when compared with a normal traffic condition, but this post-quake travel-time can be slowly reduced if appropriate reconstruction activities are executed. Thus, we use the first objective (Z1) to reflect the convenience of traveling by minimizing total travel-time of all travelers in the road-network during reconstruction. This objective is measured by summing up the product of the traffic volume by travel-time in each link during reconstruction. To construct fuzzy membership functions of our objectives, the relevant optimistic and pessimistic values in Equations (3.28)-(3.30) are all consulted from experienced engineers in the transportation-safety department of the Institute of Transportation (IOT) in Taiwan.        The achievement level of Z1 can be expressed as a membership function of μH(Z1), which is graphically shown in Figure 3.10, where H indicates that Z1 belongs to the membership function of hours (H: hours).

μH(Z1) = 0                    if Z1 ³ 100000 hours

μH(Z1) = (100000-Z1)/100000   if 100000 hours ³ Z1 0 hours   (3.28)

μH(Z1) = 0                    if Z1 0 hours

Achievement Level

1.0

0

100000

Z
, Hours

Figure 3.10  Achievement Level of Minimizing the Travel-time for Travelers in a Road-network During Reconstruction

Secondly, minimizing the total time for completed reconstruction is defined asZ2. The achievement level of Z2can also be similarly expressed as a piece-wise membership function of μD(Z2), which is shown in Equation (3.29) and as a graph in Figure 3.11, where D indicates that Z2belongs to the membership function of days (D: days).

μD(Z2)= 1               if Z2 60 days

μD(Z2)= (80 - Z2)/20     if 70 days ³Z2 60 days

μD(Z2)= (90 - Z2)/40     if 90 days ³Z2 70 days            (3.29)

μD(Z2)= 0               if Z2 70 days

Achievement Level

1.0

0.5

60

70

90

Z
, Days

Figure 3.11  Achievement Level of Minimizing the Total Time Needed for Reconstruction

Thirdly, the final fuzzy objective (Z3) is designed to minimize the work-load variance between any two work-troops. This work-load variance is measured by the longest time spent by a work-troop on heavy duty minus the shortest time spent by another work-troop on light duty. The work-load variance between the longest working time and the shortest working time within any two work-troops should be as small as possible. The achievement level of Z3 can also be similarly expressed as a piece-wise membership function of μD(Z3), which is shown in Equation (3.30) and as a graph in Figure 3.12, where D indicates that Z3belongs to the membership function of days (D: days).

μD(Z3)= 1              if Z3 10 days

μD(Z3)= (60 - Z3)/60     if 60 days ³Z3 10 days         (3.30)

μD(Z3)= 0              if Z3 60 days

Finally, we assume that a work-troop is continuously reconstructing an assigned damage point till this point is well-reconstructed; therefore, we expand Equations (3.14) –(3.23) in this study and shown in Equations (3.31) - (3.40).

Achievement Level

1.0

10

60

Z
, Days

Figure 3.12  Achievement Level of Minimizing the Work-load Variance between any Two Work-troops

Min Z1 =i∈oj=1nk=1hl=1nlQkl(xijk)×tkl(xijk)

Min Z2 = Maxj{RTj}                                          (3.31)

Min Z3 = Maxj{RTj} Maxj{RTj}

s.t

if xijk≠0, thenj=1nxijk=1, ∀i, ∀k                                (3.32)

k=1hi∈oxijk≥WTij, ∀j                                            (3.33)

k∈uxijk=WTij,∀i,∀j                                            (3.34)

k=1hi∈oj=1nxijk=i∈oj=1nWTij　　                                         (3.35)

k=1hi∈on,                                        (3.36)

o ={i| the damage point i is assigned to the work-troop j}

u ={k| the damage point i is continuously reconstructed by the work-troop j within the time-interval k}

xijk∈0,1; yijk=1–xijk, ∀i, ∀j, ∀k

if l is a damage link including the damage point i, then this link volume at any given time-interval k is subject to:

Qkl(xijk)=0,ifτ=1ki≠si,s∈o,∀i,∀j,∀k,                     (3.37)

but if l is a well link, then the link volume is subject to:

Qkl(xijk)=Qkl∗,∀k                                            (3.38)

and the varying travel-time of a well link l in each time-interval k is subject to:

tkl(xijk)=βlLlQkl(xijk),∀l,∀k                                 (3.39)

and for each work-troop j, the total working time is subject to:

RTj=i∈ok=1h(xijk+yijk),∀j                                     (3.40)

Wherexijk represents the damage point i is assigned to the work-troop j during the time-interval k, and the other variable definition are identical to that in Section 3.3.1.1.

Equations (3.31)-(3.36) are modified structures of traditional transportation problems in operations research field (OR), but Equations (3.37)-(3.39) are established so as to dynamically catch the traffic variation in this study by traffic-flow theory. Furthermore, according to the concepts of λ transformation for fuzzy multi-objective approach (Martison, 1993; Sakawa et al., 1994, 1997), Equations (3.31)-(3.40) can be efficiently transformed in Equation (3.41).

After inputting the available data (e.g., traffic-volume, travel-time, locations of work-troops and damage points, etc.) to Equation (3.41), which will output a plan so as to instruct each work-troop for its own reconstruction task. This model can be solved by many approaches, which we discussed in the Section 3.1.2. Since GA has been recently applied to many problems related to our study and reveals its potential power in such combinatorial optimization problems, we try to develop a revised GA with a lattice-gene in this study. In the next section, we will use a numerical example to illustrate how our GA works.

Maxxijk λ

s.t

λ≤μH(Z1)                        (3.41)

λ≤μD(Z2)

λ≤μD(Z3)

and Equations (3.32)-(3.40)

3.3.2.2 Numerical Example and Resolution

In this section, we use a practical freeway-network in Taiwan to illustrate how we collect required data for Equations (3.31)-(3.40) and how our GA works so as to validate the effectiveness of our model.

1. Data Collection and Input

Since the freeway-network of Taiwan had never been decimated by a large-scale earthquake like that in Japan or America. Thus, we only do our best to imagine and assume the most possible situation of a post-earthquake freeway-network in Taiwan. We firstly collect historical earthquake data from 1897 to 1996 from Central Weather Bureau (CWB). Secondly, we statistically analyze these data on a Taiwan map to check where big quakes are possible to occur– this assumed damage points are shown in Figure 3.13. The number from “1” to “15” represents the damage points by our assumption. Thirdly, we survey available work-troops in Taiwan and locate their sites in Figure 3.13– the symbol from “A” to “F” represents the location of each work-troop. Of course, all work-troops are simply assumed to be homogeneous in this study. Fourthly, the reconstruction time for a single damage point is estimated about 12~24 work-days by consulting civil engineers– not including the travel time form a work-troop to reach a damage point. The travel time between any two nodes are assumed from 1 to 24 hours ( see Appendix B ).

Figure 3.13  An Assumed Situation of Post-earthquake Freeway-network

Finally, we also survey the traffic volume between required nodes in our network– this data is shown in Appendix B. Moreover, to reflect the traffic variation during reconstruction, the traffic volume in each link is varied with two different states during reconstruction periods. These states are: (a) before-earthquake state; and (b) reconstruction-completed state. The traffic volume in each state is reasonably assigned to each link by equal-travel-time assumption, which means the traffic volume of a reconstruction-completed link will continuously increase until the travel time of this well-reconstructed link is equal to that of the nearby substituted routes (see Figure 3.13).

2. Model Resolution by Two-step Genetic Algorithm

To solve our mathematical model, a two-step approach is necessary. The first step is to decide the reconstruction order for each damage point, the second step then uses the results from the first step to assign the reconstruction task to each work-troop. Although there are many approaches developed and applied to solving our problem, we also use GA for three reasons: (a) the concept of GA is easy to be understood for readers, who lack a strong background of mathematics; (b) the GA is full of transferability– it means that the other problems which are similar to ours can be solved with almost the same steps for resolution; (c) our model is a NP-hard problem, and GA is proved to be effective in solving such a combinatorial optimization problem (Sakawa, et al., 1994, 1997). To resolve the scheduling and work assignment for reconstruction, two different GAs are developed– we will discuss these details in next Section. The traditional and basic notion of gene type in GA is to translate decimal values into binary-digit value: such as decimal 2 is equivalent to binary 0010 (Goldberg, 1989). But considering the characteristics of our model, we use a lattice-gene to describe the work assigned to each work-troop and a string-gene used to express the reconstruction order for damage points. In any case, the computational spirit of genetic reproduction here is similar to that of Goldberg’s, but the gene type, crossover, and mutation are re-defined for reconstruction order (scheduling) and work assignment, respectively as follows:

3. GA of String-gene for Optimizing Reconstruction Order

(a) Gene type

The reconstruction order for each damage point can be defined as a string-gene of Arabic numerals. For example, if there are 8 damage points:1,2,…,8 needed to be reconstructed, then the string of “13245687” indicates that the damage point 1 will be reconstructed first, the damage point 3 will be reconstructed secondly,…,and the damage point 7 will be reconstructed finally– this encoding way will be applied in this study.

(b) Crossover

Our crossover is similar to that of Sato and Ichii in 1996. First, we randomly choose a father gene and a mother gene from the gene population. Secondly, two cut-points are randomly selected for the father gene. Thirdly, the content between two cut-points from the father gene is then preserved and transmitted to the beginning location (left-hand side) of the offspring gene. At the same time, the mother gene’s elements, which are identical to the father gene’s preserved content are deleted from the mother gene. Finally, the undeleted elements of the mother gene are transmitted to the offspring gene one by one and from left to right. Furthermore, the mother gene’s undeleted part is connected to the tail (right-hand side) of the father gene’s preserved content.  This aforementioned process is illustrated as follows:

Father Gene    78|123|456  ;   Mother Gene    72468135

Offspring Gene    |123|74685

| : cut-point,

Note: 1, 2 and 3 are the same elements between parent genes. The sequence remained in the mother gene are 7, 4, 6, 8 and 5.

(c) Mutation

The mutation proceeds as follows: first, we randomly choose a gene from the gene population. Secondly, two cut-points are randomly selected from this gene and the content between cut-points is then preserved. Finally, the Arabic numerals preserved are inversely transmitted to the offspring gene one by one. This mutation can be illustrated as follows:

Parent Gene    78|123|456

Offspring Gene   78|321|456

| : cut-point

##### 4. GA of Lattice-gene for Optimizing Work Assignment

We also develop a lattice-gene in this study for work assignment, this GA is shown as follows:

(a) Gene type

In view of Figure 3.13, there are 15 damage points (e.g., 1,2,…,15) needed to be reconstructed by 6 work-troops (e.g., A, B,…,F), then a reconstruction plan can be expressed by a lattice-gene as follows:

|  | Damage pt. 1 | Damage pt. 2 | … | Damage pt. 15 |
| --- | --- | --- | --- | --- |
| Work-troop A | 1 | 1 | … | 0 |
| Work-troop B | 0 | 0 | … | 1 |
| … | … | … | … | … |
| Work-troop F | 0 | 0 | … | 0 |

The damage points in the first row are arranged by the scheduling result from GA for reconstruction order above. The emergency of reconstruction for damage points is listed in descending order from left to right– we assume the damage point 1 should be tackled first, then the damage point 2,…, finally the damage point 15 in the case above. Moreover, the value of “1” indicates a damage point is reconstructed by a specified work-troop; by contrast, the value of “0” indicates no task to be assigned. To match the assumption that each damage point is reconstructed by only one work-troop at the same time, there is at most one “1” in each column. For example, if work-troop A is assigned to tackle damage point 1 and 2, and damage point 1 should be reconstructed first, and damage point 2 secondly. And the other work-troops (e.g., B, C,…,F) would not be assigned to reconstruct damage point 1 and 2 again.

(b) Crossover

The two-point crossover for the lattice-gene is defined as follows: first, we randomly choose a father gene and a mother gene from the gene population. Secondly, two cut-points are randomly selected from the father gene as well as the mother gene. Finally, two offspring genes are formed when the father gene is partially replaced with the content, which is preserved between two cut-points of the mother gene; and when the mother gene is partially replaced with the content, which came from the cut part of the father gene. This crossover is shown as follows:

Father’s Content between Two Cut-points

Father Gene

|  | Damage pt. 1 | Damage pt. 2 | … | Damage pt. 15 |
| --- | --- | --- | --- | --- |
| Work-troop A | 0 | 1 | … | 0 |
| Work-troop B | 1 | 0 | … | 1 |
| … | … | … | … | … |
| Work-troop F | 0 | 0 | … | 0 |

Mother’s Content between Two Cut-points

Mother Gene

|  | Damage pt. 1 | Damage pt. 2 | … | Damage pt. 15 |
| --- | --- | --- | --- | --- |
| Work-troop A | 0 | 0 | … | 0 |
| Work-troop B | 1 | 0 | … | 0 |
| … | … | … | … | … |
| Work-troop F | 0 | 0 | … | 1 |

Crossover

Offspring Gene 1 (Mother Gene is Partially Replaced by Father’s Cut Content)

|  | Damage pt. 1 | Damage pt. 2 | … | Damage pt. 15 |
| --- | --- | --- | --- | --- |
| Work-troop A | 0 | 1 | … | 0 |
| Work-troop B | 1 | 0 | … | 0 |
| … | … | … | … | … |
| Work-troop F | 0 | 0 | … | 1 |

Offspring Gene2 (Father Gene is Partially Replaced by Mother’s Cut Content)

|  | Damage pt. 1 | Damage pt. 2 | … | Damage pt. 15 |
| --- | --- | --- | --- | --- |
| Work-troop A | 0 | 1 | … | 0 |
| Work-troop B | 1 | 0 | … | 1 |
| … | … | … | … | … |
| Work-troop F | 0 | 0 | … | 0 |

(c) Mutation

The mutation is defined as exchanging contents between rows and columns. Firstly, we randomly select two rows and two columns to mutate for a randomly selected gene. Considering the row mutation, the contents of two selected rows are then exchanged each other. For the column mutation, the contents of two selected columns are also exchanged each other. Two types of mutation in this study are shown as follows:

Mutation 1

Parent Gene

|  | Damage pt. 1 | Damage pt. 2 | … | Damage pt. 15 |
| --- | --- | --- | --- | --- |
| Work-troop A | 0 | 1 | … | 0 |
| Work-troop B | 0 | 0 | … | 1 |
| … | … | … | … | … |
| Work-troop F | 1 | 0 | … | 0 |

Row Mutation

|  | Damage pt. 1 | Damage pt. 2 | … | Damage pt. 15 |
| --- | --- | --- | --- | --- |
| Work-troop A | 1 | 0 | … | 0 |
| Work-troop B | 0 | 0 | … | 1 |
| … | … | … | … | … |
| Work-troop F | 0 | 1 | … | 0 |

Mutation 2

Parent Gene

|  | Damage pt. 1 | Damage pt. 2 | … | Damage pt. 15 |
| --- | --- | --- | --- | --- |
| Work-troop A | 1 | 0 | … | 0 |
| Work-troop B | 0 | 1 | … | 1 |
| … | … | … | … | … |
| Work-troop F | 0 | 0 | … | 0 |

Column Mutation

|  | Damage pt. 1 | Damage pt. 2 | … | Damage pt. 15 |
| --- | --- | --- | --- | --- |
| Work-troop A | 0 | 0 | … | 1 |
| Work-troop B | 1 | 1 | … | 0 |
| … | … | … | … | … |
| Work-troop F | 0 | 0 | … | 0 |

To sum up, the GA is developed and designed to exchange the task assignment between work-troops.

5. Optimizing the Reconstruction Order and Work Assignment

The aforementioned GA for scheduling and work assignment in this study will derive satisfying solutions after 30 generations by our experiment. A generation is defined as a process to make gene population undergo rank-selection once, crossover once and mutation once. Since there are two different GA modules developed in this study, this special optimization process is illustrated as follows:

Step 1

Input the required data of Equations (3.31)-(3.40): for example, the data of traffic volume of each link, the travel time of each link, the travel time between required nodes, locations of damage points, locations of available work-troops, the reconstruction time of each damage point, etc.

Step 2

Encode the reconstruction order by method in Section 3.3.1, then use the relevant crossover, mutation of GA to extend offspring genes– the initial population size of string-genes is set to 20 genes.

Step 3

The reconstruction order is transmitted into the lattice-genes in Section 3.3.2 by the initial 20 string-genes from step 2. The population of lattice-genes is then expanded by relevant crossover and mutation.

Step 4

The achievement level of each lattice-gene will be evaluated by λ value in Equation (3.41), then the best 20 genes will be kept by rank-selection after Step 4 is terminated. The achievement level of each gene is defined as the smaller value between μH(Z1), μD(Z2) and μD(Z3) of this gene by a fuzzy multi-objective approach.

Step 5

The reconstruction order found in the final population of lattice-genes will be extracted to Step 2 to form a new population of string-genes, and the steps from 2 to 5 will be iterated until 30 iterations generations are reached.

3.3.2.3 Computational Results and Discussion

After 20 runs– each run includes aforementioned five steps, a heuristic solution of our model can be easily derived. In view of Figure 3.14, the work-troop A should tackle the damage point 1 first, then the damage point 2; the work-troop B should reconstruct the damage point 3 first, then the damage point 4, finally the damage point 5. The reconstruction plans of the other work-troops can be recognized in a similar way. The total reconstruction time is 56 days with an aspiration level of 1.0; the cumulative travel time is 29,537 hours with an achievement level of 0.70 and the significant work-load variance between work-troops is 28 days with a achievement level of 1.0. Thus, this implies the global achievement level of three objectives is 0.70. Furthermore, our GA did show its power when we analyze the global achievement value (λ) between generations– this is shown in Figure 3.10. This reconstruction schedule in Figure 3.14 is practically available for relevant authorities for pre-quake exercises so as to reduce the negative impact of a transportation network.

Thus, our model is reasonably constructed and practically validated by a freeway-network so far. Both the aspiration level of Z2and Z3 are 1.0– this indicates that the acceptable threshold subjectively set up by us may be too optimistic for these two objectives. Our model did show its practicability and operational utility from Equations (3.31)-(3.40) and Figure 3.14, 3.15. Since this study is developed by mathematical programming, with more precise data for our model, a reconstruction schedule can be scientifically made instead of messy decisions after a large-scale earthquake. Furthermore, since this effective reconstruction schedule is powerful and efficient, it can’t only be a basis for pre-earthquake exercises of work-troops, but can also be potentially installed on a portable computer for real-time reaction of restoring damage road-networks by radioactive communication.

Figure 3.14  A Heuristic Reconstruction Schedule for Each Work-troop

Global Achievement Value

Generations

Figure 3.15  The GA Evolution between Generations of the Best Run

3.3.3 A NRP Model with multiple work-troops, multiple objectives and traffic assignment

Moreover, we consider the traffic assignment, many work-troops and objectives in this section– this situation will be more similar to the real situations. In Section 3.3.3.1, our NRP characteristics are described and the bi-level model is constructed. In Section 3.3.3.2, a revised genetic algorithm (GA) is proposed. In Section 3.3.3.3, the numerical example is presented and used to validate the applicability and efficiency of our model.

3.3.3.1 Problem Description and Model Construction

We had recognized that: after a large-scale earthquake occurring, there might be many damage points, which are scattered throughout a road-network and will make travelers fail to pass (see Figure 3.1), this situation is abstracted in Figure 3.7 in order to illustrate our formulating concept of the NRP. Meanwhile, supposing that there are also a few available work-troops, which are responsible to reconstruct damage points- this situation is also shown in the previous Figure 3.7. Each work-troop is permitted to bi-directionally move along any well link for reconstruction. To efficiently reconstruct each damage point by each work-troop, which is constrained in resources, the reconstruction task for each work-troop must be appropriately arranged in order to minimize the total reconstruction time, travel-time of travelers and the maximal idle time of any work-troop. Thus, this reconstruction guideline should not only instruct each work-troop how many damage points should be tackled, but also in what order for these damage points to be reconstructed.

To formulate the mathematical model of the NRP, we expand Equations (3.31)-(3.40) again. Furthermore, assuming that a work-troop is continuously reconstructing a assigned damage point till this point is recovered; therefore, our NRP model in this study can be formulated as in Equations (3.42) - (3.54).

The Upper Level

This level achieves to minimize the travel-time of travelers, total reconstruction time and the maximal idle time of any work-troop by the following objectives and constraints:

Min Z1 =i∈oj=1nk=1hl=1nlQkl(xijk)×tkl(xijk)

Min Z2 = Maxj{RTj}                                       (3.42)

Min Z3 = Maxj{RTj} Maxj{RTj}

s.t

if xijk≠0, thenj=1nxijk=1, ∀i, ∀k                             (3.43)

k=1hi∈oxijk≥WTij, ∀j                                         (3.44)

k∈uxijk=WTij,∀i,∀j                                         (3.45)

k=1hi∈oj=1nxijk=i∈oj=1nWTij　　                                      (3.46)

k=1hi∈on,                                      (3.47)

o ={i| the damage point i is assigned to the work-troop j}

u ={k| the damage point i is continuously reconstructed by the work-troop j within the interval k}

xijk∈0,1; yijk=1–xijk, ∀i, ∀j, ∀k

if l is a damage link including the damage point i, then this link volume at any given time-interval k is subject to:

Qkl(xijk)=0,ifτ=1ki≠si,s∈o

but if l is a well link, then the link volume is subject to:

Qkl(xijk)=Qkl∗,∀k                                             (3.49)

and the varying travel-time of a well link l in each time-interval k is subject to:

tkl(xijk)=βlLlQkl(xijk),∀l,∀k                                  (3.50)

and for each work-troop j, the total working time is subject to:

RTj=i∈ok=1h(xijk+yijk),∀j                                      (3.51)

The Lower Level

This level achieves to obtain the convergent link flows by the reconstruction states of the road-network from the upper level. Furthermore, the travel cost function in this level is non-separable and with an asymmetric Jocobian cost matrix. Thus, this variational inequality problem (VIP) is formulated as follows:

Find f∈Ω, such that:

aCa(f)(fa−fa)≥0,∀f∈Ω                                    (3.52)

Ω=                                    (3.53)

if  fa∈l then Qkl∗is subject to:

Qkl∗= afa,                                                      (3.54)

The definition of symbols and variables are the same as in the previous section; thus, we won’t duplicate this variable definition again.

The aforementioned NRP model essentially integrates the reconstruction scheduling and assignment in the upper level. Furthermore, the lower level is expressed in a variational inequality problem (Dafermos, 1980; Nagurney, 1993; Roy, 1991) based on the asymmetric traffic assignment.

After inputting available data, e.g., origin/destination (OD) pattern of traffic demand in a pre-quake road-network, sites of work-troops, locations of damage points, etc., to this model, which is able to draw a plan as to optimally instruct each work-troop for its own reconstruction task. Furthermore, since the connection status of the post-quake road-network varies with the reconstruction states; thus, Equations (3.39)-(3.41) are continuously resolved by the diagonalization method (Dafermos, 1980, 1984; Dafermos and Nagurney, 1984; Roy, 1991) to obtain the assigned traffic in the physical network.  The diagonalization method is briefly introduced as follows:

Step 1

Use all or nothing (AON) traffic assignment skill to find an initial flow set fa,it, let it (iteration) = 0.

Step 2

Use the asymmetric traffic assignment, let:

Ca,it(f)=Ca,it(f1,it,f2,it,...,fa,it,...),∀f∈Ω

Step 3

Find an optimal αit by the bi-section method, such that:

Min0≤αit≤1a0fa,it+αit(fa,it+1−fa,it)Ca,it(f)df,∀f∈Ω

Step 4

If Maxa|fa,it+1−fa,it|fa,it∀f∈Ω, then the iteration stops and the convergent flows are obtained; otherwise, go to Step 2 for another iteration, and it = it + 1. Where ε is the acceptable percent error, which is subjectively set to 10% in this study.

The assigned traffic in each reconstruction period is used as a performance index for evaluating the reconstruction effectiveness in Z1. In next section, we will illustrate how we resolve the NRP by Equations (3.42)-(3.54).

#### 3.3.3.2 Model Resolution by Genetic Algorithm

To solve the mathematical model above, the concept for optimizing a bi-level model is required (LeBlanc and Boyce, 1986; Tzeng and Tsaur, 1997). The upper level in Equations (3.42)-(3.51) will firstly outputs a reconstruction plan, which will instruct the work-troops how to share their reconstruction tasks of all damage points. Secondly, the reconstruction plan from the upper level will be an input to the lower level of Equations (3.52)-(3.54). The lower level uses this input to find the convergent link flows by the asymmetric traffic assignment. Finally, the convergent link flows will be feedback to the upper level as to evaluate the plan effectiveness in Z1.

A GA is applied in this study because that: (a) the concept of GA is easy to be understood by readers; (b) the GA is full of transferability- this means the resolution steps proposed in this NRP can be applied to other similar problems; and (c) the GA is proved to be effective in solving such a NP-hard problem (Sakawa, 1993; Sakawa, et al., 1994, 1997). Considering the characteristics of our model, the gene type, crossover, mutation, selection are re–designed as follows:

(a) Gene type

The gene is the basic unit used to evaluate the effectiveness of a reconstruction plan by Equation (3.42). Considering the Figure 3.7 as an example, assuming that there are five damage points, e.g., ,,…, , and three available work-troops, e.g., A, B and C in a road-network, which has 10 nodes, e.g., 1, 2,…, 10. Thus, a feasible solution expressed in Figure 3.16 is defined as a gene in our NRP model.

A: 26910

B: 7810

C: 1056

Figure 3.16  The Gene of a Reconstruction Plan

The gene in Figure 3.16 is recognized as follows: the work-troop A reaches the damage point 2 () by travelling from node 2, and passing through node 6. After work-troop A completely reconstructed the damage point 2 (), the work-troop A is at node 9, then A goes to the damage point 3 () by traveling through node 9. After the damage point 3 () is recovered, the troop A is at node 10. The work plan for the troop B and troop C can also be recognized in a similar way.

The reconstruction task of each work-troop is defined as a sub-gene in our study. The initial sub-gene (reconstruction route) for each work-troop is constructed as follows: firstly, we use Figure 3.7 to establish a node linkage table, which describes the from/to relation between any two nodes. Secondly, the damage points are randomly partitioned into 3 sets. Finally, using the linkage table to randomly combine the damage points and nodes for each damage point set, in order to create a feasible reconstruction route for each work-troop.

(b) Crossover

Since the work-troop routing problem in Equations (3.42)-(3.51) is similar to the vehicle routing problem (VRP) in many aspects, the crossover in this study is also designed by the route exchange concept of the VRP (Liepins, et al., 1990). The crossover is defined as follows: first, we randomly choose two sub-genes of a gene, which is randomly selected from a gene population. One of these sub-genes is defined as the father gene, and the other sub–gene is defined as the mother gene. For example, the work plan of troop A is the father gene and the work plan of troop C is the mother gene. Secondly, checking if there are the same or compatible access points between the father gene and mother gene. If the same or compatible access point (s) is found, then the cut-point (s) is decided. Compatible access points are recognized as follows, for example, the damage point 2 () can be reached from the node 6 or node 9; thus, the node 6 and node 9 are two compatible access points. Taking the Figure 3.11 as an example, the work troop A and troop C both travel through node 6 and 10; thus, two traveling points are found to be the same. The node 6 and 10 is then used for two cut-points. Thirdly, the content between two cut-points in the father gene or mother gene will be extracted. For example, the extracted content from the father gene in Figure 3.11 is “6910”, and from the mother gene is “1056”. Thus, the leftover in the father gene is “2”, and in the mother gene is a empty string. Finally, the extracted contents are swapped between the father gene and mother gene. Furthermore, this swap way is designed in order to keep a feasible reconstruction route for each work-troop. For example, in Figure 3.11, the work-troop C is sited on the node 10; thus, the elements extracted from the father gene (troop A) have to inversely attach to the leftover in the mother gene. Of course, if a feasible reconstruction route is not formed for any work-troop in such a crossover, the node linkage table will be used again to construct a newly shortest route based on the extracted elements. The crossover probability is set to 0.7. This crossover is shown in Figure 3.17.

(c) Mutation

The mutation is defined as the task exchange between any two work-troops without altering the restoration order. This process is proceeding as follows: first, we randomly choose two sub-genes of a gene, which is randomly selected in the population. Secondly, the work tasks are swapped between these two work-troops. Of course, the node linkage table is used again to find a feasible reconstruction route based on the swapped tasks. The mutation probability is set to 0.1. This mutation of work plan for the troop A and troop C can be illustrated as in Figure 3.18. The reconstruction order is held for any work–troop in such a process.

A: 26910**

C: 10**56

A: 265

C: 10**96

Figure 3.17 The Crossover of a Reconstruction Plan

A: 269**10

C: 105***6

A: 26***5

C: 10569**10

Figure 3.18 The Mutation of a Reconstruction Plan

(d) Selection

The gene evaluation used in this study is the elitism selection, which means the gene with a higher performance index is more preferred. Since our model is a multi-objective problem, the performance index is defined as finding a solution, which is more close to each individual objective optimum. Each gene will be evaluated by Equation (3.55), and the gene with a lower value of EI is more preferred and highly ranked. The evaluation index (EI) for any gene in population is presented as follows:

EIg=p=13Zp−ZpgZp∀g                                            (3.55)

where :

g: the gene in population, g=1,2,…,5;

EIg: the evaluation index of the g-th gene in the population;

p: the p-th objective, p=1,2,3;

Zpg: the p-th objective value of the g-th gene in the population;

Zp: the heuristic and individual optimum of the p-th objective.

The aforementioned GA for work scheduling and assignment in this study will derive satisfying solutions through 30 generations after 20 runs. A generation is defined as a process to make gene population undergo elitism-selection, crossover and mutation once. We omit the reproduction process in this study in order to avoid the early maturity of the gene population, because the early maturity of a population will result in a local optimum (Michalewicz, 1996). A run is defined as the re-starting process with a different random seed to iterate GA for 30 generations. This special optimizing steps of our NRP is illustrated as follows:

Step 1

Input the relevant data required for the reconstruction order and work assignment. For example, the OD traffic demand of a pre-earthquake road-network, damage points need to be reconstructed, available work-troops, etc.

Step 2

Encode the reconstruction task of each work-troop by aforementioned GA and iterate GA for 10 runs to compute the heuristic optimal value of the second and third objective, e.g., Z2 and Z3. While Z1 is exactly computed from the pre-earthquake traffic condition of a road-network. The initial gene population is set to five genes.

Step 3

Use the relevant crossover, mutation to extend offspring genes. The achievement level of each gene will be evaluated by Equation (3.55), and the gene with a lower EI is more preferred and highly ranked. The best five genes will be kept by elitism-selection after Step 3 terminated.

Step 4

Iterate Step 3 for 20 runs. The heuristic optimal value of the second, third objective and EI will be renewed in each run if any better objective value is found.

Step 5

Use the final (latest) EI to evaluate the genes remained in the final population. Thus, the heuristic reconstruction plan is obtained.

After aforementioned five steps, a heuristic solution can be easily derived. The GA process for the optimal reconstruction plan is illustrated in Figure 3.20. A simple example is used to validate our model and optimizing process in next Section.

#### 3.3.3.3 Numerical Example and Discussions

Since lacking the real traffic data and geometric design of road-network in Northridge (Figure 3.1), an assumed post-quake road-network in Figure 3.7 and Table 3.6 is used again to validate our model and revised GA. The reconstruction time for a damage point is depending on the ability of each work-troop and is estimated from 12 to 24 time- intervals by civil engineers. Moreover, the travel time from any work troop to any damage point is exactly computed based on the physical routes in Figure 3.7. ( see Appendix C ). To reflect the traffic variation in different reconstruction states, the traffic volume in each link will be re-assigned if a damage point is fully recovered. The pre-quake traffic demand of Figure 3.7 is expressed in an OD matrix, which is shown in Table 3.6. The detailed network and the detailed link costs are shown in Appendix A, which are identical to that of Nagurney’s (Nagurney, 1984).

After inputting the desired parameters of Equations (3.42)-(3.54), loading traffic demand from Table 3.6, and applying five resolution steps in Section 3.3.3.2, a heuristic reconstruction schedule is easily obtained in Figure 3.19. The heuristic objective value, Z1= 31431.29 (traffic volume  time-interval), Z2= 42 (time-interval) and Z3= 18 (time-interval), respectively. The Figure 3.19 can be recognized as follows: the work-troop A starts its reconstruction trip form the node 2, then arrives at the node 6 and begins to reconstruct the damage point 1 (). After the damage point 1 is completely recovered, the work-troop A arrives at node 5 and continuously goes to the node 10 for reconstructing damage point 3 (). The reconstruction task of work-troop B and C can also be explained in a similar way. The Figure 3.19 is practically available for relevant authorities in order to reduce the negative impact to a post-quake transportation network. Since this NRP model is established by the mathematical optimization, with more precise data collected for our model, an appropriate reconstruction schedule can be scientifically made instead of messy decisions after a large-scale quake.

Figure 3.19   A Heuristic Reconstruction Schedule for Each Work-troop

Figure 3.20  The Modified GA Process

3.3.4 A NRP Model with multiple work-troops, fuzzy multi-objectives and traffic assignment

1.0

ZkZk

Zk−

0

Zk

Figure 3.21  The Achievement Level for Fuzzy Objectives (k=1,2,3)

A fuzzy multi-objective approach is continuously applied because that the exact solution for Equations (3.42)-(3.54) is very hard to find; thus, our model is heuristically resolved for maximal satisfaction of the decision maker instead of finding the global optimization with intolerable computation time. Coincidentally, the basic concept of fuzzy multi-objective optimization is to find the maximal satisfying degree (or achievement level) among many conflicting objectives（Sakawa, et al, 1994, 1997）. Using our three objectives in Equation (3.42) as an example, and assuming that the optimistic value of kth (k=1,2,3) objective is Zk+, while the pessimistic value is Zk−, the achievement level is expressed as in Figure 3.21 (Hannan, 1981).

The μkZk represents the fuzzy membership function of Zk, it also reveals the achievement level of Zk. By using the λ transformation（Hannan, 1981）, and λ= Minkλ, Equations (3.42)-(3.54) can also be rewritten as a similar form like that in Equation (3.41)– please see Equation (3.56).

3.3.4.1 Model Resolved by Genetic Algorithm

We expand Figure 3.7 to a more real situation– this is shown in Figure 3.22. To solve the fuzzy multi-objective NRP model, the concept for optimizing a bi-level model is used again (LeBlanc and Boyce, 1986; Tzeng and Tsaur, 1997). We had discussed these resolution details in the previous section; thus, we won’t repeat these details again.

Maxkλst    λ≤Zk−Zk−Zk+−Zk−,k=1,2,3                      (3.56)

and Equations (3.43)-(3.54).

The fuzzy approach is summarized as follows:

Step 1

Input the relevant data required for the reconstruction order and work assignment. For example, the OD traffic demand of pre-earthquake road-network, damage points need to be reconstructed, available work-troops, and so on.

Step 2

Encode the reconstruction order by the aforementioned GA and expanding the initial gene population by GA with 10 runs to compute heuristically the optimistic value (Zk+) and the pessimistic value (Zk−) of the second and third objectives (k=2,3). Compute Z1 from the pre-earthquake and post-quake traffic based on Figure 3.22. Furthermore, the initial gene population is set to be ten genes.

Step 3

Use the relevant crossover, mutation to extend the offspring genes. The achievement level of each gene will be evaluated by Equation (3.57) for k =1,2 and 3. The evaluation index (EI) for any gene in the population is presented as follows:

EIg=Maxgλg                                                  (3.57)

where :

g: the gene in the population, g=1,2,…,10;

















: Work-troop for Reconstructing Damage Points

A

B

C

D





: City

: Damage Point

Figure 3.22  An Abstracted Situation of Post-earthquake Road-network

EIg: the evaluation index of g-th gene in the population;

λg: the minimal value of λ (Minkλ, k=1,2,3) for g-th gene from Equation (3.14).

The gene with higher value of EIgis more preferred and highly ranked. The best ten genes will be kept by rank-selection after Step 3 terminates.

Step 4

Iterate Step 3 for 20 runs. The values of Zk+ and EIgwill be dynamically changed in each run if better objective values are found.

Step 5

Use the final (latest) EIgto evaluate the genes remaining in the final population. Thus, the heuristic reconstruction plan is obtained.

Using our GA and the aforementioned five steps, a heuristic solution can be easily derived. A numerical example is used to validate our model and optimizing process in next Section.

#### 3.3.4.2 Numerical Example and Discussions

An assumed post-quake road-network in Figure 3.22 is used to validate our model and GA. The reconstruction time for a damage point depends on the ability of each work-troop and is estimated to be from 30 to 80 time-intervals for a single damage point by civil engineers. Moreover, the travel time and the shortest path for any work troop to reach any damage point will be recognized (see Appendix D). To reflect the dynamics of traffic affected by different recovery states of network, the traffic volume in each link will be re-assigned if a damage point is fully recovered. The pre-quake traffic demand of Figure 3.22 is expressed in an OD matrix, which is shown in Table 3.7. The detailed network and the detailed link costs are individually established by a similar concept of Nagurney’s (Nagurney, 1984).

Table 3.7  The Assumed Traffic Demand in Figure 3.22

| From\To | 10 | 11 | 14 | 15 |
| --- | --- | --- | --- | --- |
| 1 | 470 | 480 | 450 | 400 |
| 2 | 480 | 455 | 470 | 450 |
| 13 | 465 | 440 | 400 | 500 |
| 20 | 470 | 450 | 460 | 450 |

After the desired parameters of Equations are input, the traffic demand from Table 3.7 is loaded. We apply five steps in Section 3.3.4.1; therefore, a heuristic reconstruction schedule is finally obtained, this schedule is shown in Figure 3.20. The heuristic objective values are, Z1= 3,256,547 (traffic volume×time-intervals), Z2= 105 (time-intervals), Z3= 23 (time-intervals) and the global achievement level with λ= 0.63, respectively. The Figure 3.23 can be interpreted as follows: the work-troop A starts its reconstruction trip form repairing damage point 1 (). After damage point 1 is completely reconstructed, work-troop A arrives at node 2 and continues to pass nodes 1 and 3 for reconstructing damage point 3 (). And after damage point 3 is recovered, work-troop A arrives at node 12 and orderly travels through nodes 11, 10, 17 and 9 for the reconstruction of damage point 8 (). The work-troops B, C and D can also be explained in a similar way. The Figure 3.23 can be made available for relevant authorities for pre-quake simulation in order to reduce the negative impact on the transportation network. Furthermore, since this reconstruction schedule is powerful and effective, it not only can be a reaction plan for pre-quake exercise of work-troops, but also be full of potential to be installed on a portable computer for real-time restoration of the post-earthquake road-network by a radioactive command.

A: 1--1-3--11-10-17-19--20

B: 2-6-8-16-17-19-20-21--24--13

C: 13-24-23-14--10-9-5--6

D: 20-22-23--15--9-8--6

Figure 3.23   A Heuristic Reconstruction Schedule for Each Work-troop

3.4 Discussions and Summary

In this Chapter, we formulate two models: the first one is used to find the optimal location of airport firestation, and the second one is developed to find an effective restoration process for a damage roadnetwork. We had both introduced these models and validated their effectiveness so far.

In view of the airport firestation problem, from GA, three sets of firestation site are obtained, which x-y coordinates are (1, 6), (9, 6) and (6, 2). The final achievement level is 0.75. These three firestations can be easily located in Figure 3.6 by the triangle symbols. Comparing the GA and enumeration method, GA uses less time than enumeration method is firstly concluded (20.33 min vs. 73.57 min), yet still can perform a satisfying achievement level between conflicting objectives (λ=0.75 vs. λ=0.78). Therefore, the GA did show its feasibility and effectiveness in this study. Finally, the GA has potential of efficiently solving large-scale location problem of other facilities. This means many optimal sites (e.g., three or much more than three sites) of other facility can be easily derived through our model, which is very easy to be operated and can be a powerful support basis for location decision. The airport authorities should follow our systematical approach to set their own optimal firestation sites– this will help promote the efficiency of rescuing lives and fire-fighting activities. The tragedy of China Airlines crashing near the Taipei’s International Airport in 1998, stimulated the relevant authority to study more advanced topics for promoting the efficiency of fire-protection and rescue at the airport. The sites and the number of firestation are factors heavily affecting the reaction efficiency; therefore, a location model is implemented by a fuzzy multi-objective approach in order to decide the optimal number and sites of firestation at the  Taipei’s International Airport. Furthermore, because of the combinatorial complexity in resolution, the GA is then employed and proved to be effective when compared to the enumeration method. Thus, our model has the potential to be expanded for the optimal layout of other facilities.

And in view of the network restoration problem (NRP), we introduce the formulation concept and resolution steps. Since the solution space of our NRP model is unknown, we apply and generate many corresponding GAs to solve such a combinatorial optimization problem. The NRP are presented from the simplest one to the complicated one. From the simplest one, we conclude that the subjective and traditional restoration concept– firstly reconstructing the damage point, which heavily affects the link volume, is not appropriate. We should first reconstruct the damage point, which consumes less recovery time. Furthermore, from Figure 3.14, 3.19 and 3.23, we recognize these reconstruction plans are practically available for relevant authorities to pre-simulate responses in order to reduce the negative impact of a post-quake transportation network. Since this NRP is based on mathematical optimization, with more precise data for our model, a reconstruction schedule can be scientifically made instead of messy decisions after a large-scale earthquake. Furthermore, since this reconstruction schedule is powerful and effective, it can not only be a basis for pre-quake-exercise, but also be full of potential to be installed on a portable computer for real-time restoration decision by a radioactive command. Finally, this NRP model can be regarded as a basis of decision support system (DSS) for facility location in a large-scale environment. We now have devoted ourselves to combine this NRP research with a geographic information system (GIS) so as to greater reflect reality. In next Section, we will summarize the research values of these models and discuss some future issues, which should be researched and developed.