Content

Content	1

List of Pictures	3

Chapter1 Introduction	5

1.1	Motivation	5

1.2	Objective	9

1.3	Study Flowchart	10

Chapter2 Literature Review	12

2.1	Review of National Transportation Communication for ITS Protocol (NTCIP)	12

2.1.1	Introduction of NTCIP	12

2.1.2	Benefits of NTCIP	13

2.1.3	The NTCIP Standards Framework	13

2.2	Review of Transport level with TCP/IP and UDP/IP	17

2.2.1	TCP (Transmission Control Protocol) and UDP (User Datagram Protocol)	17

2.2.2	Comparison of UDP and TCP	20

2.3	Simulation in communication Network	22

2.3.1	Simulation for layered communication	22

2.3.2	Evaluation Indices of layered communications	22

Chapter3 System Framework	24

3.1	Adjustable Parameters	27

Chapter4 Data Transmission Simulation Model	28

4.1	Basic Assumptions	28

4.2	Rationale of Data Transmission Simulation Model	29

4.3	Data Flow of Simulation Model	31

Chapter5 Numerical Experiments	35

5.1	Basic Experiments	35

5.1.1	Amount of transmitted data packets	35

5.1.2	Inter-arrival Time	36

5.2	Adjustable Parameters Experiments	38

5.2.1	Adjust One Parameter	39

5.2.2	Adjust Two Parameters	45

5.3	Summary	55

Chapter6 Conclusion and Suggestion	57

6.1 Conclusion	57

6.2 Suggestion	59

References	60

# List of Pictures

Figure 1.1-1 NTCIP framework ………………………………………………………7

Figure 1.1-2 Scope of layered communication in this research ………………………8

Figure1.3-1Study Flowchart …………………………………………………………10

Figure 2.1.3-1 NTCIP Standards Framework ……………………………………….15

Figure 2.2-1 Structure of TCP/IP ……………………………………………………19

Figure 2.2-2 IP in LAN ……………………………………………………………...20

Figure 3-1 Scope of layered communication in this research ……………………….24

Figure 3-2 System Framework ………………………………………………………25

Figure 4.2-1 Queue model in the sending side ………………………………………29

Figure 4.2-2 Queue model in the receiving side …………………………………….30

Figure 4.2-3 Data transmission from sending side to receiving side ………………..30

Figure 4.3-1 Lower Part is embedded as A Process Server …………………………31

Figure 4.3-2 Simulation Model ……………………………………………………...34

Figure5.1.1-1 Amount of transmitted data packet …………………………………..36

Figure 5.1.2-1 Inter-arrival Time ……………………………………………………37

Figure 5.2.1-1. System Time of Queue Length ……………………………………...40

Figure 5.2.1-2. Throughput of Queue Length ……………………………………….40

Figure 5.2.1-3. System Time of Timeout period …………………………………….42

Figure 5.2.1-4. Throughput of Timeout period ……………………………………...42

Figure 5.2.1-5 System Time of Retrial ………………………………………………44

Figure 5.2.1-6 Throughput of Retrial ………………………………………………..44

Figure 5.2.2-1 System Time of Queue Length vs. Retrial in high scenario …………46

Figure 5.2.2-2 Throughput of Queue Length vs. Retrial in high scenario …………..46

Figure 5.2.2-3 System Time of Queue Length vs. Retrial in middle scenario ………47

Figure 5.2.2-4 Throughput of Queue Length vs. Retrial in middle scenario ………..47

Figure 5.2.2-5 System Time of Queue Length vs. Retrial in low scenario ………….48

Figure 5.2.2-6 Throughput of Queue Length vs. Retrial in low scenario …………...48

Figure 5.2.2-7 System Time of Queue Length vs. Timeout period in high scenario ..49

Figure 5.2.2-8 Throughput of Queue Length vs. Timeout period in high scenario …49

Figure 5.2.2-9 System Time of Queue Length vs. Timeout period in middle scenario ………………………………………………………………50

Figure 5.2.2-10 Throughput of Queue Length vs. Timeout period in middle scenario ………………………………………………………………50

Figure 5.2.2-11 System Time of Queue Length vs. Timeout period in low scenario .51

Figure 5.2.2-12 Throughput of Queue Length vs. Timeout period in low scenario …51

Figure 5.2.2-13 System Time of Retrial vs. Timeout period in high scenario ………52

Figure 5.2.2-14 Throughput of Retrial vs. Timeout period in high scenario ………..53

Figure 5.2.2-15 System Time of Retrial vs. Timeout period in middle scenario ……53

Figure 5.2.2-16 Throughput of Retrial vs. Timeout period in middle scenario ……..54

Figure 5.2.2-17 System Time of Retrial vs. Timeout period in low scenario ……….54

Figure 5.2.2-18 Throughput of Retrial vs. Timeout period in low scenario ………...54

Figure 5.3-1 Result of the adjustable parameters ……………………………………56

Figure 6.1-1 Result of the adjustable parameters……………..……………………...58

# Chapter1 Introduction

## Motivation

Transportation is always playing an important role of our daily life. An efficient and effective transportation system, which could provide the competition edge and reduce the social costs, is the basis for a modern country.

The traditional approach couldn’t solve the deteriorate transportation problem. Therefore, a new approach, which integrates with computer science, control, information and transportation, is emerging. We call such system as ITS, Intelligent Transportation System.

However, there are several problems we must concern to implement ITS, such as the interconnectivity, the interchangeability and the interoperability.

Interchangeability is defined as the capability to exchange devices of the same type (e.g., a signal controller from different vendors) without changing the software. Interoperability is defined as the capability to operate devices from different manufacturers, or different device types (e.g., signal controllers and dynamic message signs) on the same communications channel.

In order to solve these three issues, we need a common language for all the components’ communication in ITS. So the first problem we will encounter as ITS going is the communication protocols between different devices and components. If all the providers make their own products with different communication protocols, the devices belong to different providers will not be compatible within the same system. That is, different areas get the close systems to the others. In such case, the built system will monopolize the market so that the buyer can’t get the same devices from the other providers and the other providers can’t get into the built system, either. Besides, each system provided from different provider has it’s own standards different from the others so that providers can’t get enough demand as large as being economics scale. It will be inefficiency of the investment and may cause the related industry recession.

In order to address these issues, the USA proposes National Transportation Communications for ITS protocol (NTCIP). The NTCIP is receiving considerable attentions these days. The goal of NTCIP is to provide a set of standard protocols with interchangeability and interoperability among the various components of transportation systems could be achieved.

NTCIP is a suite of communications protocols and data definitions that have been designed to accommodate the diverse needs of various subsystems and user services of the National ITS Architecture. NTCIP uses a layered or modular approach to communications standards, similar to the layering approach adopted by the Internet and the International Standards Organization (ISO). In general, data communications between two computers or other electronic devices can be considered to involve the following primary layers, called “levels” in NTCIP to distinguish them from those defined by ISO and the Internet. It contains with five levels: Information level, Application level, Transport level, Subnetwork level, and Plant level. And in each level includes kinds of communication protocols, some are unique to transportation industry and some are the on-going standards used by the broader computer and telecommunications industries. The framework of NTCIP is shown as the Figure 1.1-1.

Figure 1.1-1 NTCIP framework

A particular message transmission can use, at most, one standard at each level or sublevel of the NTCIP framework. The series of standards used in the message transmission is called a “stack” of standards, or a “protocol stack”. It is possible for a pair of electronic devices to exchange some messages using one stack and other messages using a different stack, though usually, such stacks will differ only at one or two levels or sublevels.

The issue is how do the government or other sections choose the “right” protocol and protocol stacks, if governments or some sections related with transportation are planning to follow out ITS with NTCIP framework. Theoretically, we need to choose one from each level, and make sure the combination is optimal.

From this practical standpoint, the lower three levels are provided by commercial providers, such as GSM, Internet. The upper level is based on the data object and available devices or systems.

Considering the data transmissions in ITS, the real time requirement is needed. And most connections of controllers and centers will move to the IP base with the Internet fast developed. Therefore based on these specific applications in ITS, we have to make the real-time constraint and consider the Internet transmission efficiency.

However, the Internet is used by various applications jointly without priority for transportation. Thus, we have to adjust the related parameters which have much influence on the performance in the transport level to reduce the data transmission time.

After realizing these two conditions, we have to change some parameters in the transport level, such as retrial of ACK, Queue length and Time-out period parameters in TCP/IP and UDP/IP. The scope is shown as Figure 1.1-2.

Figure 1.1-2 Scope of layered communication in this research

## Objective

The objective of this research is to propose an approach and method to calibrate the parameters to get the better performance and determine that which the key parameters are under various conditions. The method tries to make the appropriate protocol stacks efficiently for various transportation applications, with considering in various environments and situations. It tries to determine the adaptive parameters’ values interactively for minimizing the total system time for all needed data packets transmission completed. Because some transportation applications may need real time responses and the network status also changes time over time so that the message transmissions through all levels have to consider the congestion in the transmission process and minimize the mean transmission delay.

## Study Flowchart

Figure 1.3-1 Study Flowchart

As shown in the flowchart, we first define the problem what we are concerning about. After realizing the problem we encounter, review relative literatures of queuing network models in such layered communication systems and some performance evaluation methods for the plant level and subnetwork level to understand the current developments. Also, we will define the related parameters.

Then we propose the model formulation of data transmission and also consider the relative literatures about performance evaluation methods for the plant level and subnetwork level; parameters estimated for the application level and relative transport level, such as UDP/IP and TCP/IP in order to modify our model.

We propose a queuing network model to describe the process of data transmission through these five levels, and select the simulation method to work out the solutions. Based on the solutions we get, we will put out them into output analysis and constantly modify our model until we are satisfied.

# Chapter2 Literature Review

## Review of National Transportation Communication for ITS Protocol (NTCIP)

### Introduction of NTCIP

A communications protocol is a set of rules for how messages are coded and transmitted between electronic devices. The equipment at each end of a data transmission must use the same protocol to successfully communicate. It is a bit like human languages that have an alphabet, vocabulary, and grammar rules used by everyone speaking that language.

NTCIP is a family of communications standards for transmitting primarily data and messages between microcomputer control devices used in Intelligent Transportation Systems (ITS). NTCIP is intended for use in all types of management systems dealing with the transportation environment, including those for freeways, traffic signals, transit, emergency management, traveler information, and data archiving. NTCIP is intended for use between computers in different systems or different management centers, and between a computer and devices at the roadside.

NTCIP provides communications standards for two fundamentally different types of ITS communications. The first type is between a management system or center and multiple control or monitoring devices managed by that system or center.  The second type of communication involves messages sent between two or more management systems.

NTCIP is a suite of communications protocols and data definitions that have been designed to accommodate the diverse needs of various subsystems and user services of the National ITS Architecture. It is intended principally to handle these needs in two areas: communications between a management center and field devices, and communications between two or more management centers.

### Benefits of NTCIP

NTCIP offers increased flexibility and choices for agencies operating transportation management systems. It removes barriers to interagency coordination and allows equipment of different types and manufacturers to be mixed on the same communications line. For these reasons, operating agencies will benefit from specifying that NTCIP be included in all future purchases and upgrades, even if NTCIP is not planned to be used initially.

NTCIP can provide some benefits such as: Avoid early obsolescence, Provide choice of vendor, Enable interagency coordination, and Use one communications network for all purposes.

### The NTCIP Standards Framework

NTCIP uses a layered or modular approach to communications standards, similar to the layering approach adopted by the Internet and the International Standards Organization (ISO). In general, data communications between two computers or other electronic devices can be considered to involve the following primary layers, called “levels” in NTCIP to distinguish them from those defined by ISO and the Internet:

- Information Level – This level provides standards for the data elements, objects, and messages to be transmitted (e.g., TCIP, TS3.5, MS/ETMCC, etc.).

- Application Level – This level provides standards for the data packet structure and session management. (e.g., SNMP, STMP, DATEX, CORBA, FTP, etc.). Transport Level – This level provides standards for data packet subdivision, packet reassembly, and routing when needed (e.g., TCP, UDP, IP).

- Subnetwork Level – This level provides standards for the physical interface (e.g., modem, network interface card, CSU/DSU, etc.), and the data packet transmission method (e.g., HDLC, PPP, Ethernet, ATM, etc.).

- Plant Level – This level consists of the physical transmission media used for communications (e.g., copper wire, coaxial cable, fiber optic cable, wireless)

The NTCIP Standards Framework is shown as Figure 2.1.3-1. A particular message transmission can use, at most, one standard at each level or sublevel of the NTCIP framework. The series of standards used in the message transmission is called a “stack” of standards, or a “protocol stack”. It is possible for a pair of electronic devices to exchange some messages using one stack and other messages using a different stack, though usually, such stacks will differ only at one or two levels or sublevels.

The levels shown in the framework are somewhat different from communication stack layers defined by the International Standards Organization (Open Systems Interconnect seven-layer reference model) and other standards making organizations. This document describes the NTCIP stack as extending beyond the communications stack to include informational data and interfaces to the physical communications infrastructure.

Figure 2.1.3-1 NTCIP Standards Framework

The information level standards used in Intelligent Transportation Systems (ITS) are unique to the transportation industry. The National ITS Architecture and much of the on-going standards development effort for ITS involve identification of required data elements and their compilation into standard objects or message sets for all the different domains and functions within ITS (e.g., traffic, transit, traveler information, emergency management, etc.).

For the subnetwork and transport levels, ITS can generally use existing standards used by the broader computer and telecommunications industries. NTCIP has not had to develop significantly new standards in these areas, but has merely chosen which existing standards are to be used in ITS (the Internet standards have been adopted where possible), and has specified which options to use where alternatives are available in some standards.

The application level is the primary focus of NTCIP. Although some existing standards are useful here, ITS has special requirements that have necessitated the extension of existing standards, or development of entirely new protocols for specific applications within ITS.

## Review of Transport level with TCP/IP and UDP/IP

### TCP (Transmission Control Protocol) and UDP (User Datagram Protocol)

TCP and IP were developed by a Department of Defense (DOD) research project to connect a number different networks designed by different vendors into a network of networks (the "Internet").

TCP/IP is a set of protocols developed to allow cooperating computers to share resources across a network. It was developed by a community of researchers centered on the ARPAnet. Certainly the ARPAnet is the best-known TCP/IP network. However as of June, 87, at least 130 different vendors had products that support TCP/IP, and thousands of networks of all kinds use it. Whatever it is called, TCP/IP is a family of protocols. A few provide "low-level" functions needed for many applications. These include IP, TCP, and UDP.

TCP/IP is a layered set of protocols. TCP is responsible for making sure that the commands get through to the other end. It keeps track of what is sent, and retransmits anything that did not get through. If any message is too large for one datagram, e.g. the text of the mail, TCP will split it up into several datagrams, and make sure that they all arrive correctly. It can be thought of TCP as forming a library of routines that applications can use when they need reliable network communications with another computer. Similarly, TCP calls on the services of IP. Whatever it is called, TCP/IP is a family of protocols. A few provide "low-level" functions needed for many applications. These include IP, TCP, and UDP. Others are protocols for doing specific tasks, e.g. transferring files between computers, sending mail, or finding out who is logged in on another computer.

Although the services that TCP supplies are needed by many applications, there are still some kinds of applications that don't need them. However there are some services that every application needs. So these services are put together into IP. As with TCP, you can think of IP as a library of routines that TCP calls on, but which is also available to applications that don't use TCP. Sometimes an application on a network needs to send messages to a specific application or process on another network. The UDP provides a datagram means of communication between applications on Internet hosts. Because senders do not know which processes are active at any given moment, UDP uses destination protocol ports (or abstract destination points within a machine), identified by positive integers, to send messages to one of multiple destinations on a host. The protocol ports receive and hold messages in queues until applications on the receiving network can retrieve them.

Since UDP relies on the underlying IP to send its datagrams, UDP provides the same connectionless message delivery as IP. It offers no assurance of datagram delivery or duplication protection. However, UDP does allow the sender to specify source and destination port numbers for the message and calculates a checksum of both the data and header. These two features allow the sending and receiving applications to ensure the correct delivery of a message. Applications that require reliable delivery of datagrams must implement their own reliability checks when using UDP. Applications that require reliable delivery of streams of data should use TCP. This strategy of building several levels of protocol is called "layering". We think of the applications programs such as mail, TCP, and IP, as being separate "layers", each of which calls on the services of the layer below it. Generally, TCP/IP applications use 4 layers, shown as Figure 2.2-1:

- An application protocol such as mail

- A protocol such as TCP that provides services need by many applications

- IP, which provides the basic service of getting datagrams to their destination

- The protocols needed to manage a specific physical medium, such as Ethernet or a point-to-point line.

| Application Layer | Telnet, FTP, SMTP, Http, SMTP, SNMP, DNS |  |
| --- | --- | --- |
| Transport Layer | TCP | UDP |
| Network Layer | IP |  |
| Link Layer | Ethernet |  |

Figure 2.2-1 Structure of TCP/IP

The Internet Protocol (IP) is responsible for moving packet of data from node to node. IP forwards each packet based on a four bytes destination address (the IP number). The Internet authorities assign ranges of numbers to different organizations. The organizations assign groups of their numbers to departments. IP operates on gateway machines that move data from department to organization to region and then around the world. Figure 2.2-2 shows how does IP work in LAN.

Figure 2.2-2 IP in LAN

### Comparison of UDP and TCP

The TCP/IP transport-level protocols allow application programs to communicate with other application programs. User Datagram Protocol (UDP) and the TCP are the basic transport-level protocols for making connections between Internet hosts. Both TCP and UDP allow programs to send messages to and receive messages from applications on other hosts. When an application sends a request to the Transport layer to send a message, UDP and TCP break the information into packets, add a packet header including the destination address, and send the information to the Network layer for further processing. Both TCP and UDP use protocol ports on the host to identify the specific destination of the message.

Higher-level protocols and applications use UDP to make datagram connections and TCP to make stream connections. The operating system sockets interface implements these protocols.

Since UDP relies on the underlying IP to send its datagrams, UDP provides the same connectionless message delivery as IP. It offers no assurance of datagram delivery or duplication protection. Because of this, UDP has more convenient transmissions but oppositely it has lower reliability. Once there is error, then it has to recovery this depended on upper layer, application layer. Why does UDP still work at present even if it has this problem? There are several reasons: to reduce the demand for the resource of computer. If the network system has special property such as DNS Server has to deal with huge of request messages from client, then it will occur the heavy loading to the server. Second, if the application layer has already provide the check schema for the data complete, then it won’t be necessary to deal with this kind of problem at transport layer. Third, if in such case that the system provides multicast or broadcast function, then it is going to use UDP because of the connection-oriented property of TCP only supporting point to point communications.

Unlike UDP, TCP provides reliable message delivery. TCP ensures that data is not damaged, lost, duplicated, or delivered out of order to a receiving process. This assurance of transport reliability keeps applications programmers from having to build communications safeguards into their software. Besides, the connection-oriented property of TCP contains data confirmation, retransmission and flow control schema. The function of data confirmation and retransmission will rise up the reliability of TCP, and the function of flow control will make a more efficient use of bandwidth of Internet. The schema of flow control contains sliding windows, adjusting window size. Most time, the destination port will decide the window size, because the destination port will realize how many data he can handle, if it is too busy that it can’t deal with the data sent from the original port in time, then it will notify the original port by the returned ACK so that it can adjust the data flow by this way.

## Simulation in communication Network

### Simulation for layered communication

Multi-layered communication architecture is normally used in computer communication. In order to obtain good performance in end-to-end level, protocol performance should be analyzed with taking the layered structure into consideration.

Hiroshi Inai[3] proposed layered communication analytical model for predicting the performance of protocol software.

It introduced several simulation-based approximations to assess the accuracy of the algorithm.

Yamamoto, M[4] also used simulation to investigate the performance of the window-flow-control scheme. When communication packet networks are connected, the network traffic could be calculated at the connecting points. The research also pointed out key variables affecting data transmission.

Most of researches of layered communication adapt simulation as tool to access the performance of layered communication models.

### Evaluation Indices of layered communications

Apostolopoulos, T.K. [5] proposed an SNMP-based management framework for monitoring and controlling the performance of resources in a TCP/IP network. This research proposed using retransmission probability, network service utilization and reassembly failure probability as the performance indicators.

Hiroshi Inai[6] designed a series of numerical experiments for the layered communication protocol in LAN environment and suggested some adjustable parameters, such as window size, time-out period and buffer size in the lower levels.

Jaime Jungok Bae[7] investigated the effects of protocol processing overhead of error recovery schemes. The focus is on the edge-to-edge error recovery scheme, in which retransmissions of error packets only take place between source and destination nodes. The performance measures obtained in this paper are the distribution of transfer delays and the loss probability of packets across a network.

Most researches of layered communication network evaluate system performance by using simulation and statistical method and regard the system as static one. However, the network status is changing dynamically. The network may be jammed, overload and failures. It is wise to adjust the parameters depending on the network status interactively in order to obtain the best performance.

# Chapter3 System Framework

We propose a data transmission model to reflect the data flow of NTCIP protocol levels. The data transmission model is referred to Hiroshi Inai[3] [6] and based on the framework of NTCIP, which has a level structure corresponding to those of OSI. With this model, we could obtain relevant statistics such as data processing, such as delay-time, process-time, and total flow-time.

As discussed in section 1.1, the NTCIP’s five levels could be categorized into three parts. Because the real-time requirement is needed, we have to speed up the simulation model and the results. Therefore, we simplify the layered communication model into three parts. As shown in Figure 3-1, the upper part consists of the upper two levels, information level and application level. Users must specify the application dependent parameters, such as packet sending frequency in the upper part.

The middle part, the core of this research, consists of the transport level. We attempt to study the behaviors of parameters, such as retrial and timeout period in this part. The lower part consists of the lower two levels, subnetwork level and plant level. The third part usually represents commercial networks used.

Figure 3-1 Scope of layered communication in this research

Clearly, the parameters in the upper and lower parts are either user or commercial network specified. They are regarded as given input data. The data obtained from the upper part are data size and data sending frequency. The data yielded from the lower part are data lost rate, connection time, handshaking time (including the establishment and release), transfer failure rate, packet lost rate, and duplication.

Figure 3-2 System Framework

The idea of the proposed model could be illustrated in Figure 3-2. We first pipe the given input parameters into our simulation model. The simulation model could calculate the statistics of data processing, such as delay time, process time and total system time. These outcomes reflect the current performance of the system.

According to the outcomes from the simulation model, we change the values of the parameters and feedback to the simulation model to improve the system performance. With this procedure, we can conclude the suggested values of the parameters.

Recall that the purpose of this model is to determine the suggested parameters of the middle part in NTCIP categories. And try to conclude that the key parameters under various conditions. Thus we defined several different conditions and use the simulation model to generate the statistics of data processing. After analyzing the outcomes, we can conclude the key parameters the suggested values.

## Adjustable Parameters

We need to first define the adjustable parameters in the middle part as shown in Figure 3-1. The middle part is the transport level in NTCIP. There are three protocols, TCP, UDP, and IP, in this level.

According to the published research, the adjustable parameters are client numbers [9], freshness interval [9], push interval [11], ACK timeout period [10], flow control window size [4], and retries times [9].

- Client number is the number of control centers.

- Freshness interval is the time between the generation of a response and its expiration time.

- Push interval is time interval between automatic sending requests or commands. It will affect the number of data transmission.

- ACK time-out period is the max tolerance time between data sent and ACK received.

- Window size is the size of data transmitted each time.

- Retrial times specify the max number of retries of sending data.

In this research, we only consider three parameters, ACK time-out period, Window size and retrial times.

We just consider single server in our research, so that the client number won’t affect the performance in our consideration.

All the sending data will be specified by the user in this research, so that the server won’t send the request automatically. Therefore, we don’t consider of the client number, freshness interval and push interval.

# Chapter4 Data Transmission Simulation Model

## Basic Assumptions

To simplify the analysis in our simulation model, the following assumption are made.

- The arrival of data packets follows the Poisson distribution with parameter λ(number of arrivals per unit time). The valueλis application dependent.

- At each process server, the service time follows the Exponential distribution with parameter θ (mean service time). The value of θcould be obtained from relevant literature.

- The process of data packets follows the FIFO (First In First Out) schema.

- There is no transmission time between servers.

- The probability of packet lost is pre-defined as 0.05.

- The probability of packet error is pre-defined as 0.05.

- The stopping rule is amount of arrived data packets

## Rationale of Data Transmission Simulation Model

Figure 4.2-1 illustrates the concept of data sending in a communication network. The sending side has two kinds of arrivals, data packets and ACK. When data packets are processed, they will be sent to the receiving side. Otherwise, it will be sent to Timeout queue till the waiting time is over timeout period and retransmitted. The Timeout queue is a time counter to record the time of the processed data packet sent from Q1. When the waiting time of the data packet in Timeout queue is over timeout period, it will be sent to Q1 for retransmission.

When the ACK arrives, nothing needs to be done since the transmission is completed.

Figure 4.2-1 Queue model in the sending side

Figure 4.2-2 shows the concept of the receiving side. When the arrivals are received, the receiving side issues an ACK back to the sending side for confirmation.

Figure 4.2-2 Queue model in the receiving side

Figure 4.2-3 shows the concept of the data from the sending side to the receiving side. After data packets are processed at Q1, they are sent to the receiving side. If there were any error occurred at this moment, it would be sent to Timeout queue till the waiting time is over timeout period and retransmitted. When the arrivals are received, the receiving side issues related ACKs back to the sending side for confirmation. If there were any error occurred at this moment, it would be sent to Timeout queue till the waiting time is over timeout period and retransmitted.

Figure 4.2-3 Data transmission from sending side to receiving side

## Data Flow of Simulation Model

In this section, we will introduce the layered communication structure used in this research.

In the previous section, we can know the concept of the data packet transmitted from sending side and receiving side. Despite that the sending side and receiving side, we will introduce the behavior of the data packets in the lower part.

Figure 4.3-1 shows the lower part is regarded as a process server. The time spent of each data packet in this server means the transmitted time during lower part.

Figure 4.3-1 Lower Part is embedded as A Process Server

We build up a simulation model to obtain the needed statistics. The basic concept of our simulation model is based on the data transmission model and shown in Figure 4.3-2.

The circles mean the processing nodes, including the generator and the three servers. The arrows with sequential number mean the processing steps of data flow. The gray rectangles mean the parameters given from the lower part. Each white rectangle describes the related processing node.

In the beginning, the generator generates the incoming data packets following the Poisson distribution with parameter λ. Each data has two attributes: data type (data or ACK) and status (true or false).

We assume three processing servers to be represented as each process in multiple layered communication networks. Each server follows exponential distribution with different parameter θ.

In fact, there are two sources for Q1 ‘s arrivals. The first one is the incoming data packet. The second one is the ACK form Q2 server.

The processing one means the data from generator goes to the Q1 server for encoding. After encoding, follows processing two to Q2 server.

The time spent of each data packet at Q2 server means the transmission time in lower part. As data transmitted in the lower levels, errors might occur. The possible errors are packet loss, and packet error. In our simulation model, the data lost rate Ｐ is obtained from the relative literatures. Therefore, before dealing with data, Q2 server has to check the status of data, decided by the probability depending on the data lost rate Ｐ.

If there is no error occurred, in the lower part, data follows processing three to Q3 server, otherwise follows processing four to Q4 server.

The processing three represents the data goes to the Q3 server for decoding to retrieve the original information. Q3 server represents the receiving side, which is responsible for receiving data from the sending side and sending the acknowledgment back to the sending side. Q3 server has to decode and do the error check. In our simulation model, the data error rate π is obtained from the relative literatures.

The processing four represents the data goes to the Q4 server for waiting the time-out period.

The processing five represents the ACK after receiving the data goes back to its origin. Sending ACK is just like the sending data of normal. Thus, if there is no error occurred, ACK goes to processing eight, otherwise it goes to processing six.

The processing six represents the ACK goes to the Q4 server for waiting the time-out period.

The processing seven represents that if the time out is expired without receiving the ACK, Q4 server will issue the command to Q1 server for the data resending.

The processing eight represents the ACK back to the origin completing the flow. And at this time, Q1 server will count the completed data packets and it will stop the simulation till the stopping rule is satisfied.

Figure 4.3-2 Simulation Model

# Chapter5 Numerical Experiments

In this chapter, we have two categories of experiments, basic experiments and adjustable parameters experiments. The basic experiments are used to verify our data transmission model.

The adjustable parameters experiments are used to conclude the key parameters we interest and its value.

## Basic Experiments

In this section, we design a series of experiments to verify the proposed data transmission model.

We choose the inter-arrival time and the amount of transmitted data packets as two indices to help us to verify model accuracy and rationality.

The inter-arrival time and the amount of transmitted data packets will affect the performance of our data transmission model, but given by the user not the parameters we try to adjust. Therefore, we choose these two as our indices.

### Amount of transmitted data packets

According to the queuing theory, if the arrival rate is less than the service rate, then the system will reach the steady state.

We need to define some parameters before introducing the experiments. The transmitted completed data packet means the packet is sent from the sending side and successfully received at the receiving side. Moreover, the receiving side will send a acknowledgement called ACK back to the sending side as confirmation. After the sending side received the ACK, this data packet is completed transmitted.

The settings are as following:

- Service time of data in Q1 is 10ms per unit; ACK in Q1 is 7.5ms per unit. The service time of data in Q2 is 6.0ms per unit; ACK in Q2 is 5.6ms per unit. The service time of data in Q3 is 10ms per unit; ACK in Q3 is 7.5ms per unit.

- User specified: The inter-arrival time follows the Poisson distribution withλ= 100ms

- Network status: The Data Error Rate is 0.05 and the Data Lost Rate is 0.05

- Adjustable Parameters: Queue Length is unlimited; Timeout period is 10000ms; Retrial is 5.

- The stopping rule is the amount of the transmitted packets

The simulation output is shown as figure 5.1.2-1.

| Num of Data | 5000 | 10000 | 15000 | 20000 |
| --- | --- | --- | --- | --- |
| Average Delay (ms) | 7.55424 | 7.37344 | 7.44605 | 7.4897 |

Figure5.1.1-1 Amount of transmitted data packet

Clearly, the average delay will reach a constant, as the inter-arrival time is constant. The reason is that the service rate is larger than the arrival rate so that even the number of data increasing, the average won’t change and reach a constant.

### Inter-arrival Time

Inter-arrival time mean the time interval between two packers. We can easily know that the more data packets come, the more delay caused.

The scenario settings are as following:

- Service time of data in Q1 is 10ms per unit; ACK in Q1 is 7.5ms per unit. The service time of data in Q2 is 6.0ms per unit; ACK in Q2 is 5.6ms per unit. The service time of data in Q3 is 10ms per unit; ACK in Q3 is 7.5ms per unit.

- User specified: The inter-arrival time follows the Poisson distribution withλfrom 50ms to 120ms

- Network status: The Data Error Rate is 0.05 and the Data Lost Rate is 0.05

- Adjustable Parameters: Queue Length is unlimited; Timeout period is 10000ms; Retrial is 5.

- The amount of the transmitted packets is fixed value 10000

The simulation output is shown as figure 5.1.2-1.

| Inter-Arrival Time | 50 | 60 | 70 | 80 | 90 | 100 | 110 | 120 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Average Delay | 17756.1 | 13371.4 | 9662.31 | 8180.1 | 7095.2 | 3734.4 | 465.888 | 9.33768 |

Figure 5.1.2-1 Inter-arrival Time

According to the output, we can know that as the inter-arrival time is small, the average delay is large. As the inter-arrival time is increasing, the average delay is decreasing.

When the inter-arrival time is small, there is a high possibility that the server is busy, which result in a higher delay and vice versa.

Through the basic experiment, choosing two indices, inter-arrival time and the amount of transmitted data packets, and analyzing the outputs, we can conclude that the simulation model observes the basic queuing theory.

## Adjustable Parameters Experiments

In this section, we want to determine which is the key adjustable parameter and its suggested value. The adjustable parameters are the timeout period, queue length, and retrial times.

The performance indicator is the system time. The system time is defined as the terminated time of the simulation model under the condition those given amount of data are transmitted successfully.

The reason of choosing this indicator is that what we do care is the total amount of time for transmitting the desired data in the real world.

In the simulation system, if there is no any delay occurred, the mean total process time of each data packet is 46ms. We use the three scenarios, high arrival rate, middle arrival rate and low arrival rate. High arrival rate is defined as inter-arrival time is 60ms, that is the system will be close to saturation as 46/60 = 0.767. Middle arrival rate is defined as inter-arrival time is 100ms, that is the system will be half to saturation as 46/100 = 0.46. Low arrival rate is defined as inter-arrival time is 200ms, that is the system will be less to saturation as 46/200 = 0.23.

There are two parts of this experiment. The first part is to observe the system performance with only one parameter changed. The second part is to observe the system performance with two parameter changed.

### Adjust One Parameter

- Queue Length

The scenario settings are as following:

- Service time of data in Q1 is 10ms per unit; ACK in Q1 is 7.5ms per unit. The service time of data in Q2 is 6.0ms per unit; ACK in Q2 is 5.6ms per unit. The service time of data in Q3 is 10ms per unit; ACK in Q3 is 7.5ms per unit.

- User specified: The inter-arrival time follows the Poisson distribution withλ. High arrival is set as 60ms, middle arrival is set as 100ms, and low arrival is set as 200ms.

- Network status: The Data Error Rate is 0.05 and the Data Lost Rate is 0.05

- Adjustable Parameters: Queue Length is from 1 to 200; Timeout period is 10000ms; Retrial is 5.

- The stopping rule is the amount of the transmitted packets = 20000.

The simulation output is shown as figure 5.2.1-1 and figure 5.2.1-2.

| Queue Length | 1 | 3 | 10 | 100 | 200 |
| --- | --- | --- | --- | --- | --- |
| System Time (high) ms | 1273840 | 1743030 | 2444790 | 2679580 | 2151270 |
| System Time (mid) ms | 2021700 | 1986720 | 2265090 | 2784580 | 2638710 |
| System Time (low) ms | 3982300 | 3961920 | 3962050 | 3962140 | 3961710 |

Figure 5.2.1-1. System Time of Queue Length

| Queue Length | 1 | 3 | 10 | 100 | 200 |
| --- | --- | --- | --- | --- | --- |
| Throughput (high) | 0.458804 | 0.303794 | 0.155098 | 0.179148 | 0.201308 |
| Throughput (mid) | 0.679098 | 0.792957 | 0.391138 | 0.373885 | 0.288198 |
| Throughput (low) | 0.763393 | 0.814131 | 0.814895 | 0.814111 | 0.814164 |

Figure 5.2.1-2. Throughput of Queue Length

As shown in figure 5.2.1-1 and figure 5.2-2, it is clear that when the arrival rate is low, the queue length parameter has no significant influence of the system performance. The total system time is approximately 4 seconds and the throughput is approximately 0.81. Basically, the throughput will change with the total system time. When the total system time increases, the throughput increases and vice versa.

When the arrival rate is mid and high, the queue length is expected to have more influence of the system performance. When the queue is full, the arriving packet will be discarded so that the throughput decreases.

The larger queue length results in higher probability of staying in queue for a packet. Therefore, as the Queue length is increasing, the system time will be longer for that increasing waiting time of data packets. But when the Queue length is large enough to hold the arriving packets, the system time will tend to steady.

The system time is decreasing along with the arrival rate decreasing. The arrival rate is higher means that the stopping rule will be satisfied earlier, because the amount of the transmitted data packets will be reached earlier.

- Timeout period

The settings are as following:

- Service time of data in Q1 is 10ms per unit; ACK in Q1 is 7.5ms per unit. The service time of data in Q2 is 6.0ms per unit; ACK in Q2 is 5.6ms per unit. The service time of data in Q3 is 10ms per unit; ACK in Q3 is 7.5ms per unit.

- User specified: The inter-arrival time follows the Poisson distribution withλ. High arrival is set as 60ms, middle arrival is set as 100ms, and low arrival is set as 200ms.

- Network status: The Data Error Rate is 0.05 and the Data Lost Rate is 0.05

- Adjustable Parameters: Queue Length is 5; Timeout period is from 5000ms to 10000ms; Retrial is 5.

- The stopping rule is the amount of the transmitted packets = 20000.

The simulation output is shown as figure 5.2.1-2.

| Timeout period | 5000 | 10000 | 50000 | 100000 |
| --- | --- | --- | --- | --- |
| System Time (high) ms | 2216570 | 2324940 | 2255580 | 2077220 |
| System Time (mid) ms | 2000240 | 2021330 | 2018220 | 2026140 |
| System Time (low) ms | 3960750 | 3962160 | 3971290 | 3982490 |

Figure 5.2.1-3. System Time of Timeout period

| Timeout period | 1000 | 5000 | 10000 | 50000 | 100000 |
| --- | --- | --- | --- | --- | --- |
| Throughput (high) | 0.193473 | 0.167642 | 0.152524 | 0.176429 | 0.198128 |
| Throughput (mid) | 0.765697 | 0.661443 | 0.624949 | 0.691343 | 0.752764 |
| Throughput (low) | 0.814306 | 0.813992 | 0.814813 | 0.814632 | 0.815466 |

Figure 5.2.1-4. Throughput of Timeout period

As shown in figure 5.2.1-2, it is clear that when the arrival rate is low and middle, the Timeout period parameter has no significant influence of the system performance. The total system time in low scenario is almost constant at 4 seconds and in middle scenario is almost constant at 2 seconds. The difference of the system time between these two scenarios is because of the arrival rate.

The timeout period parameter has significant influence of the system performance only under the condition that the arrival rate is high. When the timeout period is small, the data packet tends to be discard easily. Therefore it requires more time to complete all the needed transmission. When timeout period increases, the data packets are able to stay in the queue longer so that the delay and the total system time are expected to increase.

When timeout period is large enough, the delay of data packet can be contained to the timeout period so the data won’t be discarded because of too much delay and can be sent successfully soon without retransmission. And it will reduce the system time.

- Retrial

The settings are as following:

- Service time of data in Q1 is 10ms per unit; ACK in Q1 is 7.5ms per unit. The service time of data in Q2 is 6.0ms per unit; ACK in Q2 is 5.6ms per unit. The service time of data in Q3 is 10ms per unit; ACK in Q3 is 7.5ms per unit.

- User specified: The inter-arrival time follows the Poisson distribution withλ. High arrival is set as 60ms, middle arrival is set as 100ms, and low arrival is set as 200ms.

- Network status: The Data Error Rate is 0.05 and the Data Lost Rate is 0.05

- Adjustable Parameters: Queue Length is 5; Timeout period is 10000ms; Retrial is from 1 to 5.

- The stopping rule is the amount of the transmitted packets = 20000.

The simulation output is shown as figure 5.2.1-3.

| #### Retrial | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| System Time (high) ms | 1952310 | 2107880 | 2230060 | 2258880 | 2334900 |
| System Time (mid) ms | 2065800 | 2027780 | 2010240 | 2009220 | 2005400 |
| System Time (low) ms | 4102790 | 3986260 | 3966360 | 3962660 | 3961740 |

Figure 5.2.1-5 System Time of Retrial

| Retrial | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| Throughput (high) | 0.291043 | 0.210914 | 0.178528 | 0.161115 | 0.153466 |
| Throughput (mid) | 0.800199 | 0.745391 | 0.68923 | 0.651355 | 0.645019 |
| Throughput (low) | 0.815338 | 0.81438 | 0.814212 | 0.814959 | 0.814707 |

Figure 5.2.1-6 Throughput of Retrial

As shown in figure 5.2.1-3, it is clear that when the arrival rate is low and middle, the retrial parameter has no significant influence of the system performance. The total system time in low scenario is almost constant at 4 seconds and in middle scenario is almost constant at 2 seconds. The difference of the system time between these two scenarios is because of the arrival rate. But the throughput in mid scenario is decreasing. When the retrial increases, the number of data increases but number of the transmitted data don’t increase so that the throughput decreases.

The retrial parameter has a little influence of the system performance only under the condition that the arrival rate is high. When the retrial increases, the retransmission packets increase so that the system time increases but not very obviously. Because there are so many arriving packets at high scenario, the additional retransmission packets won’t make the big difference from the origin number of arriving packets.

### Adjust Two Parameters

- Queue Length vs. Retrial

The settings are as following:

- Service time of data in Q1 is 10ms per unit; ACK in Q1 is 7.5ms per unit. The service time of data in Q2 is 6.0ms per unit; ACK in Q2 is 5.6ms per unit. The service time of data in Q3 is 10ms per unit; ACK in Q3 is 7.5ms per unit.

- User specified: The inter-arrival time follows the Poisson distribution withλ. High arrival is set as 60ms, middle arrival is set as 100ms, and low arrival is set as 200ms.

- Network status: The Data Error Rate is 0.05 and the Data Lost Rate is 0.05

- Adjustable Parameters: Queue Length is from 1 to 200; Timeout period is 10000ms; Retrial is from 1 to 5.

- The stopping rule is the amount of the transmitted packets = 20000.

  - High scenario

The simulation output is shown as figure 5.2.2-1 and figure 5.2.2-2.

| Queue Length – Retrial | 1 | 3 | 10 | 100 | 200 |
| --- | --- | --- | --- | --- | --- |
| 1 | 1428510 | 1522680 | 2277420 | 3645300 | 2895980 |
| 2 | 1353090 | 1605490 | 2361750 | 2959880 | 2286540 |
| 3 | 1311240 | 1663110 | 2423460 | 2725730 | 2241200 |
| 4 | 1289050 | 1722550 | 2479570 | 2546370 | 2208870 |
| 5 | 1274710 | 1745610 | 2570800 | 2697630 | 2176340 |

Figure 5.2.2-1 System Time of Queue Length vs. Retrial in high scenario

| Queue Length – Retrial | 1 | 3 | 10 | 100 | 200 |
| --- | --- | --- | --- | --- | --- |
| 1 | 0.651358 | 0.540809 | 0.320796 | 0.193523 | 0.233422 |
| 2 | 0.594314 | 0.395285 | 0.249391 | 0.204513 | 0.239117 |
| 3 | 0.562837 | 0.329457 | 0.220021 | 0.200876 | 0.222719 |
| 4 | 0.544165 | 0.293579 | 0.206146 | 0.218222 | 0.217831 |
| 5 | 0.532353 | 0.275898 | 0.194583 | 0.195622 | 0.215447 |

Figure 5.2.2-2 Throughput of Queue Length vs. Retrial in high scenario

When the arrival rate is high, it is clear that the queue length parameter has significant influence on the system time. As the queue length is small, the retrial almost has no significant influence on the total system time. If the queue length is small, the arriving data will be mostly exceed the queue length and discarded by the system. And also, the retransmission data packets can’t be in operation because of lots of arriving data packets in queue first.

As the queue length is increasing, the retrial starts to affect the total system time. The total system time will increase with the queue length increasing because the data packets have more space to stay at queue. But if the retrial increases, it will reduce the system time because even the packet is lost or error, the retrial will issue the retransmission packets to recovery the lost or error packet. It will cost less system time to satisfy the stopping rule.

  - Middle scenario

The simulation output is shown as figure 5.2.2-3 and figure 5.2.2-4.

| Queue Length – Retrial | 1 | 3 | 10 | 100 | 200 |
| --- | --- | --- | --- | --- | --- |
| 1 | 2191740 | 2063240 | 2275660 | 3484710 | 4489770 |
| 2 | 2091090 | 2003780 | 2273460 | 2968170 | 3322980 |
| 3 | 2049650 | 1991500 | 2281300 | 2767520 | 2840200 |
| 4 | 2029370 | 1988480 | 2240970 | 2860250 | 2695360 |
| 5 | 2020640 | 1987550 | 2255170 | 2800680 | 2685350 |

Figure 5.2.2-3 System Time of Queue Length vs. Retrial in middle scenario

| Queue Length – Retrial | 1 | 3 | 10 | 100 | 200 |
| --- | --- | --- | --- | --- | --- |
| 1 | 0.736086 | 0.806784 | 0.62697 | 0.404873 | 0.310653 |
| 2 | 0.7048 | 0.798146 | 0.480105 | 0.356369 | 0.276539 |
| 3 | 0.689293 | 0.794083 | 0.42237 | 0.347482 | 0.295698 |
| 4 | 0.683412 | 0.791714 | 0.403402 | 0.340522 | 0.299439 |
| 5 | 0.679991 | 0.790797 | 0.392851 | 0.331836 | 0.288358 |

Figure 5.2.2-4 Throughput of Queue Length vs. Retrial in middle scenario

When the arrival rate is middle, it is clear that the queue length parameter has influence on the total system time. As the queue length is small, the retrial almost has no significant influence on the total system time. If the queue length is small, the arriving data will be mostly exceed the queue length and discarded by the system. And also, the retransmission data packets can’t be in operation because of lots of arriving data packets in queue first.

As the queue length is increasing, the retrial starts to affect the total system time.  The system time increases with the queue length increasing because the data packets have more space to stay at queue. But if the retrial increases, it will reduce the system time because even the packet is lost or error, the retrial will issue the retransmission packets to recovery the lost or error packet. It will cost less system time to satisfy the stopping rule.

  - Low scenario

The simulation output is shown as figure 5.2.2-5 and figure 5.2.2-6.

| Queue Length – Retrial | 1 | 3 | 10 | 100 | 200 |
| --- | --- | --- | --- | --- | --- |
| 1 | 4214080 | 4102100 | 4101520 | 4101000 | 4102340 |
| 2 | 4054150 | 3987820 | 3988060 | 3986670 | 3986070 |
| 3 | 4005360 | 3966770 | 3966520 | 3966400 | 3966690 |
| 4 | 3988050 | 3963050 | 3963070 | 3961910 | 3962300 |
| 5 | 3983960 | 3961590 | 3961620 | 3962160 | 3962350 |

Figure 5.2.2-5 System Time of Queue Length vs. Retrial in low scenario

| Queue Length – Retrial | 1 | 3 | 10 | 100 | 200 |
| --- | --- | --- | --- | --- | --- |
| 1 | 0.780556 | 0.81392 | 0.814903 | 0.815192 | 0.813889 |
| 2 | 0.769908 | 0.814091 | 0.814931 | 0.814769 | 0.815299 |
| 3 | 0.765929 | 0.814451 | 0.814856 | 0.814884 | 0.815457 |
| 4 | 0.763645 | 0.814465 | 0.814154 | 0.8149 | 0.814369 |
| 5 | 0.763387 | 0.815357 | 0.814496 | 0.814082 | 0.814297 |

Figure 5.2.2-6 Throughput of Queue Length vs. Retrial in low scenario

When the arrival rate is low, it is clear that the queue length parameter and retrial parameter almost have no influence on the total system time. The total system time is approximately 4 seconds and the throughput is approximately 0.81.

- Queue Length vs. Timeout period

The settings are as following:

- Service time of data in Q1 is 10ms per unit; ACK in Q1 is 7.5ms per unit. The service time of data in Q2 is 6.0ms per unit; ACK in Q2 is 5.6ms per unit. The service time of data in Q3 is 10ms per unit; ACK in Q3 is 7.5ms per unit.

- User specified: The inter-arrival time follows the Poisson distribution withλ. High arrival is set as 60ms, middle arrival is set as 100ms, and low arrival is set as 200ms.

- Network status: The Data Error Rate is 0.05 and the Data Lost Rate is 0.05

- Adjustable Parameters: Queue Length is from 1 to 200; Timeout period is from 5000ms to 100000ms; Retrial is 5.

- The stopping rule is the amount of the transmitted packets = 20000.

  - High scenario

The simulation output is shown as figure 5.2.2-7 and figure 5.2.2-8.

| Queue Length —Timeout period | 1 | 3 | 10 | 100 | 200 |
| --- | --- | --- | --- | --- | --- |
| 5000 | 1271350 | 1719600 | 2292370 | 2753280 | 5304990 |
| 10000 | 1274890 | 1732680 | 2445410 | 2591120 | 2233470 |
| 50000 | 1300620 | 1668100 | 2602070 | 1553910 | 1240950 |
| 100000 | 1328660 | 1606290 | 2381700 | 1564930 | 1277020 |

Figure 5.2.2-7 System Time of Queue Length vs. Timeout period in high scenario

| Queue Length —Timeout period | 1 | 3 | 10 | 100 | 200 |
| --- | --- | --- | --- | --- | --- |
| 5000 | 0.531528 | 0.279668 | 0.216322 | 0.151817 | 0.082992 |
| 10000 | 0.53272 | 0.279205 | 0.203559 | 0.204475 | 0.207517 |
| 50000 | 0.548457 | 0.312942 | 0.196059 | 0.422769 | 0.668411 |
| 100000 | 0.568602 | 0.360995 | 0.219532 | 0.429013 | 0.649771 |

Figure 5.2.2-8 Throughput of Queue Length vs. Timeout period in high scenario

When the arrival rate is high, it is clear that the queue length has significant influence on the total system time. As the queue length is small, the timeout period almost has no significant influence on the total system time. If the queue length is small, the arriving data will be mostly exceed the queue length and discarded by the system. Hence, no matter how the timeout period changes, the data packets will be discarded.

As the queue length increases, the timeout period starts to affect the total system time. When the timeout period is small and the queue length is large, the total system time will increase fast. Because the timeout period is small, the packets are easy to over the timeout period and to be discarded from the system. It requires lots of time to retransmit the packets or wait the new arrival packets. Therefore the timeout period will reduce the system time, as the queue length is large.

  - Middle scenario

The simulation output is shown as figure 5.2.2-9 and figure 5.2.2-10.

| Queue Length —Timeout period | 1 | 3 | 10 | 100 | 200 |
| --- | --- | --- | --- | --- | --- |
| 5000 | 2017590 | 1985970 | 2183740 | 3216700 | 6083860 |
| 10000 | 2019520 | 1986690 | 2255230 | 2797990 | 2676230 |
| 50000 | 2036960 | 1998570 | 2291590 | 2010060 | 2018440 |
| 100000 | 2055890 | 2009850 | 2201640 | 2037620 | 2028900 |

Figure 5.2.2-9 System Time of Queue Length vs. Timeout period in middle scenario

| Queue Length —Timeout period | 1 | 3 | 10 | 100 | 200 |
| --- | --- | --- | --- | --- | --- |
| 5000 | 0.679051 | 0.792041 | 0.404946 | 0.194494 | 0.097457 |
| 10000 | 0.679127 | 0.791543 | 0.388276 | 0.337429 | 0.283988 |
| 50000 | 0.684053 | 0.792781 | 0.41677 | 0.706242 | 0.813699 |
| 100000 | 0.689732 | 0.797532 | 0.501873 | 0.730461 | 0.813787 |

Figure 5.2.2-10 Throughput of Queue Length vs. Timeout period in middle scenario

When the arrival rate is middle, it is clear that the queue length has significant influence on the total system time. As the queue length is small, the timeout period almost has no significant influence on the total system time. If the queue length is small, the arriving data will be mostly exceed the queue length and discarded by the system. Hence, no matter how the timeout period changes, the data packets will be discarded.

As the queue length increases, the timeout period starts to affect the total system time. When the timeout period is small and the queue length is large, the total system time will be increasing fast. Because the timeout period is small, the packets are easy to over the set timeout period and discarded from the system. It requires lots of time to retransmit the packets or waits the new arrival packets. Therefore the timeout period will reduce the total system time, as the queue length is large.

  - Low scenario

The simulation output is shown as figure 5.2.2-11 and figure 5.2.2-12.

| Queue Length —Timeout period | 1 | 3 | 10 | 100 | 200 |
| --- | --- | --- | --- | --- | --- |
| 5000 | 3980500 | 3961280 | 3960400 | 3960510 | 3960740 |
| 10000 | 3983030 | 3961870 | 3961880 | 3961790 | 3961620 |
| 50000 | 3994470 | 3971030 | 3970910 | 3971060 | 3971080 |
| 100000 | 4010500 | 3981960 | 3981570 | 3982000 | 3982310 |

Figure 5.2.2-11 System Time of Queue Length vs. Timeout period in low scenario

| Queue Length —Timeout period | 1 | 3 | 10 | 100 | 200 |
| --- | --- | --- | --- | --- | --- |
| 5000 | 0.76356 | 0.814525 | 0.814268 | 0.814791 | 0.81488 |
| 10000 | 0.762414 | 0.813977 | 0.814519 | 0.814742 | 0.814881 |
| 50000 | 0.761279 | 0.813987 | 0.814966 | 0.814133 | 0.814122 |
| 100000 | 0.761695 | 0.814164 | 0.814761 | 0.814239 | 0.814654 |

Figure 5.2.2-12 Throughput of Queue Length vs. Timeout period in low scenario

When the arrival rate is low, it is clear that the queue length parameter and timeout period parameter almost have no influence on the system time. The system time is approximately 4 seconds and the throughput is approximately 0.81.

- Retrial vs. Timeout period

The settings are as following:

- Service time of data in Q1 is 10ms per unit; ACK in Q1 is 7.5ms per unit. The service time of data in Q2 is 6.0ms per unit; ACK in Q2 is 5.6ms per unit. The service time of data in Q3 is 10ms per unit; ACK in Q3 is 7.5ms per unit.

- User specified: The inter-arrival time follows the Poisson distribution withλ. High arrival is set as 60ms, middle arrival is set as 100ms, and low arrival is set as 200ms.

- Network status: The Data Error Rate is 0.05 and the Data Lost Rate is 0.05

- Adjustable Parameters: Queue Length is from 3; Timeout period is from 5000ms to 10000ms; Retrial is from 1 to 5.

- The stopping rule is the amount of the transmitted packets = 20000.

  - High scenario

The simulation output is shown as figure 5.2.2-13 and figure 5.2.2-14.

| Retrial— ##### Timeout period | 5000 | 10000 | 50000 | 100000 |
| --- | --- | --- | --- | --- |
| 1 | 1530330 | 1521150 | 1514750 | 1515260 |
| 2 | 1600420 | 1613450 | 1575300 | 1568590 |
| 3 | 1655190 | 1682220 | 1634140 | 1589510 |
| 4 | 1705080 | 1703730 | 1665250 | 1592340 |
| 5 | 1725600 | 1727480 | 1663660 | 1598970 |

Figure 5.2.2-13 System Time of Retrial vs. Timeout period in high scenario

| Retrial— ##### Timeout period | 5000 | 10000 | 50000 | 100000 |
| --- | --- | --- | --- | --- |
| 1 | 0.168842 | 0.211277 | 0.499009 | 0.497975 |
| 2 | 0.171603 | 0.22301 | 0.459815 | 0.455165 |
| 3 | 0.162693 | 0.206559 | 0.437976 | 0.440182 |
| 4 | 0.157256 | 0.21115 | 0.436551 | 0.435223 |
| 5 | 0.157444 | 0.191157 | 0.42246 | 0.430636 |

Figure 5.2.2-14 Throughput of Retrial vs. Timeout period in high scenario

When the arrival rate is high, it is clear that the timeout period parameter and retrial parameter almost have no significant influence on the total system time. The system time is almost constant at 1.5 seconds. But the throughput increases, when the timeout period parameter increases. When the timeout period increases, the data packet can stay in queue longer so that the data packet won’t be discarded so easily.

  - Middle scenario

The simulation output is shown as figure 5.2.2-15 and figure 5.2.2-16.

| Retrial— Timeout period | 5000 | 10000 | 50000 | 100000 |
| --- | --- | --- | --- | --- |
| 1 | 2063330 | 2064940 | 2070070 | 2077690 |
| 2 | 2003730 | 2004610 | 2013280 | 2024180 |
| 3 | 1990160 | 1991230 | 2000720 | 2013310 |
| 4 | 1987710 | 1988460 | 1997550 | 2010210 |
| 5 | 1985440 | 1988300 | 1997810 | 2010390 |

Figure 5.2.2-15 System Time of Retrial vs. Timeout period in middle scenario

| Retrial— Timeout period | 5000 | 10000 | 50000 | 100000 |
| --- | --- | --- | --- | --- |
| 1 | 0.170209 | 0.387793 | 0.764952 | 0.767987 |
| 2 | 0.181207 | 0.367309 | 0.733566 | 0.72091 |
| 3 | 0.197224 | 0.356949 | 0.710207 | 0.72464 |
| 4 | 0.195165 | 0.362588 | 0.726444 | 0.72747 |
| 5 | 0.1906 | 0.352553 | 0.719124 | 0.724365 |

Figure 5.2.2-16 Throughput of Retrial vs. Timeout period in middle scenario

When the arrival rate is middle, it is clear that the timeout period parameter and retrial parameter almost have no significant influence on the total system time. The system time is almost constant at 2 seconds. But the throughput increases, when the timeout period parameter increases. When the timeout period increases, the data packet can stay in queue longer so that the data packet won’t be discarded so easily.

  - Low scenario

The simulation output is shown as figure 5.2.2-17 and figure 5.2.2-18.

| Retrial— Timeout period | 5000 | 10000 | 50000 | 100000 |
| --- | --- | --- | --- | --- |
| 1 | 4101330 | 4101420 | 4107100 | 4116670 |
| 2 | 3986470 | 3986950 | 3995130 | 4005760 |
| 3 | 3965300 | 3966040 | 3975410 | 3987340 |
| 4 | 3961510 | 3962380 | 3971290 | 3982440 |
| 5 | 3961490 | 3961950 | 3970560 | 3982720 |

Figure 5.2.2-17 System Time of Retrial vs. Timeout period in low scenario

| Retrial— Timeout period | 5000 | 10000 | 50000 | 100000 |
| --- | --- | --- | --- | --- |
| 1 | 0.814367 | 0.814567 | 0.81473 | 0.814463 |
| 2 | 0.814493 | 0.813823 | 0.814078 | 0.814431 |
| 3 | 0.814373 | 0.814558 | 0.81365 | 0.814771 |
| 4 | 0.815053 | 0.815269 | 0.815381 | 0.814858 |
| 5 | 0.814597 | 0.815025 | 0.814858 | 0.814826 |

Figure 5.2.2-18 Throughput of Retrial vs. Timeout period in low scenario

When the arrival rate is low, we can know that the timeout period parameter and retrial parameter almost have no significant influence on the system time. The system time is approximately 4 seconds and the throughput is approximately 0.81.

## Summary

In the beginning, we use two indices, amount of transmitted data packet and inter-arrival time to verify our data transmission model. And based on the assumption we made, the outputs of the simulation model are all reasonable and logical reasoning.

Finally, we want to conclude the value of adjustable parameters, queue length, timeout period and retrial, so that we give three scenarios to help us. We use inter-arrival time as the criterion for high, middle and low three scenarios. And first section we adjust only one parameter and fix the other two; the second section we adjust two parameters at the same time.

By this way, we conclude that the key parameters in each scenario and try to conclude the adaptive value of these parameters.

When the arrival rate is low, all these parameters almost have no influence on the system time because there is less probability to occur the congestion.

The queue length is playing a key parameter all the time except in the low arrival rate. It has great influence on the system time. And when it will also affect the other parameters and make them increase the influence on the system time. The result of the adjustable parameters is shown as figure 5.3-1.

“+” means effective, “-” means ineffective, “↑” means effective and up tendency, and ”↓” means effective and down tendency.

|  | Queue Length | Timeout period | Retrial |  |
| --- | --- | --- | --- | --- |
| High | (+, +) 69% | (↑, ↑) Or (↓, ↓) | (↑, -) | Queue Length |
|  | (↑, ↑) Or (↓, ↓) | (+, +) 6% | (-, -) | Timeout period |
|  | (↓, -) | (-, -) | (+, +) 19.6% | Retrial |
| Middle | (+, +) 30% | (↑, ↑) Or (↓, -) | (-, ↓) *Queue length =3 | Queue Length |
|  | (↑, ↑) Or (↓, -) | (-, -) 1% | (-, -) | Timeout period |
|  | (↓, -) *Queue length =3 | (-, -) | (-, -) 2.9% | Retrial |
| Low | (-, -) 0.5% | (-, -) | (-, -) | Queue Length |
|  | (-, -) | (-, -) 0.5% | (-, -) | Timeout period |
|  | (-, -) | (-, -) | (-, -) 3.4% | Retrial |

Figure 5.3-1 Result of the adjustable parameters

# Chapter6 Conclusion and Suggestion

## 6.1 Conclusion

In this research, we have build up a simulation model for the data packet transmitted among the network. And we use the numerical experiments to verify the accuracy and the rationality about our transmission model We choose the inter-arrival time and the amount of arrived data packets, which are given by the user not the parameters we try to adjust, as two indices to help us to know the model outcomes and rely on these outcomes to determine the model accuracy and rationality.

Second, we conclude the value of adjustable parameters, queue length, timeout period and retrial three key parameters in three scenarios, which are high arrival rate, middle arrival rate and low arrival rate.

High arrival rate is defined as inter-arrival time is 60ms, that is the system will be close to saturation as 46/60 = 0.767. Middle arrival rate is defined as inter-arrival time is 100ms, that is the system will be half to saturation as 46/100 = 0.46. Low arrival rate is defined as inter-arrival time is 200ms, that is the system will be less to saturation as 46/200 = 0.23.

And we have considered two phases, adjusting only one variable and adjusting two parameters at the same time, concluded the key parameters in three scenarios. The result is shown as figure 6.1-1.

|  | Queue Length | Timeout period | Retrial |  |
| --- | --- | --- | --- | --- |
| High | (+, +) | (↑, ↑) Or (↓, ↓) | (↑, ↓) | Queue Length |
|  | (↑, ↑) Or (↓, ↓) | (+, +) | (-, -) | Timeout period |
|  | (↓, ↑) | (-, -) | (+, +) | Retrial |
| Middle | (+, +) | (↑, ↑) Or (↓, -) | (↑, ↓) *Queue length =3 | Queue Length |
|  | (↑, ↑) Or (↓, -) | (-, -) | (-, -) | Timeout period |
|  | (↓, ↑) *Queue length =3 | (-, -) | (-, -) | Retrial |
| Low | (-, -) | (-, -) | (-, -) | Queue Length |
|  | (-, -) | (-, -) | (-, -) | Timeout period |
|  | (-, -) | (-, -) | (-, -) | Retrial |

Figure 6.1-1 Result of the adjustable parameters

## 6.2 Suggestion

In this research, we assume that all the process servers are represented as different protocol procedures and have the same characteristics. The only one difference is the different service rate according to the type of data, data or ACK. Basically, different protocol procedures will have different service rate according the coding, packet headers. It needs to add the protocol characteristics into our process server in the advanced study.

In the simulation model of this research, all the distributions refer to the previous papers. We assume the arrival follows the Poisson distribution withλ given by user and the service rate follows the Exponential distribution with θreferred to the previous studies. In consideration of the completeness, it needs to distribution calibration for these assumptions in the advanced study.

The final results in this research is using the numerical experiments to conclude the value of adjustable parameters, queue length, timeout period and retrial and the key parameters in three scenarios, which are high arrival rate, middle arrival rate and low arrival rate. It can be considered with the stochastic optimization model to obtain the exact optimal value of these parameters in the advanced study.

# References

- American Association of State Highway and Transportation Officials (AASHTO), Institute of Transportation Engineers (ITE), National Electrical Manufacturers Association (NEMA), “The NTCIP Guide, NTCIP 9001 v02.06”, (1999).

- Raymond M. Bryant, Anthony E. Krzesinski, M. Seetha Lakshmi, K. Mani Chandy, “The MVA Priority Approximation”, ACM Transaction on Computer Systems, Vol. 2, NO. 4, pp335-359 (Nov., 1984).

- Hiroshi Inai, Takeshi Nishida, Tokumi Yokohira, Hideo Miyahara, “End-to-End Performance Modeling for Layered communication Protocol”, INFOCOM '90, Ninth Annual Joint Conference of the IEEE Computer and Communication Societies. The Multiple Facets of Integration. Proceedings, IEEE, pp442-449 (1990).

- Yamamoto, M.; Akiyoshi, I.; Nakanishi, H.; Sanada, H.; Tezuka, Y., ”Performance of window flow control scheme for interconnected packet networks”, Communications, 1988. ICC '88. Digital Technology - Spanning the Universe. Conference Record., IEEE International Conference on , vol.2, pp1162-1166 (1988).

- Apostolopoulos, T.K.; Daskalou, V.C. “A model for SNMP based performance management services”, Networks, 1995. Theme: Electrotechnology 2000: Communications and Networks. International Conference on Information Engineering., Proceedings of IEEE Singapore International Conference on , pp269-273, (1995 ).

- Hiroshi Inai, Tokumi Yokohira, Masayuki Murata, Hideo Miyahara, “Performance Evaluation of Layered Communication Protocol”, Electronic and Communications in Japan, part1, Vol. 74, No. 4, pp13-24 (1991).

- Jaime Jungok Bae, Tatsuya Suda, Naoya Watanabe, “Evaluation of the Effects of Protocol Processing Overhead in Error Recovery Schemes for a High-Speed Packet Switched Network: Link-by-Link versus Edge-to-Edge Schemes”, IEEE Journal on selected areas in communications, Vol. 9, No. 9, pp1496-1509 (Dec., 1991).

- Ihsan Khan, Graham Knight, “SNMP and OSI Management Information Modeling and Translation: A case study”, INFOCOM '96. Fifteenth Annual Joint Conference of the IEEE Computer Societies. Networking the Next Generation., Proceedings IEEE , pp481 -488 (1996).

- X. Du, M. Shayman, M. Rozenblit, “Implementation and Performance Analysis of SNMP on a TLS/TCP Base”, Integrated Network Management Proceedings, 2001 IEEE/IFIP International Symposium on, pp453-466 (2001).

- S. Amarnath, Anurag Kumar, “A New Technique for Link Utilization Estimation in Packet Data Networks using SNMP Variables”, Global Telecommunications Conference, 1997. GLOBECOM '97., IEEE, Vol. 1, pp212-216 (1997).

- Therdore K. Apostolopoulos, Victoria C. Daskalou, “A Model for SNMP Based Performance Management Services”, IEEE Catalogue No. 95TH8061, pp269-273 (1995).