附錄B.1

FAR Part 91  Operating And Flight Rules  ---  Subpart I--Operating Noise Limits

美國聯邦飛航規則第九十一號  操作與飛行規則  ----  I部份  營運之噪音限制

| 91.801 Applicability: Relation to part 36. | 91.801條 I部份之適用性與有關第36號規則之事項 |
| --- | --- |
| 91.803 Part 125 operators: Designation of applicable regulations. | 91.803條 第125號規則之營運人與其適用之規定 |
| 91.805 Final compliance: Subsonic airplanes. | 91.805條 達成次音速飛機噪音規定之最後期限 |
| 91.807 Phased compliance under parts 121, 125, and 135: Subsonic airplanes. | 91.807條 第121,125及135號規則次音速飛機之逐步達成規定 |

在此僅列出下數條文，其餘條文內容請參見FAA之網站資料。

| Sec. 91.801 Applicability: Relation to Part 36. |  |  | 第91.801條 I部份之適用性與有關第36號規則之事項 |
| --- | --- | --- | --- |
| (a) This subpart prescribes operating noise limits and related requirements that apply, as follows, to the operation of civil aircraft in the United States. |  |  | (a) I部份（以下稱為本部份）所規定者為營運時之噪音限制﹐及美國境內營運之民用航空器之相關規定。 |
| (1) Sections 91.803, 91.805, 91.807, 91.809, and 91.811 apply to civil subsonic turbojet airplanes with maximum weights of more than 75,000 pounds and-- |  |  | (1)第91.803, 91.805, 91.807, 91.809與91.811條之規定為適用於民用次音速噴射機﹐其最大重量超過七萬五千磅者﹐且 |
| (i) If U.S. registered, that have standard airworthiness certificates; or |  |  | (i) 登記於美國﹐並具有標準適航證明者﹔或 |
| (ii) If foreign registered, that would be required by this chapter to have a U.S. standard airworthiness certificate in order to conduct the operations intended for the airplane were it registered in the United States. Those sections apply to operations to or from airports in the United States under this part and parts 121, 125, 129, and 135 of this chapter. |  |  | (ii)登記於外國﹐假設其登記於美國時﹐依本章規定仍需具有美國標準適航證明﹐用以經營航空器者。 上述四條規定仍適用於符合本章本規則與第121, 125, 129及135號規則﹐營運航空器於美國之機場起降者。 |
| (2) Section 91.813 applies to U.S. operators of civil subsonic turbojet airplanes covered by this subpart. This section applies to operators operating to or from airports in the United States under this part and parts 121, 125, and 135, but not to those operating under part 129 of this chapter. |  |  | (2)第91.813條之規定適用於本部份規定之民用次音速噴射機之美國營運人﹔該條規定適用於營運人符合本章本規則與第121, 125及135號規則﹐營運航空器於美國之機場起降者﹐但不適用於第129號規則規定之營運人。 |
| (3) Sections 91.803, 91.819, and 91.821 apply to U.S.-registered civil supersonic airplanes having standard airworthiness certificates and to foreign-registered civil supersonic airplanes that, if registered in the United States, would be required by this chapter to have U.S. standard airworthiness certificates in order to conduct the operations intended for the airplane. Those sections apply to operations under this part and under parts 121, 125, 129, and 135 of this chapter. |  |  | (3)第91.803, 91.819及91.821條之規定適用於登記於美國﹐具有標準適航證明之民用超音速飛機﹔與登記於國外之民用超音速飛機﹐假設其登記於美國時﹐依本章規定仍須具有美國標準適航證明﹐用以經營航空器者。 上述三條規定適用於符合本章本規則與第121, 125, 129及135號規則規定之營運人。 |
| (b) Unless otherwise specified, as used in this subpart "part 36" refers to 14 CFR part 36, including the noise levels under appendix C of that part, notwithstanding the provisions of that part excepting certain airplanes from the specified noise requirements. For purposes of this subpart, the various stages of noise levels, the terms used to describe airplanes with respect to those levels, and the terms "subsonic airplane" and "supersonic airplane" have the meanings specified under part 36 of this chapter. For purposes of this subpart, for subsonic airplanes operated in foreign air commerce in the United States, the Administrator may accept compliance with the noise requirements under annex 16 of the International Civil Aviation Organization when those requirements have been shown to be substantially compatible with, and achieve results equivalent to those achievable under, part 36 for that airplane. Determinations made under these provisions are subject to the limitations of Sec. 36.5 of this chapter as if those noise levels were part 36 noise levels. |  |  | (b)除非其他說明﹐第36號規則在此表 ” 14 CFR part 36 ”﹐並包括其附錄C噪音水準之規定﹐雖然該規則因特定噪音規定﹐而排除一部份之飛機。本部份所提及不同等級之噪音水準﹑次音速或超音速飛機等專用語皆依第36號規則之定義。 於美國從事國外航空商務之次音速飛機﹐FAA局長於以下情況得接受其符合「國際民航組織（ICAO）」第16號附約之噪音規定：第十六號附約之航空器噪音規定與第36號規則實質上相容﹐且達成之結果相同。 上述規定中所提及之噪音水準﹐若為第36號規則之噪音水準﹐則於決議其符合與否﹐應受限於第36號規則第36.5條之規定。 |
| (c) Sections 91.851 through 9l.875 of this subpart prescribe operating noise limits and related requirements that apply to any civil subsonic turbojet airplane with a maximum certificated weight of more than 75,000 pounds operating to or from an airport in the 48 contiguous United States and the District of Columbia under this part, part 121, 125, 129, or 135 of this chapter on and after September 25, 1991. |  |  | (c)第91.851條至第91.875條為規定最大檢定重量超過七萬五千磅之民用次音速噴射機﹐於1991年9月25日以後﹐符合本規則與第121,125,129及135號規則規定﹐營運於美國48個相鄰的州以及哥倫比亞特區等之機場者﹐其營運時之噪音限制及其相關規定。 |

| Sec. 91.803 Part 125 operators: Designation of applicable regulations. |  |  | 第91.803條 第125號規則之營運人與其適用之規定 |
| --- | --- | --- | --- |
| For airplanes covered by this subpart and operated under part 125 of this chapter, the following regulations apply as specified: |  |  | 本部份所規定之飛機並符合第125號規則之營運者﹐適用於下列指定之規則： |
| (a) For each airplane operation to which requirements prescribed under this subpart applied before November 29, 1980, those requirements of this subpart continue to apply. |  |  | (a)每一飛機於1980年11月29日以前適用本部份相關規定之營運﹐這些規定繼續適用之。 |
| (b) For each subsonic airplane operation to which requirements prescribed under this subpart did not apply before November 29, 1980, because the airplane was not operated in the United States under this part or part 121, 129, or 135 of this chapter, the requirements prescribed under Secs. 91.805, 91.809, 91.811, and 91.813 of this subpart apply. |  |  | (b) 每一次音速飛機於1980年11月29日以前不適用本部份相關規定之營運﹐因飛機依本號規則或第121, 129, 135號等規則不在美國境內營運﹐則適用於本部份第91.805, 91.809, 91.811與 91.813 條規定。 |
| (c) For each supersonic airplane operation to which requirements prescribed under this subpart did not apply before November 29, 1980, because the airplane was not operated in the United States under this part or part 121, 129, or 135 of this chapter, the requirements of Secs. 91.819 and 91.821 of this subpart apply. |  |  | (c) 每一超音速飛機於1980年11月29日以前不適用本部份相關規定之營運﹐因飛機依本號規則或第121, 129, 135號等規則不在美國境內營運﹐則適用於本部份第91.819與 91.821 條規定。 |
| (d) For each airplane required to operate under part 125 for which a deviation under that part is approved to operate, in whole or in part, under this part or part 121, 129, or 135 of this chapter, notwithstanding the approval, the requirements prescribed under paragraphs (a), (b), and (c) of this section continue to apply. |  |  | (d) 每一飛機依第125號規則要求﹐雖有不符規定處但仍依本號規則或第121, 129, 135號等規則被（部份或全部）核准而營運時﹐縱使核准﹐但仍須遵守本條(a),(b)(c)項之規定。 |

| Sec. 91.805 Final compliance: Subsonic airplanes. |  |  | 第91.805條 達成次音速飛機噪音規定之最後期限 |
| --- | --- | --- | --- |
| Except as provided in Secs. 91.809 and 91.811, on and after January 1, 1985, no person may operate to or from an airport in the United States any subsonic airplane covered by this subpart unless that airplane has been shown to comply with Stage 2 or Stage 3 noise levels under part 36 of this chapter. |  |  | 除第91.809條與第91.811條規定者外﹐自1985年1月1日以後﹐於美國境內機場營運之任何本部份所涵蓋之次音速飛機﹐皆需符合第36號規則規定之第二﹑第三噪音等級之噪音水準。 |

| Sec. 91.807 Phased compliance under Parts 121, 125, and 135: Subsonic airplanes. |  |  | 第91.807條 第121,125及135號規則次音速飛機之逐步達成規定 |
| --- | --- | --- | --- |
| (a) General. Each person operating airplanes under part 121, 125, or 135 of this chapter, as prescribed under Sec. 91.803 of this subpart, regardless of the state of registry of the airplane, shall comply with this section with respect to subsonic airplanes covered by this subpart. |  |  | (a) 一般。依第121, 125或135號規則營運飛機之任何人﹐依本部份第91.803條規定﹐無視飛機之註冊州別﹐應遵守本條規則有關本部份所涵蓋次音速飛機之規定。 |
| (b) Compliance schedules. Except for airplanes shown to be operated in foreign air commerce under paragraph (c) of this section or covered by an exemption (including those issued under Sec. 91.811), airplanes operated by U.S. operators in air commerce in the United States must be shown to comply with Stage 2 or Stage 3 noise levels under part 36 of this chapter, in accordance with the following schedule, or they may not be operated to or from airports in the United States: |  |  | (b) 達成規定之時間表。除本條文(c)項之從事國外航空商務者﹐或為免除條文所涵蓋者（含第91.811條）之航空器外﹐航空器為美國籍營運人於美國境內從事空中商務者﹐皆需符合第36號規則規定之第二﹑第三噪音等級之噪音水準﹐並依照下列時間表﹐否則不得營運於美國境內： |
| (1) By January 1, 1981-- |  |  | (1)於1981年1月1日 -- |
| (i) At least one quarter of the airplanes that have four engines with no bypass ratio or with a bypass ratio less than two; and |  |  | (i)至少四分之一的飛機具有四引擎且無旁通比或旁通比小於2﹐且 |
| (ii) At least half of the airplanes powered by engines with any other bypass ratio or by another number of engines. |  |  | (ii)至少二分之一的飛機為引擎動力者﹐不限旁通比或其引擎數目。 |
| (2) By January 1, 1983-- |  |  | (2)於1983年1月1日 -- |
| (i) At least one-half of the airplanes that have four engines with no bypass ratio or with a bypass ratio less than two; and |  |  | (i) 至少二分之一的飛機具有四引擎且無旁通比或旁通比小於2﹐且 |
| (ii) All airplanes powered by engines with any other bypass ratio or by another number of engines. |  |  | (ii) 所有飛機為引擎動力者﹐不限旁通比或其引擎數目。 |
| (c) Apportionment of airplanes. For purposes of paragraph (b) of this section, a person operating airplanes engaged in domestic and foreign air commerce in the United States may elect not to comply with the phased schedule with respect to that portion of the airplanes operated by that person shown, under an approved method of apportionment, to be engaged in foreign air commerce in the United States. |  |  | (c) 飛機數之配額。依本條文(b)項規定﹐任何人在美國境內從事國內外航空商務者﹐於該人士從事有關國外航空商務之飛機數部份﹐在一許可分配方式下﹐該部份得選擇不遵守逐步時間表。 |

「本章」表聯邦規則彙編第十四篇（14 CFR）中第1至第199號之規則彙整於一章中﹐該章即通稱之聯邦飛航規則（FAR）﹐內容為FAA規定之行政規則。

聯邦飛航規則第121號為 “Certification and Operations: Domestic, Flag, and Supplemental Air Carriers and Commercial Operators of Large Aircraft”

第125號為 “Certification and Operations: Airplanes Having A Seating Capacity of 20 or More Passengers or A Maximum Payload Capacity of 6,000 Pounds or More”

第129號規則為 “Operations: Foreign Air Carriers and Foreign Operators of U.S.- Registered Aircraft Engaged in Common Carriage”

第135號規則為 “Air Taxi Operators and Commercial Operators”