附錄B.2

FAR Part 150 -- Airport Noise Compatibility Planning

第一五０號聯邦飛航規則—機場噪音相容性規畫

Subpart A--General Provisions

A部份 – 總則

150.1 Scope and purpose.

150.1條 範圍與目的

150.3 Ap

| plicability. 150.3條 適用性 150.5 Limitations of this part. 150.5條 本號規則之限制 150.7 Definitions. 150.7條 定義 150.9 Designation of noise systems. 150.9條 噪音系統之指定 150.11 Identification of land uses. 150.11條 土地使用之確認 150.13 Incorporations by reference. 150.13條 其他供參照之規定 Subpart B--Development of Noise Exposure Maps and Noise Compatibility Programs B部份 -- 發展噪音曝露量地圖與噪音相容性計畫 150.21 Noise exposure maps and related descriptions. 150.21條 噪音曝露量地圖與相關描述 150.23 Noise compatibility programs. | 150.23條 噪音相容性計畫 Subpart C--Evaluations and Determinations of Effects of Noise Compatibility Programs C部份 – 噪音相容性計畫之評估與決定 150.31 Preliminary review; acknowledgments. 150.31條 初步檢查與公證 150.33 Evaluation of programs. |  |
| --- | --- | --- |
| 附件B—噪音相容性計畫150.33條 評估計畫 Subpart A--General Provisions A部份 – 總則 Sec. 150.1 Scope and purpose. 第150.1條 範圍與目的 This part prescribes the procedures, standards, and methodology governing the development, submission, and review of airport noise exposure maps and airport noise compatibility programs, including the process for evaluating and approving or disapproving those programs. It prescribes single systems for-- (a) 本號規則所規定之程序﹑標準以及方法主要用來指導噪音曝露量地圖與噪音相容性計畫的相關發展﹑提交仲裁與檢查，其中包含評估及審核這些計畫的程序。所規定的單一系統包含： (a) measuring noise at airports and surrounding areas that generally provides a highly reliable relationship between projected noise exposure and surveyed reaction of people to noise; and (a) 提供量測機場及其周圍噪音的方法，該方法介於量得之噪音曝露量與民調之噪音反應間之關係具有高度可信賴的關係﹔與 (b) determining exposure of individuals to noise that results from the operations of an airport. (b) 決定各群體所受之機場噪音曝露量。 This part also identifies those land uses which are normally compatible with various levels of exposure to noise by individuals. It provides technical assistance to airport operators, in conjunction with other local, State, 150.35 Determinations; publications; effectivity. 150.35條 決議﹑公佈與生效 Appendix 附件 Appendix A to Part 150 -- Noise Exposure Maps 附件A—噪音曝露量地圖 Appendix B to Part 150 -- Noise Compatibility Programs |  |  |

and Federal au

| thorities, to prepare and execute appropriate noise compatibility planning and implementation programs. 本號規則亦提供各個群體分別對噪音的正常相容性水準。是以本號規則提供對機場經營者及關連之其他當地、州與聯邦當局的技術支援，以準備及進行適當的噪音相容性規畫，與推行該計畫。 Sec. 150.3 Applicability. 第150.3條 適用性 This part applies to the airport noise compatibility planning activities of the operators of "public use airports," including heliports, as that term is used in section 101(1) of the ASNA Act as amended (49 U.S.C. 2101) and as defined in section 503(17) of the Airport and Airway Improvement Act of 1982 (49 U.S.C. 2202). 本號規則適用於”公用機場”之營運者於規畫機場噪音相容性活動時所使用，並含直昇機機場；”公用機場”適用範圍與ASNA法案第101條第1項（即依聯邦法典第49篇第2101條）之用法相同，其定義詳見1982年Airport and Airway Improvement法案第503條第17項（依聯邦法典第49篇第2202條）之定義。 Sec. 150.5 Limitations of this part. 第150.5條 本號規則之限制 (a)Pursuant to the ASNA Act (49 U.S.C. 2101 et seq.), this part provides for airport noise compatibility planning and land use programs necessary to the purposes of those provisions. No submittal of a map, or approval or disapproval, in whole or part, of any map or program submitted under this part is a determination concerning the acceptability or unacceptability of that land use under Federal, State, or local law. (a) 依ASNA Act (49 U.S.C. 2101 et seq.)之必要目的，本號規則提出機場相容性規畫與土地使用方案；依本號規則所提出之噪音曝露量地圖(無論於任何相關計畫或地圖中，全部或部份的核准或不核准)，皆不代表該項決議在聯邦、各州或地方有關的土地使用法令下一樣地承認接受（或否定不接受）。 (b)Approval of a noise compatibility program under this part is neither a commitment by the FAA to financially assist in the implementation of the program, nor a determination that all measures covered by thible ( ? triable ?) for grant-in-aid funding from the FAA. (b) 本號規則所核准之噪音相容性計畫，既不表示FAA承諾履行資助該計畫，亦不表示為審理FAA補助基金申請案之決議。 (c)Approval of a noise compatibility program under this part does not by itself constitute an FAA implementing action. A request for Federal action or approval to implement specific noise compatibility measures may be required, and an FAA decision on the request may require an environmental assessment of the proposed action, pursuant to the National Environmental Policy Act (42 U.S.C. 4321 et seq.) and applicable regulations, directives, and guidelines. (c) 本號規則所核准之噪音相容性計畫，並不構成FAA需執行之決議。FAA可能被要求批准執行特定噪音相容性措施或申請聯邦決議，在此情況下，FAA可要求對方提出提議方案之環境評估(EA)(依NEPA 1969[聯邦法典第42篇第4321條]及適用之法規、指令以及指導方針之規範)。 (d)Acceptance of a noise exposure map does not constitute an FAA determination that any specific parcel of land lies within a particular noise contour. Responsibility for interpretation of the effects of noise contours upon subjacent land uses, including the relationship between noise contours and spe |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

ci

| fic properties, rests with the sponsor or with other state or local government. (d) 接受噪音曝露量地圖並不構成FAA對某區域土地位於特定之等噪音線內之決定；而機場營運者、州或當地政府有義務解釋等噪音線鄰近土地使用之影響(含特定等噪音線與相對特性之解釋)。 Sec. 150.7 Definitions. 第150.7條 定義 As used in this part, unless the context requires otherwise, the following terms have the following meanings. 本號規則文中所使用的專用術語定義如下： "Airport" means any public use airport, including heliports, as defined by the ASNA Act, including: (a) Any airport which is used or to be used for public purposes, under the control of a public agency, the landing area of which is publicly owned; (b) any privately owned reliever airport; and (c) any privately owned airport which is determined by the Secretary to enplane annually 2,500 or more passengers and receive scheduled passenger service of aircraft, which is used or to be used for public purposes. “機場”表公用機場，含直昇機場，如ASNA 1979所定義，為(a)任何機場其用途或被用作為公共目的，為政府機關所控制，機場降落區為公有地；(b)救援用途之私有機場；與(c)私有機場，每年承載2500位旅客以上並接受定期客機起降，且其用途或被用作為公共目的者，由運輸部長決定之。 "Airport noise compatibility program" and "program" mean that pro |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

gr

| am, and all revisions thereto, reflected in documents (and revised documents) developed in accordance with Appendix B of this part, including the measures proposed or taken by the airport operator to reduce existing noncompatible land uses and to prevent the introduction of additional noncompatible land uses within the area. “機場噪音相容性計畫”及”計畫”表該計畫及其後之修訂版本，依照本號規則附件B而發展而得，包含機場營運者提議處理有關降低現存土地使用之不相容性，以及避免引入額外之土地使用不相容性的措施。 "Airport Operator" means, the operator of an airport as defined in the ASNA Act. “機場營運者”為ASNA法案中所定義的經營機場的人。 "ASNA Act" means the Aviation Safety and Noise Abatement Act of 1979, as amended (49 U.S.C. 2101 et seq.). “ASNA法”為1979年航空安全與噪音防制法及其修正版。 "Average sound level" means the level, in decibels, of the mean-square, A- weighted sound pressure during a specified period, with reference to the square of the standard reference sound pressure of 20 micropascals. “平均聲音水準”，單位：分貝，於一特定期間中，參考基準聲壓(standard reference sound pressure)（為20微帕）平方之A加權尺度聲壓水準平均值平方之水準。 "Compatible land use" means the use of land that is identified under this part as normally compatible with the outdoor noise environment (or an adequately attenuated noise level reduction for any indoor activities involved) at the location because the yearly day-night average sound level is at or below that identified for that or similar use under Appendix A（Table 1）of this part. “相容性土地使用”表示本號規則界定的土地用途按規則與戶外噪音環境正常情況下相容（或對室內活動充分地降低噪音水準），即該區位之年平均日夜噪音水準不高於附件A表1（噪音土地使用相容性表）之建議，且土地用途符合或類似上述之建議。 "Day-night average sound level" (DNL) means the 24-hour average sound level, in decibels, for the period from midnight to midnight, obtained after the addition of ten decibels to sound levels for the periods between midnight and 7 a.m., and between 10 p.m., and midnight, local time." The symbol for DNL is Ldn. “平均日夜聲音水準”，24小時的平均噪音水準，以分貝為單位，估計的期間為第一日午夜12時至次日午夜12時；其中，午夜12時至次日7時與晚上10時至午夜12時（以當地時間為準），需加上10分貝；其符號表為Ldn。 "Noise exposure map" means a scaled, geographic depiction of an airport, its noise contours, and surrounding area developed in accordance with section A150.101 of Appendix A of this part, including the accompanying documentation setting forth the required descriptions of forecast aircraft operations at that airport during the fifth calendar year beginning after submission of the map, together with the ways, if any, those operations will affect the map (including noise contours and the forecast land uses). “噪音曝露量地圖”，依照附件A第A150之101條，使用等噪音線發展該圖，做地理上的量度與描述機場及其周圍地區，且需有附帶文件描述該機場的噪音曝露量地圖（自提出後第五年[the fifth calendar year]）之預測飛行器運作情形，如有影響該圖（含等噪音線及預測之土地使用情形）亦應一併提出。 "Noise level reduction" (NLR) means the amount of noise level reduction in decibels achieved through incorporation of noise attenuation (between outdoor and indoor levels) in the design and construction of a structure. |  | “噪音水準降低值”表噪音水準降低量，單位：分貝，透過結構的構建與設計衡量室內與室外的噪音降低值。 "Noncompatible land use" means the use of land that is identified under this part as normally not compatible with the outdoor noise environment (or an adequately attenuated noise reduction level for the indoor activities involved at the location) because the yearly day-night average sound level is above that identified for that or similar use under Appendix A (Table 1) of this part. “不相容性土地使用”，表示本號規則界定的土地用途，按規則與戶外噪音環境（或對室內降低之噪音值）不相容，即該區位之年平均日夜噪音水準高於附件A表1（噪音—土地使用相容性表）建議之噪音—土地相容（符合或類似）用途。 "Regional Airports Division Manager" means the Airports Division Manager having responsibility for the geographic area in which the airport in question is located. “各管區之分區機場管理員”，分區機場管理員需負責處理地理行政區內之機場問題。 "Restriction affecting flight procedures" means any requirement, limitation, or other action affecting the operation of aircraft, in the air or on the ground. “影響飛航程序之限制”即任何規定、限制、或其他決議行為影響飛行器於空域或地面的操作。 "Sound exposure level" means the level, in decibels, of the time integral of squared A-weighted sound pressure during a specified period or event, with reference to the square of the standard reference sound pressure of 20 micropascals and a duration of on |  |  |  |
| --- | --- | --- | --- | --- | --- |

e

| second. “聲音曝露水準”，單位：分貝，於一特定期間或事件中，參考基準聲壓(standard reference sound pressure)（為20微帕）平方，持續一秒的A加權尺度積分平方值之水準。 "Yearly day-night average sound level" (YDNL) means the 365-day average, in decibels, day-night average sound level. The symbol for YDNL is also Ldn. “年平均日夜聲音水準”，單位：分貝，表365天的平均日夜聲音水準之平均值，以符號”YDNL”與”Ldn”表之。 Sec. 150.9 Designation of noise systems. 第150.9條 指定使用之噪音系統 For purposes of this part, the following designations apply: 為達成本部之目的，於應用下列指定工具時： (a) The noise at an airport and surrounding areas covered by a noise exposure map must be measured in A-weighted sound pressure level (LA) in units of decibels (dBA) in accordance with the specifications and methods prescribed under Appendix A of this part. (a) 噪音曝露量地圖所包含之機場及周圍地區之噪音值，必須以A加權尺度聲壓水準（A-weighted sound pressure level , LA）測量，其單位為分貝(dBA)，並依照本部附件A所描述的方法與規格說明。 (b) The exposure of individuals to noise resulting from the operation of an airport must be established in terms of yearly day-night average sound level (YDNL) calculated in accordance with the specifications and methods prescribed under Appendix A of this part. (b) 機場運作之單一事件噪音值，必須依照本部附件A所描述的方法與規格說明，以年平均日夜聲音水準（yearly day-night average sound level , YDNL）累計建立之。 (c) Uses of computer models to create noise contours must be in accordance with the criteria prescribed under Appendix A of this part. (c) 於使用計算機模式建立等噪音線時，應依照本部附件A所描述之準則。 Sec. 150.11 Identification of land uses. 第150.11條 界定土地使用 For the purposes of this part, uses of land which are normally compatible or noncompatible with various noise exposure levels to individuals around airports must be identified in accordance with the criteria prescribed under Appendix A of this part. Determination of land use must be based on professional planning criteria and procedures utilizing comprehensive, or master, land use planning, zoning, and building and site designing, as appropriate. If more than one current or future land use is permissible, determination of compatibility must be based on that use most adversely affected by noise. 為達本部之目的，一般而言，土地使用對單一事件在機場周遭不同噪音曝露水準下之相容性或不相容性，需依照本部附件A所描述之準則；土地使用之決定必基於專業規畫準則與程序，利用於適當的綜合計畫、主計畫、土地使用規畫、分區以及建築與基地之設計。土地使用若有一個以上許可之用途（最近或未來），必須基於最少噪音之影響來做相容性之決議。 Sec. 150.13 Incorporations by reference. 第150.13條 聯合參照 (a)General. This part prescribes certain standards and procedures which are not set forth in full text in the rule. Those standards and procedures are hereby incorporated by reference and were approved for incorporation by reference by the Director of the Federal Register under 5 U.S.C. 552(a) and 1 CFR Part 51. (a) 一般，本部所規定之標準與程序有些並不在本規定中全文公佈，這些標準與程序依據聯邦法典第5篇第552條a項(5 U.S.C. 552(a))與聯邦管理規則彙編第一篇第51號規則(1 CFR Part 51)之規定，在負責發行聯邦公報之首長Director of the Federal Register核准下，據此聯合參照之。 (b)Changes to incorporated matter. Incorporated matter which is subject to subsequent change is incorporated by reference according to the specific reference and to the identification statement. Adoption of any subsequent change in incorporated matter that affects compliance with standards and procedures of this part will be made under 14 CFR Part 11 and 1 CFR Part 51. (b) 變更部份，聯合參照部份若後續變更，改採特定之參照與確切之陳述，而影響本號規則之主從問題，則依聯邦管理規則彙編第14篇第11號規則(14 CFR Part 11)及第1篇第51號規則(1 CFR Part 51)處理之。 (c)Identification statement. The complete title or description which identifies each published matter incorporated by reference in this part is as follows: (c) 確切之陳述，本號規則聯合參照相關之出版品，其完整名稱與描述如下：（因以下條文牽涉本研究較少，故本附錄不列入） Subpart B--Development of Noise Exposure Maps and Noise Compatibility Programs B部份—發展噪音曝露量地圖與噪音相容性計畫 Sec. 150.21 Noise exposure maps and related descriptions. 第150.21條 噪音曝露量地圖與相關規定 (a)Each airport operator may after completion of the consultations and public procedure specified under paragraph (b) of this section submit to the Regional Airports Division Manager five copies of the noise exposure map (or revised map) which identifies each noncompatible land use in each area depicted on the map, as of the date of submission, and five copies of a map each with accompanying documentation setting forth— (a) 每位機場營運者在完成本條(b)項指定之諮詢與公開程序後，可(may)向九大管區之所屬區域之分區機場管理員(the Regional Airports Division Manager)提出噪音曝露量地圖（或更新版本）五份，上面皆需載明不相容性土地使用之區位、提交仲裁之日期以及附上以下之公開文件： (1)The noise exposure based on forecast aircraft operations at the airport for the fifth calendar year beginning after the date of submission (based on reasonable assumptions concerning future type and frequency of aircraft operations, number of nighttime operations, flight patterns, airport layout including any planned airport development, planned land use changes, and demographic changes in the surrounding areas); and (1) 自提交仲裁日起第五年（the fifth calendar year），以飛行器於該機場運作為基礎之噪音預測曝露量（應基於合理假設地考慮以下各項：未來飛行器運作之頻次與機型、夜間運作之數量、飛行模式、包含機場任一發展計畫中之配置情形、變更土地用途之計畫以及機場周圍地區人口統計量之變更等），與 (2)The and extent, if any, to which those forecast operations will affect the compatibility and land uses depicted on the map. (2) 若預測運作方式將影響相容性與土地使用，需描述其在噪音曝露量地圖上之範圍與性質。 (b)Each map, and related documentation submitted under this section must be developed and prepared in accordance with Appendix A of this part, or an FAA approved equivalent, and in consultation with states, and public agencies and planning agencies whose area, or any portion of whose area, of jurisdiction is within the Ldn 65 dB contour depicted on the map, FAA regional officials, and other Federal officials having local responsibility for land uses depicted on the map. This consultation must include regular aeronautical users of the airport. The airport operator shall certify that it has afforded interested persons adequate opportunity to submit their views, data, and comments concerning the correctness and adequacy of the draft noise exposure map and descriptions of forecast aircraft operations. Each map and revised map must be accompanied by documentation describing the consultation accomplished under this paragraph and the opportunities afforded the public to review and comment during the development of the map. One copy of all written comments received during consultation shall also be filed with the Regional Airports Division Manager. (b) 在本條（第150之21條）所提出之每一噪音曝露量地圖與相關文件，必須依本號規則附件A（或FAA核准之同等效力方式）發展與準備，並諮詢位於該噪音曝露量地圖上，平均日夜聲音水準65分貝等噪音線內的區域（或部份區域）具有管轄權的州屬、公共和規畫機構之意見，以及諮詢FAA各分區主管機關與負責有關該地區土地使用之聯邦機關之意見，此意見諮詢亦需包含固定使用該機場的航空運輸業者或個人； 而機場營運者應保證每一利益團體或相關人士，對噪音曝露量地圖之草案的適當性、正確性，以及未來飛行器運作之預測描述，皆有充分機會提出觀點、資料與批評。是以每份噪音曝露量地圖（或更新版本）必須附上本條本項要求之相關諮詢結果，與發展噪音曝露量地圖時公眾之檢視與評論的文件；接受諮詢之書面意見需提供一份給FAA該管區之分區機場管理員備案用。 (c)The Regional Airports Division Manager acknowledges receipt of noise exposure maps and descriptions and indicates whether they are in compliance with the applicable requirements. The Regional Airports Division Manager publishes in the Federal Register a notice of compliance for each such noise exposure map and description, identifying the airport involved. Such notice includes information as to when and where the map and related documentation are available for public inspection. (c) 分區機場管理員於收到噪音曝露量地圖與相關書面文件後，需公佈其接收事實與指示有否符合相關規定，並於聯邦公報(the Federal Register)公告之，以確認所描述之機場；此份公告意在知會相關公眾，公開檢查該噪音曝露量地圖與相關書面文件的時間與地點。 (d)If, after submission of a noise exposure map under paragraph (a) of this section, any change in the operation of the airport would create any "substantial, new noncompatible use" in any area depicted on the map beyond that which is forecast for the fifth calendar year after the date of submission, the airport operator shall, in accordance with this section, promptly prepare and submit a revised noise exposure map. A change in the operation of an airport creates a substantial new noncompatible use if that change results in an increase in the yearly day-night average sound level of 1.5 dB or greater in either a land area which was formerly compatible but is thereby made noncompatible under Appendix A (Table 1), or in a land area which was previously determined to be noncompatible under that Table and whose noncompatibility is now significantly increased. Such updating of the map shall include a reassessment of those areas excluded under sec. A150.101(e)(5) of Appendix A because of high ambient noise levels. If the five-year forecast map is based on assumptions involving recommendations in a noise compatibility program which are subsequently disapproved by the FAA, a revised map must be submitted if revised assumptions would create a substantial, new noncompatible use not indicated on the initial five-year map. Revised noise exposure maps are subject to the same requirements and procedures as initial submissions of noise exposure maps under this part. (d) 若依本條a項，於提出噪音曝露量地圖供裁示後，機場運作產生任何「潛在上，新的不相容性使用」之變動時，即超出該圖描述之情境（提交供裁示日起第五年之預測情境），機場營運者應依本條立刻準備及提出校定版本以供裁示。 機場所產生「潛在上，新的不相容性使用」之變動，為引起年平均日夜聲音水準增加1.5分貝（或以上），無論依附件A表一之規定，原來為相容性或不相容性者皆是；除附件A之第A150之101條第e項第5款之原因外（因背景噪音水準高於機場營運所引起的噪音水準），皆需更新此「地圖」並重新評估該區域。 若所預測的第五年噪音曝露量地圖之基本假設，包含噪音相容性計畫中所建議之事項，而該建議事項稍後不為FAA所核准，則須另外提出校定後的「地圖」；但若校定後之假設情境，仍將引起潛在上、新的不相容性使用（於初次地圖中尚未標示出），該「地圖」仍需提交裁示，依本號規則，重提之更新版本噪音曝露量地圖，仍依初次所提交裁示之相同規定與程序裁決之。 (e)Each map, or revised map, and description of consultation and opportunity for public comment, submitted to the FAA, must be certified as true and complete under penalty of 18 U.S.C. 1001. (e) 每一提交FAA裁決之「地圖」（或重新校定之）與書面文件（即描述徵詢公眾之看法和時機），必須檢定其真實性與完整性，否則依聯邦法典第18篇第1001條處罰之。 (f)(1) The ASNA Act provides, in section 107 (a) (49 U.S.C. 2107(a)), that: No person who acquires property or an interest therein after the date of enactment of the Act in an area surrounding an airport with respect to which a noise exposure map has been submitted under section 103 of the Act shall be entitled to recover damages with respect to the noise attributable to such airport if such person had actual or constructive knowledge of the existence of such noise exposure map unless, in addition to any other elements for recovery of damages, such person can show that -- (f)(1)ASNA法案第107條第a項(49 U.S.C. 2107(a))提出：自該法案生效日以後，且依本法案第103條規定下，對於已呈交噪音曝露量地圖之機場周圍區域，無任何人士（若該人士「實際上( actual )或肯定地( constructive )認知」該噪音曝露量的存在）具有可於其中獲得財產或利益之資格（即具有受該機場有關噪音損害性質之補償資格），除非該人士可檢具以下要件，做為補償其所受損害 -- (i)A significant change in the type or frequency of aircraft operations at the airport; or (i)於該機場飛行器運作機型與頻次顯著之改變；或 (ii)A significant change in the airport layout; or (ii)機場配置顯著之改變；或 (iii)A significant change in the flight patterns; or (iii)飛航模式顯著之改變；或 (iv)A significant increase in nighttime operations; (iv)夜間營運顯著之改變； occurred after the date of the acquisition of such property or interest therein and that the damages for which recovery is sought have resulted from any such change or increase. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

| 上述事件發生於取得其中的財產或利益之後，且蒙受損害區域，其尋求補償之損害歸因於上述任一項之改變或增加。 (2)The Act further provides in section 107(b), (49 U.S.C. 2107(b)): That for this purpose, "constructive knowledge" shall be imputed, at a minimum, to any person who acquires property or an interest therein in an area surrounding an airport after the date of enactment of the Act if-- (2) 該法案於第107條b項(49 U.S.C. 2107(b)) 再提出：為此目的，以下為產生「肯定地認知」的最低限度﹔即任一人士於該法案生效日期後，才取得機場周圍區域中的財產或利益，並且 (i)Prior to the date of such acquisition, notice of the existence of a noise exposure map for such area was published at least three times in a newspaper of general circulation in the county in which such property is located; or (i) 先於取得財產（或利益）之日期前，有關目前該財產所在地之噪音曝露量地圖已完成並公告之（即經該財產所在地之地方性一般發行量之報紙刊載至少三次）；或 (ii)A copy of such noise exposure map is furnished to such person at the time of such acquisition. (ii) 當該人士取得之際，一份噪音曝露量地圖已完成。 (g)For this purpose, the term "significant" in paragraph (f) of this section means that change or increase in one or more of the four factors which results in a "substantial new noncompatible use" as defined in Sec. 150.21(d), affecting the property in issue. Responsibility for applying or interpreting this provision with respect to specific properties rests with local government. (g) 為此目的，本條(第150之21條)第f項所描述之「顯著」，意謂四個因子中，一個（或兩個以上）的改變或增加，導致本條d項所定義「潛在上新的不相容性使用」影響財產之結果。而地方政府( local government ) 有義務解釋或應用本規定有關一定財產之規範。 Sec. 150.23 Noise compatibility programs. 第150.23條 噪音相容性計畫 (a) Any airport operator who has submitted an acceptable noise exposure map under Sec. 150.21 may, after FAA notice of acceptability and other consultation and public procedure specified under paragraphs (b) and (c) of this section, as applicable, submit to the Regional Airports Division Manager five copies of a noise compatibility program. (a) 任一機場營運者於提出一可接受的噪音曝露量地圖（依第150之21條所發展）後，並經本條(第150之23條)第b、c項指定的諮詢與公開程序，以及FAA接受公告(notice of acceptability )後，得向所屬管區之分區機場管理員，提出噪音相容性計畫五份供裁示。 (b) An airport operator may submit the noise compatibility program at the same time as the noise exposure map. In this case, the Regional Airports Division Manager will not begin the statutory 180- |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

da

| y review period (for the program) until after FAA reviews the noise exposure map and finds that it and its supporting documentation are in compliance with the applicable requirements. (b) 機場營運者可於提出噪音曝露量地圖之際，同時提出噪音相容性計畫；此種情況下，分區機場管理員尚不立刻展開為期180天的規定檢查（檢視此計畫），直到FAA檢查完該機場營運者所提之「地圖」與相關補充文件俱符合適用之規定後，分區機場管理員才開始檢查此計畫。 (c) Each noise compatibility program must be developed and prepared in accordance with Appendix B of this part, or an FAA approved equivalent, and in consultation with FAA regional officials, the officials of the state and of any public agencies and planning agencies whose area, or any portion or whose area, of jurisdiction within the Ldn 65 dB noise contours is depicted on the noise exposure map, and other Federal officials having local responsibility of land uses depicted on the map. Consultation with FAA regional officials shall include, to the extent practicable, informal agreement from FAA on proposed new or modified flight procedures. For air carrier airports, consultation must include any air carriers and, to the extent practicable, other aircraft operators using the airport. For other airports, consultation must include, to the extent practicable, aircraft operators using the airport. (c) 每一噪音相容性計畫需依照本號規則之附件B（或FAA核准之同等效力方式）發展及準備，並諮詢位於該噪音曝露量地圖上，平均日夜聲音水準65分貝等噪音線內的區域（或部份區域）具有管轄權的州屬、公共和規畫機構之意見，以及諮詢FAA各分區主管機關與負責有關該地區土地使用之聯邦機關之意見。 諮詢FAA各分區主管機關之意見時，應包含：可實際執行的範圍、FAA對所提議新的或修正的飛航程序，所做非正式的協議。諮詢營運於該機場之航空運輸業者之意見時，應包含：所有業者與可實際執行的範圍、及其他使用該機場的駕駛員。於諮詢其他機場之意見時，應包含：可實際執行的範圍、及其他使用該機場的駕駛員。 (d) Prior to and during the development of a program, and prior to submission of the resulting draft program to the FAA, the airport operator shall afford adequate op |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

po

| rtunity for the active and direct participation of the states, public agencies and planning agencies in the areas surrounding the airport, aeronautical users of the airport, and the general public to submit their views, data, and comments on the formulation and adequacy of that program. (d) 在預備或已發展該噪音相容性計畫時，以及向FAA提出該「計畫」草案之前，機場營運者應當提供充分機會予機場周圍之州屬、公共和規畫機構、機場之飛航使用者與公眾積極且直接的參與，以提出他們對該計畫之充分性與設計內容的觀點、資料與批評。 (e) Each noise compatibility program submitted to the FAA must consist of at least the following: (e) 每一提供FAA裁決之噪音相容性計畫，至少必須由以下九點所構成： (1) A copy of the noise exposure map and its supporting documentation as found in compliance with the applicable requirements by the FAA, per Sec. 150.21(c). (1) 一份符合FAA相關規定（第150之21條第c項所規定）之噪音曝露量地圖與有關補充文件。 (2) A description and analysis of the alternative measures considered by the airport operator in developing the program, together with a discussion of why each rejected measure was not included in the program. (2) 機場營運者於發展該計畫時曾考慮過的替代措施方案的描述與分析，以及討論所駁斥之措施的原因。 (3) Program measures proposed to reduce or eliminate present and future noncompatible land uses and a description of the relative contribution of each of the proposed measures to the overall effectiveness of the program. (3) 計畫提出以降低或消除現存與未來的不相容性土地使用之措施，並描述這些預計措施的貢獻對全盤計畫之效果。 (4) A description of public participation and the consultation with officials of public agencies and planning agencies in areas surrounding the airport, FAA regional officials and other Federal officials having local responsibility for land uses depicted on the map, any air carriers and other users of the airport. (4) 描述公眾參與諮詢相關部門之結果（如機場周圍地區之公共和規畫機構之意見、FAA各分區主管機關與負責有關該地區土地使用之聯邦機關之意見，以及航空運輸業者或機場之其他使用者之意見等）。 (5) The actual or anticipated effect of the program on reducing noise exposure to individuals and noncompatible land uses and preventing the introduction of additional noncompatible uses within the area covered by the noise exposure map. The effects must be based on expressed assumptions concerning the type and frequency of aircraft operations, number of nighttime operations, flight patterns, airport layout including planned airport development, planned land use changes, and demographic changes within the Ldn 65 dB noise contours. (5) 該計畫於降低對個體的噪音曝露量與不相容性土地使用，以及避免引入額外不相容性使用至噪音曝露量地圖所涵蓋範圍的實際或預期成果。這些效果必基於明確地假設以下條件：飛行器運作之頻次與機型、夜間運作之數量、飛行模式、包含機場任一規畫中之配置情形、變更土地用途之計畫以及機場周圍地區人口統計量之變更等。 (6) A description of how the proposed future actions may change any noise control or compatibility plans or act |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

io

| ns previously adopted by the airport proprietor. (6) 需描述所提議之計畫，可能改變機場營運者前期執行的噪音控制或相容性之規畫或決議事項。 (7) A summary of the comments at any public hearing on the program and a copy of all written material submitted to the operator under paragraphs (c) and (d) of this section, together with the operator's response and disposition of those comments and materials to demonstrate the program is feasible and reasonably consistent with obtaining the objectives of airport noise compatibility planning under this part. (7) 本「計畫」之公聽會意見摘要；與按照本條(第150之23條)第c、d項規定，向營運者提出之所有書面資料一份；並附上營運者對這些意見或資料的回應與處理狀況，以證明該計畫乃基於本號規則機場噪音相容規畫的目的，所做出合理、可行之計畫。 (8) The period covered by the program, the schedule for implementation of the program, the persons responsible for implementation of each measure in the program, and, for each measure, documentation supporting the feasibility of implementation, including any essential governmental actions, costs, and anticipated sources of funding, that will demonstrate that the program is reasonably consistent with achieving the goals of airport noise compatibility planning under this part. (8) 列出本計畫之執行時期；時間表；負責執行各項措施的人員；與支持該措施可行性的書面資料，如：必要的政府決議、成本與預期的基金籌措來源，這些可表示該計畫是基於合理地達成本號規則之噪音相容性規畫的目標。 (9) Provision for revising the program if made necessary by revision of the noise exposure map. (9) 若有修正噪音曝露量地圖，則該計畫必須提出重新修訂。 Subpart C--Evaluations and Determinations of Effects of Noise Compatibility Programs C部份—評估與決定噪音相容性計畫之成效 Sec. 150.31 Preliminary review: acknowledgments. 第150.31條 初步檢查與公證 (a) Upon receipt of a noise compatibility program submitted under Sec. 150.23, the Regional Airports Division Manager acknowledges to the airport operator receipt of the program and conducts a preliminary review of the submission. (a) 緊接著在收到依第150之23條所提交仲裁的噪音相容性計畫之後，分區機場管理員需公佈其收到機場營運者該計畫之事實，並為此指示一初步檢查。 (b) If, based on the preliminary review, the Regional Airports Division Manager finds that the submission does not conform to the requirements of this part, he disapproves and returns the unacceptable program to the airport operator for reconsideration and development of a program in accordance with this Part. (b) 若基於此初步檢查，分區機場管理員認為該提交計畫並不與本號規則之規定一致，可以不核准並退回該計畫，以要求機場營運者重新考慮與發展符合本號規則規定之計畫。 (c) If, based on the preliminary review, the Regional Airports Division Manager finds that the program conforms to the requirements of this part, the Regional Airports Division Manager publishes in the Federal Register a notice of receipt of the program for comment which indicates the following: (c) 若基於此初步檢查，分區機場管理員認為該提交計畫與本號規則之規定一致，則於聯邦公報( the Federal Register )公告接受此計畫之評論，且顯示以下： (1) The airport covered by the program, and the date of receipt. (1) 該計畫所涵蓋之機場，與收到該計畫之日期。 (2) The availability of the program for examination in the offices of the Regional Airports Division Manager and the airport operator. (2) 該計畫受過FAA於該管區之機場管理單位與機場營運者考驗之有效性。 (3) That comments on the program are invited and, will be considered by the FAA. (3) 邀請對該計畫之評論，FAA亦考慮所提出之評論。 (d) The date of signature of the published notice of receipt starts the 180-day approval period for the program. (d) 自發佈收取公告日起為期180天，為該計畫的核准期限。 Sec. 150.33 Evaluation of programs. 第150.33條 評估計畫 (a) The FAA conducts an evaluation of each noise compatibility program and, based on that evaluation, either approves or disapproves the program. The evaluation includes consideration of proposed measures to determine whether they-- (a) FAA指示每一噪音相容計畫之評估工作，以核准或不核准該計畫。評估內容包含對該計畫所提議之措施： (1) May create an undue burden on interstate or foreign commerce (including unjust discrimination); (1) 決議其有否造成對州際或外國商務不當之負擔（包含不公平之歧視） (2) Are reasonably consistent with obtaining the goal of reducing existing noncompatible land uses and preventing the introduction of additional noncompatible land uses; and (2) 決議其是否與降低現存的以及避免引入新的不相容性土地使用的目標合理一致 (3) Include the use of new or modified flight procedures to control the operation of aircraft for purposes of noise control, or affect flight procedures in any way. (3) 決議其有否採用新的或修正之飛航程序，以控制飛行器操作時的噪音，或任何影響飛航程序之噪音控制方法。 (b) The evaluation may also include an evaluation of those proposed measures to determine whether they may adversely affect the exercise of the authority and responsibilities of the Administrator under the Federal Aviation Act of 1958, as amended. (b) 此評估亦可包含其提議之措施對1958年Federal Aviation Act（與修訂版）法案下，是否違背聯邦航空總署（FAA）署長之職權運作。 (c) To the extent considered necessary, the FAA may (c) 依需要，FAA於考慮範圍內可以： (1) Confer with the airport operator and other persons known to have information and views material to the evaluation; (1) 與機場營運者或其他已知人士協議，以獲得評估用的資訊與檢視用之資料。 (2) Explore the objectives of the program and the measures, and any alternative measures, for achieving the objectives. (2) 探討該計畫與其措施之目的，以及為達成這些目的之替代措施。 (3) Examine the program for developing a range of alternatives that would eliminate the reasons, if any, for disapproving the program. (3) 調查該計畫中，為避免不受核准而剔除的一系列替代方案。 (4) Convene an informal meeting with the airport operator and other persons involved in developing or implementing the program for the purposes of gathering all facts relevant to the determination of approval or disapproval of the program and of discussing any needs to accommodate or modify the program as submitted. (4) 召集一非正式會議，由機場營運者與有關該計畫發展、執行之人士所組成，目的在於收集相關事實，以決議該計畫是否核准，與討論該計畫是否需要調整或修正。 (d) If requested by the FAA, the airport operator shall furnish all information needed to complete FAA's review under (c). (d) 若FAA為完成第c項之檢查需要，機場營運者應提供所有必要資訊。 (e) An airport operator may, at any time before approval or disapproval of a program, withdraw or revise the program. If the airport operator withdraws or revises the program or indicates to the Regional Airports Division Manager, in writing, the intention to revise the program, the Regional Airports Division Manager terminates the evaluation and notifies the airport operator of that action. That termination cancels the 180-day review period. The FAA does not evaluate a second program for any airport until any previously submitted program has been withdrawn or a determination on it is issued. A new evaluation is commenced upon receipt of a revised program, and a new 180-day approval period is begun, unless the Regional Airports Division Manager finds that the modification made, in light of the overall revised program, can be integrated into the unmodified portions of the revised program without exceeding the original 180-day approval period or causing undue expense to the government. 一機場營運者可在其計畫核准與否前提出撤銷或修正，若機場營運者提出撤銷、修正該計畫，或以書面形式向分區機場管理員表示修正該計畫之意圖，則FAA之分區機場管理員應終止評估，並通知該機場營運者此項決議，並取消為期180天的檢查，直到先前提交仲裁之計畫已被撤銷或已發佈其決議，否則FAA不再評估該機場第二個計畫。 分區機場管理員於收到修訂過之計畫後，立即展開新的評估與另一為期180天的查核；除非，鑑於修訂過之整體計畫，分區機場管理員認為修正部份之檢查，不會超出原給定180天內剩餘之天數，或避免引起政府資源不當地使用。 Sec. 150.35 Determinations; publications; effectivity. 第150.35條 決議﹑公佈與生效 (a) The FAA issues a determination approving or disapproving each airport noise compatibility program (and revised program). Portions of a program may be individually approved or disapproved. No conditional approvals will be issued. A determination on a program acceptable under this part is issued within 180 days after the program is received under Sec. 150.23 of this part or it may be considered approved, except that this time period may be exceeded for any portion of a program relating to the use of flight procedures for noise control purposes. A determination on portions of a program covered by the exceptions to the 180-day review period for approval will be issued within a reasonable time after receipt of the program. Determinations relating to the use of any flight procedure for noise control purposes may be issued either in connection with the determination on other portions of the program or separately. Except as provided by this paragraph, no approval of any noise compatibility program, or any portion of a program, may be implied in the absence of the FAA's express approval. (a) FAA對每一機場噪音相容性計畫發佈其決定為核准或為不核准，其中，一計畫之部分可以為個別核准或不核准，但無條件性核准。 於該計畫達成本號規則第150之23條規定，為FAA接獲後之180天內，依本號規則規定將公佈該計畫可接受與否之決議，除因該計畫使用有關飛航程序以控制噪音，可能超出該段檢查期間（此部份FAA於收到計畫後一合理期間內，將再發佈其決議），否則可被視為核准。 有關使用任何飛航程序以達控制噪音目的之決議，可與此計畫其他部份之決議連同公佈之，或個別公佈。除本條第a項所提及之部分外，無任一噪音相容性計畫（或計畫之部分）可被引伸為缺乏FAA之表示核准與否。 (b) The Administrator approves programs under this part, if-- (b) 依照本號規則規定，FAA署長( Administrator )依下列條件核准噪音相性計畫： (1) It is found that the program measures to be implemented would not create an undue burden on interstate or foreign commerce (including any unjust discrimination) and are reasonably consistent with achieving the goals of reducing existing noncompatible land uses around the airport and of preventing the introduction of additional noncompatible land uses; (1) 執行該計畫之措施，將不致造成對州際或外國商務不當負擔（含不公平之歧視），並且與達成”降低機場現存與避免引進新的不相容性土地使用”的目標合理一致。 (2) The program provides for revision if made necessary by the revision of the noise map; and (2) 若有修訂噪音曝露量地圖，則該計畫亦必須提出重新修訂。 (3) Those aspects of programs relating to the use of flight procedures for noise control can be implemented within the period covered by the program and without-- (3) 在有關噪音相容性計畫使用飛航程序以控制噪音方面，於計畫執行時無以下缺失者： (i) Reducing the level of aviation safety provided; (i) 降低所提供之飛航安全水準 (ii) Derogating the requisite level of protection for aircraft, their occupants and persons and property on the ground; (ii) 減低飛行器、其所有人與個人及財產於地面之必要的保護水準。 (iii) Adversely affecting the efficient use and management of the Navigable Airspace and Air Traffic Control Systems; or (iii) 對所飛航空域與空中交通控制系統的有效使用與管理，具有負面影響者。 (iv) Adversely affecting any other powers and responsibilities of the Administrator prescribed by law or any other program, standard, or requirement established in accordance with law. (iv) 違背FAA署長之職權者，如FAA署長依法( Law )授與之職權，或其他依法制訂之相關計畫、標準或規定而授與之職權。 (c) When a determination is issued, the Regional Airports Division Manager notifies the airport operator and publishes a notice of approval or disapproval in the Federal Register identifying the nature and extent of the determination. (c) 決議發佈時，分區機場管理員得通知機場營運者並將核准結果公告於聯邦公報( the Federal Register )，以確認該項決議的性質與範圍。 (d) Approvals issued under this part for a program or portion thereof become effective as specified therein and may be withdrawn when one of the following occurs: (d) 依本號規則規定，對一噪音相容性計畫（或其部份）發佈核准後，其中之敘述說明即從此生效；但以下情形發生時，可撤銷其效力： (1) The program or portion thereof is required to be revised under this part or under its own terms, and is not so revised; (1) 該計畫或其部份，從此需依本號規則或該計畫之某項規定做再修訂時，而未做再修訂。 (2) If a revision has been submitted for approval, a determination is issued on the revised program or portion thereof, that is inconsistent with the prior approval. (2) 若其修訂計畫（或其部份）提交仲裁後所公佈之決議，從此與先前之核准不一致時。 (3) A term or condition of the program, or portion thereof, or its approval is violated by the responsible government body. (3) 所核准該計畫（或其部份）中的某項或條件，從此違背有責之政府主體。 (4) A flight procedure or other FAA action upon which the approved program or portion thereof is dependent is subsequently disapproved, significantly altered, or rescinded by the FAA. (4) 所核准的計畫（或其部份）中，依循的某一飛航程序或其他FAA之決議，從此不被核准、或顯著改變、或為FAA取消者。 (5) The airport operator requests rescission of the approval. (5) 機場營運者請求此計畫之核准無效。 (6) Impacts on flight procedures, air traffic management, or air commerce occur which could not be foreseen at the time of approval. (6) 於檢查核准期間內，有不可預見之衝擊（有關飛航程序、空中交通管理或空中商務之衝擊）發生。 A determination may be sooner rescinded or modified for cause with at least 30 days written notice to the airport operator of the FAA's intention to rescind or modify the determination for the reasons stated in the notice. The airport operator may, during the 30-day period, submit to the Regional Ai |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

rports Division Manager for consideration any reasons and circumstances why the determination should not be rescinded or modified on the basis stated in the notice of intent. Thereafter, the FAA either rescinds or modifies the determination consistent with the notice or withdraws the notice of intent and terminates the action.

FAA可對其所做決定立即予以取消或變更，並以書面通知機場營運者至少三十天，說明FAA取消或變更其決定的理由與狀況。而機場營運者可在此30天期限內，基於書面通知的內容，向分區機場管理員提出”不應再被取消或變更決定之理由與狀況”予以考慮之；FAA無論是依舊取消或變更該決定，或是撤回該通知，此後一律結束此項決議行動。

(e) Determinations may contain conditions which must be satisfied prior to implementation of any portion of the program relating to flight procedures affecting airport or aircraft operations.

(e) 決議中可能包含一些條件，這些條件應於執行計畫中涉及影響機場或飛行器運作之有關飛航程序以前，先被執行或完成之。

(f) Noise exposure maps for current and five year forecast conditions that are submitted and approved with noise compatibility programs are considered to be the new FAA accepted noise exposure maps for purposes of Part 150.

(f) FAA當局基於本號規則之目的，可接受”最近與預測第五年情境之噪音曝露量地圖”與”噪音相容性計畫”一併提交仲裁。

A

| ppendix A to Part 150--Noise Exposure Maps 附件A – 噪音曝露量地圖 Part A—General A部份 – 一般 A150.1 Purpose. 第A150.1條 目的 |  | A150.3 Noise descriptors. 第A150.3條 噪音描述 A150.5 Noise measurement procedures and equipment. 第A150.5條 噪音測量程序與設備 | Part B--Noise Exposure Map Development B部份 – 發展噪音曝露量地圖 | A150.101 Noise contours and land usages. 第A150.101條 等噪音線與土地使用說明 A150.103 Use of computer prediction model. 第A150.103條 電腦預測模式之使用 |  |
| --- | --- | --- | --- | --- | --- |
| A150.105 Identification of public agencies and planning agencies. 第A150.201條 一般 A150.203 Symbols. 第A150.203條 符號 A150.205 Mathematical computations. 第A150.205條 數學計算 Part A—General A部份 – 一般 Sec. A150.1 Purpose. 第A150.1條 目的 (a) This appendix establishes a uniform methodology for the development and preparation of airport noise exposure maps. That methodology includes a single system of measuring noise at airports for which there is a highly reliable relationship between projected noise exposure and surveyed reactions of people to noise along with a separate single system for determining the exposure of individuals to noise. It also identifies land uses which, for the purpose of this part are considered to be compatible with various exposures of individuals to noise around airports. (a) 本附件建立統一發展與準備機場噪音曝露量地圖之方法。該方法包含單一測量機場噪音之系統，使得依設計量測出的噪音曝露量與調查群眾對噪音之反應，具有高 第A150.105條 公家機關與規畫機構之確認 Part C--Mathematical Descriptions C部份 – 數學描述 A150.201 General. |  |  |  |  |  |

度地

| 、可靠地關連性，以及決定群眾對噪音之曝露量的單一系統。並依本號規則之目的，確認機場周圍土地，個別相容性用途之不同噪音曝露量。 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| (b) This appendix provides for the use of the FAA's Integrated Noise Model (INM) or an FAA approved equivalent, for developing standardized noise exposure maps and predicting noise impacts. Noise monitoring may be utilized by airport operators for data acquisition and data refinement, but is not required by this part for the development of noise exposure maps or airport noise compatibility programs. Whenever noise monitoring is used, under this part, it should be accomplished in accordance with Sec. A150.5 of this appendix. (b) 本附件提供FAA的整合噪音模式( Integrated Noise Model )或其他FAA核准之同等方法，以發展標準化噪音曝露量地圖，與預測噪音衝擊。機場營運者可利用噪音監測來取得或改善資料，但不被本號規則用來發展噪音曝露量地圖，或機場噪音相容性計畫；不論何時，本號規則於使用噪音監測方法時，應依照本附件之第A150.5條規定完成。 Sec. A150.3 Noise descriptors. 第A150.3條 噪音描述 (a)Airport Noise Measurement. The A-Weighted Sound Level, measured, filtered and recorded in accordance with Sec. A150.5 of this appendix, must be employed as the unit for the measurement of single event noise at airports and in the areas surrounding the airports. (a) 量測機場噪音時，依照本附件之第A150.5條規定，必須使用A—加權尺度聲音水準度量、過濾與記錄之，以量測機場與其周圍地區之單一事件噪音。 (b) Airport Noise Exposure. The yearly day-night average sound level (YDNL) must be employed for the analysis and characterization of multiple aircraft noise events and for determining the cumulative exposure of individuals to noise around airports. (b) 機場噪音曝露量之衡量指標，為年平均日夜聲音水準( the yearly day-night average sound level )，用以分析與描述多宗飛行器噪音事件，並決定機場附近噪音對個體的累積曝露量。 Sec. A150.5 Noise measurement procedures and equipment. 第A150.5條 噪音測量程序與設備 (a) Sound levels must be measured or analyzed with equipment having the "A" frequency weighting, filter characteristics, and the "slow response" characteristics as defined in International Electrotechnical Commission (IEC) Publication No. 179, entitled "Precision Sound Level Meters" as incorporated by reference in Part 150 under Sec. 150.11. For purposes of this part, the tolerances allowed for general purpose, type 2 sound level meters in IEU 179, are acceptable. (a) 聲壓水準必須使用符合國際電工協會（IEC）標準（A頻加權﹑濾特性﹑慢特性）之噪音計。 (b) Noise measurements and documentation must be in accordance with accepted acoustical measurement methodology, such as those described in American National Standards Institute publication ANSI 51.13, dated 1971 as revised 1979, entitled "ANS--Methods for the Measurement of Sound Pressure Levels"; ARP No. 796, dated 1969, entitled "Measurement of Aircraft Exterior Noise in the Field"; "Handbook of Noise Measurement," Ninth Ed. 1980, by Arnold P.G. Peterson; or "Acoustic Noise Measurement," dated Jan., 1979, by J.R. Hassell and K. Zaveri. For purposes of this part, measurements intended for comparison to a State or local standard or with another transportation noise source (including other aircraft) must be reported in maximum A- weighted sound levels (LAM); for computation or validation of the yearly day- night average level (Ldn), measure |  |  |  |  |  |

me

| nts must be reported in sound exposure level (LAE), as defined in Sec. A150.205 of this appendix. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| (b) 該法條規定測量聲壓水準時，使用噪音計需符合國際電工協會（IEC）之標準；而噪音之量測方式、條件與資料格式，則需符合美國國家標準中心（ANSI）之規定。此外，在與某州或地方標準、以及其他運具（含其他飛行器）做噪音量比較時，需使用最大A加權聲壓水準（maximum A- weighted sound levels (LAM)），而於計算或校估年平均日夜聲音水準（yearly day- night average level(Ldn)）時，測量尺度需為第A150.205條定義之聲壓水準（sound exposure level (LAE)）。 Part B--Noise Exposure Map Development B部份 – 發展噪音曝露量地圖 Sec. A150.101 Noise contours and land usages. 第A150.101條 等噪音線與土地使用說明 (a) To determine the extent of the noise impact around an airport, airport proprietors developing noise exposure maps in accordance with this part must develop Ldn contours. Continuous contours must be developed for YDNL levels of 65, 70, and 75 (additional contours may be developed and depicted when appropriate). In those areas where YDNL values are 65 YDNL or greater, the airport operator shall identify land uses and determine land use compatibility in accordance with the standards and procedures of this appendix. (a) 機場營運者依本號規則發展以年平均日夜聲音水準為單位之等噪音線( Ldn contours )，而發展噪音曝露量地圖以決定機場周圍噪音影響之範圍。連續等噪音線必須以65、70、75年平均日夜聲音水準(YDNL)為主，如有需要，可再描述其他適當之等噪音線；在YDNL為65以及更大的地區，機場營運者需確認其土地使用與決定其土地使用之相容性（依本附件A之標準與程序）。 (b) Table 1 of this appendix describes compatible land use information for several land uses as a function of YDNL values. The ranges of YDNL values in Table 1 reflect the statistical variability for the responses of large groups of people to noise. Any particular level might not, the |  |  |  |  |  |

re

| fore, accurately assess an individual's perception of an actual noise environment. Compatible or noncompatible land use is determined by comparing the predicted or measured YDNL values at a site with the values given. Adjustments or modifications of the descriptions of the land-use categories may be desirable after consideration of specific local conditions. (b) 本附件A表一描述各種YDNL值下，相容性的各種土地使用用途之資訊；表一中各範圍內之YDNL值，則反映大部份群眾對噪音反應之統計上的變異性，所以任一特定之水準值並不能精確的評估個體實際感受之噪音環境。決定土地使用的相容性或不相容性，乃藉由比較一區位的YDNL量測值（或預測值）與一給定值之差異；如有調整或修正土地使用之分類描述時，需要先考量當地特定之情形。 (c) Compatibility designations in Table 1 generally refer to the major use of the site. If other uses with greater sensitivity to noise are permitted by local government at a site, a determination of compatibility must be based on that use which is most adversely affected by noise. When appropriate, noise level reduction through incorporation of sound attenuation into the design and construction of a structure may be necessary to achieve compatibility. (c) 表一指示之相容性通常以該區位的主要使用為準，而當地政府如有允許一區位使用於對噪音更敏感之用途時，相容性之決定應以區位中最有噪音負面影響之使用為準。若建築物透過減音之設計與構造而降低噪音水準，適當的話，可視其達成相容性。 (d) For the purpose of compliance with this part, all land uses are considered to be compatible with noise levels less than Ldn 65 dB. Local needs or values may dictate further delineation based on local requirements or determinations. (d) 為遵守本號規則之規定，所有噪音水準低於年平均日夜聲音水準(Ldn)65分貝之土地使用皆視為具相容性；而當地的需求或重視程度，可基於其要求或決議，做更進一步的劃分。 (e) Except as provided in (f) below, the noise exposure maps must also contain and identify: (e) 除非下面之第(f)項所提及，噪音曝露量地圖必須包含與確認下列各項： (1) Runway locations. (1) 跑道區位 (2) Flight tracks. (2) 飛航軌跡 (3) Noise contours of Ldn 65, 70, and 75 dB resulting from aircraft operations (3) .因飛行器操作所形成之年平均日夜聲音水準(Ldn)65、70以及75分貝之等噪音線。 (4) Outline of the airport boundaries. (4) 機場邊界之輪廓 (5) Noncompatible land uses within the noise contours, including those within the Ldn 65 dB contours. (No land use has to be identified as noncompatible if the self-generated noise from that use and/or the ambient noise from other nonaircraft and nonairport uses is equal to or greater than the noise from aircraft and airport sources.) (5) 位於等噪音線內之不相容性土地使用（含Ldn 65分貝之等噪音線內）。（若為某區域自己產生噪音、或其背景噪音非機場與飛行器所產生且超過機場與飛行器所產生者，該區之土地使用不被視為不相容性土地使用） (6) Location of noise sensitive public buildings (such as schools, hospitals, and health care facilities), and properties on or eligible for inclusion in the National Register of Historic Places. (6) 對噪音較敏感之公眾建築物區位（如學校、醫院與療養設施），以及財產（或具有資格者）被包含在National Register of Historic Places登記下。 (7) Locations of any aircraft noise monitoring sites utilized for data acquisition and refinement procedures. (7) 使用飛行器噪音監測設備以收集與改善資料的位置。 (8) Estimates of the number of people residing within the Ldn 65, 70, and 75 dB contours. (8) 估計居留於Ldn 65, 70, and 75等噪音線內之民眾數目。 (9) Depiction of the required noise contours over a land use map of a sufficient scale and quality to discern streets and other identifiable geographic features. (9) 在一土地使用地圖上描述所要求之等噪音線時，需具充分比例以辨識街道與其他可識別之地標。 (f) Notwithstanding any other provision of this Part, noise exposure maps prepared in connection with studies which were either Federally funded or Federally approved and which commenced before October 1, 1981, are not required to be modified to contain the following items: (f) 噪音曝露量地圖在進行與聯邦補助或聯邦核准有關之研究時，且於1981年10月1日前著手者，可忽略本號規則之任何其他規定而不需修正，以包含下列項目： (1) Flight tracks depicted on the map. (1) 描述於地圖上之航跡 (2) Use of ambient noise to determine land use compatibility. (2) 使用背景噪音以決定土地使用之相容性。 (3) The Ldn 70 dB noise contour and data related to Ldn 70 dB contour. When determinations on land use compatibility using Table |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

1

| differ between Ldn 65- 70 dB and the Ldn 70-75 dB, determinations should either use the more conservative Ldn 70-75 dB column or reflect determinations based on local needs and values. (3) 年平均日夜聲音水準( Ldn )70分貝之等噪音線與對應於此等噪音線之資料；於使用表1以決定土地使用相容性時，此決定應使用較保守之Ldn為 70 -75分貝的那一列， |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

或反應當地需要與重要性而決定之。

(4) Estimates of the number of people residing within the Ldn 65, 70, and 75 dB contours.

(4) 估計居留於Ldn 65, 70, and 75等噪音線內之民眾數目。

TABLE 1  --  Land Use Compatibility* With Yearly Day-Night Average Sound Levels

Yearly day-night average sound level (Ldn) in decibels

Below

65-

70-

75-

80-Over

Land use

65

70

75

80

85

85

Residential

Residential, other than mobile homes and transient lodgings

| Y | N(1) N(1) |  | N N N Mobile home parks Y N | N N N N Transient lodgings Y | N(1) N(1) N(1) N N Public Use |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Schools Y(2) Y Y Y N(1) N(1) N N | N |  | Hospitals and nursing homes Y 25 30 N N N Churches, auditoriums, and concert halls Y 25 30 N N N Governmental services Y Y 25 30 | N | N Transportation | Y Y Y(2) Y(3) Y(4) Y(4) | Parking Y Y Y(2) Y(3) Y(4) N Commercial Use |
| 公共用途N(1)住宅用途70NN N(1) N(1) N N 住宅用，不含活動房屋與暫時寄宿處 Y N(1) N(1) N N N 活動房屋公園 Y N N N N N 暫時寄宿處 Y 75 80 85 以上 Y Y(5) Y(5) N N N Outdoor music shells, amphitheaters Y N N N N N Nature exhibits and zoos Y Y N N N N Amusements, parks, resorts and camps Y Y Y N N N Golf courses, riding stables and water recreation Y Y 25 30 N N 表一 – 土地使用相容性*（使用年平均日夜聲音水準，單位：分貝） 65 65- 70- 75- 80- 85 土地使用用途 以下 Outdoor sports arenas and spectator sports Y Y Recreational Mining and fishing, resource production and extraction Y Y Y Y N Y(3) Y(4) N Offices, business and professional Y Y 25 30 N N Wholesale and retail--building materials, hardware and farm equipment | Retail trade--general Y |  | Y 25 30 | N N | Utilities Y Y Y(2) Y(3) Y(4) N Communication Y Y 25 30 N N | Manufacturing and Production Manufacturing, general Y Y Y(2) Y(3) Y(4) N Photographic and optical Y | Y 25 30 N |
| 學校N Y N(1) N(1) N N N 醫院與看護中心 Y 25 30 Y(8) Y(8) Livestock farming and breeding Y Y(6) Y(7) Agriculture (except livestock) and forestry Y Y(6) Y(7) Y(8) | N N N 教堂、會議聽（禮堂）與音樂廳 Y |  | 25 30 N | N |  |  |  |
| YN一般零售業N Y(2) Y(3) Y(4) N 公用事業 Y Y Y 25 30 N N 政府服務機關 Y Y | 25 30 N |  | N 運輸類 Y Y Y(2) | Y(3) Y(4) Y(4) 停車場 Y Y Y(2) Y(3) Y(4) N 商業用途 |  | 商務與專業辦公室 Y Y 25 | 30 N N |
| NNY戶外音樂廳與劇場Y(5)戶外運動競技場與現場圍觀之運動區通訊物料、零件與農業裝備之批發與零售業 30 N N Numbers in parentheses refer to notes. 括弧內之數字請參照註解 *The designations contained in this table do not constitute a Federal determination that any use of land covered by the program is acceptable or unacceptable under Federal, State, or local law. The responsibility for determining the acceptable and permissible land uses and the relationship between specific properties and specific noise contours rests with the local authorities. FAA determinations under Part 150 are not intended to substitute federally determined land uses for those determined to be appropriate by local authorities in response to locally determined needs and values in achieving noise compatible land uses. *本表中的指示並不構成聯邦之決議，以決定此類計畫所涵蓋之土地範圍內，依聯邦、州與當地法令為可接受（或不可接受）之土地使用；而是由當地有關當局負責決定可接受與允許之土地使用，以及特定財產與特定等噪音線關係。FAA依本號規則之決議亦非取代全國對土地使用之決議，該決議應由各地有關當局，依其達成噪音/土地相容性之重要性與需要而決定之。 Key to Table 1 表一之符號含意 * SLUCM = Standard Land Use Coding Manual. * SLUCM =土地使用規劃標準手冊 * Y (Yes) = Land Use and related structures compatible without restrictions. * Y (Yes) =土地使用與有關結構物相容，不必加以限制 * N (No) = Land Use and related structures are not compatible and should be prohibited. * N (No) =土地使用與有關結構物不相容，應加以禁止 * NLR = Noise Level Reduction (outdoor to indoor) to be achieved through incorporation of noise attenuation into the design and construction of the structure. * NLR =室外至室內之噪音降低水準，透過結構物的降音構造與設計來達成 *25, 30, or 35 = Land use and related structures generally compatible; measures to achieve NLR of 25, 30, or 35 dB must be incorporated into design and construction of structure. *25, 30, or 35 =透過結構物的降音構造與設計，衡量出室內與室外的噪音降低值(NLR)分別達25, 30, 35分貝，則土地使用與有關結構物具有相容性 Notes for Table 1 註解 (1) Where the community determines that residential or school uses must be allowed, measures to achieve outdoor to indoor Noise Level Reduction (NLR) of at least 25 dB and 30 dB should be incorporated into building codes and be considered in individual approvals. Normal residential construction can be expected to provide a NLR of 20 dB, thus, the reduction requirements are often stated as 5, 10 or 15 dB over standard construction and normally assume mechanical ventilation and closed windows year round. However, the use of NLR criteria will not eliminate outdoor noise problems. (1) 表示如被公眾決定可做為住宅及學校用地時，需考量符合建築法令與群眾個別允許，並使用室外至室內噪音降低值（NLR）至少達25與30分貝之措施；正常住宅用建築物預期可提供NLR值20分貝，所以要求其降低值應超出建築物原降低效果的5、10或15分貝，且正常假設其終年關閉窗戶並具備空調設備。無論如何，使用NLR為準則並非排除戶外噪音問題。 (2) Measures to achieve NLR 25 dB must be incorporated into the design and construction of portions of these buildings where the public is received, office areas, noise sensitive areas or where the normal noise level is low. (2) 表示辦公區域、噪音敏感地區或正常噪音水準較低處之建築物中較易接收噪音的部份，其設計與結構應與NLR值可達25分貝之措施一併考量。 (3) Measures to achieve NLR of 30 dB must be incorporated into the design and construction of portions of these buildings where the public is received, office areas, noise sensitive areas or where the normal noise level is low. (3) 表示辦公區域、噪音敏感地區或正常噪音水準較低處之建築物中較易接收噪音的部份，其設計與結構應與NLR值可達30分貝之措施一併考量。 (4) Measures to achieve NLR 35 dB must be incorporated into the design and construction of portions of these buildings where the public is received, office areas, noise sensitive areas or where the normal level is low. (4) 表示辦公區域、噪音敏感地區或正常噪音水準較低處之建築物中較易接收噪音的部份，其設計與結構應與NLR值可達35分貝之措施一併考量。 (5) Land use compatible provided special sound reinforcement systems are installed. (5) 表示藉由設置特殊的隔音系統，使該地區土地使用具相容性。 (6) Residential buildings require an NLR of 25. (6) 表示住宅用建築物需要NLR值25分貝。 (7) Residential buildings require an NLR of 30. (7) 表示住宅用建築物需要NLR值30分貝。 (8) Residential buildings not permitted. (8) 表示住宅用建築物需要NLR值35分貝。 Sec. A150.103 Use of computer prediction model. 第A150.103條 電腦預測模式之使用 (a) The airport operator shall acquire the aviation operations data necessary to develop noise exposure contours using an FAA approved methodology or computer program, such as the Integrated Noise Model (INM) for airports or the Heliport Noise Model (HNM) for heliports. In considering approval of a methodology or computer program, key factors include the demonstrated capability to produce the required output and the public availability of the program or methodology to provide interested parties the opportunity to substantiate the results. (a) 機場營運者需取得有關航空營運資料，以發展等噪音線（使用FAA核准的方法或電腦程式，例如：機場用的整合噪音模式(INM)或直昇機場用的直昇機場噪音模式(HNM)）；考量各種核准之方法或電腦程式時，主要因素為：具有提供所需輸出資料的能力，以及提供相關利益團體有能力檢查其輸出結果的有效性。 (b) Except as provided in paragraph (c) of this section, the following information must be obtained for input to the calculation of noise exposure contours: (b) 除本條第c項規定外，於計算等噪音線時須輸入以下資訊： (1) A map of the airport and its environs at an adequately detailed scale (not less than 1 inch to 8,000 feet) indicating runway length, alignments, landing thresholds, takeoff start-of-roll points, airport boundary, and flight tracks out to at least 30,000 feet from the end of each runway. (1) 一份機場與其周圍環境之地圖（應具適當比例，且不得少於1英吋比8000英呎），指示跑道長度與排列位置、降落進場處、起飛滾行點、機場週界、每一跑道至少3萬英呎外（從跑道終點起算）的飛航軌跡。 (2) Airport activity levels and operational data which will indicate, on an annual average-daily-basis, the number of aircraft, by type of aircraft, which utilize each flight track, in both the standard daytime (0700-2200 hours local) and nighttime (2200-0700 hours local) periods for both landings and takeoffs. (2) 機場年度的日常平均活動與營運資料，以顯示各種飛行器之數量、於日間（當地時間0700-2200）及夜間（當地時間2200-0700）起降、所使用之飛航軌跡。 (3) For landings--glide slopes, glide slope intercept altitudes, and other pertinent information needed to establish approach profiles along with the engine power levels needed to fly that approach profile. (3) 降落時，下降斜面之海拔截距高度，與建立進場縱剖面之有關資訊，如進場通過某點時之引擎推力水準。 (4) For takeoffs--the flight profile which is the relationship of altitude to distance from start-of-roll along with the engine power levels needed to fly that takeoff profile; these data must reflect the use of noise abatement departure procedures and, if applicable, the takeoff weight of the aircraft or some proxy for weight such as stage length. (4) 起飛時，飛行縱剖面資訊，為相對於滾行點之某高度、某距離時的引擎推力水準；上述資訊必須能反映「噪音防制起飛程序」之使用（如飛行器起飛重量或機身長度等）。 (5) Existing topographical or airspace restrictions which preclude the utilization of alternative flight tracks. (5) 因現存地形或空域之限制，而預先排除使用的替選飛航軌跡。 (6) The government furnished data depicting aircraft noise characteristics (if not already a part of the computer program's stored data bank). (6) 政府提供描述飛行器噪音特徵的資料（若電腦程式已附有該類資料庫則免之）。 (7) Airport elevation and average temperature. (7) 機場之海拔高度與平均溫度。 (c) For heliports, the map scale required by paragraph (b)(1) of this section shall not be less than 1 inch to 2,000 feet and shall indicate heliport boundaries, takeoff and landing pads, and typical flight tracks out to at least 4,000 feet horizontally from the landing pad. Where these flight tracks cannot be determined, obstructions or other limitations on flight tracks in and out of the heliport shall be identified within the map areas out to at least 4,000 feet horizontally from the landing pad. For static operation (hover), the helicopter type, the number of daily operations based on an annual average, and the duration in minutes of the hover operation shall be identified. The other information required in paragraph (b) shall be furnished in a form suitable for input to the HNM or other FAA approved methodology or computer program. (c) 依本條第b項第1款（Sec. A150.103(b)(1)）規定，直昇機場之地圖比例不得少於1英吋比2,000英呎，且需標示直昇機場週界、起降台與之飛航軌跡；於起降直昇機場之飛航軌跡中有無法決定、障礙或其他限制者，應於地圖中（起降台水平距離4,000英尺外之區域）確認之。此外，於靜態操作時（盤旋），直昇機之款式、年度平均之每日營運量、與持續盤旋操作之時間（以分鐘計）皆需確認之。其他有關第b項規定提供之資訊，應以適合HNM（或其他FAA核准的方法或電腦程式）輸入的格式提供之。 Sec. A150.105 Identification of public agencies and planning agencies. N 娛樂、公園、度假及露營 Y Y Y N N N 高爾夫球場、休閒馬場與水上休閒活動 Y Y 25 N Y Y N N N N N 自然風景區與動物園 N N N Y Y(5) Y Y 25 30 N N 製造與生產業用途 Y(2) Y(3) Y(4) Y Y |  |  | 一般製造業 | Y Y Y(2) Y(3) | Y(4) N 照相與光學業 Y Y 25 30 N N 農、林業（畜牧類除外） Y | Y(6) Y(7) Y(8) Y(8) Y(8) 畜牧類 Y | Y(6) Y(7) |
| 休閒娛樂用途N 第A150.105條 公家機關與規畫機構之確認 (a) The airport proprietor shall identify each public agency and planning agency whose jurisdiction or responsibility is either wholly or partially within the Ldn 65 dB boundary. (a) 機場營運者應該確認，於Ldn65分貝之等噪音線通過之區域，具有管轄權或應負責的公家機關與規畫機構。 (b) For those agencies identified in (a) that have land use planning and control authority, the supporting documentation shall identify their geographic areas of jurisdiction. (b) 於第a項中確認的相關機構，應於補充之書面文件中載明其具有管轄權（通常為土地使用之規畫與掌控的權力）之區域。 Part C--Mathematical Descriptions C部份 – 數學描述 Sec. A150.201 General. 第A150.201條 一般 The following mathematical descriptions provide the most precise definition of the yearly day-night average sound level (Ldn), the data necessary for its calculation, and the methods for computing it. 下列數學式子中，精確地定義年平均日夜聲音水準（以下簡寫為Ldn），以及所需計算的資料、計算之方法。 N N 礦業、漁業、資源生產與萃取 Y Y Y Y Y Y | Sec. A150.203 Symbols. 第A150.203條 符號 The following symbols are used in the computation of Ldn; 計算Ldn時，將用到以下符號： Symbol「符號」 Measure (in dB)「量度為分貝」 LT Average Sound Level, During Time T 時段T之平均聲音水準 Ldni Day-Night Average Sound Level (individual day) 日夜平均聲音水準（單天） Ldn Yearly Day-Night Average Sound Level 年平均日夜聲音水準 LAE Sound Exposure Level 聲音曝露水準 Sec. A150.205 Mathematical computations. 第A150.205條 數學計算 (a) Average sound level must be computed in accordance with the following formula: (a) 平均聲壓水準需以公式（1）計算之： |  | µ § (1) where T is the length of the time period, in seconds, during which the average is taken; LA(t) is the instantaneous time varying A-weighted sound level during the time period T. T為測量時間長度，單位：秒﹔LA(t) 表T時間內A加權聲壓水準變化之平均值 (1) Note: When a noise environment is caused by a number of identifiable noise events, such as aircraft flyovers, average sound level may be conveniently calculated from the sound exposure levels of the individual events occurring within a time period T: [ 註1]：當噪音環境為可確認之噪音事件所引起時，如飛行器的飛越，則平均聲壓水準可以較方便的方式計算之，如計算時間T內單一事件之聲音曝露水準： µ § (2) where LAEi is the sound exposure level of the i-th event, in a series of n events in time period T, in seconds. LAEi為T時間內（單位：秒），n個噪音事件序列中，第i個事件之聲音曝露水準。 (2) Note: When T is one hour, LT is referred to as one-hour average sound level. [ 註2]：時間T若為一小時，LT則表為單位小時聲音水準。 (b)Day-night average sound level (individual day) must be computed in accordance with the following formula: (b) 日夜平均聲音水準（個別一天）需以公式（3）計算之： µ § (3) Time is in seconds, so the limits shown in hours and minutes are actually interpreted in seconds. It is often convenient to compute day-night average sound level from the one-hour average sound levels obtained during successive hours. 因單位為秒，故上式中之時間需轉換為秒數﹔通常為方便計算日夜平均聲音水準，常以單位小時平均聲音水準依序計算，再累加得之 (c)Yearly day-night average sound level must be computed in accordance with the following formula: (c) 年平均日夜聲音水準需以公式（4）計算之： µ § (4) where Ldni is the day-night average sound level for the i-th day out of one year. Ldni為一年中第i天之日夜平均聲音水準。 |  |  |  | (d)Sound exposure level must be computed in accordance with the following formula: (d) 聲音曝露水準需以公式（5）計算之： µ § (5) where to is one second and LA(t) is the time-varying A-weighted sound level in the time interval t1 to t2. to表單位秒，而LA(t)為時點t1至時點t2間，隨時間變動之A加權聲音水準。 The time interval should be sufficiently large that it encompasses all the significant sound of a designated event. 時段長度需足夠長，以包含一噪音事件中所有顯著的聲音。 The requisite integral may be approximated with sufficient accuracy by integrating LA(t) over the time interval during which LA(t) lies within 10 decibels of its maximum value, before and after the maximum occurs. 必要時，某些積分值可用LA(t)之積分值來逼近其近似值，並具有充分精確性，但須在LA(t)最大值發生前後，LA(t)變化在十分貝內的時段間積分。 Appendix B to Part 150--Noise Compatibility Programs 附件B – 噪音相容性計畫 B150.1 Scope and purpose. 第B150.1條 範圍與目的 B150.3 Requirement for noise map. 第B150.3條 噪音地圖之必要條件 B150.5 Program standards. 第B150.5條 計畫標準 B150.7 Analysis of program alternatives. 第B150.7條 計畫替選方案分析 B150.9 Equivalent programs. 第B150.9條 等效計畫 Sec. B150.1 Scope and purpose. 第B150.1條 範圍與目的 (a) This appendix prescribes the content and the methods for developing noise compatibility programs authorized under this part. Each program must set forth the measures which the airport operator (or other person or agency responsible) has taken, or proposes to take, for the reduction of existing noncompatible land uses and the prevention of the introduction of additional nonco |

mp

| atible land uses within the area covered by the noise exposure map submitted by the operator. (a) 依本號規則授權此附件規定，發展噪音相容性計畫之內容與方法。每一計畫需宣佈機場營運者（或其他負責之個人、機構）執行或預計執行之措施，該措施為此營運者提出之噪音曝露量地圖涵蓋地區內，為降低目前與避免引進新的不相容性土地使用之措施。 (b) The purpose of a noise compatibility program is: (b) 噪音相容性計畫之目的為： (1) To promote a planning process through which the airport operator can examine and analyze the noise impact created by the operation of an airport, as well as the costs and benefits associated with various alternative noise reduction techniques, and the responsible impacted land use control jurisdictions can examine existing and forecast areas |  |
| --- | --- |

of

| noncompatibility and consider actions to reduce noncompatible uses. (1) 透過以下二者推動此規畫過程；一為經由機場營運者，調查與分析機場運作所產生之噪音衝擊，以及各種噪音降低方法所具備之成本與利益；並透過管轄受噪音衝擊地區之單位，調查現存和預測之不相性地區，與考慮降低不相容性使用之決議。 (2) To bring together through public participation, agency coordination, and overall cooperation, all interested parties with their respective authorities and obligations, thereby facilitating the creation of an agreed upon noise abatement plan especially suited to the individual airport location while at the same time not unduly affecting the national air transportation system. (2) 經由公眾的參與、各機關的協調與合作，以及所有牽涉之利益團體（因其權利與義務），藉此促其達成對噪音防制計畫的共識，而特別符合個別機場所受用，且不過當影響全國空運系統。 (3) To develop comprehensive and implementable noise reduction techniques and land use controls which, to the maximum extent feasible, will confine severe aircraft YDNL values of Ldn 75 dB or greater to areas included within the airport boundary and will establish and maintain compatible land uses in the areas affected by noise between the Ldn 65 and 75 dB contours. (3) 發展全面性且可行之噪音降低技術與控制土地使用，在最大可行範圍內，嚴格限制年平均日夜聲音水準值(YDNL)75分貝及以上之區域（包含機場邊界內），並建立與維護噪音水準介於65分貝與75分貝間區域之土地相容性。 Sec. B150.3 Requirement for noise map. 第B150.3條 噪音地圖之必要條件 (a) It is required that a current and complete noise exposure map and its supporting documentation as found in compliance with the applicable requirements by the FAA, per Sec. 150.21(c) be included in each noise compatibility program: |  | (a) 每一噪音相容性計畫中，皆需要最近且完整之噪音曝露量地圖以及補充之相關文件（需遵守FAA之相關規定，且符合第150.21條第c項之規定）用以確認： (1) To identify existing and future noncompatible land uses, based on airport operation and off-airport land uses, which have generated the need to develop a program. (1) 基於機場運作與機場外土地使用，產生現存及未來之不相容性土地使用情形，因而需要發展一噪音相容性計畫。 (2) To identify changes in noncompatible uses to be derived from proposed program measures. (2) 因計畫中提議之措施而改變的不相容性使用。 (b) If the proposed noise compatibility program would yield maps differing from those previously submitted to FAA, the program shall be accompanied by appropriately revised maps. Such revisions must be prepared in accordance with the requirements of Sec. A150.101(e) of Appendix A and will be accepted by FAA in accordance with Sec. 150.35(f). (b) 若提議之噪音相容性計畫超出原先向FAA提出之噪音曝露量地圖範圍，該計畫應再附上適當的修訂地圖。此修訂版本應依附件A之第A150.101條第e項之規定準備之，且依第150.35條第f項規定為FAA所接受。 Sec. B150.5 Program standards. 第B150.5條 計畫標準 Based upon the airport noise exposure and noncompatible land uses identified in the map, the airport operator shall evaluate the several alternative noise control actions and develop a noise compatibility program which — 基於地圖上標示之機場噪音曝露量與不相容性土地使用，機場營運者應評估數種噪音控制方案與發展噪音相容性計畫，用以 (a) Reduces existing noncompatible uses and prevents or reduces the probability of the establishment of additional noncompatible uses; (a) 降低現存不相容性使用，與避免引入新的不相容性使用； (b) Does not impose undue burden on interstate and foreign commerce; (b) 不強加不當負擔於州際、國外商務上﹔ (c) Provides for revision in accordance with Sec. 150.23 of this part. (c) 依本號規則第150.23條規定提出修訂計畫﹔ (d) Is not unjustly discriminatory. (d) 不造成不公之差別待遇﹔ (e) Does not derogate safety or adversely affect the safe and efficient use of airspace. (e) 不減損飛安或負面影響空域使用之效率﹔ (f) To the extent practicable, meets both local needs and needs of the national air transportation system, considering tradeoffs between economic benefits derived from the airport and the noise impact. (f) 在可行範圍內，符合當地與全國空運系統之需要，並兼顧機場經濟效益與噪音衝擊之抵換關係﹔與 (g) Can be implemented in a manner consistent with all of the powers and duties of the Administrator of FAA. (g) 可以在FAA署長所有權力與賦予之義務下執行。 Sec. B150.7 Analysis of program alternatives. 第B150.7條 計畫替選方案分析 (a) Noise control alternatives must be considered and presented according to the following categories: (a) 噪音控制方案必須依據下列各範疇來考慮與表示： (1) Noise abatement alternatives for which the airport operator has adequate implementation authority. (1) 機場營運者需有適當權力以執行噪音防制方案。 (2) Noise abatement alternatives for which the requisite implementation authority is vested in a local agency or political subdivision governing body, or a state agency or political subdivision governing body. (2) 噪音防制方案所需有必要的執行權力，可由當地或州屬機關、部門授權之。 (3) Noise abatement options for which requisite authority is vested in the FAA or other Federal agency. (3) 噪音防制之選擇權由FAA授權或其他聯邦機關授權。 (b) At a minimum, the operator shall analyze and report on the following alternatives, subject to the constraint |  |  |  |
| --- | --- | --- | --- | --- | --- |

s that the strategies are appropriate to the specific airport (for example, an evaluation of night curfews is not appropriate if there are no night flights and none are forecast):

(b) 受限於特定機場適用之策略（例如對一個目前沒有且預測亦無夜間班次之機場，評估該機場宵禁之影響為不適當的），機場營運者最少應分析與報告下列替選方案：

(1) Acquisition of land and interests therein, including, but not limited to air rights, easements, and development rights, to ensure the use of property for purposes which are compatible with airport operations.

(1) 為確保於其中取得之土地與權益（但不限於飛越權、地役權與發展權）的使用用途，能與機場營運之相容性目標一致。

(2) The construction of barriers and acoustical shielding, including the soundproofing of public buildings.

(2) 隔音設備的構建，例如公眾建築之隔音設備。

(3) The implementation of a preferential runway system.

(3) 執行最適跑道系統。

(4) The use of flight procedures (including the modifications of flight tracks) to control the operation of aircraft to reduce exposure of individuals (or specific noise sensitive areas) to noise in the area around the airport.

(4) 使用飛航程序（含修正航跡）控制飛行器操作，以降低各機場鄰近區域（或特定之噪音敏感地區）對噪音之曝露量。

(5) The implementation of any restriction on the use of airport by any type or class of aircraft based on the noise characteristics of those aircraft. Such restrictions may include, but are not limited to--

(5) 依飛行器發出噪音分類或分級，而限制其使用機場之措施，可包含以下（但並不限於以下）之措施：

(i) Denial of use of the airport to aircraft types or classes which do not meet Federal noise standards;

(i) 拒絕不符合聯邦標準之飛行器使用該機場；

(ii) Capacity limitations based on the relative noisiness of different types of aircraft;

(ii) 基於不同噪音特性分類之各類型飛行器之容量限制；

(iii) Requirement that aircraft using the airport must use noise abatement takeoff or approach procedures previously approved as safe by the FAA;

(iii) 要求飛行器起降該機場時，使用經FAA飛安考量核准的噪音防制進、離場程序；

(iv) Landing fees based on FAA certificated or estimated noise emission levels or on time of arrival; and

(iv) 基於FAA認可、或依噪音排放水準、或抵達時刻收取降落費；

(v) Partial or complete curfews.

(v) 部份或完全宵禁；

(6) Other actions or combinations of actions which would have a beneficial noise control or abatement impact on the public.

(6) 其他的決議或決議之組合，其組合將有公眾受惠於此防制或控制噪音衝擊的決議。

(7) Other actions recommended for analysis by the FAA for the specific airport.

(7) 其他為FAA為特定機場所建議分析之個案。

(c) For those alternatives selected for implementation, the program must identify the agency or agencies responsible for such implementation, whether those agencies have agreed to the implementation, and the approximate schedule agreed upon.

(c) 為執行計畫中所選擇之方案，該計畫必須確認負責執行之機關是否同意執行，並配合其時間表。

Sec. B150.9  Equivalent programs.

第B150.9條  等效計畫

(a) Notwithstanding any other provision of this part, noise compatibility programs prepared in connection with studies which were either Federally funded or Federally approved and commenced before October 1, 1981, are not required to be modified to contain the following items:

(a) 噪音相容性計畫在進行與聯邦補助、聯邦核准有關之研究，且於1981年10月1日前著手者，儘管本號規則之任何其他規定，可不需修正而包含下列項目：

(1) Flight tracks.

(1) 航跡

(2) A noise contour of Ldn 70 dB resulting from aircraft operations and data related to the Ldn 70 dB contour. When determinations on land use compatibility using Table 1 of Appendix A differ between Ldn 65-70 dB and Ldn 70-75 dB, the determinations should either use the more conservative Ldn 70- 75 dB column or reflect determinations based on local needs and values.

(2) 年平均日夜聲音水準( Ldn )70分貝之等噪音線（為飛行器操作結果且資料對應於此等噪音線），於使用附件A表1以決定土地使用相容性時，此決定應使用較保守之Ldn為 70 -75分貝的那一列，或反應當地需要與重要性而決定之。

(3) The categorization of alternatives pursuant to Sec. B150.7(a), although the persons responsible for implementation of each measure in the program must still be identified in accordance with Sec. 150.23(e)(8).

(3) 雖然負責執行該計畫每一措施的人，可依據第B150.7條第a項之替代方案分類，但仍應依照第150.23條第e項第8款之規定確認之。

(4) Use of ambient noise to determine land use compatibility.

(4) 使用背景噪音以決定土地使用之相容性。

(b) Previously prepared noise compatibility program documentation may be supplemented to include these and other program requirements which have not been excepted.

(b) 原先準備的噪音相容性計畫之文件可能需要增補，用以包含該計畫（或其他計畫）尚未被排除在外的請求。