論文名稱：應用類神經網路構建視覺化夜間路口車輛偵測模式    頁數：147

之研究

校系（所）組別：淡江大學    運輸管理學系運輸科學碩士班

畢業時間及提要別：九十二學年度第二學期碩士學位論文摘要

研究生：賴文復              	指導教授：范俊海

論文摘要內容：

由於影像式偵測器能夠獲得許多傳統偵測器所無法收集之交通參數，且影像式偵測器具有成本低廉、裝設與維修容易等優點，故利用影像處理的方式析出所需之交通參數，是更有效率、更符成本的趨勢。鑒於夜間環境照明度不足、光源不穩定，及路口車流行為複雜等因素，先前的相關研究大都以白天且晴朗的路段作為研究的對象，對於夜間或是路口的環境鮮少有探討，因此，本研究將以夜間路口作為研究的環境，以期構建一適用於夜間路口車輛偵測的模式。

夜間偵測的成敗取決於特徵的切割，以夜間車輛為例，其最明顯的車輛特徵不外乎是車燈、車體反光與地面反光等亮塊，然而，由於路口車輛行為相較於路段車輛行為而言更為複雜，因此，如何在夜間路口的複雜環境中精確切割出特徵，是本研究所要克服的第一個難題，對此本研究提出以倒傳遞網路（Back-Propagation Network）架構為基礎所發展的類神經切割法（Neural Segmentation Method），用以獲得夜間路口車輛特徵，其切割結果顯示優於過去以經驗法則、統計方法等為主的切割法；此外，夜間光源的不穩定性與路口車輛轉向行為，皆會造成車輛特徵的切割隨著時間軸而變化，因此，如何將屬於同一輛車之車輛特徵予以群聚，是本研究所面臨的第二個難題，對此本研究利用模糊系統（Fuzzy System）的架構發展一模糊亮塊群聚法（Fuzzy Block Clustering Method），依亮塊之間相似的程度進行群聚的工作，其研究結果顯示在處理低密度交通流量時有不錯的效果。

本研究所析出之交通參數包括車輛種類與轉向交通流量，經過實例驗證之後，大型車、小型車與機車之辨識正確率，分別為58.6 %、85.5 %及84.5 %；在轉向交通流量偵測正確率方面，南北時相之偵測正確率為82.5 %、東西時相之偵測正確率為83.9 %。

關鍵字：夜間車輛偵測、類神經網路、模糊系統、車輛種類、轉向交通流量

Title of Thesis：                                      Total pages：147

Developing Visual Intersection Vehicle Detection Model at

Night Applied on Neural Network

Key words：Night Vehicle Detection、Neural Network、Fuzzy System、Vehicle Classification、Turning Traffic Flow

Name of Institute：

Graduate Institute of Transportation Science, Tamkang University

Graduate data：Jun 2004             Degree conferred：Master Degree

Name of student：Wen-Fu Lai             Advisor：Dr. Chun-Hai Fan

Abstract：

Because of video image detector can obtain more traffic parameters than traditional ones, and provided with many advantages, including cheap cost, easy installation and quick maintenance etc. So it is suitable and efficient to extract traffic parameters of image processing. Due to insufficient illumination and instable illuminant at night, previous researches almost focus on daytime and cloudless weather. It is few researches study on the environment of night or intersection. Therefore, our research will study the circumstances of intersection at night.

The feature segmentation is a critical technique to decide that night detection success or failure. For example, when vehicle was drove at night, the most obvious features included lamp, the light reflection of car and the light reflection of ground etc. However, owing to the vehicle behavior in intersection are more complicated than in road. Hence, how to segment features accurately from complicated intersection environment at night, which is the first arduous problem we must to overcome. In this problem, we developed a neural segmentation method based on back-propagation. Moreover obtain vehicle features by neural segmentation method, which result manifest better performance than heuristics rules and statistics method.

Besides, the instability of illuminant at night and the turning behavior of car in intersection are caused the variation of vehicle’s features. So, the second difficult problem is how to cluster the features with the same vehicle. In order to solve this one, we adopted fuzzy system to build the fuzzy block clustering method integrating the most part of features. Performing these clustered works by similarity between blocks, and the result manifest okay performance when deal with low- density traffic flow.

In our study, we successfully extract the traffic parameters are included vehicle classification and turning traffic flow. After our experimental analysis, the accurate rate of identifying big vehicle is 58.6 %, small vehicle is 85.5 %, motorcycle is 84.5 %；The accurate rate of detecting transferred traffic flow, in south-north phase is 82.5 %, and in east- west is 83.9 %.