Title of Thesis:	Total Pages: 109

Travel Time Estimation with Transfer Function Technique: A Simulation Study

Keywords: Travel time estimation, Paramics, algorithm of section density ,Transfer Function Model,

Name of Institute:

Graduate Institute of Transportation Science, Tamkang University

Graduate Date: June 2004	Degree Conferred: Master Degree

Name of Student: Ya-Hui HSU                Advisor: Dr. Chee-Chung Tong

許雅惠                             董啟崇 博士

Abstract:

Dual-loop vehicle detectors are most commonly used devices in this country to perform instantaneous (real time) traffic data collection for either traffic management or travel information.  In particular, travel time information is of the major importance which can not be obtained directly but derived from various models/algorithms using detected traffic flow related data as input.  Among these travel time estimation models/algorithms, traffic flow based algorithms were particular attractive due to their robustness in accordance to theory rather than some “black boxes” approaches.

One particular algorithm of interested was the recently derived by Oh, Jayakrishnan and Recker in 2002 with the concept of section density seems appealing due to its simplicity and theoretical clearness in presentation.  In addition, transfer function methods with the capability to calibrate speed-density relations using time series data was recognized to capture the effect of dynamic nature real time data instead of the commonly used static or steady-state speed-density relation.  It is therefore the focus of this thesis to develop an integrated procedure of the two in the application of estimation of travel time along a specific road segment.

This study accessed Paramics, a microscopic traffic simulation program, as primary research tool of data sources to evaluate the proposed issues.  It is because the vehicle detection system is under major overhaul for some sustained period hence reliable and continuous true data would not be available in the study area.  The Paramics was carefully calibrated utilizing its feature for detailed road section geometric configuration and driving behavior in the selected freeway section.

The estimation results using the proposed hybrid method (with transfer function) were evaluated in contrast to a base model with steady-state speed-density relation under two VD (vehicle detector) deployment scenarios, one with original deployment and another with incremental deployment.  The results show that the hybrid model did not always perform better than the base model in all cases especially under the incremental deployment scenario.  The reason may be identified to the poor synergy of the nature of the section density model using the deviation of flow rates and the relatively high fluctuation of simulated flow pattern in short freeway sections within relative short and varying time interval by Paramics.