The Study of Real-Time Bus Dispatching Algorithm

Student：Chia-Seng Lin						Advisor：Jin-Yuan Wang

Department of Transportation Technology and Management

National Chiao Tung University

Abstract

In order to improve the quality of service and insure the performance of operations, the most important thing in bus dispatching is how to maintain established schedule. In reality, the bus company must readjust the schedule under abnormal circumstances such as traffic jam and mechanical failures.

Readjusting the schedule may inference the whole schedule. Inappropriate adjustment will affect the performance of operation and the quality of service, so how to dispatch bus effectively becomes an important task.

The purpose of this study is to adapt a bus dispatching algorithm to improve traditional methods. A column generation approach is proposed to generate available duties. For the vehicle scheduling network, node-to-node cost matrix is calculated. Arc cost is defined as sum of node cost, dual price, time distance, delay cost, and route changed cost. We develop a Multi-Labels Correcting Method to solve the shortest path problem with resource constraints. Each path is an available duty.

In order to evaluate the practicability of the algorithm, we test how the real-time bus dispatching algorithm apply in the real world. Second, we establish different situations of schedule. Apply the solution algorithm to solve the problem. Finally, we perform sensitivity analyses by adjusting parameters on the model.

Keywords: Real-Time Dispatching, Column Generation, Multi-Labels Correcting Method.