Content

List of Figures	III

List of Tables	III

Chapter1 Introduction	1

1.1 Motivation	1

1.2 Objective	1

1.3 Scope	1

1.4 Study Flowchart	2

Chapter2 Current Operation Procedures and Literature Review	4

2.1 Current Operation Procedures	4

2.2 Literature Review	5

2.2.1 Exact Solution Approach	5

2.2.2 Heuristic Approach	6

2.2.3 AI Approach	8

2.3 Summary	10

Chapter3 Real-Time Bus Dispatching Algorithm	11

3.1 Solution Algorithm Flowchart	11

3.2 Initial Variables	13

3.3 The Master Problem	13

3.3.1 The Model of The Master Problem	13

3.3.2 The Constraint of Bus Numbers	14

3.4 The Sub-Problem	15

3.4.1 The Vehicle Scheduling Network	15

3.4.2 The Constrained Shortest Path Problem	16

3.4.3 The Sub-Problem Solution	19

Chapter4 Computational Experiments	22

4.1 Solutions For Real-Time Dispatching Problem	22

4.1.1 The maximum delay time is 10 minutes	23

4.1.2 The maximum delay time is 20 minutes	24

4.2 Different Situations of schedule	25

4.2.1 The Number of Routes	27

4.2.2 The Headway	28

4.2.3 The Round-Trip Time of Route	29

4.3 Adjustable Parameters Experiments	30

4.3.1 The Time Allowed For Drivers To Have Meal	31

4.3.2 The Meal Time	32

4.3.3 The Penalty Weight of Delay	32

4.3.4 The Penalty Weight of Different Route	33

4.4 Summary	34

Chapter5 Conclusions and Suggestions	36

5.1 Conclusions	36

5.2 Suggestions	37

Reference	38

Appendix A—The schedule with 5 routes, short headway, and long round-trip time	43

Appendix B—The schedule with 5 routes, short headway, and short round-trip time	44

Appendix C—The schedule with 5 routes, long headway, and long round-trip time	45

Appendix D—The schedule with 5 routes, long headway, and short round-trip time	45

Appendix E—The schedule with 8 routes, short headway, and long round-trip time	46

Appendix F—The schedule with 8 routes, short headway, and short round-trip time	47

Appendix G—The schedule with 8 routes, long headway, and long round-trip time	48

Appendix H—The schedule with 8 routes, long headway, and short round-trip time	49

Appendix I—The test schedule in adjustable parameters experiments	50

# List of Figures

Figure 2-1 Dispatching traditionally	4

Figure 3-1 Solution algorithm flowchart	12

Figure 3-2: Example of a vehicle scheduling network	16

Figure 3-3 Examples of different types of arcs	19

Figure 3-4 Multi-Labels Correcting Method flowchart	21

Figure 4-1 The schedule of real-time dispatching problem	22

Figure 4-2 The solution result of 10 minutes delay time allowed	23

Figure 4-3 The solution result of 20 minutes delay time allowed	24

Figure 4-4 The schedule after the perturbation be solved	25

# List of Tables

Table 3-1 Properties of the meal node	16

Table 3-2 Examples of cost of arcs	19

Table 4-1 The test results of different situations of schedule	26

Table 4-2 The test result of different time allowed for drivers to have meal	31

Table 4-3 The test result of different meal time	32

Table 4-4 The test result of different penalty weight of delay	33

Table 4-5 The test result of different penalty weight of different route	34

# Chapter1 Introduction

## 1.1 Motivation

A public transit company is often in a situation where a bus could not follow its original schedule due to traffic conditions. Thus, the company is forced to fulfil the schedule via stand by buses. Clearly, when a stand by bus is not available, disturbances arise.

Bus shortages occasionally occur during daily operation due to factors such as traffic jam and mechanical failures. Bus dispatchers need to respond to such shortages on a real-time basis by delaying or canceling tasks, swapping bus among different routes, or requesting the usage of stand by bus. The choices undertaken aim at minimizing the losses incurred while retaining an operable schedule.

Bus dispatching is aimed at efficiently assigning bus to real-time service requests. Unfortunately, it is not easy to find good and experienced dispatchers. Moreover, it takes a long time to train dispatchers. Hence, the development of a computer-aided bus dispatching system to assist human dispatchers would certainly be very useful.

## 1.2 Objective

The objective of this research is to develop a bus real-time dispatching model and solution algorithm. Based on the special structure of the model, an effective algorithm is presented, sufficient computational experiments are conducted and some results are reported.

## 1.3 Scope

In this research we focus mainly on the public transit companies that have daily pre-established schedule. If the public transit company has no daily pre-established schedule, we should establish the schedule based on the scheduling rule of the company.

## 1.4 Study Flowchart

Figure 1-1 Study Flowchart

First, we define the problem. After realizing the problem we encounter, we will review relative literatures, and capture the knowledge of dispatchers via observation and interview. We can understand the current research progress in this area.

We formulate this real-time dispatching problem as a linear programming model, which minimize the total cost. Thereafter, a solution algorithm is proposed to solve the model.

Finally by analyzing the output, we will keep modifying our model until the result satisfied the actual requirement.

# Chapter2 Current Operation Procedures and Literature Review

## 2.1 Current Operation Procedures

Figure 2-1 is the current operation procedure. We derive this figure by interviewing employees of various public transit companies.

Figure 2-1 Dispatching traditionally

When the daily operation begins, the established schedule is honored if there is no incident happened. Unfortunately the bus dispatcher will encounter unpredictable bus shortages due to some traffic conditions. Bus dispatchers need to respond to such shortages on a real-time basis by delaying or canceling tasks, swapping bus among different tasks, or requesting the usage of backup buses. The choices undertake aim at finding suitable way to maintain the schedule.

Under the current operation, the dispatchers make the decision manually base on their own individual experience. This is not an effective way because of the lack of skilled dispatchers. Moreover, it takes time to train such dispatchers. Hence, this study will develop a real-time bus dispatching algorithm to assist human dispatchers.

## 2.2 Literature Review

In this section we review the relative literature. We categorize the dispatching problems into three classifications according to solution techniques. There are exact solution approach, heuristic approach, and AI approach.

### 2.2.1 Exact Solution Approach

Christian and Kunt（1999）[5] presented a model for the vehicle and crew scheduling problem in urban public transport systems by combining models for vehicle and crew scheduling that cover a great variety of real world aspects. The main part of the model consists of a set partitioning formulation to cover each trip. A column generation algorithm is implemented to calculate the continuous relaxation which is embedded in a branch and bound approach to generate an exact solution for the problem.

Teodorovic and Guberinic（1984）[43] discussed the problem of minimizing overall passenger delays in the eventuality of a schedule perturbation. They attempt to find the least expensive set of aircraft routings using a branch and bound procedure.

### 2.2.2 Heuristic Approach

Martin and Francois（1989）[35] proposed a column generation approach to solve the transit crew scheduling problem. The set covering problem chooses a schedule from already known feasible workdays. The second sub-problem is a shortest path problem with resource constraints and is used to propose new feasible workdays to improve the current solution of the set covering problem.

Yang（1994）[12] developed a framework to help carriers in handling schedule perturbation caused by technical incidents of aircraft for the operations of single fleet and non-stop flights. The framework is based on a basic schedule perturbation model constructed as a dynamic network from which two perturbed network models are developed to study the impacts of incidents on scheduling. These network models are formulated as pure network flow problems and are suggested to solve using the network simplex method. A flow decomposition algorithm is then applied to trace each airplane route in the schedule perturbed period.

Luo（1996）[31] developed a framework for carriers to handle schedule perturbations resulting from expected maintenance of aircrafts for the operation of single fleet. The framework is based on a basic model constructed as a time-space network from which strategic models are developed to assist carriers in fleet routing and flight scheduling effectively. The models are formulated as a pure network flow problem or a network flow problem with side constraints. The former is solved using the network simplex method and the latter is solved using a Lagrangian relaxation-based algorithm.

Lu（1996）[6] developed a framework of aircraft dispatching strategy for irregular flight incidents. Following that, a procedure to find and evaluate feasible aircraft dispatching alternatives is proposed. Two mathematical programming models and corresponding heuristic method were introduced to solve this problem.

Gelman（1997）[15] posed the crew pairing problem as a set partitioning 0-1 integer program. Variables are generated as legal pairings meting all work rules. Dual values obtained from solving successive large linear program relaxations are used to prune the search tree. A graph based branching heuristic presented applied to a restricted set partitioning problem representing a collection of ‘best’ pairings. The algorithm exploits the natural integer properties of the crew pairing problem.

Cao and Kanafani（1997）[29] presented a quadratic 0-1 programming model for the integrated decision problem of flight schedule and aircraft assignment. The model is extended to formulate some special cases such as the ferrying of surplus aircraft and the replacement of different type of aircraft. An effective heuristic algorithm is presented.

Liu（1998）[16] developed a column generation based real-time bus dispatching and rescheduling system. Real-time bus location can be obtained by GPS signal and expected return time is estimated. When perturbation occurred, system will active bus rescheduling module automatically and provides dispatching suggestions. Rescheduling module advised bus from other route to cover this perturbation task. This system guarantee the schedule will return to its original one after the user setting the system recovery time.

Mohamadreza（2000）[37] presented a procedure for solving real-world large-scale multiple depot vehicle scheduling（MDVS）problems considering the route time constraints（RTCs）. Formulation of the MDVS problem, the set of constraints for considering the time restriction, and a heuristic procedure for solving the MDVS problems with RTCs are discussed.

Lu（2000）[7] proposed an algorithm to improve the efficiency for solving the drivers scheduling problem of the Taipei MRT company. Lu formulated the problem as a set-partitioning problem. A three phases algorithm, column generation, branch-and-bound, and heuristic local search, is then developed for solving this model. This study used the real case of the Taipei MRT company as the numerical example. The results show that algorithm could get better solutions with higher operational feasibility than the current one does.

Christos and Efthymios（2002）[4] presented a quick heuristic scheduling procedure named QS（Quick Shift）for the solution of the problem. The main algorithms used by QS are minimum cost matching, set partitioning and shortest path. In addition, a column generation procedure named CGQS that uses an LP-solver and the QS process as its integer solution finder is presented.

Chang（2002）[28] developed a model and two scheduling networks to help minimize cockpit crew costs and to plan for proper cockpit crew pairings. The model was formulated as a set-partitioning problem. A column generation approach was proposed to efficiently solve the problem.

### 2.2.3 AI Approach

Smith（1990）[32] developed a computerized system for assigning, parking, and dispatching buses so that each operator in the St. Louis metropolitan transit system generally drives the same bus each day. Combinations of optimizing routines, mathematical heuristics, decision rules from experienced personnel, and human intervention are used to manage the bus operations efficiently.

Shieh（1994）[30] developed a model in which the objective was set to maximize operator’s net benefit, subject to set constraints of adequate service quality. In addition, this study combined the use of an expert system for bus operations, where uncertainties are considered in conjunction with the optimization model.

Potvin（1995）[27] proposed learning by examples as a means to uncover the human expertise in this domain. Two techniques based on linear programming and neural network models are investigated and compared. Experimental results are reported on real data from a courier service company.

Wu（1997）[44] developed a bus scheduling expert system to reschedule different bus routes under abnormal circumstances. This system included a bus scheduling simulation model and a knowledge base. The simulation model was used to generate normal schedule with the objective of minimizing total social cost. To deal with abnormal circumstances of bus operations, the best alternative of mutual support can be chosen from the base for rescheduling.

Cheng（1998）[26] developed a fuzzy expert system including three modules to solve the above-mentioned problems from the viewpoint of transportation demand and supply. First, in knowledge module of extra board（frequency）, they estimate the effected line frequency for uncertain situations. Second, they calculate aggregate membership function of dispatched drivers in supply module. Finally in the feasible alternative’s module, they synthesize information of the above two modules to determine the ranking of feasible alternatives.

James（2000）[25] presented findings on initial changes in service reliability following Tri-Met’s deployment of a new bus dispatching system using automatic vehicle location and automatic passenger counter technology. Changes in on-time performance, headway variation, run time variation, and run times were determined with respect to predeployment levels. Changes in headway variation and run times were also used to estimate the initial benefits of the new system with respect to operating costs, passenger waiting, and passenger travel time.

Yuan（2001）[17] had adapted an expert system like approach to improve traditional methods. They obtain the knowledge of dispatching rules via an interview with bus companies and combine these rules with inferential engine, users can obtain dispatching strategies through the user interface of the system. A case study concerning the operation of bus companies in Taiwan is performed.

## 2.3 Summary

The disadvantage of exact solution approach is the efficiency of solving problem. When the scale of problem is large, it takes a long time to find the exact solution. Therefore the exact solution approach is not suitable to practical purpose.

One important work of the AI approach is to obtain knowledge of rules via interviews. However, The decision rules of each company differ a lot. Therefore it is not easy to find a general rules to satisfy every bus companies.

Hence, we focus on developing a heuristic approach to solve the real-time bus dispatching problem. Considering that there could be an enormous number of variables in our case, we adopt a column generation approach to solve the real-time bus dispatching problem.

The column generation approach in our model would decompose the problem into two parts: master problem and sub-problem. The master problem determines a schedule from part of the all tasks that are generated by sub-problem. The sub-problem is used to generate new feasible variables to improve the current solution of the master problem.

# Chapter3 Real-Time Bus Dispatching Algorithm

In this chapter we propose a column generation and branch-and-bound based algorithm to solve the real-time bus dispatching problem. Lower bounds are calculated by dropping the integral constraints. The relaxation problem is solved via column generation method. The detail of this algorithm is explained in the following sections.

## 3.1 Solution Algorithm Flowchart

The flowchart of solution algorithm is shown in Figure 3-1. The steps of the algorithm are listed as follows:

Step 1: We generate enough initial columns (variables) to guarantee the initial feasible solution is available.

Step 2: We construct the real-time bus dispatching problem as the set partitioning linear programming model. The master problem is a linear-relaxed version of the original problem with limited columns.

Step 3: The linear programming relaxation of this set partitioning problem is solved using the simplex algorithm.

Step 4: To price out the best columns not in the master problem, we construct the sub-problem as a constrained shortest path problem. In addition to the cost, the constrained shortest path problem also concerned other resource limitation.

Step 5: We modify the arc costs in sub-problem using the related dual variables obtained from the optimal simplex tableau in Step 3.

Step 6: We solve the sub-problem using the Multi-Labels Correcting Method to derive the shortest path.

Step 7: If the length of the shortest path in the sub-problem is less than zero, it means this variable could contribute to the master problem. This column is added to the master problem. Then return to Step 3.

If the length of the shortest path in the sub-problem is equal to or greater than zero, it means no any other variable could contribute to the master problem. Then go to Step 8.

Figure 3-1 Solution algorithm flowchart

Step 8: If the optimal linear solution is an integer one, then stop; otherwise, go to Step 9.

Step 9: We obtain the integer solution by the branch-and-bound method.

We describe below the four major issues embedded in the above algorithm. There are initial variables（section 3.2）, the master problem（section 3.3）, and the sub-problem（section 3.4）.

## 3.2 Initial Variables

We define the single service of a bus as task. The initial variables of this model are each bus serves only one task. Since the column generation method is not sensitive to initial solution. We use the easiest way to find the initial feasible solution even though they are expensive columns.

## 3.3 The Master Problem

We construct the master problem as the set partitioning linear programming model. The decision variables of this model are a set of tasks. Then we show these columns could be generated by sub-problem.

### 3.3.1 The Model of The Master Problem

We formulate a minimum cost optimization model for our master problem. A feasible vehicle schedule has to ensure that every task is served by one and only one bus. One vehicle can serve a number of tasks per day. A collection of such tasks is called a column. Therefore the number of selected columns is equal to the number of vehicles. We define the following notation:

air= 1, if column r serves task i, 0 otherwise

cr: The cost of column r

xr= 1, if column r is be selected, 0 otherwise

N: The set of the tasks

R: The set of the columns

The real-time bus dispatching problem can be formulated as follows:

Minimize

r∈Rcrxr							（3-1）

Subject to

r∈Rairxr=1			∀i∈N		（3-2）

xr∈0,1			r∈R		（3-3）

The objective function（3-1）minimizes the total cost. The constraints（3-2）ensure that each task is served once and only once.

### 3.3.2 The Constraint of Bus Numbers

In order to avoid the number of column is more than available vehicles（Nv）, we should add the following constraint into the master problem.

rxr−riaSir×aiErxr≤Nv		（3-4）

aSir= 1, if the start node connecting to the task node i in column r, 0 otherwise

aiEr= 1, if the task node i connecting to the end node in column r, 0 otherwise

When aSir and aiEr are both equal to 1, it means the column r contained only one task i. It is impractical to let one bus served only one task. We call these columns as single task columns, the others are multi-tasks columns. The constraint（3-4）ensures that the number of multi-tasks column is equal to or less than the number of vehicles. The earliest task which is not included in the selected multi-tasks columns is where we need to resolve our problem.

## 3.4 The Sub-Problem

In our sub-problem, we construct a constrained shortest path model base on the vehicle scheduling network（we will describe in 3.4.1）to generate new columns. This model could generate a feasible column with the minimum marginal cost. In section 3.4.1 the vehicle scheduling network is described. In section 3.4.2 the constraints of the shortest path problem are presented. In section 3.4.3 the solution algorithm for this sub-problem is developed.

### 3.4.1 The Vehicle Scheduling Network

We define a network for vehicle scheduling. The nodes include the start node, end node and the task nodes. The arcs are annotated with the costs to serve them.

Let G indicates the whole vehicle scheduling network, N indicates set of nodes and A indicates set of arcs. So G is defined as follow:

G=N,A

There are three kinds of arcs. The first one is from the start node to any task node, the second one is from any task node to another task node, the third one is from any task node to the end node.

In Figure 3-2, the time in the start node and end node means the starting and ending time of operation. Each task node has three numbers associated. The first number is serial number of task. The second number is the starting time to carry out the task. The third number is the ending time of that task（e.g. the starting time and ending time for the task 1 is 8:05 and 8:55）.

A column is the collection of a serial of tasks. For example, a vehicle can start the day with task 1, and then serve task 4 and finally return to the depot to end the day. Other possibilities are to start the day with task 1, and then serve task 6 and finally return to the depot to end the day. Note that if no delayed strategy is concerned, then there is no arc from the task 1 to task 5 because the ending time of task 1 is later than starting time of task 5.

Figure 3-2: Example of a vehicle scheduling network

### 3.4.2 The Constrained Shortest Path Problem

In our sub-problem, we add the meal node to conform to the real world situation. Each driver could eat lunch within meal time. Table 3-1 defines the properties of the meal node.

Table 3-1 Properties of the meal node

| Properties | Description |
| --- | --- |
| Tstartmeal | The earliest time allowed for drivers to have meal. |
| Tendmeal | The latest time allowed for drivers to have meal. |
| Tmeal | The meal time of each driver. |

In order to make each column pass the meal node, we set the cost of meal node is negative to ensure each column must pass the meal node. To avoid the negative cycle in searching shortest path, we set one column should pass the meal node once and only once.

By adding the meal node to our sub-problem network, we redefine the categories of the arcs as following（Let Trest indicates the rest time between two continuous tasks, Tdelay indicates the allowed delay time of each task, STi indicates the starting time to carry out the task i and ETi indicates the ending time for the task i.）:

  - The segment from the start node to any other reachable nodes: The first type of arcs is the connection of the start node and any other reachable nodes.

  - The segment from any task node i to another task node j: If ETi plus Trest is earlier than STj plus Tdelay（ETi+Trest≤STj+Tdelay）, then task node i could connect to task node j. It means the task j could be proceeded right after task i is finished.

  - The segment from any task node i to the meal node: If ETi is later than Tstartmeal（ETi≥Tstartmeal）, and ETi plus Tmeal is earlier than Tendmeal（ETi+Tmeal≤Tendmeal）, then the node i could connect to the meal node. It means the meal node could be proceeded right after task i is finished.

  - The segment from the meal node to any task node j: Assume the task node i is connected to the meal node. If ETi plus Tmeal is before than STj plus Tdelay（ETi+Tmeal≤STj+Tdelay）, then the meal node could connect to the task node j. It means the task j could be proceeded right after the meal is finished.

  - The segment from any node to the end node: The last type of arcs is the connection of any node and the end node.

The sub-problem is constructed to generate the shortest path. The cost of the shortest path is sum of cost of selected arcs in the shortest path. The cost of arcs are determined by following factors:

- Idle time: Let Tidle indicates the idle time between two continuous nodes. For example, the idle time between task node i and task node j is STj−ETi. To avoid a column contained two continuous nodes whose idle time is too long, we assign a penalty weight w1 to idle time.

- Different route: Let I indicates the bus is dispatched to serve the task which has different route from the original schedule（I≠0 if yes, 0 otherwise）. When one bus serves the tasks that has changed route three times（ex. Route 1→Route 2→Route 1→Route 3）, then the I equals to 3. We assign a weight w2 to represent the degree of importance of I.

- Delay cost: Let D indicates the delay cost which is a constant based on the degree of importance. For example, the task node i is connecting to the task node j, while STj≤ETi≤STj+Tdelay, then the cost of arc（i,j）should include D multiply the total time of delay.

- Meal cost: Let CM indicates the cost of meal. As describe before, we set the cost of meal node negative to ensure each column must pass the meal node.

- Dual value: Let DVi indicates the dual value of the successor task node i.

An Example of different types of arcs and cost of arcs are shown in Figure 3-3 and Table 3-2.

Figure 3-3 Examples of different types of arcs

Table 3-2 Examples of cost of arcs

| Type | Predecessor node | Successor node | Cost of arc |
| --- | --- | --- | --- |
| A | Start node | Task node | Tidle−DVi |
| B | Task node | Task node which is in different route | w1×Tidle+w2×I−DVi |
| C | Task node | Task node which is in the same route | w1×Tidle−DVi |
| D | Task node | Meal node | CM |
| E | Meal node | Task node | w1×Tidle−DVi |
| F | Task node | End node | Tidle |
| G | Task node | Task node which is in different route and with delay | w1×Tidle+w2×I+D−DVi |

### 3.4.3 The Sub-Problem Solution

We develop a Multi-Labels Correcting Method to solve the shortest path problem with resource constraints. Not alike the Label Correcting Method which is only concerned about the cost. In our study, we consider four labels. There are cost, current time, route of the predecessor node, and the number of meals.

The flowchart of the Multi-Labels Correcting Method is shown in Figure 3-4. The steps of the algorithm are listed as follows:

Step 1: Subtract the related dual values from the arc costs. Create a container to store the updated nodes. The container is empty at first. The initial current node is the start node. Let i indicates the serial number of nodes. Initiate i = 0.

Step 2: i = i + 1.

Step 3: According to the definition of arcs in section 3.4.1. If current node could connect to the node i, then go to Step 4. Otherwise return to Step 2.

Step 4: If the successor node is the meal node, then go to Step 5. Otherwise go to Step 6.

Step 5: If the path has passed the meal node, then go to Step 6. Otherwise go to Step 7.

Step 6: If current cost exceeds the cost label of the successor node, then go to Step 7. Otherwise, replace the cost label with current cost, update the current time, replace the predecessor node with present node, and add the successor node to the bottom of container, go to Step 7.

Step 7: If node i is the end node, it means all nodes are examined, then go to Step 8. Otherwise return to Step 2.

Step 8: If container is not empty then go to Step 9. Otherwise go to Step 10.

Step 9: Take the top node from container as the present node. Initiate i = 0. Then return to Step 2

Step 10: Stop.

Figure 3-4 Multi-Labels Correcting Method flowchart

# Chapter4 Computational Experiments

In this section, we test how well the real-time bus dispatching algorithm may be applied in the real world. We perform a computational experimentation regarding the operations of public transit company. In this experiment, the LINDO callable library is used as the LP solver. All the numerical problems are solved on a Pentium-III 933 PC.

First, we test how the real-time bus dispatching algorithm apply in the real world. Second, we establish different situations of schedule based on three factors（the number of routes, the headway, and the round-trip time of the route）. Then apply the solution algorithm to solve the problem. Finally, we perform sensitivity analyses by adjusting parameters of the model.

## 4.1 Solutions For Real-Time Dispatching Problem

In this section we try to do a real-time dispatching when a disturbance occurs. Figure 4-1 shows the schedule of 4 buses.

Figure 4-1 The schedule of real-time dispatching problem

The GPS signal and expected return time of all buses can be obtained. Assume Bus 1 has an incident on road due to mechanical failure at 5:50. The expected return time of Bus 1 is 10:00. Therefore the dispatcher should dispatch 3 remaining buses to serve all tasks between 6:00 to 10:00.

### 4.1.1 The maximum delay time is 10 minutes

First, we set the maximum delay time is 10 minutes. The minimum rest time between two continuous tasks is 10 minutes. The solution result is shown in Figure 4-2. When the delay strategy is used, the starting time and ending time of task 4 have been changed

Figure 4-2 The solution result of 10 minutes delay time allowed

The solution result has two new disturbed tasks as shown in the gray nodes. The one of the new disturbed tasks is at 7:30. Another new disturbed tasks is at 9:00. Because the Bus 1 fails to return to the terminal at 7:30 and 9:00, therefore the disturbed task should be cancelled.

### 4.1.2 The maximum delay time is 20 minutes

When we set the maximum delay time as 20 minutes, there is no task should be cancelled. The solution result is shown in Figure 4-3.

Figure 4-3 The solution result of 20 minutes delay time allowed

When the maximum delay time is 20 minutes, the solution result has no disturbed task. The solution algorithm successfully solves the problem of bus shortage by delaying tasks and swapping bus among different routes.

When the Bus 1 return to the terminal at 10:00 according to the expected return time, the schedule return to its original one. After the Bus 1 return to the terminal at 10:00, the schedule is shown in Figure 4-4.

Figure 4-4 The schedule after the perturbation be solved

## 4.2 Different Situations of schedule

In this section we assume Bus 1 has an incident on road due to mechanical failure at 5:50. The expected return time of Bus 1 is 8:30. Thus, the impact duration is from 6:00 to 8:30. We assume this perturbation occurs in 8 different situations of schedule. The different situations of schedule in our case study are based on three factors. The first factor is the number of routes whose buses can support each other. The second factor is headway. We define the short headway is less than 30 minutes. The long headway is larger than 31 minutes. The last factor is the round-trip time of the route. We define the long round-trip time is larger than 51 minutes. The short round-trip time is less than 50 minutes.

The parameters in these situations are as following:

  - The time allowed for drivers to have meal is from 11:00 to 13:00.

  - The meal time is 30 minutes.

  - The penalty weight of delay is 3 per minutes.

  - The penalty weight of different route is 30.

  - The maximum delay time is 20 minutes.

The 8 test schedules are shown in Appendix A~H. The test results are shown in Table 4-1. The CPU Time to solve the problem is within 2 minutes.（Situation1: 5 routes, Short headway, Long round-trip time. Situation2: 5 routes, Short headway, Short round-trip time. Situation3: 5 routes, Long headway, Long round-trip time. Situation4: 5 routes, Long headway, Short round-trip time. Situation5: 8 routes, Short headway, Long round-trip time. Situation6: 8 routes, Short headway, Short round-trip time. Situation7: 8 routes, Long headway, Long round-trip time. Situation8: 8 routes, Long headway, Short round-trip time.）

Table 4-1 The test results of different situations of schedule

| Situation | Objective Value | Number of New Disturbed Task | Earliest Time of Disturbed Task | Number of Delay | Total Delay Time | Number of Different Route | Max Number of Different Route in a column |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 21240 | 2 | 7:20 | 1 | 10 | 1 | 1 |
| 2 | 10530 | 1 | 7:20 | 3 | 40 | 1 | 1 |
| 3 | 10500 | 1 | 7:20 | 10 | 130 | 2 | 1 |
| 4 | 1870 | N/A | N/A | 13 | 150 | 3 | 1 |
| 5 | 4850 | N/A | N/A | 16 | 220 | 5 | 1 |
| 6 | 4360 | N/A | N/A | 22 | 250 | 8 | 1 |
| 7 | 4570 | N/A | N/A | 14 | 160 | 5 | 1 |
| 8 | 2690 | N/A | N/A | 20 | 220 | 7 | 1 |

In the following sections, we will compare the different result based on three factors（The number of routes, headway, and the round-trip time）.

### 4.2.1 The Number of Routes

The test result of short headway and long round-trip time is shown below:

| The Number of Routes | Objective Value | Number of New Disturbed Task | Earliest Time of Disturbed Task | Number of Delay | Total Delay Time | Number of Different Route | Max Number of Different Route in a column |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 21240 | 2 | 7:20 | 1 | 10 | 1 | 1 |
| 8 | 4850 | N/A | N/A | 16 | 220 | 5 | 1 |

The test result of short headway and short round-trip time is shown below:

| The Number of Routes | Objective Value | Number of New Disturbed Task | Earliest Time of Disturbed Task | Number of Delay | Total Delay Time | Number of Different Route | Max Number of Different Route in a column |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 10530 | 1 | 7:20 | 3 | 40 | 1 | 1 |
| 8 | 4360 | N/A | N/A | 22 | 250 | 8 | 1 |

The test result of long headway and long round-trip time is shown below:

| The Number of Routes | Objective Value | Number of New Disturbed Task | Earliest Time of Disturbed Task | Number of Delay | Total Delay Time | Number of Different Route | Max Number of Different Route in a column |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 10500 | 1 | 7:20 | 10 | 130 | 2 | 1 |
| 8 | 4570 | N/A | N/A | 14 | 160 | 5 | 1 |

The test result of long headway and short round-trip time is shown below:

| The Number of Routes | Objective Value | Number of New Disturbed Task | Earliest Time of Disturbed Task | Number of Delay | Total Delay Time | Number of Different Route | Max Number of Different Route in a column |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 1870 | N/A | N/A | 13 | 150 | 3 | 1 |
| 8 | 2690 | N/A | N/A | 20 | 220 | 7 | 1 |

When the number of routes is less, it causes less flexible to adopt more different route support strategy to solve the perturbation problem. When the number of routes is 5, it is hard to solve the disturbed task by using more delay strategy or different route strategy. When the number of routes is 8, the solution algorithm can flexibly adopt more delay strategy and different route strategy to solve the perturbation problem.

### 4.2.2 The Headway

The test result of 5 routes and long round-trip time is shown below:

| Headway | Objective Value | Number of New Disturbed Task | Earliest Time of Disturbed Task | Number of Delay | Total Delay Time | Number of Different Route | Max Number of Different Route in a column |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Short | 21240 | 2 | 7:20 | 1 | 10 | 1 | 1 |
| Long | 10500 | 1 | 7:20 | 10 | 130 | 2 | 1 |

The test result of 5 routes and short round-trip time is shown below:

| Headway | Objective Value | Number of New Disturbed Task | Earliest Time of Disturbed Task | Number of Delay | Total Delay Time | Number of Different Route | Max Number of Different Route in a column |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Short | 10530 | 1 | 7:20 | 3 | 40 | 1 | 1 |
| Long | 1870 | N/A | N/A | 13 | 150 | 3 | 1 |

The test result of 8 routes and long round-trip time is shown below:

| Headway | Objective Value | Number of New Disturbed Task | Earliest Time of Disturbed Task | Number of Delay | Total Delay Time | Number of Different Route | Max Number of Different Route in a column |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Short | 4850 | N/A | N/A | 16 | 220 | 5 | 1 |
| Long | 4570 | N/A | N/A | 14 | 160 | 5 | 1 |

The test result of 8 routes and short round-trip time is shown below:

| Headway | Objective Value | Number of New Disturbed Task | Earliest Time of Disturbed Task | Number of Delay | Total Delay Time | Number of Different Route | Max Number of Different Route in a column |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Short | 4360 | N/A | N/A | 22 | 250 | 8 | 1 |
| Long | 2690 | N/A | N/A | 20 | 220 | 7 | 1 |

When the headway is shorter, the buffer time between two continuous tasks is tighter. We see that it is hard to solve the disturbed task by using delay strategy or different route strategy. Although the disturbed task can’t be solved when headway is short, the solution algorithm still can push the time of disturbed task later.

### 4.2.3 The Round-Trip Time of Route

The test result of 5 routes and short headway is shown below:

| Round-Trip Time | Objective Value | Number of New Disturbed Task | Earliest Time of Disturbed Task | Number of Delay | Total Delay Time | Number of Different Route | Max Number of Different Route in a column |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Long | 21240 | 2 | 7:20 | 1 | 10 | 1 | 1 |
| Short | 10530 | 1 | 7:20 | 3 | 40 | 1 | 1 |

The test result of 5 routes and long headway is shown below:

| Round-Trip Time | Objective Value | Number of New Disturbed Task | Earliest Time of Disturbed Task | Number of Delay | Total Delay Time | Number of Different Route | Max Number of Different Route in a column |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Long | 10500 | 1 | 7:20 | 10 | 130 | 2 | 1 |
| Short | 1870 | N/A | N/A | 13 | 150 | 3 | 1 |

The test result of 8 routes and short headway is shown below:

| Round-Trip Time | Objective Value | Number of New Disturbed Task | Earliest Time of Disturbed Task | Number of Delay | Total Delay Time | Number of Different Route | Max Number of Different Route in a column |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Long | 4850 | N/A | N/A | 16 | 220 | 5 | 1 |
| Short | 4360 | N/A | N/A | 22 | 250 | 8 | 1 |

The test result of 8 routes and long headway is shown below:

| Round-Trip Time | Objective Value | Number of New Disturbed Task | Earliest Time of Disturbed Task | Number of Delay | Total Delay Time | Number of Different Route | Max Number of Different Route in a column |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Long | 4570 | N/A | N/A | 14 | 160 | 5 | 1 |
| Short | 2690 | N/A | N/A | 20 | 220 | 7 | 1 |

When the round-trip time is longer, it means each bus is hard to serve many tasks in one day. When the round-trip time is shorter, each bus can finish the task quickly and return to terminal to serve another task. Hence, from the table above, we can see it is hard to use delay strategy or different route strategy when the round-trip time is long. It also has more new unsolved disturbed tasks.

## 4.3 Adjustable Parameters Experiments

There are four adjustable parameters compared as following:

- The time allowed for drivers to have meal.

- The meal time.

- The penalty weight of delay.

- The penalty weight of different route.

First, we analyze the effect of solution results by adjusting the time allowed for drivers to have meal. Second, we analyze the effect of solution result by adjusting the meal time. Third, we analyze the effect of solution result by adjusting the penalty weight of delay. Finally, we analyze the effect of solution result by adjusting the penalty weight of different route.

We establish a schedule of 51 buses, 12 routes, and 182 tasks. The test schedule is shown in Appendix I. The CPU Time to solve the problem is within 10 minutes.

### 4.3.1 The Time Allowed For Drivers To Have Meal

We test three cases by adjusting the time allowed for drivers to have meal. The time in Case 1 is from 11:00 to 13:00. The time in Case 2 is from 10:00 to 14:00. The time in Case 3 is from 11:30 to 12:30. The other parameters are given as following:

- The meal time is 30 minutes.

- The penalty weight of delay is 3 per minutes.

- The penalty weight of different route is 30.

- The maximum delay time is 20 minutes.

The test result is shown in Table 4-2.

Table 4-2 The test result of different time allowed for drivers to have meal

| The Time Allowed For Drivers To Have Meal | Objective Value | Number of Delay | Total Delay Time | Number of Different Route | Max Number of Different Route in a column |
| --- | --- | --- | --- | --- | --- |
| Case1(11:00~13:00) | 10760 | 13 | 160 | 5 | 2 |
| Case2(10:00~14:00) | 9850 | 10 | 110 | 3 | 1 |
| Case3(11:30~12:30) | 13570 | 15 | 180 | 7 | 2 |

When the time range for drivers to have meal is longer, such as Case 2, it is more flexible to find better solutions. Therefore, the number of delay, and the number of different route is the lowest in Case 2.

### 4.3.2 The Meal Time

We test two cases by adjusting the meal time. The time in Case 1 is 30 minutes. The time in Case 2 is 60 minutes. The other parameters are given as following:

- The time allowed for drivers to have meal is from 11:00 to 13:00.

- The penalty weight of delay is 3 per minutes.

- The penalty weight of different route is 30.

- The maximum delay time is 20 minutes.

The test result is shown in Table 4-3.

Table 4-3 The test result of different meal time

| The Meal Time | Objective Value | Number of Delay | Total Delay Time | Number of Different Route | Max Number of Different Route in a column |
| --- | --- | --- | --- | --- | --- |
| Case1(30 minutes) | 10760 | 13 | 160 | 5 | 2 |
| Case2(60 minutes) | 13850 | 22 | 250 | 9 | 2 |

When the meal time is longer, each driver spend more time to have meal. It causes less flexible to find the better solution. Therefore the number of delay, and the number of different route is higher in Case 2.

### 4.3.3 The Penalty Weight of Delay

We test eight cases by adjusting the penalty weight of delay. There are 1, 2, 3, 4, 5, 10, 50, 900 per minute. The other parameters are given as following:

- The time allowed for drivers to have meal is from 11:00 to 13:00.

- The meal time is 30 minutes.

- The penalty weight of different route is 30.

- The maximum delay time is 20 minutes.

The test result is shown in Table 4-4.

Table 4-4 The test result of different penalty weight of delay

| The Penalty Weight of Delay (per minute) | Objective Value | Number of New Disturbed Task | Earliest Time of Disturbed Task | Number of Delay | Total Delay Time | Number of Different Route | Max Number of Different Route in a column |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10050 | N/A | N/A | 28 | 320 | 13 | 2 |
| 2 | 10340 | N/A | N/A | 25 | 300 | 15 | 2 |
| 3 | 10760 | N/A | N/A | 22 | 280 | 16 | 2 |
| 4 | 11060 | N/A | N/A | 18 | 220 | 18 | 2 |
| 5 | 12980 | N/A | N/A | 15 | 180 | 19 | 2 |
| 10 | 14110 | 1 | 9:50 | 8 | 100 | 19 | 2 |
| 50 | 16220 | 1 | 9:50 | 5 | 70 | 20 | 3 |
| 900 | 20310 | 1 | 9:50 | 2 | 30 | 21 | 3 |

When the penalty weight of delay is higher, which cause the number of delay decrease. The solution algorithm adopt more different route support strategy and generate more new disturbed task to substitute for less delay strategy.

### 4.3.4 The Penalty Weight of Different Route

We test eight cases by adjusting the penalty weight of different route. There are 10, 20, 30, 40, 50, 100, 500, 9000. The other parameters are given as following:

- The time allowed for drivers to have meal is from 11:00 to 13:00.

- The meal time is 30 minutes.

- The penalty weight of delay is 3 per minutes.

- The maximum delay time is 20 minutes.

The test result is shown in Table 4-5.

Table 4-5 The test result of different penalty weight of different route

| The Penalty Weight of Different Route | Objective Value | Number of New Disturbed Task | Earliest Time of Disturbed Task | Number of Delay | Total Delay Time | Number of Different Route | Max Number of Different Route in a column |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 10050 | N/A | N/A | 28 | 320 | 13 | 2 |
| 20 | 10180 | N/A | N/A | 30 | 330 | 8 | 2 |
| 30 | 10560 | N/A | N/A | 31 | 340 | 5 | 2 |
| 40 | 10840 | N/A | N/A | 31 | 360 | 3 | 1 |
| 50 | 10120 | N/A | N/A | 33 | 350 | 0 | 0 |
| 100 | 10120 | N/A | N/A | 33 | 350 | 0 | 0 |
| 500 | 10120 | N/A | N/A | 33 | 350 | 0 | 0 |
| 9000 | 10120 | N/A | N/A | 33 | 350 | 0 | 0 |

When the penalty weight of different route is higher, which cause the number of different route decrease. The solution algorithm adopt more delay strategy to substitute for less different route support strategy.

When the penalty weight of different route is higher than 50, there is no any different route support strategy be adopted. Also, no any new disturbed task has been generated. It means in our case study, the delay strategy is more useful to solve the disturbed tasks than different route support strategy.

## 4.4 Summary

The computational experimentation is summarized as following:

- The solution algorithm can successfully solve the problem of bus shortage by delaying tasks and swapping bus among different routes. And the schedule will return to its original one after the perturbation be solved.

- When the number of routes is less, it causes less flexible to adopt more different route support strategy to solve the perturbation problem. Oppositely, the solution algorithm can flexibly adopt more delay strategy and different route strategy to solve the perturbation problem when the number of routes is more.

- When the headway is shorter, the buffer time between two continuous tasks is tighter. It is hard to solve the disturbed task by using delay strategy or different route strategy.

- When the round-trip time is shorter, each bus can finish the task quickly and return to terminal to serve another task. It is hard to use delay strategy or different route support strategy when the round-trip time is long. It also has more new disturbed task not be solved when the round-trip time is long.

- When the time range for drivers to have meal is longer, it is more flexible to find the better solution. Therefore the number of delay, and the number of different route is lower.

- When the meal time is longer, each driver spend more time to have meal. It causes less flexible to find the better solution.

- When the penalty weight of delay is higher, which cause the number of delay decrease. The solution algorithm adopt more different route support strategy and generate more disturbed task to substitute for less delay strategy.

- When the penalty weight of different route is higher, which cause the number of different route decrease. The solution algorithm adopt more delay strategy to substitute for less different route support strategy.

- In our case study, the delay strategy is more useful to solve the disturbed tasks than different route support strategy.

# Chapter5 Conclusions and Suggestions

## 5.1 Conclusions

For the moment, the achievements progress of this research is shown as following:

- Formulate the real-time bus dispatching problem: We formulate the real-time bus dispatching problem as a set partitioning linear programming model. The decision variables of this model are a set of columns. Each task is served by one bus once and only once.

- Develop the algorithm for solving the real-time bus dispatching problem: We propose a column generation algorithm to solve the real-time bus dispatching problem. In column generation approach, the master problem is solved by simplex method. The sub-problem is constructed as a constrained shortest path problem. We solve the sub-problem using the Multi-Labels Correcting Method to derive the shortest path.

- Obtain the integer solution by the branch-and-bound method: If the solution we obtain after column generation approach is not an integer one, we use the branch-and-bound method to find an integer solution.

- Computational experiments: The computational experiments show that the solution algorithm can successfully solve the problem of bus shortage by delaying tasks and swapping bus among different routes. And the schedule will return to its original one after the perturbation be solved.

## 5.2 Suggestions

The topics we suggest for future research are:

- Consider the regulation of headway: Usually there are some constraints regulate the headway, such as the headway must be ten minutes. However, in our model we do not take this into consideration. We suggest this mode could be modified to accommodate this headway constraint.

- Generate useful columns: The final integer solution in our study is the dispatching result. The number of non-zero columns equals to the number of buses. It is much less than the all columns generated by column generation approach. Therefore the future study can improve efficiency in generating useful columns rather than all contributed columns.

- Consider other constraints in model: We use meal node to conform to the real world situation. Another constraint such as the maximum working time of the bus. It can be embed in the model to conform to the real world situation.

- Apply the real-time bus dispatching algorithm to other area: Our testing result of this algorithm is satisfactory. We can consider to apply this solution technique to other areas such as real-time aircraft dispatching, driver and crew dispatching, train dispatching et al.

# Reference

- Ahmad I.Z. Jarrah, Gang Yu,“A decision support framework for airline flight cancellations and delays”, Transportation Science, Vol.27, No.3, pp.266-280, 1993.

- Avishai Ceder, Yechezkel Israeli,“Scheduling considerations in designing transit routes at the network level”, Lecture notes in economics and mathematical system, 386, pp.113-136, 1992.

- Camil Fuchs,“Optimization in bus scheduling – driver constraints deadheading estimation and size advantage”, Lecture notes in economics and mathematical system, 386, pp.197-212, 1992.

- Christos V, Efthymios H, “Combined bus and driver scheduling”, Computers & Operations Research, 29, pp.243-259, 2002.

- Christian F, Knut H, “An exact branch and cut algorithm for the vehicle and crew scheduling problem”, LECT NOTES ECON MATH, 471, pp. 63-80, 1999.

- Ching-Chyuan Lu, “Aircraft dispatching strategy for irregular flight incidents”, Transportation Planning Journal, Vol.25, No.2, pp.255-288, 1996.

- Chung-Cheng Lu, “The study of mass rapid transit drivers’ scheduling problems: a case study of Taipei MRT Company”,master thesis, Chiao Tung University, Hsinchu, Taiwan, Republic of China, 2000.

- CS Sung, JM Hong,“Branch-and-price algorithm for a multicast routing problem”, Journal of the Operational Research Society, pp.1168-1175, 1999.

- Cynthia Barnhart, Christopher A. Hane, Pamela H. Vance,“Using branch-and-price-and cut to solve origin-destination integer multicommodity flow problems”, Operations Research, Vol.48, No.2, pp.318-326, 2000.

- Cynthia Barnhart, Ellis L. Johnson, George L. Nemhauser, Martin W. P. Savelsbergh, Pamela H. Vance,“Branch-and price：column generation for solving huge integer programs”, Operations Research, Vol.46, No.3, pp.316-329, 1998.

- Daeki Kim, Cynthia Barnhart,“Transportation service network design：models and algorithms”, Lecture notes in economics and mathematical system, 471, pp.259-283, 1999.

- Dah-Hwei Yang, “Systematic aircraft routing under schedule perturbation caused by technical incidents of aircrafts”, Transportation Planning Journal, Vol.23, No.4, pp.407-426, 1994.

- David Grove Jorgensen, Morten Meyling, “A branch-and-price algorithm for switch-box routing”, NETWORKS, Vol.40(1), pp.13-26, 2002.

- Desrochers M., Rousseau J., “Real-Time scheduling on a transit bus route”, Computer-aided transit scheduling, No.386, 1992.

- Eric Gelman, Hai D. Chu, Ellis L. Johnson, “Solving large scale crew scheduling problems”, European Journal of Operational Research, Vol.97, pp.260-268, 1997.

- Fang-Chyi Liu, “The study on urban bus scheduling and real time rescheduling: a case study of hsinchu transportation company”, master thesis, Chiao Tung University, Hsinchu, Taiwan, Republic of China, 1998.

- Feng-Ping Yuan, “The study of bus dispatching system under expected perturbation”, master thesis, Chiao Tung University, Hsinchu, Taiwan, Republic of China, 2001.

- Francois Vanderbeck,“On dantzig-wolfe decomposition in integer programming and ways to perform branching in a branch-and-price algorithm”, Operations Research, Vol.48, No.1, pp.111-128, 2000.

- Francois Vanderbeck,“Exact algorithm for minimising the number of setups in the one-dimensional cutting stock problem”, Operations Research, Vol.48,  No.6, pp.915-926, 2000.

- G.Belov, G.Scheithauer,“A cutting plane algorithm for the one-dimensional cutting stock problem with multiple stock lengths”, European journal of operational research, 141, pp.274-294, 2002.

- Gary M.Roodman,“Near-optimal solutions to one-dimensional cutting stock problems”, Comput.&Ops.Res., Vol.13, No.6, pp.713-719, 1986.

- George B.Dantzig, Philip Wolfe,“Decomposition principle for linear programs”, Mathematical Society, 1959.

- J.M.Valerio de Carvalho,“LP models for bin packing and cutting stock problems”, European journal of operational research, Vol.141, pp.253-273, 2002.

- JA Diaz, E Fernandez,“A branch-and-price algorithm for the single source capacitated plant location problem”, Journal of the Operational Research Society, pp.728-740, 2002.

- James G. Strathman,“Service reliability impacts of computer-aided dispatching and automatic vehicle location technology: a tri-met case study”, Transportation Quarterly, Vol. 54, No. 3, pp. 85-202, 2000.

- Jao-Hong Cheng, “An application of a fuzzy expert system on bus operation management under uncertain situations”, Transportation Planning Journal, Vol.27, No.1, pp.77-108, 1998.

- Jean-Yves Potvin,“Learning techniques for an expert vehicle dispatching system”, Expert Systems With Applications, Vol. 8, No. 1, pp. 101-109, 1995.

- Jei-Chi Chang, “Airline cockpit crew scheduling”, European Journal of Operational Research, 136, pp.501-511, 2002.

- Jia Ming Cao, Adib Kanafani,“Real-Time decision support for integration of airline flight cancellations and delays part I&II：mathematical formulation”, Transportation Planning and Technology, Vol.22, pp.183-199, 1997.

- Jin-Mei Shieh, “A study on applications of expert system to bus scheduling”, Transportation Planning Journal, Vol.23, No.2, pp. 199-234, 1994.

- Jyhterng Luo, “Systematic fleet routing and flight scheduling under expected maintenance of aircrafts”, Journal of the Chinese Institute of Civil and Hydraulic Engineering, Vol.8, No.3, pp. 447-456, 1996.

- L. Douglas Smith, Robert M. Nauss, David A. Bird, “Decision support for bus operations in a mass transit system”, Decision Sciences, Vol.21, pp. 183-203, 1990

- Michael Junger, Stefan Thienel,“Introduction to ABACUS – a branch-and-cut system”, Operations research letters, 22, pp.83-95, 1998.

- Marta Mesquita, J.Paixao,“Multiple depot vehicle scheduling problem：a new heuristic based on quasi-assignment algorithms”, Lecture notes in economics and mathematical system, 386, pp.167-180, 1992.

- Martin Desrochers, Francois Soumis,“A column generation approach to the urban transit crew scheduling problem”, Transportation Science, Vol.23, No.1, pp.1-13, 1989.

- Martin Savelsbergh,“A branch-and-price algorithm for the generalized assignment problem”, Operations Research, Vol.45, No.6, pp.831-842, 1997.

- Mohamadreza Banihashemi, Ali Haghani,“Optimization model for large-scale bus transit scheduling problems”, Transportation Research Record, 1733, pp.23-30, 2000.

- Osman Oguz,“Generalized column generation for linear programming”, Management Science, Vol.48, No.3, pp.444-452, 2002.

- P.C.Gilmore, R.E.Gomory,“A linear programming approach to the cutting-stock problem”, Mathematical Society, 1961.

- Powell, W.B.,“An operational planning model for the dynamic”, Transportation Research-B, Vol.21, No.3, pp.217-232, 1987.

- Powell, W.B.,“A comparative review of alternative algorithms for the dynamic vehicle allocation problem”, Vehicle Routing: Methods and Studies, pp.249-291, 1988.

- Riberro, C.C., Soumis, F.O.,“A column generation approach to the multiple depot vehicle scheduling problem”, Operations Research, Vol.42, No.1, pp.41-51, 1994.

- Teodorovic D., Guberinic S.,“Optimal dispatching strategy on an airline network after a schedule perturbation”, European journal of operational research, Vol.15, pp.178-182, 1984.

- Tzung-Hsien Wu, “A scheduling expert system on mutual support between bus routes”, Transportation Planning Journal, Vol.26, No.1, pp.159-202, 1997

- Wilson, Nigel H.M.,“Modeling Real-Time control strategies in public transit operations”, Computer-aided transit scheduling, No.471, 1999.

# Appendix A—The schedule with 5 routes, short headway, and long round-trip time

| Task | Starting Time | Ending Time | Route | Task | Starting Time | Ending Time | Route |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 06:00 | 07:00 | 1 | 28 | 08:00 | 09:20 | 3 |
| 2 | 07:10 | 08:10 | 1 | 29 | 07:00 | 08:20 | 3 |
| 3 | 08:20 | 09:20 | 1 | 30 | 08:30 | 09:50 | 3 |
| 4 | 06:10 | 07:10 | 1 | 31 | 06:00 | 07:00 | 4 |
| 5 | 07:20 | 08:20 | 1 | 32 | 07:10 | 08:10 | 4 |
| 6 | 08:30 | 09:30 | 1 | 33 | 08:20 | 09:20 | 4 |
| 7 | 06:20 | 07:20 | 1 | 34 | 06:10 | 07:10 | 4 |
| 8 | 07:30 | 08:30 | 1 | 35 | 07:20 | 08:20 | 4 |
| 9 | 06:30 | 07:30 | 1 | 36 | 08:30 | 09:30 | 4 |
| 10 | 07:40 | 08:40 | 1 | 37 | 06:20 | 07:20 | 4 |
| 11 | 06:40 | 07:40 | 1 | 38 | 07:30 | 08:30 | 4 |
| 12 | 07:50 | 08:50 | 1 | 39 | 06:30 | 07:30 | 4 |
| 13 | 06:50 | 07:50 | 1 | 40 | 07:40 | 08:40 | 4 |
| 14 | 08:00 | 09:00 | 1 | 41 | 06:40 | 07:40 | 4 |
| 15 | 07:00 | 08:00 | 1 | 42 | 07:50 | 08:50 | 4 |
| 16 | 08:10 | 09:10 | 1 | 43 | 06:50 | 07:50 | 4 |
| 17 | 06:00 | 07:10 | 2 | 44 | 08:00 | 09:00 | 4 |
| 18 | 07:20 | 08:30 | 2 | 45 | 07:00 | 08:00 | 4 |
| 19 | 06:20 | 07:30 | 2 | 46 | 08:10 | 09:10 | 4 |
| 20 | 07:40 | 08:50 | 2 | 47 | 06:00 | 07:10 | 5 |
| 21 | 06:40 | 07:50 | 2 | 48 | 07:20 | 08:30 | 5 |
| 22 | 08:00 | 09:10 | 2 | 49 | 06:20 | 07:30 | 5 |
| 23 | 07:00 | 08:10 | 2 | 50 | 07:40 | 08:50 | 5 |
| 24 | 08:20 | 09:30 | 2 | 51 | 06:40 | 07:50 | 5 |
| 25 | 06:00 | 07:20 | 3 | 52 | 08:00 | 09:10 | 5 |
| 26 | 07:30 | 08:50 | 3 | 53 | 07:00 | 08:10 | 5 |
| 27 | 06:30 | 07:50 | 3 | 54 | 08:20 | 09:30 | 5 |

# Appendix B—The schedule with 5 routes, short headway, and short round-trip time

| Task | Starting Time | Ending Time | Route | Task | Starting Time | Ending Time | Route |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 06:00 | 06:20 | 1 | 29 | 07:20 | 07:30 | 3 |
| 2 | 06:30 | 06:50 | 1 | 30 | 07:40 | 07:50 | 3 |
| 3 | 07:00 | 07:20 | 1 | 31 | 08:00 | 08:10 | 3 |
| 4 | 07:30 | 07:50 | 1 | 32 | 08:20 | 08:30 | 3 |
| 5 | 08:00 | 08:20 | 1 | 33 | 06:00 | 06:20 | 4 |
| 6 | 08:30 | 08:50 | 1 | 34 | 06:30 | 06:50 | 4 |
| 7 | 06:10 | 06:30 | 1 | 35 | 07:00 | 07:20 | 4 |
| 8 | 06:40 | 07:00 | 1 | 36 | 07:30 | 07:50 | 4 |
| 9 | 07:10 | 07:30 | 1 | 37 | 08:00 | 08:20 | 4 |
| 10 | 07:40 | 08:00 | 1 | 38 | 08:30 | 08:50 | 4 |
| 11 | 08:10 | 08:30 | 1 | 39 | 06:10 | 06:30 | 4 |
| 12 | 06:20 | 06:40 | 1 | 40 | 06:40 | 07:00 | 4 |
| 13 | 06:50 | 07:10 | 1 | 41 | 07:10 | 07:30 | 4 |
| 14 | 07:20 | 07:40 | 1 | 42 | 07:40 | 08:00 | 4 |
| 15 | 07:50 | 08:10 | 1 | 43 | 08:10 | 08:30 | 4 |
| 16 | 08:20 | 08:40 | 1 | 44 | 06:20 | 06:40 | 4 |
| 17 | 06:00 | 06:30 | 2 | 45 | 06:50 | 07:10 | 4 |
| 18 | 06:40 | 07:10 | 2 | 46 | 07:20 | 07:40 | 4 |
| 19 | 07:20 | 07:50 | 2 | 47 | 07:50 | 08:10 | 4 |
| 20 | 08:00 | 08:30 | 2 | 48 | 08:20 | 08:40 | 4 |
| 21 | 06:20 | 06:50 | 2 | 49 | 06:00 | 06:30 | 5 |
| 22 | 07:00 | 07:30 | 2 | 50 | 06:40 | 07:10 | 5 |
| 23 | 07:40 | 08:10 | 2 | 51 | 07:20 | 07:50 | 5 |
| 24 | 08:20 | 08:50 | 2 | 52 | 08:00 | 08:30 | 5 |
| 25 | 06:00 | 06:10 | 3 | 53 | 06:20 | 06:50 | 5 |
| 26 | 06:20 | 06:30 | 3 | 54 | 07:00 | 07:30 | 5 |
| 27 | 06:40 | 06:50 | 3 | 55 | 07:40 | 08:10 | 5 |
| 28 | 07:00 | 07:10 | 3 | 56 | 08:20 | 08:50 | 5 |

# Appendix C—The schedule with 5 routes, long headway, and long round-trip time

| Task | Starting Time | Ending Time | Route | Task | Starting Time | Ending Time | Route |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 06:00 | 07:00 | 1 | 12 | 06:50 | 08:10 | 3 |
| 2 | 07:10 | 08:10 | 1 | 13 | 08:20 | 09:40 | 3 |
| 3 | 08:20 | 09:20 | 1 | 14 | 06:00 | 07:00 | 4 |
| 4 | 06:50 | 07:50 | 1 | 15 | 07:10 | 08:10 | 4 |
| 5 | 08:00 | 09:00 | 1 | 16 | 08:20 | 09:20 | 4 |
| 6 | 06:00 | 07:10 | 2 | 17 | 07:00 | 08:00 | 4 |
| 7 | 07:20 | 08:30 | 2 | 18 | 08:10 | 09:10 | 4 |
| 8 | 06:40 | 07:50 | 2 | 19 | 06:00 | 07:10 | 5 |
| 9 | 08:00 | 09:10 | 2 | 20 | 07:20 | 08:30 | 5 |
| 10 | 06:00 | 07:20 | 3 | 21 | 06:40 | 07:50 | 5 |
| 11 | 07:30 | 08:50 | 3 | 22 | 08:00 | 09:10 | 5 |

# Appendix D—The schedule with 5 routes, long headway, and short round-trip time

| Task | Starting Time | Ending Time | Route | Task | Starting Time | Ending Time | Route |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 06:00 | 06:20 | 1 | 15 | 07:20 | 07:30 | 3 |
| 2 | 06:30 | 06:50 | 1 | 16 | 07:40 | 07:50 | 3 |
| 3 | 07:00 | 07:20 | 1 | 17 | 08:00 | 08:10 | 3 |
| 4 | 07:30 | 07:50 | 1 | 18 | 08:20 | 08:30 | 3 |
| 5 | 08:00 | 08:20 | 1 | 19 | 06:00 | 06:20 | 4 |
| 6 | 08:30 | 08:50 | 1 | 20 | 06:30 | 06:50 | 4 |
| 7 | 06:00 | 06:30 | 2 | 21 | 07:00 | 07:20 | 4 |
| 8 | 06:40 | 07:10 | 2 | 22 | 07:30 | 07:50 | 4 |
| 9 | 07:20 | 07:50 | 2 | 23 | 08:00 | 08:20 | 4 |
| 10 | 08:00 | 08:30 | 2 | 24 | 08:30 | 08:50 | 4 |
| 11 | 06:00 | 06:10 | 3 | 25 | 06:00 | 06:30 | 5 |
| 12 | 06:20 | 06:30 | 3 | 26 | 06:40 | 07:10 | 5 |
| 13 | 06:40 | 06:50 | 3 | 27 | 07:20 | 07:50 | 5 |
| 14 | 07:00 | 07:10 | 3 | 28 | 08:00 | 08:30 | 5 |

# Appendix E—The schedule with 8 routes, short headway, and long round-trip time

| Task | Starting Time | Ending Time | Route | Task | Starting Time | Ending Time | Route |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 06:00 | 07:00 | 1 | 43 | 06:50 | 07:50 | 4 |
| 2 | 07:10 | 08:10 | 1 | 44 | 08:00 | 09:00 | 4 |
| 3 | 08:20 | 09:20 | 1 | 45 | 07:00 | 08:00 | 4 |
| 4 | 06:10 | 07:10 | 1 | 46 | 08:10 | 09:10 | 4 |
| 5 | 07:20 | 08:20 | 1 | 47 | 06:00 | 07:10 | 5 |
| 6 | 08:30 | 09:30 | 1 | 48 | 07:20 | 08:30 | 5 |
| 7 | 06:20 | 07:20 | 1 | 49 | 06:20 | 07:30 | 5 |
| 8 | 07:30 | 08:30 | 1 | 50 | 07:40 | 08:50 | 5 |
| 9 | 06:30 | 07:30 | 1 | 51 | 06:40 | 07:50 | 5 |
| 10 | 07:40 | 08:40 | 1 | 52 | 08:00 | 09:10 | 5 |
| 11 | 06:40 | 07:40 | 1 | 53 | 07:00 | 08:10 | 5 |
| 12 | 07:50 | 08:50 | 1 | 54 | 08:20 | 09:30 | 5 |
| 13 | 06:50 | 07:50 | 1 | 55 | 06:00 | 07:20 | 6 |
| 14 | 08:00 | 09:00 | 1 | 56 | 07:30 | 08:50 | 6 |
| 15 | 07:00 | 08:00 | 1 | 57 | 06:30 | 07:50 | 6 |
| 16 | 08:10 | 09:10 | 1 | 58 | 08:00 | 09:20 | 6 |
| 17 | 06:00 | 07:10 | 2 | 59 | 07:00 | 08:20 | 6 |
| 18 | 07:20 | 08:30 | 2 | 60 | 08:30 | 09:50 | 6 |
| 19 | 06:20 | 07:30 | 2 | 61 | 06:00 | 07:00 | 7 |
| 20 | 07:40 | 08:50 | 2 | 62 | 07:10 | 08:10 | 7 |
| 21 | 06:40 | 07:50 | 2 | 63 | 08:20 | 09:20 | 7 |
| 22 | 08:00 | 09:10 | 2 | 64 | 06:10 | 07:10 | 7 |
| 23 | 07:00 | 08:10 | 2 | 65 | 07:20 | 08:20 | 7 |
| 24 | 08:20 | 09:30 | 2 | 66 | 08:30 | 09:30 | 7 |
| 25 | 06:00 | 07:20 | 3 | 67 | 06:20 | 07:20 | 7 |
| 26 | 07:30 | 08:50 | 3 | 68 | 07:30 | 08:30 | 7 |
| 27 | 06:30 | 07:50 | 3 | 69 | 06:30 | 07:30 | 7 |
| 28 | 08:00 | 09:20 | 3 | 70 | 07:40 | 08:40 | 7 |
| 29 | 07:00 | 08:20 | 3 | 71 | 06:40 | 07:40 | 7 |
| 30 | 08:30 | 09:50 | 3 | 72 | 07:50 | 08:50 | 7 |
| 31 | 06:00 | 07:00 | 4 | 73 | 06:50 | 07:50 | 7 |
| 32 | 07:10 | 08:10 | 4 | 74 | 08:00 | 09:00 | 7 |
| 33 | 08:20 | 09:20 | 4 | 75 | 07:00 | 08:00 | 7 |
| 34 | 06:10 | 07:10 | 4 | 76 | 08:10 | 09:10 | 7 |
| 35 | 07:20 | 08:20 | 4 | 77 | 06:00 | 07:10 | 8 |
| 36 | 08:30 | 09:30 | 4 | 78 | 07:20 | 08:30 | 8 |
| 37 | 06:20 | 07:20 | 4 | 79 | 06:20 | 07:30 | 8 |
| 38 | 07:30 | 08:30 | 4 | 80 | 07:40 | 08:50 | 8 |
| 39 | 06:30 | 07:30 | 4 | 81 | 06:40 | 07:50 | 8 |
| 40 | 07:40 | 08:40 | 4 | 82 | 08:00 | 09:10 | 8 |
| 41 | 06:40 | 07:40 | 4 | 83 | 07:00 | 08:10 | 8 |
| 42 | 07:50 | 08:50 | 4 | 84 | 08:20 | 09:30 | 8 |

# Appendix F—The schedule with 8 routes, short headway, and short round-trip time

| Task | Starting Time | Ending Time | Route | Task | Starting Time | Ending Time | Route |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 06:00 | 06:20 | 1 | 45 | 06:50 | 07:10 | 4 |
| 2 | 06:30 | 06:50 | 1 | 46 | 07:20 | 07:40 | 4 |
| 3 | 07:00 | 07:20 | 1 | 47 | 07:50 | 08:10 | 4 |
| 4 | 07:30 | 07:50 | 1 | 48 | 08:20 | 08:40 | 4 |
| 5 | 08:00 | 08:20 | 1 | 49 | 06:00 | 06:30 | 5 |
| 6 | 08:30 | 08:50 | 1 | 50 | 06:40 | 07:10 | 5 |
| 7 | 06:10 | 06:30 | 1 | 51 | 07:20 | 07:50 | 5 |
| 8 | 06:40 | 07:00 | 1 | 52 | 08:00 | 08:30 | 5 |
| 9 | 07:10 | 07:30 | 1 | 53 | 06:20 | 06:50 | 5 |
| 10 | 07:40 | 08:00 | 1 | 54 | 07:00 | 07:30 | 5 |
| 11 | 08:10 | 08:30 | 1 | 55 | 07:40 | 08:10 | 5 |
| 12 | 06:20 | 06:40 | 1 | 56 | 08:20 | 08:50 | 5 |
| 13 | 06:50 | 07:10 | 1 | 57 | 06:00 | 06:40 | 6 |
| 14 | 07:20 | 07:40 | 1 | 58 | 06:50 | 07:30 | 6 |
| 15 | 07:50 | 08:10 | 1 | 59 | 07:40 | 08:20 | 6 |
| 16 | 08:20 | 08:40 | 1 | 60 | 08:30 | 09:10 | 6 |
| 17 | 06:00 | 06:30 | 2 | 61 | 06:30 | 07:10 | 6 |
| 18 | 06:40 | 07:10 | 2 | 62 | 07:20 | 08:00 | 6 |
| 19 | 07:20 | 07:50 | 2 | 63 | 08:10 | 08:50 | 6 |
| 20 | 08:00 | 08:30 | 2 | 64 | 06:00 | 06:50 | 7 |
| 21 | 06:30 | 07:00 | 2 | 65 | 07:00 | 07:50 | 7 |
| 22 | 07:10 | 07:40 | 2 | 66 | 08:00 | 08:50 | 7 |
| 23 | 07:50 | 08:20 | 2 | 67 | 06:10 | 07:00 | 7 |
| 24 | 08:30 | 09:00 | 2 | 68 | 07:10 | 08:00 | 7 |
| 25 | 06:00 | 06:10 | 3 | 69 | 08:10 | 09:00 | 7 |
| 26 | 06:20 | 06:30 | 3 | 70 | 06:20 | 07:10 | 7 |
| 27 | 06:40 | 06:50 | 3 | 71 | 07:20 | 08:10 | 7 |
| 28 | 07:00 | 07:10 | 3 | 72 | 08:20 | 09:10 | 7 |
| 29 | 07:20 | 07:30 | 3 | 73 | 06:30 | 07:20 | 7 |
| 30 | 07:40 | 07:50 | 3 | 74 | 07:30 | 08:20 | 7 |
| 31 | 08:00 | 08:10 | 3 | 75 | 08:30 | 09:20 | 7 |
| 32 | 08:20 | 08:30 | 3 | 76 | 06:40 | 07:30 | 7 |
| 33 | 06:00 | 06:20 | 4 | 77 | 07:40 | 08:30 | 7 |
| 34 | 06:30 | 06:50 | 4 | 78 | 06:50 | 07:40 | 7 |
| 35 | 07:00 | 07:20 | 4 | 79 | 07:50 | 08:40 | 7 |
| 36 | 07:30 | 07:50 | 4 | 80 | 06:00 | 06:10 | 8 |
| 37 | 08:00 | 08:20 | 4 | 81 | 06:20 | 06:30 | 8 |
| 38 | 08:30 | 08:50 | 4 | 82 | 06:40 | 06:50 | 8 |
| 39 | 06:10 | 06:30 | 4 | 83 | 07:00 | 07:10 | 8 |
| 40 | 06:40 | 07:00 | 4 | 84 | 07:20 | 07:30 | 8 |
| 41 | 07:10 | 07:30 | 4 | 85 | 07:40 | 07:50 | 8 |
| 42 | 07:40 | 08:00 | 4 | 86 | 08:00 | 08:10 | 8 |
| 43 | 08:10 | 08:30 | 4 | 87 | 08:20 | 08:30 | 8 |
| 44 | 06:20 | 06:40 | 4 |  |  |  |  |

# Appendix G—The schedule with 8 routes, long headway, and long round-trip time

| Task | Starting Time | Ending Time | Route | Task | Starting Time | Ending Time | Route |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 06:00 | 07:00 | 1 | 19 | 06:00 | 07:10 | 5 |
| 2 | 07:10 | 08:10 | 1 | 20 | 07:20 | 08:30 | 5 |
| 3 | 08:20 | 09:20 | 1 | 21 | 06:40 | 07:50 | 5 |
| 4 | 06:50 | 07:50 | 1 | 22 | 08:00 | 09:10 | 5 |
| 5 | 08:00 | 09:00 | 1 | 23 | 06:00 | 07:20 | 6 |
| 6 | 06:00 | 07:10 | 2 | 24 | 07:30 | 08:50 | 6 |
| 7 | 07:20 | 08:30 | 2 | 25 | 06:50 | 08:10 | 6 |
| 8 | 06:40 | 07:50 | 2 | 26 | 08:20 | 09:40 | 6 |
| 9 | 08:00 | 09:10 | 2 | 27 | 06:00 | 07:00 | 7 |
| 10 | 06:00 | 07:20 | 3 | 28 | 07:10 | 08:10 | 7 |
| 11 | 07:30 | 08:50 | 3 | 29 | 08:20 | 09:20 | 7 |
| 12 | 06:50 | 08:10 | 3 | 30 | 07:00 | 08:00 | 7 |
| 13 | 08:20 | 09:40 | 3 | 31 | 08:10 | 09:10 | 7 |
| 14 | 06:00 | 07:00 | 4 | 32 | 06:00 | 07:10 | 8 |
| 15 | 07:10 | 08:10 | 4 | 33 | 07:20 | 08:30 | 8 |
| 16 | 08:20 | 09:20 | 4 | 34 | 06:40 | 07:50 | 8 |
| 17 | 07:00 | 08:00 | 4 | 35 | 08:00 | 09:10 | 8 |
| 18 | 08:10 | 09:10 | 4 |  |  |  |  |

# Appendix H—The schedule with 8 routes, long headway, and short round-trip time

| Task | Starting Time | Ending Time | Route | Task | Starting Time | Ending Time | Route |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 06:00 | 06:20 | 1 | 23 | 08:00 | 08:20 | 4 |
| 2 | 06:30 | 06:50 | 1 | 24 | 08:30 | 08:50 | 4 |
| 3 | 07:00 | 07:20 | 1 | 25 | 06:00 | 06:30 | 5 |
| 4 | 07:30 | 07:50 | 1 | 26 | 06:40 | 07:10 | 5 |
| 5 | 08:00 | 08:20 | 1 | 27 | 07:20 | 07:50 | 5 |
| 6 | 08:30 | 08:50 | 1 | 28 | 08:00 | 08:30 | 5 |
| 7 | 06:00 | 06:30 | 2 | 29 | 06:00 | 06:40 | 6 |
| 8 | 06:40 | 07:10 | 2 | 30 | 06:50 | 07:30 | 6 |
| 9 | 07:20 | 07:50 | 2 | 31 | 07:40 | 08:20 | 6 |
| 10 | 08:00 | 08:30 | 2 | 32 | 08:30 | 09:10 | 6 |
| 11 | 06:00 | 06:10 | 3 | 33 | 06:00 | 06:50 | 7 |
| 12 | 06:20 | 06:30 | 3 | 34 | 07:00 | 07:50 | 7 |
| 13 | 06:40 | 06:50 | 3 | 35 | 08:00 | 08:50 | 7 |
| 14 | 07:00 | 07:10 | 3 | 36 | 06:00 | 06:10 | 8 |
| 15 | 07:20 | 07:30 | 3 | 37 | 06:20 | 06:30 | 8 |
| 16 | 07:40 | 07:50 | 3 | 38 | 06:40 | 06:50 | 8 |
| 17 | 08:00 | 08:10 | 3 | 39 | 07:00 | 07:10 | 8 |
| 18 | 08:20 | 08:30 | 3 | 40 | 07:20 | 07:30 | 8 |
| 19 | 06:00 | 06:20 | 4 | 41 | 07:40 | 07:50 | 8 |
| 20 | 06:30 | 06:50 | 4 | 42 | 08:00 | 08:10 | 8 |
| 21 | 07:00 | 07:20 | 4 | 43 | 08:20 | 08:30 | 8 |
| 22 | 07:30 | 07:50 | 4 |  |  |  |  |

# Appendix I—The test schedule in adjustable parameters experiments

| Task | Starting Time | Ending Time | Route | Task | Starting Time | Ending Time | Route |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 08:00 | 08:40 | 1 | 94 | 10:10 | 10:50 | 7 |
| 2 | 08:50 | 09:30 | 1 | 95 | 11:00 | 11:40 | 7 |
| 3 | 09:40 | 10:20 | 1 | 96 | 08:40 | 09:20 | 7 |
| 4 | 10:30 | 11:10 | 1 | 97 | 09:30 | 10:10 | 7 |
| 5 | 11:20 | 12:00 | 1 | 98 | 10:20 | 11:00 | 7 |
| 6 | 08:10 | 08:50 | 1 | 99 | 11:10 | 11:50 | 7 |
| 7 | 09:00 | 09:40 | 1 | 100 | 08:50 | 09:30 | 7 |
| 8 | 09:50 | 10:30 | 1 | 101 | 09:40 | 10:20 | 7 |
| 9 | 10:40 | 11:20 | 1 | 102 | 10:30 | 11:10 | 7 |
| 10 | 11:30 | 12:10 | 1 | 103 | 11:20 | 12:00 | 7 |
| 11 | 08:20 | 09:00 | 1 | 104 | 09:00 | 09:40 | 7 |
| 12 | 09:10 | 09:50 | 1 | 105 | 09:50 | 10:30 | 7 |
| 13 | 10:00 | 10:40 | 1 | 106 | 10:40 | 11:20 | 7 |
| 14 | 10:50 | 11:30 | 1 | 107 | 11:30 | 12:10 | 7 |
| 15 | 08:30 | 09:10 | 1 | 108 | 09:10 | 09:50 | 7 |
| 16 | 09:20 | 10:00 | 1 | 109 | 10:00 | 10:40 | 7 |
| 17 | 10:10 | 10:50 | 1 | 110 | 10:50 | 11:30 | 7 |
| 18 | 11:00 | 11:40 | 1 | 111 | 08:20 | 08:50 | 8 |
| 19 | 08:40 | 09:20 | 1 | 112 | 09:00 | 09:30 | 8 |
| 20 | 09:30 | 10:10 | 1 | 113 | 09:40 | 10:10 | 8 |
| 21 | 10:20 | 11:00 | 1 | 114 | 10:20 | 10:50 | 8 |
| 22 | 11:10 | 11:50 | 1 | 115 | 11:00 | 11:30 | 8 |
| 23 | 08:20 | 09:10 | 2 | 116 | 08:30 | 09:00 | 8 |
| 24 | 09:20 | 10:10 | 2 | 117 | 09:10 | 09:40 | 8 |
| 25 | 10:20 | 11:10 | 2 | 118 | 09:50 | 10:20 | 8 |
| 26 | 11:20 | 12:10 | 2 | 119 | 10:30 | 11:00 | 8 |
| 27 | 08:40 | 09:30 | 2 | 120 | 08:40 | 09:10 | 8 |
| 28 | 09:40 | 10:30 | 2 | 121 | 09:20 | 09:50 | 8 |
| 29 | 10:40 | 11:30 | 2 | 122 | 10:00 | 10:30 | 8 |
| 30 | 09:00 | 09:50 | 2 | 123 | 10:40 | 11:10 | 8 |
| 31 | 10:00 | 10:50 | 2 | 124 | 08:50 | 09:20 | 8 |
| 32 | 11:00 | 11:50 | 2 | 125 | 09:30 | 10:00 | 8 |
| 33 | 09:00 | 09:30 | 3 | 126 | 10:10 | 10:40 | 8 |
| 34 | 09:40 | 10:10 | 3 | 127 | 10:50 | 11:20 | 8 |
| 35 | 10:20 | 10:50 | 3 | 128 | 08:50 | 09:30 | 9 |
| 36 | 11:00 | 11:30 | 3 | 129 | 09:40 | 10:20 | 9 |
| 37 | 09:10 | 09:40 | 3 | 130 | 10:30 | 11:10 | 9 |
| 38 | 09:50 | 10:20 | 3 | 131 | 11:20 | 12:00 | 9 |
| 39 | 10:30 | 11:00 | 3 | 132 | 09:10 | 09:50 | 9 |
| 40 | 11:10 | 11:40 | 3 | 133 | 10:00 | 10:40 | 9 |
| 41 | 09:20 | 09:50 | 3 | 134 | 10:50 | 11:30 | 9 |
| 42 | 10:00 | 10:30 | 3 | 135 | 09:30 | 10:10 | 9 |
| 43 | 10:40 | 11:10 | 3 | 136 | 10:20 | 11:00 | 9 |
| 44 | 11:20 | 11:50 | 3 | 137 | 11:10 | 11:50 | 9 |
| 45 | 09:30 | 10:00 | 3 | 138 | 08:10 | 08:50 | 10 |
| 46 | 10:10 | 10:40 | 3 | 139 | 09:00 | 09:40 | 10 |
| 47 | 10:50 | 11:20 | 3 | 140 | 09:50 | 10:30 | 10 |
| 48 | 11:30 | 12:00 | 3 | 141 | 10:40 | 11:20 | 10 |
| 49 | 09:00 | 09:50 | 4 | 142 | 11:30 | 12:10 | 10 |
| 50 | 10:00 | 10:50 | 4 | 143 | 08:20 | 09:00 | 10 |
| 51 | 11:00 | 11:50 | 4 | 144 | 09:10 | 09:50 | 10 |
| 52 | 09:10 | 10:00 | 4 | 145 | 10:00 | 10:40 | 10 |
| 53 | 10:10 | 11:00 | 4 | 146 | 10:50 | 11:30 | 10 |
| 54 | 11:10 | 12:00 | 4 | 147 | 08:30 | 09:10 | 10 |
| 55 | 09:20 | 10:10 | 4 | 148 | 09:20 | 10:00 | 10 |
| 56 | 10:20 | 11:10 | 4 | 149 | 10:10 | 10:50 | 10 |
| 57 | 11:20 | 12:10 | 4 | 150 | 11:00 | 11:40 | 10 |
| 58 | 09:30 | 10:20 | 4 | 151 | 08:40 | 09:20 | 10 |
| 59 | 10:30 | 11:20 | 4 | 152 | 09:30 | 10:10 | 10 |
| 60 | 11:30 | 12:20 | 4 | 153 | 10:20 | 11:00 | 10 |
| 61 | 09:40 | 10:30 | 4 | 154 | 11:10 | 11:50 | 10 |
| 62 | 10:40 | 11:30 | 4 | 155 | 08:50 | 09:30 | 10 |
| 63 | 09:50 | 10:40 | 4 | 156 | 09:40 | 10:20 | 10 |
| 64 | 10:50 | 11:40 | 4 | 157 | 10:30 | 11:10 | 10 |
| 65 | 08:40 | 09:20 | 5 | 158 | 11:20 | 12:00 | 10 |
| 66 | 09:30 | 10:10 | 5 | 159 | 09:00 | 09:30 | 11 |
| 67 | 10:20 | 11:00 | 5 | 160 | 09:40 | 10:10 | 11 |
| 68 | 11:10 | 11:50 | 5 | 161 | 10:20 | 10:50 | 11 |
| 69 | 09:00 | 09:40 | 5 | 162 | 11:00 | 11:30 | 11 |
| 70 | 09:50 | 10:30 | 5 | 163 | 09:10 | 09:40 | 11 |
| 71 | 10:40 | 11:20 | 5 | 164 | 09:50 | 10:20 | 11 |
| 72 | 11:30 | 12:10 | 5 | 165 | 10:30 | 11:00 | 11 |
| 73 | 09:20 | 10:00 | 5 | 166 | 11:10 | 11:40 | 11 |
| 74 | 10:10 | 10:50 | 5 | 167 | 09:20 | 09:50 | 11 |
| 75 | 11:00 | 11:40 | 5 | 168 | 10:00 | 10:30 | 11 |
| 76 | 09:00 | 09:50 | 6 | 169 | 10:40 | 11:10 | 11 |
| 77 | 10:00 | 10:50 | 6 | 170 | 11:20 | 11:50 | 11 |
| 78 | 11:00 | 11:50 | 6 | 171 | 09:30 | 10:00 | 11 |
| 79 | 09:10 | 10:00 | 6 | 172 | 10:10 | 10:40 | 11 |
| 80 | 10:10 | 11:00 | 6 | 173 | 10:50 | 11:20 | 11 |
| 81 | 11:10 | 12:00 | 6 | 174 | 11:30 | 12:00 | 11 |
| 82 | 09:20 | 10:10 | 6 | 175 | 08:30 | 09:10 | 12 |
| 83 | 10:20 | 11:10 | 6 | 176 | 09:20 | 10:00 | 12 |
| 84 | 11:20 | 12:10 | 6 | 177 | 10:10 | 10:50 | 12 |
| 85 | 09:30 | 10:20 | 6 | 178 | 11:00 | 11:40 | 12 |
| 86 | 10:30 | 11:20 | 6 | 179 | 08:50 | 09:30 | 12 |
| 87 | 11:30 | 12:20 | 6 | 180 | 09:40 | 10:20 | 12 |
| 88 | 09:40 | 10:30 | 6 | 181 | 10:30 | 11:10 | 12 |
| 89 | 10:40 | 11:30 | 6 | 182 | 11:20 | 12:00 | 12 |
| 90 | 09:50 | 10:40 | 6 | 183 | 09:10 | 09:50 | 12 |
| 91 | 10:50 | 11:40 | 6 | 184 | 10:00 | 10:40 | 12 |
| 92 | 08:30 | 09:10 | 7 | 185 | 10:50 | 11:30 | 12 |
| 93 | 09:20 | 10:00 | 7 |  |  |  |  |