摘   要

為使國土區域得以均衡發展，在進行公路路網投資決策時應同時兼顧效率與公平的課題。然而以往的相關研究多偏重效率卻忽略公平，因此本研究乃以發展一套實用且兼顧效率與公平的路網投資決策方法為目的。

在路網投資時，應提供各區域主要都市的民眾到該區域中心之合理可及性以滿足生活所需。由兼顧效率與公平的觀點，不僅要針對各主要都市至區域中心的可及性予以最佳化以達成旅運成本面的效率，亦需同時令各區域內都市間該項可及性的差異及區域間該項可及性的差異最小化以達成水平式與垂直式的公平。基於此觀念，本研究首先提出多目標之基本型模式，再予以改良提出修正型模式以使得公路投資決策程序更為合理與實用。

本研究以台灣西部公路網為例，以上述模式進行多種情境之示範操作與分析。決策者或管理者可藉以了解在不同的總經費預算、預期行駛速率水準、路段改善的互補關係與互斥方案等各種假設下對公路投資的影響。由研究結果顯示，以模糊規劃法求解各案例之多目標模式，皆可有效獲得兼顧效率與公平的妥協解。

本研究亦探討公路投資預算分配的公平課題，其方法是在模式中依據各都市的社經特性給定預算的比例上限作為其限制式。此外，為擴充修正型模式的使用彈性，亦探討模式中可及性指標的替代方案如旅運需求的消費者剩餘。最後亦嘗試將模式中的公平目標式予以簡化，提出旅運時間標的模式與旅運速率標的模式。並以時速六十公里為標的，示範給定多目標妥協度下最小預算之求解，以展示更先進的決策程序。

關鍵詞: 模糊規劃法、路網投資決策、效率、可及性、水平式的公平、垂直式的公平、妥協解

ABSTRACT

To facilitate equal regional development, the decision-making of highway investment should consider not only cost efficiency but also issue of equity, which is seldom mentioned in past researches. The goal of this research is to develop a practical method for highway investment decision-making to deal with the issues of both efficiency and equity.

In highway investment, we should provide people reasonable accessibility from every main city in every region to the regional center. We could optimize the accessibility for main cities in view of efficiency, and simultaneously minimize the difference in accessibility for main cities in the same region and to minimize that difference among regions to maintain both horizontal equity and vertical equity. Based on this concept, the basic model with multi-objective formulation was proposed first in this research and then modified as a revised model to make the decision-making process more reasonable and practical.

Case studies of highway investment for western Taiwan were formulated in both models with different scenarios to show the impact of assumptions in future travel speed, total budget, complementary property among links, and exclusive property among alternatives. Using the fuzzy programming, the revised model effectively yielded the compromised solutions for all the cases in view of efficiency and equity.

The research also covers the equity in budget allocation among cities, which was processed by restricting the upper bound of the budget share for each main city according to its socio-economic attributes. Besides, to expand the revised model for more flexible use, alternative travel cost/benefit measures were discussed and consumers’ surplus in elastic travel demand was proposed for the future study. For the simplification of objectives in equity, the travel-time target model and travel-speed target model were proposed with alternative equity measures. Furthermore, for the travel-speed target model with a target of 60 km/hr, each minimal budget was obtained in terms of given compromise grades to demonstrate the advanced decision-making process.

Keywords: fuzzy programming; highway investment decision-making; efficiency; accessibility; horizontal equity; vertical equity; compromised solution

### CONTENTS

ABSTRACT (in Chinese)…………………………...……………………………..i

ABSTRACT……………………………………………………………………….ii

CONTENTS………………………………………………………………………iii

TABLE CONTENTS..…………………………………………………………….vi

FIGURE CONTENTS..………………………………………………………….viii

### CHAPTER 1  INTRODUCTION………………………………………………..1

1.1  Research Background and Objective.……………………………….1

1.2  Concept of Equity…………………………………………………...1

1.3  Research Scope…………………………………………………..….2

1.4  Research Approach……………………..………………………..….3

1.5  Dissertation Framework…………………………………………….4

### CHAPTER 2  LITERATURE REVIEW……………………...…...…………….5

2.1  Past Researches for Highway Investment Decision-Making…….....5

2.2  Efficiency-Equity Trade-off in Facility Location Problem…………6

2.3  Features of Fuzzy Programming……………………..……………..7

### CHAPTER 3  BASIC MODEL AND FUZZY PROGRAMMING…………….10

3.1  Problem Statements and Basic Concepts…………………..……....10

3.2  Formulation of Basic model………………………………………..11

3.3  Solution Algorithm: Fuzzy Programming…………………………14

3.4  Properties of Basic Model……………………………………….....16

3.5  Test with a Simple Case……………………………………………20

### CHAPTER 4  DEVELOPMENT OF REVISED MODEL…………………….33

4.1  Weaknesses of Basic Model……………………………………….33

4.1.1 Undefined Alternatives of Exclusion and Complement…...33

4.1.2 Unreasonable Travel Cost Measure for Equity…...……….34

4.1.3 Overestimated Degree of Satisfaction for Equity……..…35

4.2  Development of Revised Model…………………………………...37

4.2.1 Define Exclusion and Complement Properties………..…..37

4.2.2 Adjust Travel Cost Measures in Equity Objectives……….38

4.2.3 Justify Objective Formulas with Membership Functions…38

4.3  Formulation of the Revised Model………………………………..39

4.4  Summary of Key Features of Revised Model……………………..41

### CHAPTER 5  CASE STUDIES……………………………………………….42

5.1  Usage of Geographic Information System………………………..42

5.2  Process of Case Study…………………………………………….50

5.3  Analysis of Results………………………………………………..60

5.4  More Cases of Revised Model……………………………………68

5.4.1 Upgrade the Assumed Future Travel Speed………………68

5.4.2 Raise the Total Budget of Highway Improvement………..71

5.4.3 Include Complementary Property among Links………….73

5.4.4 Build Exclusive Property among Alternatives……………75

### CHAPTER 6  EXPANSION OF REVISED MODEL………………………...83

6.1 Equity in Allocation of Budget for Highway Investment…………..83

6.2 Alternatives of Travel Cost/Benefit Measure……………………….88

6.2.1 Using Travel Time Weighted by Fixed Demand………….88

6.2.2 Using Travel Time Weighted by Elastic Demand.……......89

6.2.3 Using Consumers’ Surplus as Travel Benefit Measure…..90

6.3 Alternative Equity Objectives………………………………………92

CHAPTER 7  CONCLUSIONS AND SUGGESTIONS……………………...97

7.1 Conclusions………………………………………………………….97

7.2 Suggestions………………………………………………………….99

### REFERENCES………..………………………………….…………………….101

### APPENDIX CONTENTS..……………………………….……………….…...105

### TABLE CONTENTS

### TABLE 3.1  Propositions for Link Selection in View of Efficiency………...…17

### TABLE 3.2  Characteristics of Simple Case for Test…………….…………….20

### TABLE 3.3  Link Attributes in Simple Case for Basic Model…………………21

### TABLE 3.4  Excel Worksheet for Solving Simple Case (Initial State)………...26

### TABLE 3.5  Excel Worksheet for Solving Simple Case (Min W1)………….….27

### TABLE 3.6  Excel Worksheet for Solving Simple Case (Min W2)……………..28

TABLE 3.7  Excel Worksheet for Solving Simple Case (Min W3)……………..29

TABLE 3.8  Excel Worksheet for Solving Simple Case (Max λ)……………....30

### TABLE 3.9  Summary of Results for Simple Case…………….….……………31

### TABLE 3.10  Squared Intra-regional Difference for each Region……………...32

### TABLE 3.11  Squared Inter-regional Difference for each Region……………...32

### TABLE 5.1  The Regions and Main Cities in the Western Taiwan……………..42

### TABLE 5.2  Link Attributes for each Shortest Path…………………………....46

### TABLE 5.3  Parameters for the Input of Both Models………………….……...50

### TABLE 5.4A  Summary of Results from the Basic Model for Case A…...……52

TABLE 5.4B  Summary of Results from the Basic Model for Case B………...53

### TABLE 5.5A  Summary of Results from the Revised Model for Case A……...54

TABLE 5.5B  Summary of Results from the Revised Model for Case B….…..55

TABLE 5.6  Values of Decision Variables in Case A and CaseB……………....56

### TABLE 5.7.  Effects of Revised Model on Regional Equity……..…………….62

TABLE 5.8A  Comparison of Travel Time in Case A between Basic Model

and Revised Model…..……….………………………………….……63

### TABLE 5.8B  Comparison of Travel Time in Case B between Basic Model

### and Revised Model…..…………………………………………….….64

TABLE 5.9A  Comparison of Average Travel Speed in Case A between

Basic Model and Revised Model…..……….……………….……..…65

TABLE 5.9B  Comparison of Average Travel Speed in Case B between

Basic Model and Revised Model…..……….……………………...…66

TABLE 5.10  Summary of Results from the Revised Model for Case C…..….70

TABLE 5.11  Summary of Results from the Revised Model for Case D….…..72

TABLE 5.12  Summary of Results from the Revised Model for Case E….…..74

TABLE 5.13  Summary of Results from the Revised Model for Case F….…..77

TABLE 5.14  Intra-regional Difference in Case A, C, D, E and F…………….78

TABLE 5.15  Inter-regional Difference in Case A, C, D, E and F…………….78

TABLE 5.16  Values of Decision Variables in Case C, D, E and F……………79

TABLE 6.1  Upper Bound of Budget Share for Each City…………………….86

TABLE 6.2  Revised model with/without Budget Share Constraints………….87

TABLE 6.3  Summary of Results from Both Target models for Case A………94

TABLE 6.4  Intra-regional Difference in travel speed  ………………………95

TABLE 6.5  Inter-regional Difference in travel speed  ………………………95

TABLE 6.6  Budget Required for Given Compromise Grades………………...96

### FIGURE CONTENTS

### FIG. 3.1  Shape of Simple Networks for Test…………………………………..20

### FIG. 3.2  Procedure for Min W1 by SOLVER( Step 1)…………………………24

### FIG. 3.3  Procedure for Min W1 by SOLVER( Step 2)…………………………25

### FIG. 3.4  Procedure for Min W1 by SOLVER( Step 3)…………………………25

### FIG. 4.1  Degree of Satisfaction DSs ( Ws ) is less than DSs' ( Ws' )…………….36

### FIG. 5.1  Highway Network in Taiwan………………………………………...44

### FIG. 5.2  Shortest Paths from Main Cities to Regional Centers………..………45

### FIG. 5.3  Comparison between Results of Basic model and Revised Model…..67

### FIG 6.1  Consumers’ Surplus in Travel Demand………………………………91