TABLE 3.9  Summary of Results for Simple Case

| Process Results | Initial State | Min W1 | Min W2 | Min W3 | Maxλ |
| --- | --- | --- | --- | --- | --- |
| Budget used | 0 | 200 | 100 | 100 | 200 |
| Time saved | 0 | 50 | 25 | 15 | 45 |
| Value of W1 (minutes) | 66.25 | *53.75 | 60 | 62.5 | 55 |
| Value of W2 (minutes)2 | 53.125 | 115.625 | *6.25 | 156.25 | 31.25 |
| Value of W3 (minutes)2 | 14.0625 | 76.5625 | 6.25 | *0 | 6.25 |
| DS1 |  | 1 |  | 0 | 0.8571 |
| DS2 |  |  | 1 | 0 | 0.8333 |
| DS3 |  | 0 |  | 1 | 0.9184 |

TABLE 3.10 Squared Intra-regional Difference for each Region (minutes)2

| Process Results | Initial State | Min W1 | Min W2 | Min W3 | Maxλ |
| --- | --- | --- | --- | --- | --- |
| Region 1 | 100 =(10) 2 | 225 =(15 )2 | 6.25 =(2.5) 2 | 306.25 =(17.5) 2 | 6.25 =(2.5) 2 |
| Region 2 | 6.25 =(2.5) 2 | 6.25 =(2.5) 2 | 6.25 =(2.5) 2 | 6.25 =(2.5) 2 | 56.25 =(7.5) 2 |

TABLE 3.11 Squared Inter-regional Difference for each Region (minutes)2

| Process Results | Initial State | Min W1 | Min W2 | Min W3 | Maxλ |
| --- | --- | --- | --- | --- | --- |
| Region 1 | 14.0625 =(3.75) 2 | 76.5625 =(-8.75) 2 | 6.25 =(-2.5) 2 | 0 | 6.25 =(2.5) 2 |
| Region 2 | 14.0625 =(-3.75) 2 | 76.5625 =(8.75) 2 | 6.25 =(2.5) 2 | 0 | 6.25 =(-2.5) 2 |

### CHAPTER 4

### DEVELOPMENT OF REVISED MODEL

The revised model is developed to make the highway investment decision-making process more reasonable and practical than the basic model. For the improvements of basic model, its weakness is reviewed first in 4.1. The modifications are described in 4.2. The formulation and key features of the revised model are presented and listed in section 4.3 and 4.4, respectively.

4.1 Weakness of Basic Model

The weakness of basic model covers: (1) The exclusion and complement properties among alternatives are not defined; (2) The travel time is not a reasonable travel cost measure for equity objectives in general case; and (3) The degree of satisfaction for the achievement of the equity is overestimated. They are reviewed in 4.1.1~4.1.3.

4.1.1 Undefined Alternatives of Exclusion and Complement

In the basic model, the shortest path connecting each main city to the regional center is generated first based on the current travel time collected by real field survey. Then, the links of those shortest paths are treated as the alternatives for improvement by default. Alternatively, any given set of links that serve as the substitute for the shortest path, can be allowed to be the alternatives for improvement or addition. However, due to the lack of definition of exclusive alternatives in the basic model, we cannot evaluate those alternatives for the shortest path for a city in the model at the same time.

Practically, we should put all the alternatives together in the mathematical model for evaluation to optimize the objectives. Consequently, we need to revise the model to accommodate the exclusion property among shortest paths for each main city.

Besides, some relative links should be improved together to raise the highway performance, especially for some adjacent links. They must be defined as complementary links in the model to generate reasonable solution.

After those modifications, the links of shortest path and other substitution paths could be simultaneously evaluated in the model and those relative links could be improved together.

4.1.2 Unreasonable Travel Cost Measure for Equity

In the basic model, travel time serves as the travel cost measure in all of the objectives for cost efficiency, horizontal equity and vertical equity. However, the travel time is not always a good measure for horizontal (intra-regional) equity, especially for big differences in travel distance from every city to the regional center. It is only suitable for the special case where the regional center is also the geographic center with equal distance from cities in that region. In general case, to minimize the difference in travel time from each city to the regional center is not reasonable to achieve the horizontal equity.

For similar reasons, travel time is not an appropriate measure for vertical (inter-regional) equity especially when the area size of each region varies greatly. Average travel time from every city to the regional center is usually longer in a region with a large area than a region with a small area. Therefore, an adjustment for the travel cost measure in the basic model is necessary. We may use different travel cost measures in objectives for the horizontal equity and vertical equity for general cases.

4.1.3 Overestimated Degree of Satisfaction for Equity

In the basic model (M1), the objective functions for horizontal equity and vertical equity are formulated in the form of an ‘average square’ of the difference in travel cost measure. Actually, we intended to evaluate equity in terms of the ‘difference’, rather than the ‘square of the difference’ in travel cost measure. The reason for using the square operation is to eliminate the deduction effect of the signs (positive or negative) while accumulating items of difference for each city and region.

While using fuzzy programming for the basic model, the membership function DSs(Ws) defined in equation (1) indicates the degree of satisfaction for each Objective_Ws ( = Min Ws ). Using the form of ‘square’ in objective function Ws (s=2,3) will ‘overestimate’ the real degree of satisfaction for the achievement of that equity, as illustrated in Fig.4.1. Owing to the increasing and concave-up curve of square function Ws' = (Ws)2, the value of DSs(Ws) for the objective function Ws is less than the value of DS s'(Ws') for the objective function Ws'. The basic model should be refined to eliminate the overestimation in degree of satisfaction.

1

0

0

Ws#

Ws *

Ws'#

# Ws'*

DSs(Ws)

DSs'(Ws')

1

Ws' = (Ws)2

(Ws,Ws')

FIG. 4.1 Degree of Satisfaction DSs ( Ws ) is less than DSs'( Ws' )

4.2 Development of Revised Model

Based on the review of weakness in basic model, the revised model is developed. More constraints and decision variables are added to the model to handle the exclusion and complement alternatives. Besides, adjustments for travel cost measures and objective functions are proposed to justify the horizontal equity and vertical equity. These modifications are illustrated in section 4.2.1~4.2.3.

4.2.1 Defining Exclusion and Complement Properties

To evaluate all the alternatives together to optimize the objectives, we revised the basic model by introducing the new decision variable yijk to accommodate the exclusion property among alternatives for the shortest path for a city. The variable yijk = 1 while the alternative k for the shortest path SPij connecting the city to the regional center is chosen; otherwise, yijk = 0. The exclusive property of alternatives is specified in the constraint Σk yijk = 1∀i,j. With this modification, we may simultaneously evaluate the links of all the substitution paths in the model and still follow the exclusion property among alternatives.

Besides, complementary alternatives should also be defined in the model to generate reasonable solutions. In real cases, relative links should be improved together to enhance highway performance, and they must be defined as complementary alternatives. Therefore, the new constraintsxl'=xl'',∃l',l''∈Cm,∀m were added to the model to handle the complementary property, Cm represents the set of relative links that should be improved or added together to raise highway performance. Judgment for the set of relative links is dependent on professional experience, and those sets could be given for the model.

4.2.2 Adjusting Travel Cost Measures in Equity Objectives

In the basic model, travel time is not always a good measure for horizontal (intra-regional) equity, especially for big difference in travel distance from every city to the regional center. For similar reasons, the travel time is not an appropriate measure for the vertical (inter-regional) equity, especially when the area size of each region varies greatly.

To revise the model, HTCMij and VTCMij respectively serve as the travel cost measure for horizontal equity and vertical equity instead of TCMij. Both HTCMij and VTCMij are given as Lij / TCMij where Lij is the length of the chosen shortest path SPij. After this adjustment, both horizontal equity and vertical equity are measured by average travel speed rather than travel time. Average travel speed is a good indicator of highway service quality and a reasonable measure of equity in highway investment, even for regions of various shapes or area sizes.

4.2.3 Justifying Objective Formulas with Membership Functions

As mentioned in 4.1.3, using the form of ‘square of the difference’ in travel cost measure in objective function Ws' (s=2,3) will ‘overestimate’ the real degree of satisfaction for the achievement of that equity. To refine the overestimated degree of satisfaction, we revised the objective function Ws' ( s=2,3) by applying operation of the square root (fractional exponent = 1/2) to their original formula; then the membership functions for these two objectives can genuinely reflect the extent to which horizontal equity and vertical equity can be achieved.

4.3 Formulation of the Revised Model

With the above modifications, formulation of the revised model (M2) becomes the following:

HTCMij =Lij / TCMij ; VTCMij =Lij / TCMij ; Lij =Σk Lijk · yijk

Objective:MIN1Ni1NijVTCMij−1NiijVTCMij212

Subject toTCMij=kl∈SPijkTlf⋅yijk=k

Σl Al · xl ≦ B

Objective:MIN1NiijHTCMij−1NijHTCMij212Objective:MIN1NiijTCMij		  xl ∈{0,1}, ∀l

yijk ∈{0,1} ∀i,j,k

Σk yijk = 1  ∀i,j

xl'=xl'',∃l',l''∈Cm,∀m

Notation:

Al : cost needed for improvement of link l

B : total budget

Cm: the mth set of given links that are complementary

Dl : length of link l

HTCMij: the adjusted travel cost measure for horizontal equity

Lij : the length of the chosen alternative of shortest path SPij.

Lijk : the length of shortest path SPijk

Ni : number of main cities in region i ( regional center is not counted )

N : number of regions in problem

Slf : future travel speed along link l while it is improved

SPij : the set of links that comprise shortest path connecting main city j of region i to regional center

SPijk : set of links that comprise alternative k for shortest path SPij

Tlc : current travel time along link l

Tlf : future travel time along link l while it is improved

Tls : saved travel time along link l while it is improved

( Tls = max(0 , Tlc -Tlf) = max(0 , Tlc - Dl / Slf ))

TCMij: travel cost measure for city j in region i

TTij : current travel time along SPij

VTCMij: adjusted travel cost measure for vertical equity

xl : decision variable for improvement of link l

yijk : decision variable for choice of alternative k of SPij

In the revised model, Objective_W1 is to optimize the cost efficiency measured by average travel time from all the main cities to the regional centers, and Objective_W2 and Objective_W3 aim to optimize the horizontal equity and vertical equity, which are measured respectively by intra-regional and inter-regional differences in average travel speed from every main city to the regional center.

Most of the symbols used in the revised model are the same as those of the basic model. The new symbol SPijk represents the set of links that comprise the alternative k for the shortest path connecting the main city j in region i to the regional center, Lijk represents the length of the shortest path SPijk and Lij is the length of the chosen alternative for SPij.

4.4 Summary of Key Features of Revised Model

In summary, the revised model has the following key features in formulation:

- With the new decision variables yijk ∈{0,1}, constraintsΣk yijk = 1 ∀i,j and xl'=xl'',∃l',l''∈Cm,∀m for handling the exclusive and complementary property among the alternatives, the model becomes more practical for the highway investment problem in the real world;

- With the adjusted travel cost measure HTCMij = VTCMij = Lij / TCMij, the horizontal equity and vertical equity are measured by the average travel speed, so the model becomes more flexible and reasonable for dealing with regions of various shapes or area sizes in highway investment planning; and

- With operation of the square root for both original equity objectives, membership functions can genuinely reflect the degree of satisfaction of horizontal equity and vertical equity. Based on the justified membership function, the model can yield reasonable compromise solutions in view of both efficiency and equity.

### CHAPTER 5

### CASE STUDIES

The basic model and revised model are tested in highway investment decision-making for western Taiwan, which covers three regions with four regional centers and 11 main cities (Table 5.1). In the case study, the real highway network stored in geographic information system is depicted in section 5.1. The process and the results of case study for both models are presented in section 5.2 and 5.3. More cases for revised model are processed with different scenarios in section 5.4. Decision maker could realize the impact of different assumptions on highway investment.

| TABLE 5.1 The Regions and Main Cities in the Western Taiwan |  |  |
| --- | --- | --- |
| Region | Regional center | Main city |
| North Region | Taipei City | Keelung City, Taoyuan City, Hsinchu City, I-lan City |
| Middle Region | Taichung City | Miaoli City, Nantou City, Changhwa City, Douliou City |
| South Region | Tainan City* | Chiayi City, Shinying City |
|  | Kaohsiung City* | Pingtung City |

* The South Region has two regional centers

5.1 Usage of Geographic Information System

The highway network with 1929 links (as Fig. 5.1) is built in the geographic information system (GIS) using TransCAD, and the 1998 travel time data for links are collected by a field survey and stored in the GIS. Four types of highways are included in the test case: national, provincial, county, and Accessory (or connector) highways. TransCAD is used to generate the shortest path that connects each main city to the regional center. After this procedure, the number of links that comprise all the shortest paths is 120, as illustrated in Fig. 5.2. All the link attributes of the shortest path for every main city to regional center are listed in Table 5.2.

In the study, using the GIS is not necessary but is really helpful. The advantages of using the GIS are listed as follows:

- It’s easier to generate a shortest path in a network by giving any two locations.

- It’s easier to justify the rationality of the shortest path by using different colors to display the network in terms of the road type.

- It’s easier to check the accuracy of link length stored in attribute database by comparing it with the value generated automatically from geographic database.

Considering the cost and time in the development of the GIS database, the geographic data of network could be collected simultaneously using GPS (Global Positioning System) during the travel time survey.

FIG. 5.1 Highway Network in Taiwan

FIG. 5.2 Shortest Paths from Main Cities to Regional Centers

| TABLE 5.2 Link Attributes for each Shortest Path |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Shortest Path (Main city to Regional center) | Link no. | Road type | Road Name | LNl : No. of lanes | Tlc : Current travel time (minutes) | Dl : Length of link (Km) |
| Keelung -Taipei | link591 | 4 | A1 | 2 | 2.2 | 0.9 |
|  | link471 | 4 | A1 | 1 | 8.71 | 2.2 |
|  | link10843 | 1 | N1 | 2 | 1.32 | 1.1 |
|  | link10002 | 1 | N1 | 2 | 1.23 | 1.2 |
|  | link10003 | 1 | N1 | 2 | 5 | 5.8 |
|  | link10165 | 1 | N1 | 2 | 2.46 | 3.2 |
|  | link10006 | 1 | N1 | 3 | 0.71 | 0.7 |
|  | link10007 | 1 | N1 | 3 | 0.78 | 0.5 |
|  | link10057 | 1 | N1 | 4 | 2.34 | 2.15 |
|  | link10511 | 1 | N1 | 2 | 2.79 | 3.4 |
|  | link10141 | 4 | A6 | 2 | 9.28 | 2.6 |
|  | link10139 | 4 | A6 | 3 | 5.74 | 1.1 |
| Taoyuan -Taipei | link10220 | 2 | P1 | 2 | 2.39 | 0.8 |
|  | link1059 | 2 | P1 | 2 | 2.79 | 0.75 |
|  | link509 | 3 | C110 | 2 | 4.19 | 1.9 |
|  | link10224 | 4 | A20 | 2 | 2.17 | 1.1 |
|  | link10221 | 1 | N2 | 3 | 2.32 | 3.2 |
|  | link10807 | 1 | N1 | 4 | 2.55 | 3.2 |
|  | link10064 | 1 | N1 | 4 | 7.23 | 8.5 |
|  | link10063 | 1 | N1 | 5 | 4.75 | 5.1 |
|  | link10844 | 1 | N1 | 4 | 1.39 | 1.4 |
|  | link10817 | 1 | N1 | 4 | 0.735 | 1.1 |
|  | link10159 | 1 | N1 | 3 | 3.2 | 4.75 |
|  | link10161 | 1 | N1 | 2 | 3.32 | 5.3 |
|  | link10142 | 4 | A9 | 3 | 8.73 | 3.4 |
|  | link10139 | 4 | A6 | 3 | 5.74 | 1.1 |
| Hsinchu -Taipei | link424 | 2 | P1 | 2 | 5.8 | 4 |
|  | link10247 | 3 | C120 | 2 | 4.08 | 1.8 |
|  | link10249 | 4 | A15 | 2 | 1.9 | 0.7 |
|  | link10074 | 1 | N1 | 2 | 3.06 | 4.2 |
|  | link10073 | 1 | N1 | 2 | 2.08 | 3.2 |
|  | link10072 | 1 | N1 | 2 | 7.04 | 8.8 |
|  | link10071 | 1 | N1 | 2 | 2.1 | 2.5 |
|  | link10792 | 1 | N1 | 3 | 1.34 | 2 |
|  | link10796 | 1 | N1 | 3 | 3.68 | 5 |
|  | link10068 | 1 | N1 | 3 | 4.55 | 5.9 |
|  | link10067 | 1 | N1 | 3 | 2.07 | 1.7 |
|  | link10066 | 1 | N1 | 3 | 2.66 | 2.6 |

Road type =1: National Highway; 2: Provincial Highway; 3: County Highway; 4: Accessory Highway or Connector

| TABLE 5.2 Link Attributes for each Shortest Path (continued) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Shortest Path (Main city to Regional center) | Link no. | Road type | Road Name | LNl : No. of lanes | Tlc : Current travel time (minutes) | Dl : Length of link (Km) |
| Hsinchu -Taipei | link10807 | 1 | N1 | 4 | 2.55 | 3.2 |
|  | link10064 | 1 | N1 | 4 | 7.23 | 8.5 |
|  | link10063 | 1 | N1 | 5 | 4.75 | 5.1 |
|  | link10844 | 1 | N1 | 4 | 1.39 | 1.4 |
|  | link10817 | 1 | N1 | 4 | 0.735 | 1.1 |
|  | link10159 | 1 | N1 | 3 | 3.2 | 4.75 |
|  | link10161 | 1 | N1 | 2 | 3.32 | 5.3 |
|  | link10142 | 4 | A9 | 3 | 8.73 | 3.4 |
|  | link10139 | 4 | A6 | 3 | 5.74 | 1.1 |
| I-lan - Taipei | link635 | 2 | P9 | 1 | 4.89 | 1.6 |
|  | link10301 | 2 | P9 | 1 | 7.86 | 5.6 |
|  | link511 | 2 | P9 | 1 | 6.54 | 3.5 |
|  | link10300 | 2 | P9 | 1 | 1.03 | 0.9 |
|  | link10299 | 2 | P9 | 1 | 14.12 | 10.6 |
|  | link808 | 2 | P9 | 1 | 31.51 | 21.8 |
|  | link10298 | 3 | C106 | 1 | 27.54 | 13.3 |
|  | link846 | 3 | C106 | 1 | 5.96 | 2.5 |
|  | link474 | 3 | C106 | 1 | 7.85 | 4.4 |
|  | link10145 | 3 | C106 | 1 | 2.25 | 1.1 |
|  | link10019 | 4 | A22 | 2 | 2.79 | 1.8 |
|  | link10131 | 1 | N3A | 2 | 2.5 | 1.6 |
|  | link10018 | 1 | N3A | 2 | 2.57 | 2.8 |
|  | link10137 | 4 | A10 | 3 | 11.2 | 2.6 |
|  | link10128 | 2 | P5 | 3 | 7.1 | 2.8 |
| Miaoli -Taichung | link370 | 2 | P6 | 2 | 2.21 | 1.85 |
|  | link980 | 2 | P6 | 2 | 2.42 | 1.5 |
|  | link10331 | 2 | P6 | 2 | 2.98 | 1.5 |
|  | link10029 | 2 | P13 | 2 | 9.11 | 10.9 |
|  | link10771 | 1 | N1 | 2 | 5.87 | 8.5 |
|  | link10770 | 1 | N1 | 2 | 2.9 | 4.1 |
|  | link10768 | 1 | N1 | 2 | 3.27 | 4.8 |
|  | link10371 | 1 | N1 | 2 | 4.58 | 6.4 |
|  | link10766 | 1 | N1 | 2 | 3.19 | 4.4 |
|  | link295 | 2 | P12 | 3 | 1.31 | 0.8 |
|  | link330 | 3 | C127 | 1 | 6.92 | 3.4 |
|  | link985 | 3 | C136 | 1 | 6.04 | 4 |

Road type =1: National Highway; 2: Provincial Highway; 3: County Highway; 4: Accessory Highway or Connector

| TABLE 5.2 Link Attributes for each Shortest Path (continued) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Shortest Path (Main city to Regional center) | Link no. | Road type | Road Name | LNl : No. of lanes | Tlc : Current travel time (minutes) | Dl : Length of link (Km) |
| Nantou - Taichung | link10437 | 2 | P3 | 2 | 1.93 | 1 |
|  | link866 | 2 | P3 | 2 | 6.39 | 4.5 |
|  | link10417 | 2 | P3 | 2 | 1.23 | 1.3 |
|  | link10420 | 2 | P14 | 3 | 2.43 | 2.6 |
|  | link10419 | 2 | P14 | 3 | 1.46 | 1.3 |
|  | link10424 | 2 | P63 | 3 | 3.65 | 2.2 |
|  | link10428 | 2 | P63 | 3 | 5.58 | 7.4 |
|  | link10427 | 2 | P63 | 3 | 2.31 | 2.8 |
|  | link10426 | 2 | P63 | 3 | 1.05 | 1.1 |
|  | link10425 | 2 | P63 | 3 | 2 | 2 |
|  | link10430 | 4 | A32 | 3 | 2.91 | 1.4 |
|  | link10408 | 2 | P3 | 3 | 3.01 | 1.7 |
|  | link839 | 2 | P3 | 2 | 3.86 | 1.5 |
|  | link10405 | 2 | P3 | 2 | 1.14 | 1 |
| Changhwa - Taichung | link309 | 2 | P1 | 2 | 5.94 | 2.5 |
|  | link10438 | 2 | P1 | 2 | 1.39 | 0.7 |
|  | link308 | 2 | P1 | 3 | 2.19 | 1.4 |
|  | link310 | 2 | P1 | 3 | 1.36 | 1.2 |
|  | link311 | 2 | P1B | 3 | 4.65 | 2 |
|  | link313 | 2 | P1B | 3 | 3.81 | 2.2 |
|  | link10411 | 2 | P1B | 2 | 3.145 | 1.2 |
|  | link325 | 2 | P1B | 3 | 13.01 | 5.6 |
| Douliou - Taichung | link559 | 2 | P1D | 2 | 9.75 | 7.2 |
|  | link10497 | 2 | P1D | 2 | 1.02 | 0.7 |
|  | link10502 | 2 | P1 | 2 | 3.95 | 3.2 |
|  | link10760 | 1 | N1 | 2 | 0.47 | 0.7 |
|  | link10847 | 1 | N1 | 2 | 6.96 | 11 |
|  | link10761 | 1 | N1 | 2 | 5.18 | 7.4 |
|  | link10037 | 1 | N1 | 2 | 10.37 | 12.4 |
|  | link10036 | 1 | N1 | 2 | 7.31 | 9.3 |
|  | link10407 | 1 | N1 | 2 | 10.29 | 10.2 |
|  | link295 | 2 | P12 | 3 | 1.31 | 0.8 |
|  | link330 | 3 | C127 | 1 | 6.92 | 3.4 |
|  | link985 | 3 | C136 | 1 | 6.04 | 4 |

Road type =1: National Highway; 2: Provincial Highway; 3: County Highway; 4: Accessory Highway or Connector

| TABLE 5.2 Link Attributes for each Shortest Path (continued) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Shortest Path (Main city to Regional center) | Link no. | Road type | Road Name | LNl : No. of lanes | Tlc : Current travel time (minutes) | Dl : Length of link (Km) |
| Chiayi - Tainan | link10558 | 3 | C159 | 1 | 3.63 | 0.8 |
|  | link910 | 3 | C159 | 2 | 1.09 | 0.6 |
|  | link10555 | 3 | C159 | 2 | 7.64 | 4.9 |
|  | link10554 | 1 | N1 | 2 | 3.5 | 6.1 |
|  | link10754 | 1 | N1 | 2 | 6.25 | 10.7 |
|  | link10753 | 1 | N1 | 2 | 1.83 | 3 |
|  | link10046 | 1 | N1 | 2 | 2.5 | 3.9 |
|  | link10047 | 1 | N1 | 2 | 9.67 | 15.1 |
|  | link10750 | 1 | N1 | 2 | 2.21 | 3.7 |
|  | link10749 | 1 | N1 | 2 | 3.29 | 5.9 |
|  | link10746 | 1 | N1 | 2 | 4.04 | 6.2 |
|  | link10590 | 1 | N1 | 2 | 4.63 | 7.7 |
|  | link96 | 3 | C182 | 2 | 6.14 | 2.3 |
|  | link97 | 3 | C182 | 2 | 8.95 | 3 |
| Shinying - Tainan | link925 | 3 | C172 | 2 | 3.1 | 2.6 |
|  | link10047 | 1 | N1 | 2 | 9.67 | 15.1 |
|  | link10750 | 1 | N1 | 2 | 2.21 | 3.7 |
|  | link10749 | 1 | N1 | 2 | 3.29 | 5.9 |
|  | link10746 | 1 | N1 | 2 | 4.04 | 6.2 |
|  | link10590 | 1 | N1 | 2 | 4.63 | 7.7 |
|  | link96 | 3 | C182 | 2 | 6.14 | 2.3 |
|  | link97 | 3 | C182 | 2 | 8.95 | 3 |
| Pingtung - Kaohsiung | link948 | 2 | P3 | 2 | 7.81 | 3.6 |
|  | link10622 | 2 | P1 | 2 | 7.52 | 4 |
|  | link10623 | 2 | P1 | 2 | 6.99 | 3.3 |
|  | link46 | 2 | P1 | 3 | 6.73 | 3.2 |
|  | link760 | 2 | P1 | 2 | 4.67 | 2.5 |
|  | link762 | 2 | P25 | 1 | 2.42 | 1.1 |
|  | link10731 | 2 | P1E | 2 | 1.9 | 0.4 |
|  | link10629 | 2 | P1E | 2 | 4.28 | 1.5 |
|  | link954 | 2 | P1E | 2 | 3.79 | 0.8 |
|  | link10632 | 2 | P1E | 2 | 2.2 | 1 |

Road type =1: National Highway; 2: Provincial Highway; 3: County Highway; 4: Accessory Highway or Connector

5.2 Process of Case Study

For testing, the values of parameters for the input of the both models are given as Table 5.3.

| TABLE 5.3 Parameters for the Input of Both Models |  |
| --- | --- |
| Case A | Case B |
| Al =5/km * Dl , if road type=1 or 4 =1/km * Dl , otherwise), | Al =5/km * Dl * LNl , if road type=1 or 4 =1/km * Dl* LNl , otherwise), |
| Al : cost needed for the improvement of link l Dl : length of link l LNl : number of lanes of link l Road type =1 for National Highway =2 for Provincial Highway =3 for County Highway =4 for Accessory Highway or Connector |  |
| Slf : future travel speed along link l while it is improved ( = 80km/hr, if link l is national highway = 45km/hr, otherwise) |  |
| B : total budget (=200) |  |

In Case A, the cost needed for improvement of link l is assumed to be proportional to the length of the link. In Case B, this cost is assumed to be proportional to both the length and number of lanes of that link.

For estimation of the saved travel time by improvement of a link, we need to forecast the future travel speed along the link to derive its future travel time. In both cases, the future travel speed is assigned to be the value specified where the traffic flow reaches the capacity of the link. This value could underestimate the future travel speed while the improvement is just finished. Nevertheless, it is a conservative but reasonable estimate for travel speed on account of increasing demand from highway improvement.

We minimized the objective Ws (s = 1,2,3) one by one and got the ideal solution I* and anti-ideal solution I# mentioned in section 3.2. Then, the fuzzy programming was processed to derive the compromised solutions. The computing time of each process is less than 5 minutes on a personal computer Pentium 4-1.6G by the SOLVER of EXCEL.

The summary of results from the basic model for Case A and Case B are presented in Table 5.4A and 5.4 B, respectively. Also, the summary of results from the revised model for Case A and Case B are presented separately in Table 5.5A and 5.5B. For both model with both cases, the values of decision variables for link improvement are listed in Table 5.6. The outputs and the details of process through fuzzy programming are listed in Appendix 1.1~4.4.

TABLE 5.4A  Summary of Results from the Basic Model for Case A

| Process Results | Initial State | Min W1 | Min W2 | Min W3 | Maxλ |
| --- | --- | --- | --- | --- | --- |
| Budget used | 0 | 199.65 | 187.9 | 196.85 | 193.85 |
| Time saved | 0 | 141.57 | 60.01 | 81.91 | 101.55 |
| Value of W1 (minutes) | 59.85 | *46.98 | 54.39 | 52.40 | 50.61 |
| Value of W2 (minutes)2 | 571.37 | 457.8 | *238.77 | 406.64 | 318.08 |
| Value of W3 (minutes)2 | 159.87 | 73.27 | 89.53 | *10.39 | 47.53 |
| DS1 |  | 1 | 0 |  | 0.509 |
| DS2 |  | 0 | 1 |  | 0.527 |
| DS3 |  |  | 0 | 1 | 0.531 |

* : optimal value

TABLE 5.4B  Summary of Results from the Basic Model for Case B

| Process Results | Initial State | Min W1 | Min W2 | Min W3 | Maxλ |
| --- | --- | --- | --- | --- | --- |
| Budget used |  | 199.90 | 183.2 | 197.8 | 196.1 |
| Time saved | 0 | 113.44 | 47.61 | 71.18 | 88.74 |
| Value of W1 (minutes) | 59.85 | *49.53 | 55.52 | 53.37 | 51.78 |
| Value of W2 (minutes)2 | 571.37 | 510.35 | *252.63 | 428.57 | 338.38 |
| Value of W3 (minutes)2 | 159.87 | 113.31 | 76.53 | *19.11 | 53.85 |
| DS1 |  | 1 | 0 |  | 0.625 |
| DS2 |  | 0 | 1 |  | 0.667 |
| DS3 |  | 0 |  | 1 | 0.631 |

* : optimal value

TABLE 5.5A  Summary of Results from the Revised Model for Case A

| Process Results | Initial State | Min W1 | Min W2 | Min W3 | Maxλ |
| --- | --- | --- | --- | --- | --- |
| Budget used | 0 | 199.65 | 170.05 | 123.35 | 198.55 |
| Time saved | 0 | 141.57 | 89.07 | 77.39 | 120.78 |
| Value of W1 (minutes) | 59.85 | *46.98 | 51.75 | 52.81 | 48.87 |
| Value of W2 (Δkm/hr) | 14.23 | 12.01 | *7.21 | 13.15 | 9.13 |
| Value of W3 (Δkm/hr) | 3.96 | 5.22 | 3.30 | *0.19 | 1.07 |
| DS1 |  | 1 |  | 0 | 0.676 |
| DS2 |  |  | 1 | 0 | 0.677 |
| DS3 |  | 0 |  | 1 | 0.827 |

* : optimal value

TABLE 5.5B  Summary of Results from the Revised Model for Case B

| Process Results | Initial State | Min W1 | Min W2 | Min W3 | Maxλ |
| --- | --- | --- | --- | --- | --- |
| Budget used | 0 | 199.9 | 198.7 | 118.9 | 196.9 |
| Time saved | 0 | 113.44 | 76.10 | 53.19 | 94.13 |
| Value of W1 (minutes) | 59.85 | *49.53 | 52.93 | 55.01 | 51.29 |
| Value of W2 (Δkm/hr) | 14.23 | 11.93 | *7.54 | 13.70 | 9.52 |
| Value of W3 (Δkm/hr) | 3.96 | 6.70 | 4.16 | *0.025 | 1.53 |
| DS1 |  | 1 |  | 0 | 0.680 |
| DS2 |  |  | 1 | 0 | 0.680 |
| DS3 |  | 0 |  | 1 | 0.775 |

* : optimal value

| TABLE 5.6 Values of Decision Variables in Case A and Case B |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Shortest Path (Main city to Regional center) | Link no. | Road Name | Basic Model xl in Case A | Basic Model xl in Case B | Revised Model xl in Case A | Revised Model xl in Case B |
| Keelung -Taipei | link591 | A1 | 0 | 0 | 1 | 0 |
|  | link471 | A1 | 1 | 1 | 1 | 1 |
|  | link10843 | N1 | 0 | 0 | 0 | 0 |
|  | link10002 | N1 | 0 | 0 | 0 | 0 |
|  | link10003 | N1 | 0 | 0 | 0 | 0 |
|  | link10165 | N1 | 0 | 0 | 0 | 0 |
|  | link10006 | N1 | 0 | 0 | 0 | 0 |
|  | link10007 | N1 | 0 | 0 | 1 | 0 |
|  | link10057 | N1 | 0 | 0 | 0 | 0 |
|  | link10511 | N1 | 0 | 0 | 0 | 0 |
|  | link10141 | A6 | 0 | 0 | 1 | 1 |
|  | link10139 | A6 | 1 | 1 | 1 | 1 |
| Taoyuan -Taipei | link10220 | P1 | 0 | 0 | 1 | 1 |
|  | link1059 | P1 | 1 | 0 | 1 | 1 |
|  | link509 | C110 | 0 | 0 | 1 | 1 |
|  | link10224 | A20 | 1 | 0 | 0 | 0 |
|  | link10221 | N2 | 0 | 0 | 0 | 0 |
|  | link10807 | N1 | 0 | 0 | 0 | 0 |
|  | link10064 | N1 | 0 | 0 | 0 | 0 |
|  | link10063 | N1 | 0 | 0 | 0 | 0 |
|  | link10844 | N1 | 0 | 0 | 0 | 0 |
|  | link10817 | N1 | 0 | 0 | 0 | 0 |
|  | link10159 | N1 | 0 | 0 | 0 | 0 |
|  | link10161 | N1 | 0 | 0 | 0 | 0 |
|  | link10142 | A9 | 1 | 1 | 1 | 0 |
|  | link10139 | A6 | 1 | 1 | 1 | 1 |
| Hsinchu -Taipei | link424 | P1 | 1 | 0 | 0 | 0 |
|  | link10247 | C120 | 1 | 1 | 1 | 1 |
|  | link10249 | A15 | 1 | 0 | 1 | 0 |
|  | link10074 | N1 | 0 | 0 | 0 | 0 |
|  | link10073 | N1 | 0 | 0 | 0 | 0 |
|  | link10072 | N1 | 0 | 0 | 0 | 0 |
|  | link10071 | N1 | 0 | 0 | 0 | 0 |
|  | link10792 | N1 | 0 | 0 | 0 | 0 |
|  | link10796 | N1 | 0 | 0 | 0 | 0 |
|  | link10068 | N1 | 0 | 0 | 0 | 0 |
|  | link10067 | N1 | 0 | 0 | 0 | 0 |
|  | link10066 | N1 | 1 | 0 | 1 | 0 |

| TABLE 5.6 Values of Decision Variables in Case A and Case B (continued) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Shortest Path (Main city to Regional center) | Link no. | Road Name | Basic Model xl in Case A | Basic Model xl in Case B | Revised Model xl in Case A | Revised Model xl in Case B |
| Hsinchu -Taipei | link10807 | N1 | 0 | 0 | 0 | 0 |
|  | link10064 | N1 | 0 | 0 | 0 | 0 |
|  | link10063 | N1 | 0 | 0 | 0 | 0 |
|  | link10844 | N1 | 0 | 0 | 0 | 0 |
|  | link10817 | N1 | 0 | 0 | 0 | 0 |
|  | link10159 | N1 | 0 | 0 | 0 | 0 |
|  | link10161 | N1 | 0 | 0 | 0 | 0 |
|  | link10142 | A9 | 1 | 1 | 1 | 0 |
|  | link10139 | A6 | 1 | 1 | 1 | 1 |
| I-lan - Taipei | link635 | P9 | 1 | 1 | 1 | 1 |
|  | link10301 | P9 | 1 | 0 | 0 | 1 |
|  | link511 | P9 | 1 | 1 | 1 | 1 |
|  | link10300 | P9 | 0 | 0 | 0 | 0 |
|  | link10299 | P9 | 0 | 0 | 0 | 0 |
|  | link808 | P9 | 1 | 1 | 1 | 0 |
|  | link10298 | C106 | 1 | 1 | 1 | 1 |
|  | link846 | C106 | 1 | 1 | 1 | 1 |
|  | link474 | C106 | 1 | 1 | 1 | 1 |
|  | link10145 | C106 | 1 | 1 | 1 | 1 |
|  | link10019 | A22 | 1 | 0 | 0 | 0 |
|  | link10131 | N3A | 1 | 0 | 1 | 0 |
|  | link10018 | N3A | 1 | 0 | 0 | 0 |
|  | link10137 | A10 | 1 | 1 | 1 | 0 |
|  | link10128 | P5 | 1 | 1 | 1 | 1 |
| Miaoli -Taichung | link370 | P6 | 0 | 0 | 0 | 0 |
|  | link980 | P6 | 1 | 0 | 1 | 0 |
|  | link10331 | P6 | 1 | 0 | 1 | 0 |
|  | link10029 | P13 | 0 | 0 | 0 | 0 |
|  | link10771 | N1 | 0 | 0 | 0 | 0 |
|  | link10770 | N1 | 0 | 0 | 0 | 0 |
|  | link10768 | N1 | 0 | 0 | 0 | 0 |
|  | link10371 | N1 | 0 | 0 | 0 | 0 |
|  | link10766 | N1 | 0 | 0 | 0 | 0 |
|  | link295 | P12 | 1 | 0 | 1 | 0 |
|  | link330 | C127 | 1 | 1 | 1 | 1 |
|  | link985 | C136 | 1 | 0 | 0 | 1 |

| TABLE 5.6 Values of Decision Variables in Case A and Case B (continued) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Shortest Path (Main city to Regional center) | Link no. | Road Name | Basic Model xl in Case A | Basic Model xl in Case B | Revised Model xl in Case A | Revised Model xl in Case B |
| Nantou - Taichung | link10437 | P3 | 0 | 1 | 1 | 1 |
|  | link866 | P3 | 0 | 0 | 0 | 0 |
|  | link10417 | P3 | 0 | 0 | 0 | 0 |
|  | link10420 | P14 | 0 | 0 | 0 | 0 |
|  | link10419 | P14 | 0 | 0 | 0 | 0 |
|  | link10424 | P63 | 0 | 0 | 1 | 1 |
|  | link10428 | P63 | 0 | 0 | 0 | 0 |
|  | link10427 | P63 | 0 | 0 | 0 | 0 |
|  | link10426 | P63 | 0 | 0 | 0 | 0 |
|  | link10425 | P63 | 0 | 0 | 0 | 0 |
|  | link10430 | A32 | 0 | 0 | 0 | 0 |
|  | link10408 | P3 | 0 | 0 | 1 | 0 |
|  | link839 | P3 | 0 | 0 | 1 | 1 |
|  | link10405 | P3 | 0 | 0 | 0 | 0 |
| Changhwa - Taichung | link309 | P1 | 0 | 0 | 1 | 1 |
|  | link10438 | P1 | 0 | 0 | 1 | 1 |
|  | link308 | P1 | 0 | 0 | 1 | 0 |
|  | link310 | P1 | 0 | 0 | 0 | 0 |
|  | link311 | P1B | 0 | 0 | 1 | 1 |
|  | link313 | P1B | 0 | 0 | 1 | 1 |
|  | link10411 | P1B | 0 | 0 | 1 | 1 |
|  | link325 | P1B | 0 | 0 | 1 | 1 |
| Douliou - Taichung | link559 | P1D | 1 | 0 | 0 | 0 |
|  | link10497 | P1D | 0 | 0 | 1 | 0 |
|  | link10502 | P1 | 0 | 0 | 0 | 0 |
|  | link10760 | N1 | 0 | 0 | 0 | 0 |
|  | link10847 | N1 | 0 | 0 | 0 | 0 |
|  | link10761 | N1 | 0 | 0 | 0 | 0 |
|  | link10037 | N1 | 0 | 0 | 0 | 0 |
|  | link10036 | N1 | 0 | 0 | 0 | 0 |
|  | link10407 | N1 | 0 | 0 | 0 | 0 |
|  | link295 | P12 | 1 | 0 | 1 | 0 |
|  | link330 | C127 | 1 | 1 | 1 | 1 |
|  | link985 | C136 | 1 | 0 | 0 | 1 |

| TABLE 5.6 Values of Decision Variables in Case A and Case B (continued) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Shortest Path (Main city to Regional center) | Link no. | Road Name | Basic Model xl in Case A | Basic Model xl in Case B | Revised Model xl in Case A | Revised Model xl in Case B |
| Chiayi - Tainan | link10558 | C159 | 1 | 1 | 0 | 0 |
|  | link910 | C159 | 1 | 0 | 0 | 0 |
|  | link10555 | C159 | 1 | 0 | 0 | 0 |
|  | link10554 | N1 | 0 | 0 | 0 | 0 |
|  | link10754 | N1 | 0 | 0 | 0 | 0 |
|  | link10753 | N1 | 0 | 0 | 0 | 0 |
|  | link10046 | N1 | 0 | 0 | 0 | 0 |
|  | link10047 | N1 | 0 | 0 | 0 | 0 |
|  | link10750 | N1 | 0 | 0 | 0 | 0 |
|  | link10749 | N1 | 0 | 0 | 0 | 0 |
|  | link10746 | N1 | 0 | 0 | 0 | 0 |
|  | link10590 | N1 | 0 | 0 | 0 | 0 |
|  | link96 | C182 | 1 | 1 | 0 | 0 |
|  | link97 | C182 | 1 | 1 | 0 | 0 |
| Shinying - Tainan | link925 | C172 | 0 | 0 | 0 | 0 |
|  | link10047 | N1 | 0 | 0 | 0 | 0 |
|  | link10750 | N1 | 0 | 0 | 0 | 0 |
|  | link10749 | N1 | 0 | 0 | 0 | 0 |
|  | link10746 | N1 | 0 | 0 | 0 | 0 |
|  | link10590 | N1 | 0 | 0 | 0 | 0 |
|  | link96 | C182 | 1 | 1 | 0 | 0 |
|  | link97 | C182 | 1 | 1 | 0 | 0 |
| Pingtung - Kaohsiung | link948 | P3 | 0 | 0 | 1 | 1 |
|  | link10622 | P1 | 0 | 0 | 1 | 1 |
|  | link10623 | P1 | 0 | 0 | 1 | 1 |
|  | link46 | P1 | 0 | 0 | 1 | 0 |
|  | link760 | P1 | 0 | 0 | 1 | 1 |
|  | link762 | P25 | 0 | 0 | 1 | 1 |
|  | link10731 | P1E | 1 | 0 | 1 | 1 |
|  | link10629 | P1E | 0 | 0 | 1 | 1 |
|  | link954 | P1E | 1 | 1 | 1 | 1 |
|  | link10632 | P1E | 0 | 0 | 1 | 1 |

5.3 Analysis of Results

From the results of the basic model for Case A (Case B) in Table 5.4A(5.4B), the used budget is 193.85(196.1) and the travel time saved for all the main cities to the regional centers is 101.55(88.74) minutes in total. With the highway investment, we may improve cost efficiency from 59.85 to 50.61(51.78) minutes, horizontal equity from 571.37 to 318.08(338.38) minutes2, and vertical equity from 159.87 to 47.53(53.85) minutes2. The compromise-grade is 0.509 and 0.625 in Case A and Case B, respectively.

From the results of the revised model for Case A(Case B) in Table 5.5A(5.5B), the budget 198.55(196.9) is used and travel time saved is 120.78(94.13) minutes in total. With the highway investment, we may improve the cost efficiency from 59.85 to 48.87(51.29) minutes, horizontal equity from 14.23 to 9.13(9.52) km/hr, and vertical equity from 3.96 to 1.07(1.53) km/hr. The compromise-grade is 0.676 and 0.680 in Case A and Case B, respectively.

In the case study, the revised model not only improves the average travel speed of the shortest paths in every region, but also improves the horizontal equity and vertical equity by reducing the intra-regional and inter-regional differences in average travel speed. These improvements have been shown in Table 5.7 for both Case A and Case B. In either the same region or different regions, the average travel speed of each shortest path connecting main cities to regional centers becomes closer through use of the revised model.

After adjusting the travel cost measure from travel time to travel speed in view of equity, the revised model has obtained properties superior to those of the basic model. The results of comparison are listed in the Table 5.8A(B) and Table 5.9A(B) in terms of travel time and average travel speed for each shortest path.

In Case A and Case B, the cost efficiency measured by averaged travel time that resulted from the basic model is similar to that obtained from the revised model, but the basic model yielded some unreasonable results in terms of equity in service quality of highway. For example in Table 5.9A, with the basic model, the average travel speed was greatly improved from 66.38 to 82.04 Km/Hr from Shinying to Tainan, but was only slightly improved from 26.58 to 29.04 Km/Hr from Pingtung to Kaohsiung, even though these cities belong to the same region. However, with the revised model, no link of the shortest path from Shinying to Tainan was improved, but the average travel speed was greatly improved from 26.58 to 45 Km/Hr from Pingtung to Kaohsiung,

From Table 5.9A and 5.9B, the revised model yields better results than the basic model in view of equity because the service quality (average travel speed) of each shortest path has been equalized to some extent. Using the Case A, the comparison of average travel speed between basic model and revised model is also illustrated in Fig. 5.3.

This case study has proved that both models can be solved efficiently by fuzzy programming, and in addition, the estimated results indicated that the revised model is a practical and effective for acquiring reasonably compromised solutions for efficiency and equity of highway investment.

| TABLE 5.7 Effects of Revised Model on Regional Equity |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Region | Average Travel Speed (km/hr) |  | Intra-regional Difference (Δkm/hr) |  | Inter-regional Difference (Δkm/hr) |  |
|  | Initial | Case A | Initial | Case A | Initial | Case A |
|  |  | Case B |  | Case B |  | Case B |
| North Region | 44.04 | 59.74 | 10.19 | 8.90 | 4.74 | 0.82 |
|  |  | 54.82 |  | 8.35 |  | 1.73 |
| Middle Region | 49.90 | 57.49 | 13.37 | 8.26 | 1.13 | 1.43 |
|  |  | 56.80 |  | 8.32 |  | 0.24 |
| South Region | 53.59 | 59.74 | 19.11 | 10.44 | 4.82 | 0.82 |
|  |  | 58.54 |  | 12.12 |  | 1.99 |

| TABLE 5.8A Comparison of Travel Time in Case A between Basic Model and Revised Model |  |  |  |
| --- | --- | --- | --- |
| Shortest Path ( Main city  Regional center ) | Travel time (minutes) |  |  |
|  | Initial Status | Basic Model | Revised Model |
| 1.Keelung  Taipei | 42.56 | 32.51 | 25.29 |
| 2.Taoyuan  Taipei | 51.50 | 40.54 | 38.26 |
| 3.Hsinchu  Taipei | 78.00 | 65.71 | 66.18 |
| 4.I-lan  Taipei | 135.71 | 99.78 | 101.04 |
| 5.Miaoli  Taichung | 50.80 | 46.06 | 46.77 |
| 6.Nantou  Taichung | 38.95 | 38.95 | 35.03 |
| 7.Changhwa  Taichung | 35.49 | 35.49 | 22.16 |
| 8.Douliou  Taichung | 69.57 | 66.08 | 66.85 |
| 9.Chiayi  Tainan | 65.37 | 53.39 | 65.37 |
| 10.Shinying  Tainan | 42.03 | 34.01 | 42.03 |
| 11.Pingtung  Kaohsiung | 48.31 | 44.22 | 28.53 |

| TABLE 5.8B Comparison of Travel Time in Case B between Basic Model and Revised Model |  |  |  |
| --- | --- | --- | --- |
| Shortest Path ( Main city  Regional center ) | Travel time (minutes) |  |  |
|  | Initial Status | Basic Model | Revised Model |
| 1.Keelung  Taipei | 42.56 | 32.51 | 26.70 |
| 2.Taoyuan  Taipei | 51.50 | 43.03 | 42.46 |
| 3.Hsinchu  Taipei | 78.00 | 67.85 | 72.05 |
| 4.I-lan  Taipei | 135.71 | 102.34 | 112.12 |
| 5.Miaoli  Taichung | 50.80 | 48.41 | 47.71 |
| 6.Nantou  Taichung | 38.95 | 38.35 | 35.78 |
| 7.Changhwa  Taichung | 35.49 | 35.49 | 22.48 |
| 8.Douliou  Taichung | 69.57 | 67.18 | 66.48 |
| 9.Chiayi  Tainan | 65.37 | 54.78 | 65.37 |
| 10.Shinying  Tainan | 42.03 | 34.01 | 42.03 |
| 11.Pingtung  Kaohsiung | 48.31 | 45.59 | 31.00 |

| TABLE 5.9A Comparison of Average Travel Speed in Case A between Basic Model and Revised Model |  |  |  |
| --- | --- | --- | --- |
| Shortest Path ( Main city  Regional center ) | Average travel speed (Km/Hr) |  |  |
|  | Initial Status | Basic Model | Revised Model |
| 1.Keelung  Taipei | 35.03 | 45.86 | 58.95 |
| 2.Taoyuan  Taipei | 48.46 | 61.57 | 65.23 |
| 3.Hsinchu  Taipei | 58.65 | 69.62 | 69.13 |
| 4.I-lan  Taipei | 34.00 | 46.24 | 45.67 |
| 5.Miaoli  Taichung | 61.59 | 67.93 | 66.90 |
| 6.Nantou  Taichung | 48.98 | 48.99 | 54.46 |
| 7.Changhwa  Taichung | 28.40 | 28.40 | 45.49 |
| 8.Douliou  Taichung | 60.63 | 63.83 | 63.09 |
| 9.Chiayi  Tainan | 67.83 | 83.05 | 67.83 |
| 10.Shinying  Tainan | 66.38 | 82.04 | 66.38 |
| 11.Pingtung  Kaohsiung | 26.58 | 29.04 | 45.00 |

| TABLE 5.9B Comparison of Average Travel Speed in Case B between Basic Model and Revised Model |  |  |  |
| --- | --- | --- | --- |
| Shortest Path ( Main city  Regional center ) | Average travel speed (Km/Hr) |  |  |
|  | Initial Status | Basic Model | Revised Model |
| 1.Keelung  Taipei | 35.03 | 45.86 | 55.85 |
| 2.Taoyuan  Taipei | 48.46 | 58.00 | 58.78 |
| 3.Hsinchu  Taipei | 58.65 | 67.42 | 63.50 |
| 4.I-lan  Taipei | 34.00 | 45.09 | 41.15 |
| 5.Miaoli  Taichung | 61.59 | 64.63 | 66.59 |
| 6.Nantou  Taichung | 48.98 | 49.75 | 53.33 |
| 7.Changhwa  Taichung | 28.40 | 28.40 | 44.83 |
| 8.Douliou  Taichung | 60.63 | 62.78 | 63.45 |
| 9.Chiayi  Tainan | 67.83 | 80.94 | 67.83 |
| 10.Shinying  Tainan | 66.38 | 82.04 | 66.38 |
| 11.Pingtung  Kaohsiung | 26.58 | 28.17 | 41.42 |

FIG. 5.3 Comparison between Results of Basic model and Revised Model

5.4 More Cases of Revised Model

In previous section, the case study was processed with Case A and Case B. In Case A, the cost needed for the improvement of link l is assumed to be proportional to the length of the link. In Case B, this cost is assumed to be proportional to both the length and number of lanes of that link. The total budget and the assumed future travel speed for highway improvement are the same in both cases. For illustrating the application of revised model, there are more cases processed in this section.

Based on the assumption of parameters in the Case A, we create Case C~F for revised model as follows:

- Case C — to upgrade the assumed future travel speed

- Case D — to raise the total budget of highway improvement

- Case E — to include complementary property among links

- Case F — to build exclusive property among alternatives

Each case is depicted and compared with Case A in section 5.4.1~5.4.4.

5.4.1 Upgrade the Assumed Future Travel Speed

In Case A, the future travel speed is assigned to be the value specified where the traffic flow reaches the capacity of the link. This value is a conservative but reasonable estimate for travel speed on account of increasing demand from highway improvement. However, it could underestimate the future travel speed while the improvement is just finished. If the decision maker expects to know the impact of different assumptions of future travel speed on investment planning, this model is allowed to change the value easily in EXCEL worksheet. In Case C, the future travel speed for any type of highway has been upgraded by 5 km/hr. Namely, the future travel speed for national highway is given to be 85 km/hr instead of 80 km/hr in Case A. Simultaneously, this value is assigned as 50 km/hr for provincial highway, county highway and accessory highway (or connector) instead of 45 km/hr.

From the summary of results in Table 5.10, the Case C achieved the optimal compromised objective-values (W1 = 46.51, W2 = 8.20 and W3 = 1.74) by fuzzy programming, which are better than the results from Case A (W1 = 48.87, W2 = 9.13 and W3 = 1.07). These outcomes are reasonable because higher future travel speed could make average travel time saved more.

TABLE 5.10  Summary of Results from the Revised Model for Case C

| Process Results | Min W1 | Min W2 | Min W3 | Maxλ | Case A |
| --- | --- | --- | --- | --- | --- |
| Budget used | 199.15 | 184.3 | 197.35 | 197.15 | 198.55 |
| Time saved | 162.55 | 106.81 | 109.50 | 146.67 | 120.78 |
| Value of W1 (minutes) | *45.07 | 50.14 | 49.89 | 46.51 | 48.87 |
| Value of W2 (Δkm/hr) | 10.93 | *5.34 | 15.50 | 8.20 | 9.13 |
| Value of W3 (Δkm/hr) | 5.71 | 3.09 | *0.16 | 1.74 | 1.07 |
| DS1 | 1 | 0 |  | 0.715 |  |
| DS2 |  | 1 | 0 | 0.719 |  |
| DS3 | 0 |  | 1 | 0.715 |  |

* : optimal value

5.4.2 Raise the Total Budget of Highway Improvement

With the revised model, the decision maker could understand the impact of different budget on investment planning just by the easy change of the constraint in SOLVER in EXCEL. In Case D, the total budget was changed from 200 to 250.

From the summary of results in Table 5.11, the Case D achieved the optimal compromised objective-values (W1 = 48.76, W2 = 9.05 and W3 = 1.42) by fuzzy programming, which are similar to the results from Case A (W1 = 48.87, W2 = 9.13 and W3 = 1.07). It seems that the higher budget usually yields better objective-values, but these outcomes could not fit this assumption. The reason is that the time saved (146.46) in the process of Min W1 is just a little higher than that value (141.57) of Case A (in Table 5.5A). As the result, even having raised 25% of total budget, it only yields a little difference in performance. Although the sensitivity analysis is not suitable to the integer programming, the decision maker could still test different budgets in the model to make better utilization of resources.

TABLE 5.11  Summary of Results from the Revised Model for Case D

| Process Results | Min W1 | Min W2 | Min W3 | Maxλ | Case A |
| --- | --- | --- | --- | --- | --- |
| Budget used | 248.75 | 190.2 | 249.8 | 235.04 | 198.55 |
| Time saved | 146.46 | 89.40 | 61.54 | 121.91 | 120.78 |
| Value of W1 (minutes) | *46.53 | 51.72 | 54.25 | 48.76 | 48.87 |
| Value of W2 (Δkm/hr) | 11.98 | *7.20 | 13.93 | 9.05 | 9.13 |
| Value of W3 (Δkm/hr) | 5.03 | 3.19 | *0.086 | 1.42 | 1.07 |
| DS1 | 1 |  | 0 | 0.710 |  |
| DS2 |  | 1 | 0 | 0.725 |  |
| DS3 | 0 |  | 1 | 0.730 |  |

* : optimal value

5.4.3 Include Complementary Property among Links

In real cases, relative links should be improved together to enhance the highway performance. The complementary property among links could be defined in the constraintsxl'=xl'',∃l',l''∈Cm,∀m in the revised model to generate reasonable solutions. The Cm represents the set of relative links that should be improved or added together. The judgment for the set of relative links is dependent on the professional experience.

In Case E, there are two sets of relative links assumed to need improvement together to assure the performance of improvement. The first set comprises the link 10437 and link 866, which are adjacent parts of provincial highway P3 and belong to the shortest path from Nantou to Taichung. The second set comprises the link 559 and link 10497, which are adjacent parts of provincial highway P1D and belong to the shortest path from Douliou to Taichung. Two constraints were added in the revised model (x10437 = x866, x559 = x10497) and processed in SOVLER of EXCEL (C76 = C77, C98 = C99 in Appendix 7.1~7.4) using fuzzy programming.

From the summary of results in Table 5.12, the Case E achieved the optimal compromised objective-values (W1 = 48.98, W2 = 9.23 and W3 = 1.55), which are a little worse than the results from Case A (W1 = 48.87, W2 = 9.13 and W3 = 1.07). These outcomes are reasonable owing to more constraints in the model. Including the complementary property among links could make the objective-values lower but make the planning process much more practical.

TABLE 5.12  Summary of Results from the Revised Model for Case E

| Process Results | Min W1 | Min W2 | Min W3 | Maxλ | Case A |
| --- | --- | --- | --- | --- | --- |
| Budget used | 199.65 | 170.05 | 123.35 | 185.9 | 198.55 |
| Time saved | 141.57 | 89.07 | 77.39 | 119.51 | 120.78 |
| Value of W1 (minutes) | *46.98 | 51.75 | 52.81 | 48.98 | 48.87 |
| Value of W2 (Δkm/hr) | 12.01 | *7.21 | 13.15 | 9.23 | 9.13 |
| Value of W3 (Δkm/hr) | 5.22 | 3.30 | *0.19 | 1.55 | 1.07 |
| DS1 | 1 |  | 0 | 0.656 |  |
| DS2 |  | 1 | 0 | 0.659 |  |
| DS3 | 0 |  | 1 | 0.730 |  |

* : optimal value

5.4.4 Build Exclusive Property among Alternatives

In the previous cases, only the links of the shortest paths are treated as the alternatives for improvement. Actually, any given set of links, which serves as the substitute of shortest path, is allowed to be the alternatives for improvement or addition. Through the definition of exclusive alternatives in the revised model, we may evaluate those alternatives of shortest path for each city at the same time.

The decision variable yijk is used to accommodate the exclusion property among alternatives of shortest path for a city. The yijk is equal to 1 while the alternative k of shortest path SPij connecting the city to the regional center is chosen. The exclusive property of alternatives is specified in the constraint Σk yijk = 1∀i,j.

In Case F, there is a link_new assigned to be the substitute of link 762, which is a part of provincial highway P25 and belong to the shortest path from Pingtung to Kaohsiung. The link_new is assumed to be a provincial highway with 0.8 km in length and 3 minutes in current travel time. The link_new and other links of old shortest path become a new shortest path. These two alternatives of shortest path for Pingtung to Kaohsiung are included in revised model and listed in the EXCEL worksheet (as the Appendix 8.1~8.4). Two decision variables y1 and y2 are used to be the indicator of choice between these two alternatives.

From the summary of results in Table 5.13, the Case F achieved the optimal compromised objective-values (W1 = 48.86, W2 = 9.09 and W3 = 1.46), which are similar to the results from Case A (W1 = 48.87, W2 = 9.13 and W3 = 1.07). During each process of Min W1 , Min W2 , Min W1 and Maxλ, the new shortest path with link_new was always selected to optimize the objective-value.

The intra-regional difference and inter-regional difference in Case C, D, E and F are shown with Case A in Table 5.14 and Table 5.15, respectively. The values of decision variables in these cases are listed in Table 5.16. The process of fuzzy programming with EXCEL worksheets for each case is included in Appendix 5.1~8.4.

TABLE 5.13  Summary of Results from the Revised Model for Case F

| Process Results | Min W1 | Min W2 | Min W3 | Maxλ | Case A |
| --- | --- | --- | --- | --- | --- |
| Budget used | 199.35 | 169.75 | 123.35 | 197.95 | 198.55 |
| Time saved | 142.55 | 90.05 | 77.39 | 121.45 | 120.78 |
| Value of W1 (minutes) | *46.94 | 51.71 | 52.86 | 48.86 | 48.87 |
| Value of W2 (Δkm/hr) | 12.01 | *7.21 | 13.24 | 9.09 | 9.13 |
| Value of W3 (Δkm/hr) | 5.22 | 3.30 | *0.04 | 1.46 | 1.07 |
| DS1 | 1 |  | 0 | 0.676 |  |
| DS2 |  | 1 | 0 | 0.689 |  |
| DS3 | 0 |  | 1 | 0.726 |  |

* : optimal value

TABLE 5.14  Intra-regional Difference in Case A, C, D, E and F

| Region | Intra-regional Difference (Δkm/hr) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Case A | Case C | Case D | Case E | Case F |
| North Region | 8.90 | 7.85 | 8.63 | 9.13 | 8.58 |
| Middle Region | 8.26 | 6.59 | 7.80 | 7.55 | 7.70 |
| South Region | 10.44 | 10.31 | 10.75 | 11.21 | 11.21 |

TABLE 5.15  Inter-regional Difference in Case A, C, D, E and F

| Region | Inter-regional Difference (Δkm/hr) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Case A | Case C | Case D | Case E | Case F |
| North Region | 0.82 | 0.52 | 1.14 | 0.68 | 0.26 |
| Middle Region | 1.43 | 1.43 | 1.92 | 1.96 | 1.67 |
| South Region | 0.82 | 2.61 | 1.04 | 1.71 | 1.88 |

Case C — Upgrade the assumed future travel speed.

Case D — Raise the total budget of highway improvement.

Case E — Include the complementary property among links.

Case F — Build the constraints of exclusive property among alternatives.

| TABLE 5.16 Values of Decision Variables in Case C, D, E and F |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Shortest Path (Main city to Regional center) | Link no. | Road Name | Revised Model xl in Case C | Revised Model xl in Case D | Revised Model xl in Case E | Revised Model xl in Case F |
| Keelung -Taipei | link591 | A1 | 1 | 1 | 1 | 1 |
|  | link471 | A1 | 1 | 1 | 1 | 1 |
|  | link10843 | N1 | 0 | 1 | 0 | 0 |
|  | link10002 | N1 | 0 | 0 | 0 | 1 |
|  | link10003 | N1 | 0 | 0 | 0 | 0 |
|  | link10165 | N1 | 0 | 0 | 0 | 0 |
|  | link10006 | N1 | 0 | 1 | 0 | 0 |
|  | link10007 | N1 | 0 | 1 | 0 | 0 |
|  | link10057 | N1 | 0 | 0 | 1 | 0 |
|  | link10511 | N1 | 0 | 0 | 0 | 0 |
|  | link10141 | A6 | 1 | 1 | 1 | 1 |
|  | link10139 | A6 | 1 | 1 | 1 | 1 |
| Taoyuan -Taipei | link10220 | P1 | 1 | 1 | 1 | 1 |
|  | link1059 | P1 | 1 | 1 | 1 | 1 |
|  | link509 | C110 | 1 | 1 | 1 | 0 |
|  | link10224 | A20 | 0 | 1 | 0 | 0 |
|  | link10221 | N2 | 0 | 0 | 0 | 0 |
|  | link10807 | N1 | 0 | 1 | 0 | 0 |
|  | link10064 | N1 | 0 | 0 | 0 | 0 |
|  | link10063 | N1 | 0 | 0 | 0 | 0 |
|  | link10844 | N1 | 0 | 0 | 1 | 0 |
|  | link10817 | N1 | 0 | 0 | 0 | 0 |
|  | link10159 | N1 | 0 | 0 | 0 | 0 |
|  | link10161 | N1 | 0 | 0 | 0 | 0 |
|  | link10142 | A9 | 1 | 1 | 1 | 1 |
|  | link10139 | A6 | 1 | 1 | 1 | 1 |
| Hsinchu -Taipei | link424 | P1 | 0 | 0 | 1 | 0 |
|  | link10247 | C120 | 1 | 1 | 1 | 1 |
|  | link10249 | A15 | 1 | 0 | 0 | 1 |
|  | link10074 | N1 | 0 | 0 | 0 | 0 |
|  | link10073 | N1 | 0 | 0 | 0 | 0 |
|  | link10072 | N1 | 0 | 0 | 0 | 0 |
|  | link10071 | N1 | 0 | 0 | 0 | 0 |
|  | link10792 | N1 | 0 | 0 | 0 | 0 |
|  | link10796 | N1 | 0 | 0 | 0 | 0 |
|  | link10068 | N1 | 0 | 0 | 0 | 0 |
|  | link10067 | N1 | 0 | 0 | 0 | 1 |
|  | link10066 | N1 | 0 | 0 | 0 | 0 |

| TABLE 5.16 Values of Decision Variables in Case C, D, E and F (continued) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Shortest Path (Main city to Regional center) | Link no. | Road Name | Revised Model xl in Case C | Revised Model xl in Case D | Revised Model xl in Case E | Revised Model xl in Case F |
| Hsinchu -Taipei | link10807 | N1 | 0 | 1 | 0 | 0 |
|  | link10064 | N1 | 0 | 0 | 0 | 0 |
|  | link10063 | N1 | 0 | 0 | 0 | 0 |
|  | link10844 | N1 | 0 | 0 | 1 | 0 |
|  | link10817 | N1 | 0 | 0 | 0 | 0 |
|  | link10159 | N1 | 0 | 0 | 0 | 0 |
|  | link10161 | N1 | 0 | 0 | 0 | 0 |
|  | link10142 | A9 | 1 | 1 | 1 | 1 |
|  | link10139 | A6 | 1 | 1 | 1 | 1 |
| I-lan - Taipei | link635 | P9 | 1 | 1 | 1 | 1 |
|  | link10301 | P9 | 0 | 1 | 1 | 0 |
|  | link511 | P9 | 1 | 1 | 1 | 1 |
|  | link10300 | P9 | 0 | 0 | 0 | 0 |
|  | link10299 | P9 | 0 | 0 | 0 | 0 |
|  | link808 | P9 | 1 | 1 | 0 | 1 |
|  | link10298 | C106 | 1 | 1 | 1 | 1 |
|  | link846 | C106 | 1 | 1 | 1 | 1 |
|  | link474 | C106 | 1 | 1 | 1 | 1 |
|  | link10145 | C106 | 1 | 1 | 1 | 1 |
|  | link10019 | A22 | 0 | 1 | 0 | 0 |
|  | link10131 | N3A | 1 | 1 | 1 | 1 |
|  | link10018 | N3A | 0 | 0 | 0 | 0 |
|  | link10137 | A10 | 1 | 1 | 1 | 1 |
|  | link10128 | P5 | 1 | 1 | 1 | 1 |
| Miaoli -Taichung | link370 | P6 | 0 | 0 | 0 | 0 |
|  | link980 | P6 | 0 | 0 | 0 | 1 |
|  | link10331 | P6 | 0 | 0 | 0 | 0 |
|  | link10029 | P13 | 0 | 0 | 0 | 0 |
|  | link10771 | N1 | 0 | 0 | 0 | 0 |
|  | link10770 | N1 | 0 | 0 | 0 | 0 |
|  | link10768 | N1 | 0 | 0 | 0 | 0 |
|  | link10371 | N1 | 0 | 0 | 0 | 0 |
|  | link10766 | N1 | 0 | 0 | 0 | 0 |
|  | link295 | P12 | 1 | 0 | 0 | 0 |
|  | link330 | C127 | 1 | 1 | 1 | 1 |
|  | link985 | C136 | 1 | 1 | 0 | 0 |

| TABLE 5.16 Values of Decision Variables in Case C, D, E and F (continued) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Shortest Path (Main city to Regional center) | Link no. | Road Name | Revised Model xl in Case C | Revised Model xl in Case D | Revised Model xl in Case E | Revised Model xl in Case F |
| Nantou - Taichung | link10437 | P3 | 1 | 0 | 1 | 1 |
|  | link866 | P3 | 1 | 1 | 1 | 0 |
|  | link10417 | P3 | 0 | 0 | 0 | 0 |
|  | link10420 | P14 | 0 | 0 | 0 | 0 |
|  | link10419 | P14 | 0 | 0 | 0 | 0 |
|  | link10424 | P63 | 1 | 1 | 1 | 0 |
|  | link10428 | P63 | 0 | 0 | 0 | 0 |
|  | link10427 | P63 | 0 | 0 | 0 | 0 |
|  | link10426 | P63 | 0 | 0 | 0 | 0 |
|  | link10425 | P63 | 0 | 0 | 0 | 0 |
|  | link10430 | A32 | 1 | 0 | 0 | 1 |
|  | link10408 | P3 | 1 | 1 | 1 | 1 |
|  | link839 | P3 | 1 | 1 | 1 | 1 |
|  | link10405 | P3 | 0 | 0 | 0 | 0 |
| Changhwa - Taichung | link309 | P1 | 1 | 1 | 1 | 1 |
|  | link10438 | P1 | 1 | 1 | 1 | 1 |
|  | link308 | P1 | 1 | 1 | 1 | 1 |
|  | link310 | P1 | 0 | 0 | 0 | 0 |
|  | link311 | P1B | 1 | 1 | 1 | 1 |
|  | link313 | P1B | 1 | 1 | 1 | 1 |
|  | link10411 | P1B | 1 | 1 | 1 | 1 |
|  | link325 | P1B | 1 | 1 | 1 | 1 |
| Douliou - Taichung | link559 | P1D | 0 | 0 | 0 | 0 |
|  | link10497 | P1D | 0 | 0 | 0 | 0 |
|  | link10502 | P1 | 0 | 0 | 0 | 0 |
|  | link10760 | N1 | 0 | 0 | 0 | 0 |
|  | link10847 | N1 | 0 | 0 | 0 | 0 |
|  | link10761 | N1 | 0 | 0 | 0 | 0 |
|  | link10037 | N1 | 0 | 0 | 0 | 0 |
|  | link10036 | N1 | 0 | 0 | 0 | 0 |
|  | link10407 | N1 | 0 | 0 | 0 | 0 |
|  | link295 | P12 | 1 | 0 | 0 | 0 |
|  | link330 | C127 | 1 | 1 | 1 | 1 |
|  | link985 | C136 | 1 | 1 | 0 | 0 |

| TABLE 5.16 Values of Decision Variables in Case C, D, E and F (continued) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Shortest Path (Main city to Regional center) | Link no. | Road Name | Revised Model xl in Case C | Revised Model xl in Case D | Revised Model xl in Case E | Revised Model xl in Case F |
| Chiayi - Tainan | link10558 | C159 | 0 | 0 | 1 | 1 |
|  | link910 | C159 | 0 | 0 | 0 | 0 |
|  | link10555 | C159 | 0 | 1 | 0 | 0 |
|  | link10554 | N1 | 0 | 0 | 0 | 0 |
|  | link10754 | N1 | 0 | 0 | 0 | 0 |
|  | link10753 | N1 | 0 | 0 | 0 | 0 |
|  | link10046 | N1 | 0 | 0 | 0 | 0 |
|  | link10047 | N1 | 0 | 0 | 0 | 0 |
|  | link10750 | N1 | 0 | 0 | 0 | 0 |
|  | link10749 | N1 | 0 | 0 | 0 | 0 |
|  | link10746 | N1 | 0 | 0 | 0 | 0 |
|  | link10590 | N1 | 0 | 0 | 0 | 0 |
|  | link96 | C182 | 1 | 0 | 0 | 0 |
|  | link97 | C182 | 0 | 0 | 0 | 0 |
| Shinying - Tainan | link925 | C172 | 0 | 0 | 0 | 0 |
|  | link10047 | N1 | 0 | 0 | 0 | 0 |
|  | link10750 | N1 | 0 | 0 | 0 | 0 |
|  | link10749 | N1 | 0 | 0 | 0 | 0 |
|  | link10746 | N1 | 0 | 0 | 0 | 0 |
|  | link10590 | N1 | 0 | 0 | 0 | 0 |
|  | link96 | C182 | 1 | 0 | 0 | 0 |
|  | link97 | C182 | 0 | 0 | 0 | 0 |
| Pingtung - Kaohsiung | link948 | P3 | 1 | 1 | 1 | 1 |
|  | link10622 | P1 | 1 | 1 | 1 | 1 |
|  | link10623 | P1 | 1 | 1 | 1 | 1 |
|  | link46 | P1 | 1 | 1 | 1 | 1 |
|  | link760 | P1 | 1 | 1 | 1 | 1 |
|  | link762 | P25 | 1 | 1 | 1 | 1 |
|  | link10731 | P1E | 1 | 1 | 1 | 1 |
|  | link10629 | P1E | 1 | 1 | 1 | 1 |
|  | link954 | P1E | 1 | 1 | 1 | 1 |
|  | link10632 | P1E | 1 | 1 | 1 | 1 |
|  | Link_new | P-new |  |  |  | 1 |
|  |  |  |  |  |  | y1 = 0 |
|  |  |  |  |  |  | y2 = 1 |

### CHAPTER 6

### EXPANSION OF REVISED MODEL

To expand the revised model for more practical and flexible use, the equity issue in allocation of budget for highway investment is presented in section 6.1, and some alternative formulations for travel cost/benefit measure and equity objective are discussed in section 6.2 and 6.3, respectively.

6.1 Equity in Allocation of Budget for Highway Investment

Allocation of the budget for highway investment is another issue of equity but is not mentioned in the basic model and revised model. This issue can be considered by restricting the upper bound of the budget-share for each city according to its socio-economic attributes (for example, population, household income, household tax payment, or travel demand). The decision-maker may adjust the budget share in response to some social or political consensus to achieve equity in highway investment. Here is an example.

We may use either household income or tax payment to calculate the percentage share of financial contribution CSHij (i.e.Σi,j CSHij = 1) for each city j of region i. We may also use either population or travel demand to calculate another percentage share of travel need NSHij (i.e.Σi,j NSHij = 1) for the city. The upper bound of budget UBij for improving the shortest path SPij could be assigned as the value of B*CSHij , B*NSHij or max(B*CSHij , B*NSHij ) where B is the total budget. Taking into consideration the financial contribution or/and travel need of each city, the following budget-share constraints and the total budget constraint (Σl Al ·xl ≦ B ) can be joined together in the revised model to improve the equity in budget allocation.

l∈SPijAl⋅xl≤UBij,∀i,j (3)

For testing this idea, the revised model with these new constraints is processed using the input data from the above case study. We calculate the NSHij based on the 1998’s population for each main city. The CSHij could be calculated based on the total house income of each city, which is proportional to the product of household income and population. With the NSHij and CSHij, the upper bound of budget share for each city (as the Table 6.1) is derived and the budget-share constraints are added to the revised model.

Based on the ideal objective-values (W1*= 46.98, W2*=7.21 and W3*=0.19) of Case A in Table 5.5A, the revised model with different types of budget-share constraints (1)UBij=B*CSHij, (2)UBij=B*NSHij and (3)UBij=max(B*CSHij , B*NSHij ) were processed in fuzzy programming separately. From the results in Table 6.2, the performance of objectives in the revised model has been sacrificed to some extent after adding those constraints. In the revised model with budget-share constraint of type 1~ type 3, all the objective-values W1, W2 and W3 are worse than those of original revised model (M2). For example, with the type 1, the average travel time along shortest path has been increased from 48.87 to 51.24 minutes; the intra-regional difference and inter-regional difference in travel speed have been changed from 9.13 to 11.53 km/hr and from 1.07 to 3.79 km/hr, respectively. This phenomenon is also illustrated in the change of compromise degree in the table. In the case of type 1, the compromise degree has been decreased from 0.676 to 0.269.

In the original Case A, the time saved is 120.78 minutes and the budget is almost fully used (198.55). However, the total budget could not be fully used through the enforcement of budget-share constraints in the case study. With the budget-share constraint of type 1~ 3, only 114.2~116.9 of budget has been used and the time saved from highway investment is only 94.66~97.33 minutes.

| TABLE 6.1 Upper Bound of Budget Share for Each City |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| City | Population | House Income | NSHij | CSHij | B*NSHij | B*CSHij | Max (B*CSHij ,B*NSHij ) |
| 1.Keelung | 380744 | 1037722 | 0.173 | 0.169 | 34.643 | 33.891 | 34.643 |
| 2.Taoyuan | 299916 | 1186947 | 0.136 | 0.153 | 27.288 | 30.536 | 30.536 |
| 3.Hsinchu | 354022 | 1262533 | 0.161 | 0.192 | 32.211 | 38.339 | 38.339 |
| 4.I-lan | 92026 | 1024380 | 0.042 | 0.040 | 8.373 | 8.086 | 8.373 |
| 5.Miaoli | 90015 | 973722 | 0.041 | 0.038 | 8.190 | 7.518 | 8.190 |
| 6.Nantou | 104465 | 906808 | 0.048 | 0.041 | 9.505 | 8.125 | 9.505 |
| 7.Changhwa | 225579 | 973219 | 0.103 | 0.094 | 20.524 | 18.831 | 20.52 |
| 8.Douliou | 98361 | 808764 | 0.045 | 0.034 | 8.949 | 6.823 | 8.9496 |
| 9.Chiayi | 262936 | 1092044 | 0.120 | 0.123 | 23.923 | 24.630 | 24.630 |
| 10.Shinying | 75567 | 867293 | 0.034 | 0.028 | 6.875 | 5.621 | 6.875 |
| 11.Pingtung | 214471 | 956460 | 0.098 | 0.088 | 19.514 | 17.595 | 19.514 |

| TABLE 6.2 Revised model with/without Budget Share Constraints |  |  |  |  |
| --- | --- | --- | --- | --- |
| Result Process | Original Revised Model Case A | Type 1 | Type 2 | Type 3 |
|  |  | Ubij= B*CSHij | Ubij= B*NSHij | Ubij= Max (B*CSHij ,B*NSHij ) |
| Budget used | 198.55 | 116.9 | 115.75 | 114.2 |
| Time saved | 120.78 | 94.66 | 97.33 | 95.57 |
| Value of W1 (minutes) | 48.87 | 51.24 | 51.00 | 51.16 |
| Value of W2 (Δkm/hr) | 9.13 | 11.53 | 11.23 | 11.35 |
| Value of W3 (Δkm/hr) | 1.07 | 3.79 | 3.16 | 3.78 |
| DS1 | 0.676 | 0.269 | 0.311 | 0.283 |
| DS2 | 0.677 | 0.272 | 0.322 | 0.303 |
| DS3 | 0.827 | 0.285 | 0.411 | 0.287 |

##### 6.2 Alternatives of Travel Cost/Benefit Measure

In the revised model, the travel time serves as the travel cost measure TCM for cost efficiency in highway investment. Alternatively, the average travel speed is chosen to be an adjusted TCM for horizontal equity and vertical equity. To make the revised model more flexible in practice, the planners may redefine the TCM. The alternatives of formula for travel cost/benefit measure are discussed in 6.2.1~6.2.3.

6.2.1 Using Travel Time Weighted by Fixed Demand

The travel time weighted by the travel demand would probably be considered an alternative to the TCM. Most existing models for highway network investment problem assume that the demand for travel is given and fixed. Thus, the future demand DMijf is equal to current demand DMij. Then, the TCMij could be represented as follows:

TCMij= TTijf * DMijf =( TTij - TTij) * DMijf

=(TTij−l∈SPijTl⋅xl)∗DMij(4)

The TTijf is the future travel time along the shortest path SPij after improvement. We may get the value by subtracting the estimated saved travel timel∈SPijTlSxl from current travel time TTij collected by a real survey along the shortest path. The estimation of the saved travel time Tls occurred by the improvement of the link l is already described in section 3.2.

Estimation of travel demand usually requires knowledge of the level of service in the network (e.g. travel time between O-D pairs). Since the future transport network remains to be designed, its level of service is unknown, thus the future demand is usually estimated with an assumed level of service. However, the value of the assumed level of service for O-D demand estimation may be inconsistent with the resultant value obtained from the solution of the highway investment problem. Therefore, it might be inappropriate to tackle the problem with an assumed and fixed travel demand (Asakura and Sasaki, 1990).

6.2.2 Using Travel Time Weighted by Elastic Demand

Once the highway is improved, the associated travel demand will shift to a new level. To ensure the highway investment problem fit the real situations, we try to consider the elasticity of travel demand.

In the case where the demand is given by the monotonically decreasing demand function Dij(tij) for the shortest path SPij, and the tij is the expected, perceived travel time along SPij. Then using travel time weighted by elastic demand, the travel cost measure TCMij is represented as follows:

=(TTij−l∈SPijTlSxl)∗(DMij+Dij(TTij)'∗ΔTTij)TCMij=TTijf*DMijf=(TTij-TTij)* Dij(TTijf )

(5)

We may get the future demand DMijf from the demand function Dij for the shortest path SPij. The increment of demand could be obtained by the product of the saved travel time ⊿TTij and the Dij (TTij)', which is the slope of demand function specified at the current travel time.

Unfortunately, using travel time weighted by elastic demand as travel cost measure could result in undesirable solutions involving less investment. The objective function could be minimized just through the minimization of travel demand instead of travel time. Therefore, the alternative to TCM must be chosen deliberately.

6.2.3 Using Consumers’ Surplus as Travel Benefit Measure

Instead of the TCM in the revised model, the consumers’ surplus in travel demand could be an alternative to the travel benefit measure TBM for the highway investment. The use of consumers' surplus reflects an economic tradition that public investments should be operated to maximize social benefits. This measure has been suggested by many researchers (Kocur and Hendrickson, 1982; Williams and Lam, 1991; Yang and Bell, 1998) to evaluate the benefits of transport systems but has never been proposed for the equity issue in highway investment.

While using consumers’ surplus CSij as the travel benefit measure TBMij, it is formulated as follows:

TBMij=CSij=Dij−1(x)⋅dx−TTijfDMijf=Dij−1(x)⋅dx−(TTij−l∈SPijTlSxl)⋅(DMij+Dij(TTij)'∗ΔTTij)	(6)

The function Dij-1(dij) is the inverse of the monotonically decreasing demand function Dij(tij) for the shortest path SPij, and the tij is the averaged perceived travel time along SPij. The CSij derived by the demand function is illustrated in Figure 6.1. The practical issues of using consumers’ surplus as the travel benefit measure could be investigated in future study.

tij

dij

TTijf

DMijf

CSij

Dij-1(dij)

FIG 6.1. Consumers’ Surplus in Travel Demand

6.3 Alternative Equity Objectives

For the simplification of equity objectives in the revised model, the travel-time target model and travel-speed target model were proposed with alternative equity objectives. To replace both the Objective_W2 and Objective_W3, for horizontal and vertical equity, a single equity objective Objective_W2t for the travel-time target model and another Objective_W2s for the travel-speed target model are formulated as follows:

TTT : travel-time targetObjectives:MIN1NiijLijTCMij−TST212Objectivet:MIN1NiijTCMij−TTT212

TST: travel-speed target

The case A is processed by fuzzy programming with the TTT = 60 minutes and TST= 60 km/hr in the travel-time target model and travel-speed target model, respectively. The results for both target models are listed in Table 6.3 with results of the revised model. The EXCEL worksheet for solving these two target models are listed in Appendix 10.1~11.3. From the results, the equity objective Objective_W2s in the travel-speed target model has similar value to the Objective_W2 for the horizontal equity in the revised model. Besides, both travel-speed target model and the revised model have similar outputs in the budget used, time saved, objective values and compromise grade.

To realize the similarity of both models, the intra-regional difference and inter-regional difference in travel speed for each region are listed in Table 6.4 and 6.5. From figures in the tables, both models are similar in view of intra-regional difference but not in view of inter-regional difference.

From the Table 6.3, the solution for travel-speed model is with compromise grade about 0.64 and the budget is almost fully used. To get more information of budget required for the improvement of compromise grade, the process of Min B is proposed to replace the process of Max .

Min B =Σl Al•xl

Subject to

λ≦DSs(Ws)   s=1,2

xl ∈{0,1}, l

λ is given

Finally, for the travel-speed target model with a target of 60 km/hr, each minimal budget was obtained with different values of compromise grade and the results are listed in Table 6.6. From this table, we could realize that the budget required for the improvement of compromise grade from 0.645 to 0.70 is much less than the budget required for that improvement from 0.70 to 0.73. This information of trade-off between budget and compromise grade of solution is very important to decision-makers.

All the EXCEL worksheets for the advanced decision-making process are listed in Appendix 12.1~12.4.

| TABLE 6.3 Summary of Results from Both Target models for Case A |  |  |  |
| --- | --- | --- | --- |
| Result Process | Revised Model | Travel-time Target Model | Travel-speed Target Model |
| Budget used | 198.55 | 194.50 | 197.25 |
| Time saved | 120.78 | 105.76 | 128.46 |
| Objective value for efficiency (minutes) | 48.87 | 50.23 | 48.17 |
| Objective value for equity | 9.13 /1.07 (Δkm/hr) | 21.80 (minutes) | 9.95 (Δkm/hr) |
| Intra-regional difference | 9.13 (Δkm/hr) | 18.09 (minutes) | 9.78 (Δkm/hr) |
| Inter-regional difference | 1.07 (Δkm/hr) | 33.38 (minutes) | 1.92 (Δkm/hr) |
| DS1 | 0.676 | 0.589 | 0.646 |
| DS2 | 0.677 | 0.589 | 0.645 |
| DS3 | 0.827 | -- | -- |

| TABLE 6.4 Intra-regional Difference in travel speed (Δkm/hr) |  |  |  |
| --- | --- | --- | --- |
| Model Region | Initial State | Revised Model | Travel-speed Target Model |
| North Region | 10.19 | 8.90 | 8.71 |
| Middle Region | 13.37 | 8.26 | 8.35 |
| South Region | 19.11 | 10.44 | 12.52 |

| TABLE 6.5 Inter-regional Difference in travel speed (Δkm/hr) |  |  |  |
| --- | --- | --- | --- |
| Model Region | Initial State | Revised Model | Travel-speed Target Model |
| North Region | 4.74 | 0.82 | 0.35 |
| Middle Region | 1.13 | 1.43 | 1.75 |
| South Region | 4.82 | 0.82 | 2.80 |

| TABLE 6.6 Budget Required for Given Compromise Grades |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| λ>= Results | 0.645 | 0.70 | 0.71 | 0.72 | 0.73 |
| Budget used | 197.25 | 256.7 | 295.3 | 306.25 | 421.15 |
| Time saved | 128.46 | 130.48 | 130.87 | 131.25 | 131.60 |
| W1 (minutes) | 48.17 | 47.98 | 47.95 | 47.91 | 47.88 |
| W2 (Δkm/hr) | 9.95 | 9.71 | 9.67 | 9.60 | 9.57 |
| DS1 | 0.646 | 0.700 | 0.711 | 0.721 | 0.730 |
| DS2 | 0.645 | 0.704 | 0.713 | 0.728 | 0.732 |

### CHAPTER 7

### CONCLUSIONS AND SUGGESTIONS

### After the formulation of basic model, development of revised model, a series of case studies, and expansion of models, the conclusions and suggestions of the research are summarized in 7.1 and 7.2, respectively.

### 7.1 Conclusions

### The aim of achieving the equity in highway investment is to provide people reasonable accessibility from every city in every region to the regional center. Accessibility can be measured by the travel cost or benefit along the shortest path from the city to the regional center. To facilitate equal regional development, optimization of highway investment considers not only cost efficiency, but also horizontal equity and vertical equity.

In the decision-making of highway investment, horizontal equity and vertical equity should be considered to minimize the intra-regional difference in accessibility for the main cities in the same region and to minimize that inter-regional difference among regions, respectively.

Based on this concept, we have proposed the basic model first and modified it by the revised model to make the decision-making process more reasonable and practical for both efficiency and equity issues in highway investment. The modification has improved the capacity of the method in the following procedures:

- Necessary decision variables and constraints are included to handle the exclusive and complementary properties among alternatives.

- Travel cost measures are adjusted as the average travel speed for horizontal equity and vertical equity to deal with regions of various shapes or area sizes.

- Both objective functions for equity are revised by applying operation of the square root to their original formulas to refine the overestimated degree of satisfaction for intended objectives.

To demonstrate the decision-making process, the case study of highway investment for western Taiwan was formulated in the basic model and revised model. Through the case study, we found the following outcomes:

- Both the basic model and revised model can be solved efficiently by fuzzy programming;

- The revised model can yield practical and reasonable results to achieve cost efficiency, horizontal equity, and vertical equity in highway investment.

- The revised model can yield better results than the basic model in terms of equity.

To illustrate the flexibility and practical use of the revised model, a series of cases have been processed with different scenarios. Decision maker could realize the impact of different assumptions on highway investment. These assumptions cover the future travel speed, total budget, complementary property among links, and exclusive property among alternatives. The change of assumption is easy with the EXCEL worksheet for decision maker or planner.

For expansion of the revised model, equity in budget allocation is considered by restricting the upper bound of the budget share for each city according to its socio-economic attributes like population and household income. Different types of budget-share constraints are included in the revised model. However, according to our test, the budget-share constraints could seriously lower achievement of the objectives.

For the simplification of objectives in equity, the travel-time target model and travel-speed target model were proposed with alternative equity objectives. In the travel-speed target model with a target of 60 km/hr, each minimal budget was obtained in terms of given compromise grades for the demonstration of advanced decision-making process.

###### 7.2 Suggestions

To make the revised model more practical in use, some professional criteria and database for the inputs of model are required. Firstly, the necessary unit cost for the improvement of each type of links should be built into the database. Secondly, the information of current travel time and link length should be collected routinely by field surveys with travel speed and mileage recorder. With the GPS receiver, the topology data of highway network can be easily collected and it is better to use GIS to store those data for more applications. Thirdly, the standard for assuming the future travel speed for each type of links and the criteria for defining compliment property among links should be collected by capacity analyses or interviews with the professionals.

To make the revised model more flexible in application, alternative travel cost/benefit measures were examined and consumers’ surplus in elastic travel demand was proposed as a travel benefit measure in the future study. Besides, the weight for each objective could be included in the model and then the compromised grade derived by the membership functions will be affected by the given weights. The impact of the integration of different methods of weighting will be an interesting topic for decision-making.

Excel was used to process the fuzzy programming to solve the multi-objective model. With Excel, changing parameters and initial states to obtain the solutions is easy. Owing to the nonlinear equity objectives, the local optimal may be unintentionally treated as the global optimal. If we integrate a Macro program to handle the mutation of decision variables such as the genetic algorithm, the local optimal could have more chances to jump to the global optimal.

For expansive applications, the travel cost/benefit measures in the revised model are not only used in the model for the decision-making of highway investment, but also used to show the historical performance of highway in view of efficiency and equity, which is important for sustainable development in transportation system.

REFERENCES

- Asakura, Y., and Sasaki, T. (1990). “Formulation and feasibility test of optimal road network design model with endogenously determined travel demand.” The 5th World Conference on Transport Research, Yokohama, Japan, 351-365.

- Bhattacharya, U., Rao, J. R. and Tiwari, R.N. (1992). “Fuzzy multi-criteria facility location problem.” Fuzzy Sets and Systems, 51, 277-287.

- Bit, A. K., Biswal, M. P. and Alam, S .S. (1992). “Fuzzy programming approach to multicriteria decision making transportation problem.” Fuzzy Sets and Systems, 50, 135-141.

- Bit, A. K., Biswal, M. P. and Alam, S .S. (1993). “Fuzzy programming approach to multiobjective solid transportation problem.” Fuzzy Sets and Systems, 57, 183-194.

- Boyce, D. E. (1984). “Urban transportation network equilibrium and design models: recent achievements and future prospectives.” Environment and Planning, 16 A, 1445-1474.

- Boyce, D. E., and Jason, B. N. (1980). “A discrete transportation network design problem with combined trip distribution and assignment.” Transportation Research, 14B, 147-154.

- Chang, C. J. and Chang S. H. (1992). “A heuristic algorithm for solving the discrete network design problem.” Transportation Planning and Technology, 16, 39-50.

- Chen, M., and Alfa, A. S. (1991). “A network design algorithm using a stochastic incremental traffic assignment approach.” Transportation Science, 25, 215-224.

- Chitwood, S. R. (1974). “Social equity and social service productivity.” Public Administration Review, 34, 29-35.

- Current, J. R., Revelle, C. S. and Cohon, J. L. (1987). “The median shortest path problem: a multiobjective approach to analyze cost vs. accessibility in the design of transportation networks.” Transportation Science, 21, 188-197.

- Dear M. J. (1978). “Planning for mental Health care: a reconsideration of public Facility Location Theory.” International Regional Science Review, 3, 93-111.

- Dyson, R.G.. (1980) “Maximin programming, fuzzy linear programming and multi-criteria decision making.” Journal of Operational Research Society, 31, 263-267.

- Friesz, T. L. (1985). “Transportation network equilibrium, design and aggregation: key developments and research opportunities.” Transportation Research, 19A, 413-427.

- Friesz, T. L., Anandalingam, G., Mehta, N. J., Nam, K., Shan, S. J. and Tobin, R. L. (1993). “The multiobjective equilibrium network design problem revisited: a simulated annealing approach.” European Journal of Operational Research, 65, 44-57.

- Kocur, G., and Hendrickson, C. (1982). “Design of local bus service with demand equilibrium.” Transportation Science, 16, 149-170.

- LeBlanc, L. (1975). “An algorithm for the discrete network design problem.” Transportation Science, 9, 183-199.

- LeBlanc, L. J. and Boyce, D. E. (1986). “A bilevel programming algorithm for each solution of the network design problem with user-optimal flows.” Transportation Research, 21B, 259-265.

- Li, R. J. and Lee, E.S. (1990) “Fuzzy approaches to multicriteria de novo programs.” Journal of Mathematical Analysis and Applications, 153, 97-111.

- Lee, E. S. and Li, R. J. (1993). “Fuzzy multiple objective programming and compromise programming with Pareto optimum.” Fuzzy Sets and Systems, 53, 275-288.

- Magnanti, T. L. and Wong, R. T. (1984). “Network design and transportation planning: models and algorithms.” Transportation Science, 18,1-55.

- Martinson, F. K.(1993). “Fuzzy vs. minmax weighted multiobjective linear programming illustrative comparisons.” Decision Sciences, vol. 24, No. 4, 809-824.

- McAllister, D. M. (1976). “Equity and efficiency in public facility location.” Geographical Analysis, vol. 8, 47-63.

- Mohanty, B.K. and T. A. S. Vijayaraghavan (1995). “A multi-objective programming problem and its equivalent goal programming problem with appropriate priorities and aspiration levels: a fuzzy approach.” Computers Ops. Res., vol.22, No. 8,771-778.

- Morrill, R. L., Symons, J. (1977). “ Efficiency and equity aspects of optimal location.” Geographical Analysis, vol. 9, 215-225.

- Sasaki, M., Gen, M. and Yamashiro, M. (1995). “A method for solving fuzzy de novo programming problem by genetic algorithms.” Computers Ind. Eng., 29(1-4), 507-511.

- Suwansirikul, C., Friesz, T. L. and Tobin, R. L. (1987). “Equlibrium decomposed optimization: a heuristic for the continuous equilibrium network design problem.” Transportation Science, 21, 254-263.

- Truelove, M. (1993). “Measurement of spatial equity.” Environment and Planning C: Government and Policy 11,19-34.

- Tzeng, G. H. and Chen, C. H. (1993). “Multiobjective decision making for traffic assignment.” IEEE Transaction on Engineering Management, 40, 180-187.

- Tzeng, G. H. and Tsaur, S. H. (1997). “Application of multiple criteria decision making for network improvement.” Journal of Advanced Transportation, 31(1), 49-74.

- William, H. C. W. L., and Lam, W. M. (1991). “Transport policy appraisal with equilibrium models 1: generalized traffic and highway investment benefits.” Transportation Research, 25B, 253-279.

- Yang, H. and Bell, M. G. H. (1998). “Models and algorithms for road network design: a review and some new developments.” Transport Reviews, 18(3), 257-278.

- Zimmermann, H. J. (1978). “Fuzzy programming and linear programming with several objective functions.” Fuzzy Sets and Systems 1, 45-55.

APPENDIX CONTENTS

### Appendix 1.1  Basic Model for Case A—Min W1

### Appendix 1.2  Basic Model for Case A—Min W2

### Appendix 1.3  Basic Model for Case A—Min W3

### Appendix 1.4  Basic Model for Case A—Max Lamda

### Appendix 2.1  Basic Model for Case B—Min W1

### Appendix 2.2  Basic Model for Case B—Min W2

### Appendix 2.3  Basic Model for Case B—Min W3

### Appendix 2.4  Basic Model for Case B—Max Lamda

### Appendix 3.1  Revised Model for Case A—Min W1

### Appendix 3.2  Revised Model for Case A—Min W2

### Appendix 3.3  Revised Model for Case A—Min W3

### Appendix 3.4  Revised Model for Case A—Max Lamda

### Appendix 4.1  Revised Model for Case B—Min W1

### Appendix 4.2  Revised Model for Case B—Min W2

### Appendix 4.3  Revised Model for Case B—Min W3

Appendix 4.4  Revised Model for Case B—Max Lamda

Appendix 5.1  Revised Model for Case C—Min W1

Appendix 5.2  Revised Model for Case C—Min W2

Appendix 5.3  Revised Model for Case C—Min W3

Appendix 5.4  Revised Model for Case C—Max Lamda

Appendix 6.1  Revised Model for Case D—Min W1

Appendix 6.2  Revised Model for Case D—Min W2

Appendix 6.3  Revised Model for Case D—Min W3

Appendix 6.4  Revised Model for Case D—Max Lamda

Appendix 7.1  Revised Model for Case E—Min W1

Appendix 7.2  Revised Model for Case E—Min W2

Appendix 7.3  Revised Model for Case E—Min W3

Appendix 7.4  Revised Model for Case E—Max Lamda

Appendix 8.1  Revised Model for Case F—Min W1

Appendix 8.2  Revised Model for Case F—Min W2

Appendix 8.3  Revised Model for Case F—Min W3

Appendix 8.4  Revised Model for Case F—Max Lamda

Appendix 9.1  Revised Model for Case A—Type 1 of Budget-share Constraints

Appendix 9.2  Revised Model for Case A—Type 2 of Budget-share Constraints

Appendix 9.3  Revised Model for Case A—Type 3 of Budget-share Constraints

Appendix 10.1  60minutes Model for Case A—Min W1

Appendix 10.2  60minutes Model for Case A—Min W2

Appendix 10.3  60minutes Model for Case A—Max Lamda

Appendix 11.1  60 Km/Hr Model for Case A—Min W1

Appendix 11.2  60 Km/Hr Model for Case A—Min W2

Appendix 11.3  60 Km/Hr Model for Case A—Max Lamda

Appendix 12.1  60 Km/Hr Model for Case A—Lamda>0.7

Appendix 12.2  60 Km/Hr Model for Case A—Lamda>0.71

Appendix 12.3  60 Km/Hr Model for Case A—Lamda>0.72

Appendix 12.4  60 Km/Hr Model for Case A—Lamda>0.73

PS. If the Excel file in each appendix is required, please contact the author.