### CHAPTER 1

### INTRODUCTION

The background and objective of this research are presented in section 1.1. To deal with the equity issues in highway investment decision-making, the concept of equity and its relation to the highway investment are briefly described in 1.2. The research scope and approach are separately introduced in 1.3 and 1.4, and the research framework of this dissertation is presented in 1.5.

1.1 Research Background and Objective

Highway investment involves making optimal decisions on the improvement and expansion of a highway network system to meet the growing demand for travel in order to select improvement or addition of links to an existing network under budget constraints for the maximization of social welfare. Past researches for solving this problem mostly focus on the issue of efficiency in cost or travel-time instead of the issue of equity. However, to facilitate equal regional development, the achievement of equity in highway investment is very important. The objective of the research is to develop a practical method for highway investment decision-making to deal with both issues of efficiency and equity.

1.2 Concept of Equity

Equity implies social or political consensus about the fairness of the distribution of costs and benefits of a policy or program (Dear, 1978). In the decision-making of highway investment, the concepts of ‘horizontal equity’ and ‘vertical equity’ should be adopted to justify the distribution of travel costs and benefits for people. Horizontal equity means that persons in like circumstances should be treated identically (Truelove, 1993), and vertical equity refers to developing a rationale for allocating services among people who are in different circumstances and have various socio-economic attributes (Chitwood, 1974). In highway investment, horizontal equity should focus on people who live in the same region or in regions with similar circumstances, while vertical equity could focus on people who live in different regions of various sizes, population, income levels, or travel demands.

1.3 Research Scope

To facilitate equal regional development, we should provide people reasonable accessibility from every main city in every region to the regional center. The equity issues in research scope cover horizontal (intra-regional) equity and vertical (inter-regional) equity of the accessibility from main cities to the regional center as well as the equity of budget allocation among main cities. Accessibility can be measured by the travel cost or benefit along the shortest path from the city to the regional center.

Based on the idea of horizontal equity, all the main cities in the same region should have the same travel cost or benefit measure to the greatest extent to connect with the regional center. Also based on the idea of vertical equity, accessibility of each city in different regions is expected to be as reasonably fair as possible. Consequently, the research focuses on the formulation of models and the problem-solving algorithm for achieving the efficiency with both vertical equity and horizontal equity in highway investment decision-making.

1.4 Research Approach

Based on the concept of horizontal equity and vertical equity, a basic model of multi-objective formulation is first proposed to achieve both efficiency and equity in the decision-making of highway investment. In the model, we not only optimize the accessibility for all main cities in view of efficiency, but also minimize the difference in accessibility among the cities in the same region and justify that difference among all the regions simultaneously to maintain both horizontal equity and vertical equity.

Then, the basic model is modified as a revised model to make the decision-making process more reasonable and practical for the equity issues. The procedures of development for the revised model are as follows: Necessary constraints and decision variables are added to the basic model to handle the exclusion and complement properties among alternatives; Necessary adjustments for travel cost measures and objective functions are proposed to justify the horizontal equity and vertical equity for general cases.

For the trade-off between objectives of efficiency and equity, the fuzzy programming approach is used to design the weights for objectives and to solve the multi-objective models to achieve compromised solutions for the problem of highway investment.

The highway investment also involves the equity in budget allocation among main cities, which could be processed in models by restricting the upper bound of the budget share for each city according to its socio-economic attributes.

Case studies of highway investment for western Taiwan are processed with different scenarios to demonstrate the performance of the proposed models and fuzzy programming approach, and also show the impact of assumptions (for example, future travel speed, total budget, complementary property among links, and exclusive property among alternatives) on compromised solutions.

1.5 Dissertation Framework

The Chapter 1 gives an overview of this research in terms of background and objective, concept of equity, research scope and approach, and the framework of dissertation. In Chapter 2, past researches for highway investment decision-making and the features of fuzzy programming are reviewed. Owing to the lack of research in the trade-off between efficiency and equity in highway investment, the similar topic in facility location problem is reviewed instead. Then, the basic model and its detailed solving process by fuzzy programming will be described in Chapter 3. After that, the formulation of the revised model is developed and elaborated in Chapter 4. Case studies of highway investment for western Taiwan with different scenarios are processed in Chapter 5 to demonstrate the performance of the models. For the expansion of the revised model for more flexible uses, the equity in budget allocation, alternative travel cost/benefit measures and equity objectives are explored in Chapter 6. Finally, the conclusions and suggestions of the research are summarized in Chapter 7.

### CHAPTER 2

### LITERATURE REVIEW

Past researches for the highway investment decision-making mostly focus on the issue of efficiency instead of the equity. These traditional methods are reviewed in section 2.1. Owing to the lack of research in the trade-off between efficiency and equity in highway investment, the similar topic in facility location problem is reviewed instead in section 2.2. The fuzzy programming approach is reviewed in 2.3 to realize its capacity to achieve compromised solutions for the multi-objective problem.

2.1 Past Researches for Highway Investment Decision-Making

For the highway investment decision-making, some transportation planners regard it as a network design problem (NDP) to determine the appropriate investment alternatives. Friesz (1985), Magnanti and Wong (1984), Boyce (1984) and Yang and Bell (1998) conducted comprehensive surveys on the modeling and algorithmic development for those mathematical programming-based NDPs.

From those surveys, we observed that ‘cost efficiency’ in travel time or budget was usually concerned in single-objective NDPs (LeBlanc, 1975; Boyce and Jason, 1980; Suwansirikul et al., 1987; Chen and Alfa, 1991). Also, we found that various kinds of cost items were proposed in multi-objective NDPs. Corrent et al. (1987) considered the trade-off between operator cost for paths and user cost to reach the path. Tzeng and Chen (1993) evaluated the social cost by total travel distance, total travel time and total air pollution. Tzeng and Tsaur (1997) concerned total travel time and total investment cost in their model. Friesz et al. (1993) used total user transport costs, total construction costs, total vehicle miles traveled, and total dwelling units taken for rights-of-way. In those studies, researchers pursued the optimization of investment performance for the network only in view of efficiency, without considering the equity issues. In general, the issue of equity is seldom mentioned in either single-objective NDPs or multi-objective NDPs.

Actually, it is difficult to solve the NDP since two sets of decision makers with different objectives are inherently involved (LeBlanc and Boyce, 1986): the road users are engaged in ‘user optimization’ at the lower level and the government in ‘system optimization’ at the upper level. According to past researches (for example, Tzeng and Tsaur, 1997), the bi-level model is NP-hard (Nondeterministic Polynomial Hard) and that problem could not be solved by a polynomial algorithm. Especially for a nonlinear bi-level model, the problem becomes non-convex and the global optimal solution becomes even more unreachable. Therefore, only some heuristic algorithms (for example, Suwansirikul et al., 1987; Chang and Chang, 1992) have been used to solve large-scale network design problems.

Besides, while solving the user optimization in NDP, the travel demands for all O-D pairs in the network and travel-cost functions for all the links are necessary inputs but those data are hard to obtain.

2.2 Efficiency-Equity Trade-off in Facility Location Problem

In optimal location model, the most frequently used measure of efficiency is transportation distance or cost. Usually, total transportation distances of the system under study are minimized. However, to achieve greater equity in accessibility of population to facilities, Morrill and Symons (1977) suggest three possible approaches, all of which might decrease efficiency and increase costs.

Equity and efficiency are two distinct goals of a system of public facilities that often conflict. The optimal location of an extra facility from an efficiency viewpoint is likely to be different from the optimal location from an equity viewpoint (Truelove, 1993). To maximize efficiency, an additional facility might be located in an area of high population density to minimize average travel costs; to maximize equity, it might be located in a distant low-density area, to minimize the maximum distance that people must travel. The question then becomes: how much efficiency to trade off for equity?

From the research of McAllister (1976), there is no objective method of determining the ‘optimal’ size and spacing of public facilities. Fortunately, Bhattacharya, Rao and Tiwari (1992) solved this multi-objective problem by fuzzy programming to obtain the compromised solution. The first objective is to maximize the minimum distances from the facility to be located and the existing facilities. The second objective is to minimize the maximum distances from the facility to the demand points. The third objective is to minimize the sum of all the transportation costs. As the result, the efficiency and equity have been considered simultaneously. The fuzzy programming is reviewed in next section.

2.3 Features of Fuzzy Programming

Fuzzy programming is the fuzzy set theory allied with multi-criteria decision-making problems. The objective functions are represented by fuzzy set, and the decision rule is to select the solution having the highest membership of the decision sets. Zimmermann (1978) first applied fuzzy set theory concept with suitable choices of membership functions and derived a fuzzy linear program which is identical to the maximin program (Dyson, 1980). He shows that solution obtained by the fuzzy programming are always efficient solutions and gives an optimal comprised solution. Dyson (1980) claims that the fuzzy linear programming model of Zimmermann is an innovation in the field of multi-criteria decision-making. This approach is similar, in many respects, to the weighted linear goal programming method (Bit et al., 1992) The weighted linear goal programming depends on the development of weights, whereas fuzzy programming uses fuzzy membership functions (Mohanty, 1995).

The fuzzy programming and compromise programming are essentially equivalent under certain conditions (Lee and Li, 1993). In the compromise programming, the weights express the importance of the relative deviation of the objectives from the ideal, but in the fuzzy programming, they express the importance of the deviations from the anti-ideal (Martinson, 1993).

Currently the fuzzy programming technique is applied to solve multi-objective linear, as well as nonlinear, programming problems. In many researches, the fuzzy programming serves as a good method to find compromised solutions for the multi-objective optimization problem. Bit, Biswal and Alam (1992,1993) concluded that using fuzzy programming for the large problem, one can easily find a compromise solution, and this algorithm is applicable to all types of multi-objective transportation problem.

The fuzzy programming has been used in many fields. Lee and Li (1990) proposed a two-phase approach to get non-dominated solution and adapt it to de Novo programming with fuzzy parameters in objectives and constraints. Bhattacharya et al. (1992) used the approach to solve a multi-objective facility location problem, which has been mentioned in section 2.2. Besides, genetic algorithm has been proved to be able to solve fuzzy programming with fuzzy goal of nonlinear function and fuzzy nonlinear constraints (Sasaki et al. 1995). All the achievements in past researches make fuzzy programming more practical in applications.

### CHAPTER 3

### BASIC MODEL AND FUZZY PROGRAMMING

In this chapter, the basic method for highway investment is proposed to deal with both issues of equity and efficiency. The basic model is formulated in the multi-objective form, and it is solved by the fuzzy programming approach to get the compromised solution. The problem statements and basic concepts for the model are described in 3.1. The formulation and the solution algorithm of the model are illustrated in 3.2 and 3.3, respectively. The Properties of the basic model are specified in 3.4 and a simple case is processed to show those properties in 3.5.

3.1 Problem Statements and Basic Concepts

The highway investment decision-making is to select link improvement or link additions to an existing network under budget constraints for the maximization of social welfare. For solving this problem, past researches mostly pursue the minimization of cost or travel-time to achieve the maximization of social welfare in view of efficiency. Actually, the equity issue is important as the efficiency in social welfare and we need a model/procedure to handle the trade-off between efficiency and equity in highway investment. The concept for the development of a basic model for highway investment to deal with both issues is explained as follows.

In highway investment, we should provide people reasonable accessibility from every main city in every region to the regional center because much more large-scale or high-quality facilities (for example, hospitals, entertainment places, museums, etc.) are generally located in regional centers. The accessibility can be measured by the travel cost or benefit along the shortest path from the main city to the regional center.

As mentioned in Chapter 1, the horizontal equity and the vertical equity are relative to develop the rationale for allocating services among people who are in similar and different circumstances, respectively. In highway investment, the horizontal equity could deal with the people who live in the same region; the vertical equity could deal with the people who live in different regions.

In view of horizontal equity, all the main cities in the same region should have the same accessibility to the greatest extent to connect with the regional center. In view of vertical equity, the accessibility of each city in different regions is expected to be as reasonably fair as possible.

Consequently, while improving the accessibility for all the main cities to achieve efficient highway investment, we must simultaneously attend to equal accessibility among the cities to maintain both horizontal equity and vertical equity.

3.2 Formulation of the Basic Model

In the basic model, the objective of efficiency in highway investment is achieved by the optimization of accessibility for all the main cities. The horizontal equity and vertical equity are assured by the minimization of the difference in accessibility among main cities in the same region and that difference in various regions, respectively. The multi-objective formulation of the basic model (M1) is as follows:

TCMij=l∈SPijTlf=TTij−l∈SPijTls⋅xl=l∈SPijTlc−l∈SPijTls⋅xlObjective:MIN1Ni1NijTCMij−1NiijTCMij2Objective:MIN1NiijTCMij−1NijTCMij2Objective:MIN1NiijTCMij

Subject to

Σl Al ·xl ≦ B

xl ∈{0,1}, ∀l

Notation:

Al : cost needed for improvement of link l

B : total budget

Dl : length of link l

Ni : number of main cities in region i ( regional center is not counted )

N : number of regions in problem

Slf : future travel speed along link l while it is improved

SPij : the set of links that comprise shortest path connecting main city j of region i to regional center

Tlc : current travel time along link l

Tlf : future travel time along link l while it is improved

Tls : saved travel time along link l while it is improved

( Tls = max(0 , Tlc -Tlf) = max(0 , Tlc - Dl / Slf ))

TCMij: travel cost measure for city j in region i

TTij : current travel time along SPij

xl : decision variable for improvement of link l

Actually, accessibility in this model is measured by the travel time along the shortest path SPij from each main city j of region i to the regional center. Travel time serves as an anti-accessibility measure and we may call it travel cost measure TCMij. Therefore, Objective_W1 is to optimize the cost efficiency by minimizing the average travel cost measure for all the main cities in all regions. Objective_W2 is to optimize the horizontal (intra-regional) equity by minimizing the difference in travel cost measure among main cities in the same region. Objective_W3 is to optimize the vertical (inter-regional) equity by minimizing the difference in travel cost measure among all the main cities in various regions.

TheΣl Al·xl ≦ B is the budget constraint for investment, and the discrete decision variables xl are used in the model, xl will be equal to 1 when link l is selected to be improved. For a practical reason, links of the shortest paths are selected to be the alternatives for improvement, and those shortest paths are determined by the field survey of travel time.

The TCMij is defined as l∈SPijTlf, the future-travel-time along the shortest path SPij. The value of l∈SPijTlf is derived from the difference between the current-travel-time TTij (=l∈SPijTlc) and the estimated saved-travel-timel∈SPijTls⋅xl along SPij. The value of TTij can be obtained from the field travel survey.

Estimation of Tls, the time saved by the improvement of the link l, requires information on future-travel-speed Slf along that link. Value of Tls is necessary to be positive for link improvement, so we define that Tls = max(0 , Tlc -Tlf) = max(0 , Tlc - Dl / Slf ), where Tlc is the current-travel-time along link l and Dl is the length of link l. The future-travel-speed could be assumed as a conservative value on account of the increasing demand for highway improvement.

3.3 Solution Algorithm: Fuzzy Programming

The solution algorithm for the basic model is fuzzy programming. The features of the algorithm have been reviewed in section 2.3. From the review, some researchers concluded that fuzzy programming is applicable to all types of multi-objective transportation problem. Beside, using this algorithm, one can easily find a compromised solution for the large problem.

To follow the procedures of that method, we first need to get the ideal solution I* = (W1*, W2*, W3*) and anti-ideal solution I# = (W1#, W2#, W3#) for the basic model. Each Ws* shows the independently optimal performance of its Objective_ Ws ( = Min Ws , s=1,2,3), and each Ws# shows the worst possible performance of its objective while optimizing other objectives. Both Ws* and Ws# are used as the reference points to define the membership function DSs(Ws), which indicates the degree of satisfaction for each Objective_Ws. The membership functions are defined as follows:

1			     Ws ≦Ws*

DSs(Ws) =  (Ws#-Ws)/( Ws#- Ws*)   Ws * < Ws ≦Ws#			           		      0              Ws > Ws# ,

s=1,2,3      (1)

A compromise-gradeλis defined to represent the degree of overall satisfaction of the optimization and is expressed as

λ=MINs{ DSs(Ws)| s=1,2,3}.	 							(2)

Through maximization of theλ, we may get compromised solutions for the basic model (M1). Therefore, the multi-objective problem is the following problem:

Max λ

Subject to

λ≦(Ws#- Ws)/( Ws#- Ws*),   s=1,2,3

Σl Al•xl ≦ B

xl ∈{0,1}, ∀l

λ∈[0,1]

By solving this problem, we may get all the values of decision variable xl , compromise-gradeλ, and compromised objective Ws with its degree of satisfaction DSs in the basic model.

For the purpose of achieving the highest compromise-grade, the budget may not be fully used. To deal with this issue, Lee and Li (1990) proposed a two-phase fuzzy programming by introducing a fully compensatory operator ‘averaging’ and restrictingλs≧λ,s in the model. While adopting this approach, the following program is processed after obtaining the compromised-gradeλ.

Max  13(λ1 +λ2+λ3 )

Subject to

λ≦λs ≦(Ws#- Ws)/( Ws#- Ws*),  s=1,2,3

Σl Al•xl ≦ B

xl ∈{0,1}, ∀l

λs ∈[0,1],  s=1,2,3

In this research of highway investment decision-making, the main purpose is to get compromised solution for efficiency and equity, rather than to get full utilization of budget. Therefore, only the process of Min W1, Min W2, Min W3 and Max  will be executed in the fuzzy programming.

3.4 Properties of Basic Model

From the formulation of the basic model with its solution algorithm, the properties for this model are specified in this section.

Objective:MIN1NiijTCMij	In view of efficiency, the Objective_W1 is to minimize the average travel cost measure for all the main cities in all regions.

With the definition of the travel cost measure TCMij in the basic model, there exist equations as follows:

TCMij=TTij−l∈SPijTls⋅xl

when Tls> 0,   Tl = Tlc -Tlf = Tlc - Dl / Slf = Dl / Slc -Tlf

= Dl / Slc - Dl / Slf = Dl (1/ Slc - 1 / Slf )

Through these equations, we may derive the property 1 for the Objective_W1.

Property 1:

Based on the Objective_W1, the criteria for link selection under budget is to make the travel time saved as more as possible. Besides, with the assumption that the improvement cost for each link is equal, the propositions for link selection in view of efficiency are listed in the Table 3.1.

| TABLE 3.1 Propositions for Link Selection in View of Efficiency |  |  |  |  |
| --- | --- | --- | --- | --- |
| Proposition no. | Fixed items | If | Then | Equation for deriving the proposition |
| 1 | TTij | Tls  | TCMij  | TCMij=TTij−l∈SPijTls⋅xl |
| 2 | Tlc | Tlf  | Tls  and TCMij  | Tls = Tlc -Tlf |
| 3 | Tlf | Tlc  | Tls  and TCMij  |  |
| 4 | Tlc , Dl | Slf  | Tls  and TCMij  | Tls = Tlc - Dl / Slf |
| 5 | Tlc , Slf | Dl  | Tls  and TCMij  |  |
| 6 | Dl , Tlf | Slc  | Tls  and TCMij  | Tls = Dl / Slc -Tlf |
| 7 | Slc , Tlf | Dl  | Tls  and TCMij  |  |
| 8 | Slc , Slf | Dl  | Tls  and TCMij  | Tls = Dl (1/ Slc - 1 / Slf ) |
| 9=4 | Dl , Slc | Slf  | Tls  and TCMij  |  |
| 10=6 | Dl , Slf | Slc  | Tls  and TCMij  |  |

In view of horizontal equity, The Objective_W2 is to minimize intra-regional difference in travel cost measure among main cities in the same region. Thus, all the main cities in the same region will have the same travel cost measure to the greatest extent to connect with the regional center.

The usage of square operation could give penalty to some alternative, which has extra-small deviation for one region and extra-large deviation for another region. The property for Objective_W2 is represented as follows:

Property 2:

Based on the Objective_W2, the criteria for link selection under budget is to make the intra-regional difference TCMij−1NijTCMijObjective:MIN1NiijTCMij−1NijTCMij2 for each region as lower as possible and the budget may not be totally used.

Objective:MIN1Ni1NijTCMij−1NiijTCMij2In view of vertical equity, the accessibility (travel cost measure) of each city in different regions is expected to be as reasonably fair as possible. The Objective_W3 could minimize the inter-regional difference in travel cost measure among all the main cities in various regions to meet the requirement of horizontal equity.

As the same as Objective_W2, the usage of square operation could give penalty to some alternative, which has extra-small deviation for one region and extra-large deviation for another region. The property for Objective_W3 is specified as follows:

Property 3:

Based on the Objective_W3, the criteria for link selection under budget is to make the inter-regional difference 1NijTCMij−1NiijTCMij for each region as lower as possible and the budget may not be totally used.

In view of compromise grade, the weight for each objective is derived by membership function in fuzzy programming (in section 3.2) rather than given by a manual operation. Each Objective_Ws (= Min Ws , s=1,2,3) is transferred to a membership function DSs(Ws), which indicates the degree of satisfaction for each Objective_Ws, by the best value and worst value of that objective function. The best value can be obtained by independent optimization of that objective. The worst value of a certain objective could be derived from results of the optimization of other two objectives and this value could serve as a bottom line for that objective. The property for the compromised solution from the fuzzy programming is specified as follows:

Property 4:

Based on the objective (=Max λ, λ DSs(Ws), s=1,2,3) in fuzzy programming, the criteria for link selection under budget is to make the membership function DSs(Ws) for each Objective_Ws (= Min Ws , s=1,2,3) as higher as possible and simultaneously make those DSs(Ws) as closer as possible to each other.

3.5 Test with a Simple Case

To show the properties of the basic model, a test with a simple case is processed. The shape of the networks in the case is illustrated in Fig. 3.1. The characteristics of the simple case and its link attributes for inputs of the basic model are listed in Table 3.2 and Table 3.3. The total budget is 200 units and the improvement cost of each link is proportional to its length.

Region 1

Region 2

A

B

a1

a2

b2

b1

1

2

3

4

5

6

7

8

FIG. 3.1 Shape of Simple Networks for Test

| TABLE 3.2 Characteristics of Simple Case for Test |  |  |  |  |
| --- | --- | --- | --- | --- |
| Region | Regional Center | Main City | Shortest Path | Links |
| 1 | A | a1 | SP11: a1 A | 1,2 |
|  |  | a2 | SP12: a2 A | 3,4 |
| 2 | B | b1 | SP21: b1 B | 5,6 |
|  |  | b2 | SP22: b2 B | 7,8 |

| TABLE 3.3 Link Attributes in Simple Case for Basic Model |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Shortest Path | Link No. | Decision variables | Improvement Cost (units/km) | Future Travel Speed Slf (Km/Hr) | Current Travel Time Tlc (minutes) | Link Length Dl (Km) |
| SP11: a1 A | 1 | x1 | 5 | 80 | 40 | 20 |
|  | 2 | x2 | 5 | 80 | 40 | 20 |
| SP12: a2 A | 3 | x3 | 5 | 80 | 30 | 20 |
|  | 4 | x4 | 5 | 80 | 30 | 20 |
| SP21: b1 B | 5 | x5 | 5 | 80 | 35 | 20 |
|  | 6 | x6 | 5 | 80 | 30 | 20 |
| SP22: b2 B | 7 | x7 | 5 | 80 | 30 | 20 |
|  | 8 | x8 | 5 | 80 | 30 | 20 |

To formulate the basic model for the simple case, the travel cost measure TCMij for each shortest path is needed. The Tls, the time saved by the improvement of the link l, is also needed to compute the TCMij as follows:

TCMij=l∈SPijTlc−l∈SPijTls⋅xl

when Tls> 0, Tls = Tlc -Tlf = Tlc – 60*(Dl / Slf)

 TCM11 =(40+40)-(25x1+25x2)

 TCM12=(30+30)-(15x3+15x4)

 TCM21=(35+30)-(20x5+15x6)

 TCM22=(30+30)-(15x7+15x8)

i jTCMij= (TCM11+TCM12+TCM21+TCM22)

Then the objectives and constraints of basic model for the simple case are represented as follows:

Objective_W1: MIN (i jTCMij)/4

Objective_W2: MIN ((TCM11 –(TCM11 + TCM12 )/2)2

+ (TCM12 –(TCM11 + TCM12 )/2)2

+ (TCM21 –(TCM21 + TCM22 )/2)2

+ (TCM22 –(TCM21 + TCM22 )/2)2)/4

Objective_W3: MIN (((TCM11 + TCM12 )/2 -i jTCMij /4)2

+ ((TCM21 + TCM22 )/2-i jTCMij /4)2)/2

Subject to

100(x1+ x2 + x3+ x4+ x5+ x6+ x7+ x8) ≦ 200

xl ∈{0,1}, l =1,2…8

Using the SOLVER of Excel, a linear/non-linear (integer) programming tool, the simple case is solved by the fuzzy programming. The procedure for solving a linear or non-linear programming has three steps: (1) design the worksheet for the objective and decision variables and define constraints; (2) set up the options for the search method, iteration, tolerance, precision and convergence; and (3) execute and wait for the message of results. Each step of the procedure for the process of Min W1 by SOLVER is illustrated in Fig. 3.2~3.4. The Excel worksheet for initial state is shown in the Table 3.4. Each process of Min W1, Min W2, Min W3 and Max  in the fuzzy programming with its Excel worksheet is shown in Table 3.5~3.8, separately.

Each DSs(Ws) for Objective_Ws (= Min Ws) is derived by the best value Ws* and worst value Ws# of the Ws from the process of Min W1, Min W2 and Min W3.

DSs(Ws)=(Ws#- Ws)/( Ws#- Ws*)

DS1(W1) =(62.5-W1)/(62.5-53.75)

DS2(W2) =(156.25-W2)/(156.25-6.25)

DS3(W3) =(76.563-W3)/(76.563-0)

Then, the simple case is transformed into the process of Max  as follows:

Max λ

Subject to

λ DS1(W1)

λ DS2(W2)

λ DS3(W3)

100(x1+ x2 + x3+ x4+ x5+ x6+ x7+ x8)  200

xl ∈{0,1}, l =1,2…8

λ∈[0,1]

The summary of results for the case is listed in Table 3.9. In the process of Min W1, the efficiency is achieved with the highest saved travel-time and fully used budget. The horizontal equity and vertical equity are separately acquired in the process of Min W2 and Min W3, but only 50% of budget is used in both processes. Finally, the compromised solution and the degree of satisfaction for each objective are obtained in the process of Max . To realize the status of horizontal equity and vertical equity for each process, the squared intra-regional difference and squared inter-regional difference for each region are listed in Table 3.10 and 3.11, respectively. All the properties mentioned in section 3.4 are shown in the demonstration of the simple case.

FIG. 3.2 Procedure for Min W1 by SOLVER(Step 1)

FIG. 3.3 Procedure for Min W1 by SOLVER ( Step 2)

FIG.3.4. Procedure for Min W1 by SOLVER (Step 3)