關鍵詞：模糊數排序、模糊隨機變數

近年來隨著模糊理論逐漸發展成熟，已有大量研究文獻投入不確定性問題討論範疇，在國內部份，亦有許多學者積極參與對模糊理論應用於不確定性問題的探討，其中在模糊數排序課題上之應用研究，包括方案評選、路徑選擇等諸多課題，亦多直接套用現成的隨機模式，或透過去模糊化程序，直接跳過問題核心的不確定性，然而此種方式，並未針對模糊數選擇之隨機結果，進行本質及特性的深入探究。

交通路網環境由於受到很多因素的影響，存在著高度不確定性，因此當使用者根據交通資訊進行決策時，亦存有不確定的現象。以往國內、外對於多應用隨機分佈的假設研究，係以邏輯特模式進行分析，但對於交通資訊存有模糊或同時具有模糊隨機特性的資訊，在比較與選擇上則無法有效的進行評估。本研究主要針對連續型態的模糊隨機性資訊進行排序與比較，證明並提出全新的機率預測通式與演算法，據此，並針對淡水路網資料，以運量指派問題，驗證此通式於大型路網之適用性。

Keywords: ordered fuzzy number  fuzzy random variable

Although, the fuzzy theorem is in blossom today, a lot of research papers named fuzzy number in transportation area remain in the use of stochastic logit approach.  These studies overlook the differences between fuzzy and stochastic in nature.  Therefore, some problem is over-simplified and not really solved in realistic uncertain circumstances.

The traffic environment is complicated and highly stochastic due to uncertain and changing key factors.  When facing several alternatives, decision makers based on their preferences usually make choice to maximize their benefits, all of which in a macro scale displays uncertain outcomes.  In the past decade, the stochastic model, such as logit model or probit model, is the exclusive tool to solve the uncertain problem in the field of transportation.  While traffic information is both fuzzy and stochastic, there is not any theoretical way to interpret this situation right now.

In this study, we propose a new approach to comparing fuzzy numbers motivated by a probabilitic view of the underling uncertainty.  We discuss the effect of decision attitude and show that this approach is particularly useful for aiding decision makers while information is fuzzy. Finally, we try to use this new approach in traffic assignment problem in Tamsui Area.