Chapter 5

Results and Discussion

Our study proposed a new method for constructing a model for motorcycle traffic flow at mixed traffic. In this chapter, we act the preliminary test to check if the motorcycle traffic flow is the chaotic firstly. Next, we start to work the first part of CANN test. Then, we experiment and compare with difference network frame to define as the optimal CANN model for motorcycle traffic flow at mixed traffic. Finally, we verify the CANN and compare with the other analysis of motorcycle traffic flow to ensure the veracity of our model.

5.1 Preliminary Test

According to the literature review and video observation, we originally hypothesized that the motorcycle traffic flow is chaotic. To ascertain the assumption, the preliminary test of chaos analysis for macroscopic motorcycle traffic flow data was administered. The test worked into following two points. First, we gathered the relative time series data of macroscopic motorcycle traffic flow by re-watching video in the laboratory. These factors included density, velocity and flow at 7/25/2002 at motorcycle exclusive lane of the Hua Chiang bridge. Second, we calculated each parameter’s correlation integral function and fractal dimension to understand if the motorcycle traffic flow exists the chaotic phenomenon.

In the first place, we marked there lines such as line 1, line 2, line 3 on the TV screen. Then the number of motorcycles between line 1 and line 3 in every 5 seconds, the number of motorcycles processing line 2 in every 5 second, and the average time of 5 motorcycles chosen randomly processing line 2 in every 5 seconds were recorded. Finally, we regarded these as density, flow and speed and got the 866 efficient samples for calculating.

In the second place, we calculated the autocorrelation function. This Figure 5.4 presented the autocorrelation function of each variable with unit time (5s). As can be seen from the figure, there was no cut-off phenomenon after lag n or damped sine wave phenomenon of the autocorrelation function of variables. It indicated that two consequences. One is that the variables (k, q, v) of motorcycle traffic flow are not suit for conventional time series data. The other is these variables of the motorcycle traffic flow may not use the time series model such as AR, MA and ARIMA to foretell.

Furthermore, we computed the correlation dimension of each variable and drew the correlation dimension V.S. embedding dimension diagram. The correlation dimension of density, flow and speed over embedding dimension are shown in Fig 5.1, 5.2, 5.3. These figures summarized that the more the embedding dimension, the more fixed correlation dimension is. These results provided substantial evidence for original hypothesis. That is to say, the conclusion of preliminary analysis demonstrates that the motorcycle traffic flow indeed exist the chaotic behavior and we can apply the parameter in the chaotic system to explain the dynamic change with time of motorcycle traffic flow.

5.2 Chaos Analysis

This section started to act the first part of CANN work. The experimental study was design to calculate the embedding dimension, average predicted time scale, Renvi entropy and attractor dimension of each relative factor of motorcycle traffic flow at mixed traffic including the difference speed between auto and motorcycle (ams), area occupation rate (aor), speed of auto vehicle (as), horizontal speed of motorcycle (ms), motorcycle acceleration (ma), mixed rate (mr), total vehicle amount (ta), motorcycle disarray rate (man), motor speed variance (ms_st). The algorithm process was from the Eq. (3.14) to Eq. (3.37).

The chaos analysis of each factor is presented in Table 5.1. As can be seen from the table, it showed the four important results. For one thing, the most of Renyi entropy of each factor were between zero and infinite, which indicated that most of the factors of motorcycle traffic flow at mixed traffic were chaotic except one factor, motorcycle disarray (man). For second thing, most of the factors existed the chaotic attractor, which correlation dimensions were fixed after certain embedding dimension, as shown in Fig. 5.5, Fig 5.6, Fig 5.7, and Fig 5.8. It indicated that the finding were in substantial agreement with those of Renyi entropy analysis.

What is more, all of the predicted time scales were under one. Conversing to the time in the real world, these data suggested that the predicted time scale of each factor was under 0.5 second, which was the unit time of our system. The finding appeared that the short-term prediction of motorcycle traffic flow at mixed traffic is expected, not long-term prediction and the theoretical assumption agrees well with the experiment calculating outcome. However, the result was difficult to practice, so we will apply the 0.5 second to be the unit time in our system.

One final point is that the attractor dimensions of most factors were independent of the difference time scale if the factor was chaotic. On the other words, they not only had the self-similar feature on time scale but also existed the of obvious fractal structure. So it implied that the factor of motorcycle traffic flow at mixed traffic may be analyzed and solved by fractal dimension techniques. But the motorcycle acceleration (ma) was the exception to the rule.

According to the table 5.1 shown, the value of attractor dimension of motorcycle acceleration (ma) when time lag was 5 second was lower than that of the others. A possible explanation for this is that the number of the sample was not enough to act the chaos analysis and influenced on the result of motorcycle acceleration (ma). Moreover, we considered that the motorcycle acceleration (ma) was a more complicated factor than other factor because its attractor dimension was the highest in the all factors. Eventually the motorcycle acceleration (ma) had a severe effect with the number of the sample decreasing. However, we still lacked the definite on the theory explanation, so there is room for argument on this point. For simplifying our design system, we will assumed that the motorcycle traffic flow at mixed traffic had the self-similar feature on time scale and existed the of obvious fractal structure at under analysis.

About the motorcycle disarray rate (man), the only factor is not chaotic as the time lag was 0.5 second and 1 second, as shown in Fig. 5.9 and Fig. 5.10. Moreover, because all of correlation integral functions were 1, the attractor dimension of motorcycle disarray rate (man) did not exist as the time lag was 5 seconds. These findings displayed that the change of motorcycle anarchy (man) due to exterior factors, not its nonlinear structure. This data strongly supported Wang’s (2003) suggestion that the horizontal displacement seemed to be influenced by exterior surrounding such as overtaking, parking, or bus stop.

Hence, although we considered the motorcycle disarray rate (man) as the most important factor to illustrate the unique characteristic of motorcycle traffic flow at mixed traffic, this factor could not include in the element of CANN system. To solve problem, we try to fine the new parameters to replace the motorcycle disarray rate (man). Then, because of themv=msx2+msy2 and man=ΔmsyΔmsx∗10, we thought that we could foretell  the motorcycle disarray (man) indirectly by prediction of total motor velocity.  Finally we add the factor, the total motor velocity, to construct the model.

Besides, the results of compared with attractor dimension of pure motorcycle traffic flow and motorcycle traffic flow at mixed traffic is shown in Table 5.2. The attractor dimensions of factors of pure motorcycle traffic flow were significant higher than that of motorcycle traffic flow at mixed traffic. These results can be explained by assuming that purifying the motorcycle traffic flow caused the internal structure of traffic flow in phase space to complicate. For instance, the speed of pure motorcycle traffic flow needed more factors to illustrate the dynamic characteristic of the factor.

Now, we already fined the each average predicted time scale to define as the length of the forecasting vector and size of moving-window. We will construction CANN model and experiment the optimal CANN model frame.

5.3 CANN Model Construction

After the chaos analysis, the chaos artificial neural network was designed to construct for forecasting the motorcycle traffic flow at mixed flow. There were several aims to accomplish in this part. The first point was to understand the spatiotemporal relationship of each variable. The second point was to test the efficacy and accuracy of difference algorithm. The third was to test the efficacy and accuracy of difference network frame. Finally, we tested efficacy of difference network learning epochs and defined the optimal frame of CANN.

5.3.1 Regression Analysis

Before the CANN test, there were two important preparing works. One is to ensure that the linear method or regression analysis could not solve the “complicate ” problem. If the determination coefficient of the no hidden-layer network was higher than that of one or two hidden-layer network, there was no reason that using the complex network replaced the statistic formulation. It was explored in comparing with the consecutive prediction of back- propagation network of supervised learning network constantly. So we created the regression model according the Table 3.5 and calculated their determination coefficients. These results were presented as Table 5.3. As the table shown, there were no any determination coefficients above 0.6. We usually adjusted that the model was well when its determination coefficients exceed 0.6. It meant that the hidden-layer network may be the better than linear method to solve the problem about motorcycle traffic flow at mixed traffic.

5.3.2 CANN Test

The other is to define the basic frame of test network. According to the above-mention analysis, we already fined the each average predicted time scale to define as the length of the forecasting vector and size of moving-window. However, because the embedding dimensions of each factor were not only one value, choosing network frame confused us once. Ultimately, we chose the median to be the size of moving-window to proceed initial test for simplifying test. The frames of each model were shown in Table 5.4.

Now, we could administer formal test. The test may be divided into three subtests.  The first experiment was administrated to compare with the difference type models if these output layer factor was also in the input layer and test the efficacy and accuracy of difference algorithm for finding auto spatiotemporal relationship of variables and optimal algorithm. About the algorithm selecting, we applied two famous type algorithms used in the back-propagation network including Heuristic method and Numerical method. Then the Heuristic method included variables learning rate back-propagation algorithm and resilient back-propagation algorithm. The Numerical method included conjugate gradient algorithm, Quasi-Newton method, and Levenber-marquardt algorithm. Finally, we chose the variables learning rate algorithm, resilient algorithm and Levenber-marquardt algorithm to be the testing algorithm. Moreover, because the commands of variables learning rate algorithm, resilient algorithm and Levenber-marquardt algorithm were trainda, trainrp, and trainlm in the MATLAB program, we will use the commands to replace the complete denotation at upper composition.

Furthermore, all of the training parameters of initial test network explored the original set-up value in Matlab program. For avoiding the over-training, the epoch of each test was 1000. About the conversing test and verifying test, we evaluated acceptable level of the network by mean square root and scatter diagram. The range of mean square error was from 0 to 1. The formulation was presented as follow.

(5.1)

Where the Ajn is the estimating output values of thejth output element of the nth sample, which Ajn=(Yi,Yi+τ,⋯,Yi+mτ−1), Tjn is the original output values of thejth output element of the nth sample, which Tjn=(Yi,Yi+τ,⋯,Yi+mτ−1), M is the number of sample, and N is the number of the element in the output layer.

The scatter diagram was that the x-axis was the original output values and the y-axis was estimating output values. If the outcomes of each sample located at the diagonal, it demonstrated that the forecasting results were well.

Besides, correlation coefficient was a quantifiable index to measure the collected condition at the diagonal of the scatter diagram. Its relative formulations were shown at follows:

ρ=(TiAi)−n∗μT∗μA(n−1)∗σT∗σA							    (5.2)

σT=i(Ti−μT)(n−1)									(5.3)

σT=i(Ti−μT)(n−1)									(5.4)

Where the σT is the standard deviation of original output values, the σA is the standard deviation of estimating output values, the μT is mean of the original output values and the μA is mean of the estimating output values. The range of the correlation coefficient was from -1 to 1. If the correlation coefficient was above 0.8, it indicated that the forecasting results were well.

The table 5.5 showed the mean square error of each model and the table 5.6 displayed the correlation coefficient of each model after training. Else, we made the Fig. 5.11 and Fig. 5.12 by data of table 5.6 to illustrate analysis more clearly. As the 5.5 tables shown, all models had the well conversion. Besides, the Fig 5.11 depict that average correlation coefficient of trainrp was the highest in the all algorithm. Moreover, although difference of the forecasting ability between trainlm and trainrp was small, the program running time of trainlm was almost 10 times to that of trainrp. It is clear that the resilient back-propagation algorithm was the better algorithm in our system for considering the accuracy and efficiency.

Then, because the characteristics of motorcycle traffic flow at mixed traffic were influence by not only contemporary traffic flow sounding but also antecedent environment, we needed to understand the auto-spatiotemporal relationship of factors of motorcycle traffic flow at mixed traffic. The all models may be divided into two main types, which the type I models only existed the exterior environment factors in the input layer and the type II models have the output factor in the input element. The Fig. 5.12 depicted that the line of type II models was almost above the line of type I models except trainda. The finding showed that the forecasting ability of type I was better than that of type II. On the other words, all of the factors of motorcycle traffic flow at mixed traffic had conspicuous auto-spatiotemporal relationships. From this point one may say that the difference speed between auto and motorcycle (ams) was most closely related to itself among all factors.

Next, the second experiment was administrated to define the optimal CANN frame. Traditionally, when the number of the hidden layers was one or two, the network had better conversing ability. The excessive number of the hidden layers let the network too complex to converse because it caused the local minimum condition. So the one or two hidden layer was enough to reflect the interaction of element in the input layer and prevent the above mentioned condition from happening. Besides, choosing the appreciate number of the elements in the hidden layer was also important for considering the quality and executing cost. General speaking, the principles were shown as follows:

(5.5)

Nhid=(NinpNout)12								(5.6)

Where Nhid is the number of elements in the hidden layer, Ninp is the number of elements in the input layer and Nout is the number of elements in the output layer. The Table 5.7, The table 5.8, Fig. 5.13 and Fig. 5.14 displayed the results of the second experiment.

As can be seen from the Fig. 5.13, the average correlation coefficient of all models was slight rose as the number of the number of first hidden layer increasing. After the 10 elements, the average correlation coefficient was steady at the range from 0.86 to 0.87. So we thank that the 15 was the better choose to the number of elements of hidden layer. Next, we test the benefit of the second hidden layer according to the judgment from above. The finding of the experiment was that the average correlation coefficient at the 5 elements of second hidden layer was the highest point as shown in Fig 5.14.

Finally, the third experiment was administrated to understand the efficacy and accuracy of difference epoch. For facilitating the test, we only test one of the models, model_1_b, to be out example. The Table 5.9 showed the correlation coefficient, mean square error, and executing time. Overall, the findings of the test suggested that accuracy was correlated with the iterative epoch but program executing time also increased as the accuracy increasing. The speaking results, unless experimentalist considered that the accuracy of the test was more important than other objectives, 1000 epoch is enough in CANN. On account of the above analysis, we finally defined the optimal frame of CANN, which can be schematized as Table 5.10.

5.4 Choosing Optimal Delay Time

We already knew that choosing optimal delay time τ was a crucial thing to influence the quality of reconstructed phase space in Chapter 3 above. In this section, we would try to define as the most optimal delay time in our CANN model. There were three methods to help us choosing the optimal delay time including observing the correlation dimensions V.S. embedding dimension diagrams, calculating the autocorrelation function, and comparing with accuracy of difference delay time models.

5.4.1 Correlation Dimensions and Autocorrelation Function Observation

First, Fig. 5.6, Fig 5.7 and Fig 5.8 were the correlation dimensions V.S. embedding dimension diagrams of each factor with difference time lag. These depicted that most of correlation dimensions of factors were fixed after certain embedding dimension; however, these correlation dimensions were more scattered when the embedding dimension increased. It seemed that the chaotic characteristic disappeared gradually as the delay time rose.

Second, we calculated the autocorrelation function of each factor of motorcycle traffic flow at mixed traffic and depicted the results as Fig 5.15 (a), (b), (c), (d), (e), (f), (g), (h) and (i). These figures expressed that there were sudden dip at all of the figures initially, then the autocorrelation functions were shown as cycle oscillation by the similar structure. The findings suggested that there were no cut-off after lag n or damped sine wave of the autocorrelation of variables. Furthermore, most of time, which ACF equaled to null, were upper 20 seconds as shown in table 5.11. It was likely that the time which ACF equaled to null were too long to demonstrate the dynamic character of motorcycle. The point of view helped account for that applying the time when autocorrelation function first passes through the null was not the appropriate value of the optimal delay time.

5.4.2 Comparing with Accuracy of Difference Delay Time Models

Eventually, we compared with accuracy of difference delay time models. All of the outcomes were represented as the Table 5.12 and Fig. 5.16(a) and (b). Our results indicated that the model of which delay time was one unite time had better and stable forecasting ability. One reason for this could be that the optimal delay time was the 0.5 second in CANN model for motorcycle traffic flow at mixed traffic.

5.5 Verification

5.5.1 Comparing with single variable analysis

Wang (2003) analyzed the relationship between the mixed ratio and motorcycle disarray rate. Its formulation is and the correlation coefficient was 0.5451. The results in this study indicated that the motorcycle disarray was negatively correlated with the mixed ratio at mixed traffic. The outcome can be explained by that the separating traffic flow cloud decrease the motorcycle disarray and promote mixed traffic flow order. In addition, Hsu (2003) also analyzed the controlling motorcycle strategy by the mixed ratio and motorcycle disarray rate. We reanalyzed the data of Hsu (2003) and find that the formulation is y = -118.75x6 + 353.46x5 - 403.72x4 + 222.37x3 - 62.202x2 + 9.0175x - 0.5215 and the correlation coefficient was 0.7143, shown in Fig. 5.17. The findings indicated that the certain mixed ratio at mixed traffic causes the motorcycle disarray rate increasing.

Comparing with study of Wang (2003) and Hsu (2003), our findings are in strong agreement with those of those studies. However, there were more advantages in our model than Wang analysis. First, the predicting accuracy of CANN model for motorcycle traffic flow at mixed ratio was greater than the reported by Wang (2003). Second, we may predict the other factors which were the indices to illustrate the homogeneity of motorcycle traffic flow such as the difference speed between auto and motorcycle (ams) and motor speed variance (ms_st). It should be concluded from what has been said above, that the CANN model analysis is better than single variable analysis.

5.5.2 Network Verification

After gathering data, there were 1922 samples which are applied to construct chaos neural network model for motorcycle traffic flow at mixed traffic. To ascertain the practical usage of CANN model, we needed to compare with the MSE and correlation coefficient of training samples and testing samples. According to tradition, we choose three-quarter data to be the training samples and fourth data to be the testing samples. Before comparing, we understand whether there is any marked difference between the training samples and testing samples firstly. The Table 5.13 presented that the distributions of training samples and testing samples are not extreme different. So we still considered our testing samples reasonable. Next, we compared with the MSE and correlation coefficient of training samples and testing samples. The results in Table 5.14 appeared that the conversing condition is well. Because the errors of testing samples is near and bigger than that of training samples. This finding seemed to show three things. One is that the number of samples is enough. Two is that the samples are exemplary. Finally, it meant that the chaos artificial neural network for motorcycle traffic flow at mixed traffic is practical.

5.5.3 Comparing with Conventional the Spatiotemporal Neural Network

Traditionally, the spatiotemporal neural network also could treat the temporal data and spatial data such as the pure motorcycle traffic flow data. However, there were three shortcomings in this model. First, its algorithm was more difficult than general back-propagation network. Second, executing program need more memory and time than general back-propagation network. Finally, it is difficult to define the appropriate size of moving-window. So many researchers applied try and error to find the better the frame of the spatiotemporal neural network.

According reduction, using the chaos analysis of CANN not only can illustrate the physical complexity of each parameter of motorcycle traffic flow at mixed traffic but also can design the length of size of the moving-window by chaos analysis to capture the relationships of variables and the characteristics of the particle movement in the nonlinear dynamic system. So it may decrease the memory and time when we executed program.

From the theoretical view, there were indeed a lot of the adjust weights in the artificial neural network, so the famous drawback of the spatiotemporal neural network was to need a considerable of the memory and cost long time for executing program. For example, it needed Ninp∗Nhid+Nhid∗Noutbits to memory the adjusting weights for one leaning epoch in aNinp−Nhid−Nout network. If we used the real to record values, it needed 2(Ninp∗Nhid+Nhid∗Nout)bits to memory the adjusting weight and correcting value. The same observation applied that it needed 8(Ninp∗Nhid+Nhid∗Nout) bits to memory for one epoch to record float values. Because the algorithm of CANN was similar to that of the spatiotemporal neural network, the executing time of CANN also may be calculated by the same method. The formulation of one sample for one leaning epoch is presented as follows:

Nmul≈5∗Nhid∗Nout+4∗Ninp∗Nhid				 (5.7)

Ntransfer=Nhid+Nout                      (5.8)

Where Nmul is the multiplication operator, Ntransfer is the transferring function operator, Ninp is the number of elements in the input layer, Nhid is the number of elements in the hidden layer, and Nout is the number of elements in the output layer.  Because the optimal frame of our CANN model for motorcycle traffic flow at mixed traffic had two hidden layers, the formulation yields

Nmul≈5∗Nhid2∗Nout+4∗(Ninp∗Nhid1+Nhid2∗Nhid1)   (5.9)

Ntransfer=Nhid2+Nhid1+Nout                   (5.10)

Where theNhid1 is the number of elements in the first hidden layer and Nhid2 is the number of elements in the second hidden layer. The Table 5.15, Table 5.16, Fig. 5.18 and Fig. 5.19 showed the results about comparison of CANN and different frame of spatiotemporal neural network. The findings demonstrated that the CANN for motorcycle traffic flow at mixed traffic may reduce the input element and decrease the number of the multiplication operators markedly, but its predicting ability could approximate the model of which had more input information (spatiotemporal neural network model t=10) (see Table 5.17). There is a high level of agreement between the theoretical predictions and the experiment results.