Chapter 1

Introduction

  - Motivation

Motorcycle plays a crucial role in most of Asia countries, especial in Taiwan. According to statistic data of Institute by Transportation (MOTC), the present average ownership of motorcycles of per capita is about 0.5 in Taiwan, which is the highest ownership of motorcycle around the world. Therefore, high dense motorcycle traffic flow has become the distinctive phenomenon in Taiwan and cause serious traffic problems. So we need an applicative and useful motorcycle traffic flow model for searching the countermeasures to solve traffic problems at mixed traffic motorcycle.

It is obvious different of traffic flow characteristics between motorcycle and passenger car both microscopic and macroscopic. Motorcycle traffic analyses inherent high mobility and risk. For instance, the acceleration rate and the change of direction of motorcycle are usually grater than those of passenger car. This will suffer hobble to predict or simulate the dynamic behaviors of motorcycle’s drivers. Therefore, “motorcycle traffic ”usually is seen as the ringleader to traffic accident in the mixed traffic.

Although to provide exclusive motorcycle lane and motorcycle waiting zone indeed can help to regulate a part of motorcycle traffic flow and to alleviate some of the conflicts at intersections, these approaches about separating traffic flow still can not solve all of the motorcycle traffic problems. After all, in most traffic flow on urban street, the motorcycle traffic flow can not be completely purified because of many circumscriptions to establish motorcycle exclusive lane. On the other words, mostly motorcycle is driving under mixed traffic with other vehicles. It indicates that it is necessary to establish the predicting model of motorcycle traffic at mixed traffic for understanding and treating motorcycle traffic features.

In general, no matter passenger car or motorcycle, it is rather an overcome of a complex process of interaction by involving the driver, vehicle and road environment. Traditionally, most of researchers in the past aimed to investigate dynamics to demonstrate the behavior of passenger car such as car-following theory or kinetic equation. However, there is no mathematical model to present the motorcycle traffic flow at mixed traffic contains the mixed traffic conspicuous dynamic character.

The recent researches in chaos and fractal geometry have been conducted since 1960. Chaos theory has a big step because of its most important characteristics in terms of nonlinear dynamic analysis, can capture the hidden rules or autocorrelation of the similar random series data. Chaos has been applied to a lot of fields, including forecasting traffic volume. We consider that the chaotic system which is nonlinear and sensitive to initial value and adjust factors, can illustrate the conspicuous dynamic character of motorcycle traffic flow at mixed traffic. Its adoption can overcome the hassle about application of motorcycle traffic engineering. It means that we will discover the unknown force which contains nonlinear structure of motorcycle traffic flow at mixed traffic.

It is sometime difficult to discuss the advantages and disadvantages about using motorcycle because it is such a multifaceted topic. Some people seem to antagonize motorcycle because of its noise and exhaust, while others prefer to using motorcycle. In my opinion, if developing a low-pollution motorcycle, the motorcycle may enhance the traffic throughput in terms of accelerating rapidly and varying direction nimbly by safe control system. Finally I believe that motorcycle will become the efficient and safety vehicle at Taiwan urban.

  - Objectives

This study attempts to combine chaos theory and artificial neural network and establish a predicting model for motorcycle traffic flow at mixed traffic. The model includes new prediction model incorporating with chaos theory and neural net work.  It is called “CANN” not only utilize in optional diverse flow system in order to increase the volume and security on the road but also fine the thresholds of relative traffic parameters to control motorcycle traffic flow. Its application becomes major principles of motorcycle traffic engineering or motorcycle intelligent transportation system.

  - Scope

This study aimed to analyze the motorcycle traffic flow at mixed traffic at road section. The term “mixed traffic ”is that the traffic flow contains multi-vehicle which have different operation capability and driving ability. Moreover, it usually unmakes to separate driving space or act regulation by diverse flow. The road section is defined as the chosen section has enough distance from intersection to avoid the mixed traffic flow affected by downstream or upstream traffic signal.

  - Contents

According to above-mentioned objectives, the study contents are as follows.

- Observe motorcycle traffic flow at mixed traffic.

- Analyze the attributes of motorcycle traffic flow at mixed

traffic.

- Find out the relative parameters influencing motorcycle

traffic flow by literature review, watching the traffic flow

video and the Duncan’s multiple Range test.

- Detect the chaotic phenomenon of motorcycle traffic flow.

- Construct the chaos artificial neural network (CANN).

- Comparing with the results of CANN and other traditional

tests.

- Conclusion and suggestion.

  - Procedure

There are four ways to study the behavior the traffic flow, which are controlled test-track experiment, theoretical simulation, studies of phenomena, and computer simulation. This study applies the collected the data of traffic flow, and then to construct the motorcycle traffic flow model. The flowchart is presented as Fig. 1.1.

Problem Definition

Literature Review

Traffic Flow Photograph

Model Construction

Case Validation

Conclusion and Suggestion

Fig. 1.1 The Flowchart of This Study