Chapter 2

Literature Review

The literatures reviews fall into three categories. The first part describes the basic theorem of traffic flow and its application. The second part introduces the problems of motorcycle traffic flow, which help to identify the main factors. The third explains the chaotic theorem and some application of chaotic theorem at different fields. Some comments on these literatures are summarized.

2.1 The study method of traffic flow

There are four ways to study the behavior in traffic flow:

- Controlled Test-Track Experiment

It needs the ideal flied to test. It costs too much for collect the complete traffic flow data. Although it could be similar to real world, the testing result still can not comprise all traffic flow characteristics.

- Theoretical Simulation

On the assumption, it is using ex ante gathering data to establish the abstract model. The shortcoming of this method is that he reducing result only can explain the condition of traffic flow behavior and can not demonstrate the phenomena of behavior of independent vehicle.

- Observation Studies of Phenomena

The studies of phenomena only observe the characteristic of integrate behavior and can not establish complete model.

- Computer Simulation

The method can establish the abstract model by collecting traffic flow data and simulate the real traffic flow by programming computer program.

Generally speaking, the more popular way to study traffic flow in the past were the controlled test-track experiment and studies of phenomena. Especially for studying ally car following theory microscopic, most researchers get the factors by working experiments. For instances, GM (1960’s) car following studies, Hoefs (1972) and Wiedemann (1974) study of threshold of car following behavior are belong to that ways. These two methods must need a lot of labors, cost hung funds and take a long time so the methods are seldom used at resent years. Furthermore, there are a lot of advantages on Theoretical simulation method such as max generalization, various applications, and low cost, but the method still only can be used to simulate the certain condition if it is difficult to describe real traffic flow efficiently. Therefore, in this study, we will try to collect the data of traffic flow by VCR for analysis of basic traffic characteristic. It will be the best ideal method to study traffic flow. In this study, we will use this way to establish the motorcycle traffic flow.

For studying the traffic flow, theory can fall into three different categories to be used. One is macroscopic, and other is microscopic and the third type is mesoscopic.

- Macroscopic Traffic Flow Model

The fundamental assumption of macroscopic models is that aggregate behavior of a group of vehicles. On the other words, macroscopic the traffic flow is seen water current and shows the aggregate traffic variables by hydrokinetics. The parameters such as density, capacity, and speeds are related each other by relationships derived by considering the analogy between vehicular flow and flow of liquids and gases. Most macroscopic traffic flow models describe the dynamics of the density ,the velocity u,the flow q,and the relationship between them is q=k∗u. Some researchers investigate the speeds, flows, and densities from low quality time lapse, including Payne-type models (1979) or Helbing-type models (1996)

Due to their aggregate nature, the variables are easy to observe, valid and handle. However, the macroscopic model still suffers the drawback because they can not adequately distinguish individual vehicle characteristics and depict specific man oeuvres. Although there are more detailed macroscopic models such as the cell transmission models (Daganzo,1994) and gas kinetic equation(Bovey and Hoogendoorn,1999), it remains an unsettled questions. So, the model is often used by of which generalize and average result at planning net work, analyzing capacity, forecasting demand and evaluating study result.

- Microscopic Traffic Flow Model

In the contract, the term “microscopic modeling” is meaning to allow for tracing of individual vehicles right from entry into the net work until departure and for assigning to each vehicle type specific performance capabilities. Its parameters include maximum speeds, gap, acceleration rates and deceleration rates. In addition, this model is more different to validate because the behavior of drivers in the real world the model try to depict is hard to measure and observe. Despite these individual vehicle movements can be described by use of appropriate model that can be drawn for both lateral and longitudinal movements.

Comparing with macroscopic model, microscopic, this approach still remains the most logical option to study traffic flow and there are a number of articles about this. Summarized these articles, we can understand that the applications of microscopic concept are usually on traffic control and decision making.

- Mesoscopic Traffic Flow Model

∂∂t+v⋅∇r+Fm⋅∇vf(r,v,t)=(∂f∂t)collision					      (2.1)

The concept of mesoscopic traffic flow model locates middle between macroscopic and microscopic model. So it contains some characteristics both of two. It comprehends the fleet and various rate of traffic capacity of the dummy lane.

It seems reasonable to consider updating the traffic flow data through two types of organization, especially in traffic simulation. The data structure for establishing a traffic flow model can be one of them.

- Even-Scanning

If any accident should occur, the systems will scan and treat the incidents. So the scanning time of the system is not fixed and needs the clearly accidental definition. The method often use on the terminal organization system of airport or bunder.

- Time-Scanning

After the fixed interval, the system is scanned once time and updated the new attribution of every unit. The shorter scanning time is, the more closely the real world. Then, most of traffic flow studies use this way.

Based on the advantages and disadvantages, we consider that the disadvantages of certain methods are more significant than advantages. So the time-scanning and mesoscopic method is the best way to study the motorcycle traffic flow forecasting model.

2.2 The study of motorcycle traffic flow

For studying the motorcycle traffic flow, we must understand what are the main characteristics of behaviors of motorcyclists. Over the past few decades, a considerable number of studies have been made on motorcycle traffic flow in Taiwan. They can be classified into three groups. One group of studies is to study car following theory. The second group of the studies is to investigate the characteristics of motorcycles. The third group of study is to analyze traffic conflict study of motorcycle traffic flow.

2.2.1 The study of motorcycle traffic flow

“Car Following Theory” is to study the action of following drivers corresponding to the preceding vehicle. The rules become applicable when a vehicle in a traffic stream can not move freely and choose its own speed as well, but must adjust its speed due to the presence of other vehicles. Generally speaking, there are five distinct genres of car following, reviewed as follows:

- Restriction equation

The restriction equations proposed by Pipes, 1953 and Forbes 1963, were based on safe time and minimum distance headway, which to protect from the rear end collision. There are four restriction equations to describe the driving behavior, which includes spacing restriction, acceleration restriction, stopping and turning restriction. The spacing restriction equation is:

S=P+K1V1+K2Vt2−Vt2D∗C                            (2.2)

S:	the minimum safe headway

P: effective vehicle length

K1:	the reactive time of rear driver

K2: the constant, 1m/sec

Vt:	the speed of following vehicle

Vt′:	the speed of preceding vehicle

D:	the average deceleration of vehicle, where Vt−1>Vt−1' 	C=1

Vt−1≤Vt−1'	C=0

Some researchers incorporate the visual computer simulation technique and restriction equation to study motorcycle traffic engineering design. For example, and Hsu, Yar-Ming, et al. (2001) use visual computer simulation technique to establish the virtual system, which be based on the action of passenger cars and motorcycle by four restriction equations

- Two-Dimensional Traffic Flow Simulation

For simulating characteristics of mix traffic flow in Taiwan, the Two-Dimensional Traffic Simulation model is deduced for various sizes of vehicles. The fundamental assumption of two-dimensional traffic flow simulation is that motorcycle will proceed at normal acceleration when its condition is allowed by arrayal of other vehicles.  Tay-Lin Hwang, et al. (1994) proposed the new concepts about dynamic length width of vehicle, straight-line restriction, oblique motion restriction, the restriction of lane width, and proceeded logic choice to show the phenomenon of proceeding vehicle at two-dimension. Yu-Ju Chou, et al. (2000) developed collision avoidance strategy and collision avoidance system which use Two-Dimensional Traffic Flow Simulation in mix traffic flow.

The behavior of motorcycle is anyway not closed to the car with following maneuver side by side by passing behavior model is proposed by Tien-Pen Hsu, et al. (1994), which was to fine the phenomenon that the drivers of motorcycle will change direction rubbery and adjust the lateral net space when proceeding. The situation is defined as ‘’bypassing behavior”. It demonstrates that the motorcycle can keep shorter fore-and-after gap and gain the independent driving space by lateral distance.

- The Study of Neural Network and Fuzzy Theory

Recently, fuzzy theory and neural network have both been applied to various fields. Each of them could be exploited imitate the thinking of human being and suited for simulating the reaction and stimulus of passengers cars. So some researchers construct the car following model.  Kakuchi, et al. (1993) developed the fuzzy car-following model of which input data are relative speeds, distance between two vehicle, and alteration of speeds of advance vehicle. Lawrence W. Lan, et al. (1994) added the consideration about optimal distance and avoided the fuzzy model form the unstable oscillation of speeds of vehicle. There are some relative researches such as by Tay-Lin Hwang (1994), by Yuar-Nan Fu (1995), and by Yin-Biau Liu (1996).

However, since the motorcycle is not a major mode in other countries, the development of traffic flow models in abroad has been confined only to automobiles. There is not any model developing for motorcycle in the other countries. On the other hand, Yu-Jui Lin, et al. (2002) had tried to establish the motorcycle traffic flow model by using neural networks as well as its practical usage in the real world traffic flow conditions.

- Behavioural Threshold Model

The foundation of the assumption of behavioral threshold model is that the following vehicle will accelerate to decrease gap or decelerate to increase gap, depending on the distance to the preceding vehicle. For instance, the driver will thinks that the distance is too far to be affected, when the gap is enough long. In the contrast, if the distance is too short, if it will affect the driving behavior, and then automobile will oscillate from accelerating to decelerate, (Hoefs 1972).

Winedamann, et al. (1974) developed a physiology and psychology model for microscopic traffic flow simulation. Shih-Chuan Chen, (1992) studied the Behavioural Threshold Model of motorcycle of mix traffic flow.

- Stimulus-Response Equation

In the 1950s, a number of researchers’ effort (Picking 1958, Herman 1959, Gazis 1961) focused on the filed experiments and develop a series of models on the basis of the response being a function of stimulus and sensitivity parameter, which related with the relative acceleration and deceleration between two near by vehicles. The response is the braking or the acceleration of the following vehicle, delayed by a overall reaction time T; then, the sensitivity term was initially a dimensionless constant but was revised progressively to finally cover the distance headway and the speed of the following car.

2.2.2 The characteristics of motorcycle vehicle

Most of the traffic flow models have been constructed for automobiles. But Taiwan has more than 11 million motorcycles, which shares 70% of traffic volume in Taiwan. To establishing the mix traffic flow model is necessary. For this purpose, it is important to understand the basic characteristics of motorcycles vehicle firstly.

1. Size of motorcycle

They may be divided into three types based on drivers or speeds of motorcycles including physical size of motorcycle, static size of motorcycle, and dynamic size of motorcycle. The physical size demonstrates that the space is occupied by the motorcycle body itself. The static size means that it comprises driver and motorcycle body stop on the street. Finally, the motorcyclists keep the other vehicles away from colliding to maintain a gap and the static space is different as dynamic size.

2. Width of motorcycle lane

When motorcycle is proceeding, its position changing could not be gathering in original direction. In general, the required width of motorcycle lane depends on the speed of motorcycle. The faster motorcycle needs the broader the motorcycle lane. Tun-Yuan Hsu, et al.(2001) defined as the dummy lane meter width for motorcycle, which considered the static width of motorcycle in mix traffic flow, to show the horizontal movement.

2.2.3 Traffic Confliction of Motorcycle Traffic Flow

Perkins and Harris (1967) provided a better insight to causal factors and identified of “traffic conflicts”. The definition agreed upon for “traffic conflict is an observable situation in which two or more road users approach each other in space and time to such an extant that a collision is imminent if their movements remain unchanged.”(Amundsen and Hyden, 1977). Horst and Hogema (1993) established a relationship between time to collision (TTC) and the severity of traffic conflicts. Similar examples are abundant such as Post-encroachment time (Allen, 1978) and gap time (Glauz and Miglegz, 1980) etc.

Most of the researches related to traffic conflict technique have been carried out in the countries whose traffic stream consists of mainly passenger vehicles and therefore the design of traffic conflict technique reflects the situation in relatively homogeneous traffic; however, Taiwan presents a very different mixed or heterogeneous of traffic situation compared to other countries. There are many scholars to study on the phenomenon mixed traffic stream phenomenon in Taiwan.

Jiann Chyuan Ju, et al. (1998) exploreed the reaction of motorcyclist as facing the traffic conflicts based on the established conceptual framework and experimental survey. The stimulus variables of study include TTC (time to collision) and the width of obstacle. The conclusion demonstrates that traffic-flow analysis should not ignore motorcyclists’ perception of danger.

Yu-Jui Lin (2002) described that the driving maneuver behavior of motorcyclists is more sensitive than that of automobile. Yuan Xiang Ye and Tien Pen Hsu (1994) established the direction disarray rate to show the horizontal displacement of microscopic motorcycle traffic flow. Outreaching the concept about the direction disarray (DA), Tun Yuan Hsu, (2001) found that “Dummy lane variation of motorcycle using (ΔDL)” is established as an index of measured the motorcycle traffic flow.

2.3 Application of Chaos Theory

For the study of dynamics, began and ended with Newton’s Law of F=ma, considering that if the forces between particles and their initial positions and velocities were given, one could predict the motion or history of a system forever into future, when given a enough power; However, it has been proven recently that the motion of very simple dynamical systems is not always able to be predicted. Such motion is labeled chaotic has thrown a new light on the modern scientific subject.

Chaos is a Greek verb which means to gape open and often refers to the primeval emptiness of the universe before things come into being. In the physical sciences, the existence of chaotic or unpredictable motions from the classical equations of physics was introduces by Poincar. Considering the following excerpt from this essay on Science and Method:

It may happen that small differences in the initial conditions produce very great ones in the final phenomena. A small error in the former will produce an enormous error in the latter. Prediction becomes impossible.

Chaotic is a term assigned to that class of motions in deterministic physical and mathematical systems whose time history has a sensitive dependence on initial condition. At present, due to new measuring method such as fractal dimension and Lyapunov exponents, chaotic phenomenon is not difficult to quantify and can be observed in many physical system.

2.3.1Chaotic phenomenon of car-following model

Disbro and Frame, et al. (1989) explored the chaotic phenomenon of GM car-following model. Because of its maximum Lyapunov exponents greater than zero, they consider that the GM car-following model exist the chaotic phenomenon. Comparing to the result of Disbro and Frame’s study, Jarrett and Zhang (1993) analyzed one of parameter of car-following model (α=165.3, m=0, l=2) and discovered its attractor not the strange attractor and has not chaotic phenomenon. So the chaotic phenomenon of GM car-following has become a subject of controversy. Until 1996 and 1998, that Addision and Low analyze the corrected car-following model and ascertained that there is the chaotic phenomenon only at certain range of parameters.

Ui Kie Hung, et al. (1996) predicted out the reaction time plays a key role in the car-following model, and a regular periodic perturbation to the equilibrium state of the GM car-following model will generate a chaotic oscillations. Chia Shiou Chou, et al. (1997) studied the traffic wave equations by using chaotic theory. It shows that there is a critical value large than that the solution of the dynamic traffic model is stable and less than that which the motion becomes highly unstable.

2.3.2Chaotic phenomenon of traffic volume

Dendrinos (1994) checked the Hausdorff dimension of traffic volume and found that there is fractal structure in the traffic volume from view point both of macroscopic and microscopic. Lokibe, Koyama, and Taniguchi et al. (1997) calculated the maximum Lyapunov exponents of 4000 time- series traffic volume of freeway. The value is 0.343, and validated that the chaotic behavior existed. Shang Yi Lin (2001) applied phase space local approximation method to construct the urban traffic forecasting model and capture the nonlinear dynamics of traffic flow pattern in different time intervals such as five, ten, and fifteen minutes. The results of prediction show that correlation coefficients between observations and model outputs are greater than 0.6. The relative error of prediction also is located within a practically acceptable range.

2.3.3 Application on other fields

Chaotic analysis is adopted at many other fields such as physic, chemistry, medicinal, economics, or engineering. Some studies, on engineering systems, have developed the equations of system and use to analyze the chaotic phenomenon and bifurcations of certain equations, such as kinematics equation, nonlinear acoustic systems, and vibrations of buckled elastic structures. The others are the researches investigate chaotic behavior of time series data. Their methods include Brock-Dechert-Scheinkman, Rescaled Range Analysis, and Correlation Dimension etc.

For one example, in the first category, Jin Po Ye (1995) Jiun Jung Fu (1995) proposed a new chaotic predictor which contained the nearest neighbor concept of the local predictor (LP) and the artificial neural network (ANN). He shows in full detail that chaotic dynamics could be controlled to these extracted orbits. In the second category, Jiann Long Chen et al. (1998) applied the p phase space local approximation method to predict the hourly ozone concentrations. The results showed that the method can act the nonparametric short term prediction and solve the problem about gathering a considerable of atmospheric data.

Even though the chaos theory is one kind of mathematical methods to solve and explain the unexpected turbulent in a traditional linear system, many chaotic detecting methods still can not be suit for all of the time series systems. Therefore, so far, the study of chaos theory is still superficial need deeper consideration.