Chapter 4

Field Investigation

In order to analyses and understand the motorcycle traffic flow, we deduce the simple chaos artificial neural network using field observation data. Although field observation costs a lot of money, time, and labor, it is the best method to obtain the characteristics of traffic flow in detail and establish the database for the study. So in this study, field observation data are collected and used to construct for CANN of motorcycle traffic flow.

4.1 Field Observation

4.1.1 The Principle of Choosing Investigation Location

Because we need to analyze the behavior of motorcycle in the mixed traffic, the investigation location should contain the typical kind of mixed traffic flow. The main rules of choosing the investigation location must have acceptable typical traffic flow under prevail traffic environment, and so on.  There are four critical principles for choosing investigation location : (1) The lengths of investigated road section need to be long enough to gather the completed analysis data; (2) For simplifying the analysis condition, on the location, there are no intrusive factors or special geometry of roads which influence the prevailing traffic flow; (3) For gathering the normal and stable traffic flow, there are two important points: One is that the choosing section must have enough distance from intersection to avoid the mixed traffic flow affected by downstream or upstream traffic signal; Furthermore, there are no detectors or police  controlling the traffic in the section during the field observation; (4) For collecting the acceptable data (mixed traffic flow), the motorcycle exclusive lane and fast lane also are excluded from the observation location.

On the other hand, the place for setting the digital camera to take a video also is selected by following principles: (1) the view from this place to the traffic street on needs to avoid the covering by signboard and tree on the road; (2) Because the survey range should be not too short, the building chosen for setting the camera must be high enough; (3) and nearby building is near to road the section, in order to avoid the decline view.

4.1.2	Survey Procedure

After ascertaining the investigation items and method, we start to conduct the field observation. The flowchart of field observation is shown in Fig. 4-1. The investigation location is located chosen on Sec. 2, Jilung Rd., Taipei. In order to avoid the mixed traffic flow affected by traffic signal downstream or upstream too significantly, the road section located with a distance of 90 meters from upstream intersection and 80 meters distance from downstream. The total investigation location has the length approximate 60 meters. On the other hand, the survey time also eschews the rush hours, night time and the time interrupted by pedestrian. Finally, we take the video at AM 9:00 ~ 10:00 at Jan, 28, 2003. The sampled examples of the observed traffic are illustrated in Fig. 4-2, 4-3, and 4-4.

4.1.3 The treatment and transformation of survey data

We treat the documentary data through following four steps: (1) converting the digital image data, (2) reading the traffic flow data from the pictures, ( 3) choosing the suitable sample, (4) and programming the treating data code.

The first step is converting the digital image data. Because the original file is digital format, we just use the image capture card to store the digital image data to hard desk. Next, we apply the transformation software form dynamic image file ( *.mpeg ) to stable image file (*.bmp). The time interval is 0.5 second, in other that  the track of motorcycle can be identified more clearly. The distance driving by 0.5 seconds which is about 14 meters in every picture, and then we can avoid the too hung amount of the observing data.

Second, we start the data reading work. We watch the stable image pictures one by one, and record the coordination of every vehicle at every time lag. There are five procedures to get the samples. (1) First is amplifying the pictures to the appreciate size. (2) Two step is attaching a transparent plastic slice on the computer monitor. (3) Thrid step is remarking the x-coordinate and y-coordinate on the plastic slice. The unit of x-coordinate is 3 meters and the y-coordinate is 1.7 meter. The critical coordinate considers mainly whether there is enough space for motorcycles at the mixed traffic flow. (4) Fourth step is watching the stable image pictures one by one. (5) Fifth step is recording the two-dimensions of every vehicle at each time lag.

Third, for gathering all the effective traffic flow data, we need to pick out the ill-suited samples such as pulling alongside the road, and diverse or merge vehicles. Finally, we execute write computer program to calculate and treat the original data from two-dimension coordination to data set of model variables time series such as the speed difference between auto and motorcycle (ams), area occupation rate (aor), speed of auto vehicle (as), speed of motorcycle (ms), motorcycle acceleration (ma), mixed rate (mr), total vehicle amount (ta), motorcycle disarray rate (man), motor velocity (mv), motor speed variance (ms_st). The definition of the variables are showed as Table. 3.1 . Furthermore, we also design program to implement the Statistics analysis and Duncan’s multiple Range test in order to understand the relationship among two variables.

4.2 Depicture and Analysis of Observing data

4.2.1 Depicture of Video Observation

After observing digital video, there are a few results as follows.

- Because acceleration ability of motorcycle is better than passenger car, the average acceleration of motorcycle is greater than that of passenger car. Besides, the average variation speed of motorcycle is also greater than that of passenger car. It demonstrates that the motorcycle traffic flow is indeed more disorder the pure passenger car traffic flow.

- Motorcycle will proceed act including acceleration and diversion at the same time when the motorcycle’s driver feel danger.

- If the motorcycles are agglomerate and the density at unit lane space arrives certain value, all of the motorcycles will change speed and direction to readjust the composition of motorcycle glomeration.

- Due to “the big car effect”, the motorcycle is influenced by bus or truck to change speed and direction. So the more number of bus or truck on the road, the bigger average ancharcy of motorcycle at the road section is.

- We discover that certain variables of motorcycle traffic flow, including speed of motorcycle (ms), motorcycle acceleration (ma) and motor speed variance (ms_st), not only impacted by exterior factors such as area occupation rate (aor), speed of auto vehicle (as), mixed rate (mr), and total vehicle amount (ta). We guess it due to habits of difference drivers. Sometimes, the behavior of drivers most depends on their comportment, not the external surrounding. So it explicates the part of the change of certain variables of motorcycle traffic flow due to parameter itself.

- We find that if traffic flow at is non-uniform or homogeneous, the difference speed between auto and motorcycle (ams), motorcycle acceleration (ma), motorcycle disarray rate (man), and motor speed variance (ms_st) will have increasing trend.

- According to elemental observation, we find that the behavior of motorcycle changing direction is caused by external environment such as flow, density, or speed, not itself effect. It means that the behavior of changing lane do not exist unknown force which contains nonlinear structure. So we conjecture that the behavior of motorcycle changing direction should not have any chaotic phenomenon. Then we will try to use theory verification to re-explain the phenomenon of motorcycle from observation

4.2.2 Basic Statistics Analysis and Duncan’s multiple Range test for variables

In this section, we start with some basic analysis works, which falls into three categories. First, in order to check the universality of our survey data, we re-investigate the foundational traffic flow condition at bigger time scale to understand if there is any bias of our samples. Second, we program to basic statistics analysis of each variable, which are determined by literature review and video observation, including the difference speed between auto and motorcycle (ams), area occupation rate (aor), speed of auto vehicle (as), speed of motorcycle (ms), motorcycle acceleration (ma), mixed rate (mr), total vehicle amount (ta), motorcycle disarray rate (man), motor velocity (mv), motor speed variance (ms_st). Finally, we do Duncan’s multiple range test between two variables for finding out the relationship of two relative variables.

The Fig. 4.6 and Fig 4.7 illustrate the result of re-investigate foundational traffic flow at mixed traffic at bigger time scale. We can fine that most of the area occupation rate (aor) and the total vehicle amount (ta) time series demonstrate stable condition expect the total vehicle amount (ta) at peak hour. It shows that our survey data has universality, not stock enormously with difference time.

Besides, we also seek to capture the relationship of relative variables to the average anarchy of motorcycle at mixed traffic by Duncan’s multiple Range test for variables. This Table 4.1 shows that the average motorcycle disarray rate(man) at mixed traffic has the conspicuous grouping condition with the difference speed between auto and motorcycle (ams), the total vehicle amount (ta) and area occupied rate changing. On the other words, the average motorcycle disarray rate (man) could rise or fall sharply on certain values of the difference speed between auto and motorcycle (ams), the total vehicle amount (ta) and area occupation rate. So we may say that it is possible to build up the controlling the average motorcycle disarray rate (man) mechanism by the difference speed between auto and motorcycle (ams), the total vehicle amount (ta) and area occupation rate. On contrary, we also find that the other variables such as mixed rate (mr) and speed of motorcycle (ms) do not influence on the motorcycle disarray rate (man). It means that these two variables are not suit for being controlling factors.

The same observation and method applied that Table 4.2, 4.3, 4.4. We also may indicate by these charts what there are grouping condition at the difference speed between auto and motorcycle (ams), motorcycle acceleration (ma) and motor speed variance (ms_st) with area occupation rate (aor), speed of auto vehicle (as), speed of motorcycle (ms), mixed rate (mr), total vehicle amount (ta) increasing.

Summarized the result, we could apply Duncan’s multiple Range test to understand the relationship of difference external or internal variables at mixed traffic; however, because we can not avoid the bias, which is caused by no vehicle at continuous time, this conclusion is a considerable rough true for initiative analysis. We may further observe the spatiotemporal alteration of motorcycle traffic flow at mixed traffic when the chaos artificial neural network (CANN) builds up.

The relation of variables is disproportioned. The motorcycle traffic flow at mixed flow could be a simple, chaotic and dynamical system. To control the certain variables could arrest the increase of motorcycle anarchy and acceleration. To define the threshold could facilitate traffic safety.

Certain the Principle of Choosing Road

Choose Section and Exploration

Practice Preparing Work

Photo Mixed Traffic Flow

Treat the Gathering Basic Data

Pass Data

Application

Re-survey

Preceding Chaos Test

YES

NO

Input the CANN Analysis

Fig 4.1 The flow chart of survey procedure

Fig. 4.2 The e-map of survey location

Fig. 4.3 The stable image picture of investing mixed traffic

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

Fig. 4.4 The sketch of observing two-dimension coordination

Fig. 4.6 The line graph of mixed rate in every minute

Fig. 4.7 The line graph of total vehicles amount in every minute

Table 4.1 The Basic Statistics Analysis and Duncan’s multiple Range test for the difference speed between auto and motorcycle (ams), area occupation rate (aor), speed of motorcycle (ms), mixed rate (mr), total vehicle amount (ta) to motorcycle disarray rate (man)

| aor | man | variance | percentage | cumulative percentage | treat |
| --- | --- | --- | --- | --- | --- |
| 0.00 ~ 0.113 | 0.018 | 0.004 | 52.393% | 52.393% | A |
| 0.113 ~ 0.227 | 0.053 | 0.009 | 28.980% | 81.374% | B |
| 0.227 ~ 0.339 | 0.117 | 0.024 | 12.227% | 93.600% | C |
| 0.339 ~ 0.453 | 0.009 | 0.012 | 4.683% | 98.283% | A |
| 0.453 ~ 0.567 | 0.005 | 0.008 | 1.717% | 100.000% | A |
| ams | man | variance | percentage | cumulative percentage | treat |
| - 12.727 ~ - 6.154 | 0.069 | 0.009 | 4.115% | 4.115% | A |
| - 6.154 ~ 0.419 | 0.038 | 0.007 | 74.427% | 78.542% | B |
| 0.419 ~ 6.993 | 0.064 | 0.016 | 19.219% | 97.760% | A |
| 6.993 ~ 13.567 | 0.040 | 0.016 | 2.188% | 99.948% | B |
| 13.567 ~ 20.13 | 0.000 | 0.000 | 0.052% | 100.000% | none |
| ta | man | variance | percentage | cumulative percentage | treat |
| 0 ~ 5 | 0.001 | 0.004 | 45.525% | 45.525% | A |
| 5 ~ 10 | 0.006 | 0.013 | 31.113% | 76.639% | A |
| 10 ~ 14 | 0.009 | 0.011 | 11.759% | 88.398% | A |
| 14 ~ 19 | 0.008 | 0.008 | 6.139% | 94.537% | A |
| 19 ~ 23 | 0.106 | 0.012 | 5.463% | 100.000% | B |
| mr | man | variance | percentage | cumulative percentage | treat |
| 0.0 ~ 0.2 | 0.001 | 0.004 | 0.000% | 45.525% | A |
| 0.2 ~ 0.4 | 0.007 | 0.016 | 0.000% | 76.639% | A |
| 0.4 ~ 0.6 | 0.008 | 0.011 | 0.000% | 88.398% | A |
| 0.6 ~ 0.8 | 0.007 | 0.008 | 0.000% | 94.537% | A |
| 0.8 ~ 1.0 | 0.005 | 0.006 | 0.000% | 100.000% | A |
| ms | man | variance | percentage | cumulative percentage | treat |
| 0.000 ~ 5.455 | 0.002 | 0.007 | 0.000% | 45.525% | A |
| 5.455 ~ 10.909 | 0.009 | 0.015 | 0.000% | 76.639% | A |
| 10.909 ~ 16.363 | 0.004 | 0.004 | 0.000% | 88.398% | A |
| 16.363 ~ 21.828 | 0.002 | 0.002 | 0.000% | 94.537% | A |
| 21.818 ~ 27.273 | 0.007 | 0.008 | 0.000% | 100.000% | A |

Table 4.2 The Basic Statistics Analysis and Duncan’s multiple Range test for the difference speed between auto and motorcycle (ams), area occupation rate (aor), mixed rate (mr) and total vehicle amount (ta) to motorcycle acceleration (ma).

| ams | ma | variance | percentage | cumulative percentage | treat |
| --- | --- | --- | --- | --- | --- |
| - 12.727 ~ - 6.154 | 4.516 | 9.073 | 4.115% | 4.115 | A |
| - 6.154 ~ 0.419 | 2.338 | 10.617 | 74.427% | 78.542 | B |
| 0.419 ~ 6.993 | 3.913 | 13.710 | 19.219% | 97.760 | A |
| 6.993 ~ 13.567 | 1.975 | 8.481 | 2.188% | 99.948 | B |
| 13.567 ~ 20.13 | 0.000 | 0.000 | 0.052% | 100.000 | none |
| aor | ma | variance | percentage | cumulative percentage | treat |
| 0.00 ~ 0.113 | 1.838 | 8.318 | 52.393% | 52.393 | A |
| 0.113 ~ 0.227 | 3.690 | 8.975 | 28.980% | 81.374 | B |
| 0.227 ~ 0.339 | 3.491 | 4.738 | 12.227% | 93.600 | B |
| 0.339 ~ 0.453 | 3.782 | 3.461 | 4.683% | 98.283 | B |
| 0.453 ~ 0.567 | 3.954 | 2.722 | 1.717% | 100.000 | B |
| man | ma | variance | percentage | cumulative percentage | treat |
| 0.0 ~ 0.2 | 2.628 | 11.592 | 93.704% | 93.704% | A |
| 0.2 ~ 0.4 | 4.115 | 9.067 | 4.891% | 98.595% | B |
| 0.4 ~ 0.6 | 4.215 | 15.090 | 1.145% | 99.740% | B |
| 0.6 ~ 0.8 | 4.999 | 1.961 | 0.156% | 99.896% | B |
| 0.8 ~ 1.0 | 5.999 | 0.000 | 0.104% | 100.000% | C |
| mr | ma | variance | percentage | cumulative percentage | treat r |
| 0.0 ~ 0.2 | 0.376 | 6.220 | 38.241% | 38.241% | A |
| 0.2 ~ 0.4 | 3.589 | 10.684 | 18.366% | 56.608% | B |
| 0.4 ~ 0.6 | 4.399 | 12.684 | 21.176% | 77.784% | C |
| 0.6 ~ 0.8 | 4.678 | 9.576 | 13.996% | 91.779% | C |
| 0.8 ~ 1.0 | 4.192 | 9.653 | 8.221% | 100.000% | C |
| ta | ma | variance | percentage | cumulative percentage | treat |
| 0 ~ 5 | 1.194 | 8.498 | 45.525% | 38.241% | A |
| 5 ~ 10 | 3.906 | 14.736 | 31.113% | 56.608% | B |
| 10 ~ 14 | 4.331 | 6.473 | 11.759% | 77.784% | B |
| 14 ~ 19 | 4.081 | 3.132 | 6.139% | 91.779% | B |
| 19 ~ 23 | 3.769 | 4.501 | 5.463% | 100.000% | B |

Table 4.3 The Basic Statistics Analysis and Duncan’s multiple Range test for area occupation rate (aor), mixed rate (mr), total vehicle amount (ta) to the difference speed between auto and motorcycle (ams)

| aor | ams | variance | percentage | cumulative percentage | treat |
| --- | --- | --- | --- | --- | --- |
| 0.00 ~ 0.113 | 0.318 | 5.331 | 52.393 | 52.393 | A |
| 0.113 ~ 0.227 | -0.448 | 7.274 | 28.980 | 81.374 | B |
| 0.227 ~ 0.339 | -1.394 | 6.269 | 12.227 | 93.600 | C |
| 0.339 ~ 0.453 | -3.650 | 2.706 | 4.683 | 98.283 | D |
| 0.453 ~ 0.567 | -5.532 | 1.552 | 1.717 | 100.000 | E |
| ta | ams | variace | percentage | cumulative percentage | treat |
| 0 ~ 5 | 0.590 | 4.835 | 45.525 | 45.525 | A |
| 5 ~ 10 | -0.474 | 7.433 | 31.113 | 76.639 | B |
| 10 ~ 14 | -1.332 | 5.070 | 11.759 | 88.398 | C |
| 14 ~ 19 | -2.732 | 5.912 | 6.139 | 94.537 | D |
| 19 ~ 23 | -3.602 | 4.991 | 5.463 | 100.000 | E |
| mr | ams | variance | percentage | cumulative percentage | treat |
| 0.0 ~ 0.2 | 0.002 | 1.989 | 38.241 | 38.241 | A |
| 0.2 ~ 0.4 | -0.882 | 13.177 | 18.366 | 56.608 | B |
| 0.4 ~ 0.6 | -0.822 | 10.281 | 21.176 | 77.784 | A |
| 0.6 ~ 0.8 | -0.316 | 8.090 | 13.996 | 91.779 | A |
| 0.8 ~ 1.0 | -0.333 | 2.204 | 8.221 | 100.000 | A |

Table 4.4 The Basic Statistics Analysis and Duncan’s multiple Range test for area occupation rate (aor) speed of motorcycle (ms), mixed rate (mr) and total vehicle amount (ta) to motor speed variance (ms_st).

| aor | ms_st | variance | percentage | cumulative percentage | treat |
| --- | --- | --- | --- | --- | --- |
| 0.00 ~ 0.113 | 0.615 | 1.670 | 52.393 | 52.393 | A |
| 0.113 ~ 0.227 | 1.555 | 2.748 | 28.980 | 81.374 | B |
| 0.227 ~ 0.339 | 1.867 | 2.555 | 12.227 | 93.600 | C |
| 0.339 ~ 0.453 | 2.246 | 1.742 | 4.683 | 98.283 | D |
| 0.453 ~ 0.567 | 2.020 | 0.893 | 1.717 | 100.000 | C |
| mr | ma | variance | percentage | cumulative percentage | treat |
| 0.0 ~ 0.2 | 0.026 | 0.066 | 38.241 | 38.241 | A |
| 0.2 ~ 0.4 | 1.182 | 1.947 | 18.366 | 56.608 | B |
| 0.4 ~ 0.6 | 2.108 | 2.876 | 21.176 | 77.784 | C |
| 0.6 ~ 0.8 | 2.328 | 2.130 | 13.996 | 91.779 | C |
| 0.8 ~ 1.0 | 1.724 | 2.671 | 8.221 | 100.000 | D |
| ms | ms_st | variance | percentage | cumulative percentage | treat |
| 0.000 ~ 5.455 | 0.1591 | 0.334 | 42.0395 | 42.0395 | A |
| 5.455 ~ 10.909 | 1.4257 | 1.736 | 30.8533 | 72.8928 | B |
| 10.909 ~ 16.363 | 1.2500 | 2.212 | 20.8117 | 93.7045 | C |
| 16.363 ~ 21.828 | 0.7506 | 1.667 | 5.9834 | 99.6878 | B |
| 21.818 ~ 27.273 | 1.2513 | 5.050 | 0.3122 | 100.0000 | B |
| ta | ms_st | variance | percentage | cumulative percentage | treat |
| 0 ~ 5 | 0.153 | 0.470 | 45.525 | 45.525 | A |
| 5 ~ 10 | 1.076 | 1.931 | 31.113 | 76.639 | B |
| 10 ~ 14 | 1.848 | 1.762 | 11.759 | 88.398 | C |
| 14 ~ 19 | 1.840 | 0.909 | 6.139 | 94.537 | C |
| 19 ~ 23 | 1.487 | 0.641 | 5.463 | 100.000 | D |